3GPP TSG-RAN5 Meeting #100	R5-235613

# Toulouse, France, August 21st – 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0522 | rev | 1 | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for 2Tx switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 specified the 2Tx-2Tx UL switching ON/OFF time mask requirements. The corresponding test case needs to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding PICS for 2Tx-2Tx UL switching, so that the test applicability could be specified. The exising R16 UL switching capability is updated to ‘1Tx-2Tx’ UL switching to distinguish from R17 capability, which is aligned with TS 38.306. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PICS is missing in RAN5 specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-234955 |  |  |  |  |  |  |  |  |

<Start of Changes>

A.4.3.2	Physical Layer Baseline Implementation Capabilities

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item |  |  |  | UE Physical Layer Baseline Implementation Capabilities |  |  |  | Ref. |  |  |  | Release |  |  |  | Mnemonic |  |  |  | M |  |  |  | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release |  |  |  | Comments |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | Support PDSCH reception based on semi-persistent scheduling |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_downlinkSPS |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  | Support 256QAM for PDSCH for FR1 |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_pdsch_256QAM_FR1 |  |  |  | CY |  |  |  |  |  |  |  | Mandatory for non-RedCap UEs and optional for RedCap UEs. |  |  |  |
| 3 |  |  |  | Support 256QAM for PDSCH for at least one NR FR2 band |  |  |  | 38.306, 4.2.7.2 |  |  |  | Rel-15 |  |  |  | pc_pdsch_256QAM_FR2 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  | Support 256QAM for PUSCH for at least one NR FR1 band |  |  |  | 38.306, 4.2.7.2 |  |  |  | Rel-15 |  |  |  | pc_pusch_256QAM_FR1 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 4a |  |  |  | Support 256QAM for PUSCH for at least one NR FR2 band |  |  |  | 38.306, 4.2.7.2 |  |  |  | Rel-15 |  |  |  | pc_pusch_256QAM_FR2 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  | Support receiving PDSCH using PDSCH mapping type A with less than seven symbols |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_pdsch_MappingTypeA |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |
| 6 |  |  |  | Support receiving PDSCH using PDSCH mapping type B |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_pdsch_MappingTypeB |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  | Support resource allocation Type 0 for PUSCH |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_ra_Type0_PUSCH |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  | Support scaling factor 0.75 is applied to the band in the max data rate calculation |  |  |  | 38.306, 4.2.7 |  |  |  | Rel-15 |  |  |  | pc_scalingFactor0dot75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  | Support reconfiguration with sync using a contention free random access on PRACH resources that are associated with CSI-RS resources of the target cell |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_csi_RS_CFRA_ForHO |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  | Support Type 1 PUSCH transmissions with configured grant |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_configuredUL_GrantType1 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  | Support Type 2 PUSCH transmissions with configured grant |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_configuredUL_GrantType2 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  | Support PDSCH Reception when configured with higher layer parameter aggregationFactorDL > 1 |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_pdsch_RepetitionMultiSlots |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  | Supports supplemental uplink with dynamic switch (DCI based selection of PUSCH carrier) |  |  |  | 38.306, 4.2.7.7 |  |  |  | Rel-15 |  |  |  | pc_dynamicSwitchSUL |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  | Supports MIMO layers at the UE for PUSCH transmission with codebook precoding. UE indicating support of this feature shall also indicate support of PUSCH codebook coherency subset |  |  |  | 38.306, 4.2.7.8 |  |  |  | Rel-15 |  |  |  | pc_nrMIMO_CB_PUSCH |  |  |  | No |  |  |  |  |  |  |  | Set to true if maxNumberMIMO-LayersCB-PUSCH has value different from "oneLayer" |  |  |  |
| 15 |  |  |  | Supports MIMO layers at the UE for PUSCH transmission using non-codebook precoding |  |  |  | 38.306, 4.2.7.8 |  |  |  | Rel-15 |  |  |  | pc_nrMIMO_NonCB_PUSCH |  |  |  | No |  |  |  |  |  |  |  | Set to true if maxNumberMIMO-LayersNonCB-PUSCH has value different from "oneLayer" |  |  |  |
| 16 |  |  |  | Support receiving PDSCH with interleaved VRB-to-PRB mapping |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_interleavingVRB_ToPRB_PDSCH |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  | Support dynamic EN-DC power sharing for at least one EN-DC band combination_FR1 only |  |  |  | 38.306, 4.2.7.9 |  |  |  | Rel-15 |  |  |  | pc_dynamicPowerSharingENDC |  |  |  | Yes |  |  |  |  |  |  |  | If the UE supports this capability it will dynamically share the power between NR and LTE if P_LTE + P_NR > Pcmax. |  |  |  |
| 18 |  |  |  | Supports up to 10 search spaces in a SCell per BWP |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_maxNumberSearchSpaces |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  | Supports spatial bundling of HARQ-ACK bits carried on PUCCH or PUSCH per PUCCH group. With spatial bundling, two HARQ-ACK bits for a DL MIMO data is bundled into a single bit by logical "AND" operation |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_spatialBundlingHARQ_ACK |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  | Support alternative additional DMRS position for co-existence with LTE CRS |  |  |  | 38.306, 4.2.7.5 |  |  |  | Rel-15 |  |  |  | pc_additionalDMRS_DL_Alt |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 21 |  |  |  | Supports transmitting PUSCH scheduled by DCI format 0_0 or 0_1 when configured with higher layer parameter aggregationFactorIUL > 1 |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_pusch_RepetitionMultiSlots |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 22 |  |  |  | Support beam correspondence without UL beam sweeping |  |  |  | 38.306, 4.2.7.2 |  |  |  | Rel-15 |  |  |  | pc_beamCorrespondenceWithoutUL_BeamSweeping |  |  |  | Yes |  |  |  |  |  |  |  | A UE that can fulfil the requirements without UL beam sweeping then set the bit to 1. A UE that can fulfil the requirements with UL beam sweeping then set the bit to 0. |  |  |  |
| 22A |  |  |  | Support beam correspondence based on SSB |  |  |  | 38.306, 4.2.7.2 |  |  |  | Rel-16 |  |  |  | pc_beamCorrespondence_SSBbased |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 22B |  |  |  | Support beam correspondence based on CSI-RS |  |  |  | 38.306, 4.2.7.2 |  |  |  | Rel-16 |  |  |  | pc_beamCorrespondence_CSI-RSbased |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 23 |  |  |  | The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 8 Layers. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier |  |  |  | 38.306, 4.2.7.6 |  |  |  | Rel-15 |  |  |  | pc_maxNumberMIMO_LayersPDSCH_eightLayers |  |  |  | CY |  |  |  |  |  |  |  | Set to false if Table A.4.3.2-1/23A or 23B set to true. |  |  |  |
| 23A |  |  |  | The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 4 Layers. |  |  |  | 38.306, 4.2.7.6 |  |  |  | Rel-15 |  |  |  | pc_maxNumberMIMO_LayersPDSCH_fourLayers |  |  |  | CY |  |  |  |  |  |  |  | Set to false if Table A.4.3.2-1/23 or 23B set to true. |  |  |  |
| 23B |  |  |  | The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 2 Layers. |  |  |  | 38.306, 4.2.7.6 |  |  |  | Rel-15 |  |  |  | pc_maxNumberMIMO_LayersPDSCH_twoLayers |  |  |  | CY |  |  |  |  |  |  |  | Set to false if Table A.4.3.2-1/23 or 23A set to true. |  |  |  |
| 24 |  |  |  | Supports DCI and timer based active BWP switching delay type1 |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_bwp_SwitchingDelay_Type1 |  |  |  | No |  |  |  |  |  |  |  | It is mandatory to report one among BWP switching delay type1 or type 2 as supported |  |  |  |
| 24A |  |  |  | Supports DCI and timer based active BWP switching delay type2 |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_bwp_SwitchingDelay_Type2 |  |  |  | No |  |  |  |  |  |  |  | It is mandatory to report one among BWP switching delay type1 or type 2 as supported |  |  |  |
| 25A |  |  |  | Support modified MPR behaviour bit 0 |  |  |  | 38.306 4.2.7.2 |  |  |  | Rel-15 |  |  |  | pc_modifiedMPR_behaviour_bit0 |  |  |  | No |  |  |  |  |  |  |  | Applicable to FR2 bands n257, n258, n260 and n261 |  |  |  |
| 25 |  |  |  | Support modified MPR behaviour |  |  |  | 38.306 4.2.7.2 |  |  |  | Rel-15 |  |  |  | pc_modifiedMPR_behaviour |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 26 |  |  |  | Support dynamic switching between resource allocation Types 0 and 1 for PDSCH |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_dynamicSwitchRA_Type0_1_PDSCH |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 27 |  |  |  | Support dynamic switching between resource allocation Types 0 and 1 for PUSCH |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_dynamicSwitchRA_Type0_1_PUSCH |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  |  |  | Support almost contiguous UL CP-OFDM transmissions in FR1 |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_almostContiguousCP_OFDM_UL_FR1 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Support almost contiguous UL CP-OFDM transmissions in FR2 |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_almostContiguousCP_OFDM_UL_FR2 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 30 |  |  |  | Support dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1, and the minimum scheduling offset for PDSCH and aperiodic CSI-RS triggering offset (K0), and PUSCH (K2), and the extended value range for aperiodic CSI-RS triggering offset |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-16 |  |  |  | pc_crossSlotScheduling |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 31 |  |  |  | Supports pi/2-BPSK modulation scheme for PUSCH in FR1 |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_pusch_halfpiBPSK_FR1 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 31a |  |  |  | Supports pi/2-BPSK modulation scheme for PUSCH in FR2 |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-15 |  |  |  | pc_pusch_halfpiBPSK_FR2 |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 32 |  |  |  | Support multi-DCI based multi-TRP and support of fully/partially overlapping PDSCHs in time and non-overlapping in frequency |  |  |  | 38.306, 4.2.7.6 |  |  |  | Rel-16 |  |  |  | pc_multiDCI_MultiTRP_r16 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 33 |  |  |  | Support receiving PDSCH with resource mapping that excludes the REs determined by the higher layer configuration LTE-carrier configuring common RS |  |  |  | 38.306, 4.2.7.2 |  |  |  | Rel-15 |  |  |  | pc_rateMatchingLTE_CRS |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 34 |  |  |  | Support of BWP operation without bandwidth restriction |  |  |  | 38.306, 4.2.7.2 |  |  |  | Rel-15 |  |  |  | pc_bwp_WithoutRestriction |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 35 |  |  |  | Support of receiving SCell dormancy indication on SPCell using DCI format 2_6 outside the active time |  |  |  | 38.306, 4.2.7.4 |  |  |  | Rel-16 |  |  |  | pc_scellDormancyOutsideActiveTime_r16 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 36 |  |  |  | Supports pi/2-BPSK modulation scheme for power boosting in FR1 |  |  |  | 38.306, 4.2.7.2 |  |  |  | Rel-15 |  |  |  | pc_powerBoosting_pi2BPSK |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 37 |  |  |  | Support of dynamic 1Tx-2Tx UL Tx switching |  |  |  | 38.306, 4.2.7.1 |  |  |  | Rel-16 |  |  |  | pc_ULTxSwitchingBandPair |  |  |  | No |  |  |  |  |  |  |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2B.2.3.1-2 and Table A.4.3.2C.2-1 |  |  |  |
| 38 |  |  |  | Support uplink transmission power boost by suspension of in-band emission (IBE) in FR2 |  |  |  | 38.306 4.2.7.2 |  |  |  | Rel-16 |  |  |  | pc_mpr_PowerBoost_FR2 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 39 |  |  |  | Supports the alternative 64QAM MCS table for PDSCH |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-16 |  |  |  | pc_dl_64qam_mcs_tableAlt |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 40 |  |  |  | Supports the CQI table with target BLER of 10^-5 |  |  |  | 38.306, 4.2.7.10 |  |  |  | Rel-16 |  |  |  | pc_cqi_tableAlt |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 41 |  |  |  | Supports of single DCI based spatial division multiplexing scheme |  |  |  | 38.306, 4.2.7.5 |  |  |  | Rel-16 |  |  |  | pc_singledci_sdm |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| <Unchanged Lines Skipped> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 118 |  |  |  | Supports repetition of TB processing over multi-slot PUSCH |  |  |  | 38.306 4.2.7.2 |  |  |  | Rel-17 |  |  |  | pc_tb_ProcessingRepMultiSlotPUSCH_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 119 |  |  |  | Support SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH |  |  |  | 38.306, 4.2.7.5 |  |  |  | Rel-17 |  |  |  | pc_sfn_schemeA-r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 120 |  |  |  | Support SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH |  |  |  | 38.306, 4.2.7.5 |  |  |  | Rel-17 |  |  |  | pc_sfn_schemeB-r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 1yy |  |  |  | Support of dynamic 2Tx-2Tx UL Tx switching |  |  |  | 38.306, 4.2.7.1 |  |  |  | Rel-17 |  |  |  | pc_2Tx_ULTxSwitchingBandPair_r17 |  |  |  | No |  |  |  |  |  |  |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |  |  |  |

Table A.4.3.2A.4.1-3: Supported configurations for NR Inter-band CA within FR1 and two bands

| NR FR1 Inter-band CA configuration / Item (Note 1, 9) | Release | Supported | Supported CA Bandwidth Class(es) in UL (Note 2,5) | Supported Bandwidth Combination Set(s) (Note 3) | Supported 1Tx-2Tx ULTxSwitching Band Pair (Note 7, 8) | Supported 2Tx-2Tx ULTxSwitching Band Pair (Note 7, 8) | Supported uplinkTxSwitching-DL-Interruption-r16 (Note 10) | Supported simultaneousRxTx (Note 11) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A | Rel-16 |  |  |  |  |  |  |  |
| CA_n1(2A)-n3A | Rel-17 |  |  |  |  |  |  |  |
| CA_n1(2A)-n5A | Rel-17 |  |  |  |  |  |  |  |
| CA_n1A-n8A | Rel-16 |  |  |  |  |  |  |  |
| CA_n1(2A)-n8A | Rel-17 |  |  |  |  |  |  |  |
| CA_n1A-n77A | Rel-16 |  |  |  |  |  | Not supported |  |
| CA_n1A-n78A | Rel-16 |  |  |  |  |  | Not supported |  |
| CA_n1(2A)-n78A | Rel-17 |  |  |  |  |  | Not supported |  |
| CA_n1A-n78(2A) | Rel-17 |  |  |  |  |  | Not supported |  |
| CA_n1A-n78C | Rel-16 |  |  |  |  |  | Not supported |  |
| CA_n1A-n79A | Rel-16 |  |  |  |  |  | Not supported |  |
| CA_n2A-n5A | Rel-16 |  |  |  |  |  |  |  |
| CA_n2A-n48A | Rel-16 |  |  |  |  |  |  |  |
| CA_n2A-n77A | Rel-16 |  |  |  |  |  |  |  |
| CA_n3A-n5A | Rel-17 |  |  |  |  |  |  |  |
| CA_n3(2A)-n5A | Rel-17 |  |  |  |  |  |  |  |
| CA_n3A-n8A | Rel-16 |  |  |  |  |  |  |  |
| CA_n3(2A)-n8A | Rel-17 |  |  |  |  |  |  |  |
| CA_n3A-n41A | Rel-16 |  |  |  |  |  | Not supported |  |
| CA_n3A-n78A | Rel-15 |  |  |  |  |  | Not supported |  |
| CA_n3A-n78(2A) | Rel-17 |  |  |  |  |  | Not supported |  |
| CA_n3(2A)-n78A | Rel-17 |  |  |  |  |  | Not supported |  |
| CA_n5A-n66A | Rel-16 |  |  |  |  |  |  |  |
| CA_n5A-n78(2A) | Rel-17 |  |  |  |  |  |  |  |
| CA_n5A-n7A | Rel-16 |  |  |  |  |  |  |  |
| CA_n5A-n48A | Rel-17 |  |  |  |  |  |  |  |
| CA_n5A-n77A | Rel-16 |  |  |  |  |  |  |  |
| CA_n5A-n78A | Rel-16 |  |  |  |  |  | Not supported |  |
| CA_n7A-n78A | Rel-16 |  |  |  |  |  |  |  |
| CA_n8A-n78A | Rel-15 |  |  |  |  |  | Not supported |  |
| CA_n8A-n78(2A) | Rel-17 |  |  |  |  |  | Not supported |  |
| CA_n24A-n41A | Rel-17 |  |  |  |  |  |  |  |
| CA_n24A-n41(2A) | Rel-17 |  |  |  |  |  |  |  |
| CA_n24A-n48A | Rel-17 |  |  |  |  |  |  |  |
| CA_n24A-n48B | Rel-17 |  |  |  |  |  |  |  |
| CA_n24A-n48(2A) | Rel-17 |  |  |  |  |  |  |  |
| CA_n24A-n77A | Rel-17 |  |  |  |  |  |  |  |
| CA_n24A-n77C | Rel-17 |  |  |  |  |  |  |  |
| CA_n26A-n66A | Rel-17 |  |  |  |  |  |  |  |
| CA_n26A-n66(2A) | Rel-17 |  |  |  |  |  |  |  |
| CA_n26A-n70A | Rel-17 |  |  |  |  |  |  |  |
| CA_n28A-n41A | Rel-16 |  |  |  |  |  |  |  |
| CA_n28A-n78A | Rel-16 |  |  |  |  |  |  |  |
| CA_n28A-n79A | Rel-17 |  |  |  |  |  |  | Yes |
| CA_n29A-n66A | Rel-16 |  |  |  |  |  |  |  |
| CA_n29A-n66B | Rel-16 |  |  |  |  |  |  |  |
| CA_n29A-n66(2A) | Rel-16 |  |  |  |  |  |  |  |
| CA_n29A-n70A | Rel-16 |  |  |  |  |  |  |  |
| CA_n29A-n71A | Rel-17 |  |  |  |  |  |  |  |
| CA_n39A-n41A | Rel-16 |  |  |  |  |  |  |  |
| CA_n41A-n66A | Rel-17 |  |  |  |  |  |  |  |
| CA_n41A-n71A | Rel-16 |  |  |  |  |  |  |  |
| CA_n41A-n79A | Rel-16 |  |  |  |  |  | Not supported |  |
| CA_n41C-n79A | Rel-16 |  |  |  |  |  | Not supported |  |
| CA_n48A-n66A (Note 6) | Rel-16 |  |  |  |  |  |  |  |
| CA_n48A-n66(2A) (Note 6) | Rel-17 |  |  |  |  |  |  |  |
| CA_n48A-n70A | Rel-17 |  |  |  |  |  |  |  |
| CA_n48A-n71A | Rel-17 |  |  |  |  |  |  |  |
| CA_n48A-n71(2A) | Rel-17 |  |  |  |  |  |  |  |
| CA_n48A-n77A | Rel-17 |  |  |  |  |  |  |  |
| CA_n48B-n66A | Rel-17 |  |  |  |  |  |  |  |
| CA_n48B-n70A | Rel-17 |  |  |  |  |  |  |  |
| CA_n48B-n71A | Rel-17 |  |  |  |  |  |  |  |
| CA_n48(2A)-n66A | Rel-17 |  |  |  |  |  |  |  |
| CA_n48(2A)-n66(2A) | Rel-17 |  |  |  |  |  |  |  |
| CA_n48(2A)-n70A | Rel-17 |  |  |  |  |  |  |  |
| CA_n48(2A)-n71A | Rel-17 |  |  |  |  |  |  |  |
| CA_n48(2A)-n71(2A) | Rel-17 |  |  |  |  |  |  |  |
| CA_n66A-n70A (Note 6) | Rel-16 |  |  |  |  |  |  |  |
| CA_n66B-n70A (Note 6) | Rel-16 |  |  |  |  |  |  |  |
| CA_n66(2A)-n70A (Note 6) | Rel-16 |  |  |  |  |  |  |  |
| CA_n66A-n71A (Note 6) | Rel-16 |  |  |  |  |  |  |  |
| CA_n66A-n71(2A) (Note 6) | Rel-17 |  |  |  |  |  |  |  |
| CA_n66B-n71A (Note 6) | Rel-16 |  |  |  |  |  |  |  |
| CA_n66(2A)-n71A (Note 6) | Rel-16 |  |  |  |  |  |  |  |
| CA_n66(2A)-n71(2A) (Note 6) | Rel-17 |  |  |  |  |  |  |  |
| CA_n66A-n77A | Rel-16 |  |  |  |  |  |  |  |
| CA_n70A-n71A | Rel-16 |  |  |  |  |  |  |  |
| CA_n70A-n71(2A) | Rel-17 |  |  |  |  |  |  |  |
| CA_n71A-n77A | Rel-17 |  |  |  |  |  |  |  |
| CA_n78A-n79A | Rel-15 |  |  |  |  |  |  |  |
| Note 1: Notation used for inter-band CA Bands is according to TS 38.101-1 [23] Table 5.5A.3.1-1, e.g. ‘CA_n1A-n78C’ indicates CA operation on NR band n1 and n78 with DL CA Bandwidth Class A and C respectively. Note 2: The UL CA capabilities as per Table A.4.3.2A.4.1-2 can be supported on a single or multiple CA Band(s). The UE supplier shall indicate all supported UL CA Bandwidth Class(es), in uplink of the supported CA Band(s), as per TS 38.101-1 [23] Table 5.5A.3.1-1. For this release of specification valid choices are ’N’, ‘nXA-nYA’, ‘nX(2A)’, ‘nXB’ and ‘nXC’, where both nX and nY are the NR bands. For example, for CA_n1A-n77A, ‘N’ would mean only DL CA, ‘n1A-n77A’ would mean both DL and UL CA. Note 3: The UE supplier shall indicate the supported Bandwidth Combination Set(s) as per TS 38.101-1 [23] Table 5.5A.3.1-1. Note 4: Void. Note 5: See UL(table_index) in in Note 1 of Table 4.0-3 and UL_nCC(table_index) in Note 2 of Table 4.0-3 in TS 38.522 [9]. Note 6: A UE that supports NR Band n66 (Table A.4.3.1-1) and CA operation in any CA band shall also support the DL CA configurations CA_n66B and CA_n66(2A), as per Note 7, in Table 5.2-1, in TS 38.521-1 [5]. Note 7: The ULTxSwitching capability can be reported on inter-band CA band combinations. The UE supplier shall indicate inter-band CA band pairs on which it supports 1Tx-2Tx or 2Tx-2Tx ULTxSwitching. For this release of specification valid choices are ’N’ and ‘nX-nY’, where both nX and nY are NR bands. For example, for CA_n1A-n77A, ‘N’ would mean not supporting ULTxSwitching, ‘n1-n77’ would mean supporting of ULTxSwitching on this band pair. If UE supplier indicates supporting of ULTxSwitching on a band pair, they shall indicate at least one inter-band UL CA configuration on the same band pair in the column “Supported CA Bandwidth Class(es) in UL”. The ULTxSwitching is only tested with 2 UL CCs, so UE is allowed to report ‘N’ by default for CA configuration with > 2 component carriers. Note 8: See ULTxSwitching(table_index) and 2Tx_ULTxSwitching(table_index) in Note 6 of Table 4.0-3 in TS 38.522 [9]. Note 9: See DL_nCC(table_index) in Note 4 of Table 4.0-3 in TS 38.522 [9]. Note 10: A UE that supports ULTxSwitching on a band pair might report the uplinkTxSwitching-DL-Interruption-r16 capability on the same band pair. If UE doesn’t report this capability, no DL interruption is allowed during UL Tx switching. For certain band configurations DL interruption is not allowed according to Note 8 in Table 5.2A.2.1-1 of TS 38.101-1 [23], therefore the corresponding entry is prefilled by ‘Not Supported’. Note 11: For configurations with Note 1 in Table 5.2A.2.1-1 of TS 38.521-1 [5], UE capability simultaneousRxTxInterBandCA is mandatory. |  |  |  |  |  |  |  |  |

< End of Changes >