3GPP TSG-RAN5 Meeting #100	R5-235706

# Toulouse, France, 21 – 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2495 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of DL interruptions at switching between two uplink bands with two transmit antenna connectors for 6.5.7C.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA test case have been defined in 38.133 but are currently missing in RAN5 test specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added DL interruption switching testcase - Added cell mapping configuration for the above test case in Annex E.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would be missing in the test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.7C.1(new), E.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0305 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Meeting number and title corrected to resolve 3GU issues. |  |  |  |  |  |  |  |  |

-------- Start of first change --------

### 6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors

#### 6.5.7C.0	Minimum conformance requirements

The DL interruption requirements at dynamic switching between two uplink bands specified in this clause are

applicable for an uplink band pair of an inter-band UL CA configuration when the capability

uplinkTxSwitchingPeriod2T2T is present, and is only applicable for uplink switching mechanism specified in clause 6.1.6 of TS 38.214 [9], where NR UL carrier 1 in band A is capable of two transmit antenna connectors, NR UL carrier 2 and carrier 3 in band B are capable of two transmit antenna connectors. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers, and band A and band B are different bands with different carrier frequencies.

When dynamic switching between two uplink bands is conducted, UE is allowed to cause DL interruption of X OFDM symbols in NR downlink carrier(s) as indicated by uplinkTxSwitching-DL-Interruption [13]. The DL interruption starts from the first OFDM symbol which fully or partially overlaps with the UL switching period located in either NR band A or band B as indicated in RRC signalling [13]. The DL interruption lengths of X are defined in Table 6.5.7C.0-1.

No DL interruption is allowed in the NR downlink carrier(s) which is not indicated by uplinkTxSwitching-DLInterruption. No DL interruption is allowed for some inter-band UL CA configurations as specified in clause 5.2A.2 of TS 38.101-1 [2].

Table 6.5.7C.0-1: DL interruption length on NR carrier(s) in the unit of OFDM symbols (X) for switching between two uplink carriers

|  | NR Slot length (ms) | Uplink Tx switching period Note1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | 7 |
| 2 | 0.25 | 4 | 10 | 14 |
| Note 1: Uplink Tx switching period depends on UE capability [uplinkTxSwitchingPeriod2T2T]. |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 8.2.2.2.10C.

#### 6.5.7C.1 	NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA

6.5.7C.1.1	Test purpose

The purpose of this test is to verify DL interruption requirements during UE dynamic switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA.

6.5.7C.1.2	Test applicability

This test applies to all types of NR UE release 16 onwards configured with uplinkTxSwitchingPeriod2T2T.

6.5.7C.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.7C.0.

The normative reference for this requirement is TS 38.133 [6] clause 8.2.2.2.10C and A.6.5.7C.1

6.5.7C.1.4	Test description

6.5.7C.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.7C.1.4.1-1.

Table 6.5.7C.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 6.5.7C.1-1 | NR Cell 1: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 6.5.7C.1.4.1-2.

Table 6.5.7C.1.4.1-2: Initial conditions for DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.7C.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2a | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. - Without LTE link - Without Faders |  |  |

1. The general test parameter settings are set up according to Table 6.5.7C.1.4.1-3.

2. Message contents are defined in clause 6.5.7C.1.4.3.

3. The test scenario comprises of three NR Cells. (Cell 1, Cell 2 and Cell 3). Cell 1, Cell 2 and Cell 3 are configured according to Annex C.1.2 and C.1.3.

Table 6.5.7C.1.4.1-3: General test parameters for DL interruptions at switching between two uplink bands with two antenna transmit connectors in FDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2, 3 | Three radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 Scell Cell 3: FR1 SCell | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 FR1 SCell on RF channel number 3 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.1.5 FDD Cell 2: CSI-RS.2.5 TDD Cell 3: CSI-RS.2.5 TDD |  |
| T1 | s | Config 1 | 5 |  |

6.5.7C.1.4.2	Test procedure

The test consists of three active NR cells: FR1 FDD PCell (Cell 1), FR1 TDD SCell (Cell 2) and FR1 TDD SCell (Cell 3). The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE shall be connected to Cell1, Cell2 and Cell3 and the RRC message including uplinkTxSwitching is received at the UE antenna connector.

UE is configured to transmit SRS on PCell (Cell 1), Scell (Cell 2) and SCell (Cell 3) on the last 2 symbols of special slot. DL interruption is expected to take place prior to the SRS symbols.

- Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Configure FDD PCell (Cell 1), TDD SCell(Cell 2) and TDD SCell(Cell 3) according to Table 6.5.7C.1.5-1. Propagation conditions are set according to Annex C clause C.2.2.

3.	The SS shall transmit an RRCReconfiguration message to configure FDD PCell (Cell 1), TDD SCell(Cell 2) and TDD SCell(Cell 3) as per TS 38.508-1 [7] clause 7 with the message content exceptions defined in clause 6.5.7C.1.4.3. UplinkTxSwitching is configured to the UE.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	Set the parameters according to T1 in Tables 6.5.7C.1.5-1. T1 starts.

6.	SS schedules UL transmission on PCell continuously on the DL slots of SCell (Cell 2 and Cell 3).

7.	SS triggers aperiodic CSI-RS for L1-RSRP reporting with power boosting 6dB for NR FDD carrier (Cell 1), on the following symbol in the slot overlapping with the special slot of the NR TDD carrier (Cell 2) and NR TDD carrier (Cell 3):

FDD PCell (Cell1):

- symbol#12 if UE does not report uplinkTxSwitching-DL-Interruption;

- otherwise,

- symbol #8 if UE capability uplinkTxSwitchingPeriod2T2T is 210us or

- symbol #9 if UE capability uplinkTxSwitchingPeriod2T2T is 140us or

- symbol #10 if UE capability uplinkTxSwitchingPeriod2T2T is 35us.

8.   SS triggers aperiodic CSI-RS for L1-RSRP reporting with power boosting 6dB for NR TDD carrier (Cell 2) and NR TDD carrier (Cell 3), on the following symbol in the special slot:

TDD SCell(Cell2 and Cell3):

-	symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption;

-    otherwise,

- symbol #4 if UE capability uplinkTxSwitchingPeriod2T2T is 210us or

- symbol #5 if UE capability uplinkTxSwitchingPeriod2T2T is 140us or

- symbol #8 if UE capability uplinkTxSwitchingPeriod2T2T is 35us.

9.	After SS transmits the DCI trigger, the UE shall send L1-RSRP report containing valid L1-RSRP report for CSI-RS#0, CSI-RS#1 and CSI-RS#2 at slot 5 from the reception of DCI trigger, if so, increase the number of passed iterations by one otherwise increase the number of failed iterations by one and switch off the UE.

10.	SS transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED TS 38.508-1 [14] clause 4.5),

or

-	switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat step 3-9 until a test verdict has been achieved.

6.5.7C.1.4.3	Message contents

Message contents are same as in 6.5.7.1.4.3 with the following exceptions:

6.5.7C.1.4.3-1: SRS-Config

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-182 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SEQUENCE { |  |  |  |
| SRS-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SEQUENCE { |  |  |  |
| SRS-Resource[1] SEQUENCE { |  | entry 1 |  |
| nrofSRS-Ports | ports2 |  | 2TX_UL_MIMO |
| resourceMapping SEQUENCE { |  |  |  |
| startPosition | 0 |  |  |
| nrofSymbols | n2 |  |  |
| } |  |  |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS | 0 |  |  |
| } |  |  |  |
| groupOrSequenceHopping | neither |  |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| periodicityAndOffset-p CHOICE { |  |  |  |
| sl8 | 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.7C.1.4.3-2: CSI-MeasConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { |  |  |  |
| NZP-CSI-RS-Resource[1] SEQUENCE { |  | entry 1 |  |
| nzp-CSI-RS-ResourceId | 0 |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000001 |  |  |
| } |  |  |  |
| nrofPorts | p1 |  |  |
| firstOFDMSymbolInTimeDomain | 12 |  | UE does not report uplinkTxSwitching-DL-Interruption |
|  | 8 |  | UE capability uplinkTxSwitchingPeriod is 210us |
|  | 9 |  | UE capability uplinkTxSwitchingPeriod is 140us |
|  | 10 |  | UE capability uplinkTxSwitchingPeriod is 35us |
| firstOFDMSymbolInTimeDomain2 | Not present |  |  |
| cdm-Type | noCDM |  |  |
| density CHOICE { |  |  |  |
| three |  |  |  |
| } |  |  |  |
| freqBand SEQUENCE { |  |  |  |
| startingRB | 0 |  |  |
| nrofRBs | 52 |  |  |
| } |  |  |  |
| } |  |  |  |
| powerControlOffset | 0 |  |  |
| powerControlOffsetSS | db6 |  | UE configured with CSI-RS power boosting of 6dB |
| scramblingID | 0 |  |  |
| periodicityAndOffset | Not Present |  |  |
| qcl-InfoPeriodicCSI-RS | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| NZP-CSI-RS-Resource[2] SEQUENCE { |  | entry 2 |  |
| nzp-CSI-RS-ResourceId | 1 |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000001 |  |  |
| } |  |  |  |
| nrofPorts | p1 |  |  |
| firstOFDMSymbolInTimeDomain | 10 |  | UE does not report uplinkTxSwitching-DL-Interruption |
|  | 4 |  | UE capability uplinkTxSwitchingPeriod is 210us |
|  | 5 |  | UE capability uplinkTxSwitchingPeriod is 140us |
|  | 8 |  | UE capability uplinkTxSwitchingPeriod is 35us |
| firstOFDMSymbolInTimeDomain2 | Not present |  |  |
| cdm-Type | noCDM |  |  |
| density CHOICE { |  |  |  |
| three |  |  |  |
| } |  |  |  |
| freqBand SEQUENCE { |  |  |  |
| startingRB | 0 |  |  |
| nrofRBs | 108 |  |  |
| } |  |  |  |
| } |  |  |  |
| powerControlOffset | 0 |  |  |
| powerControlOffsetSS | db6 |  | UE configured with CSI-RS power boosting of 6dB |
| scramblingID | 0 |  |  |
| periodicityAndOffset | Not Present |  |  |
| qcl-InfoPeriodicCSI-RS | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| NZP-CSI-RS-Resource[2] SEQUENCE { |  | entry 3 |  |
| nzp-CSI-RS-ResourceId | 2 |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000001 |  |  |
| } |  |  |  |
| nrofPorts | p1 |  |  |
| firstOFDMSymbolInTimeDomain | 10 |  | UE does not report uplinkTxSwitching-DL-Interruption |
|  | 4 |  | UE capability uplinkTxSwitchingPeriod is 210us |
|  | 5 |  | UE capability uplinkTxSwitchingPeriod is 140us |
|  | 8 |  | UE capability uplinkTxSwitchingPeriod is 35us |
| firstOFDMSymbolInTimeDomain2 | Not present |  |  |
| cdm-Type | noCDM |  |  |
| density CHOICE { |  |  |  |
| three |  |  |  |
| } |  |  |  |
| freqBand SEQUENCE { |  |  |  |
| startingRB | 0 |  |  |
| nrofRBs | 108 |  |  |
| } |  |  |  |
| } |  |  |  |
| powerControlOffset | 0 |  |  |
| powerControlOffsetSS | db6 |  | UE configured with CSI-RS power boosting of 6dB |
| scramblingID | 0 |  |  |
| periodicityAndOffset | Not Present |  |  |
| qcl-InfoPeriodicCSI-RS | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet | entry 1 Table 6.5.7C.1.4.3-3 |  |
| } |  |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | 1 entry |  |  |
| CSI-ResourceConfig[1] |  | entry 1 Table 6.5.7C.1.4.3-4 |  |
| } |  |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { | 1 entry |  |  |
| CSI-ReportConfig[1] | CSI-ReportConfig | entry 1 Table 6.5.7.1.4.3-9 |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.7C.1.4.3-3: NZP-CSI-RS-ResourceSet

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp-CSI-ResourceSetId | 0 | For CSI-RS#0 |  |
|  | 1 | For CSI-RS#1 |  |
|  | 2 | For CSI-RS#2 |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceId[1] | 0 | entry 1 |  |
| } |  |  |  |
| repetition | off |  |  |
| aperiodicTriggeringOffset | 6 |  |  |
| trs-Info | Not present |  |  |
| } |  |  |  |

Table 6.5.7C.1.4.3-4: CSI-ResourceConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | 0 | For CSI-RS#0 |  |
|  | 1 | For CSI-RS#1 |  |
|  | 2 | For CSI-RS#2 |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { |  |  |  |
| NZP-CSI-RS-ResourceSetId [1] | 0 |  |  |
| } |  |  |  |
| csi-SSB-ResourceSetList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | 0 |  |  |
| resourceType | aperiodic |  |  |
| } |  |  |  |

6.5.7C.1.5	Test requirements

Table 6.5.7C.1.5-1 defines the primary level settings including test tolerances for DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA.

Table A.6.5.7C.1.5-1: Cell specific test parameters for DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 | FR1 |
| Duplex mode | Config 1 |  | FDD | TDD | TDD |
| TDD configuration | Config 1 |  | N/A | TDDConf.2.1 except that: S=’11DL: 1GP:2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 | TDDConf.2.1 except that: S=’11DL: 1GP:2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 |
| BWchannel | Config 1 |  | 10 MHz: NRB,c = 52 | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosit ion: 0resourceMappingnrofSy mbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPos ition: 0 resourceMappingnrofSym bols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPos ition: 0 resourceMappingnrofSym bols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.1 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | 2x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 + TT | -104 + TT | -104 + TT |
| SS-RSRP Note 3 |  | dBm/SCS | -87 + TT | -87 + TT | -87 + TT |
| Ês/Iot |  | dB | 17 + TT | 14 + TT | 14 + TT |
| Ês/Noc |  | dB | 17 + TT | 14 + TT | 14 + TT |
| NocNote 2 | Config 1 | dBm/SCS | -104 + TT | -101 + TT | -101 + TT |
| IoNote3 | Config 1 | dBm/9.36 MHz | -58.96 + TT | - | - |
|  |  | dBm/ 38.16MHz | - | -55.79 + TT | -55.79 + TT |
| Time offset to Cell1 Note 5 |  | s | - | 0 | 0 |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |

UE shall send L1-RSRP report while meeting the accuracy requirements defined in TS 38.133 clause 10.1.19.2.

The DL interruption lengths of X are defined in Table 6.5.7C.1.5-2.

Table 6.5.7C.1.5-2: DL interruption length on NR carrier(s) in the unit of OFDM symbols (X) for switching between two uplink bands

|  | NR Slot length (ms) | Uplink Tx switching period Note1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | 7 |
| 2 | 0.25 | 4 | 10 | 14 |
| Note 1: Uplink Tx switching period depends on UE capability [uplinkTxSwitchingPeriod2T2T]. |  |  |  |  |

The UE shall send L1-RSRP report at slot 5 from the reception of DCI trigger. The L1-RSRP report shall include the results of CSI-RS#0, CSI-RS#1 and CSI-RS#2. Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 6.5.7C.1.5-3 and 6.5.7C.1.5-4.

Table 6.5.7C.1.5-3: L1-RSRP absolute accuracy requirements for the reported values of CSI-RS#0

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value | 64 |
| Highest reported value | 84 |

Table 6.5.7C.1.5-4: L1-RSRP absolute accuracy requirements for the reported values of CSI-RS#1 and CSI-RS#2

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value | 67 |
| Highest reported value | 87 |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

----- End of first change ------

----Unchanged Clauses Skipped -----

-------- Start of Second change --------

# E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6

Table E.4-1 defines the cell configuration mapping for NR/5GC FR1 test cases in chapter 6 of this test specification.

Table E.4-1: Cell configuration mapping for SA FR1 RRM testing

| TC |  | Description |  | 38.533 NR Cell1 |  | 38.533 NR Cell2 |  | 38.533 NR Cell3 |  | 38.533 NR Cell4 |  | CA Type |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1 |  | NR SA FR1 cell re-selection |  | NR Cell 1 |  | NR Cell 11 |  |  |  |  |  |  |  |  |
| 6.1.1.2 |  | NR SA FR1-FR1 cell re-selection |  | NR Cell 6 |  | NR Cell 23 |  |  |  |  |  |  |  |  |
| 6.1.1.3 |  | NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | NR Cell 1 |  | NR Cell 11 |  |  |  |  |  |  |  |  |
| 6.1.1.4 |  | NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | NR Cell 1 |  | NR Cell 11 |  |  |  |  |  |  |  |  |
| 6.1.1.5 |  | NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | NR Cell 1 |  | NR Cell 23 |  |  |  |  |  |  |  |  |
| 6.1.1.6 |  | NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | NR Cell 1 |  | NR Cell 23 |  |  |  |  |  |  |  |  |
| 6.1.1.7 |  | NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 |  | NR Cell 1 |  | NR Cell 11 |  |  |  |  |  |  |  |  |
| 6.1.1.8 |  | NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17 |  | NR Cell 1 |  | NR Cell 11 |  |  |  |  |  |  |  |  |
| 6.3.1.1 |  | NR SA FR1 handover with known target cell |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.1.2 |  | NR SA FR1 handover with unknown target cell |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.1.3 |  | NR SA FR1-FR1 Handover with unknown Target Cell |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |
| 6.3.1.7 |  | NR SA FR1 synchronous DAPS handover |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.1.8 |  | NR SA FR1 asynchronous DAPS handover |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.1.9 |  | NR SA FR1 Intra-band inter-frequency synchronous DAPS handover |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |
| 6.3.1.10 |  | NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |
| 6.3.1.11 |  | NR SA FR1 Inter-band inter-frequency synchronous DAPS handover |  | NR Cell 1 |  | NR Cell 10 |  |  |  |  |  |  |  |
| 6.3.1.12 |  | NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover |  | NR Cell 1 |  | NR Cell 10 |  |  |  |  |  |  |  |
| 6.3.2.1.1 |  | NR SA FR1 RRC re-establishment |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 6.3.2.1.2 |  | NR SA FR1 - FR1 RRC re-establishment |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 6.3.2.1.3 |  | NR SA FR1 RRC re-establishment without serving cell timing |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 6.3.2.2.1 |  | NR SA FR1 contention based random access |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.3.2.2.2 |  | NR SA FR1 non-contention based random access |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.3.2.2.3 |  | NR SA FR1 2-step contention based random access |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.3.2.2.4 |  | NR SA FR1 2-step non-contention based random access |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.3.2.3.1 |  | NR SA FR1 RRC connection release with redirection |  | NR Cell 6 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 6.3.3.1 |  | NR SA FR1 conditional handover |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 6.3.3.2 |  | NR SA FR1-FR1 conditional handover |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  |  |  |  |
| 6.4.3.1 |  | NR SA FR1 timing advance adjustment accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.5.1.1 |  | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.5.1.2 |  | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.5.1.3 |  | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.5.1.4 |  | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.5.1.5 |  | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.5.1.6 |  | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.5.1.7 |  | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.5.1.8 |  | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.5.2.1 |  | NR SA FR1 interruptions during measurements on deactivated NR SCC |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  | intra-band |  |  |
|  |  |  |  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |  |
| 6.5.3.1 |  | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  | intra-band |  |  |
|  |  |  |  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |  |
| 6.5.3.2 |  | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  | intra-band |  |  |
|  |  |  |  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |  |
| 6.5.3.3 |  | NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  | intra-band |  |  |
|  |  |  |  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |  |
| 6.5.3.4 |  | Direct SCell activation at SCell addition of known SCell in FR1 |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  | intra-band |  |  |
| 6.5.4.1 |  | NR SA FR1 UE UL carrier RRC reconfiguration delay |  | NR Cell 1 |  | NR Cell 33 |  |  |  |  |  |  |  |  |
| 6.5.5.1 |  | NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.5.5.2 |  | NR SA FR1 SSB-based beam failure detection and link recovery in DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.5.5.3 |  | NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.5.5.4 |  | NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.5.5.5 |  | NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX |  | NR Cell 1 |  | NR Cell 10 |  |  |  |  |  |  |  |  |
| 6.5.5.6 |  | NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX |  | NR Cell 1 |  | NR Cell 10 |  |  |  |  |  |  |  |  |
| 6.5.6.1.1 |  | NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 6.5.6.1.2 |  | NR SA FR1 DCI-based DL active BWP switch in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.5.6.2.1 |  | NR SA FR1 RRC-based DL active BWP switch in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.5.7.1 |  | NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA |  | NR Cell 1 |  | NR Cell 10 |  |  |  |  |  | inter-band |  |  |
| 6.5.7.2 |  | NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA |  | NR Cell 1 |  | NR Cell 10 |  |  |  |  |  | inter-band |  |  |
| 6.5.7C.1 |  | DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA |  | NR Cell 1 |  | NR Cell 6 |  | NR Cell 13 |  |  |  | Inter-band |  |  |
| 6.5.8.1 |  | UE specific CBW change on PCell in FR1 in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.6.1.1 |  | SA event triggered reporting tests without gap under non-DRX |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 6.6.1.2 |  | SA event triggered reporting tests without gap under DRX |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 6.6.1.3 |  | SA event triggered reporting tests with per-UE gaps under non-DRX |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 6.6.1.4 |  | SA event triggered reporting tests with per-UE gaps under DRX |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 6.6.1.5 |  | SA event triggered reporting tests without gap under non-DRX with SSB index reading |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 6.6.1.6 |  | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 6.6.1.7 |  | NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 6.6.1.8 |  | NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17 |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 6.6.2.1 |  | NR SA FR1-FR1 event-triggered reporting in non-DRX |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 6.6.2.2 |  | NR SA FR1-FR1 event-triggered reporting in DRX |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 6.6.2.5 |  | NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 6.6.2.6 |  | NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 6.6.2.12 |  | NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 6.6.4.1 |  | NR SA FR1 SSB-based L1-RSRP measurement in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.6.4.2 |  | NR SA FR1 SSB-based L1-RSRP measurement in DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.6.4.3 |  | NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.6.4.4 |  | NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.6.4.5 |  | NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.6.6.1 |  | NR SA FR1 SRS-RSRP measurement in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.6.8.1 |  | NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.6.8.2 |  | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.6.8.3 |  | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.7.8.1 |  | NR SA FR1 SRS-RSRP measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.6.9.1 |  | NR SA FR1 Idle mode CA/DC measurement for FR1 |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  | intra-band |  |  |
|  |  |  |  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |  |
| 6.6.17.1 |  | NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |  |
| 6.6.17.2 |  | NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |  |
| 6.6.18.1 |  | NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers |  | NR Cell 1 |  | NR Cell 3 |  | NR Cell 6 |  |  |  |  |  |  |
| 6.6.18.2 |  | NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers |  | NR Cell 1 |  | NR Cell 3 |  | NR Cell 6 |  |  |  |  |  |  |
| 6.6.18.4 |  | NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used |  | NR Cell 6 |  | NR Cell 3 |  | NR Cell 13 |  |  |  |  |  |  |  |
| 6.6.19.1 |  | NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1 |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |  |
| 6.6.19.2 |  | NR SA FR1 event-triggered reporting tests for FR1 with NCSG for inter-frequency measurement |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |  |
| 6.6.19.4 |  | NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX |  | NR Cell 1 |  | NR Cell 6 |  | NR Cell 13 |  |  |  | intra-band |  |  |  |
|  |  |  |  |  |  | NR Cell 10 |  | NR Cell 30 |  |  |  | inter-band |  |  |  |
| 6.6.20 |  | UE Rx-Tx time difference measurement for propagation delay compensation |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.6.21 |  | UE Rx-Tx time difference measurement for propagation delay compensation with TRS |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.7.1.1.1 |  | NR SA FR1 SS-RSRP absolute measurement accuracy |  | NR Cell 489 |  | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.7.1.1.2 |  | NR SA FR1 SS-RSRP relative measurement accuracy |  | NR Cell 489 |  | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.7.1.2.1 |  | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 6.7.1.2.2 |  | NR SA FR1-FR1 SS-RSRP relative measurement accuracy |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 6.7.1.3.1 |  | NR SA FR1-FR2 SS-RSRP absolute measurement accuracy |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 6.7.1.3.2 |  | NR SA FR1-FR2 SS-RSRP relative measurement accuracy |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 6.7.2.1 |  | NR SA FR1 SS-RSRQ measurement accuracy |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 6.7.2.2.1 |  | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 6.7.2.2.2 |  | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 6.7.3.1 |  | NR SA FR1 SS-SINR measurement accuracy |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 6.7.3.2.1 |  | NR SA FR1-FR1 SS-SINR absolute measurement accuracy |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 6.7.3.2.2 |  | NR SA FR1-FR1 SS-SINR relative measurement accuracy |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 6.7.4.1.1 |  | NR SA FR1 SSB based L1-RSRP absolute measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.7.4.1.2 |  | NR SA FR1 SSB based L1-RSRP relative measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.7.4.2.1 |  | NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.7.4.2.2 |  | NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.7.9.1 |  | NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.7.9.2 |  | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 6.7.9.3 |  | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |

----- End of Second change ------