3GPP TSG-RAN5 Meeting #100	R5-234787

# Toulouse, France, 21st – 25th Aug. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2394 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 6.2G.2 MPR for Power Class 1.5 for TxD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_TxD-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - MPR test requirements for PC1.5 FWA is in place but the corresponding test configration is missing. - In Table 6.2G.2.5-2b the value of PPowerClass andΔPPowerClass does not follow the definition in core specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Reuse PC1.5 UE test configuration table for PC1.5 FWA. - Correct PPowerClass andΔPPowerClass as 29dBm and 3dB respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case may not be performed properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2G.2	UE maximum output power reduction for Tx Diversity

6.2G.2.1	Test purpose

The number of RB identified in Table 6.2D.2.3-1 and Table 6.2D.2.3-2 is based on meeting the requirements for adjacent channel leakage ratio and the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2G.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5G.2.3.1 is executed.

6.2G.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the allowed MPR for the maximum output power is specified in Table 6.2.2-1, Table 6.2D.2.3-1, Table 6.2D.2.3-2 and Table 6.2D.2.3-3 for UE power class 3, 2 and 1.5 respectively. For UE power class 1.5, the allowed maximum power reduction (MPR) defined in Table 6.2D.2.3-3 is in accordance with the indicated modifiedMPR-Behavior specified in Table L.1-1 for channel bandwidths ≤ 100 MHz. The maximum output power is defined as the sum of the maximum output power at each UE antenna connector. If a UE that supports PC1.5 has to apply the requirements of PC2 according to the rules in clause 6.2.1, the MPR requirements in Table 6.2.2-2 apply.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2G.4 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2G.2.

6.2G.2.4	Test description

6.2G.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2G.2.4.1-1, Table 6.2G.2.4.1-2, and Table 6.2G.2.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2G.2.4.1-1: Test Configuration Table for power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

Table 6.2G.2.4.1-2: Test Configuration Table for power class 1.5 UE and power class 1.5 FWA (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Default |  | DFT-s-OFDM_64_QAM | Inner Full |
| 14 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 17 | Default |  | DFT-s-OFDM_256_QAM | Inner Full |
| 18 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 19 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 20 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 21 | Default |  | CP-OFDM QPSK | Inner Full |
| 22 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 23 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM QPSK | Outer Full |
| 25 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 26 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 27 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 28 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 29 | Default |  | CP-OFDM 64 QAM | Inner Full |
| 30 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 33 | Default |  | CP-OFDM 256 QAM | Inner Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

Table 6.2G.2.4.1-2a: Test Configuration Table for power class 3 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 13 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 23 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 33 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 43 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 54 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 64 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 74 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 84 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 10 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 13 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 14 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 17 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 18 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 19 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 20 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 21 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 22 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 23 | Default |  | CP-OFDM QPSK | Inner Full |
| 24 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 25 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM QPSK | Outer Full |
| 27 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 28 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 29 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 30 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 31 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 32 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 33 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 374,5 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 384,5 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 394,5 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 5: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |

Table 6.2G.2.4.1-3: Test Configuration Table for power class 2 and 3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Default |  | CP-OFDM QPSK | Outer Full |
| 3 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 4 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 5 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 6 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.2.2.4.1-4, 6.2.2.4.1-4a and 6.2.2.4.1-4b. NOTE 2: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.2.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2G.2.4.1-1, Table 6.2G.2.4.1-2 and Table 6.2G.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2G.2.4.3.

6.2G.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2G.2.4.1-1, Table 6.2G.2.4.1-2, Table 6.2G.2.4.1-2a and Table 6.2G.2.4.1-3. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	For UEs supporting Power Class 1.5 and Power Class 2, repeat steps 1~3 for Test ID 16 and 22 in Table 6.2G.2.4.1-2a on the applicable bands with message exception of P-Max defined in Table 6.2G.2.4.3-2.

5.	For UEs supporting Power Class 1.5, repeat steps 1~3 for all Test IDs in Table 6.2G.2.4.1-1 on the applicable bands with message exception of P-Max defined in Table 6.2G.2.4.3-3.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2G.2.4.1-1 and Table 6.2G.2.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2G.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions:

Table 6.2G.2.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.2G.2.4.3-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | PC2 UE or PC1.5 UE |

Table 6.2G.2.4.3-3: P-Max (Step 5)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 26 |  | PC1.5 UE |

6.2G.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.2.5-1 to Table 6.2G.2.5-12.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.2.5-5 to Table 6.2G.2.5-12.

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.2.5-2b.

Table 6.2G.2.5-1: UE Power Class test requirements (for Bands n34, n39, n41, n77, n78, n79) for Power Class 2 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0- TT2） |
| 4 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5- TT2） |
| 5 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.0 - TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 8 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 9 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 12 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.0 - TT2） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0- TT2） |
| 15 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 16 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 17 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 18 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 19 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 20 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 21 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5- TT2） |
| 22 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 23 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.0 - TT2） |
| 24 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 26 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 27 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 28 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 29 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 30 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 31 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 32 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-2: UE Power Class test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | (27.0²) | 3.0 |  | 3 | 31.0 + TT | 25.5 - TT | (24.0 - TT²) |
| 2 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | (21.5²) | 3.0 | （5.02） | 3 | 31.0 + TT | 20.0 - TT | (16.5 - TT²) |
| 3 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | (21.5²) | 3.0 | （5.02） | 3 | 31.0 + TT | 20.0 - TT | (16.5 - TT²) |
| 4 | 29 | 0 | 2 | 0 | （1.52） | 27.0 | (25.5²) | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | (22.5 - TT²) |
| 5 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | (27.0²) | 3.0 |  | 3 | 31.0 + TT | 25.5 - TT | (24.0 - TT²) |
| 6 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 7 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 8 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | (25.0²) | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | (22.0 - TT²) |
| 9 | 29 | 0 | 1.5 | 0 | （1.52） | 27.5 | (26.0²) | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | (23.0 - TT²) |
| 10 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 11 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 12 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | (24.0²) | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | (21.0 - TT²) |
| 13 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | (24.0²) | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | (21.0 - TT²) |
| 14 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 15 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 16 | 29 | 0 | 4 | 0 | （1.52） | 25.0 | (23.5²) | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | (20.5 - TT²) |
| 17 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 18 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 19 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 20 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 21 | 29 | 0 | 2 | 0 | （1.52） | 27.0 | (25.5²) | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | (22.5 - TT²) |
| 22 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 23 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 24 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | (23.0²) | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | (20.0 - TT²) |
| 25 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | (25.0²) | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | (22.0 - TT²) |
| 26 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 27 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | (23.0²) | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | (20.0 - TT²) |
| 29 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | (23.0²) | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | (20.0 - TT²) |
| 30 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 31 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 32 | 29 | 0 | 5 | 0 | （1.52） | 24.0 | (22.5²) | 3.0 | （5.02） | 3 | 31.0 + TT | 21.0 - TT | (17.5 - TT²) |
| 33 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | (19.0²) | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | (14.0 - TT²) |
| 34 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | (19.0²) | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | (14.0 - TT²) |
| 35 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | (19.0²) | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | (14.0 - TT²) |
| 36 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | (19.0²) | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | (14.0 - TT²) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-2a: UE Power Class test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 FWA (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 0 | 0 | （1.53） | 29.0 | (27.53) | 3.0 |  | 3 | 31.0 + TT | 26.0 - TT | (24.5 – TT3) |
| 2 | 29 | 0 | 6 | 0 | （1.53） | 23.0 | (21.53) | 3.0 | (5.03) | 3 | 31.0 + TT | 20.0 - TT | (16.5 – TT3) |
| 3 | 29 | 0 | 6 | 0 | （1.53） | 23.0 | (21.53) | 3.0 | (5.03) | 3 | 31.0 + TT | 20.0 - TT | (16.5 – TT3) |
| 4 | 29 | 0 | 1.5 | 0 | （1.53） | 27.5 | (26.03) | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | (23.0 – TT3) |
| 5 | 29 | 0 | 0 | 0 | （1.53） | 29.0 | (27.53) | 3.0 |  | 3 | 31.0 + TT | 26.0 - TT | (24.5 – TT3) |
| 6 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 7 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 8 | 29 | 0 | 2 | 0 | （1.53） | 27.0 | (25.53) | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | (22.5 – TT3) |
| 9 | 29 | 0 | 1 | 0 | （1.53） | 28.0 | (26.53) | 3.0 |  | 3 | 31.0 + TT | 25.0 - TT | (23.5 – TT3) |
| 10 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 11 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 12 | 29 | 0 | 3.5 | 0 | （1.53） | 25.5 | (24.03) | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | (21.0 – TT3) |
| 13 | 29 | 0 | 3 | 0 | （1.53） | 26.0 | (24.53) | 3.0 |  | 3 | 31.0 + TT | 23.0 - TT | (21.5 – TT3) |
| 14 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 15 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 16 | 29 | 0 | 3.5 | 0 | （1.53） | 25.5 | (24.03) | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | (21.0 – TT3) |
| 17 | 29 | 0 | 5.5 | 0 | （1.53） | 23.5 | (22.03) | 3.0 |  | 3 | 31.0 + TT | 20.5 - TT | (19.0 – TT3) |
| 18 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 19 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 20 | 29 | 0 | 5.5 | 0 | （1.53） | 23.5 | (22.03) | 3.0 | (5.03) | 3 | 31.0 + TT | 20.5 - TT | (17.0 – TT3) |
| 21 | 29 | 0 | 1.5 | 0 | （1.53） | 27.5 | (26.03) | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | (23.0 – TT3) |
| 22 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 23 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 24 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | (23.53) | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | (20.5 – TT3) |
| 25 | 29 | 0 | 2 | 0 | （1.53） | 27.0 | (25.53) | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | (22.5 – TT3) |
| 26 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 27 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | (23.53) | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | (20.5 – TT3) |
| 29 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | (23.53) | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | (20.5 – TT3) |
| 30 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 31 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 32 | 29 | 0 | 4.5 | 0 | （1.53） | 24.5 | (23.03) | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | (20.0 – TT3) |
| 33 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | (20.03) | 5.0 | （6.03） | 3 | 31.0 + TT | 16.5 - TT | (14.0 – TT3) |
| 34 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | (20.03) | 5.0 | （6.03） | 3 | 31.0 + TT | 16.5 - TT | (14.0 – TT3) |
| 35 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | (20.03) | 5.0 | （6.03） | 3 | 31.0 + TT | 16.5 - TT | (14.0 – TT3) |
| 36 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | (20.03) | 5.0 | （6.03） | 3 | 31.0 + TT | 16.5 - TT | (14.0 – TT3) |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-2b: UE Power Class test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 falling back to Power Class 2 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 3 | 0 | 0 | （1.52） | 26.0 | (24.5²) | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | (21.5 - TT²) |
| 2 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 3 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 4 | 29 | 3 | 0.5 | 0 | （1.52） | 25.5 | (24.0²) | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | (21.0 - TT²) |
| 5 | 29 | 3 | 0 | 0 | （1.52） | 26.0 | (24.5²) | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | (21.5 - TT²) |
| 6 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 7 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 8 | 29 | 3 | 1 | 0 | （1.52） | 25.0 | (23.5²) | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | (20.5 - TT²) |
| 9 | 29 | 3 | 1 | 0 | （1.52） | 25.0 | (23.5²) | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | (20.5 - TT²) |
| 10 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 11 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 12 | 29 | 3 | 2 | 0 | （1.52） | 24.0 | (22.5²) | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | (17.5 - TT²) |
| 13 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 14 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 15 | 29 | 3 | 2.5 | 0 | （1.52） | 23.5 | (22.0²) | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | (17.0 - TT²) |
| 16 | 29 | 3 | 4.5 | 0 | （1.52） | 21.5 | (20.0²) | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | (14.0 - TT²) |
| 17 | 29 | 3 | 4.5 | 0 | （1.52） | 21.5 | (20.0²) | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | (14.0 - TT²) |
| 18 | 29 | 3 | 4.5 | 0 | （1.52） | 21.5 | (20.0²) | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | (14.0 - TT²) |
| 19 | 29 | 3 | 1.5 | 0 | （1.52） | 24.5 | (23.0²) | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | (20.0 - TT²) |
| 20 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 21 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 22 | 29 | 3 | 3 | 0 | （1.52） | 23.0 | (21.5²) | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | (16.5 - TT²) |
| 23 | 29 | 3 | 2 | 0 | （1.52） | 24.0 | (22.5²) | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | (17.5 - TT²) |
| 24 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 25 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 26 | 29 | 3 | 3 | 0 | （1.52） | 23.0 | (21.5²) | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | (16.5 - TT²) |
| 27 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 28 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 29 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 30 | 29 | 3 | 6.5 | 0 | （1.52） | 19.5 | (18.0²) | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | (13.0 - TT²) |
| 31 | 29 | 3 | 6.5 | 0 | （1.52） | 19.5 | (18.0²) | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | (13.0 - TT²) |
| 32 | 29 | 3 | 6.5 | 0 | （1.52） | 19.5 | (18.0²) | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | (13.0 - TT²) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of Changes>