3GPP TSG-RAN5 Meeting #100	R5-235703

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2499 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR SA FR2 SSB based Inter-cell L1-RSRP measurement in non-DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SSB based Inter-cell L1-RSRP measurement in non-DRX test case have been defined in 38.133 but are currently missing in RAN5 test specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added SSB based Inter-cell L1-RSRP measurement in non-DRX test case - Added cell mapping configuration for above test case in Annex E.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would be missing in the test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.3.6 (new), E.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0306 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Title correction to resolve 3GU issue. |  |  |  |  |  |  |  |  |

### 7.6.3	L1-RSRP measurement for beam reporting

#### 7.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting

-------- Unchanged Sections Skipped --------

-------- Start of first change --------

##### 7.6.3.0.3	Minimum conformance requirements for Inter-cell SSB based L1-RSRP Reporting

If a cell with PCI different from serving cell is known according to TS 38.133 [6] clause 9.13.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_CDP. The requirements specified in this clause are only applicable when

-	highSpeedMeasFlag-r16 is not configured, and

-	highSpeedMeasFlagFR2-r17 is not configured, and

-	highSpeedMeasCA-Scell-r17 is not configured, and

-	SSBs of CDP outside SMTCs for L1-RSRP measurement are not overlapped with concurrent gaps.

The value of TL1-RSRP_Measurement_Period_SSB_CDP is defined in Table 7.6.3.6.3-1 for FR2 when highSpeedMeasFlagFR2-r17 is not configured, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N= 8.

For FR2,

-	P is PL1_sharing*Psharing factor, CDP, when SSB is not overlapped with measurement gap and SSB is fully overlapped with SMTC period (TSSB_CDP = TSMTCperiod), and TSSB_SC = TSMTCperiod.

-	P is  PL1_sharing*Psharing factor1-TSSBxRP, when SSB is partially overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP), and TSSB_SC = TSMTCperiod.

-	P2=11-TSSB_CDPTSMTCperiod, when SSB is not overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB_CDP < TSMTCperiod).

-	P2= 11-TSSB_CDPMGRP-TSSB_CDPTSMTCperiod, when SSB is partially overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB_CDP < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB_CDP < 0.5*TSMTCperiod

-	P2= 11-TSSB_CDPmin(TSMTCperiod,MGRP),when SSB is partially overlapped with measurement gap (TSSB_CDP <MGRP) and SSB is partially overlapped with SMTC occasion (TSSB_CDP < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	If SSB resource from serving cell is configured for L1-RSRP measurements, and P1 is valid accoding to TS 38.133 [6] clause 9.5.4.1, and any symbol of the SSBs from serving cell and cell with different PCI are overlapping or adjacent (in time domain)

-	P = P21-P2*TSSB_CDPP1*TSSB_SC ,   if P2*TSSB_CDP < P1*TSSB_SC.

-	P = P2, if P2*TSSB_CDP> P1*TSSB_SC.

-	P = 2*P2, if P1*TSSB_SC = P2*TSSB_CDP.

-	Otherwise, P = P2

-	TSSB_CDP = SSB periodicity of the cell with PCI different from serving cell

-	TSMTCperiod = the configured SMTC period

-	TSSB_SC = ssb-periodicityServingCell of the serving cell

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = 3, otherwise.

-	PL1_sharing = 2, if SSB resource from serving cell is configured for L1-RSRP measurements, and Psharing_factor is used in 9.5.4.1, and any symbol of the SSBs from serving cell and cell with different PCI are overlapping or adjacent (in time domain). PL1_sharing = 1, otherwise.

If the high layer in TS 38.331 [6] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For FR2 cell with PCI different from serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For FR2 cell with PCI different from serving cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 7.6.3.6.3-1: Inter-cell L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_CDP for known cells with different PCIs in FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB_CDP (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB_CDP) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB_CDP)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB_CDP is the periodicity of the SSB-Index configured for inter-cell L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

The normative reference for this requirement is TS 38.133 [6] clause 9.13.4.1

-------- End of First change --------

-------- Unchanged Sections Skipped --------

-------- Start of Second change --------

#### 7.6.3.6 	NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used

Editor’s note: This test case is incomplete. The following aspects are missing:

- Message contents is TBD

- Measurement Uncertainties and Test Tolerances are missing

7.6.3.6.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in non-DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.13.4.1.

7.6.3.6.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

7.6.3.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.3.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.3.6.

7.6.3.6.4	Test description

7.6.3.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.3.6.4.1-1.

Table 7.6.3.6.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR PCell 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR SCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR PCell 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode NR SCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | NR PCell 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR SCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 4 | NR PCell 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR SCell 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 5 | NR PCell 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode NR SCell 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 6 | NR PCell 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR SCell 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 7.6.3.6.4.1-2: Cell specific test parameters for FR1 PCell

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~6 |  | freq1 |
| Duplex mode | 1,4 |  | FDD |
|  | 2,5 |  | TDD |
|  | 3,6 |  | TDD |
| TDD Configuration | 1,4 |  | N/A |
|  | 2,5 |  | TDDConf.1.1 |
|  | 3,6 |  | TDDConf.2.1 |
| BWchannel | 1,4 | MHz | 10: NRB,c = 52 |
|  | 2,5 |  | 10: NRB,c = 52 |
|  | 3,6 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1,4 |  | SR.1.1 FDD |
|  | 2,5 |  | SR.1.1 TDD |
|  | 3,6 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1,4 |  | CR.1.1 FDD |
|  | 2,5 |  | CR.1.1 TDD |
|  | 3,6 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1,4 |  | CCR.1.1 FDD |
|  | 2,5 |  | CCR.1.1 TDD |
|  | 3,6 |  | CCR.2.1 TDD |
| SSB configuration | 1,4 |  | SSB.3 FR1 |
|  | 2,5 |  | SSB.3 FR1 |
|  | 3,6 |  | SSB.4 FR1 |
| OCNG Patterns | 1~6 |  | OP.1 |
| Initial BWP Configuration | 1~6 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~6 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~6 |  | SMTC.1 |
| CSI-RS for tracking | 1,4 |  | TRS.1.1 FDD |
|  | 2,5 |  | TRS.1.1 TDD |
|  | 3,6 |  | TRS.1.2 TDD |
| DRX configuration | 1~6 |  | Off |
| SSB index assigned as RLM RS | 1~6 |  | 0,1 |
| SSB Index assigned as BFD RS (q0) | 1~6 |  | 0 |
| SSB Index assigned as CBD RS (q1) | 1~6 |  | 1 |
| EPRE ratio of PSS to SSS | 1~6 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note2 | 1~6 | dBm/15 kHz | -104 |
|  | 1~6 | dB | 17 |
|  | 1~6 | dB | 17 |
| SS-RSRP Note3 | 1~6 | dBm/15 kHz | -87 |
| IoNote3 | 1,2,4,5 | dBm/ 9.36MHz | -58.96 |
|  | 3,6 | dBm/ 38.16MHz | -52.86 |
| Propagation condition | 1~6 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselvess. |  |  |  |

Table 7.6.3.6.4.1-3: Cell specific test parameters for FR2 SCell

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~6 |  | Freq2 |
| Duplex mode | 1~6 |  | TDD |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |
| BWchannel | 1~6 | MHz | 100: NRB,c = 66 |
| Data RBs allocated | 1~6 |  | 66 |
| PDSCH Reference measurement channel | 1,2,3 |  | SR.3.2 TDD |
|  | 4,5,6 |  | SR.3.3 TDD |
| RMSI CORESET Reference Channel | 1,2,3 |  | CR.3.1 TDD |
|  | 4,5,6 |  | CR.3.2 TDD |
| Dedicated CORESET Reference Channel | 1,2,3 |  | CCR.3.1 TDD |
|  | 4,5,6 |  | CCR.3.7 TDD |
| SSB configuration | 1,2,3 |  | SSB.1 FR2 |
|  | 4,5,6 |  | SSB.2 FR2 |
| OCNG Patterns | 1~6 |  | OP.1 |
| Initial BWP Configuration | 1~6 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~6 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1~6 |  | SMTC.1 |
| TRS Configuration | 1~6 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~6 |  | TCI.State.2 |
| DRX configuration | 1~6 |  | Off |
| reportConfigType | 1~6 |  | periodic |
| reportQuantity | 1~6 |  | ssb-Index-RSRP |
| Number of reported RS | 1~6 |  | 2 |
| SSB index associated to serving PCI | 1~6 |  | 0 |
| SSB index associated to a PCI different from serving cell | 1~6 |  | 1 |
| Timing offset between two SSBs associated to different PCI | 1~6 |  | <CP |
| L1-RSRP reporting period | 1~6 | slot | 320 |
| T1 | 1~6 | s | 5 |
| T2 | 1~6 | s | 2 |
| EPRE ratio of PSS to SSS | 1~6 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~6 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Configure the test equipment and the DUT according to the parameters in Table 7.6.3.6.4.1-4.

Table 7.6.3.6.4.1-4: Initial conditions for NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.3.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.3.3.1-1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1. Cell specific test parameter settings are set up according to Table 7.6.3.6.4.1-2 and 7.6.3.6.4.1-3

2. Message contents are defined in clause 7.6.3.6.4.3.

7.6.3.6.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1 and T2 respectively. Prior to the start of the time duration T1, the UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 7.6.3.6.4.1-2 and 7.6.3.6.4.1-3. Before the test, UE is configured to perform RLM and BFD on Cell 1 in FR1 and perform L1-RSRP measurements on the SSB#0 in FR2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.6.3.6.5-1. T1 starts.

3.	The UE shall be transmitting CSI on PUCCH with a periodicity of 320 slots.

4.	When T1 expires, the SS shall set the parameters according to T2 in 7.6.3.6.5-1. T2 starts. SSB#1 starts transmission, and the UE is configured for L1-RSRP measurement on SSB#1.

5.	The UE shall start sending valid L1-RSRP reports. The SS shall check the following requirements:

- R1: the UE shall start to transmit valid L1-RSRP reports no later than 2200ms for UE supporting power class 1 in configuration 1,2 and 3, no later than 2180 ms for UE supporting power class 1 in configuration 4, 5 and 6, no later than 1720 ms for UE supporting power class 2, 3 and 4 in configuration 1 and no later than 1700 ms for UE supporting power class 2, 3 and 4 in configuration 2 from the beginning of time period T2. A valid report shall meet the absolute L1-RSRP requirement for SSB#1 Table 7.6.3.1.5-2 for test configuration 1 and the corresponding absolute accuracy requirements in Table 7.6.3.1.5-3 for test configuration 2. If the first valid report is received before the specified time, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2: the UE shall transmit L1-RSRP reports every 320 slots. If the reports are received accordingly, the number of passed iterations for R2 is increased by one. Otherwise, the number of failed iterations for R2 is increased by one.

-R3: The L1-RSRP value of SSB#1 reported by the UE is compared to the expected L1-RSRP value for SSB#1. In all consecutive reports after the first valid value is received, if the resulting value is outside the corresponding absolute accuracy requirements in Table 7.6.3.1.5-2 for test configuration 1 and the corresponding absolute accuracy requirements in Table 7.6.3.1.5-3 for test configuration 2 the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

-R4: The DIFF-RSRP value of SSB#0 reported by the UE is compared to the expected DIFF-RSRP value. In all consecutive reports after the first valid value is received, if the resulting value is outside the corresponding relative accuracy requirements in Table 7.6.3.1.5-4 for all test configurations, the number of failed iterations for R4 is increased by one. Otherwise, the number of passed iterations for R4 is increased by one.

6.	The SS waits until T2 expires.

7.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

9.	Repeat steps 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.6.3.6.4.3	Message contents

TBD

7.6.3.6.5	Test requirements

Table 7.6.3.6.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.3.6.5-1: SSB specific test parameters for FR2 SCell

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration | 1~6 |  | Setup 3 according to A.9.3 |  |  |  |
|  |  |  | AoA1 |  | AoA2 |  |
| Beam AssumptionNote 4 | 1~6 |  | Rough |  | Rough |  |
| Note2 | 1~6 | dBm/15kHz | -105+ TT |  | -105+ TT |  |
| Note2 | 1~3 | dBm/SSB SCS | -96+ TT |  | -96+ TT |  |
|  | 4~6 |  | -93+ TT |  | -93+ TT |  |
|  | 1~2 | dB | 0+ TT | 0+ TT | -Infinity | 9+ TT |
| SSB_RP Note3 | 1 | dBm/SSB SCS | -96+ TT | -96+ TT | -Infinity | -87+ TT |
|  | 2 |  | -93+ TT | -93+ TT | -Infinity | -84+ TT |
| Io Note3 | 1 | dBm/95.04MHz | -63.97+ TT | -63.97+ TT | -66.98+ TT | -57.47+ TT |
|  | 2 |  | -63.97+ TT | -63.97+ TT | -66.98+ TT | -57.47+ TT |
|  | 1~2 | dB | 0+ TT | 0+ TT | -Infinity | 9+ TT |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for ￼ to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 320 slots. No later than X ms plus 320 slots from the beginning of time period T2, UE shall send L1-RSRP report including the results for SSB#1 while meeting the accuracy requirements defined in clause 10.1.20.1, where X is

-	2160 for UE supporting power class 1

-	1680 for UE supporting power class 2, 3 or 4.

Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 7.6.3.6.5-2 for test configuration 1 and the corresponding absolute accuracy requirements in Table 7.6.3.6.5-3 for test configuration 2 and the corresponding relative accuracy requirements in Table 7.6.3.6.5-4 for all test configurations.

Table 7.6.3.6.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configuration 1 (R1 and R3)

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#0 and SSB#1) | - | 40 |
| Highest reported value (SSB#0 and SSB#1) | - | 99 |

Table 7.6.3.6.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configuration 2 (R1 and R3)

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#0 and SSB#1) | - | 43 |
| Highest reported value (SSB#0 and SSB#1) | - | 102 |

Table 7.6.3.6.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations (R4)

|  | T1 | T2 |
| --- | --- | --- |
| Lowest DIFF RSRP reported (SSB#0) | - | 1 |
| Highest DIFF RSRP reported (SSB#0) | - | 7 |

For the test to pass, the ratio of successful reported valued for each requirement (R1 to R4) shall be at least 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

----- End of Second change ------

----Unchanged Clauses Skipped -----

-------- Start of Third change --------

# E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7

Table E.5-1 defines the cell configuration mapping for NR/5GC FR2 test cases in chapter 7 of this test specification.

Table E.5-1: Cell configuration mapping for SA FR2 RRM testing

| TC |  | Description |  | 38.533 NR Cell1 |  | 38.533 NR Cell2 |  | 38.533 NR Cell3 |  | 38.533 NR Cell4 |  | CA Type |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7.1.1.1 |  | NR SA FR2 cell re-selection |  | NR Cell 1 |  | NR Cell 11 |  |  |  |  |  |  |  |  |
| 7.1.1.2 |  | NR SA FR2-FR2 cell re-selection |  | NR Cell 1 |  | NR Cell 23 |  |  |  |  |  |  |  |  |
| 7.1.1.3 |  | NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.1.1.4 |  | NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.1.1.5 |  | NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.1.1.6 |  | NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.2.1.1 |  | TA validation for CG-SDT in FR2 |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |
| 7.3.1.1 |  | NR SA FR1-FR2 Inter-frequency handover; unknown target cell |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  |  |  |  |
| 7.3.1.2 |  | NR SA FR2 Intra-frequency handover; unknown target cell |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.3.1.3 |  | NR SA FR2-FR2 Inter-frequency handover; unknown target cell |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  |  |  |  |
| 7.3.1.4 |  | NR SA FR1-FR2 synchronous DAPS handover |  | NR Cell 1 |  | NR Cell 10 |  |  |  |  |  |  |  |  |
| 7.3.1.5 |  | NR SA FR1-FR2 asynchronous DAPS handover |  | NR Cell 1 |  | NR Cell 10 |  |  |  |  |  |  |  |  |
| 7.3.2.1.1 |  | NR SA FR2 RRC re-establishment |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.3.2.1.2 |  | NR SA FR2-FR2 re-establishment |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.3.2.1.3 |  | NR SA FR2 RRC re-establishment without serving cell timing |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.3.2.2.1 |  | NR SA FR2 contention based random access |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.3.2.2.2 |  | NR SA FR2 non-contention based random access |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.3.2.2.4 |  | NR SA FR2 2-step non-contention based random access |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.3.2.3.1 |  | NR SA FR2-FR2 RRC connection release with redirection |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  |  |  |  |
| 7.3.3.1 |  | NR SA FR2 conditional handover |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.3.3.2 |  | NR SA FR2-FR2 conditional handover |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  |  |  |  |
| 7.4.1.1 |  | NR SA FR2 UE transmit timing accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.4.3.1 |  | NR SA FR2 timing advance adjustment accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.1.1 |  | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.1.2 |  | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.1.3 |  | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.1.4 |  | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.1.9 |  | NR SA FR2 radio link monitoring UE scheduling restrictions |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.5.1 |  | NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.5.2 |  | NR SA FR2 SSB-based beam failure detection and link recovery in DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.5.3 |  | NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.5.4 |  | NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.5.6 |  | NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in non-DRX |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  |  |  |  |
| 7.5.5.7 |  | NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in DRX |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  |  |  |  |
| 7.5.6.1 |  | NR SA FR2 DCI-based DL active BWP switch in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.11.1 |  | NR SA FR2 UE UL carrier RRC reconfiguration delay |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.6.1.1 |  | NR SA FR2 event-triggered reporting without gap in non-DRX |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.6.1.2 |  | NR SA FR2 event-triggered reporting without gap in DRX |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.6.1.3 |  | NR SA FR2 event-triggered reporting with gap in non-DRX |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.6.1.4 |  | NR SA FR2 event-triggered reporting with gap in DRX |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.6.2.1 |  | NR SA FR2-FR2 event-triggered reporting in non-DRX |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.6.2.2 |  | NR SA FR2-FR2 event-triggered reporting in DRX |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.6.2.3 |  | NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.6.2.4 |  | NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.6.2.5 |  | NR SA FR1-FR2 event-triggered reporting in non-DRX |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.6.2.6 |  | NR SA FR1-FR2 event-triggered reporting in DRX |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.6.2.7 |  | NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.6.2.8 |  | NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.6.3.1 |  | NR SA FR2 SSB-based L1-RSRP measurement in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.6.3.2 |  | NR SA FR2 SSB-based L1-RSRP measurement in DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.6.3.3 |  | NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.6.3.4 |  | NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.6.3.6 |  | NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used |  | NR Cell 10 |  | NR Cell 1 |  |  |  |  |  | Inter-band |  |  |
| 7.6.6.1 |  | NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.6.6.2 |  | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.6.6.3 |  | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.6.13.1 |  | UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2 |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.7.1.1 |  | NR SA FR2 SS-RSRP measurement accuracy |  | NR Cell 489 |  | NR Cell 1 |  |  |  |  |  |  |  |  |
| 7.7.1.2 |  | NR SA FR2-FR2 SS-RSRP measurement accuracy |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.7.1.3.1 |  | NR SA FR1-FR2 SS-RSRP absolute measurement accuracy |  | NR Cell 1 |  | NR Cell 10 |  |  |  |  |  |  |  |  |
| 7.7.1.3.2 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7.2.1 |  | NR SA FR2 SS-RSRQ measurement accuracy |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.7.2.2 |  | NR SA FR2-FR2 SS-RSRQ measurement accuracy |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.7.3.1 |  | NR SA FR2 SS-SINR measurement accuracy |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.7.3.2 |  | NR SA FR2-FR2 SS-SINR measurement accuracy |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.7.6.1 |  | NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.7.6.2 |  | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.7.6.3 |  | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |

----- End of Third change ------