3GPP TSG-RAN5 Meeting #100	R5-234997

Toulouse, France, August 21 - 25 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2628 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of cell configuration mapping for event triggered measurement RedCap test cases to Annex E |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RRM RedCap event triggered measurement test cases are missing in the Annex E of TS 38.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Annex E has been updated. Cell configuration mapping for vent triggered measurement test cases has been defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Event triggered measurement test cases cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of changes

# E.14	Cell configuration mapping for SA FR1 test cases for RedCap in Chapter 16

Table E.14-1 defines the cell configuration mapping for NR/5GC FR1 test cases in chapter 16 of this test specification.

Table E.14-1: Cell configuration mapping for SA FR1 RRM testing for RedCap

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 |
| --- | --- | --- | --- | --- | --- |
| 16.1.1.1 | NR SA FR1 Cell reselection for 1 Rx UE | NR Cell 1 | NR Cell 11 |  |  |
| 16.1.1.2 | NR SA FR1 Cell reselection for 2 Rx UE | NR Cell 1 | NR Cell 11 |  |  |
| 16.1.1.3 | NR SA FR1-FR1 Cell reselection for 1 Rx UE | NR Cell 6 | NR Cell 23 |  |  |
| 16.1.1.4 | NR SA FR1-FR1 Cell reselection for 2 Rx UE | NR Cell 6 | NR Cell 23 |  |  |
| 16.1.1.5 | NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | NR Cell 1 | NR Cell 11 |  |  |
| 16.1.1.6 | NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | NR Cell 1 | NR Cell 11 |  |  |
| 16.1.1.7 | NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | NR Cell 6 | NR Cell 23 |  |  |
| 16.1.1.8 | NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | NR Cell 6 | NR Cell 23 |  |  |
| 16.3.1.1 | NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.3.1.2 | NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.3.1.3 | NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.3.1.4 | NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.3.1.5 | NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.3.1.6 | NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.3.2.1.1 | NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.3.2.1.2 | NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.3.2.1.3 | NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.3.2.1.4 | NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.3.2.1.5 | NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |
| 16.3.2.1.6 | NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |
| 16.3.2.2.1 | NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE | Cell 1 |  |  |  |
| 16.3.2.2.2 | NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE | Cell 1 |  |  |  |
| 16.3.2.2.3 | NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE | Cell 1 |  |  |  |
| 16.3.2.2.4 | NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE | Cell 1 |  |  |  |
| 16.3.2.2.5 | NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE | Cell 1 |  |  |  |
| 16.3.2.2.6 | NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE | Cell 1 |  |  |  |
| 16.3.2.2.7 | NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE | Cell 1 |  |  |  |
| 16.3.2.2.8 | NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE | Cell 1 |  |  |  |
| 16.3.2.3.1 | NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 1 Rx UE | NR Cell 6 | NR Cell 2 |  |  |
| 16.3.2.3.2 | NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 2 Rx UE | NR Cell 6 | NR Cell 2 |  |  |
| 16.4.1.1 | NR SA FR1 NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE | NR Cell 1 |  |  |  |
| 16.4.1.2 | NR SA FR1 NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE | NR Cell 1 |  |  |  |
| 16.4.3.1 | NR SA FR1 SA FR1 timing advance adjustment accuracy for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.4.3.2 | NR SA FR1 SA FR1 timing advance adjustment accuracy for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.1 | NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.2 | NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.3 | NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.4 | NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.7 | NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.8 | NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.9 | NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.10 | NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.11 | NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.12 | NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.13 | NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.14 | NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.15 | NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.16 | NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.2.1 | NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.2.2 | NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.2.5 | NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.2.6 | NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.2.7 | NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.2.8 | NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.4.1 | NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.4.2 | NR SA FR1 UE specific CBW change on PCell in non-DRX for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.6.1.1 | NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.2 | NR SA FR1 Event triggered reporting tests without gap under non-DRX for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.3 | NR SA FR1 Event triggered reporting tests without gap under DRX for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.4 | NR SA FR1 Event triggered reporting tests without gap under DRX for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.5 | NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.6 | NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.7 | NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.8 | NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.9 | NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.10 | NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.11 | NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.12 | NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.2.1 | NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.2 | NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.3 | NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.4 | NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.5 | NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.6 | NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.7 | NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.8 | NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.9 | NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.10 | NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.11 | NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.12 | NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.4.1 | NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.6.4.2 | NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.6.4.3 | NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.6.4.4 | NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.6.4.5 | NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.6.4.6 | NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.6.4.7 | NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.6.4.8 | NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE | NR Cell 1 |  |  |  |

Table E.14-2 defines the cell configuration mapping for NR/5GC FR1 – E-UTRA Inter-RAT test cases (serving cell in NR) for RedCap in chapter 16 of this test specification.

Table E.14-2: Cell configuration mapping for SA FR1 – E-UTRA Inter-RAT RRM testing for RedCap

| TC | Description | 38.533 NR Cell1 | 38.533 LTE Cell2 | 38.533 LTE Cell3 | 38.533 LTE Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 16.1.2.1 | NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.1.2.2 | NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2RX | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.1.2.3 | NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1RX | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.1.2.4 | NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2RX | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.1.2.5 | NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.1.2.6 | NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.3.1.7 | NR - E-UTRA handover for 1Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.3.1.8 | NR - E-UTRA handover for 2Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.3.1.9 | NR - E-UTRA handover with unknown target cell for 1 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.3.1.10 | NR - E-UTRA handover with unknown target cell for 2 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.3.2.3.3 | NR SA FR1-FR1 Redirection from NR in FR1 to E-UTRAN for 1 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.3.2.3.4 | NR SA FR1-FR1 Redirection from NR in FR1 to E-UTRAN for 2 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |

///End of changes