3GPP TSG-RAN5 Meeting #100	R5-234646

# Toulouse, France, 21st Aug 2023 – 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2548 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NTN intra-frequency measurement test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm India Pvt Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NTN intra-frequency measurement test cases 14.5.1.1, 14.5.1.2 and 14.5.1.3 have been completed in 38.133 and are missing in 38.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added basic structure for new chapter 14 to the spec - Added new general section 14.0 with testing principles and general setup configuration for NTN - Added new core section 14.5.1.0 - Added new test cases 14.5.1.1, 14.5.1.2 and 14.5.1.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases will be missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.0, 14.0.1, 14.0.2, 14.0.3, 14.0.4, 14.5.1, 14.5.1.0, 14.5.1.1, 14.5.1.2, 14.5.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

# 12 to 13	FFS

# 14	NR standalone tests for Satellite access

## 14.0	General

Editor's Note:

## ephemerisInfo is still FFS

The Satellite Access tests defined in this clause apply provided that UE indicates nonTerrestrialNetwork and is accessing a cell served by a Satellite Access Node (SAN). The requirements apply provided that serving and all neighbour satellites on the same layer are of same satellite type (LEO or GEO).

### 14.0.1	Principle of testing GSO and NGSO scenarios

The test cases related to satellite access are defined for both GSO and NGSO. The testing principle for these test cases is as follows:

-	A UE capable of GSO only is required to pass the test cases with GSO.

-	A UE capable of NGSO only is required to pass the test cases with NGSO.

-	A UE capable of both GSO and NGSO is required to pass the test cases with NGSO only.

Support of GSO and NGSO scenario is indicated via ntn-ScenarioSupport-r17.

### 14.0.2	Principle of testing different RRM requirements

Satellite Access RRM test cases are defined for all applicable RRM requirements. The testing principle for these test cases is as follows:

-	A UE capable of NTN only is required to pass all the test cases defined in clause 14.

-	A UE capable of both TN and NTN is required to pass the test cases for NTN specific requirements in Table 14.0.2-1.

Table 14.0.2-1: Test cases for NTN specific requirements

| Clause | Test case slogan |
| --- | --- |
| 14.1.2 | SA FR1 Cell reselection for UE configured with [capability for enhanced requirements] for NR satellite access |
| 14.1.3 | SA FR1 Time-based cell reselection for NR satellite access |
| 14.1.4 | SA FR1 Location-based cell reselection for NR satellite access |
| 14.1.7 | SA FR1-FR1 Cell reselection for UE configured with [capability for enhanced requirements] for NR satellite access |
| 14.1.8 | SA FR1-FR1 Time-based Cell reselection for NR satellite access |
| 14.1.9 | SA FR1-FR1 Location-based Cell reselection for NR satellite access |
| 14.2.1.3 | SA FR1 SAN time-based conditional Handover for NR satellite access |
| 14.2.1.4 | SA FR1-FR1 SAN time-based conditional Handover for NR satellite access |
| 14.2.1.5 | SA FR1 SAN distance-based conditional Handover for NR satellite access |
| 14.2.1.6 | SA FR1-FR1 SAN distance-based conditional Handover for NR satellite access |
| 14.3.1.1 | SA FR1 NR UE Transmit Timing Test for NR satellite access |
| 14.5.1.1 | SA FR1 event triggered reporting tests without gap under non-DRX for NR satellite access |
| 14.5.1.2 | SA FR1 event triggered reporting tests without gap under DRX for NR satellite access |
| 14.5.1.3 | SA FR1 event triggered reporting tests without gap under non-DRX with SSB index reading for NR satellite access |
| 14.5.1.4 | SA FR1 event triggered reporting tests with single measurement gap under non-DRX for satellite access for NR satellite access |
| 14.5.1.5 | SA FR1 event triggered reporting tests with FNO concurrent gaps under DRX for satellite access for NR satellite access |
| 14.5.1.6 | SA FR1 event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access for NR satellite access |
| 14.5.2.1 | SA FR1-FR1 Event triggered reporting test without gap under non-DRX for NR satellite access |
| 14.5.2.2 | SA FR1-FR1 Event triggered reporting tests without gap under DRX for NR satellite access |
| 14.6.3.1 | SA FR1 SA measurement accuracy for NR satellite access |
| 14.6.3.2 | SA FR1-FR1 SA measurement accuracy for NR satellite access |
| 14.6.4.1 | SA FR1 SSB based L1-RSRP measurement for NR satellite access |
| 14.6.4.2 | SA FR1 CSI-RS based L1-RSRP measurement on resource set with repetition off for NR satellite access |

### 14.0.3	Principle of testing different ephemeris formats

Satellite access RRM test cases are defined such that satellite ephemeris information is sent to UE in each test case, according to Tables 14.0.3-1 and 14.0.3-2.

Table 14.0.3-1: Test cases configuring EphemerisInfo as PositionVelocity

| Functional Area | Test Case |
| --- | --- |
| RRC_IDLE state mobility | 14.1.3 |
|  | 14.1.4 |
|  | 14.1.5 |
|  | 14.1.6 |
|  | 14.1.9 |
|  | 14.1.10 |
| Handover | 14.2.1.3 |
|  | 14.2.1.4 |
|  | 14.2.1.5 |
|  | 14.2.1.6 |
| Timing | 14.3.1 |
| Radio Link Monitoring | 14.4.1.1 |
|  | 14.4.1.2 |
|  | 14.4.1.5 |
|  | 14.4.1.6 |
| BFD and LR procedures | 14.4.2.1 |
|  | 14.4.2.3 |
|  | 14.4.2.5 |
| Active BWP switch | 14.4.3.1 |
| UE specific CBW change | 14.4.4.1 |
| PL-RS switching delay | 14.4.5.1 |
| Intra-frequency measurements | 14.5.1.1 |
|  | 14.5.1.3 |
|  | 14.5.1.5 |
| Inter-frequency measurements | 14.5.2.1 |
|  | 14.5.2.3 |
|  | 14.5.2.7 |
| L1-RSRP measurements | 14.5.3.1 |
|  | 14.5.3.3 |
| SS-RSRP accuracy | 14.6.1.1 |
| SS-RSRQ accuracy | 14.6.2.1 |
| SS-SINR accuracy | 14.6.3.1 |
| L1-RSRP accuracy | 14.6.4.1 |

Table 14.0.3-2: Test cases configuring EphemerisInfo as Orbital

| Functional Area | Test Case |
| --- | --- |
| RRC_IDLE state mobility | 14.1.1 |
|  | 14.1.2 |
|  | 14.1.5 |
|  | 14.1.6 |
| Handover | 14.2.1.1 |
|  | 14.2.1.2 |
| RRC Connection Mobiliy Control | 14.2.2.1 |
|  | 14.2.2.2 |
|  | 14.2.2.3 |
| Timing | 14.3.2 |
| Radio Link Monitoring | 14.4.1.3 |
|  | 14.4.1.4 |
|  | 14.4.1.7 |
|  | 14.4.1.8 |
| BFD and LR procedures | 14.4.2.2 |
|  | 14.4.2.4 |
|  | 14.4.2.6 |
| Active BWP switch | 14.4.3.2 |
| Intra-frequency measurements | 14.5.1.2 |
|  | 14.5.1.4 |
|  | 14.5.1.6 |
| Inter-frequency measurements | 14.5.2.2 |
|  | 14.5.2.4 |
|  | 14.5.2.6 |
|  | 14.5.2.8 |
| L1-RSRP measurements | 14.5.3.2 |
|  | 14.5.3.4 |
| SS-RSRP accuracy | 14.6.1.2 |
| SS-RSRQ accuracy | 14.6.2.2 |
| SS-SINR accuracy | 14.6.3.2 |
| L1-RSRP accuracy | 14.6.4.2 |

### 14.0.4	General setup for SIB19

The general parameters for SIB19 setup is specified in Table 14.0.4-1.

Table 14.0.4-1: SIB19 configuration for NTN test cases

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Interval between adjacent epoch time | s | LEO: 2.56 GEO: 10.24 |
| ntn-UlSyncValidityDuration | s | LEO: 5s GEO: 900s |
| cellSpecificKoffset | slot | LEO: 8 GEO: 256 |
| kmac | slot | Not configured |
| ta-Common |  | 0 |
| ta-CommonDrift |  | 0 |
| ta-CommonDriftVariant |  | 0 |
| ntn-PolarizationDL |  | linear |
| ntn-PolarizationUL |  | linear |
| ephemerisInfo |  | FFS |
| ta-Report |  | Not configured |

## 14.1	RRC_IDLE state mobility

FFS

## 14.2	RRC_CONNECTED state mobility

FFS

## 14.3	Timing for Satellite Access

FFS

## 14.4	Signalling characteristics

FFS

## 14.5	Measurement procedure

### 14.5.1	Intra-frequency Measurements

#### 14.5.1.0	Minimum conformance requirements

14.5.1.0.1	Minimum conformance requitements for intra-frequency measurements without measurement gaps

14.5.1.0.1.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 14.5.1.0.1.1-1

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 14.5.1.0.1.1-2

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 14.5.1.0.1.2-1

Kmulti_SMTC is the scaling factor for measurement of multiple SMTCs or multiple satellites, and

if SMTCs do not overlap with each other,

-	Kmulti_SMTC=1, if GEO satellites are measured on the carrier;

-	Kmulti_SMTC=NLEO,iNLEO,simul, if LEO satellites are measured on the carrier;

if SMTCs partially overlap with each other,

-	Kmulti_SMTC=NSMTC,overlap, if only GEO satellites are measured on the carrier;

-	Kmulti_SMTC=i=1NSMTC,overlapNLEO,iNLEO,simul, if only LEO satellites are measured on the carrier;

where

-	NLEO,i is the number of LEO satellites to be measured within i-th SMTC,

-	NLEO,simul is the number of LEO satellites that UE can measure in parallel within an SMTC,

-	NSMTC,overlap is the number of SMTCs that partially overlap with each other.

CSSFintra: it is a carrier specific scaling factor and is determined

according to CSSFoutside_gap,i in 38.133 [6] clause 9.1.5.1 for measurement conducted outside measurement gaps, i.e. when intra-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps,  or according to CSSFwithin_gap,i in 38.133 [6] clause 9.1.5.2 for measurement conducted within measurement gaps, i.e. when intra-frequency SMTC is fully overlapping with measurement gaps.

if the high layer in TS 38.331 [13] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1.

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal_SAN / Navailable_SAN, where Navailable_SAN and Ntotal_SAN are calculated as follows:

-	For a window W of duration max(SMTC period,  MGRP_max), where

-	If UE supports parallelMeasurementGap-r17 and is configured with concurrent measurement gaps, MGRP max is the maximum MGRP across all configured per-UE measurement gap. Otherwise, MGRP max is the MGRP of configured measurement gap.

-	Starting from the beginning of any SMTC occasion:

-	Ntotal_SAN is the total number of SMTC occasions within the window, including those overlapped and non-overlapped with measurement gap occasions within the window, and

-	Navailable_SAN is the number of SMTC occasions within the window W that don’t collide with any non-dropped MG occasion within or outside the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in 38.133 [6] section 9.1C.8.3. The collision rule between SMTC occasion and measurement gap occasion is defined in 38.133 [6] section 9.1C.9.1

Kp = [1] when Navailable_SAN = 0 and measurement gap sharing in 38.133 [6] clause 9.1.2.1a shall apply.

Kp = 1 when intra-frequency SMTC is fully non overlapping with measurement gaps.

For calculation of Kp, if the high layer signalling (TS 38.331 [13]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.

Klayer1_measurement: it is scaling factor for sharing between L3 and L1 measurement, and Klayer1_measurement =1, if GEO satellites are measured on the carrier, or if LEO satellites are measured on the carrier and UE supports parallelMeasurementWithoutRestriction, otherwise

Klayer1_measurement =1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement =1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

If the higher layer signaling in TS38.331 [13] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index.

Table 14.5.1.0.1.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5 x 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(5 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 14.5.1.0.1.1-2: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(3 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

The requirements in clause 14.5.1.0.1.1 and 14.5.1.0.1.2 are not applicable when the overall overhead ratio due to scheduling restriction caused by all configured SMTCs (i.e. scheduling restriction overhead of all SMTCs in one SMTC periodicity), is larger than 75%.

The normative reference for this requirement is TS 38.133 [6] clause 9.2C.5.1.

14.5.1.0.1.2	Measurement period

The measurement period for intra-frequency measurements without gaps is as shown in table 14.5.1.0.1.2-1.

If the higher layer signaling in TS38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for TSSB_measurement_period_intra.

Table 14.5.1.0.1.2-1: Measurement period for intra-frequency measurements without gaps (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

The normative reference for this requirement is TS 38.133 [6] clause 9.2C.5.2.

14.5.1.0.1.3	Scheduling availability of UE during intra-frequency measurements

When any of the conditions in the following clauses is met, there are restrictions on the scheduling availability; otherwise, there is no scheduling restriction. Note that the SSB symbols indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged (in TS 38.331 [13]), if it is configured; otherwise, all L SSB symbols within the SMTC window duration defined in clause 4.1 of TS 38.213 [8] are included.

The normative reference for this requirement is TS 38.133 [6] clause 9.2C.5.3.

14.5.1.0.1.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UE which do not support simultaneousRxDataSSB-DiffNumerology [11] the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement:

-	If deriveSSB_IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer signalling of smtc2 is configured (in TS 38.331 [13]), the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

-	If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [13], the SMTC periodicity follows smtc2; Otherwise, the SMTC periodicity follows smtc1.

The normative reference for this requirement is TS 38.133 [6] clause 9.2C.5.3.1.

14.5.1.0.1.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO

For UE which do not support TBD the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement on a neighbor cell served by a different satellite in LEO.

-	If deriveSSB_IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer signalling of smtc2 is configured (in TS 38.331 [13]), the SMTC periodicity follows smtc2; Otherwise, the SMTC periodicity follows smtc1.

-	If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [13], the SMTC periodicity follows smtc2; Otherwise, the SMTC periodicity follows smtc1.

-	If the following conditions are met:

-	The UE has been notified about system information update through paging,

-	The gap between the UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots

-	The UE is expected to receive the PDCCH that the UE monitors in the Type0-PDCCH CSS set, and/or the corresponding PDSCH, on SSB symbols to be measured.

The normative reference for this requirement is TS 38.133 [6] clause 9.2C.5.3.2.

#### 14.5.1.1	SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Exceptionts to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets

14.5.1.1.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 14.5.1.0.1.1 and 14.5.1.0.1.2.

14.5.1.1.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access.

14.5.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.5.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.5.1.1.

14.5.1.1.4	Test description

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in Table 14.5.1.1.4.1-1 and 14.5.1.1.4.1-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

During the test, the test system shall emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in clause B.4.1. The UE shall be provided with the valid information about the SAN serving each cell in the test before the start of the test.

UE is configured with 2 non-overlapping SMTCs for the intra-frequency measurement. The SMTC periodicity is 40ms, and SMTC1 is associated with Cell 1 with offset 0, and SMTC2 is associated with Cell 2 with offset 20ms.

14.5.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.5.1.1.4.1-1.

Table 14.5.1.1.4.1-1: Supported test configurations for SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access

| Configuration | Description |
| --- | --- |
| 14.5.1.1-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.5.1.1-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 14.5.1.1.4.1-2.

Table 14.5.1.1.4.1-2: Initial conditions for SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.5.1.1.4.1-3.

2.	Message contents are defined in clause 14.5.1.1.4.3.

3.	Cell 1 and Cell 2 are NR cells (PCell and neighbor cell, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1 and Cell 2 are satellite access cells].

Table 14.5.1.1.4.1-3: General test parameters for SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | Cell 1 |  |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  |
| NTN reference configuration |  | 1 | TBD | For GSO |
|  |  | 2 | TBD | For NGSO |
| SMTC1 configuration |  | 1 | TBD | Period: 40ms, offset: 0 |
| SMTC2 configuration |  | 2 | TBD | Period: 40ms, offset: 20ms |
| A3-Offset | dB | 1, 2 | -4.5 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 |  | OFF |
| Time offset between serving and neighbour cells |  | 1, 2 | 20 ms | Asynchronous cells. The timing of Cell 2 is 20ms later than the timing of Cell 1. |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

14.5.1.1.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

The UE is not required to read the neighbour cell SSB index in this test.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.5.1.1.5-1.

3.	SS shall transmit an RRCReconfiguration message configuring a MeasId consisting in an intra-frequency measurement object, event A3, and SMTC configuration according to Table 14.5.1.1.4.1-3.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.5.1.1.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by event A3. If the overall delay measured from the beginning of time period T2 is less than [800] ms, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. 	Once the connection is released, the SS [shall reset the satellite access channel model and then] switch OFF and then ON the UE and proceed with step 1.

10. Repeat step 2-9 until the confidence level [according to Tables G.2.3-1 in Annex G clause G.2 is achieved].

14.5.1.1.4.3	Message contents

FFS

14.5.1.1.5	Test requirements

Tables 14.5.1.1.4.1-3 and 14.5.1.1.5-1 define the primary level settings including test tolerances for SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access.

Table 14.5.1.1.5-1: Cell specific test parameters for SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| SSB configuration |  | 1, 2 | SSB.1 FR1 |  | TBD |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.1.1 FDD |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1, 2 | TRS.1.1 FDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1, 2 | -98+TT |  |  |  |
| Note 2 | dBm/15 kHz | 1, 2 | -98+TT |  |  |  |
|  | dB | 1, 2 | 4+TT | -1.46+TT | -Infinity | -1.46+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 4+TT |
| SS-RSRP Note 3 | dBm/SCS kHz | 1, 2 | -94+TT | -94+TT | -Infinity | -94+TT |
| Io | dBm/9.36 MHz | 1, 2 | -64.60+TT | -62.25+TT | -64.60+TT | -62.25+TT |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 14.5.1.2	SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Exceptionts to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets

14.5.1.2.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 14.5.1.0.1.1 and 14.5.1.0.1.2.

14.5.1.2.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access.

14.5.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.5.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.5.1.2.

14.5.1.2.4	Test description

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in Table 14.5.1.2.4.1-1 and 14.5.1.2.4.1-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

During the test, the test system shall emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in clause B.4.1. The UE shall be provided with the valid information about the SAN serving the each cell in the test before the start of the test.

UE is configured with 1 SMTC for the intra-frequency measurement. Both Cell 1 and Cell 2 are associated with the configured SMTC.

14.5.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.5.1.2.4.1-1.

Table 14.5.1.2.4.1-1: Supported test configurations for SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access

| Configuration | Description |
| --- | --- |
| 14.5.1.2-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.5.1.2-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 14.5.1.2.4.1-2.

Table 14.5.1.2.4.1-2: Initial conditions for SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.5.1.2.4.1-3.

2.	Message contents are defined in clause 14.5.1.2.4.3.

3.	Cell 1 and Cell 2 are NR cells (PCell and neighbor cell, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1 and Cell 2 are satellite access cells].

Table 14.5.1.2.4.1-3: General test parameters for SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2 | Cell 2 |  | Cell to be identified. |
| NTN reference configuration |  | 1 | TBD |  | For GSO |
|  |  | 2 | TBD |  | For NGSO |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  |  |
| SMTC configuration |  | 1, 2 | SMTC.2 |  |  |
| A3-Offset | dB | 1, 2 | -4.5 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| Time To Trigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.1 | DRX. 7 |  |
| Time offset between serving and neighbour cells |  | 1, 2 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | 5 | 10 |  |

14.5.1.2.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

The UE is not required to read the neighbour cell SSB index in this test.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.5.1.2.5-1.

3.	SS shall transmit an RRCReconfiguration message configuring a MeasId consisting in an intra-frequency measurement object, event A3, and SMTC configuration according to Table 14.5.1.2.4.1-3.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.5.1.2.5-1. T2 starts.

6. 	UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delay measured from the beginning of time period T2 is less than [TBD] ms for Test 1, or [TBD] ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. 	Once the connection is released, the SS [shall reset the satellite access channel model and then] switch OFF and then ON the UE and proceed with step 1.

10. Repeat step 2-9 until the confidence level [according to Tables G.2.3-1 in Annex G clause G.2 is achieved].

11. Repeat step 1-10 for each sub-test in Table 14.5.1.2.4.1-3 as appropriate.

14.5.1.2.4.3	Message contents

FFS

14.5.1.2.5	Test requirements

Tables 14.5.1.2.4.1-3 and 14.5.1.2.5-1 define the primary level settings including test tolerances for SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access.

Table 14.5.1.2.5-1: Cell specific test parameters for SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| SSB configuration |  | 1, 2 | SSB.1 FR1 |  | SSB.1 FR1 |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.1.1 FDD |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1, 2 | TRS.1.1 FDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1, 2 | -98+TT |  |  |  |
| Note 2 | dBm/15 kHz | 1, 2 | -98+TT |  |  |  |
|  | dB | 1, 2 | 4+TT | -1.46+TT | -Infinity | -1.46+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 4+TT |
| SS-RSRP Note 3 | dBm/SCS kHz | 1, 2 | -94+TT | -94+TT | -Infinity | -94+TT |
| Io | dBm/9.36 MHz | 1, 2 | -64.60+TT | -62.25+TT | -64.60+TT | -62.25+TT |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than TBD ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than TBD ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 14.5.1.3	SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are FFS

- Call setup and test procedure needs to be updated

- Applicability needs to be updated

- Exceptionts to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets

14.5.1.3.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clauses 14.5.1.0.1.1 and 14.5.1.0.1.2.

14.5.1.3.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access.

14.5.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.5.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.5.1.3.

14.5.1.3.4	Test description

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for FDD PCell and neighbour cell are given in Table 14.5.1.3.4.1-1 and 14.5.1.3.4.1-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

During the test, the test system shall emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in clause B.4.1. The UE shall be provided with the valid information about the SAN serving each cell in the test before the start of the test.

UE is configured with 2 overlapping SMTC for the intra-frequency measurement. The SMTC periodicity is 40ms, and SMTC1 is associated with Cell 1 with offset 0, and SMTC2 is associated with Cell 2 with offset 3ms.

14.5.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.5.1.3.4.1-1.

Table 14.5.1.3.4.1-1: Supported test configurations for SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access

| Configuration | Description |
| --- | --- |
| 14.5.1.3-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.5.1.3-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 14.5.1.3.4.1-2.

Table 14.5.1.3.4.1-2: Initial conditions for SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.5.1.3.4.1-3.

2.	Message contents are defined in clause 14.5.1.3.4.3.

3.	Cell 1 and Cell 2 are NR cells (PCell and neighbor cell, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1 and Cell 2 are satellite access cells].

Table 14.5.1.3.4.1-3: General test parameters for SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | Cell 1 |  |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. |
| NTN reference configuration |  | 1 | TBD | For GSO |
|  |  | 2 | TBD | For NGSO |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1, 2 | SSB.1 FR1 |  |
| SMTC1 configuration |  | 1, 2 | TBD |  |
| SMTC2 configuration |  | 1, 2 | TBD |  |
| A3-Offset | dB | 1, 2 | -4.5 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2 |  | OFF |
| Time offset between serving and neighbour cells |  | 1, 2 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

14.5.1.3.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.5.1.3.5-1.

3.	SS shall transmit an RRCReconfiguration message configuring a MeasId consisting in an intra-frequency measurement object, event A3, and SMTC configuration according to Table 14.5.1.3.4.1-3.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.5.1.3.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by event A3. If the overall delay measured from the beginning of time period T2 is less than [TBD] ms, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. 	Once the connection is released, the SS [shall reset the satellite access channel model and then] switch OFF and then ON the UE and proceed with step 1.

10. Repeat step 2-9 until the confidence level [according to Tables G.2.3-1 in Annex G clause G.2 is achieved].

14.5.1.3.4.3	Message contents

FFS

14.5.1.3.5	Test requirements

Tables 14.5.1.3.4.1-3 and 14.5.1.3.5-1 define the primary level settings including test tolerances for SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access.

Table 14.5.1.3.5-1: Cell specific test parameters for SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| SSB configuration |  | 1, 2 | SSB.1 FR1 |  | TBD |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.1.1 FDD |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1, 2 | TRS.1.1 FDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2 | DLBWP.0,1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1, 2 | -98+TT |  |  |  |
| Note 2 | dBm/15 kHz | 1, 2 | -98+TT |  |  |  |
|  | dB | 1, 2 | 4+TT | -1.46+TT | -Infinity | -1.46+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 4+TT |
| SS-RSRP Note 3 | dBm/SCS kHz | 1, 2 | -94+TT | -94+TT | -Infinity | -94+TT |
| Io | dBm/9.36 MHz | 1, 2 | -64.60+TT | -62.25+TT | -64.60+TT | -62.25+TT |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than TBD ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

# 15	FFS

<< End of Changes >>