3GPP TSG-RAN5 Meeting #100	R5-234493

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5236 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IoT NTN TC 22.2.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., ROHDE & SCHWARZ, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Accoring to the description in TS 24.301 Cl. 5.5.3.2, UE may trigger the TAU request when camp on Ncell 11 in T1. The UE in state EMM-REGISTERED shall initiate the tracking area updating procedure by sending a TRACKING AREA UPDATE REQUEST message to the MME, a) when the UE detects that the current TAI is not in the list of tracking areas that the UE previously registered in the MME, unless the UE is configured for "AttachWithIMSI" as specified in 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17] and is entering a tracking area in a new PLMN that is neither the registered PLMN nor in the list of equivalent PLMNs; To avoid this unexpecyted failure, it`s proposed to change the Ncell 11 as a multi-TAC cell too. So UE will select TAI-1 as current TAI due to below requirement in TS 24.301 Cl. 4.11.3 When a UE camps on a satellite E-UTRAN cell, the UE may receive multiple TACs from the lower layers. The UE shall construct TAIs from the multiple TACs (i.e. concatenate the identity of the current PLMN and each of the TACs) and select a TAI as follows: a) if at least one TAI belongs to the TAI list of the UE, the UE shall select a TAI which belongs to the TAI list. If there are multiple TAIs which belong to the TAI list the UE shall consider each of these TAIs equal and: - if the UE can determine a TAI which represents the best the tracking area of the UE's geographical location, the UE shall select a TAI which represents the best the tracking area of the UE's geographical location; and - otherwise, the UE shall select a TAI in an implementation-specific way. … The UE shall consider the selected TAI as the current TAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated conformance requirements and the specific SIB1-NB applied to Ncell 11 too. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN CR R5s230550 associated. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

22.2.13	NB-IoT / NTN / Multi-TAC

22.2.13.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED/RRC-IDLE and UE finds a cell which provides access by NB-IoT NTN RAT }

ensure that {

when { systemInformationBlockType1-NB broadcast more than one TAC per PLMN }

then { UE forwards the trackingAreaList to upper layers for mobility management procedure }

}

22.2.13.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clauses 5.2.2.7 and 6.7.2, TS 24.301 clause 4.11.3. Unless otherwise stated these are Rel-17 requirements.

[TS 36.331, clause 5.2.2.7]

Upon receiving the SystemInformationBlockType1-NB, the UE shall:

…

1>	else:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, trackingAreaList and cellIdentity for the cell as received in the cellAccessRelatedInfo;

1>	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE; or

1>	if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE:

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

2>	forward the trackingAreaList to upper layers, if present;

[TS 36.331, clause 6.7.2]

trackingAreaList A list of tracking area codes for the PLMN listed. For the first entry in plmn-IdentityList-v1700: If this field is present, the list of tracking area codes include the tracking area code in trackingAreaCode-r13 and the tracking area codes in trackingAreaList. If this field is absent, only trackingAreaCode-r13 applies. For other entries in plmn-IdentityList-v1700: If this field is present, the list of tracking area codes include the tracking area codes in trackingAreaList. If this field is absent, the list of tracking area codes of the preceding entry in plmn-IdentityList-v1700 applies. The total number of signalled tracking area codes across all PLMNs cannot be more than maxTAC-NB-r17.

[TS 24.301, clause 4.11.3]

When a UE camps on a satellite E-UTRAN cell, the UE may receive multiple TACs from the lower layers. The UE shall construct TAIs from the multiple TACs (i.e. concatenate the identity of the current PLMN and each of the TACs) and select a TAI as follows:

a)	if at least one TAI belongs to the TAI list of the UE, the UE shall select a TAI which belongs to the TAI list. If there are multiple TAIs which belong to the TAI list the UE shall consider each of these TAIs equal and:

-	if the UE can determine a TAI which represents the best the tracking area of the UE's geographical location, the UE shall select a TAI which represents the best the tracking area of the UE's geographical location; and

-	otherwise, the UE shall select a TAI in an implementation-specific way.

…

The UE shall consider the selected TAI as the current TAI. The UE shall select a TAI as described above when:

a)	the UE receives multiple TACs from the lower layers; or

b)	the UE has received multiple TACs from the lower layers upon starting to camping on the current cell and the TAI list, the list of "forbidden tracking areas for roaming", or the list of "forbidden tracking areas for regional provision of service" is updated.

22.2.13.3	Test description

22.2.13.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 and Ncell 11 as specified in TS 36.508 [18] clause 8.1.4.1.2 are configured.

-	System information combination 9 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used for both cells.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The pre-configured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	The UE is in State 3-NB Registered, Idle Mode on Ncell 1 according to TS 36.508 [18].

22.2.13.3.2	Test procedure sequence

Table 22.2.13.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 22.2.13.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 [18].

Table 22.2.13.3.2-1: Ncell configuration changes over time

|  | Parameter | Unit | Ncell 1 | Ncell 11 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | NRS EPRE | dBm/15kHz | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T1 | NRS EPRE | dBm/15kHz | “Off” | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 22.2.13.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts Ncell levels according to row T1 of table 22.2.13.3.2-1. | - | - | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 11 within 3 minutes? | --> | RRCConnectionRequest-NB | 1 | F |
| 3 | Check: Does the 'Test procedure to check UE response to Paging for Control Plane CIoT MT access' as described in TS 36.508 [18], clause 8.1.5A.2 take place on Ncell 11? | - | - | 1 | - |

22.2.13.3.3	Specific message contents

Table 22.2.13.3.3-1: SystemInformationBlockType1-NB for Ncell 1 and Ncell 11 (preamble and all steps, table 22.2.13.3.2-2)

| Derivation Path: TS 36.508 [18], Table 8.1.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-NTN-r17 SEQUENCE { |  |  |  |
| cellBarred-NTN-r17 | notBarred |  |  |
| plmn-IdentityList-v1700 SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-NB-v1700 SEQUENCE | 1 entry |  |  |
| PLMN-IdentityInfo-NB-v1700[1] SEQUENCE { |  | Entry 1 |  |
| trackingAreaList-r17 SEQUENCE (SIZE (1..maxTAC-NB-r17)) OF TrackingAreaCode SEQUENCE { | 2 entries |  | Ncell 1 |
| TrackingAreaCode[1] | TAC-1 | Entry 1 |  |
| TrackingAreaCode[2] | TAC-2 | Entry 2 |  |
| } |  |  |  |
| trackingAreaList-r17 SEQUENCE (SIZE (1..maxTAC-NB-r17)) OF TrackingAreaCode SEQUENCE { | 2 entries |  | Ncell 11 |
| TrackingAreaCode[1] | TAC-2 | Entry 1 |  |
| TrackingAreaCode[2] | TAC-1 | Entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.2.13.3.3-2: SystemInformationBlockType31-NB for Ncell 11 (preamble and all steps, table 22.2.13.3.2-2)

| Derivation Path: TS 36.508 [18], Table 8.1.4.3.3-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  | GSO |
| positionX-r17 | -23550424 |  |  |
| positionY-r17 | 22301093 |  |  |
| positionZ-r17 | 0 |  |  |
| } |  |  |  |
| orbitalParameters SEQUENCE { |  |  | NGSO |
| semiMajorAxis-r17 | FFS |  |  |
| eccentricity-r17 | FFS |  |  |
| periapsis-r17 | FFS |  |  |
| longitude-r17 | FFS |  |  |
| inclination-r17 | FFS |  |  |
| anomaly-r17 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

[End of change]