3GPP TSG-RAN5 Meeting #100	R5-235473

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4750 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NTN-IoT: NB-IoT Test Model updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160, ROHDE & SCHWARZ, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - New MMI commands are required for the configuration of NTN test cases. - During verification of the first NB-IoT NTN test case it has been exposed that because of rounding of the calculated delay, the resulting NtnUeSpecificTA may differ from the NtnUeSpecificTA calculated in the UE. Therefore, it is proposed instead of rounding the calculated delay, to round the resulting NtnUeSpecificTA. - The current test model requires updates to take into account the NTN delay (Koffset and Kmac). - RAN5 has decided, for test cases that are applicable to both GSO and NGSO, to give the flexibility to the test operator to select either GSO NTN scenario or NGSO NTN scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added new MMI commands. - Updated values of the delay and NtnUeSpecificTA correspondingly. - Clarified timing aspects related to Table 7A.4.1-1, Table 7A.4.2-1, Table 7A.4.3-1 and the RRC connection release sequence. - Introduced px_NTN_ScenarioUnderTest to Table 9.1-4 to control the satellite scenario type for the NTN configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - NTN can not be configured correctly. - The value of NtnUeSpecificTA provided to SS may appear to have a bigger deviation that one calculated by UE. - The NB-IoT NTN test model will remain incomplete. - RAN5 decision will not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 7A.3, 7A.14.2.2, 7A.14.2.2, 7A.14.2.3, 7A.14.2.4(new), 9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Submission CR R5s230535, this CR has TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 1 revision of R5-233827 & 1 revision of R5-235443. |  |  |  |  |  |  |  |  |

<Start of modification>

## 5.2	Upper Tester ASPs

There is one primitive defined carrying either an MMI or an AT command to be sent to the system adaptor and one common confirmation primitive to be sent by the system adaptor.

| TTCN-3 ASP Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name | UT_SYSTEM_REQ |  |  |
| TTCN-3 Type | Record |  |  |
| Cmd |  | TTCN-3 Type | union |
| AT |  | charstring carrying the AT command as defined in TS 27.007 [32], TS 27.005 [31] and TS 27.060 [33] |  |
| MMI |  | - Cmd (charstring) - List of parameters: - Name (charstring) - Value (charstring) |  |
| CnfRequired |  | TTCN-3 Type | Ut_CnfReq_Type |
|  |  | CNF_REQUIRED: SS shall reply with one confirmation (NOTE) NO_CNF_REQUIRED: SS shall swallow any confirmation generated LOCAL_CNF_REQUIRED: SS shall immediately send one confirmation when the command is submitted to the UE i.e. in case of operator interaction when the operator has confirmed the command, but SS shall not wait for the UE responding(NOTE) |  |
| NOTE: In the TTCN, a confirmation shall only be requested in cases when there is no signalling from the UE being triggered by the MMI/AT command |  |  |  |

| TTCN-3 ASP Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name | UT_COMMON_CNF |  |  |
| TTCN-3 Type | Record |  |  |
| Result |  | TTCN-3 Type | boolean |
|  |  | true: success false: failure NOTE: The return value false will result in the test immediately finishing with a fatal error. In case of AT commands, the SS shall return false if and only if the UE comes back with "ERROR" as result code according to ITU-T Rec V.250 clause 5.7 [60]. In case of e.g. "+CME ERROR" the SS shall return true and put the UE's response into the ResultString |  |
| ResultString |  | TTCN-3 Type | charstring |
|  |  | response by the UE for commands which request the UE to return a result, optional In case of AT commands, ResultString shall contain the complete line according to the specification of the AT command. Any carriage return and linefeed characters, defined in TS 27.007 [32] clause 3.1, contained in the response shall be included as the textual representations "<CR>" and "<LF>" respectively. |  |

The following MMI commands are defined.

Table 5.1: MMI commands

| Command | Parameters |  |  |
| --- | --- | --- | --- |
|  | Name | Value |  |
| "SWITCH_ON" | (none) |  |  |
| "SWITCH_OFF" | (none) |  |  |
| "POWER_ON" | (none) |  |  |
| "POWER_OFF" | (none) |  |  |
| "INSERT USIM" | "USIM" | <USIM> |  |
| "REMOVE_USIM" | (none) |  |  |
| "CHECK_PLMN" | "PLMN" | <PLMN ID> |  |
| "CHECK_ETWS_INDICATION" | “WARNING1” |  | <WARNING1> |
|  | “WARNING2” |  | <WARNING2> |
| “CHECK_ETWS_ALERT” | (none) |  |  |
| “CHECK_ETWS_NO_ALERT” | (none) |  |  |
| "CHECK_CMAS_INDICATION" | “WARNING1” |  | <WARNING1> |
|  | “WARNING2” |  | <WARNING2> |
| “CHECK_CMAS_ALERT” | (none) |  |  |
| “CHECK_CMAS_NO_ALERT” | (none) |  |  |
| "HRPD_PDN_CONNECTION” | (none) |  |  |
| “CHECK_SMS_LENGTH_CONTENTS" | "Length" |  | <Length> |
|  | "Msg" |  | <Msg> |
| "CONFIGURE_SMS_ONLY" | (none) |  |  |
| "DISABLE EPS CAPABILITY" | (none) |  |  |
| "DETACH_NON_EPS" | (none) |  |  |
| "CLEAR_STORED_ASSISTANCE_DATA" | (none) |  |  |
| "CHECK_DTCH_THROUGHCONNECTED" | (none) |  |  |
| "GERAN_UPLINK_DATA" | (none) |  |  |
| "SELECT_CSG" | "PLMN" |  | <PLMN ID> |
|  | "CSG" |  | <CSG ID> |
| “TRIGGER_USER_RESELECTION” | (none) |  |  |
| "REQUEST_NON_CALL_RELATED_SS" | (none) |  |  |
| "MBMS_SERVICE_INTEREST" | "PLMN" |  | <PLMN ID> |
|  | "Service" |  | <MBMS Service ID> |
|  | "Interest" |  | "ON" / "OFF" |
|  | "SAI" |  | <MBMS SAI> |
| “MBMS_SERVICE_ACTIVE” | "Service" |  | <MBMS Service ID> |
|  | "SAI" |  | <MBMS SAI> |
|  | “Active” |  | "ON" / "OFF" |
| “MBMS_PRIORITY_OVER_UNICAST” | "Priority Over Unicast" |  | "FALSE" / "TRUE" |
| "CHECK_ETWS_NO_INDICATION" | “WARNING1” |  | <WARNING1> |
|  | “WARNING2” |  | <WARNING2> |
| "CHECK_CMAS_NO_INDICATION" | “WARNING1” |  | <WARNING1> |
|  | “WARNING2” |  | <WARNING2> |
| "CHECK_CALL_DISPLAY" | "ToDisplay" |  | <NAME>/<NUMBER> |
| "CHECK_CSG" | "CSG" |  | < CSG ID > |
|  | "Included" |  | "FALSE" / "TRUE" |
| "CHECK_NITZ_DST" | "DaylightSavingTime:" |  | DST |
| "CHECK_RESPONSE_DISPLAY" | "ToDisplay" |  | <STRING> |
| "CHECK_SS_USER_IND" | "Supplementary Service" |  | <SUPPLEMENTARY SERVICE> |
|  | "ToDisplay" |  | <STRING> |
| “CHECK_SS_USER_IND_FAILURE” | (none) |  |  |
| "CONFIGURE_OPERATIONMODE" | "OPERATION MODE" |  | <OPERATION MODE> |
| “CONFIGURE_ANDSF_RULE” | “ANDSF Rule” |  | <ANDSF RULE> |
| “WLAN_ANDSF_CONFIGURED” | (none) |  |  |
| “WLAN_ANDSF_NOT_CONFIGURED” | (none) |  |  |
| "CMTLR_DUMMY" (see Note) | ”Notification Type” |  | <NOTIFICATION TYPE> |
| "CCFC_DUMMY" (see Note) | ”Status” |  | <STATUS> |
| "CNAP_DUMMY" (see Note) | ”Testname” |  | <TESTNAME> |
| "CTZE_DUMMY" (see Note) | "Year |  | <YEAR> |
|  | "Month" |  | <MONTH> |
|  | "Day" |  | <DAY> |
|  | "Hour" |  | <HOUR> |
|  | "Minutes" |  | <MINUTES> |
|  | "Seconds" |  | <SECONDS> |
|  | “Daylight SavingTime” |  | <DAYLIGHT SAVING TIME> |
|  | "TimeZone" |  | <TIMEZONE> |
| "CMWN_DUMMY" (see Note) | (none) |  |  |
| “D2D_DISCOVERY_CLEAR” | (none) |  |  |
| “D2D_DISCOVERY_CONFIGURATION” | “ProSeApplicationID” |  | <charstring> |
|  | "PLMN_List" |  | <PLMN ID_List> |
|  | T4005 |  | <T4005> |
| “D2D_DISCOVERY” | “ProSeApplicationID” |  | <charstring> |
|  | “Announce” |  | "ON" / "OFF" |
|  | “Monitor” |  | "ON" / "OFF" |
| “D2D_COMMUNICATION” | “ProSe Layer-2 Group ID” |  | <charstring> |
|  | “Transmit” |  | "ON" / "OFF" |
|  | “Receive” |  | "ON" / "OFF" |
|  | “Max size of UE transmitted Direct Communication data” |  | <bytes> |
| “D2D_COMMUNICATION_CONFIGURATION” | “ProSe Layer-2 Group ID” |  | <charstring> |
|  | "PLMN_List" |  | <PLMN ID_List> |
|  | T4005_Minutes |  | <T4005> |
|  | “Security Life time timer in minutes” (Default: 0) |  | <SecurityTimer> |
| "V2X_SIDELINK" | "Data Type" |  | "IP" / "NON-IP" |
| “INSERT_USIM_OR_PRECONFIGURE” | "USIM" |  | <USIM> |
| “DELETE_NTN_PLMN_NA” | (none) |  |  |
| “CONFIGURE_NTN_UE_POSITION” | "SpecReference" |  | <charstring> |
| Note: These MMI commands are used to indicate to the SS that a UT response is expected by the TTCN in the case when the AT command is not supported. They can be ignored if the UE supports the AT command. The prefix of these commands matches the start of the AT response expected by the TTCN |  |  |  |

<End of Modification>

<Start of next modification>

## 7A.3	Search space configurations

There are 3 kinds of search spaces in NB-IoT (TS 36.213 [30] clause 16.6):

-	Type1-NPDCCH common search space (Type1CSS): Paging

-	Type2-NPDCCH common search space (Type2CSS): Random access

-	NPDCCH UE-specific search space (UESS): UL grants and DL assignments for UE in connected mode

The search spaces define in which time windows the network may schedule DL assignments and UL grants.

The NPDCCH period is defined in TS 36.321 [16] clause 3.1 and corresponds to one search space cycle.

Scheduling of DL assignments and UL grants is not possible in subframes occupied by system information or synchronisation signals or in subframes outside the respective search space.

-	Timing of NB-IoT conformance test cases strongly depends on the system information scheduling (see clause 7A.2.2) and the search space configurations described in this clause.

In addition it is the responsibility of the SS to determine the exact timing for scheduling of DL assignments and UL grants.

Table 7A.3-1 shows the relevant parameters for search space calculations.

Table 7A.3-1: Parameters for search space calculations

| Rmax | Type1CSS | configured in SIB2-NB (PCCH-Config-NB-r13) |
| --- | --- | --- |
|  | Type2CSS | configured in SIB2-NB (NPRACH-Parameters-NB-r13) |
|  | UESS | configured in RadioResourceConfigDedicated-NB-r13 (NPDCCH-ConfigDedicated-NB-r13) |
| G | Type1CSS | (not applicable) |
|  | Type2CSS | configured in SIB2-NB (NPRACH-Parameters-NB-r13) |
|  | UESS | configured in RadioResourceConfigDedicated-NB-r13 (NPDCCH-ConfigDedicated-NB-r13) |
| offset | Type1CSS | (not applicable) |
|  | Type2CSS | configured in SIB2-NB (NPRACH-Parameters-NB-r13) |
|  | UESS | configured in RadioResourceConfigDedicated-NB-r13 (NPDCCH-ConfigDedicated-NB-r13) |
| T | Type1CSS | DRX cycle according to PCCH-Config-NB-r13.defaultPagingCycle-r13 |
|  | Type2CSS | search space cycle: Rmax * G |
|  | UESS | search space cycle: Rmax * G |
| R | Type1CSS | depends on Rmax and the "DCI subframe repetition number" in the DCI format N2 as shown in TS 36.213 [30] table 16.6-2 |
|  | Type2CSS | depends on Rmax and the "DCI subframe repetition number" in the DCI format N1 as shown in TS 36.213 [30] table 16.6-3 |
|  | UESS | depends on Rmax and the "DCI subframe repetition number" in the DCI format N0/N1 as shown in TS 36.213 [30] table 16.6-1 |

<End of Modification>

<Start of next modification>

## 7A.14	Non-Terrestrial Network (NTN)

### 7A.14.1	NTN cell configuration

An NB-IoT cell to be used in a Non-Terrestrial Network is an NB-IoT cell that is broadcasting a system block combination 8 or 9 as defined in clause 8.1.4.3.1.1 of TS 36.508 [3], i.e. it shall at least broadcast SystemInformationBlockType31-NB (SIB31-NB). In addition to that, the NB-IoT NTN cell shall be configured as "barred" in cellAccessRelatedInfo-r13 IE and as "notBarred" in cellAccessRelatedInfo-NTN-r17 IE of SystemInformationBlockType1-NB.

TTCN implementation is responsible for maintaining the system information up-to-date and to ensure that the updated system information is being broadcast in the NB-IoT NTN cell before the timer T317 expires at UE.

### 7A.14.2	NTN timing considerations

#### 7A.14.2.1	General

DL and UL are frame aligned at the UL time synchronization reference point (RP). To accommodate the long propagation delays in NTN, the timing relationships are defined using a Common Timing Advance (Common TA) and two scheduling offsets: Koffset and Kmac, as illustrated in Figure 7A.14.2.1-1.

Figure 7A.14.2.1-1: NTN timing relationship parameters

#### 7A.14.2.2	NTN timing advance

The Timing Advance (TA) formula, specified in TS 36.211 [35] clause 8.1, can be summarized as follows:

TTA = TTA,legacy + TTA,NTN

TTA,NTN = TTA,common + TTA,UE-specific

with:

-	TTA,legacy = (NTA + NTA,offset) * Ts: the (legacy) part of the TA formula that applies also to Terrestrial Networks,

-	TTA,common =  * Ts: the Common TA between the RP and the NTN payload,

-	TTA,UE-specific =  * Ts: the TA corresponding to the service link delay.

The GSO test environment in TS 36.508 [3] specifies that both the serving satellite (NTN payload) and the UE under test have a static position during the lifetime of a test case execution. Therefore the corresponding TTA is also static for a given NTN cell. TTCN provides to the SS the following inputs to enable the SS to calculate TTA:

Table 7A.14.2.2-1: NTN timing advance - input parameters

| TTA element | Associated parameter(s) |  | Relationship |
| --- | --- | --- | --- |
| (in sec) | Protocol parameter | ASP field | (TS 36.213 [30] clause 16.1.2) |
| TTA,legacy |  | InitialValue integer (0..2047) | NTA = InitialValue * 16 |
| TTA,common |  | NtnCommonTA integer (0..8316827) | See below. |
| TTA,UE-specific |  | NtnUeSpecificTA integer (0..8316827) | NtnUeSpecificTA See below. |
| NOTE: The time unit TS is defined in TS 36.211 [35] clause 4. |  |  |  |

From the nta-CommonParameters-17 specified in TS 36.331 [19], only the value of nta-Common-r17 is defined in TS 36.508 [3] SIB31-NB for the GSO scenario. TTCN will forward this value to the SS as NtnCommonTA for the calculation by the SS of  according to the formula given in TS 36.213 [30] clause 16.1.2.

NOTE:	For the GSO scenario it is assumed that the SS uses the calculation:  = NtnCommonTA.

is calculated using the GSO serving satellite static position specified in TS 36.508 [3] SIB31-NB's ephemeris data and the UE static position specified in TS 36.508 [3] clause 4.13. The end result is provided to the SS by TTCN. The calculation is provided hereafter:

The two-way propagation delay in sec between the GSO serving satellite and the non-moving UE is assumed to be constant at any time and is calculated according to the formula:

where c is the speed of light (299792458 m/sec) and d is the distance in meters between the serving satellite and the UE.

For two points in Euclidian geometry d in meters is derived by:

with

and

, ,  being serving satellite coordinates, in m

, ,  being UE coordinates, in m

To use above formulas the positioning data provided in TS 36.508 [3] is first converted to the coordinates accordingly.

Example of the calculation of :

given UE latitude 25.08439333 and positionX-r17 of -16976014:

-16976014* 1.3 m = -22068818.2 m

-3025296.94 m

Following the above calculations, the final value t results in 0.24338972709427653 sec that yields  to have a value of 7476932.416336175, that is rounded to 7476932.

#### 7A.14.2.3	NTN scheduling offsets

According to TS 36.213 [30] clause 16.5.1 and TS 36.321 [16] clause 7.7, NPUSCH transmissions in NTN are delayed by the sum of the NTN scheduling offsets Koffset and Kmac, further referred to as tNTNdelay.

TTCN configures the NTN scheduling offsets Koffset and Kmac in the SS as part of the NTN cell configuration. The SS shall use those parameters to adjust the timing & scheduling of DL & UL transmissions specified in clauses 7A.4, 7A.5 and 7A.6 as follows:

-	Table 7A.4.1-1: Timing of random access procedure

| tdelay3 | 12ms + tNTNdelay | 12ms: scheduling delay according to RAR grant (TS 36.213 [30] clause 16.3.3) tNTNdelay: based on the sum of Koffset and Kmac values in TS 36.508 [3] |
| --- | --- | --- |

-	Table 7A.4.2-1: Timing of uplink transmissions

| tdelay1 | 8ms + tNTNdelay | 8ms: according to TS 36.213 [30] table 16.5.1-1 tNTNdelay: based on the sum of Koffset and Kmac values in TS 36.508 [3] |
| --- | --- | --- |

-	Table 7A.4.3-1: Timing of downlink transmissions

| tdelay2 | 12ms + tNTNdelay | 12ms: depending HARQ-ACK resource allocation according to TS 36.213 [30] clause 16.4.2 tNTNdelay: based on the sum of Koffset and Kmac values in TS 36.508 [3] |
| --- | --- | --- |

#### 7A.14.2.4	RRC connection release sequence

The sequence specified in clause 7.1.7A applies, with the following differences: the timings defined for steps 2, 3 and 4 shall be prolonged by the value of tNTNdelay.

<End of Modification>

<Start of next modification>

## 9.1	E-UTRAN PIXIT

Table 9.1-1: Common PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_AccessPointName | octetstring |  |  | Access Point Name, as defined in TS 23.003 [48] and used in TS 24.008 [20], clause 10.5.6.1 |
| px_AttachTypeTested | EUTRA_ATTACH_TESTED_Type | EPS_ATTACH_ONLY | EPS_ATTACH_ONLY, COMBINED_ATTACH | Attach Type to be tested, if UE supports both pc_Attach and pc_Combined_Attach |
| px_eAuthRAND | B128_Type | oct2bit(‘A3DE0C6D363E30C364A4078F1BF8D577’O) |  | Random Challenge |
| px_EllipsoidPointWithAltitude | O8_Type |  |  | Ellipsoid Point With Altitude value to be provided in Update UE location information (see 36.509 cl. 6.12) |
| px_HorizontalVelocity | O3_Type |  |  | Horizontal Velocity value to be provided in Update UE location information (see 36.509 cl. 6.12) |
| px_eJapanMCC_Band6 | NAS_Mcc | ‘442’H |  | Japan MCC code to be used for Band 6. The same value will be used for E-UTRA and Inter-RAT cells. Type is different to that defined in TS 34.123-3 [7] |
| px_PWS_CB_DataPage1 | charstring |  |  | ETWS or CMAS Page 1 warning data message |
| px_PWS_CB_DataPage2 | charstring |  |  | ETWS or CMAS Page 2 warning data message |
| px_PWS_CB_DataPage3 | charstring |  |  | ETWS or CMAS Page 3 warning data message |
| px_PWS_CB_DataPage4 | charstring |  |  | ETWS or CMAS Page 4 warning data message |
| px_PWS_CB_DataCodingScheme | bitstring |  |  | ETWS or CMAS data coding scheme of the alphabet/coding and the applied language [see TS 23.041] |
| px_IPv4_Address1_UE | charstring |  |  | IPv4 Address connected to PDN1 |
| px_IPv4_Address2_UE | charstring |  |  | IPv4 Address connected to PDN2 |
| px_IPv4_Address2a_UE | charstring |  |  | IPv4 Address connected to PDN2a |
| px_IPv4_Address3_UE | charstring |  |  | IPv4 Address connected to PDN3 |
| px_IPv4_Address1_NW | charstring |  |  | IPv4 Gateway Address in PDN1 |
| px_IPv4_Address2_NW | charstring |  |  | IPv4 Gateway Address in PDN2 |
| px_IPv4_Address3_NW | charstring |  |  | IPv4 Gateway Address in PDN3 |
| px_IPv4_Address1_RemoteUE | charstring |  |  | IPv4 Address of remote UE connected to PDN1 |
| px_IPv4_Address2_RemoteUE | charstring |  |  | IPv4 Address of remote UE connected to PDN2 |
| px_IPv4_Address3_RemoteUE | charstring |  |  | IPv4 Address of remote UE connected to PDN3 |
| px_IPv4_Address_HomeAgent | charstring |  |  | IPv4 Home Agent Address |
| px_IPv6_Address1_UE | charstring |  |  | IPv6 Address connected to PDN1 |
| px_IPv6_Address2_UE | charstring |  |  | IPv6 Address connected to PDN2 |
| px_IPv6_Address2a_UE | charstring |  |  | IPv6 Address connected to PDN2a |
| px_IPv6_Address3_UE | charstring |  |  | IPv6 Address connected to PDN3 |
| px_IPv6_Address1_NW | charstring |  |  | IPv6 Gateway Address in PDN1 |
| px_IPv6_Address2_NW | charstring |  |  | IPv6 Gateway Address in PDN2 |
| px_IPv6_Address3_NW | charstring |  |  | IPv6 Gateway Address in PDN3 |
| px_IPv6_Address1_RemoteUE | charstring |  |  | IPv6 Address of remote UE connected to PDN1 |
| px_IPv6_Address2_RemoteUE | charstring |  |  | IPv6 Address of remote UE connected to PDN2 |
| px_IPv6_Address3_RemoteUE | charstring |  |  | IPv6 Address of remote UE connected to PDN3 |
| px_IPv6_Address_HomeAgent | charstring |  |  | IPv6 Home Agent Address |
| px_ePrimaryFrequencyBand | integer | 1 |  | E-UTRA primary frequency band |
| px_SMS_ChkMsgReceived | boolean | true |  | Whether the operator can check an MT Short Message received |
| px_RATComb_Tested | RATComb_Tested_Type | EUTRA_UTRA | EUTRA_UTRA, EUTRA_GERAN, EUTRA_Only | This parameter represents the network RAT capability / preference and indicates which, if any is supported, RAT combination is to be tested. |
| px_SinglePLMN_Tested | SinglePLMN_Tested_Type | MultiPLMN | SinglePLMN, MultiPLMN, MultiPLMNinPrimaryBand, MultiPLMNinSecondaryBand | This parameter represents the network capability/preference to support multi PLMNs on the same test Band and indicates the preference of multi PLMNs or single PLMN test environment. This PIXIT shall be set to SinglePLMN when only one frequency is defined for the test band. |
| px_UE_CS_PS_UsageSetting_Tested | CS_PS_MODE | VOICE_CENTRIC | VOICE_CENTRIC, DATA_CENTRIC | Specifies which CS/PS mode is under test |
| px_UE_PS_UsageSetting_Tested | PS_MODE | VOICE_CENTRIC | VOICE_CENTRIC, DATA_CENTRIC | Specifies which PS mode is under test |
| px_UTRAN_ModeUnderTest | UTRAN_FDD_TDD | UTRAN_FDD | UTRAN_FDD, UTRAN_TDD | Specifies which radio access technology is being tested in UTRAN |
| px_TestLoopModeB_Delay | O1_Type | ‘5A’ |  | This parameter represents the IP_PDU_delay to be used for UE test loop mode B in test cases, where long delay may be needed e.g. because of user interaction. |
| px_IP_MTU_Size | integer | 65535 |  | MTU Size. This value is specific to the SS. |
| px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI | integer |  |  | A not supported UTRAN frequency band that is overlapping with a supported band (px_MFBI_UTRAN_FrequencyBand). This PIXIT shall be set also in accordance with the values of the 3 PIXIT items: px_UARFCN_D_High, px_UARFCN_D_Mid,px_UARFCN_L_Mid. Applied to MFBI test case scenario. |
| px_MFBI_UTRAN_FrequencyBand | integer |  |  | A supported UTRAN frequency band overlapping with px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI. Applied to MFBI test case scenario. |
| px_WLAN_PDN1_ToOffload | boolean | true |  | If set to true, the first PDN connection is offloaded to WLAN. If set to false, the second PDN connection is offloaded to WLAN. |
| px_AquireGNSS_MaxTime | integer | 60 |  | The maximum time (in seconds) taken for the UE to aquire GNSS and derive UTC time (and normally position) and then start to transmit or receive as appropriate |
| px_MobileDataOn | boolean | true |  | Specifes whether mobile data, associated with Internet PDN, is enabled on the DUT. |

Table 9.1-2: E-UTRAN PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_eTDDsubframeConfig | TDD_SubframeAssignment_Type | 1 |  | TDD uplink-downlink subframe configuration |
| px_ePrimaryBandChannelBandwidth | Dl_Bandwidth_Type | n25 |  | Channel bandwidth used on px_ePrimaryFrequencyBand |
| px_eSecondaryBandChannelBandwidth | Dl_Bandwidth_Type | n25 |  | Channel bandwidth used on px_eSecondaryFrequencyBand |
| px_EUTRA_CA_BandCombination | CA_BandCombination_Type | CA_1C |  | Band combination for CA test cases |
| px_EUTRA_DC_BandCombination | DC_BandCombination_Type | DC_1A-3A |  | Band combination for DC test cases |
| px_eMaxNumberROHC_ContextSessions | MaxNumberROHC_ContextSessions_Type | Cs16 |  | Maximum number of ROHC context sessions |
| px_MFBI_FrequencyBand | integer | 26 |  | A supported E-UTRA frequency band overlapping with px_OverlappingNotSupportedFrequencyBand_MFBI. Applied to MFBI test case scenario. |
| px_MFBI_BandChannelBandwidth | Dl_Bandwidth_Type | n25 |  | E-UTRAN channel bandwidth used on px_MFBI_FrequencyBand. Applied to MFBI test case scenario. |
| px_OverlappingNotSupportedFrequencyBand_MFBI | integer | 27 |  | A not supported E-UTRA frequency band that is overlapping with a supported band (px_MFBI_FrequencyBand). Applied to MFBI test case scenario. |
| px_OverlappingSupportedFrequencyBand_MFBI | integer | 26 |  | A supported E-UTRA frequency band overlapping with px_OverlappingNotSupportedFrequencyBand_MFBI. Applied to MFBI enhancement test case scenario. |
| px_NoTDD_LCR_onLTE | boolean | false |  | Only applicable to UE supporting (at least) LTE and LCR TDD. If set to true, the UE is configured not to report support of LCR TDD (IE utraTDD128 in UECapabilityInformation) when being camped on an LTE cell with no detectable LCR TDD cell in the environment |

Table 9.1-3: D2D PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_EllipsoidPointWithAltitude_ProSe_Area1 | O8_Type |  |  | Ellipsoid Point With Altitude value corresponding to geographical area 1 provided in EFPROSE_RADIO_COM to be provided in Update UE location information (see 36.509 cl. 6.12) |
| px_EllipsoidPointWithAltitude_ProSe_Area2 | O8_Type |  |  | Ellipsoid Point With Altitude value corresponding to geographical area 2 provided in EFPROSE_RADIO_COM to be provided in Update UE location information (see 36.509 cl. 6.12) |
| px_EllipsoidPointWithAltitude_NotProSe | O8_Type |  |  | Ellipsoid Point With Altitude value not corresponding to geographical area provided in EFPROSE_RADIO_COM to be provided in Update UE location information (see 36.509 cl. 6.12) |
| px_ProSeAnnApplicationIdentity1 | charstring | mcc345.mnc012.ProSeApp.Food.Restaurants.Italian |  | A ProSe Application ID with which data structure the UE is preconfigured and for which the UE can request and perform Direct Discovery Announcing. TS 23.003, section 24.2. |
| px_ProSeAnnApplicationIdentity2 | charstring | mcc345.mnc012.ProSeApp.mcptt |  | An MCPTT relevant ProSe Application ID with which data structure the UE is preconfigured and for which the UE can request and perform Group Member Discovery Announcement. TS 23.003, section 24.2. |
| px_ProSeMonApplicationIdentity1 | charstring | mcc300.mnc165.ProSeApp.*.Sports.Surfing |  | A ProSe Application ID with which data structure the UE is preconfigured and for which the UE can request and perform Direct Discovery Monitoring. TS 23.003, section 24.2 |
| px_ProSeMonApplicationIdentityNot1 | charstring | mcc208.mnc*.ProSeApp.Shops.Food.Wine |  | A ProSe Application ID with which data structure the UE is NOT preconfigured to request and perform Direct Discovery Monitoring. TS 23.003, section 24.2 |
| px_ProSe_TLS_CipherSuite | TLS_CIPHER_Type | TLS_PSK_WITH_3DES_EDE_CBC_SHA |  | TLS cipher suite to be used for ProSe communication |
| px_SL_AdditionalSupportedBandwidth | Ul_Bandwidth_Type | n25 |  | Supported sidelink bandwidth different from px_ePrimaryBandChannelBandwidth |

Table 9.1-4: EUTRA_NB PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_eSecondaryFrequencyBand | integer | 2 |  | E-UTRA secondary frequency band |
| px_NAS_CipheringAlgorithm | B3_Type | 001’B |  | NAS Ciphering Algorithm (eea1, see NOTE) |
| px_NAS_IntegrityProtAlgorithm | B3_Type | 001’B |  | NAS Integrity Algorithm (eia1, see NOTE) |
| px_RRC_CipheringAlgorithm | CipheringAlgorithm_r12 | eea1 |  | Ciphering Algorithm (see NOTE) |
| px_RRC_IntegrityProtAlgorithm | IntegrityProtAlgorithm | eia1 |  | Integrity Algorithm (see NOTE) |
| px_SMSTransport_CP_CIoT | boolean | false |  | UE configured to utilise the SMS service as transport mechanism for user data for Control Plane CIoT Optimisation |
| px_nonSMSTransport_CP_CIoT | boolean | false |  | UE configured to utilise non-SMS services as transport mechanism for user data for Control Plane CIoT Optimisation |
| px_NTN_ScenarioUnderTest | NTN_ScenarioUnderTest | GSO | GSO, NGSO | NTN satellite scenario under test if UE supports NTN. |
| NOTE: Unless specified otherwise in the test case prose, the null algorithm shall not be used for verification. |  |  |  |  |

<End of Modification>