3GPP TSG-RAN5 Meeting #100	R5-235468

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1439 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of System information for NGSO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Lack of ephemeris information for NGSO. - Restructure the SIB31(-NB) for GSO and NGSO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Ephemeris information for NTN, especially NGSO was added. - Message contents of SIB31(-NB) were restructured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test configuration for NGSO will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.3.3, 8.1.4.3.3, 6.3.5 (new) and Annex AA (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-235077 -> R5-235444 -> R5-235468 |  |  |  |  |  |  |  |  |

[Start of change]

4.4.3.3	Common contents of system information blocks

-	SystemInformationBlockType2

[Skip unchanged text]

-	SystemInformationBlockType31

[Skip unchanged text]

-	SystemInformationBlockType31

The IE SystemInformationBlockType31 contains satellite assistance information for the serving cell. SystemInformationBlockType31 is only signalled in an NTN cell.

Table 4.4.3.3-21: SystemInformationBlockType31

| Derivation Path: TS 36.331 [17], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  | GSO |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -16976014 |  |  |
| positionY-r17 | 27636499 |  |  |
| positionZ-r17 | 0 |  |  |
| velocityVX-r17 | 0 |  |  |
| velocityVY-r17 | 0 |  |  |
| velocityVZ-r17 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 8243100 |  |  |
| nta-CommonDrift-r17 | 0 |  |  |
| nta-CommonDriftVariation-r17 | 0 |  |  |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s240 |  |  |
| epochTime-r17 | Not present |  |  |
| k-Offset-r17 | 550 |  |  |
| k-Mac-r17 | Not present |  |  |
| } |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  | SIG AND NGSO |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors | See cl. 6.3.5.2 |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 0 |  |  |
| nta-CommonDrift-r17 | 0 |  |  |
| nta-CommonDriftVariation-r17 | 0 |  |  |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s10 |  |  |
| epochTime-r17 | Not present |  |  |
| k-Offset-r17 | See cl. 6.3.5.2 |  |  |
| k-Mac-r17 | See cl. 6.3.5.2 |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

[Skip unchanged text]

6.3.5	Ephemeris information for NTN

6.3.5.1	GSO NTN

This ephemeris information of GSO is defined in Table 4.4.3.3-21 for eMTC NTN and Table  8.1.4.3.3-10 for NB-IoT NTN with below assumption:

-	Inclination: 0

-	Eccentricity: 0

-	Latitude: 0

-	Longitude: 121.56076999

-	Altitude: 35786 km

In case of multi GSO NTN cell scenario, a maximum 3 cells can be used. The ephemeris information is defined in Table 6.3.5.1-1 unless otherwise specified in test cases.

Table 6.3.5.1-1: ephemeris information of multi GSO cells

| Cell number (Note 1) | positionX | positionY | positionZ | velocityVX/Y/Z | nta-Common-r17 | Koffset |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | -16976014 | 27636499 | 0 | 0 | 8243100 | 550 |
| 2 | -23550424 | 22301093 | 0 | 0 | 8248322 | 550 |
| 3 | -14502736 | 29010891 | 0 | 0 | 8248322 | 550 |
| Note 1: Cell number 1 is the default serving cell to be used. Cell 2 and 3 are used in multi-NTN scenario with cell 2 as the 1st neighbour cell and cell 3 as the 2nd neighbour cell. |  |  |  |  |  |  |

For example, in TC 22.5.23 runing in GSO scenario, Ncell 50 is the serving cell in the preamble and it uses the ephemeris information of Cell number 1 while Ncell 62 uses the ephemeris information of Cell number 2.

6.3.5.2	NGSO NTN

This ephemeris information of NGSO is defined in Table 4.4.3.3-21 for eMTC NTN and Table 8.1.4.3.3-10 for NB-IoT NTN with below assumption:

-	Inclination: 90

-	Eccentricity: 0

-	Longitude: 121.56076999

-	Altitude: 600 km

-	Speed: 7.56 km/s

UE position defined in Clause 4.13 is the projection position from one of the satellite information to the ground.

As the NGSO satellite is moving and to avoid varying nta-Common-r17, nta-CommonDrift-r17 and nta-CommonDriftVariation-r17, the Reference Point (RP) is defined as the satellite itself.

Considering the Min Elevation angle for both sat-gateway and IoT device is 10°, the initial latitude of the satellite is: 25.0843933 (UE latitude) - 15.8394129 (the max intersection angle between UE and the satelline in ECEF coordinates)  = 9.24498034°.

The table of the ephemeris information of NGSO NTN lasts 510 seconds as the NGSO can serve the UE for about 8.5 minutes with the periodicity set to the si-WindowLength-r13 = 160 ms. The ephemeris information in systemInformationBlockType31 or systemInformationBlockType31-NB is defined and where appropriate is given in Excel format in .xlsx files for each time instance in the appropriate data file specified in Annex B.1.

In case of multi NGSO NTN cell scenario, a maximum 3 cells can be used. The ephemeris information of each following cell has the same value as the default, but the epoch time is shifted with respect to the previous cell by 20 sec.

Table 6.3.5.2-1: ephemeris information of multi NGSO cells

| Cell number (Note 1) | Time offset (second) | positionY | positionZ | velocityVX/Y/Z | nta-Common-r17 | Koffset |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | Refer to the table of the ephemeris information of NGSO NTN |  |  |  |  |
| 2 | +20 | Refer to the table of the ephemeris information of NGSO NTN |  |  |  |  |
| 3 | +40 | Refer to the table of the ephemeris information of NGSO NTN |  |  |  |  |
| Note 1: Cell number 1 is the default serving cell to be used. Cell 2 and 3 are used in multi-NTN scenario with cell 2 as the 1st neighbour cell and cell 3 as the 2nd neighbour cell. |  |  |  |  |  |  |

For example, in TC 22.5.23 runing in NGSO scenario, Ncell 50 is the serving cell in the preamble and it uses the ephemeris information of Cell number 1 while Ncell 62 uses the ephemeris information of Cell number 2.

[Skip unchanged text]

8.1.4.3.3	NB-IoT Common contents of system information blocks

-	SystemInformationBlockType2-NB

[Skip unchanged text]

-	SystemInformationBlockType31-NB

The IE SystemInformationBlockType31-NB contains satellite assistance information for the serving cell. SystemInformationBlockType31-NB is only signalled in an NTN cell.

Table 8.1.4.3.3-10: SystemInformationBlockType31-NB

| Derivation Path: TS 36.331 [17], clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  | GSO |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -16976014 |  |  |
| positionY-r17 | 27636499 |  |  |
| positionZ-r17 | 0 |  |  |
| velocityVX-r17 | 0 |  |  |
| velocityVY-r17 | 0 |  |  |
| velocityVZ-r17 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 8243100 |  |  |
| nta-CommonDrift-r17 | 0 |  |  |
| nta-CommonDriftVariation-r17 | 0 |  |  |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s240 |  |  |
| epochTime-r17 | Not present |  |  |
| k-Offset-r17 | 550 |  |  |
| k-Mac-r17 | Not present |  |  |
| } |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  | SIG AND NGSO |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors | See cl. 6.3.5.2 |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 0 |  |  |
| nta-CommonDrift-r17 | 0 |  |  |
| nta-CommonDriftVariation-r17 | 0 |  |  |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s10 |  |  |
| epochTime-r17 SEQUENCE { | Not present |  |  |
| k-Offset-r17 | See cl. 6.3.5.2 |  |  |
| k-Mac-r17 | See cl. 6.3.5.2 |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

[Skip unchanged text]

Annex B (normative): NTN data files

B.1	NGSO data files for IoT NTN signalling tests

The NGSO data files for use in IoT NTN signalling tests defined in TS 36.523-1 [18] are contained in archive NGSO_Data_for_IoT_NTN_Sig_V1.zip which accompanies this document.

[End of change]