3GPP TSG-RAN5 Meeting #100	R5-235440

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3910 | rev | 2 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of UAS Test Case for Initial Registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of UAS Test Case for Initial Registrations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced TC 9.1.5.1.17 to cover UAS Initial Registrations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the work plan will remain unfulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.17 (New) |  |  |  |  |  |  |  |  |
| sss |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-234323r1 -> Removed “Present not checked” value\remark from Table 9.1.5.1.17.3.3-2: REGISTRATION ACCEPT as it is a DL Message. Fixed the Derivation path in Table 9.1.5.1.17.3.3-2 Removed Step 16 as it part of postamble R5-235385 is final version of R5-234323r1 with editorial corrections R5-235440 is replacement Tdoc for R5-235385 |  |  |  |  |  |  |  |  |

<Start of modified section>

###### 9.1.5.1.17	Initial registration / Success / UAS

9.1.5.1.17.1	Test Purpose (TP)

(1)

with { UE is powered on and has sent a REGISTRATION REQUEST message }

ensure that {

when { UE receives Service-level-AA container IE with service-level-AA pending indication in REGISTRATION ACCEPT }

then { UE shall send a REGISTRATION COMPLETE message to acknowledge reception of the service-level-AA pending indication, and the UE shall not attempt to perform another registration procedure or or to establish a PDU session for USS communication }

9.1.5.1.17.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.501, clauses 4.22.1, 5.5.1.2.2, 5 5.1.2.4. Unless otherwise stated these are Rel-17 requirements.

[TS 24.501, clause 5.5.1.2.2]

5.5.1.2.2	Initial registration initiation

…

If the UE supports UAS services, the UE shall set the UAS bit to "UAS services supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

When the UE supporting UAS services initiates an initial registration for UAS services, the UE shall include the service-level device ID in the Service-level-AA container IE of the REGISTRATION REQUEST message and set the value to the CAA-level UAV ID. The UE shall include the service-level-AA server address in the Service-level-AA container IE of the REGISTRATION REQUEST message and set the value to the USS address, if it is provided by the upper layers.

…

[TS 24.501, clause 5.5.1.2.2]

5.5.1.2.4	Initial registration accepted by the network

If the UE has included the service-level device ID set to the CAA-level UAV ID in the Service-level-AA container IE of the REGISTRATION REQUEST message, and if:

-	the UE has a valid aerial UE subscription information;

-	the UUAA procedure is to be performed during the registration procedure according to operator policy;

-	there is no valid successful UUAA result for the UE in the UE 5GMM context; and

-	the REGISTRATION REQUEST message was not received over non-3GPP access,

then the AMF shall initiate the UUAA-MM procedure with the UAS-NF as specified in TS 23.256 [6AB] and shall include a service-level-AA pending indication in the Service-level-AA container IE of the REGISTRATION ACCEPT message. The AMF shall store in the UE 5GMM context that a UUAA procedure is pending. The AMF shall start timer T3550 and enter state 5GMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.1.3.2.3.3. If the REGISTRATION REQUEST message was received over non-3GPP access, the AMF shall not initiate UUAA-MM procedure.

…

If the UE has included the service-level device ID set to the CAA-level UAV ID in the Service-level-AA container IE of the REGISTRATION REQUEST message and the REGISTRATION ACCEPT message contains the service-levelAA pending indication in the Service-level-AA container IE, the UE shall return a REGISTRATION COMPLETE message to the AMF to acknowledge reception of the service-level-AA pending indication, and the UE shall not attempt to perform another registration procedure for UAS services until the UUAA-MM procedure is completed, or to establish a PDU session for USS communication or a PDU session for C2 communication until the UUAA-MM procedure is completed successfully. If the UE has included the service-level device ID set to the CAA-level UAV ID in the Service-level-AA container IE of the REGISTRATION REQUEST message and the REGISTRATION ACCEPT message does not contain the service-level-AA pending indication in the Service-level-AA container IE, the UE shall consider the UUAA-MM procedure is not triggered.

[TS 24.501, clause 5.4.4.2]

9.1.5.17.3	Test description

9.1.5.1.17.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A

UE:

-	The UE is capable of UAS operation and provisioned with CAA-level UAV ID.

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [4].

9.1.5.1.17.3.2	Test procedure sequence

Table 9.1.5.1.17.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does UE transmit a REGISTRATION REQUEST message? | --> | REGISTRATION REQUEST | - | - |
| 3-11 | Steps 5 to 13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 12 | The SS transmits a REGISTRATION ACCEPT message including | <-- | REGISTRATION ACCEPT | - | - |
| 13 | Check: Does the UE transmit a REGISTRATION COMPLETE message? | --> | REGISTRATION COMPLETE | 1 | P |
| 14 | Check: Does the UE transmit a PDU SESSION ESTABLISHMENT REQUEST message within 60 seconds? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | 1 | F |
| 15 | The SS releases the RRC connection. | - | - | - | - |

9.1.5.1.17.3.3	Specific message contents

Table 9.1.5.1.17.3.3-1: REGISTRATION REQUEST (Step 2, Table 9.1.5.1.17.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Service-level-AA container |  | Present, Contents not checked |  |  |
| Service-level-AA payload type |  | Present, Contents not checked |  |  |

Table 9.1.5.1.17.3.3-2: REGISTRATION ACCEPT (Step12, Table 9.1.5.1.17.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Service-level-AA container |  |  |  |  |
| Service-level-AA pending indication |  | ‘1010 0001’B |  |  |

<End of modified section>