3GPP TSG-RAN5 Meeting #100	R5-235861

# Toulouse, France, 21 – 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2358 | rev | 1 | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of PC1 transmitter requirements for railway bands n100 and n101 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_HPUE_FWVM_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Missing PC1 requirements for railway bands n100 and n101 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of PC1 requirements for bands other than Band n14. Added PC1 requirements for railway bands n100 and n101 into: - Table 6.2.1.3-1: UE Power Class - Table 6.2.1.5-2a: Maximum Output Power test requirement for PC1 - Clause 6.2.2.3, UE MPR Minimum conformance requirements - New Table 6.2.2.3-4b: MPR for PC 1 for bands other than Band n14 - Clause 6.2.2.4.1 UE MPR Initial conditions - New Table 6.2.2.4.1-2c: Test Configuration Table for PC1 for bands other than Band n14 (contiguous allocation) - 6.2.2.4.2 UE MPR Test procedure - New Table 6.2.2.5-4c: UE Power Class test requirements for bands other than Band n14 for PC1 (contiguous allocation) - Clause 6.5.2.4.2.3 UTRA ACLR Minimum conformance requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC1 requirements for railway bands n100 and n101 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.2, 6.5.2.4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 CR R4-2311476 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Revise MPR test requirements in Table 6.2.2.5-4c r2: Clarify test channel BWs, add NOTE 4 into Table 6.2.2.4.1-2c r3: Align minimum conformance text with 38.101-1 |  |  |  |  |  |  |  |  |

-------- Start of changes --------

## 6.2	Transmitter power

### 6.2.1	UE maximum output power

6.2.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.1.2	Test applicability

This test case applies to all types of NR Power Class 1 UE release 15 and forward.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and don't support Redcap.

6.2.1.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1.3-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n2 |  |  |  |  | 23 | ± 23 |
| n3 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n5 |  |  |  |  | 23 | ± 2 |
| n7 |  |  |  |  | 23 | ± 23 |
| n8 |  |  |  |  | 23 | ± 23 |
| n12 |  |  |  |  | 23 | ± 23 |
| n13 |  |  |  |  | 23 | ±2 |
| n14 | 316 | +2/-3 |  |  | 23 | ± 2 |
| n20 |  |  |  |  | 23 | ± 23 |
| n24 |  |  |  |  | 23 | +2/-33 |
| n25 |  |  |  |  | 23 | ± 23 |
| n26 |  |  |  |  | 23 | ± 23 |
| n28 |  |  |  |  | 23 | +2/-2.5 |
| n30 |  |  |  |  | 23 | ± 2 |
| n34 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n38 |  |  |  |  | 23 | ± 2 |
| n39 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n40 |  |  |  |  | 23 | ± 2 |
| n41 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n48 |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  | 23 | ± 2 |
| n51 |  |  |  |  | 23 | ± 2 |
| n53 |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  | 23 | ±2 |
| n66 |  |  |  |  | 23 | ± 2 |
| n70 |  |  |  |  | 23 | ± 2 |
| n71 |  |  |  |  | 23 | +2/-2.5 |
| n74 |  |  |  |  | 23 | ± 2 |
| n77 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n78 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  | 23 | ± 2 |
| n81 |  |  |  |  | 23 | ± 2 |
| n82 |  |  |  |  | 23 | ± 2 |
| n83 |  |  |  |  | 23 | + 2/-2.5 |
| n84 |  |  |  |  | 23 | ± 2 |
| n86 |  |  |  |  | 23 | ± 2 |
| n91 |  |  |  |  | 23 | ±23, 4 |
| n92 |  |  |  |  | 23 | ±23, 4 |
| n93 |  |  |  |  | 23 | ±23, 4 |
| n94 |  |  |  |  | 23 | ±23, 4 |
| n95 |  |  |  |  | 23 | ±2 |
| n97 |  |  |  |  | 23 | ±2 |
| n99 |  |  |  |  | 23 | +2/-33 |
| n100 | 316 | +2/-3 |  |  | 23 | ±2 |
| n101 | 316 | +2/-3 |  |  | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: FFS NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

- if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

- if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.331 [6] (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [6] is provided and set to the maximum output power of the default power class or lower;

-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in sub-clause 6.2.4;

- else if the UE does not support a power class with higher maximum output power than PC2; or

-	if the IE P-Max as defined in TS 38.331 [6] is provided and set to the maximum output power of the power class 2 or lower;

-	shall apply all requirements for power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2.4;

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.1.

6.2.1.4	Test description

6.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest (NOTE 3) |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | power test case | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For band n28, the Highest test channel bandwidth is replaced by 20MHz due to MPR is always larger than 0dB for 30MHz bandwidth. NOTE 4: For P-Max test (Step 4 in Section 6.2.1.4.2) and maxUplinkDutyCycle test (Step 5 in Section 6.2.1.4.2), only “Normal” test environment, “Low range” test frequencies, “Lowest” Test Channel Bandwidth, “Lowest” test SCS and “Inner Full” RB allocation to be used. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.1.4.3.

6.2.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

4.	For UEs supporting Power Class 2 or Power Class 1, repeat steps 1~3 on the applicable bands with message exception of P-Max defined in Table 6.2.1.4.3-2.

5.	For TDD UEs supporting Power Class 2 with UE capability maxUplinkDutyCycle-PC2-FR1, repeat steps 1~3 on the applicable bands with message exception of TDD UL-DL pattern defined in Table 6.2.1.4.3-4 and Table 6.2.1.4.3-5 which the UplinkDutyCycle is closest less than or equal to the maxUplinkDutyCycle-PC2-FR1 reported by UE. Before Step 3, wait for at least 10ms for the UE to complete the evaluation period.

6.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

Table 6.2.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2.1.4.3-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 | PC2 UE or PC1 UE will fallback to PC3 UE with P-Max=23 | PC2 UE or PC1 UE |

Table 6.2.1.4.3-3: Void

Table 6.2.1.4.3-4: TDD UL-DL pattern for SCS 15 KHz (UplinkDutyCycle=60%)

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
| TDD Slot Configuration pattern (Note 1) |  |  | DSUUU |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 15 |
|  | dl-UL-TransmissionPeriodicity | ms | 5 |
|  | nrofDownlinkSlots |  | 1 |
|  | nrofDownlinkSymbols |  | 10 |
|  | nrofUplinkSlot |  | 3 |
|  | nrofUplinkSymbols |  | 2 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | [4] if mod(i,5) = 0 [6] if mod(i,5) = 1 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,9} Note 4: There shall be no PUSCH or PUCCH or SRS transmitted in slot 1 and slot 6 to meet the specific UplinkDutyCycle. |  |  |  |

Table 6.2.1.4.3-5: TDD UL-DL pattern for SCS 30 KHz (UplinkDutyCycle=60%)

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
| TDD Slot Configuration pattern (Note 1) |  |  | DSUUU |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 30 |
|  | dl-UL-TransmissionPeriodicity | ms | 2.5 single |
|  | nrofDownlinkSlots |  | 1 |
|  | nrofDownlinkSymbols |  | 10 |
|  | nrofUplinkSlot |  | 3 |
|  | nrofUplinkSymbols |  | 2 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | [4] if mod(i,5) = 0 [6] if mod(i,5) = 1 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,19} Note 4: There shall be no PUSCH or PUCCH or SRS transmitted in slot 1, slot 6, slot 11 and slot 16 to meet the specific UplinkDutyCycle. |  |  |  |

6.2.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1 for Power Class 3, Table 6.2.1.5-2 for Power Class 2, Table 6.2.1.5-2a for Power Class 1.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1.

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-2 for Power Class 2 as defined in TS 38.306.

Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ±2±TT |
| n2 |  |  |  |  | 23 | ±23±TT |
| n3 |  |  |  |  | 23 | ±23±TT |
| n5 |  |  |  |  | 23 | ±2±TT |
| n7 |  |  |  |  | 23 | ±23±TT |
| n8 |  |  |  |  | 23 | ±23±TT |
| n12 |  |  |  |  | 23 | ±23±TT |
| n13 |  |  |  |  | 23 | ±2±TT |
| n14 |  |  |  |  | 23 | ±2±TT |
| n20 |  |  |  |  | 23 | ±23±TT |
| n24 |  |  |  |  | 23 | +2+TT/-3.03-TT |
| n25 |  |  |  |  | 23 | ±23±TT |
| n26 |  |  |  |  | 23 | ±23±TT |
| n28 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n30 |  |  |  |  | 23 | ±2±TT |
| n34 |  |  |  |  | 23 | ±2±TT |
| n38 |  |  |  |  | 23 | ±2±TT |
| n39 |  |  |  |  | 23 | ±2±TT |
| n40 |  |  |  |  | 23 | ±2±TT |
| n41 |  |  |  |  | 23 | ± 23±TT |
| n48 |  |  |  |  | 23 | +2+TT/-3-TT |
| n50 |  |  |  |  | 23 | ±2±TT |
| n51 |  |  |  |  | 23 | ±2±TT |
| n53 |  |  |  |  | 23 | ±2±TT |
| n65 |  |  |  |  | 23 | ±2±TT |
| n66 |  |  |  |  | 23 | ±2±TT |
| n70 |  |  |  |  | 23 | ±2±TT |
| n71 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n74 |  |  |  |  | 23 | ±2±TT |
| n77 |  |  |  |  | 23 | +2+TT/-3-TT |
| n78 |  |  |  |  | 23 | +2+TT/-3-TT |
| n79 |  |  |  |  | 23 | +2+TT/-3-TT |
| n91 |  |  |  |  | 23 | ± 23, 5±TT |
| n92 |  |  |  |  | 23 | ± 23, 5±TT |
| n93 |  |  |  |  | 23 | ± 23, 5±TT |
| n94 |  |  |  |  | 23 | ± 23, 5±TT |
| n100 |  |  |  |  | 23 | ±2±TT |
| n101 |  |  |  |  | 23 | ±2±TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 NOTE 5: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. |  |  |  |  |  |  |

Table 6.2.1.5-2: Maximum Output Power test requirement for Power Class 2

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2+TT/-3-TT |  |  |
| n3 |  |  | 26 | +2+TT/-33-TT |  |  |
| n34 |  |  | 26 | +2+TT/-3-TT |  |  |
| n39 |  |  | 26 | +2+TT/-3-TT |  |  |
| n41 |  |  | 26 | +2+TT/-33-TT |  |  |
| n77 |  |  | 26 | +2+TT/-3-TT |  |  |
| n78 |  |  | 26 | +2+TT/-3-TT |  |  |
| n79 |  |  | 26 | +2+TT/-3-TT |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |

Table 6.2.1.5-2a: Maximum Output Power test requirement for Power Class 1

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n145 | 31 | +2+TT/-3-TT |  |  |  |  |
| n1005 | 31 | +2+TT/-3-TT |  |  |  |  |
| n1015 | 31 | +2+TT/-3-TT |  |  |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3. NOTE 5: Generally, PC1 UE is not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |

Table 6.2.1.5-2b: Void

Table 6.2.1.5-3: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

### 6.2.2	UE maximum output power reduction

6.2.2.1	Test purpose

The number of RB identified in Table 6.2.2.3-1 is based on meeting the requirements for adjacent channel leakage ratio and the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2.2.2	Test applicability

This test case applies to all types of NR Power Class 1 UE release 15 and forward.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5.2.4.1 is executed.

6.2.2.3	Minimum conformance requirements

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 2 and 3 and UE power class 1, the allowed maximum power reduction (MPR) is defined in Table 6.2.2.3-2, Table 6.2.2.3-1, Table 6.2.2.3-4b and Table 6.2.2.3-5, respectively for channel bandwidths ≤ 100 MHz.

If the relative channel bandwidth ≤ 4% for TDD bands or ≤ 3% for FDD band, the ∆MPR is set to zero.

If the relative channel bandwidth is larger than 4% for TDD bands or 3% for FDD bands, the ∆MPR is defined in Table 6.2.2.3-3.

Where relative channel bandwidth = 2*BWChannel / (FUL_low + FUL_high).

The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation.

Table 6.2.2.3-1: Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Edge RB allocations |  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 |  | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 |  |  | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52 | 02 |  |  |
|  | QPSK | ≤ 1 |  |  | 0 |
|  | 16 QAM | ≤ 2 |  |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |  |
| CP-OFDM | QPSK | ≤ 3 |  |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0dB MPR is 26dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40% of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |  |

Table 6.2.2.3-2: Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Table 6.2.2.3-3: ∆MPR

| NR Band | Power class | Channel bandwidth | ∆MPR (dB) |
| --- | --- | --- | --- |
| n28 and n83 | Power class 3 | 30 MHz | 0.5 |
| n40 and n97 | Power class 3 and power class 2 | 100 MHz | 1 |

Table 6.2.2.3-4: Void

Table 6.2.2.3-4a: Void

Table 6.2.2.3-4b: Maximum power reduction (MPR) for power class 1 for bands other than Band n14

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | (NOTE 1) | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | (NOTE 1) | 0 | 0 |
|  | QPSK | (NOTE 1) | ≤ 1 | 0 |
|  | 16 QAM | (NOTE 1) | ≤ 2 | ≤ 1 |
|  | 64 QAM | (NOTE 1) | ≤ 2.5 |  |
|  | 256 QAM | (NOTE 1) | ≤ 4.5 |  |
| CP-OFDM | QPSK | (NOTE 1) | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | (NOTE 1) | ≤ 3 | ≤ 2 |
|  | 64 QAM | (NOTE 1) | ≤ 3.5 |  |
|  | 256 QAM | (NOTE 1) | ≤ 6.5 |  |
| NOTE 1: MPR for all modulations for Edge RB allocation is defined as following for two distinguished channel bandwidths groups as: Within the <50MHz channel bandwidth group: MPR= CEIL(7.2 dB- 6 dB∙ CBW100 MHz, 0.5 dB) Within the ≥50MHz channel bandwidth group: MPR= CEIL5.35 dB+3.15 dB∙ CBW100 MHz, 0.5 dB where CEIL(x,0.5 dB) means rounding x upwards to the closest multiple of 0.5 dB. |  |  |  |  |

Table 6.2.2.3-5: Maximum power reduction (MPR) for power class 1 for Band n14

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.5 | 0 | 0 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart ≤ RBStart,High, and

LCRB ≤ ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel with LCRB ≤ 2 RBs.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation or Edge RB allocation.

If CP-OFDM allocation satisfies following conditions, it is considered as almost contiguous allocation

NRB_gap / (NRB_alloc + NRB_gap) ≤ 0.25

and NRB_alloc + NRB_gap is larger than 106, 51 or 24 RBs for 15 kHz, 30 kHz or 60 kHz SCS respectively where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs. The size and location of allocated and unallocated RBs are restricted by RBG parameters specified in sub-clause 6.1.2.2 of TS 38.214 [12]. For these almost contiguous signals in power class 2 and 3, the allowed maximum power reduction defined in Table 6.2.2.3-2 and Table 6.2.2.3-1 are increased by

CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB,

where CEIL{x,0.5} means x rounding upwards to closest 0.5dB. The parameters of RBStart,Low and RBStart,High to specify valid RB allocation ranges for Outer and Inner RB allocations are defined as following:

RBStart,Low = max(1, floor((NRB_alloc + NRB_gap)/2))

RBStart,High = NRB – RBStart,Low – NRB_alloc –NRB_gap

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.4 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.2.

6.2.2.4	Test description

6.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-2a, Table 6.2.2.4.1-2c and Table 6.2.2.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.2.4.1-1: Test Configuration Table for power class 3 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 13 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 23 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 33 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 43 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 54 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 64 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 74 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 84 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 10 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 13 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 14 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 17 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 18 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 19 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 20 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 21 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 22 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 23 | Default |  | CP-OFDM QPSK | Inner Full |
| 24 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 25 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM QPSK | Outer Full |
| 27 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 28 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 29 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 30 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 31 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 32 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 33 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 374,5 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 384,5 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 394,5 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 5: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |

Table 6.2.2.4.1-1a: Void

Table 6.2.2.4.1-2: Test Configuration Table for power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

Table 6.2.2.4.1-2a: Test Configuration Table for power class 1 for Band n14 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 333 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 343 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 353 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for Ues which supports Pi/2 BPSK in FR1. NOTE 3: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |

Table 6.2.2.4.1-2b: Void

Table 6.2.2.4.1-2c: Test Configuration Table for power class 1 for bands other than Band n14 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest for all RB allocations Additionally, for edge RB allocations, the maximum channel bandwidth <50MHz and the minimum channel bandwidth ≥50MHz that are supported by the UE (NOTE 4) |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 333 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 343 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 353 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for Ues which supports Pi/2 BPSK in FR1. NOTE 3: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. NOTE 4: The maximum channel bandwidth <50MHz and the minimum channel bandwidth ≥50MHz are tested only with the highest order modulation supported by the UE. |  |  |  |  |

Table 6.2.2.4.1-3: Test Configuration Table for power class 2&3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Default |  | CP-OFDM QPSK | Outer Full |
| 3 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 4 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 5 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 6 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Tables 6.2.2.4.1-4, 6.2.2.4.1-4a and 6.2.2.4.1-4b. NOTE 2: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |

Table 6.2.2.4.1-4: Uplink configuration for almost contiguous allocation for low frequency range

| Channel Bandwidth(MHz) | SCS(kHz) | OFDM | Outer Full |  | Inner Full |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cluster1 RB allocations | Cluster2 RB allocations | Cluster1 RB allocations | Cluster2 RB allocations |
|  |  |  | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) |
| 25 | 15 | CP | 48@0 | 53@80 | N/A | N/A |
|  | 30 | CP | 24@0 | 25@40 | N/A | N/A |
|  | 60 | CP | 12@0 | 13@18 | N/A | N/A |
| 30 | 15 | CP | 64@0 | 64@96 | N/A | N/A |
|  | 30 | CP | 32@0 | 30@48 | N/A | N/A |
|  | 60 | CP | 16@0 | 14@24 | N/A | N/A |
| 35 | 15 | CP | 80@0 | 76@112 | N/A | N/A |
|  | 30 | CP | 40@0 | 36@56 | N/A | N/A |
|  | 60 | CP | 20@0 | 16@28 | N/A | N/A |
| 40 | 15 | CP | 80@0 | 88@128 | N/A | N/A |
|  | 30 | CP | 40@0 | 42@64 | N/A | N/A |
|  | 60 | CP | 20@0 | 19@32 | 12@12 | 8@28 |
| 45 | 15 | CP | 80@0 | 114@128 | 48@64 | 48@128 |
|  | 30 | CP | 40@0 | 55@64 | 24@32 | 24@64 |
|  | 60 | CP | 20@0 | 26@32 | 12@16 | 12@32 |
| 50 | 15 | CP | 96@0 | 110@160 | 48@64 | 48@144 |
|  | 30 | CP | 48@0 | 53@80 | 24@32 | 24@72 |
|  | 60 | CP | 24@0 | 25@40 | 12@16 | 12@36 |
| 60 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 64@0 | 66@96 | 32@32 | 16@80 |
|  | 60 | CP | 32@0 | 31@48 | 16@16 | 8@40 |
| 70 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 80@0 | 77@112 | 32@32 | 16@80 |
|  | 60 | CP | 40@0 | 37@56 | 16@16 | 8@40 |
| 80 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 80@0 | 89@128 | 32@32 | 16@80 |
|  | 60 | CP | 40@0 | 43@64 | 16@16 | 8@40 |
| 90 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 96@0 | 101@144 | 32@32 | 16@80 |
|  | 60 | CP | 48@0 | 49@72 | 16@16 | 8@40 |
| 100 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 112@0 | 97@176 | 48@64 | 48@144 |
|  | 60 | CP | 48@0 | 55@80 | 24@32 | 24@72 |

Table 6.2.2.4.1-4a: Uplink configuration for almost contiguous allocation for TDD high frequency range

| Channel Bandwidth(MHz) | SCS(kHz) | OFDM | Outer Full |  | Inner Full |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cluster1 RB allocations | Cluster2 RB allocations | Cluster1 RB allocations | Cluster2 RB allocations |
|  |  |  | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) |
| 25 | 15 | CP | 48@0 | 53@80 | N/A | N/A |
|  | 30 | CP | 24@0 | 25@40 | N/A | N/A |
|  | 60 | CP | 12@0 | 13@18 | N/A | N/A |
| 30 | 15 | CP | 56@0 | 64@88 | N/A | N/A |
|  | 30 | CP | 32@0 | 30@48 | N/A | N/A |
|  | 60 | CP | 16@0 | 14@24 | N/A | N/A |
| 35 | 15 | CP | 72@0 | 84@104 | N/A | N/A |
|  | 30 | CP | 40@0 | 36@56 | N/A | N/A |
|  | 60 | CP | 20@0 | 16@28 | N/A | N/A |
| 40 | 15 | CP | 72@0 | 96@120 | N/A | N/A |
|  | 30 | CP | 40@0 | 42@64 | N/A | N/A |
|  | 60 | CP | 20@0 | 19@32 | 12@12 | 8@28 |
| 45 | 15 | CP | 72@0 | 122@120 | 48@56 | 48@120 |
|  | 30 | CP | 40@0 | 55@64 | 24@32 | 24@64 |
|  | 60 | CP | 20@0 | 26@32 | 12@16 | 12@32 |
| 50 | 15 | CP | 88@0 | 112@152 | 48@72 | 48@152 |
|  | 30 | CP | 48@0 | 53@80 | 24@32 | 24@72 |
|  | 60 | CP | 24@0 | 25@40 | 12@16 | 12@36 |
| 60 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 56@0 | 74@88 | 32@40 | 16@88 |
|  | 60 | CP | 32@0 | 31@48 | 16@16 | 8@40 |
| 70 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 72@0 | 80@104 | 32@40 | 16@88 |
|  | 60 | CP | 40@0 | 37@56 | 16@16 | 8@40 |
| 80 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 72@0 | 96@120 | 32@40 | 16@88 |
|  | 60 | CP | 40@0 | 43@64 | 16@16 | 8@40 |
| 90 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 88@0 | 109@136 | 32@40 | 16@88 |
|  | 60 | CP | 48@0 | 49@72 | 16@16 | 8@40 |
| 100 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 104@0 | 105@168 | 48@72 | 48@152 |
|  | 60 | CP | 48@0 | 55@80 | 24@32 | 24@72 |

Table 6.2.2.4.1-4b: Uplink configuration for almost contiguous allocation for FDD high frequency range

| Channel Bandwidth(MHz) | SCS(kHz) | OFDM | Outer Full |  | Inner Full |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cluster1 RB allocations | Cluster2 RB allocations | Cluster1 RB allocations | Cluster2 RB allocations |
|  |  |  | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) |
| 25 | 15 | CP | 42@0 | 56@74 | N/A | N/A |
|  | 30 | CP | 22@0 | 27@38 | N/A | N/A |
|  | 60 | CP | 12@0 | 13@18 | N/A | N/A |
| 30 | 15 | CP | 58@0 | 64@90 | N/A | N/A |
|  | 30 | CP | 26@0 | 32@42 | N/A | N/A |
|  | 60 | CP | 14@0 | 16@22 | N/A | N/A |
| 35 | 15 | CP | 74@0 | 82@106 | N/A | N/A |
|  | 30 | CP | 42@0 | 34@58 | N/A | N/A |
|  | 60 | CP | 22@0 | 18@26 | N/A | N/A |
| 40 | 15 | CP | 74@0 | 94@122 | N/A | N/A |
|  | 30 | CP | 34@0 | 48@58 | N/A | N/A |
|  | 60 | CP | 18@0 | 20@30 | 12@14 | 8@30 |
| 45 | 15 | CP | 74@0 | 120@122 | 48@58 | 48@122 |
|  | 30 | CP | 34@0 | 56@58 | 24@34 | 24@66 |
|  | 60 | CP | 18@0 | 28@30 | 12@14 | 12@30 |
| 50 | 15 | CP | 90@0 | 112@154 | 48@74 | 48@154 |
|  | 30 | CP | 42@0 | 56@74 | 24@34 | 24@74 |
|  | 60 | CP | 22@0 | 27@38 | N/A | N/A |
| 60 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 58@0 | 72@90 | 32@42 | 16@90 |
|  | 60 | CP | 26@0 | 32@42 | 16@18 | 8@42 |
| 70 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 74@0 | 80@106 | 32@42 | 16@90 |
|  | 60 | CP | 34@0 | 40@50 | 16@18 | 8@42 |
| 80 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 74@0 | 95@122 | 32@42 | 16@90 |
|  | 60 | CP | 34@0 | 48@58 | 16@18 | 8@42 |
| 90 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 90@0 | 107@138 | 32@42 | 16@90 |
|  | 60 | CP | 42@0 | 55@66 | 16@18 | 8@42 |
| 100 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 106@0 | 103@170 | 48@74 | 48@154 |
|  | 60 | CP | 42@0 | 56@74 | 24@34 | 24@74 |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.2.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-2a, Table 6.2.2.4.1-2c and Table 6.2.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

6.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-2a, Table 6.2.2.4.1-2c and Table 6.2.2.4.1-3. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	For UEs supporting Power Class 1 and Power Class 2, repeat steps 1~3 for Test ID 22 and 36 in Table 6.2.2.4.1-1 on the applicable bands with message exception of P-Max defined in Table 6.2.2.4.3-2.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions:

Table 6.2.2.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.2.2.4.3-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | PC2 UE or PC1 UE |

Table 6.2.2.4.3-2a: Void

Table 6.2.2.4.3-3: DMRS-UplinkConfig (Test ID 37 - 39 in Table 6.2.2.4.1-1)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-51 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecodingEnabled SEQUENCE { |  |  |  |
| dmrs-UplinkTransformPrecoding-r16 SEQUENCE { |  |  |  |
| pi2BPSK-ScramblingID0 | Not present |  |  |
| pi2BPSK-ScramblingID1 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.4.3-4: ServingCellConfig

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 1 |  | Test IDs where NOTE 3 in Table 6.2.2.4.1-1 applies. |
|  | 0 |  | Test IDs where NOTE 4 in Table 6.2.2.4.1-1 applies. |
| } |  |  |  |
| } |  |  |  |

6.2.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1 to Table 6.2.2.5-9a.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1, Table 6.2.2.5-4, Table 6.2.2.5-4a and Table 6.2.2.5-4c.

Table 6.2.2.5-1: UE Power Class test requirements(for Bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n65, n66, n70, n74, n91, n92, n93, n94, n100, n101) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | （1.52） | 25.8 | （24.32） | 2.0 |  | 2 | 28.0 + TT | 23.8 - TT | （22.3 - TT2） |
| 2 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 3 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 4 | 23 | -3 | 1.2 | 0 | （1.52） | 24.8 | （23.32） | 2.0 |  | 2 | 28.0 + TT | 22.8 - TT | （21.3 - TT2） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 2 | 25.0 + TT | 19.5 - TT | （17.5 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | (19.0 - TT2) |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n25 n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: For bands n91, n92, n93 and n94 the maximum power reduction requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-1a: UE Power Class test requirements Band 24 for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 – TT | （15.5 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 - TT | （15.5 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 - TT | （15.5 - TT2） |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 25.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 – TT | （14.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 – TT | （14.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| 6 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 7 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 8 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 9 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| 10 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 11 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 12 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 13 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 14 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 15 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 16 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 17 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 18 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 19 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 20 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 21 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 22 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 23 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2.5 | 25.0 + TT | 19.0 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 27 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 28 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 29 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 30 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 31 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 32 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 33 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 34 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 35 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 36 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 37 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 38 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 39 | 23 | 0 | 0 | 0 | 23 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2a: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 6 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 7 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 8 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 9 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 10 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 11 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 12 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 13 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 14 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 15 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 16 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 17 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 18 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 19 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 20 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 21 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 22 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 23 | 23 | 0 | 2 | 0 | 21 | 2 | 2.5 | 25.0 + TT | 18.5-TT |
| 24 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 25 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 26 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 27 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 28 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 29 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 30 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 31 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 32 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 33 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 34 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 35 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 36 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 37 | 23 | 0 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 38 | 23 | 0 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 39 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-3: UE Power Class test requirements (for Bands n48, n77, n78, n79) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | 25.8 | 2.0 | 3 | 28.0 + TT | 22.8 - TT |
| 2 | 23 | -3 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28.0 + TT | 19.5 - TT |
| 3 | 23 | -3 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28.0 + TT | 19.5 - TT |
| 4 | 23 | -3 | 1.2 | 0 | 24.8 | 2.0 | 3 | 28.0 + TT | 21.8 - TT |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| 6 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 7 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 8 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 9 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| 10 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 11 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 12 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 13 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 14 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 15 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 16 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 17 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 18 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 19 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 20 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 21 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 22 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 23 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 3 | 25.0 + TT | 18.5 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 27 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 28 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 29 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 30 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 31 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 32 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 33 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 34 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 35 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 36 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 37 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 38 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 39 | 23 | 0 | 0 | 0 | 23 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4: UE Power Class test requirements (for Bands n34, n39, n41, n77, n78, n79) for Power Class 2 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 2.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0- TT2） |
| 4 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 2.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.0 - TT2） |
| 5 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 2.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 8 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 2.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 9 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 2.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 12 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 2.0 |  | 3 | 28.0 + TT | 21.0 - TT | （19.5 - TT2） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0- TT2） |
| 15 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 16 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 17 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 18 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 19 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 2.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 20 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 21 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 22 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 28.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 23 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 2.0 |  | 3 | 28.0 + TT | 21.0 - TT | （19.5 - TT2） |
| 24 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 26 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 28.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 27 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 28 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 29 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 30 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 31 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4a: UE Power Class test requirements for Band n14 for Power Class 1 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| 2 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 3 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 4 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 5 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| 6 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 7 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 8 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 9 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 10 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 11 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 12 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 13 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 14 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 15 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 16 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 17 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 18 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 19 | 31 | 0 | 1.5 | 0 | 29.5 | 2.0 | 3 | 33.0 + TT | 26.5 — TT |
| 20 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 21 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 22 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 23 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 24 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 25 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 26 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 27 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 28 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 29 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 30 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 31 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 32 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 33 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 34 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 35 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4b: Void

Table 6.2.2.5-4c: UE Power Class test requirements for bands other than Band n14 for Power Class 1 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) |  | ΔTC,c (dB) | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | <50 MHz CBW | ≥50 MHz CBW |  | <50 MHz CBW | ≥50 MHz CBW |  |  |  | <50 MHz CBW | ≥50 MHz CBW |
| 1 | 31 | 0 | 0 | 0 | 0 | 31.0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 - TT | 28.0 - TT |
| 2 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 3 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 4 | 31 | 0 | 0.5 | 0.5 | 0 | 30.5 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 - TT | 27.5 - TT |
| 5 | 31 | 0 | 0 | 0 | 0 | 31.0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 - TT | 28.0 - TT |
| 6 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 7 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 8 | 31 | 0 | 1 | 1 | 0 | 30.0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 - TT | 27.0 - TT |
| 9 | 31 | 0 | 1 | 1 | 0 | 30.0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 - TT | 27.0 - TT |
| 10 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 11 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 12 | 31 | 0 | 2 | 2 | 0 | 29.0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 - TT | 26.0 - TT |
| 13 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 14 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 15 | 31 | 0 | 2.5 | 2.5 | 0 | 28.5 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 - TT | 25.5 - TT |
| 16 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 17 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 18 | 31 | 0 | 4.5 | 4.5 | 0 | 26.5 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 - TT | 23.5 - TT |
| 19 | 31 | 0 | 1.5 | 1.5 | 0 | 29.5 | 29.5 | 2.0 | 3 | 33.0 + TT | 26.5 - TT | 26.5 - TT |
| 20 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 21 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 22 | 31 | 0 | 3 | 3 | 0 | 28.0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 - TT | 25.0 - TT |
| 23 | 31 | 0 | 2 | 2 | 0 | 29.0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 - TT | 26.0 - TT |
| 24 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 25 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 26 | 31 | 0 | 3 | 3 | 0 | 28.0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 - TT | 25.0 - TT |
| 27 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 28 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 29 | 31 | 0 | 3.5 | 3.5 | 0 | 27.5 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 - TT | 24.5 - TT |
| 30 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 31 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 32 | 31 | 0 | 6.5 | 6.5 | 0 | 24.5 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 - TT | 21.5 - TT |
| 33 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 34 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 35 | 31 | 0 | 0 | 0 | 0 | 31.0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 - TT | 28.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-5: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

-------- Skipped unchanged clauses --------

##### 6.5.2.4.2	UTRA ACLR

6.5.2.4.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5.2.4.2.2	Test applicability

This test case applies for network signalling values NS_03U, NS_05U, NS_43U, and NS_100 to all types of NR Power Class 3 UE release 15 and forward that don’t support Tx diversity.

6.5.2.4.2.3	Minimum conformance requirements

UTRA adjacent channel leakage power ratio (UTRAACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent(s) UTRA channel frequency.

UTRAACLR is specified for the first adjacent UTRA channel (UTRAACLR1) which centre frequency is ± 2.5 MHz from NR channel edge and for the 2nd adjacent UTRA channel (UTRAACLR2) which centre frequency is ± 7.5 MHz from NR channel edge.

The UTRA channel power is measured with an RRC filter with roll-off factor =0.22 and bandwidth of 3.84 MHz. The assigned NR channel power is measured with a rectangular filter with measurement bandwidth specified in Table 6.5.2.4.1.3-1.

If the measured adjacent channel power is greater than - 50dBm then the UTRAACLR1 and UTRAACLR2 shall be higher than the value specified in Table 6.5.2.4.2.3-1.

UTRAACLR is not applicable to the power class 3 UE operating in Band n12, n14, n17, and n30.

UTRAACLR is not applicable to the power class 1 UE operating in Band n14, n100 and n101.

Table 6.5.2.4.2.3-1: UTRA ACLR requirement

|  | Power class 3 |
| --- | --- |
| UTRAACLR1 | 33 dB |
| UTRAACLR2 | 36 dB |

UTRA ACLR requirement is applicable when signalled by the network with network signalling value indicated by the field additionalSpectrumEmission.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.2.4.2.

-------- End of changes --------