3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230611

# Online, , 13th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3168 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ENDC RRC testcase 8.2.3.17.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add test case 8.2.3.17.1 to the 5G/ENDC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.2.3.17.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.17.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	6

3	Branches executed	11

4	Execution Log Files	11

4.1	Mediatek MT6815	11

5	References	11

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.2.3.17.1 which is part of the 5G ENDC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	8.2.3.17.1

ATS Version:	iwd-TTCN3-B2022-09_D23wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Mediatek MT6815

Verification Status:	PASS

# Corrections required

Changes 2 and 3 of R5s230554 are also needed.

## Change 1

| Function name | f_TC_8_2_3_17_1_ENDC_NR() |
| --- | --- |
| Reason for change | There is coordination issues EUTRA_5GS_PTC awaits for Eutra_NR_Coordination_MSG |
| Summary of change | Added f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}) |
| TTCN module | RRCMeasurement_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_2_3_17_1_ENDC_NR() runs on ENDC_NR_PTC { //Measurement configuration control and reporting / SFTD / EN-DC f_ENDC_NR_Init(); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_TestBody_Set(true); fl_TC_8_2_3_17_1_ENDC_NR_TestBody (); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change

function f_TC_8_2_3_17_1_ENDC_NR() runs on ENDC_NR_PTC { //Measurement configuration control and reporting / SFTD / EN-DC f_ENDC_NR_Init(); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); f_NR_TestBody_Set(true); fl_TC_8_2_3_17_1_ENDC_NR_TestBody (); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

## Change 2

| Function name | fl_TC_8_2_3_17_1_ENDC_NR_TestBody() |
| --- | --- |
| Reason for change | Coordination issues gets resolve with added TTCN changes |
| Summary of change | Added f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}) & f_EUTRA_NR_WaitForCoOrd_Data(EUTRA, "Complete") |
| TTCN module | RRCMeasurement_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_8_2_3_17_1_ENDC_NR_TestBody() runs on ENDC_NR_PTC { var template (value) CellGroupConfig v_SCGConfig; var template (value) RadioBearerConfig v_RadioBearerConfig; //@siclog "Steps 1-3" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup inter-RAT SFTD measurement on NR neighbour cell (NR Cell 1) //The UE transmits an RRCConnectionReconfigurationComplete message. //Check: Does the UE transmit a MeasurementReport message including measResultCellListSFTD within the next 5s? NOTE 1 //Run on EUTRA side //@siclog "Step 4-5" siclog@ //The SS transmits an RRCConnectionReconfiguration including nr-SecondaryCellGroupConfig messages to add NR Cell 1 as PSCell. f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {f_NR_GetRadioBearerConfig_FullAM_DRB2(nr_Cell1)}); v_RadioBearerConfig := cs_NR_RadioBearer(2, secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()); v_SCGConfig := f_NR_GetSCGConfigDef(nr_Cell1); f_NR_SendRRCReconfigurationContentsToEUTRA(v_RadioBearerConfig, v_SCGConfig, omit, omit, false, reconfigurationWithSync_CheckRACH); // Get Security info from EUTRA, calculate latest keys and configure SS f_ENDC_GetKeysActivateSecurity(nr_Cell1, EUTRA); //@siclog "Step 6-8" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup intra-frequency SFTD measurement on NR PSCell (NR Cell 1) //The UE transmits an RRCConnectionReconfigurationComplete message. //Check: Does the UE transmit a MeasurementReport message including measResultCellListSFTD within the next 3s? NOTE 2 //Run on EUTRA side }//End of l_TC_8_2_3_17_1_ENDC_NR_TestBody()

After Change

function fl_TC_8_2_3_17_1_ENDC_NR_TestBody() runs on ENDC_NR_PTC { var template (value) CellGroupConfig v_SCGConfig; var template (value) RadioBearerConfig v_RadioBearerConfig; var octetstring v_OctetData; //@siclog "Steps 1-3" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup inter-RAT SFTD measurement on NR neighbour cell (NR Cell 1) //The UE transmits an RRCConnectionReconfigurationComplete message. //Check: Does the UE transmit a MeasurementReport message including measResultCellListSFTD within the next 5s? NOTE 1 //Run on EUTRA side //@siclog "Step 4-5" siclog@ //The SS transmits an RRCConnectionReconfiguration including nr-SecondaryCellGroupConfig messages to add NR Cell 1 as PSCell. f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {f_NR_GetRadioBearerConfig_FullAM_DRB2(nr_Cell1)}); v_RadioBearerConfig := cs_NR_RadioBearer(2, secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()); v_SCGConfig := f_NR_GetSCGConfigDef(nr_Cell1); f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); f_NR_SendRRCReconfigurationContentsToEUTRA(v_RadioBearerConfig, v_SCGConfig, omit, omit, false, reconfigurationWithSync_CheckRACH); // Get Security info from EUTRA, calculate latest keys and configure SS v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(EUTRA, "Complete"); f_ENDC_GetKeysActivateSecurity(nr_Cell1, EUTRA); //@siclog "Step 6-8" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup intra-frequency SFTD measurement on NR PSCell (NR Cell 1) //The UE transmits an RRCConnectionReconfigurationComplete message. //Check: Does the UE transmit a MeasurementReport message including measResultCellListSFTD within the next 3s? NOTE 2 //Run on EUTRA side }//End of l_TC_8_2_3_17_1_ENDC_NR_TestBody()

## Change 3

| Function name | fl_TC_8_2_3_17_1_ENDC_EUTRA_TestBody() |
| --- | --- |
| Reason for change | UE requires switching time as per core spec TS 38.133 cl 9.2.1, below snippet When measurement gaps are needed, the UE is not expected to detect SSB which start earlier than the gap starting time + switching time, nor detect SSB which end later than the gap end – switching time. Switching time is 0.5ms for frequency range FR1 and 0.25ms for frequency range FR2. #Draft ProseCR would be raised for next RAN5 meeting |
| Summary of change | Added template for Meas gap with offset (19) which allows UE to read/search NR cell. |
| TTCN module | RRCMeasurement_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_8_2_3_17_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var Frequency_IE_Type v_Frequency_IE_f1; var ChannelBandwidthDependency_Type v_ChBandDependency_f1; var EUTRA_NR_CoOrd_SysInfo_Type v_EUTRA_NR_SysInfo; var integer v_NR_PhysChId_Cell1; var integer v_NR_ARFCN_f1; var RS_ConfigSSB_NR_r15.subcarrierSpacingSSB_r15 v_SCS; var FreqBandIndicatorNR_r15 v_FreqBandIndicatorNR_r15; var template (omit) MeasConfig v_MeasConfig; var template (value) ReportConfigToAddMod_ReportConfig_Type v_ReportConfig; var template (present) UL_DCCH_Message v_MeasurementReport; var template (present) MeasResultCellSFTD_r15 v_MeasResultCellSFTD_r15; var template (omit) octetstring v_NR_SCGConfig; var template (omit) octetstring v_NR_RBConfig; var EUTRA38_SecurityParamsExtd_Type v_Security_Params; var float v_WaitingTime; timer t_WaitTime; //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup inter-RAT SFTD measurement on NR neighbour cell (NR Cell 1) //The UE transmits an RRCConnectionReconfigurationComplete message. v_Frequency_IE_f1 := f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1); v_ChBandDependency_f1 := f_EUTRA_BandDependentParam(v_Frequency_IE_f1.DL_ChBandwidth, v_Frequency_IE_f1.UL_ChBandwidth); v_EUTRA_NR_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(NR); v_NR_PhysChId_Cell1 := v_EUTRA_NR_SysInfo.NR[0].PhysCellId; v_NR_ARFCN_f1 := v_EUTRA_NR_SysInfo.NR[0].Arfcn; v_SCS := f_NR_ConvertIntToSCS(v_EUTRA_NR_SysInfo.NR[0].SCS); v_FreqBandIndicatorNR_r15 := v_EUTRA_NR_SysInfo.NR[0].Band; //Table 8.2.3.17.1.3.3-3 v_ReportConfig := f_EUTRA_Create_ReportConfig_SFTD(neighborCells, cs_ReportQuantityNR_r15(false, false, false)); //@sic R5-222272 sic@ //Table 8.2.3.17.1.3.3-2 v_MeasConfig := cs_MeasConfig(omit, cs_MeasObjectToAddModList_2Entries(tsc_IdMeasObject_f1, cs_MeasObject_measObjectEUTRA_Common(f_EUTRA_ARFCN_Value(v_Frequency_IE_f1.UL_DL_Earfcn.dl_CarrierFreq_r9), v_ChBandDependency_f1.AllowedMeasBandwidth), tsc_IdMeasObject_NR_f1, cs_MeasObject_measObjectNR(v_NR_ARFCN_f1,v_SCS, v_FreqBandIndicatorNR_r15)), omit, cs_ReportConfigToAddModList_1Entry(1, v_ReportConfig), omit, cs_MeasIdToAddModList_1Entry(1, tsc_IdMeasObject_NR_f1, 1), -, cs_508_MeasGapConfig_GP1, -, -, -, f_ReturnMeasObjectToAddModList_v9e0(v_Frequency_IE_f1.UL_DL_Earfcn.dl_CarrierFreq_r9) ); f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_Common(v_RRC_TI, v_MeasConfig)); //@siclog "Step 3" siclog@ //Check: Does the UE transmit a MeasurementReport message including measResultCellListSFTD within the next 5s? NOTE 1 v_WaitingTime := 5.0; //Table 8.2.3.17.1.3.3-4 v_MeasurementReport := cr_MeasurementReport(1,?,?,*,*); v_MeasResultCellSFTD_r15 :=cr_MeasResultCellSFTD_r15(v_NR_PhysChId_Cell1, ?); v_MeasurementReport.message_.c1.measurementReport.criticalExtensions.c1.measurementReport_r8.measResults.measResultCellListSFTD_r15 := {v_MeasResultCellSFTD_r15}; t_WaitTime.start(v_WaitingTime); while(t_WaitTime.running){ SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, v_MeasurementReport)); t_WaitTime.stop; } f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.2.3.17.1 Step 3"); //@siclog "Steps 4-5" siclog@ //The SS transmits an RRCConnectionReconfiguration including nr-SecondaryCellGroupConfig messages to add NR Cell 1 as PSCell. //The UE transmits an RRCConnectionReconfigurationComplete message including scg-ConfigResponseNR-r15 v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); f_EUTRA38_508_ENDC_SendReconfiguration(eutra_Cell1, -, -, v_NR_SCGConfig, v_NR_RBConfig); v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); f_EUTRA38_Security_Set(v_Security_Params); //@siclog "Step 6-7" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup intra-frequency SFTD measurement on NR PSCell (NR Cell 1) //The UE transmits an RRCConnectionReconfigurationComplete message. //Table 8.2.3.17.1.3.3-3 v_ReportConfig := f_EUTRA_Create_ReportConfig_SFTD(pSCell); v_MeasConfig.reportConfigToAddModList := cs_ReportConfigToAddModList_1Entry(1, v_ReportConfig); f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_Common(v_RRC_TI, v_MeasConfig)); //@siclog "Step 8" siclog@ //Check: Does the UE transmit a MeasurementReport message including measResultCellListSFTD within the next 3s? NOTE 2 v_WaitingTime := 3.0; //Table 8.2.3.17.1.3.3-4 v_MeasResultCellSFTD_r15 :=cr_MeasResultCellSFTD_r15(v_NR_PhysChId_Cell1); v_MeasurementReport.message_.c1.measurementReport.criticalExtensions.c1.measurementReport_r8.measResults.measResultCellListSFTD_r15 := {v_MeasResultCellSFTD_r15}; t_WaitTime.start(v_WaitingTime); while(t_WaitTime.running){ SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, v_MeasurementReport)); t_WaitTime.stop; } f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.2.3.17.1 Step 8"); }

After Change

function fl_TC_8_2_3_17_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var Frequency_IE_Type v_Frequency_IE_f1; var ChannelBandwidthDependency_Type v_ChBandDependency_f1; var EUTRA_NR_CoOrd_SysInfo_Type v_EUTRA_NR_SysInfo; var integer v_NR_PhysChId_Cell1; var integer v_NR_ARFCN_f1; var RS_ConfigSSB_NR_r15.subcarrierSpacingSSB_r15 v_SCS; var FreqBandIndicatorNR_r15 v_FreqBandIndicatorNR_r15; var template (omit) MeasConfig v_MeasConfig; var template (value) ReportConfigToAddMod_ReportConfig_Type v_ReportConfig; var template (present) UL_DCCH_Message v_MeasurementReport; var template (present) MeasResultCellSFTD_r15 v_MeasResultCellSFTD_r15; var template (omit) octetstring v_NR_SCGConfig; var template (omit) octetstring v_NR_RBConfig; var EUTRA38_SecurityParamsExtd_Type v_Security_Params; var float v_WaitingTime; timer t_WaitTime; template (value) MeasGapConfig cs_508_MeasGapConfig_GP2 := cs_MeasGapConfig_setup_gp1(19); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup inter-RAT SFTD measurement on NR neighbour cell (NR Cell 1) //The UE transmits an RRCConnectionReconfigurationComplete message. v_Frequency_IE_f1 := f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1); v_ChBandDependency_f1 := f_EUTRA_BandDependentParam(v_Frequency_IE_f1.DL_ChBandwidth, v_Frequency_IE_f1.UL_ChBandwidth); v_EUTRA_NR_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(NR); v_NR_PhysChId_Cell1 := v_EUTRA_NR_SysInfo.NR[0].PhysCellId; v_NR_ARFCN_f1 := v_EUTRA_NR_SysInfo.NR[0].Arfcn; v_SCS := f_NR_ConvertIntToSCS(v_EUTRA_NR_SysInfo.NR[0].SCS); v_FreqBandIndicatorNR_r15 := v_EUTRA_NR_SysInfo.NR[0].Band; //Table 8.2.3.17.1.3.3-3 v_ReportConfig := f_EUTRA_Create_ReportConfig_SFTD(neighborCells, cs_ReportQuantityNR_r15(true, false, false)); //@sic R5-222272 sic@ //Table 8.2.3.17.1.3.3-2 v_MeasConfig := cs_MeasConfig(omit, cs_MeasObjectToAddModList_2Entries(tsc_IdMeasObject_f1, cs_MeasObject_measObjectEUTRA_Common(f_EUTRA_ARFCN_Value(v_Frequency_IE_f1.UL_DL_Earfcn.dl_CarrierFreq_r9), v_ChBandDependency_f1.AllowedMeasBandwidth), tsc_IdMeasObject_NR_f1, cs_MeasObject_measObjectNR(v_NR_ARFCN_f1,v_SCS, v_FreqBandIndicatorNR_r15)), omit, cs_ReportConfigToAddModList_1Entry(1, v_ReportConfig), omit, cs_MeasIdToAddModList_1Entry(1, tsc_IdMeasObject_NR_f1, 1), -, cs_508_MeasGapConfig_GP2, -, -, -, f_ReturnMeasObjectToAddModList_v9e0(v_Frequency_IE_f1.UL_DL_Earfcn.dl_CarrierFreq_r9) ); f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_Common(v_RRC_TI, v_MeasConfig)); //@siclog "Step 3" siclog@ //Check: Does the UE transmit a MeasurementReport message including measResultCellListSFTD within the next 5s? NOTE 1 v_WaitingTime := 5.0; //Table 8.2.3.17.1.3.3-4 v_MeasurementReport := cr_MeasurementReport(1,?,?,*,*); v_MeasResultCellSFTD_r15 :=cr_MeasResultCellSFTD_r15(v_NR_PhysChId_Cell1, ?); v_MeasurementReport.message_.c1.measurementReport.criticalExtensions.c1.measurementReport_r8.measResults.measResultCellListSFTD_r15 := {v_MeasResultCellSFTD_r15}; t_WaitTime.start(v_WaitingTime); while(t_WaitTime.running){ SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, v_MeasurementReport)); t_WaitTime.stop; } f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.2.3.17.1 Step 3"); //@siclog "Steps 4-5" siclog@ //The SS transmits an RRCConnectionReconfiguration including nr-SecondaryCellGroupConfig messages to add NR Cell 1 as PSCell. //The UE transmits an RRCConnectionReconfigurationComplete message including scg-ConfigResponseNR-r15 v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); f_EUTRA38_508_ENDC_SendReconfiguration(eutra_Cell1, -, -, v_NR_SCGConfig, v_NR_RBConfig); v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); f_EUTRA38_Security_Set(v_Security_Params); //@siclog "Step 6-7" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup intra-frequency SFTD measurement on NR PSCell (NR Cell 1) //The UE transmits an RRCConnectionReconfigurationComplete message. //Table 8.2.3.17.1.3.3-3 v_ReportConfig := f_EUTRA_Create_ReportConfig_SFTD(pSCell); v_MeasConfig.reportConfigToAddModList := cs_ReportConfigToAddModList_1Entry(1, v_ReportConfig); f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_Common(v_RRC_TI, v_MeasConfig)); //@siclog "Step 8" siclog@ //Check: Does the UE transmit a MeasurementReport message including measResultCellListSFTD within the next 3s? NOTE 2 v_WaitingTime := 3.0; //Table 8.2.3.17.1.3.3-4 v_MeasResultCellSFTD_r15 :=cr_MeasResultCellSFTD_r15(v_NR_PhysChId_Cell1); v_MeasurementReport.message_.c1.measurementReport.criticalExtensions.c1.measurementReport_r8.measResults.measResultCellListSFTD_r15 := {v_MeasResultCellSFTD_r15}; t_WaitTime.start(v_WaitingTime); while(t_WaitTime.running){ SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, v_MeasurementReport)); t_WaitTime.stop; } f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.2.3.17.1 Step 8"); }

# Branches executed

The test case was executed on Eutra band 3 and NR band n78 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## Mediatek MT6815

The Mediatek MT6815 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_2_3_17_1_LOG.html

- PICS/PIXIT parameter file:

TC_8_2_3_17_1_PIXIT.xml

# References

| [1] | R5s230612: Supporting information for addition of ENDC RRC testcase 8.2.3.17.1 in FR1 |
| --- | --- |