3GPP TSG-RAN5 Meeting #100	R5-234080

# Toulouse, France, 21st – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5459 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test configuration for LTE inter-band DL CA and UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CA confiuration (PCC NRB: 100/SCC NRB: 50) is missed for LTE inter-band DL CA and UL CA |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of test configuration to add CA confiuration (PCC NRB: 100/SCC NRB: 50) for LTE inter-band DL CA and UL CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA confiuration (PCC NRB: 100/SCC NRB: 50) for LTE inter-band DL CA and UL CA will be still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2A.2, 6.3.5A.1.2, 6.3.5A.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

6.3.2A.2	Minimum Output Power for CA (inter-band DL CA and UL CA)

Many information & Tables are skipped

Table 6.3.2A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/Afor this test | QPSK | 31 | P_6@0 | S_25@0 | - | - |
| 6 | 50 |  | QPSK | 56 | P_6@0 | S_50@0 | - | - |
| 15 | 25 |  | QPSK | 12 | S_4@0 | P_8@0 | - | - |
| 25 | 6 |  | QPSK | 13 | P_8@0 | P_5@0 | - | - |
| 25 | 15 |  | QPSK | 40 | P_25@0 | S_15@0 | - | - |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 25 | 75 |  | QPSK | 24 | P_8@0 | S_16@0 | - | - |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

6.3.5A.1.2	Power Control Absolute power tolerance for CA (inter-band DL CA and UL CA)

Many information & Tables are skipped

Table 6.3.5A.1.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Mid range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_ alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/A for this test | QPSK | 31 | P_6@0 | S_25@0 | - | - |
| 6 | 50 |  | QPSK | 56 | P_6@0 | S_50@0 | - | - |
| 15 | 25 |  | QPSK | 12 | S_4@0 | P_8@0 | - | - |
| 25 | 6 |  | QPSK | 13 | P_8@0 | P_5@0 | - | - |
| 25 | 15 |  | QPSK | 40 | P_25@0 | S_15@0 | - | - |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 25 | 75 |  | QPSK | 24 | P_8@0 | S_16@0 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 | - | - |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

6.3.5A.3.2	Aggregate power control tolerance for CA (inter-band DL CA and UL CA)

Many information & Tables are skipped

Table 6.3.5A.3.2.4.1-1: Test Configuration Table: PUSCH

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] clause 4.1 |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Mid range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in clause 5.4.2A.1 for the CA Configuration |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/A for this test | QPSK | 31 | P_6@0 | S_25@0 | - | - |
| 6 | 50 |  | QPSK | 56 | P_6@0 | S_50@0 | - | - |
| 15 | 25 |  | QPSK | 40 | S_15@0 | S_25@0 | - | - |
| 25 | 6 |  | QPSK | 31 | S_25@0 | P_6@0 | - | - |
| 25 | 15 |  | QPSK | 40 | P_25@0 | S_15@0 | - | - |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 25 | 75 |  | QPSK | 100 | P_25@0 | S_75@0 | - | - |
| 25 | 100 |  | QPSK | 125 | P_25@0 | S_100@0 | - | - |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 50 | 75 |  | QPSK | 125 | P_50@0 | S_75@0 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 | - | - |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. |  |  |  |  |  |  |  |  |

## << End of changes >>