3GPP TSG-RAN5 Meeting #100	R5-234795

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5240 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RRC test case 8.1.2.15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In Table 8.1.2.15.3.3-1, the value of IE “schedulingInfoSIB1-BR-r13” in message MasterInformationBlock depends on UE category, when UE under test is CAT M1 then it should be 10 otherwise 0 as it is required to be scheduled only for the case of CAT M1 UEs. This is in accordance to TS 36.331, clause 6.2.2: schedulingInfoSIB1-BR Indicates the index to the tables that define SystemInformationBlockType1-BR scheduling information. The tables are specified in TS 36.213 [23], Table 7.1.6-1 and Table 7.1.7.2.7-1. Value 0 means that SystemInformationBlockType1-BR is not scheduled. - The specific message contents defined for SystemInformationBlockType1-BR-r13 in Table 8.1.2.15.3.3-3 are only applicable when UE category is M1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modified Table 8.1.2.15.3.3-1 to define the value of IE “schedulingInfoSIB1-BR-r13” for non-CAT M1 UEs as well. - Added condition CAT M1 for Table 8.1.2.15.3.3-3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.1.2.15	RRC connection establishment / Extended and spare fields in SI

8.1.2.15.1	Test Purpose (TP)

(1)

with { UE is powered on and receives system information }

ensure that {

when { UE receives an optional spare or extended field in system information that it does not comprehend }

then { UE treats system information as if the spare or extended field were absent and system information is not ignored, and UE establishes an RRC connection }

}

8.1.2.15.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.2.2.3, 5.7.1 and 5.7.3.

[TS 36.331 clause 5.2.2.3]

The UE shall:

1>	ensure having a valid version, as defined below, of (at least) the following system information, also referred to as the 'required' system information:

2>	if in RRC_IDLE:

3>	if the UE is a NB-IoT UE:

4>	the MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB and SystemInformationBlockType1-NB as well as SystemInformationBlockType2-NB through SystemInformationBlockType5-NB, SystemInformationBlockType22-NB;

3>	else:

4>	the MasterInformationBlock and SystemInformationBlockType1 (or SystemInformationBlockType1-BR depending on whether the UE is a BL UE or the UE in CE) as well as SystemInformationBlockType2 through SystemInformationBlockType8 and SystemInformationBlockType24 (depending on support of the concerned RATs), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking when the UE is connected to EPC), SystemInformationBlockType25 (depending on support of E-UTRA/5GC);

2>	if in RRC_INACTIVE:

3>	the MasterInformationBlock and SystemInformationBlockType1 as well as SystemInformationBlockType2 through SystemInformationBlockType8 (depending on support of the concerned RATs), SystemInformationBlockType25;

2>	if in RRC_CONNECTED; and

2>	the UE is not a BL UE; and

2>	the UE is not in CE; and

2>	the UE is not a NB-IoT UE:

3>	the MasterInformationBlock, SystemInformationBlockType1 and SystemInformationBlockType2 as well as SystemInformationBlockType8 (depending on support of CDMA2000), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking when the UE is connected to EPC), SystemInformationBlockType25 (depending on support of E-UTRA/5GC);

2>	if in RRC_CONNECTED and T311 is running; and

2>	the UE is a BL UE or the UE is in CE or the UE is a NB-IoT UE;

3>	the MasterInformationBlock (or MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT), SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) and SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT), and for NB-IoT SystemInformationBlockType22-NB;

1>	delete any stored system information after 3 hours or 24 hours from the moment it was confirmed to be valid as defined in 5.2.1.3, unless specified otherwise;

1>	consider any stored system information except SystemInformationBlockType10, SystemInformationBlockType11, systemInformationBlockType12 and systemInformationBlockType14 (systemInformationBlockType14-NB in NB-IoT) to be invalid if systemInfoValueTag included in the SystemInformationBlockType1 (MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT) is different from the one of the stored system information and in case of NB-IoT UEs, BL UEs and UEs in CE, systemInfoValueTagSI is not broadcasted. Otherwise consider system information validity as defined in 5.2.1.3;

[TS 36.331 clause 5.7.1]

The generic error handling defined in the subsequent sub-clauses applies unless explicitly specified otherwise e.g. within the procedure specific error handling.

The UE shall consider a value as not comprehended when it is set:

-	to an extended value that is not defined in the version of the transfer syntax supported by the UE.

-	to a spare or reserved value unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/ reserved value.

The UE shall consider a field as not comprehended when it is defined:

-	as spare or reserved unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/ reserved field.

[TS 36.331 clause 5.7.3]

The UE shall, when receiving an RRC message on any logical channel:

1>	if the message includes a field that has a value that the UE does not comprehend:

2>	if a default value is defined for this field:

3>	treat the message while using the default value defined for this field;

2>	else if the concerned field is optional:

3>	treat the message as if the field were absent and in accordance with the need code for absence of the concerned field;

2>	else:

3>	treat the message as if the field were absent and in accordance with sub-clause 5.7.4;

8.1.2.15.3	Test description

8.1.2.15.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

UE:

None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

8.1.2.15.3.2	Test procedure sequence

Table 8.1.2.15.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | System information that includes extended and spare fields that the UE does not comprehend is broadcasted on BCCH and BCCH-BR. NOTE: Spare field testing is not applicable for Rel-12 UE because a spare field in MIB or SIB1 does not exist in Rel-12. | <-- | RRC: SYSTEM INFORMATION (BCCH and BCCH-BR) | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | Does the UE transmit an RRCConnectionRequest message on Cell 1? | --> | RRCConnectionRequest | 1 | P |
| 4 | The SS transmits an RRCConnectionSetup message on Cell 1. | <-- | RRCConnectionSetup | - | - |
| 5 | Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the Attach procedure by including the ATTACH REQUEST message? The PDN CONNECTIVITY REQUEST message is piggybacked in ATTACH REQUEST. | --> | RRCConnectionSetupComplete NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST | 1 | P |
| 6-18 | Steps 5 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. | - | - | - | - |
| NOTE: SIB-Type and SIB-Type-v12j0 in SystemInformationBlockType1 and SystemInformationBlockType1-BR are set to values in Rel-17to check spare and extended fields in UE up to Rel-16. |  |  |  |  |  |

8.1.2.15.3.3	Specific message contents

Table 8.1.2.15.3.3-1: Message MasterInformationBlock (step 1, Table 8.1.2.15.3.2-1)

| Derivation Path: 36.508 Table 4.4.3.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| MasterInformationBlock ::= SEQUENCE { |  |  |  |  |
| schedulingInfoSIB1-BR-r13 | 10 |  | when UE under test is CAT M1 |
|  | 0 |  |  |
| partEARFCN-17 CHOICE { |  |  |  |
| spare | ‘11’B |  |  |
| } |  |  |  |
| spare | ‘1’B | From Rel-8 to Rel-16, the lowest bit is considered spare according to TS 36.331 [17]. | For Rel-8 to Rel-16 |
| } |  |  |  |

Table 8.1.2.15.3.3-2: Message SystemInformationBlockType1 (step 1, Table 8.1.2.15.3.2-1)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |  |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE { |  | 3 entries for Rel-8 to Rel-15 2 entries for Rel-16 |  |  |
| si-Periodicity[1] | rf16 |  |  |  |
| sib-MappingInfo[1] |  | SIB2, SIB3 |  |  |
| si-Periodicity[2] | As per TS 36.508[18] clause 4.4.3.4 |  |  |  |
| sib-MappingInfo[2] | sibType3 |  |  |  |
| si-Periodicity[3] | rf32 |  | For Rel-8 to Rel-15 |  |
| sib-MappingInfo[3] SEQUENCE (SIZE (0..maxSIB-1)) OF { | 1 entry |  |  |  |
| SIB-Type[1] | sibType29-v1610 | From Rel-8 to Rel-15, the field used for sibType29-v1610 is considered an extended field according to TS 36.331 [17]. |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension { |  |  | For Rel-16 |
| lateNonCriticalExtension { |  |  |  |
| nonCriticalExtension { |  |  |  |
| nonCriticalExtension { |  |  |  |
| nonCriticalExtension { |  |  |  |
| nonCriticalExtension { |  |  |  |
| nonCriticalExtension { |  |  |  |
| schedulingInfoExt-r12 (SIZE (1..maxSI-Message)) OF SEQUENCE { | 1 entry |  |  |
| si-Periodicity-r12[1] | rf32 |  |  |
| sib-MappingInfo-v12j0[1] SEQUENCE (SIZE (0..maxSIB-1)) OF { | 1 entry |  |  |
| SIB-Type-v12j0[1] | sibType32-v1700 | For Rel-16, the field used for sibType32-v1700 is considered an extended field according to TS 36.331 [17]. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.15.3.3-3: Message SystemInformationBlockType1-BR-r13 (step 1 when UE under test is CAT M1, Table 8.1.2.15.3.2-1)

| Derivation Path: 36.508 Table 4.4.3.2-3A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |  |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE { |  | 3 entries for Rel-13 to Rel-15 2 entries for Rel-16 |  |  |
| si-Periodicity[1] | rf16 |  |  |  |
| sib-MappingInfo[1] |  | SIB2, SIB3 |  |  |
| si-Periodicity[2] | As per TS 36.508[18] clause 4.4.3.4. |  |  |  |
| sib-Mappinginfo[2] | sibType3 |  |  |  |
| si-Periodicity[3] | rf32 |  | For Rel-13 to Rel-15 |  |
| sib-MappingInfo[3] SEQUENCE (SIZE (0..maxSIB-1)) OF { | 1 entry |  | For Rel-13 to Rel-15 |  |
| SIB-Type[1] | sibType29-v1610 | From Rel-13 to Rel-15, the field used for sibType29-v1610 is considered an extended field according to TS 36.331 [17]. |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension { |  |  | For Rel-16 |
| lateNonCriticalExtension { |  |  |  |
| nonCriticalExtension { |  |  |  |
| nonCriticalExtension { |  |  |  |
| nonCriticalExtension { |  |  |  |
| nonCriticalExtension { |  |  |  |
| nonCriticalExtension { |  |  |  |
| schedulingInfoExt-v12j0 (SIZE (1..maxSI-Message)) OF SEQUENCE { | 1 entry |  |  |
| si-Periodicity-r12[1] | rf32 |  |  |
| sib-MappingInfo-v12j0[1] SEQUENCE (SIZE (0..maxSIB-1)) OF { | 1 entry |  |  |
| SIB-Type-v12j0[1] | sibType32-v1700 | For Rel-16, the field used for sibType32-v1700 is considered an extended field according to TS 36.331 [17]. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |