3GPP TSG-RAN5 Meeting #100	R5-233849

# Toulouse, France, 21st – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0315 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.3.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - R5-233491 has introduced condition FUNCTIONAL_ALIAS for Table 5.5.3.2.1-3; for sake of consistency the same shall be done for Table 5.5.3.2.1-1 - In general the elements of <anyExt> provide information for particular features or use cases like ambient listening or functional alias and in other cases these elements shall not be present (e.g. when no functional alias is configured, the client shall not provide any <functional-alias-URI> in <anyExt>); allowing <anyExt> to contain further elements with any value hides non-conformant behaviour The elements of <anyExt> shall be present only in particular cases and not present otherwise - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Condition FUNCTIONAL_ALIAS added to Table 5.5.3.2.1-1 - Clarifications added to tables 5.5.3.2.1-1 and 5.5.3.2.1-3, that there shall be no further elements in <anyExt> unless specified otherwise in the specific message content of a test case; in table 5.5.3.2.1-2 <anyExt> is set to "not present" - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification, test specification will still allow non-conformant UE behaviour |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.5.3.2.1	MCS Info Lists from the UE

##### -	MCPTT

Table 5.5.3.2.1-1: MCPTT-Info from the UE

| Derivation Path: TS 24.379 [9] clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-access-token | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcptt-access-token> with mcpttString set to access token as assigned to the UE in the Token Response | The access token is opaque to the MCPTT client | TS 33.180 [94], clause B.4 RFC 6749 [77] | CONFIG, GROUPCONFIG |
| session-type | not present |  |  |  |
|  | "prearranged" |  |  | GROUP-CALL AND INVITE_REFER |
|  | "private" |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | “chat” |  |  | CHAT-GROUP-CALL AND INVITE_REFER |
|  | "first-to-answer" |  |  | FIRST-TO-ANSWER AND INVITE_REFER |
| mcptt-request-uri | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcptt-request-uri> with mcpttURI set to px_MCPTT_Group_A_ID | The URI of the group |  | (GROUP-CALL OR CHAT-GROUP-CALL) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) <mcptt-request-uri> with mcpttURI set to px_MCPTT_ID_User_B | The URI of the invited MCPTT Client |  | PRIVATE-CALL AND INVITE_REFER |
|  | encrypted (NOTE 2) <mcptt-request-uri> with mcpttURI set to px_MCPTT_ID_User_A |  |  | POC-SETTINGS-EVENT |
| mcptt-calling-user-id | not present or encrypted (NOTE 2) <mcptt-calling-user-id> with mcpttURI set to px_MCPTT_ID_User_A |  |  |  |
|  | not present |  |  | CONFIG, GROUPCONFIG, POC-SETTINGS-EVENT |
| mcptt-called-party-id | not present |  |  |  |
|  | not present or encrypted (NOTE 2) <mcptt-called-party-id> with mcpttURI set to px_MCPTT_ID_User_A |  |  | INVITE-RSP |
| mcptt-calling-group-id | not present |  |  |  |
| required | not present |  |  |  |
| emergency-ind | not present or encrypted (NOTE 2) <emergency-ind> with mcpttBoolean set to "false" |  |  |  |
|  | Encrypted (NOTE 2) <emergency-ind> with mcpttBoolean set to "true" |  |  | EMERGENCY-CALL AND INVITE_REFER |
| alert-ind | not present or encrypted (NOTE 2) <alert-ind> with mcpttBoolean set to "false" |  |  |  |
|  | Encrypted (NOTE 2) <alert-ind> with mcpttBoolean set to pc_MCX_EmergencyIndWithAlertInd |  |  | EMERGENCY-CALL AND INVITE_REFER |
| imminentperil-ind | not present or encrypted (NOTE 2) <imminentperil-ind> with mcpttBoolean set to "false" |  |  |  |
|  | Encrypted (NOTE 2) < imminentperil -ind> with mcpttBoolean set to "true" |  |  | IMMPERIL-CALL AND INVITE_REFER |
| broadcast-ind | not present or “false” |  |  |  |
|  | "true" |  |  | BROADCAST-CALL |
| mc-org | not present |  |  |  |
| floor-state | not present |  |  |  |
| associated-group-id | not present |  |  |  |
|  | px_MCPTT_Group_A_ID if mcptt-request-uri contains a temporary group identity; otherwise, not present | if the <mcptt-request-uri> element contains a group identity then this element can include an MCPTT group ID associated with the group identity in the <mcptt-request-uri> element. E.g. if the <mcptt-request-uri> element contains a temporary group identity (TGI), then the <associated-group-id> element can contain the constituent MCPTT group ID | TS 24.379 [9] clause F.1.3 | GROUP-CALL |
| originated-by | not present |  |  |  |
| MKFC-GKTPs | not present |  |  |  |
| mcptt-client-id | not present |  |  |  |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | The UUID URN of the MCPTT Client | RFC 4122 [106] TS 24.379 [9] clause 4.10 | (GROUP-CALL OR CHAT-GROUP-CALL OR EMERGENCY-CALL OR IMMPERIL-CALL) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) |  |  | (PRIVATE-CALL OR FIRST-TO-ANSWER) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | in general mcptt-client-id is not mandatory (e.g. for SIP SUBSCRIBE) | RFC 4122 [106] TS 24.379 [9] clause 4.10 | CONFIG, GROUPCONFIG |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | mcptt-client-id is mandatory in the SIP REGISTER or SIP PUBLISH for service authorisation according to TS 24.379 [9] clauses 7.2.1 and 7.2.2 | RFC 4122 [106] TS 24.379 [9] clause 4.10 | CONFIG AND REGISTER_PUBLISH |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | mcptt-client-id is mandatory in SIP PUBLISH for MCPTT service settings only, according to TS 24.379 [9] clause 7.2.3 | RFC 4122 [106] TS 24.379 [9] clause 4.10 | POC-SETTINGS-EVENT |
| alert-ind-rcvd | not present |  |  |  |
| anyExt | not present |  | TS 24.379 [9], clause F.1.3 |  |
| anyExt |  | anyExt shall not contain any further elements than listed below unless specified otherwise in the specifc message content of a test case | TS 24.379 [9], clause F.1.3 | FUNCTIONAL_ALIAS |
| functional-alias-URI | encrypted (NOTE 2) <functional-alias-URI> with mcpttURI set to px_MCPTT_ID_FA_A | set to the value of the functional alias that is used together with the "mcptt-calling-user-id" |  |  |
| NOTE 1: The SS shall check the mcptt-client-id - at the first time being sent by the UE to be a valid UUID URN with a format like "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" according to RFC 4122 [106] - to be all the same UUID URN in subsequent messages. NOTE 2: Encrypted element as described in Table 5.5.3.2.1-1A |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| REGISTER_PUBLISH | MCPTT-Info in SIP REGISTER or SIP PUBLISH request for service authorisation |
| INVITE_REFER | MCPTT-Info in SIP INVITE or SIP REFER request for call establishment |
| INVITE-RSP | MCPTT-Info in SIP response to a SIP INVITE NOTE: INVITE-RSP is inherited from the SIP response, i.e. it shall be considered as true whenever set for the SIP response |
| FUNCTIONAL_ALIAS | An active Functional Alias is used |
| For further conditions see table 5.5.1-1 |  |

Table 5.5.3.2.1-1A: Encrypted MCPTT info parameter sent by the UE

| Derivation Path: TS 24.379 [9] clauses F.1.2, F.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the mcptt parameter |  |  |  |

##### -	MCVideo

Table 5.5.3.2.1-2: MCVideo-Info from the UE

| Derivation Path: TS 24.281 [86] Clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-access-token | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcvideo-access-token> with mcvideoString set to access token as assigned to the UE in the Token Response | The access token is opaque to the MCVideo client | TS 33.180 [94], clause B.4 RFC 6749 [77] | CONFIG GROUPCONFIG |
| session-type | not present |  |  |  |
|  | "prearranged" |  |  | GROUP-CALL AND INVITE_REFER |
|  | "private" |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | “chat” |  |  | CHAT-GROUP-CALL AND INVITE_REFER |
| mcvideo-request-uri | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_Group_A_ID | The URI of the group |  | (GROUP-CALL OR CHAT-GROUP-CALL) AND INVITE_REFER |
|  | not present or Encrypted (NOTE 2) <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_User_B_ID | The URI of the invited MCVideo Client |  | PRIVATE-CALL AND INVITE_REFER |
|  | Encrypted (NOTE 2) <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_User_A_ID |  |  | POC-SETTINGS-EVENT |
| mcvideo-calling-user-id | not present or Encrypted (NOTE 2) <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_ID_User_A |  |  |  |
|  | not present |  |  | CONFIG, GROUPCONFIG, POC-SETTINGS-EVENT |
| mcvideo-called-party-id | not present |  |  |  |
|  | not present or Encrypted (NOTE 2) <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_ID_User_A |  |  | INVITE-RSP |
| mcvideo-calling-group-id | not present |  |  |  |
| required | not present |  |  |  |
| emergency-ind | not present or encrypted (NOTE 2) <emergency-ind> with mcvideoBoolean set to "false" |  |  |  |
|  | encrypted (NOTE 2) <emergency-ind> with mcvideoBoolean set to true |  |  | EMERGENCY-CALL AND INVITE-REFER |
| alert-ind | not present or encrypted (NOTE 2) <alert-ind> with mcvideoBoolean set to "false" |  |  |  |
|  | encrypted (NOTE 2) <alert-ind> with mcvideoBoolean set to pc_MCX_EmergencyIndWithAlertInd |  |  | EMERGENCY-CALL AND INVITE_REFER |
| imminentperil-ind | not present or encrypted (NOTE 2) <imminentperil-ind> with mcvideoBoolean set to "false" |  |  |  |
|  | encrypted (NOTE 2) <imminentperil-ind> with mcvideoBoolean set to true |  |  | IMMPERIL-CALL AND INVITE-REFER |
| broadcast-ind | not present or “false” |  |  |  |
|  | "true" |  |  | BROADCAST-CALL |
| mc-org | not present |  |  |  |
| associated-group-id | not present |  |  |  |
|  | px_MCVideo_Group_A_ID if mcvideo-request-uri contains a temporary group identity; otherwise, not present | if the <mcvideo-request-uri> element contains a group identity then this element can include an MCVideo group ID associated with the group identity in the <mcvideo-request-uri> element. E.g. if the <mcvideo-request-uri> element contains a temporary group identity (TGI), then the <associated-group-id> element can contain the constituent MCVideo group ID | TS 24.281 [86] clause F.1.3 | GROUP-CALL |
| originated-by | not present |  |  |  |
| MKFC-GKTPs | not present |  |  |  |
| mcvideo-client-id | not present |  |  |  |
|  | encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) | The UUID URN of the MCVIDEO Client | RFC 4122 [106] TS 24.281 [86] clause 4.9 | (GROUP-CALL OR CHAT-GROUP-CALL OR EMERGENCY-CALL OR IMMPERIL-CALL) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) | in general mcvideo-client-id is not mandatory (e.g. for SIP SUBSCRIBE) | RFC 4122 [106] TS 24.281 [86] clause 4.9 | CONFIG, GROUPCONFIG |
|  | encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) | mcvideo-client-id is mandatory in the SIP REGISTER or SIP PUBLISH for service authorisation according to TS 24.281 [86] clauses 7.2.1 and 7.2.2 | RFC 4122 [106] TS 24.281 [86] clause 4.9 | CONFIG AND REGISTER_PUBLISH |
|  | encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) | mcvideo-client-id is mandatory in SIP PUBLISH for MCVideo service settings only, according to TS 24.281 [86] clause 7.2.3 | RFC 4122 [106] TS 24.281 [86] clause 4.9 | POC-SETTINGS-EVENT |
| alert-ind-rcvd | not present |  |  |  |
| anyExt | not present |  | TS 24.281 [86] clause F.1.3 |  |
| NOTE 1: The SS shall check the mcvideo-client-id - at the first time being sent by the UE to be a valid UUID URN with a format like "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" according to RFC 4122 [106] - to be all the same UUID URN in subsequent messages. NOTE 2: Encrypted element as described in Table 5.5.3.2.1-2A |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| REGISTER_PUBLISH | MCVideo-Info in SIP REGISTER or SIP PUBLISH request for service authorisation |
| INVITE_REFER | MCVideo-Info in SIP INVITE or SIP REFER request for call establishment |
| INVITE-RSP | MCVideo-Info in SIP response to a SIP INVITE |
| For further conditions see table 5.5.1-1 |  |

Table 5.5.3.2.1-2A: Encrypted MCVideo info parameter sent by the UE

| Derivation Path: TS 24.281 [86] clauses F.1.2, F.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the mcvideo parameter |  |  |  |

##### -	MCData

Table 5.5.3.2.1-3: MCData-Info from the UE

| Derivation Path: TS 24.282 [87], Clause D.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-access-token | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcdata-access-token> with mcdataString set to access token as assigned to the UE in the Token Response | The access token is opaque to the MCData client | TS 33.180 [94], clause B.4 RFC 6749 [77] | CONFIG GROUPCONFIG |
| request-type | not present |  |  |  |
|  | "one-to-one-sds" |  |  | MCD_1to1 |
|  | “group-sds” |  |  | MCD_grp |
| mcdata-request-uri | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcdata-request-uri> with mcdataURI set to px_MCData_Group_A_ID |  |  | MCD_grp |
|  | Encrypted (NOTE 2) <mcdata-request-uri> with mcdataURI set to px_MCData_Group_A_ID |  |  | POC-SETTINGS-EVENT |
| mcdata-calling-user-id | not present |  |  |  |
| mcdata-called-party-id | not present |  |  |  |
| mcdata-calling-group-id | not present |  |  |  |
| alert-ind | not present |  |  |  |
| originated-by | not present |  |  |  |
| mcdata-client-id | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcdata-client-id> with mcdataString set to valid UUID URN (NOTE 1) |  |  | MCD_grp |
|  | Encrypted (NOTE 2) <mcdata-client-id> with mcdataString set to valid UUID URN (NOTE 1) |  |  | CONFIG AND PUBLISH |
|  | not present or encrypted (NOTE 2) <mcdata-client-id> with mcdataString set to valid UUID URN (NOTE 1) | in general mcdata-client-id is not mandatory (e.g. for SIP SUBSCRIBE) |  | (CONFIG OR GROUPCONFIG) AND NOT REGISTER (NOTE 3) |
|  | Encrypted (NOTE 2) <mcdata-client-id> with mcdataString set to valid UUID URN (NOTE 1) | mcdata-client-id is mandatory in SIP PUBLISH for MCData service settings only, according to TS 24.282 [87] clause 7.2.3 | RFC 4122 [106] | POC-SETTINGS-EVENT |
| mcdata-controller-psi | not present |  |  |  |
| anyExt | not present |  |  |  |
| anyExt |  | anyExt shall not contain any further elements than listed below unless specified otherwise in the specifc message content of a test case |  | PRE_ESTABLISHED_SESSION OR FUNCTIONAL_ALIAS |
| pre-established-session-ind | “true” | TS 24.282 [87], Clause 18.3.2.1 |  | PRE_ESTABLISHED_SESSION |
| functional-alias-URI | encrypted (NOTE 2) <functional-alias-URI> with mcdataURI set to px_MCData_ID_FA_A | set to the value of the functional alias that is used together with the "mcdata-calling-user-id" |  | FUNCTIONAL_ALIAS |
| NOTE 1: The SS shall check the mcdata-client-id - at the first time being sent by the UE to be a valid UUID URN with a format like "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" according to RFC 4122 [106] - to be all the same UUID URN in subsequent messages. NOTE 2: Encrypted element as described in Table 5.5.3.2.1-3A NOTE 3: In contrast to MCPTT and MCVideo for MCData TS 24.282 [87] clause 7.2.1 does not specify the client-id to be included in the REGISTER request. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| MCD_1to1 | A one-to-one MCData call |
| MCD_grp | A goup MCData call |
| REGISTER | MCData-Info in SIP REGISTER request for service authorisation |
| PUBLISH | MCData-Info in SIP PUBLISH request for service authorisation |
| PRE_ESTABLISHED_SESSION | A pre-established sessions is being established |
| FUNCTIONAL_ALIAS | An active Functional Alias is used |
| For further conditions see table 5.5.1-1 |  |

Table 5.5.3.2.1-3A: Encrypted MCData info parameter sent by the UE

| Derivation Path: TS 24.282 [87] clauses D.1.2, D.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the mcdata parameter |  |  |  |