3GPP TSG-RAN5 Meeting #100	R5-235093

# Toulouse, France, 21st Aug 2023 – 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0554 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to IMS test case 7.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Align with the outcome of the Tdoc R5-225274 to update IMS TCs with NG.114 default EVS configuration A2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case has been updated to use the generic procedure A.5.2a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The aligment to use default EVS configuration cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.8	MTSI MT Voice Call without preconditions at originating UE and with preconditions at terminating UE / 5GS

7.8.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and configured to use preconditions }

ensure that {

when { UE receives INVITE for voice call without precondition option-tag in Require or Supported header }

then { UE completes setup of voice call without preconditions }

}

7.8.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229, clause 5.1.4.1]

If an initial INVITE request is received the terminating UE shall check whether the terminating UE requires local resource reservation.

NOTE 1:	The terminating UE can decide if local resource reservation is required based on e.g. application requirements, current access network capabilities, local configuration, etc.

During the session initiation, if local resource reservation is required at the terminating UE and the terminating UE supports the precondition mechanism, and:

a)	the received INVITE request includes the "precondition" option-tag in the Supported header field or Require header field and the precondition mechanism is enabled as specified in subclause 5.1.5A, the terminating UE shall use the precondition mechanism and shall include a Require header field with the "precondition" option-tag:

-	in responses to that INVITE request if those responses include an SDP body;

-	in responses to subsequent requests received in-dialog that include an SDP body and include "precondition" option-tag in Supported header field or Require header field; and

-	in subsequent requests that include an SDP body, that it sends towards the originating UE during the session initiation;

b)	the received INVITE request includes the "precondition" option-tag in the Supported header field, and the precondition mechanism is disabled as specified in subclause 5.1.5A, the terminating UE shall not use the precondition mechanism:

c)	the received INVITE request includes the "precondition" option-tag in the Require header field, and the precondition mechanism is disabled as specified in subclause 5.1.5A, the terminating UE shall reject the INVITE request with a 420 (Bad Extension) response; and

d)	the received INVITE request does not include the "precondition" option-tag in the Supported header field or Require header field, the terminating UE shall not use the precondition mechanism.

7.8.3	Test description

7.8.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	UE is configured to use preconditions.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1 [21]) and registered to IMS.

-	The UE has registered to IMS and set up an MO voice call, by executing the generic test procedure in Annex A.2 up to the last step and thereafter executing the generic test procedure in Annex A.4.1a. The SS then ends the MO voice call by sending BYE.

7.8.3.2	Test procedure sequence

Table 7.8.3.2-1: Main Behaviour

| St |  | Procedure |  | Message Sequence |  |  |  | TP |  | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | U - S |  | Message |  |  |  |  |  |
| 1-8 |  | Steps 1-8 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [21] are performed. |  | - |  | - |  | - |  | - |  |
| 9-13 |  | Steps 1-5 of Annex A.5.2a happens |  | - |  | - |  | - |  | - |  |
| 13A-13C |  | Steps 10-12 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [21] are performed. |  | - |  | - |  | - |  | - |  |
| 14-16 |  | Steps 6-8 of Annex A.5.2a happen. |  | - |  | - |  | - |  | - |  |
| 18 |  | Step 10 of Annex A.5.2a happens |  | <-- |  | ACK |  | - |  | - |  |

7.8.3.3	Specific message contents

None as fully described in annex A.5.2a.