3GPP TSG-RAN5 Meeting #100	R5-234476

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2854 | rev |  | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of DCI 1-0 DCI 1-1 and DCI 1-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The field “Identifier for DCI formats” in DCI Format 1_0 is 1bit - The field “VRB-to-PRB mapping” in DCI Format 1_1 is 0bit if interleaved VRB-to-PRB mapping is not configured by high layers. The default configure in 38.508-1 is that PDSCH-Config->vrb-ToPRB-Interleaver = Not present. - The field “VRB-to-PRB mapping” in DCI Format 1_2 is 0bit if the higher layer parameter vrb-ToPRB-InterleaverDCI-1-2 is not configured. The default configure in 38.508-1 is that PDSCH-Config-> vrb-ToPRB-InterleaverDCI-1-2 = Not present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the value of “Identifier for DCI formats” in DCI Format 1_0 to “1” - Correct the value of “VRB-to-PRB mapping” in DCI Format 1_1 to “Not present” - Correct the value of “VRB-to-PRB mapping” in DCI Format 1_2 to “Not present” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.3.6.1.2	Physical layer parameters for scheduling of PDSCH

4.3.6.1.2.1	Physical layer parameters for DCI format 1_0

DCI format 1_0 is used for the scheduling of PDSCH in one cell.

Default physical layer parameters for DCI format 1_0 are specified in table 4.3.6.1.2.1-1 to 4.3.6.1.2.1-4.

Table 4.3.6.1.2.1-1: Physical layer parameters for DCI format 1_0

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Identifier for DCI formats | Indicating a DL DCI format | “1” |
| Frequency domain resource assignment | Dependent on test parameters | - |
| Time domain resource assignment | Indicating the first entry of PDSCH-TimeDomainResourceAllocationList to be used | “0000” |
| VRB-to-PRB mapping | Non-interleaved | “0” |
| Modulation and coding scheme | Dependent on test parameters | - |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 38.321 | - |
| Redundancy version | Dependent on test parameters | - |
| HARQ process number | Dependent on test parameters | - |
| Downlink assignment index | Dependent on test parameters | - |
| TPC command for scheduled PUCCH | 0 dB (accumulated TPC) as per Table 7.2.1-1 in TS 38.213 | “01” |
| PUCCH resource indicator | PUCCH-ResourceId[7] = 6 in pucch-ResourceSetID[1] or PUCCH-ResourceId[7] = 14 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | “110” |
| PDSCH-to-HARQ_feedback timing indicator | K1 slots as specified in 9.2.3 in TS 38.213 |  |
|  | µ=0 (SCS=15kHz): K1=7 | '110'B |
|  | µ=1 (SCS=30kHz): K1=5 | '100'B |
|  | µ=3 (SCS=120kHz): K1=8 | '111'B |

<End of modified section 1>

<Start of modified section 2>

4.3.6.1.2.2	Physical layer parameters for DCI format 1_1

DCI format 1_1 is used for the scheduling of PDSCH in one cell.

Default physical layer parameters for DCI format 1_1 are specified in table 4.3.6.1.2.2-1.

Table 4.3.6.1.2.2-1: Physical layer parameters for DCI format 1_1

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Carrier indicator | Not present | - |  |
| Identifier for DCI formats | Indicating a DL DCI format | “1” |  |
| Bandwidth part indicator | Not present | - |  |
| Frequency domain resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | Indicating the first entry of PDSCH-TimeDomainResourceAllocationList to be used | “0000” |  |
| VRB-to-PRB mapping | Not present (0 bits, vrb-ToPRB-Interleaver = Not present as per Table 4.6.3-100) | - |  |
| PRB bundling size indicator | Not present (semi-static PRB_bundling) | - |  |
| Rate matching indicator | Not present | - |  |
| ZP CSI-RS trigger | Not present | - |  |
| Modulation and coding scheme (TB1) | Dependent on test parameters | - |  |
| New data indicator (TB1) | Set for every data transmission/retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version (TB1) | Dependent on test parameters | - |  |
| Modulation and coding scheme (TB2) | Dependent on test parameters | - |  |
| New data indicator (TB2) | Set for every data transmission/retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version (TB2) | Dependent on test parameters | - |  |
| HARQ process number | Dependent on test parameters | - |  |
| Downlink assignment index | Dependent on test parameters | - |  |
| TPC command for scheduled PUCCH | 0 dB (accumulated TPC) as per Table 7.2.1-1 in TS 38.213 [22] | “01” |  |
| PUCCH resource indicator | PUCCH-ResourceId[7] = 6 in pucch-ResourceSetID[1] or PUCCH-ResourceId[7] = 14 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213 [22]) | “110” |  |
| PDSCH-to-HARQ_feedback timing indicator | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 |  |  |
|  | µ=0 (SCS=15kHz): K1=7 | “101” |  |
|  | µ=1 (SCS=30kHz): K1=5 | “011” |  |
|  | µ=3 (SCS=120kHz): K1=8 | “110” |  |
|  | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 For 60KHz SCS K1 = 4 if mod(i,5) = 0 K1 = 3 if mod(i,5) = 1 K1 = 7 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,39} For 120KHz SCS K1 = 4 if mod(i,5) = 0 K1 =3 if mod(i,5) = 1 K1 =7 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,79} |  | RF_FR2_DL |
|  | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 For TDD K1 = 4 if mod(i,5) = 0 K1 = 3 if mod(i,5) = 1 K1 = 2 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,9} For FDD K1 = 2 |  | RF_FR1_15kHz |
|  | For TDD corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 K1 = 8 if mod(i,10) = 0 K1 = 7 if mod(i,10) = 1 K1 = 6 if mod(i,10) = 2 K1 = 5 if mod(i,10) = 3 K1 = 4 if mod(i,10) = 4 K1 = 3 if mod(i,10) = 5 K1 = 2 if mod(i,10) = 6 where i is slot index per frame; i = {0,…,19} For FDD K1 = 2 |  | RF_FR1_30kHz |
|  | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 For TDD K1 = 13 if mod(i,20) = 2 K1 = 12 if mod(i,20) = 3 K1 = 11 if mod(i,20) = 4 K1 = 10 if mod(i,20) = 5 K1 = 9 if mod(i,20) = 6 K1 = 8 if mod(i,20) = 7 K1 = 7 if mod(i,20) = 8 K1 = 6 if mod(i,20) = 9 K1 = 6 if mod(i,20) = 10 K1 = 6 if mod(i,20) = 11 K1 = 6 if mod(i,20) = 12 K1 = 6 if mod(i,20) = 13 where i is slot index per frame; i = {0,…,39} For FDD K1 = 2 |  | RF_FR1_60kHz |
| Antenna port(s) | DMRS port 0 as per Table 7.3.1.2.2-1 in TS 38.212 [27] (dmrs-Type = DMRS type 1 and maxLength = len1 as per Table 4.6.3-50) | “0000” |  |
|  |  | “0011” | RF |
| Transmission configuration indication | Not present (0 bits, tci-PresentInDCI = Not present as per Table 4.6.3-28) | - |  |
| SRS request | No aperiodic SRS resource set triggered as per Table 7.3.1.1.2-24 in TS 38.212 [27] (no SUL configured) | “00” |  |
| CBG transmission information | Not present | - |  |
| CBG flushing out information | Not present | - |  |
| DMRS sequence initialization | fix length of 1 bit; '0'B for DMRS-DownlinkConfig.scramblingID0 (or physCellId if scramblingID0 is not present); see Table 4.6.3-50 | “0” |  |

| Condition | Explanation |
| --- | --- |
| RF_FR1_15kHz | RF testing in TS 38.521-x for FR1. SCS is set to 15kHz. |
| RF_FR1_30kHz | RF testing in TS 38.521-x for FR1. SCS is set to 30kHz. |
| RF_FR1_60kHz | RF testing in TS 38.521-x for FR1. SCS is set to 60kHz. |
| RF_FR2_DL | RF testing in TS 38.521-x for FR2. SCS is set to 60kHz or 120kHz |
| RF | RF testing in TS 38.521-x for FR1 and FR2. |

4.3.6.1.2.3	Physical layer parameters for DCI format 1_2

DCI format 1_2 is used for the scheduling of PDSCH in one cell.

Default physical layer parameters for DCI format 1_2 are specified in table 4.3.6.1.2.3-1.

Table 4.3.6.1.2.3-1: Physical layer parameters for DCI format 1_2

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Identifier for DCI formats | Indicating a DL DCI format | “1” |  |
| Carrier indicator | Not present |  |  |
| Bandwidth part indicator | Not present | - |  |
| Frequency domain resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | Indicating the first entry of PDSCH-TimeDomainResourceAllocationList to be used | “0000” |  |
| VRB-to-PRB mapping | Not present (0 bits, vrb-ToPRB-InterleaverDCI-1-2= Not present as per Table 4.6.3-100) | - |  |
| PRB bundling size indicator | Not present (semi-static PRB_bundling) | - |  |
| Rate matching indicator | Not present | - |  |
| ZP CSI-RS trigger | Not present | - |  |
| Modulation and coding scheme | Dependent on test parameters | - |  |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version | Dependent on test parameters | - |  |
| HARQ process number | Dependent on test parameters | - |  |
| Downlink assignment index | Dependent on test parameters | - |  |
| TPC command for scheduled PUCCH | 0 dB (accumulated TPC) as per Table 7.2.1-1 in TS 38.213 [22] | “01” |  |
| PUCCH resource indicator | PUCCH-ResourceId[7] = 6 in pucch-ResourceSetID[1] or PUCCH-ResourceId[7] = 14 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213 [22]) | “110” |  |
| PDSCH-to-HARQ_feedback timing indicator | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 |  |  |
|  | µ=0 (SCS=15kHz): K1=7 | “101” |  |
|  | µ=1 (SCS=30kHz): K1=5 | “011” |  |
|  | µ=3 (SCS=120kHz): K1=8 | “110” |  |
| Antenna port(s) | Not present (0 bit if higher layer parameter antennaPortsFieldPresenceDCI-1-2 is not configured) | - |  |
| Transmission configuration indication | Not present (0 bits, tci-PresentInDCI = Not present as per Table 4.6.3-28) | - |  |
| SRS request | Not present (0 bit if higher layer parameter srs-RequestDCI-1-2 is not configured) | - |  |
| DMRS sequence initialization | Not present (0 bit if higher layer parameter dmrs-SequenceInitializationDCI-1-2 is not configured) | - |  |
| Priority indicator | Not present (0 bit if higher layer parameter priorityIndicatorDCI-1-2 is not configured) | - |  |

<End of modified section 2>