3GPP TSG-RAN5 Meeting #100	R5-235084

# Toulouse, France, 21st Aug 2023 – 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2911 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on Dedicated coreset configuration for RF and RRM testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In some RF and RRM scenarios there is a full overlap between Coreset 0 (SS1) and Coreset 1 (SS2) that makes same frequency domain, DCI length, AL. Additionally, Coreset 0 and Coreset 1 uses same scrambling as pdcch-DMRS-ScramblingID is not present in both coresets configuration. This scenario makes potentially that UEs are not able to distinguish the DCI0-0 and DCI0-1. This scenario is considered invalid in following RAN1 agreement: From TS 38.213 section 10.1 v15.15.0: Hence, UE does not expect a NW configuration where there is a full overlap between Coreset 0 (SS1) and Coreset 1 (SS2) that makes same frequency domain, DCI length, AL and identical scrambling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to configure a PDCCH DMRS Scrambling ID for dedicated coreset in RF and RRM to avoid first PDCCH candidate and secod PDCCH candidate would have identical scrambling. PDCCH DMRS SCID proposed value is 1008 as a value that will not match with any Physical Cell ID. Physical Cell ID range defined in TS 38.331 is from 0 to 1007. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance UE will fail RF and RRM test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1, 5.4.2, 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

PDCCH-config

Table 5.4.1-16: PDCCH ControlResourceSet

| Derivation Path: Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP | RFACS AND AL8 |
|  | 11111111 11111111 00000000 00000000 00000000 00000 | CORESET to use the least significant 96 RBs of the BWP | RFACS AND AL16 |
| pdcch-DMRS-ScramblingID | 1008 | DMRS scrambling ID different from Physical Cell ID | SA AND FR1 |

| Condition | Explanation |
| --- | --- |
| RFACS | Used in RF Adjacent Channel Selectivity testing |
| AL8 | PDCCH aggregationLevel 8 is configured |
| AL16 | PDCCH aggregationLevel 16 is configured |
| SA | NR standalone RF tests cases |

Table 5.4.1-17: PDCCH Search Space

| Derivation Path: Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n2 |  | RFACS |
| aggregationLevel8 | n2 |  | RFACS |
| aggregationLevel16 | n2 |  | RFACS |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RFACS | Used in RF Adjacent Channel Selectivity testing |

## <<< Skip unchanged sections >>>

PDCCH-config

Table 5.4.2.0-6: PDCCH-ControlResourceSet

| Derivation Path: Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | Table 5.2-2 for tested channel bandwidth and subcarrier spacing |  | FR1 |
| frequencyDomainResources | Table 7.2-2 for tested channel bandwidth and subcarrier spacing |  | FR2 |
| Duration | 2 | SearchSpace duration of 2 symbols | DEMOD_FR1, CSI_FR1, CSI_FR2 |
|  | 1 | SearchSpace duration of 1 symbols | DEMOD_FR2, PDCCH_FR1, PDCCH_FR2 |
| cce-REG-MappingType CHOICE { |  |  |  |
| nonInterleaved | Null |  |  |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | 0 | entry 1 TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| pdcch-DMRS-ScramblingID | 1008 | DMRS scrambling ID different from Physical Cell ID | SA AND FR1 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DEMOD_FR1 | DEMOD testing in FR1. |
| DEMOD_FR2 | DEMOD testing in FR2. |
| PDCCH_FR1 | PDCCH testing in FR1 |
| PDCCH_FR2 | PDCCH testing in FR2 |
| CSI_FR1 | CSI testing in FR1 |
| CSI_FR2 | CSI testing in FR2 |
| SA | NR standalone Demod, PDCCH, SDR and CSI tests cases |

Table 5.4.2.0-7: PDCCH Search Space

| Derivation Path: Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Starting symbol 0 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  |  |
| ue-Specific SEQUENCE { |  |  |  |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## <<< Skip unchanged sections >>>

ControlResourceSet

Table 7.3.1-15: ControlResourceSet

| Derivation Path: Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 |  | CCR.3.7 OR SCS240 |
| duration | 1 |  | CCR.3.x |
| cce-REG-MappingType CHOICE { |  |  |  |
| interleaved ::= SEQUENCE { |  |  | CCR.X.Y |
| reg-BundleSize | n6 |  |  |
| interleaverSize | n2 |  |  |
| shiftIndex | 0 |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList | Not present |  |  |
| tci-StatesPDCCH-ToAddList SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | TCI-StateId-RRM(2) | TCI State #2, QCLed to TRS resource #4 in the first resource set entry 1 | TRS |
| TCI-StateId[1] | TCI-StateId-RRM(3) | TCI State #3, QCLed to TRS resource #4 in the second resource set entry 1 | SECOND_SET |
| } |  |  |  |
| pdcch-DMRS-ScramblingID | 1008 | DMRS scrambling ID different from Physical Cell ID | SA AND FR1 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CCR.x.y | Refers to CCR.x.y as defined in A.1.3 of TS 38.533 [18] |
| TRS | When at least one TRS resource set is configured. |
| SECOND_SET | Two resource sets for TRS are configured in test, only applies to FR2. |
| SA | NR standalone RRM tests cases |

-	SchedulingRequestResourceConfig

Table 7.3.1-16: SchedulingRequestResourceConfig

| Derivation Path: Table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl10 | 7 | With SCS = kHz15 results in repetition every 10 ms | SCS_15kHz |
| sl20 | 7 | With SCS = kHz30 results in repetition every 10 ms | SCS_30kHz |
| } |  |  |  |
| } |  |  |  |

## <<< END OF CHANGES >>>