3GPP TSG-RAN5 Meeting #100	R5-235749

# Toulouse, France, 21st Aug 2023 – 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0958 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2c MU - Rx test cases update in 38.521-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In R5-233971, progress has been made for MU and TT analysis in FR2c for certain Rx tests. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defined MU and TT for REFSENS and EIS Spherical coverage. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete for FR2c. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2, 7.3.4, F.1.3, F.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: -Added Anritsu as co-source company. -Set in [] the new MU and TT defined for FR2c. Reverted changes in editor’s notes since MTSU analysis is not concluded yet. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### 7.3.2	Reference sensitivity power level

Editor’s note: The following aspects of the clause are for future consideration:

- Measurement Uncertainties and Test Tolerances are FFS for power class 2, 4 and 7.

- The test case is incomplete for band n259 and for band n262.

The following aspects of the clause are for future consideration:

- The 3D EIS scan test time optimization in RAN 4/ RAN 5 is FFS (existing EIS based test time needs to be re-evaluated for 200/266 grid points).

- Statistical model in Annex H.2 (currently based on LTE model) needs to be validated to confirm that it is also applicable for FR2

7.3.2.1	Test purpose

To verify the UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area of an g-NodeB.

7.3.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

7.3.2.3	Minimum conformance requirements

The reference sensitivity power level REFSENS is defined as the EIS level at the centre of the quiet zone in the RX beam peak direction, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

###### 7.3.2.3.1	Reference sensitivity power level for power class 1

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 7.3.2.3.1-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

Table 7.3.2.3.1-1: Reference sensitivity for power class 1

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| n257 | -97.5 | -94.5 | -91.5 | -88.5 | N/A | N/A | N/A |
| n258 | -97.5 | -94.5 | -91.5 | -88.5 | N/A | N/A | N/A |
| n260 | -94.5 | -91.5 | -88.5 | -85.5 | N/A | N/A | N/A |
| n261 | -97.5 | -94.5 | -91.5 | -88.5 | N/A | N/A | N/A |
| n262 | -92.5 | -89.5 | -86.5 | -83.5 | N/A | N/A | N/A |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 |  |  |  |  |  |  |  |

The REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3.1-2.

Table 7.3.2.3.1-2: Uplink configuration for reference sensitivity

| Operating band | NR Band / Channel bandwidth / NRB / SCS / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz | SCS | Duplex Mode |
| n257 | 32 | 64 | 128 | 256 | N/A | N/A | N/A | 120 kHz | TDD |
| n258 | 32 | 64 | 128 | 256 | N/A | N/A | N/A | 120 kHz | TDD |
| n260 | 32 | 64 | 128 | 256 | N/A | N/A | N/A | 120 kHz | TDD |
| n261 | 32 | 64 | 128 | 256 | N/A | N/A | N/A | 120 kHz | TDD |
| n262 | 32 | 64 | 128 | 256 | N/A | N/A | N/A | 120 kHz | TDD |

Unless given by Table 7.3.2.3.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3.3.1-1) configured.

Table 7.3.2.3.1-3: Reserved

| Operating band | Network Signalling value |
| --- | --- |

###### 7.3.2.3.2	Reference sensitivity power level for power class 2

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 7.3.2.3.2-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

Table 7.3.2.3.2-1: Reference sensitivity for power class 2

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| n257 | -92 | -89 | -86 | -83 | N.A | N.A | N.A |
| n258 | -92 | -89 | -86 | -83 | N.A | N.A | N.A |
| n261 | -92 | -89 | -86 | -83 | N.A | N.A | N.A |
| n262 | -86.8 | -83.8 | -80.8 | -77.8 | N.A | N.A | N.A |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 |  |  |  |  |  |  |  |

The REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3.1-2.

Unless given by Table 7.3.2.3.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3.3.1-1) configured.

###### 7.3.2.3.3	Reference sensitivity power level for power class 3

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 7.3.2.3.3-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

For the power class 3 UEs that support multiple FR2 bands, the minimum requirement for Reference sensitivity in Table 7.3.2.3.3-1 shall be increased per band, respectively, by the reference sensitivity relaxation parameter ∑MBP and ∆MBP,n as specified in Table 7.3.2.3.3-1a and 7.3.2.3.3-1b.

Table 7.3.2.3.3-1: Reference sensitivity for power class 3

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| n257 | -88.3 | -85.3 | -82.3 | -79.3 | N.A | N.A | N.A |
| n258 | -88.3 | -85.3 | -82.3 | -79.3 | N.A | N.A | N.A |
| n259 | -84.7 | -81.7 | -78.7 | -75.7 | N.A | N.A | N.A |
| n260 | -85.7 | -82.7 | -79.7 | -76.7 | N.A | N.A | N.A |
| n261 | -88.3 | -85.3 | -82.3 | -79.3 | N.A | N.A | N.A |
| n262 | -82.8 | -79.8 | -76.8 | -73.8 | N.A | N.A | N.A |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 |  |  |  |  |  |  |  |

Table 7.3.2.3.3-1a: UE multi-band relaxation factors for power class 3 (Rel-15)

| Supported bands | ∑MBP (dB) | ∑MBS (dB) |
| --- | --- | --- |
| n257, n258 | ≤ 1.3 | ≤ 1.25 |
| n257, n260 | ≤ 1.0 | ≤ 0.753 |
| n258, n260 | ≤ 1.0 | ≤ 0.753 |
| n258, n261 | ≤ 1.0 | ≤ 1.25 |
| n260, n261 | 0.0 | ≤ 0.752 |
| n257, n258, n260 | ≤ 1.7 | ≤ 1.753 |
| n257, n258, n261 | ≤ 1.7 | ≤ 1.75 |
| n257, n260, n261 | ≤ 0.5 | ≤ 1.253 |
| n258, n260, n261 | ≤ 1.5 | ≤ 1.253 |
| n257, n258, n260, n261 | ≤ 1.7 | ≤ 1.753 |
| NOTE 1: The requirements in this table are applicable to UEs which support only the indicated bands NOTE 2: For supported bands n260 + n261, ΔMBS,n is not applied for band n260 NOTE 3: For n260, maximum applicable MBS,n is 0.4 dB and MBP,n is 0.75 dB NOTE 4: For all bands except n260, the maximum applicable MBP,n and MBS,n is 0.75 dB |  |  |

Table 7.3.2.3.3-1b: UE multi-band relaxation factors for power class 3 (Rel-16 and forward)

| Band | MBP,n (dB) | MBS,n (dB) |
| --- | --- | --- |
| n257 | 0.73 | 0.73 |
| n258 | 0.6 | 0.7 |
| n259 | 0.5 | 0.4 |
| n260 | 0.51 | 0.41 |
| n261 | 0.52,4 | 0.74 |
| NOTE 1: n260 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n260 NOTE 2: n261 peak relaxation is 0 dB for UE that exclusively supports n261+n260 NOTE 3: n257 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257 NOTE 4: n261 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257 |  |  |

The REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3.1-2.

Unless given by Table 7.3.2.3.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3.3.1-1) configured.

###### 7.3.2.3.4	Reference sensitivity power level for power class 4

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 7.3.2.3.4-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

Table 7.3.2.3.4-1: Reference sensitivity for power class 4

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -97.0 | -94.0 | -91.0 | -88.0 |
| n258 | -97.0 | -94.0 | -91.0 | -88.0 |
| n260 | -95.0 | -92.0 | -89.0 | -86.0 |
| n261 | -97.0 | -94.0 | -91.0 | -88.0 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 |  |  |  |  |

The REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3.1-2.

Unless given by Table 7.3.2.3.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3.3.1-1) configured.

The normative reference for this requirement is TS 38.101-2 [3] clause 7.3.2.

###### 7.3.2.3.5	Reference sensitivity power level for power class 5

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 7.3.2.3.4-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

Table 7.3.2.3.5-1: Reference sensitivity for power class 5

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -92.6 | -89.6 | -86.6 | -83.6 |
| n258 | -92.8 | -89.8 | -86.8 | -83.8 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 |  |  |  |  |

The REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3.1-2.

Unless given by Table 7.3.2.3.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3.3.1-1) configured.

The normative reference for this requirement is TS 38.101-2 [3] clause 7.3.2.

###### 7.3.2.3.6

###### 7.3.2.3.7	Reference sensitivity power level for power class 7

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 7.3.2.3.7-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

Table 7.3.2.3.7-1: Reference sensitivity for power class 7

| Operating band | REFSENS (dBm) / Channel bandwidth |  |
| --- | --- | --- |
|  | 50 MHz | 100 MHz |
| n257 | -85.3 | -82.3 |
| n258 | -85.3 | -82.3 |
| n261 | -85.3 | -82.3 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |

The REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3.1-2.

Unless given by Table 7.3.2.3.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3.3.1-1) configured.

The normative reference for this requirement is TS 38.101-2 [3] clause 7.3.2.

7.3.2.4	Test description

7.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and subcarrier spacing are shown in Table 7.3.2.4.1-1, Table 7.3.2.4.1-2, and Table 7.3.2.4.1-3 The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | Lowest, 100MHz, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB allocation | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |

Table 7.3.2.4.1-2: Downlink Configuration of each RB allocation

| Channel Bandwidth | SCS kHz | LCRBmax | RB allocation (LCRB@RBstart) |
| --- | --- | --- | --- |
| 50MHz | 120 | 32 | 32@0 |
| 100MHz | 120 | 66 | 66@0 |
| 200MHz | 120 | 132 | 132@0 |
| 400MHz | 120 | 264 | 264@0 |
| NOTE 1: Test Channel Bandwidths are checked separately for each NR band, the applicable channel bandwidths are specified in Table 5.3.5-1. NOTE 2: The 200MHz and 400MHz bandwidths are not applicable to PC7 RedCap UEs |  |  |  |

Table 7.3.2.4.1-3: Uplink configuration for reference sensitivity, LCRB@RBstart format

| Operating Band | SCS kHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 120 | 32@0 | 64@0 | 128@0 | 256@0 | TDD |
| n258 | 120 | 32@0 | 64@0 | 128@0 | 256@0 | TDD |
| n259 | 120 | 32@0 | 64@0 | 128@0 | 256@0 | TDD |
| n260 | 120 | 32@0 | 64@0 | 128@0 | 256@0 | TDD |
| n261 | 120 | 32@0 | 64@0 | 128@0 | 256@0 | TDD |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The DL and UL Reference Measurement channels are set according to Table 7.3.2.4.1-1, Table 7.3.2.4.1-2, and Table 7.3.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 7.3.2.4.3.

7.3.2.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3.2.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 7.3.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200msec for the UE to reach PUMAX.

4.	Set the UE in the Rx beam peak direction found with a 3D EIS scan as performed in Annex K.1.2. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Rx beam selection to complete.

5.	Perform EIS procedure as stated in Annex K.1.4 to calculate “averaged EIS”. At each power level, by changing the power level of the wanted signal with a step size of 0.2dB (coarse and fine searches are not precluded as long as the fine search is using the 0.2dB step size near the sensitivity level). For each power step measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2. The downlink power step size shall be no more than 0.2 dB when the RF power level is near the sensitivity level.

6.	Compare the dB value of the “averaged EIS” value corresponding to the Rx beam peak direction identified in step 5 to the test requirement in Table 7.3.2.5-1 to Table 7.3.2.5-4. If the EIS value is lower or equal to the value in Table 7.3.2.5-1 to Table 7.3.2.5-4, pass the UE. Otherwise fail the UE.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.2.

7.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

7.3.2.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Tables 7.3.2.5-1 to 7.3.2.5-4. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

Table 7.3.2.5-1: Reference sensitivity for power class 1

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -97.5+TT | -94.5+TT | -91.5+TT | -88.5+TT |
| n258 | -97.5+TT | -94.5+TT | -91.5+TT | -88.5+TT |
| n260 | -94.5+TT | -91.5+TT | -88.5+TT | -85.5+TT |
| n261 | -97.5+TT | -94.5+TT | -91.5+TT | -88.5+TT |

Table 7.3.2.5-1a: Test Tolerance (Reference sensitivity for power class 1)

| Test Metric | FR2a, FR2b |
| --- | --- |
| IFF (Max device size ≤ 30 cm) | 2.51 dB , NTC 2.62 dB , ETC |

Table 7.3.2.5-2: Reference sensitivity for power class 2

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -92+TT | -89+TT | -86+TT | -83+TT |
| n258 | -92+TT | -89+TT | -86+TT | -83+TT |
| n261 | -92+TT | -89+TT | -86+TT | -83+TT |

Table 7.3.2.5-3: Reference sensitivity for power class 3 for single band UE or multi-band UE declaring MBp = 0 in all FR2 bands

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -88.3+TT | -85.3+TT | -82.3+TT | -79.3+TT |
| n258 | -88.3+TT | -85.3+TT | -82.3+TT | -79.3+TT |
| n259 | -84.7+TT | -81.7+TT | -78.7+TT | -75.7+TT |
| n260 | -85.7+TT | -82.7+TT | -79.7+TT | -76.7+TT |
| n261 | -88.3+TT | -85.3+TT | -82.3+TT | -79.3+TT |

Table 7.3.2.5-3a: Reference sensitivity for power class 3 for multi-band UE declaring MBp > 0 in any FR2 band (Rel-15)

| Operating band | REFSENS (dBm) / Channel bandwidth (NOTE 1) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -88.3+TT+MBp | -85.3+TT+MBp | -82.3+TT+MBp | -79.3+TT+MBp |
| n258 | -88.3+TT+MBp | -85.3+TT+MBp | -82.3+TT+MBp | -79.3+TT+MBp |
| n260 | -85.7+TT+MBp | -82.7+TT+MBp | -79.7+TT+MBp | -76.7+TT+MBp |
| n261 | -88.3+TT+MBp | -85.3+TT+MBp | -82.3+TT+MBp | -79.3+TT+MBp |
| NOTE 1: Refer Table 7.3.2.5-3b for details for MBp allowance corresponding to supported FR2 bands set NOTE 2: For a Rel-15 UE supporting FR2 bands set not defined in Table 7.3.2.3.3-1a, Table 7.3.2.5-3c applies. |  |  |  |  |

Table 7.3.2.5-3b: Reference sensitivity multi-band relaxation factors for power class 3 (Rel-15)

| ID | Supported FR2 bands set | Maximum sum of MBP, ∑MBP (dB) (Note 3) | Comments |
| --- | --- | --- | --- |
| 1 | n257, n258 | 1.3 | Maximum 0.75 dB relaxation allowed for each band |
| 2 | n257, n260 | 1.0 | Maximum 0.75 dB relaxation allowed for each band |
| 3 | n258, n260 | 1.0 | Maximum 0.75 dB relaxation allowed for each band |
| 4 | n258, n261 | 1.0 | Maximum 0.75 dB relaxation allowed for each band |
| 5 | n260, n261 | 0.0 | No relaxation factor allowed |
| 6 | n257, n258, n260 | 1.7 | Maximum 0.75 dB relaxation allowed for each band |
| 7 | n257, n258, n261 | 1.7 | Maximum 0.75 dB relaxation allowed for each band |
| 8 | n257, n260, n261 | 0.5 | Maximum 0.75 dB relaxation allowed for each band |
| 9 | n258, n260, n261 | 1.5 | Maximum 0.75 dB relaxation allowed for each band |
| 10 | n257, n258, n260, n261 | 1.7 | Maximum 0.75 dB relaxation allowed for each band |
| NOTE 1: MBp is the Multiband Relaxation factor declared by the UE for the tested band in table A.4.3.9-2 of TS38.508-2. This declaration shall fulfil the requirements in Table 7.3.2.3.3-1a. NOTE 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant NOTE 3: Max allowed sum of MBp over all supported FR2 bands as defined in clause 7.3.2.3.3. |  |  |  |

Table 7.3.2.5-3c: Reference sensitivity for power class 3 (Rel-16 and forward)

| Operating band | REFSENS (dBm) / Channel bandwidth (NOTE 1) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -88.3+TT+MBP,n | -85.3+TT+MBP,n | -82.3+TT+MBP,n | -79.3+TT+MBP,n |
| n258 | -88.3+TT+MBP,n | -85.3+TT+MBP,n | -82.3+TT+MBP,n | -79.3+TT+MBP,n |
| n259 | -84.7+TT+MBP,n | -81.7+TT+MBP,n | -78.7+TT+MBP,n | -75.7+TT+MBP,n |
| n260 | -85.7+TT+MBP,n | -82.7+TT+MBP,n | -79.7+TT+MBP,n | -76.7+TT+MBP,n |
| n261 | -88.3+TT+MBP,n | -85.3+TT+MBP,n | -82.3+TT+MBP,n | -79.3+TT+MBP,n |
| NOTE 1: Refer Table 7.3.2.5-3d for details for MBP,n allowance corresponding to supported FR2 bands set |  |  |  |  |

Table 7.3.2.5-3d: Reference sensitivity multi-band relaxation factors for power class 3 (Rel-16 and forward)

| ID | FR2 bands/set | MBP,n (dB) | Comments |
| --- | --- | --- | --- |
| 1 | n257 | 0.7 |  |
| 2 | n258 | 0.6 |  |
| 3 | n259 | 0.5 |  |
| 4 | n260 | 0.5 |  |
| 5 | n261 | 0.5 |  |
| 6 | n257, n261 | 0 | MBP,n relaxation is 0 dB |
| 7 | n260, n261 | 0 | MBP,n relaxation is 0 dB |
| NOTE 1: MBP,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 7.3.2.3.3-1b. |  |  |  |

Table 7.3.2.5-3e: Test Tolerance (Reference sensitivity for power class 3)

| Test Metric | FR2a, FR2b | FR2c |
| --- | --- | --- |
| IFF (Max device size ≤ 30 cm) | 2.41 dB, NTC 2.52 dB, ETC | [2.68] dB, NTC TBD dB, ETC |

Table 7.3.2.5-4: Reference sensitivity for power class 4

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -97+TT | -94+TT | -91+TT | -88+TT |
| n258 | -97+TT | -94+TT | -91+TT | -88+TT |
| n260 | -95+TT | -92+TT | -89+TT | -86+TT |
| n261 | -97+TT | -94+TT | -91+TT | -88+TT |

Table 7.3.2.5-5: Reference sensitivity for power class 5

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -92.6+TT | -89.6+TT | -86.6+TT | -83.6+TT |
| n258 | -92.8+TT | -89.8+TT | -86.8+TT | -83.8+TT |

Table 7.3.2.5-5a: Test Tolerance (Reference sensitivity for power class 5)

| Test Metric | FR2a |
| --- | --- |
| IFF (Max device size ≤ 30 cm) | 2.51 dB , NTC 2.62 dB , ETC |

Table 7.3.2.5-6: FFS

Table 7.3.2.5-7: Reference sensitivity for power class 7

| Operating band | REFSENS (dBm) / Channel bandwidth |  |
| --- | --- | --- |
|  | 50 MHz | 100 MHz |
| n257 | -85.3+TT | -82.3+TT |
| n258 | -85.3+TT | -82.3+TT |
| n261 | -85.3+TT | -82.3+TT |

Table 7.3.2.5-7a: Test Tolerance (Reference sensitivity for power class 7)

| Test Metric | FR2a |
| --- | --- |
| IFF (Max device size ≤ 30 cm) | [TBD], NTC [TBD], ETC |

### 7.3.4	EIS spherical coverage

Editor’s Note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for power class 2, 4 and 7.

- The test case is incomplete for band n259 and band n262.

7.3.4.1	Test purpose

To verify that the EIS spherical coverage of the UE receiver is acceptable under conditions of low signal level, ideal propagation and no added noise.

7.3.4.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

7.3.4.3	Minimum conformance requirements

The reference sensitivity power level REFSENS at a single grid point of the spherical grid is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The reference measurement channels and throughput criterion shall be as specified in section 7.3.2.3.

For power class 1, the maximum EIS at the 85th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.3-1 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

Table 7.3.4.3-1: EIS spherical coverage for power class 1

| Operating band | EIS at 85th%ile CCDF (dBm) / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| n257 | -89.5 | -86.5 | -83.5 | -80.5 | N/A | N/A | N/A |
| n258 | -89.5 | -86.5 | -83.5 | -80.5 | N/A | N/A | N/A |
| n260 | -86.5 | -83.5 | -80.5 | -77.5 | N/A | N/A | N/A |
| n261 | -89.5 | -86.5 | -83.5 | -80.5 | N/A | N/A | N/A |
| n262 | -84.3 | -81.3 | -78.3 | -75.3 | N/A | N/A | N/A |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |  |  |  |

For power class 2, the maximum EIS at the 60th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.3-2 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

Table 7.3.4.3-2: EIS spherical coverage for power class 2

| Operating band | EIS at 60th%ile CCDF (dBm) / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| n257 | -81 | -78 | -75 | -72 | N.A | N.A | N.A |
| n258 | -81 | -78 | -75 | -72 | N.A | N.A | N.A |
| n261 | -81 | -78 | -75 | -72 | N.A | N.A | N.A |
| n262 | -74.9 | -71.9 | -68.9 | -65.9 | N.A | N.A | N.A |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |  |  |  |

For power class 3, the maximum EIS at the 50th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.3-3 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

For power class 3, the UEs that support operation in multiple FR2 bands, the minimum requirement for EIS spherical coverage in Table 7.3.4.3-3 shall be increased per band, respectively, by the reference sensitivity relaxation parameter ∑MBS and ∆MBS,n as specified in Table 7.3.2.3.3-1a and 7.3.2.3.3-1b..

Table 7.3.4.3-3: EIS spherical coverage for power class 3

| Operating band | EIS at 50th%ile CCDF (dBm) / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| n257 | -77.4 | -74.4 | -71.4 | -68.4 | N.A | N.A | N.A |
| n258 | -77.4 | -74.4 | -71.4 | -68.4 | N.A | N.A | N.A |
| n259 | -71.9 | -68.9 | -65.9 | -62.9 | N.A | N.A | N.A |
| n260 | -73.1 | -70.1 | -67.1 | -64.1 | N.A | N.A | N.A |
| n261 | -77.4 | -74.4 | -71.4 | -68.4 | N.A | N.A | N.A |
| n262 | -69.7 | -66.7 | -63.7 | -60.7 | N.A | N.A | N.A |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |  |  |  |

For power class 4, the maximum EIS at the 20th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.3-4 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

Table 7.3.4.3-4: EIS spherical coverage for power class 4

| Operating band | EIS at 20th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -88.0 | -85.0 | -82.0 | -79.0 |
| n258 | -88.0 | -85.0 | -82.0 | -79.0 |
| n260 | -83.0 | -80.0 | -77.0 | -74.0 |
| n261 | -88.0 | -85.0 | -82.0 | -79.0 |
| n262 | -78.9 | -75.9 | -72.9 | -69.9 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |

For power class 5, the maximum EIS at the 85th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.3-5 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

Table 7.3.4.3-5: EIS spherical coverage for power class 5

| Operating band | EIS at 85th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -84.6 | -81.6 | -78.6 | -75.6 |
| n258 | -84.8 | -81.8 | -78.8 | -75.8 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |

Table 7.3.4.3-6: EIS spherical coverage for power class 6

FFS

Table 7.3.4.3-7: EIS spherical coverage for power class 7

| Operating band | EIS at 50th %-tile CCDF (dBm) / Channel bandwidth |  |
| --- | --- | --- |
|  | 50 MHz | 100 MHz |
| n257 | -74.4 | -71.4 |
| n258 | -74.4 | -71.4 |
| n261 | -74.4 | -71.4 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in Annex E.2.1. |  |  |

The REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.4.3-9.

Table 7.3.4.3-8: Uplink configuration for reference sensitivity

| NR Band / Channel bandwidth / NRB / SCS / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR Band | 50 MHz | 100 MHz | 200 MHz | 400 MHz | SCS | Duplex Mode |
| n257 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n258 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n260 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n261 | 32 | 64 | 128 | 256 | 120 kHz | TDD |

Unless given by Table 7.3.4.3-7, the minimum requirements specified in Table 7.3.4.3-1, Table 7.3.4.3-2, Table 7.3.4.3-3, Table 7.3.4.3-4, Table 7.3.4.3-5 and Table 7.3.4.3-7 shall be verified with the network signalling value NS_200 configured.

Table 7.3.4.3-9: Network Signalling value for reference sensitivity

| NR Band | Network Signalling value |
| --- | --- |
| n258 | NS_201 |

For the UE which supports inter-band carrier aggregation, the minimum requirement for reference sensitivity in Table 7.3.4.3-1, Table 7.3.4.3-2, Table 7.3.4.3-3, Table 7.3.4.3-4 and Table 7.3.4.3-7 shall be increased by the amount given in ΔRIB,P,n defined in subclause 7.3A.2.0.3 for the applicable operating bands.

The normative reference for this requirement is TS 38.101-2 [3] clause 7.3.4.

7.3.4.4	Test description

7.3.4.4.1	Initial conditions

Same initial conditions as in clause 7.3.2.4.1 except that only normal condition is tested.

7.3.4.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3.2.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 7.3.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200msec for the UE to reach PUMAX.

4.	Measure UE EIS value for each grid point according to EIS spherical coverage procedure defined in Annex K.1.6.0, and obtain a Complimentary Cumulative Distribution Function (CCDF) of all EIS dBm values. Alternatively, UE EIS measurement for each grid point could be done according to Rx Fast spherical coverage procedure defined in Annex K.1.6.1. After a rotation, allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for UE to find the best beam to use. EIS is calculated considering both polarizations, theta and phi.

5.	Identify the EIS dBm value corresponding to %-tile (UE power class dependent) value in the applicable test requirement table in section 7.3.4.5.

6.	Compare the EIS dBm value identified in step 5, to the limit value in the applicable test requirement table in section 7.3.4.5. If the EIS dBm value is lower or equal to the limit value, pass the UE. Otherwise fail the UE.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.2.

7.3.4.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

7.3.4.5	Test requirement

The reference measurement channels and throughput criterion shall be as specified in section 7.3.2.5.

Table 7.3.4.5-1: EIS spherical coverage for power class 1

| Operating band | EIS at 85th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -89.5 +TT | -86.5 +TT | -83.5 +TT | -80.5 +TT |
| n258 | -89.5 +TT | -86.5 +TT | -83.5 +TT | -80.5 +TT |
| n260 | -86.5 +TT | -83.5 +TT | -80.5 +TT | -77.5 +TT |
| n261 | -89.5 +TT | -86.5 +TT | -83.5 +TT | -80.5 +TT |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |

Table 7.3.4.5-1a: Test Tolerance (Reference sensitivity for power class 1)

| Test Metric | f ≤ 40.8 GHz |
| --- | --- |
| IFF (Max device size ≤ 30 cm) | 2.28 dB |

Table 7.3.4.5-2: EIS spherical coverage for power class 2

| Operating band | EIS at 60th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -81 +TT | -78 +TT | -75 +TT | -72 +TT |
| n258 | -81 +TT | -78 +TT | -75 +TT | -72+TT |
| n261 | -81 +TT | -78 +TT | -75 +TT | -72 +TT |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |

Table 7.3.4.5-3: EIS spherical coverage for power class 3 for single band UE or multi-band UE declaring MBs = 0 in all FR2 bands

| Operating band | EIS at 50th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -77.4 +TT | -74.4 +TT | -71.4 +TT | -68.4 +TT |
| n259 | -71.9 +TT | -68.9 +TT | -65.9 +TT | -62.9 +TT |
| n258 | -77.4 +TT | -74.4 +TT | -71.4 +TT | -68.4 +TT |
| n260 | -73.1 +TT | -70.1 +TT | -67.1 +TT | -64.1 +TT |
| n261 | -77.4 +TT | -74.4 +TT | -71.4 +TT | -68.4 +TT |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |

Table 7.3.4.5-3a: EIS spherical coverage for power class 3 for multi-band UE declaring MBs > 0 in any FR2 band (Rel-15)

| Operating band | EIS at 50th%ile CCDF (dBm) / Channel bandwidth (NOTE 3) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -77.4 +TT+MBs | -74.4 +TT+MBs | -71.4 +TT+MBs | -68.4 +TT+MBs |
| n258 | -77.4 +TT+MBs | -74.4 +TT+MBs | -71.4 +TT+MBs | -68.4 +TT+MBs |
| n260 | -73.1 +TT+MBs | -70.1 +TT+MBs | -67.1 +TT+MBs | -64.1 +TT+MBs |
| n261 | -77.4 +TT+MBs | -74.4 +TT+MBs | -71.4 +TT+MBs | -68.4 +TT+MBs |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. NOTE 3: Refer Table 7.3.4.5-3b for details for MBs allowance corresponding to supported FR2 band set combination NOTE 4: For a Rel-15 UE supporting FR2 bands set not defined in Table 7.3.2.3.3-1a, Table 7.3.4.5-3c applies. |  |  |  |  |

Table 7.3.4.5-3b: EIS spherical coverage multiband relaxation factors for power class 3 (Rel-15)

| ID | Supported FR2 bands set | Maximum sum of MBs, ∑MBs (dB) (Note 3) | Comments |
| --- | --- | --- | --- |
| 1 | n257, n258 | 1.25 | Maximum 0.75 dB relaxation allowed for each band |
| 2 | n257, n260 | 0.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 3 | n258, n260 | 0.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 4 | n258, n261 | 1.25 | Maximum 0.75 dB relaxation allowed for each band |
| 5 | n260, n261 | 0.75 | No relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 6 | n257, n258, n260 | 1.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 7 | n257, n258, n261 | 1.75 | Maximum 0.75 dB relaxation allowed for each band |
| 8 | n257, n260, n261 | 1.25 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 9 | n258, n260, n261 | 1.25 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 10 | n257, n258, n260, n261 | 1.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| NOTE 1: MBs is the Multiband Relaxation factor declared by the UE for the tested band in Table A.4.3.9-3 of TS38.508-2 [11]. This declaration shall fulfil the requirements in Table 7.3.2.3.3-1a. NOTE 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant NOTE 3: Max allowed sum of MBs over all supported FR2 bands as defined in clause 7.3.2.3.3. |  |  |  |

Table 7.3.4.5-3c: EIS spherical coverage for power class 3 (Rel-16 and forward)

| Operating band | EIS at 50th%ile CCDF (dBm) / Channel bandwidth (NOTE 3) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -77.4 +TT+MBs,n | -74.4 +TT+MBs,n | -71.4 +TT+MBs,n | -68.4 +TT+MBs,n |
| n258 | -77.4 +TT+MBs,n | -74.4 +TT+MBs,n | -71.4 +TT+MBs,n | -68.4 +TT+MBs,n |
| n259 | -71.9 +TT+MBs,n | -68.9 +TT+MBs,n | -65.9 +TT+MBs,n | -62.9 +TT+MBs,n |
| n260 | -73.1 +TT+MBs,n | -70.1 +TT+MBs,n | -67.1 +TT+MBs,n | -64.1 +TT+MBs,n |
| n261 | -77.4 +TT+MBs,n | -74.4 +TT+MBs,n | -71.4 +TT+MBs,n | -68.4 +TT+MBs,n |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. NOTE 3: Refer Table 7.3.4.5-3d for details for MBs allowance corresponding to supported FR2 band set combination |  |  |  |  |

Table 7.3.4.5-3d: EIS spherical coverage multi-band relaxation factors for power class 3 (Rel-16 and forward)

| ID | FR2 bands/set | Comments |
| --- | --- | --- |
| 1 | n257 |  |
| 2 | n258 |  |
| 3 | n259 |  |
| 4 | n260 |  |
| 5 | n261 |  |
| 6 | n257, n261 | MBs,n relaxation is 0 dB |
| 7 | n260, n261 | MBs,n relaxation is 0 dB |
| NOTE 1: MBs,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 7.3.2.3.3-1b. |  |  |

Table 7.3.4.5-3e: Test Tolerance (Reference sensitivity for power class 3)

| Test Metric | FR2a, FR2b | FR2c |
| --- | --- | --- |
| IFF (Max device size ≤ 30 cm) | 2.28 dB | [2.55] dB |

Table 7.3.4.5-4: EIS spherical coverage for power class 4

| Operating band | EIS at 20th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -88.0 +TT | -85.0 +TT | -82.0 +TT | -79.0 +TT |
| n258 | -88.0 +TT | -85.0 +TT | -82.0 +TT | -79.0 +TT |
| n260 | -83.0 +TT | -80.0 +TT | -77.0 +TT | -74.0 +TT |
| n261 | -88.0 +TT | -85.0 +TT | -82.0 +TT | -79.0 +TT |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |

Table 7.3.4.5-5: EIS spherical coverage for power class 5

| Operating band | EIS at 85th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -84.6 +TT | -81.6 +TT | -78.6 +TT | -75.6 +TT |
| n258 | -84.8 +TT | -81.8 +TT | -78.8 +TT | -75.8 +TT |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |

Table 7.3.4.5-5a: Test Tolerance (Reference sensitivity for power class 5)

| Test Metric | FR2a |
| --- | --- |
| IFF (Max device size ≤ 30 cm) | 2.28 dB |

Table 7.3.4.5-6: EIS spherical coverage for power class 6

FFS

Table 7.3.4.5-7: EIS spherical coverage for power class 7

| Operating band | EIS at 50th %-tile CCDF (dBm) / Channel bandwidth |  |
| --- | --- | --- |
|  | 50 MHz | 100 MHz |
| n257 | -74.4+TT | -71.4+TT |
| n258 | -74.4+TT | -71.4+TT |
| n261 | -74.4+TT | -71.4+TT |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in Annex E.2.1. |  |  |

Table 7.3.4.5-7a: Test Tolerance (Reference sensitivity for power class 7)

| Test Metric | FR2a |
| --- | --- |
| IFF (Max device size ≤ 30 cm) | [TBD] |

## <<< Skip unchanged sections >>>

## F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty (MTSU) for receiver tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 7.3.2 Reference sensitivity power level | PC3 Max Device size ≤ 30 cm ±5.36 dB (FR2a, FR2b, NTC testing) [±5.95] dB (FR2c NTC testing) ±5.61 dB (FR2a, FR2b, ETC testing) TBD (FR2c ETC testing) PC1 Max Device size ≤ 30 cm ±5.58 dB (FR2a, FR2b, NTC testing) ±5.83 dB (FR2a, FR2b, ETC testing) PC5 Max Device size ≤ 30 cm ±5.58 dB (FR2a, NTC testing) ± 5.83 dB (FR2a, ETC testing) | MTSU = 1.00 x MU (from Table B.19-1 in TR 38.903) |
| 7.3.4 EIS spherical coverage | PC3 ±5.07 dB (Max Device size ≤ 30 cm, FR2a, FR2b) [±5.66] dB (Max Device size ≤ 30 cm, FR2c) PC1 ±5.07 dB (Max Device size ≤ 30 cm, FR2a, FR2b) PC5 ±5.07 dB (Max Device size ≤ 30 cm, FR2a) | MTSU = 1.00 x MU (from Table B.19-2 in TR 38.903) |

## <<< Skip unchanged sections >>>

## F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 7.3.2 Reference sensitivity power level | PC3 IFF (Max Device size ≤ 30 cm) 2.41 dB (FR2a, FR2b, NTC) 2.52 dB (FR2a, FR2b, ETC) [2.68] dB (FR2c, NTC) TBD (FR2c, ETC) PC1 IFF (Max Device size ≤ 30 cm) 2.51 dB (FR2a, FR2b, NTC) 2.62 dB (FR2a, FR2b, ETC) PC5 IFF (Max Device size ≤ 30 cm) 2.51 dB (FR2a, NTC) 2.62 dB (FR2a, ETC) | TT = 0.45 x MTSUIFF |
| 7.3.4 EIS spherical coverage | PC3 IFF (Max Device size ≤ 30 cm, FR2a, FR2b) 2.28 dB IFF (Max Device size ≤ 30 cm, FR2c) [2.55] dB PC1 IFF (Max Device size ≤ 30 cm, FR2a, FR2b) 2.28 dB PC5 IFF (Max Device size ≤ 30 cm, FR2a) 2.28 dB | TT = 0.45 x MTSUIFF |

## <<< END OF CHANGES >>>