3GPP TSG-RAN5 Meeting #100 	R5-234896

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0974 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ACLR TT values for PC3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TT values for ACLR (PC3) in Table F.3.2-1 are incorrect. Background: Update of TT values was discussed and agreed in R5-215826. However, the values were updated in Table 6.5.2.3.5-1a but not in Table F.3.2-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TT values for ACLR (PC3) in Table F.3.2-1 were corrected according to Table 6.5.2.3.5-1a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ACLR (PC3) could not be tested correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Unchanged sections skipped>>

#### 6.5.2.3	Adjacent channel leakage ratio

<<Unchanged sections skipped>>

Table 6.5.2.3.5-1a: Test Tolerance (Adjacent channel leakage ratio) for PC3

|  |  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test ID | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| NRACLR for band n257, n258, n261 | 1-2, 4-5 | 4.10 | 4.49 | 4.66 | 5.06 |
|  | 3, 6 | 4.08 | 4.45 | 4.59 | 5.06 |
|  | 7-9 | 4.15 | 4.59 | 4.85 | 3.34 |
|  | 10-12 | 4.36 | 4.98 | 4.06 | 1.46 |
|  | 13-15 | 4.17 | 4.62 | 4.91 | 2.99 |
| NRACLR for band n260 | 1-2, 4-5 | 4.48 | 4.65 | 4.97 | - |
|  | 3, 6 | 4.45 | 4.58 | 4.84 | - |
|  | 7-9 | 4.58 | 4.84 | 5.31 | - |
|  | 10-12 | 4.97 | - | - | - |
|  | 13-15 | 4.62 | 4.90 | - | - |
| NRACLR for band n259 | 1-15 | TBD | TBD | TBD | TBD |

## <<Unchanged sections skipped>>

## F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| <<Unchanged rows skipped>> |  |  |
| 6.5.2.3 Adjacent Channel Leakage Ratio | Absolute requirement 0 dB Relative requirement PC3 IFF (Max Device size ≤ 30 cm) FR2a: BW ≤ 50MHz: 4.10 dB (Test ID 1-2, 4-5) 4.08 dB (Test ID 3, 6) 4.15 dB (Test ID 7-9) 4.36 dB (Test ID 10-12) 4.17 dB (Test ID 13-15) 50MHz < BW ≤ 100MHz: 4.49 dB (Test ID 1-2, 4-5) 4.45 dB (Test ID 3, 6) 4.59 dB (Test ID 7-9) 4.98 dB (Test ID 10-12) 4.62 dB (Test ID 13-15) 100MHz < BW ≤ 200MHz: 4.66 dB (Test ID 1-2, 4-5) 4.59 dB (Test ID 3, 6) 4.85 dB (Test ID 7-9) 4.06 dB (Test ID 10-12) 4.91 dB (Test ID 13-15) 200MHz < BW ≤ 400MHz: 5.06 dB (Test ID 1-6) 3.34 dB (Test ID 7-9) 1.46 dB (Test ID 10-12) 2.99 dB (Test ID 13-15) FR2b: BW ≤ 50MHz: 4.48 dB (Test ID 1-2, 4-5) 4.45 dB (Test ID 3, 6) 4.58 dB (Test ID 7-9) 4.97 dB (Test ID 10-12) 4.62 dB (Test ID 13-15) 50MHz < BW ≤ 100MHz: 4.65 dB (Test ID 1-2, 4-5) 4.58 dB (Test ID 3, 6) 4.84 dB (Test ID 7-9) 4.90 dB (Test ID 13-15) 100MHz < BW ≤ 200MHz: 4.97 dB (Test ID 1-2, 4-5) 4.84 dB (Test ID 3, 6) 5.31 dB (Test ID 7-9) PC1 IFF (Max Device size ≤ 30 cm) FR2a: ±5.26 dB (BW ≤ 400MHz) FR2b: ±5.26 dB (BW ≤ 400MHz) | PC3 TT = max(R, ΔSNRmr+0.65 x (MTSUIFF-1.0)) -R + TT due to metric change TT due to metric change : 1.0 dB R: Relaxation needed to limit influence of TE noise to 1 dB (specified in clause 6.5.2.3.5) ΔSNRmr: Systematic offset due to noise when measuring ACP at minimum requirement level PC1 TT = max(R, ΔSNRmr+0.65 x (MTSUIFF-0.95)) -R + TT due to metric change |
| <<Unchanged rows skipped>> |  |  |

## <<End of change>>