3GPP TSG-RAN5 Meeting #100	R5-234289r2

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0731 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to FR2 Demod CA test cases for Agg BW upto 400MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current FR2 Demod CA test cases only testable for Aggregated ChBW of 200MHz. The MU has been defined for Agg BW upto 400MHz. Hence more test points can be made testable. Also for existing 2CC and 3CC test cases noticed several missing aspects such as test point selection table, CA bandwidth combination selection, addition of SCC. Current test procedure is ambiguous on how to do the throughput measurement per component carrier. FR2 SDR CA test cases is incorrectly listed as incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated test applicability table to enable test points upto 400MHz Agg BW Updated 2CC and 3CC test cases with test point selection table, CA bandwidth combination Updated test procedure on SCC addition and configuration. Clarified test procedure to do the throughput measurements per component carrier simultaneously to save test time. Added Fr2 4CC Demod test Updated editor’s note for the FR2 SDR CA test cases to make them complete for Agg ChBW upto 400MHz R1: corrected the applicability for n259 > 300 MHz Agg BW R2: Editor’s note added to indicate testability of test requirements table is only applicable for SC and intra-band contiguous CA. Also indicated n259 and n260 values for CA test cases are in [] |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 2CC, 3CC, 4CC test case will not implemented correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2A.2.1, 7.2A.2.2, 7.2A.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-234289r2 R5-235877 |  |  |  |  |  |  |  |  |

Table 7.1.1_1-2: Testability of test requirements due to maximum achievable SNR per band for PC3 and for intra-band contiguous CA

Editor’s note: n259 and n260 test point applicability for > 200MHz Aggregated ChBW is in []

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 7.2.2.2.1_1 | 1-1 | 100 | Yes | 1.4 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 3.6 | x | x | x | x | x |
|  | 1-3 | 100 | Yes | 14.2 | x | x | x | x | x |
|  | 2-1 | 100 | Yes | 5.8 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 16.0 | x | x | x | x | x |
|  | 2-3 | 50 | Yes | 15.7 | x | x | x | x | x |
|  | 2-4 | 200 | Yes | 15.8 | x | x | - | x | x |
|  | 2-5 | 50 | Yes | 16 | x | x | x | x | x |
|  | 2-6 | 100 | Yes | 20.3 | x | x | - | x | x |
| 7.2.2.2.1_2 | 3-1 | 100 | Yes | 20.7 | x | x | - | x | x |
| 7.2.2.2.1_3 | 1-4 | 50 | Yes | 22.0 | x | x |  | x | x |
| 7.2.2.2.2_1 | 1-1 | 100 | Yes | TBD | TBD | TBD | TBD | TBD | TBD |
| 7.2.2.2.3_1 | 1-1 | 100 | Yes | 3.1 | x | x | x | x | x |
| 7.2A.2.1 | - | BWagg ≤ 300 | Yes | 12.0 | x | x | [x] | [x] | x |
|  | - | 300 < BWagg ≤ 400 | Yes | 12.0 | x | x | - | [x] | x |
|  | - | BWagg > 400 | Yes | 12.0 | TBD | TBD | - | TBD | TBD |
| 7.2A.2.2 | - | BWagg ≤ 300 | Yes | 12.0 | x | x | [x] | [x] | x |
|  | - | 300 < BWagg ≤ 400 | Yes | 12.0 | x | x | - | X | x |
|  | - | BWagg > 400 | Yes | 12.0 | TBD | TBD | - | TBD | TBD |
| 7.2A.2.3 | - | BWagg ≤ 300 | Yes | 12.0 | x | x | [x] | [x] | x |
|  | - | 300 < BWagg ≤ 400 | Yes | 12.0 | x | x | - | [x] | x |
|  | - | BWagg > 400 | Yes | 12.0 | TBD | TBD | - | TBD | TBD |
| 7.3.2.2.1 | 1-1 | 100 | Yes | 7.7 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 4.3 | x | x | x | x | x |
| 7.3.2.2.2 | 2-1 | 100 | Yes | 3.2 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 0.2 | x | x | x | x | x |
| 7.3.2.2.3 | 1-1 | 100 | Yes | 4.7 | x | x | x | x | x |
| 7.5.1 | - | 100 | No | NA | x | x | x | x | x |
|  | - | 200 | No | NA | x | x | x | x | x |
|  | - | 400 | No | NA | TBD | TBD | TBD | TBD | TBD |
| 7.5A.1 | - | BWagg ≤ 400 | No | NA | x | x | x | x | x |
|  | - | BWagg > 400 | No | NA | TBD | TBD | TBD | TBD | TBD |

Table 7.1.1_1-2a: Testability of test requirements due to maximum achievable SNR per band for PC1

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 7.2.2.2.1_1 | 1-1 | 100 | Yes | 1.4 | x | x | TBD | x | x |
|  | 1-2 | 100 | Yes | 3.6 | x | x | TBD | x | x |
|  | 1-3 | 100 | Yes | 14.2 | x | x | TBD | x | x |
|  | 2-1 | 100 | Yes | 5.8 | x | x | TBD | x | x |
|  | 2-2 | 100 | Yes | 16.0 | x | x | TBD | x | x |
|  | 2-3 | 50 | Yes | 15.7 | x | x | TBD | x | x |
|  | 2-4 | 200 | Yes | 15.8 | x | x | TBD | x | x |
|  | 2-5 | 50 | Yes | 16 | x | x | TBD | x | x |
|  | 2-6 | 100 | Yes | 20.3 | x | x | TBD | x | x |
| 7.2.2.2.1_2 | 3-1 | 100 | Yes | 20.7 | x | x | TBD | x | x |
| 7.2.2.2.1_3 | 1-4 | 50 | Yes | 22.0 | x | x | TBD | x | x |
| 7.2.2.2.2_1 | 1-1 | 100 | Yes | TBD | TBD | TBD | TBD | TBD | TBD |
| 7.2.2.2.3_1 | 1-1 | 100 | Yes | 3.1 | x | x | TBD | x | X |
| 7.2A.2.1 | 1,2 | BWagg ≤ 200 | Yes | 12.0 | x | x | TBD | x | x |
|  | 3,4 | BWagg > 200 | Yes | 12.0 | TBD | TBD | TBD | TBD | TBD |
| 7.2A.2.2 | 1,2 | BWagg ≤ 200 | Yes | 12.0 | x | x | TBD | x | x |
|  | 3,4 | BWagg > 200 | Yes | 12.0 | TBD | TBD | TBD | TBD | TBD |
| 7.3.2.2.1 | 1-1 | 100 | Yes | 7.7 | x | x | TBD | x | x |
|  | 1-2 | 100 | Yes | 4.3 | x | x | TBD | x | x |
| 7.3.2.2.2 | 2-1 | 100 | Yes | 3.2 | x | x | TBD | x | x |
|  | 2-2 | 100 | Yes | 0.2 | x | x | TBD | x | x |
| 7.3.2.2.3 | 1-1 | 100 | Yes | 4.7 | x | x | TBD | x | x |
| 7.5.1 | - | 100 | No | NA | x | x | TBD | x | x |
|  | - | 200 | No | NA | x | x | TBD | x | x |
|  | - | 400 | No | NA | TBD | TBD | TBD | TBD | TBD |
| 7.5A.1 | - | BWagg ≤ 200 | No | NA | x | x | TBD | x | x |
|  | - | BWagg > 200 | No | NA | TBD | TBD | TBD | TBD | TBD |

#### <several sections skipped>

#### 7.2A.2.1	2Rx TDD FR2 CA requirements for normal PDSCH Demodulation Performance for both SA and NSA (2DLCA)

7.2A.2.1.1	Test Purpose

Verify the PDSCH mapping Type A normal performance with different channel models, MCSs and number of MIMO layers.

7.2A.2.1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward that support NR 2DL CA.

7.2A.2.1.3	Test Description

7.2A.2.1.3.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-2.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

CA capability to be tested: Any one of the supported CA capability as per the selection criteria specified in Table 7.1.1.5.2-2.

CA Bandwidth combination: Largest aggregated bandwidth as per Table 7.2A.2.1.3.1-1

For EN-DC within FR2 operation, setup the LTE radiated link according to Annex D:

Table 7.2A.2.1.3.1-1: Test point selection for CA UE demodulation tests

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test 1 in Section 7.2A.2.0-3 | CA_C, CA_N, CA_AX | Table 7.1.1.5.2-2 | Largest aggregated CA bandwidth combination | Any of CCs |

1.	Connection between SS, the faders, AWGN noise source and the UE is shown in TS 38.508-1 [6] Annex A, Figure A.3.3.2 for TE diagram and Figure A.3.4.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 7.2-1, 7.2A.2.0-1 and Table 7.2A.2.0-2 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-2 [8].

4.	Propagation conditions for NR cell are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 7.2A.2.1.3.3.

7.2A.2.1.3.2	Test Procedure

1.	Set the UE in a direction that satisfies the 3 normative criteria specified in Annex H.0. If no direction found, mark the test as inconclusive.

2.	Configure SCC(s) according to Annex C.0, C.1 and C.2 for all downlink physical channels.

3.	The SS shall configure SCC(s) as per TS 38.508-1 [6] clause 5.5.1. Message contents are defined in clause 7.2 A.2.1.1.3.3.

4.	SS activates SCC(s) by sending the activation MAC-CE (Refer TS 38.321 [24], clauses 5.9, 6.1.3.10). Wait for at least 1 second (Refer TS 38.133[25], clause9.3).

5.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.2A.2.1.4-1 on each NR CC. The SS sends downlink MAC padding bits on the DL RMC.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table  7.2A.2.1.4-1 on each NR CC

7.	Measure the average throughput on each component carrier simultaneously for a duration sufficient to achieve statistical significance according to Annex G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Tables G.1.5-1 in Annex G.

7.2A.2.1.3.3	Message Contents

Message contents are according to TS 38.508-1 [6] subclause 4.6.1 and 5.4.2

7.2A.2.1.4	Test Requirements

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A 3.2.2 for each throughput test per component carrier shall meet or exceed the specified value as per Table 7.2A.2.1.4_1.4-1 for the specified SNR including test tolerances.

Table 7.2A.2.1.4-1: Single carrier performance for TDD 120 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 50 | R.PDSCH.5-9.1 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 12.1 |
| 100 | R.PDSCH.5-9.2 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 11.9 |
| 200 | R.PDSCH.5-9.3 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 12.0 |
| 400 | R.PDSCH.5-9.4 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 12.0 |

#### 7.2A.2.2	2Rx TDD FR2 CA requirements for normal PDSCH Demodulation Performance for both SA and NSA (3DLCA)

7.2A.2.2.1	Test Purpose

Same as 7.2A.2.1.1

7.2A.2.2.2	Test Applicability

This test applies to all types of NR UE release 15 and forward that support NR 3DL CA.

7.2A.2.2.3	Test Description

Same as 7.2A.2.1.3

7.2A.2.2.3.2	Test Procedure

Same as 7.2A.2.1.3.2

7.2A.2.2.3.3	Message Contents

Same as 7.2A.2.1.3.3

7.2A.2.2.4	Test Requirements

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A 3.2.2 for each throughput test per component carrier shall meet or exceed the specified value as per Table 7.2A.2.2.4_1.4-1 for the specified SNR including test tolerances.

Table 7.2A.2.2.4-1: Single carrier performance for TDD 120 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 50 | R.PDSCH.5-9.1 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 12.1 |
| 100 | R.PDSCH.5-9.2 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 11.9 |
| 200 | R.PDSCH.5-9.3 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 12.0 |
| 400 | R.PDSCH.5-9.4 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 12.0 |

#### 7.2A.2.3	2Rx TDD FR2 CA requirements for normal PDSCH Demodulation Performance for both SA and NSA (4DLCA)

7.2A.2.3.1	Test Purpose

Same as 7.2A.2.1.1

7.2A.2.3.2	Test Applicability

This test applies to all types of NR UE release 15 and forward that support NR 4DL CA.

7.2A.2.3.3	Test Description

Same as 7.2A.2.1.3

7.2A.2.3.3.2	Test Procedure

Same as 7.2A.2.1.3.2

7.2A.2.3.3.3	Message Contents

Same as 7.2A.2.1.3.3

7.2A.2.3.4	Test Requirements

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A 3.2.2 for each throughput test per component carrier shall meet or exceed the specified value as per Table 7.2A.2.3.4_1.4-1 for the specified SNR including test tolerances.

Table 7.2A.2.3.4-1: Single carrier performance for TDD 120 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 50 | R.PDSCH.5-9.1 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 12.1 |
| 100 | R.PDSCH.5-9.2 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 11.9 |
| 200 | R.PDSCH.5-9.3 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 12.0 |
| 400 | R.PDSCH.5-9.4 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 12.0 |

<Several sections skipped>

#### 7.5A.1.2	FR2 SDR performance for CA (3DL CA)

Editor’s Note: MU analysis is complete for upto 400MHz Aggregated Channel BW

7.5A.1.2.1	Test Purpose

Same as in clause 7.5A.1.2.1

7.5A.1.2.2	Test Applicability

This test applies to all types of NR UE release 15 and forward supporting 3DLCA.

7.5A.1.2.3	Minimum conformance requirements

Same as in 7.5.1.3

7.5A.1.2.4	Test description

Same as in clause 7.5A.1.1.4

7.5A.1.2.5	Test requirement

The TB success rate of greater than 85% with no PDCP SDU loss shall be sustained during at least 300 frames on each CC.

#### 7.5A.1.3	FR2 SDR performance for CA (4DL CA)

Editor’s Note: MU analysis is complete for upto 400MHz Aggregated Channel BW

7.5A.1.3.1	Test Purpose

Same as in clause 7.5A.1.3.1

7.5A.1.3.2	Test Applicability

This test applies to all types of NR UE release 15 and forward supporting 4DLCA.

7.5A.1.3.3	Minimum conformance requirements

Same as in 7.5.1.3

7.5A.1.3.4	Test description

Same as in clause 7.5A.1.1.4

7.5A.1.3.5	Test requirement

The TB success rate of greater than 85% with no PDCP SDU loss shall be sustained during at least 300 frames on each CC.

#### 7.5A.1.4	FR2 SDR performance for CA (5DL CA)

Editor’s Note: MU analysis is complete for upto 400MHz Aggregated Channel BW

7.5A.1.4.1	Test Purpose

Same as in clause 7.5A.1.3.1

7.5A.1.4.2	Test Applicability

This test applies to all types of NR UE release 15 and forward supporting 5DLCA.

7.5A.1.4.3	Minimum conformance requirements

Same as in 7.5.1.3

7.5A.1.4.4	Test description

Same as in clause 7.5A.1.1.4

7.5A.1.4.5	Test requirement

The TB success rate of greater than 85% with no PDCP SDU loss shall be sustained during at least 300 frames on each CC.

#### 7.5A.1.5	FR2 SDR performance for CA (6DL CA)

Editor’s Note: MU analysis is complete for upto 400MHz Aggregated Channel BW

7.5A.1.5.1	Test Purpose

Same as in clause 7.5A.1.3.1

7.5A.1.5.2	Test Applicability

This test applies to all types of NR UE release 15 and forward supporting 6DLCA.

7.5A.1.5.3	Minimum conformance requirements

Same as in 7.5.1.3

7.5A.1.5.4	Test description

Same as in clause 7.5A.1.1.4

7.5A.1.5.5	Test requirement

The TB success rate of greater than 85% with no PDCP SDU loss shall be sustained during at least 300 frames on each CC.

#### 7.5A.1.6	FR2 SDR performance for CA (7DL CA)

Editor’s Note: MU analysis is complete for upto 400MHz Aggregated Channel BW

7.5A.1.6.1	Test Purpose

Same as in clause 7.5A.1.3.1

7.5A.1.6.2	Test Applicability

This test applies to all types of NR UE release 15 and forward supporting 7DLCA.

7.5A.1.6.3	Minimum conformance requirements

Same as in 7.5.1.3

7.5A.1.6.4	Test description

Same as in clause 7.5A.1.1.4

7.5A.1.6.5	Test requirement

The TB success rate of greater than 85% with no PDCP SDU loss shall be sustained during at least 300 frames on each CC.

#### 7.5A.1.7	FR2 SDR performance for CA (8DL CA)

Editor’s Note:  MU analysis is complete for upto 400MHz Aggregated Channel BW

7.5A.1.7.1	Test Purpose

Same as in clause 7.5A.1.3.1

7.5A.1.7.2	Test Applicability

This test applies to all types of NR UE release 15 and forward supporting 8DLCA.

7.5A.1.7.3	Minimum conformance requirements

Same as in 7.5.1.3

7.5A.1.7.4	Test description

Same as in clause 7.5A.1.1.4

7.5A.1.7.5	Test requirement

The TB success rate of greater than 85% with no PDCP SDU loss shall be sustained during at least 300 frames on each CC.