3GPP TSG-RAN5 Meeting #100 	R5-234388

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2510 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 5.5.1.6 EN-DC FR2 RLM test for PSCell configured with CSI-RS-based RLM RS including TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test parameter needs to be updated in FR2 RLM tests for PSCell based on the previously agreed RAN4 CR (R4-2307267). Also some values for SNR on RLM-RS1 and RS2 are corrected to align with the core spec. Associated Requirements and TT values also need to be updated in Annex F.1.3.2. There is a typo at the parameter “SNR on RLM-RS1" during T2 in Table 5.5.1.6.1-1. +3.9 dB should be -3.9 dB according to the TT analysis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replced Figure 5.5.1.6.4-1 to swap SNR level of RLM-RS 1 and 2 during T4 and T5. Swapped SNR on RLM-RS1 and RS2 during T4 and T5 in Table 5.5.1.6.5-1. SNR on RLM-RS2 during T2 in Table 5.5.1.6.5-1: -14 dB -> -15 dB . Corrected typo with SNR on RLM-RS1 during T2 in Table 5.5.1.6.1-1: 3.9 dB -> -3.9 dB. Updated following values. Test 5.5.1.6 in Table F.1.3.2-3. Minimum requirement Es2 / Noc during T2: -14 -> -15 dB Es1 / Noc during T4: -4.5 -> -15 dB Es2 / Noc during T4: -15 -> -4.5 dB Es1 / Noc during T5: +2 -> -15 dB Es2 / Noc during T5: -14 -> +2 dB Test tolerance TT for Es1/Noc during T5: 2.1 -> 0 dB TT for Es2/Noc during T5: 0 -> 2.1 dB Test requirement Es2 / Noc during T2: -14 -> -15 dB Es1 / Noc during T4: -4.5 -> -15 dB Es2 / Noc during T4: -15 -> -4.5 dB Es1 / Noc during T5: 4.1 -> -15 dB Es2 / Noc during T5: -14 -> +4.1 dB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies between the core/test requirements (TS38.133/ TS38.533) remain in the spec. UE beam tracking aspects might not be verified in RLM test cases when the RLM resources are received from different angles. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.1.6, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated CR for 38.903: R5-234389 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

<<Start of change>>

5.5.1.6	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode

Editor's note: This test case is incomplete. The following aspects are either missing or TBD

- TT analysis is made at Test frequency f ≤ 40.8 GHz- Connection diagram is TBD.

- Test Procedure and message contents will need further editing and review

5.5.1.6.1	Test purpose

The purpose of this test is to verify that the UE properly detects the in-sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PSCell when no DRX is used. This test will partly verify the FR2 TDD PSCell CSI-RS in-sync radio link monitoring requirements in TS 38.133 clause 8.1.

5.5.1.6.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward supporting EN-DC and CSI-RS based RLM.

5.5.1.6.3	Minimum conformance requirements

The minimum requirements are specified in clause 4.5.1.0.4. DRX configuration is not used for this test.

The normative reference for this requirement is TS 38.133 [6] clause A.5.5.1.6.

5.5.1.6.4	Test description

There are two cells configured in this test, the E-UTRA PCell and NR PSCell. This test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 5.5.1.6.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate in-sync states.

Figure 5.5.1.6.4-1: SNR variation for in-sync testing

5.5.1.6.4.1	Initial conditions

Test 5.5.1.6 can be run in one of the configurations defined in Table 5.5.1.6.4.1-1.

Table 5.5.1.6.4.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 5.5.1.6-1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 5.5.1.6-2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Configure the test equipment and the DUT according to the parameters in Table 5.5.1.6.4.1-2

Table 5.5.1.6.4.1-2: Initial conditions for CSI-RS in-sync radio link monitoring in non-DRX mode

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.2-1 and TS 38.508-1 [14] clause 4.3.1 for E-UTRA and 7.2.3 for NR. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.5.1.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The test parameters are given in Table 5.5.1.6.4.1-3 below.

2. Message contents are defined in clause 5.5.1.6.4.3.

3. There are two cells in the test, where Cell 1 is the E-UTRAN PCell on the E-UTRA carrier, and Cell 2 is the NR PSCell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to TS 38.133 [6] Table A.6.1.1-1 for this test. Cell 2 is configured according to clause C.1.1 and C.1.2.

Table 5.5.1.6.4.1-3: General test parameters for FR2 PSCell for CSI-RS in-sync testing in non-DRX mode

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active E-UTRA PCell |  |  |  | Cell 1 |
| E-UTRA RF Channel Number |  |  |  | 1 |
| Active PSCell |  |  |  | Cell 2 |
| RF Channel Number |  |  |  | 2 |
| Duplex Mode |  |  |  | TDD |
| BWchannel |  | Config 1, 2 |  | 100: NRB,c = 66 |
| Data RBs allocated |  | Config 1, 2 |  | 24 |
| BWoccupied |  | Config 1, 2 |  | 24 |
| TDD | Config 1 |  |  | TDDConf.3.1 |
| Configuration | Config 2 |  |  | TDDConf.3.1 |
| DL initial BWP configuration | Config 1, 2 |  |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2 |  |  | DLBWP.1.4 |
| UL initial BWP configuration | Config 1, 2 |  |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2 |  |  | ULBWP.1.4 |
| RMSI CORESET Reference Channel | Config 1 |  |  | CR.3.1 TDD |
|  | Config 2 |  |  | CR.3.1 TDD |
| Dedicated CORESET Reference | Config 1 |  |  | CCR.3.1 TDD CCR.3.3 TDD |
| Channel | Config 2 |  |  | CCR.3.1 TDD CCR.3.3 TDD |
| SSB | Config 1 |  |  | SSB.1 FR2 |
| Configuration | Config 2 |  |  | SSB.1 FR2 |
| SMTC | Config 1 |  |  | SMTC.1 |
| Configuration | Config 2 |  |  | SMTC.1 |
| PDSCH/PDCCH | Config 1 |  |  | 120 KHz |
| subcarrier spacing | Config 2 |  |  | 120 KHz |
| CSI-RS for RLM | Config 1, 2 |  |  | Resource #4 in TRS.2.1 TDD Resource #4 in TRS.2.2 TDD |
| OCNG parameters |  |  |  | OP.5 |
| TRS configuration |  |  |  | TRS.2.1 TDD TRS.2.2 TDD |
| TCI configuration for PDCCH#1/PDSCH |  |  |  | TCI.State.2 |
| TCI configuration for PDCCH#2 |  |  |  | TCI.State.3 |
| CP length |  |  |  | Normal |
| Out of sync | DCI format |  |  | 1-0 |
| transmission parameters | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| In sync | DCI format |  |  | 1-0 |
| transmission parameters | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 1000 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS for CSI | Config 1 |  |  | CSI-RS.3.1 TDD |
| reporting | Config 2 |  |  | CSI-RS.3.1 TDD |
| T1 |  |  | s | 0.2 |
| reportConfigType |  |  |  | periodic |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| CSI reporting periodicity |  |  | slot | 40 |
| CSI reporting offset |  |  | slot | 4 |
| T2 |  |  | s | 0.2 |
| T3 |  |  | s | 0.24 |
| T4 |  |  | s | 0.2 |
| T5 |  |  | s | 0.88 |
| D1 |  |  | s | 0.84 |
| NOTE 1: UE-specific PDCCH is not transmitted after T1 starts. NOTE 2: E-UTRAN is in non-DRX mode under test. |  |  |  |  |

5.5.1.6.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity defined in CSI-RS configuration. In the test, DRX configuration is not enabled.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.4.

2. Set the parameters of Cell 2 according to T1 in Table 5.5.1.6.5-1. Propagation conditions are set according to clause C.2.3. T1 starts.

3. When T1 expires the SS shall change the SNR value to T2 as specified in Table 5.5.1.6.5-1. T2 starts.

4. When T2 expires the SS shall change the SNR value to T3 as specified in Table 5.5.1.6.5-1. T3 starts.

5. When T3 expires the SS shall change the SNR value to T4 as specified in Table 5.5.1.6.5-1. T4 starts.

6. When T4 expires the SS shall change the SNR value to T5 as specified in Table 5.5.1.6.5-1. T5 starts.

7. If the SS detects uplink power on NR carrier in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point A to time point F (T6 ms after the start of time duration T5) the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	If the UE has not re-established the connection in at least 1s, the SS shall ensure PSCell is released.

9.	The SS then shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message.

10.	If the Reconfiguration fails, switch off and on the UE and ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5].

11.	Repeat steps 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

5.5.1.6.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 5.5.1.6.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-2 with Condition INTRA-FREQ, L3 FILTERING NEEDED; Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR2, SMTC.1 and RLM Table H.3.1-8 with Condition CSI RLM Table H.3.1-9 |

5.5.1.6.5	Test requirement

Tables 5.5.1.6.4.1-2 and 5.5.1.6.5-1 define the primary level settings including test tolerances for Radio Link Monitoring in-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode.

Table 5.5.1.6.5-1: Cell specific test parameters for FR2 for CSI-RS is-sync radio link monitoring in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 3 defined in A.9.3 |  |  |  |  |  |  |  |  |  |
|  |  |  | AoA1 |  |  |  |  | AoA2 |  |  |  |  |
| Assumption for UE beamsNote 10 |  |  | Rough |  |  |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  | Not sent |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| SNR on RLM-RS1 | Config 1, 2 | dB | 4.1Note 11 | -3.9Note 11 | -15 | -15 | -15 |  |  |  |  |  |
| SNR on RLM-RS2 | Config 1, 2 |  | Not sent |  |  |  |  | 4.1Note 11 | -15 | -15 | -4.5 | 4. 1Note 11 |
|  | Config 1, 2 | dBm/ 15KHz | -94.8 |  |  |  |  | -94.8 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure 5.5.1.6.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TS 38.133 [6] A.3.6. Note 10: Information about types of UE beam is given in TS 38.133 [6] B.2.1.3, and does not limit UE implementation or test system implementation Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband |  |  |  |  |  |  |  |  |  |  |  |  |

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (T6 after the start of time duration T5) the UE shall transmit uplink signal at least in all slots configured for CSI transmission according to the configured periodic CSI reporting on the PSCell.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

<<Unchanged sections skipped>>

F.1.3	Test Tolerance and Derivation of Test Requirements (informative)

See TS 38.521-1 [17] Annex F.3.

F.1.3.1	Measurement of test environments

See TS 38.521-1 [17] Annex F.3.1.

F.1.3.2	Measurement of RRM requirements

Because the relationships between the test system uncertainties and the test tolerances are often complex, it is not always possible to give a simple derivation of the test requirement in this document. The analysis is recorded in 3GPP TR 38 903 [22].

The derivation of the test requirements for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.3.2-1.

The derivation of the test requirements for the NE-DC FR1 test cases in chapter 4 is defined in Table F.1.3.2-1a.

The derivation of the test requirements for the NR/5GC FR1 test cases in chapter 6 is defined in Table F.1.3.2-2.

The derivation of the test requirements for the EN-DC FR2 test cases in chapter 5 is defined in Table F.1.3.2-3.

The derivation of the test requirements for the NR/5GC FR2 test cases in chapter 7 is defined in Table F.1.3.2-4.

The derivation of the test requirements for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases in chapter 8 is defined in Table F.1.3.2-5.

The derivation of the test requirements for the FR1 NR sidelink test cases in chapter 9 is defined in Table F.1.3.2-6.

The derivation of the test requirements for the NR/5GC FR1 test cases for RedCap in chapter 16 is defined in Table F.1.3.2-7.

The derivation of the test requirements for the NR/5GC FR2 test cases for RedCap in chapter 17 is defined in Table F.1.3.2-8.

The derivation of the test requirements for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases for RedCap in chapter 18 is defined in Table F.1.3.2-9.

<<Unchanged sections skipped>>

Table F.1.3.2-3: Derivation of test requirements for EN-DC FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 5.3.2.2.1 EN-DC FR2 contention based random access | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 5.3.2.2.2 EN-DC FR2 non-contention based random access | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB |
| 5.4.3.1 EN-DC FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |
| 5.5.1.1 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |
| 5.5.1.2 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -92.1dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -92.1dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -15 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB During T4: -2.7 dB 0 dB 0 dB During T5: -2.7 dB 2.1 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -94.8 dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -94.8 dBm/15kHz Ês1 / Noc: 4.1 dB Ês2 / Noc: -15 dB |
| 5.5.1.3 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB |
| 5.5.1.4 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -14 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB During T4: 0 dB -0.4 dB -0.4 dB During T5: 0 dB 1.3 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.9 dB Ês2 / Noc: -15.4 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: 3.3 dB Ês2 / Noc: -15.4 dB |
| 5.5.1.5 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |
| 5.5.1.6 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -4.5 dB During T5: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: +2 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB During T4: -2.7 dB 0 dB 0 dB During T5: -2.7 dB 0dB 2.1 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -4.5 dB During T5: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: +4.1 dB |
| 5.5.1.7 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB |
| 5.5.1.8 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -104.7dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -104.7dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -15 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB During T4: 0 dB -0.4 dB -0.4 dB During T5: 0 dB 1.3 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.9 dB Ês2 / Noc: -15.4 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: 3.3 dB Ês2 / Noc: -15.4 dB |

<<End of change>>