3GPP TSG-RAN5 Meeting #100	R5-234436

# Toulouse, France, Augest 21nd - May 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2544 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR SA RRM TC 6.7.1.2.1_RSRP absolute accuracy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - BG_offset is wrongly calculated twice for Cell 1 in TC 6.7.1.2.1 sub-test 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test parameters in TC 6.7.1.2.1 is corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Core Spec Alignment |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.7.1.2.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy

6.7.1.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

6.7.1.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.1.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.1.2.

6.7.1.2.1.4	Test description

6.7.1.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.1.2.1.4.1-1.

Table 6.7.1.2.1.4.1-1: NR SA FR1-FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.1.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.1.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.1.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.1.2.1.4.1-2.

Table 6.7.1.2.1.4.1-2: Initial conditions for SS-RSRP inter frequency absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] sclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.1.2.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in a different FR1 frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.7.1.2.1.4.2	Test procedure

Same as in clause 6.7.1.1.1.4.2 but replacing Table 6.7.1.1.1.5-1 and 6.7.1.1.1.5-2 with 6.7.1.2.1.5-1 and 6.7.1.2.1.5-2, respectively.

6.7.1.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.1.2.1.4.3-1: Common Exception messages for NR SA FR1-FR1 SS-RSRP absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-5 Table H.3.1-7 with condition INTER-FREQ Table H.3.1-6 with condition Pattern #0 |
| Specific message contents exceptions for Test Configuration 6.7.1.2.1-1 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.1.2.1-2 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.1.2.1-3 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.1.2.1.4.3-2: ReportConfigNR-DEFAULT(Periodical) for NR SA FR1 SS-RSRP Accuracy

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.1.2.1.5	Test requirement

Table 6.7.1.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.1.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.1.2.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.1.2.1.5-3 for test configuration 3.

Table 6.7.1.2.1.5-1: SS-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |  |
| SSB ARFCN |  | 1~3 |  | freq1 | freq2 |  | freq1 | freq2 |  |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 2 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 3 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| Gap pattern ID |  |  |  | 0 |  |  | 0 |  |  |
| Duplex mode |  | 1 |  | FDD |  |  | FDD |  |  |
|  |  | 2 |  | TDD |  |  | TDD |  |  |
|  |  | 3 |  | TDD |  |  | TDD |  |  |
| TDD configuration |  | 1 |  | N/A |  |  | N/A |  |  |
|  |  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - |  |
|  |  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - |  |
|  |  | 2 |  | CR.1.1 TDD | - |  | CR.1.1 TDD | - |  |
|  |  | 3 |  | CR.2.1 TDD | - |  | CR.2.1 TDD | - |  |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - |  |
|  |  | 2 |  | CCR.1.1 TDD | - |  | CCR.1.1 TDD | - |  |
|  |  | 3 |  | CCR.2.1 TDD | - |  | CCR.2.1 TDD | - |  |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  | - |
|  |  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| SMTC configuration |  | 1 |  | SMTC.2 |  |  | SMTC.2 |  |  |
|  |  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |  |
| Time offset between Cell 2 and Cell 3 |  | 1 | ms | 3 |  |  | 3 |  |  |
|  |  | 2,3 | s | 3 |  |  | 3 |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |  | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Note2 | Depending on band group | 1,2 | dBm/15kHz | -94.65 | -94.65 |  | ( for Cell 2 +8dB) | -115+ ΔBG_offset |  |
| Note2 | Depending on band group | 3 | dBm/15kHz | -96 | -96 |  | ( for Cell 2 +8dB) | -115+ ΔBG_offset |  |
| Note2 | Depending on band group | 1,2 | dBm/SSB SCS | -94.65 | -94.65 |  | ( for Cell 2 +8dB) | -115+ ΔBG_offset |  |
|  | Depending on band group | 3 |  | -93 | -93 |  | ( for Cell 2 +8dB) | -112.00+ ΔBG_offset |  |
|  |  | 1~3 | dB | 10 | 10 |  | 13 | -3 |  |
| SS-RSRPNote3 | Depending on band group | 1,2, | dBm/SCS | -84.65 | -84.65 |  | (RSRP for Cell 2 +25dB) | -118.00+ ΔBG_offset |  |
|  | Depending on band group | 3 |  | -83 | -83 |  | (RSRP for Cell 2 +25dB) | -115.00+ ΔBG_offset |  |
| IoNote3 | Depending on band group | 1,2 | dBm/ 9.36MHz | 56.28 | 56.28 |  | (Io for Channel 2 +19.75dB) | -85.28+ ΔBG_offset |  |
|  | Depending on band group | 3 | dBm/ 38.16MHz | -51.53 | -51.53 |  | (Io for Channel 2 +19.75dB) | -79.19+ ΔBG_offset |  |
|  |  | 1~3 | dB | 10 | 10 |  | 13 | -3 |  |
| Propagation condition |  | 1~3 | - | AWGN |  |  | AWGN |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5 The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. Note 6: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |  |  |  |  |  |

Table 6.7.1.2.1.5-2: SS-RSRP Inter frequency absolute accuracy requirements for the reported values for test configurations 1, 2, 4 and 5

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 62 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 32 |
|  |  | Bands NR_FDD_FR1_B | 33 |
|  |  | Bands NR_TDD_FR1_C | 33 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 34 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 34 |
|  |  | Bands NR_FDD_FR1_G | 35 |
|  |  | Bands NR_FDD_FR1_H | 36 |
| Highest reported value (Cell 2) | 81 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 45 |
|  |  | Bands NR_FDD_FR1_B | 45 |
|  |  | Bands NR_TDD_FR1_C | 46 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 46 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 47 |
|  |  | Bands NR_FDD_FR1_G | 48 |
|  |  | Bands NR_FDD_FR1_H | 48 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 59 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 28 |
|  |  | Bands NR_FDD_FR1_B | 28 |
|  |  | Bands NR_TDD_FR1_C | 29 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 29 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 30 |
|  |  | Bands NR_FDD_FR1_G | 31 |
|  |  | Bands NR_FDD_FR1_H | 31 |
| Highest reported value (Cell 2) | 84 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 49 |
|  |  | Bands NR_FDD_FR1_B | 50 |
|  |  | Bands NR_TDD_FR1_C | 50 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 51 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 51 |
|  |  | Bands NR_FDD_FR1_G | 52 |
|  |  | Bands NR_FDD_FR1_H | 53 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 6.7.1.2.1.5-3: SS-RSRP Inter frequency absolute accuracy requirements for the reported values for test configurations 3 and 6

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 64 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 35 |
|  |  | Bands NR_FDD_FR1_B | 36 |
|  |  | Bands NR_TDD_FR1_C | 36 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 37 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 37 |
|  |  | Bands NR_FDD_FR1_G | 38 |
|  |  | Bands NR_FDD_FR1_H | 39 |
| Highest reported value (Cell 2) | 83 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 48 |
|  |  | Bands NR_FDD_FR1_B | 48 |
|  |  | Bands NR_TDD_FR1_C | 49 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 49 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 50 |
|  |  | Bands NR_FDD_FR1_G | 51 |
|  |  | Bands NR_FDD_FR1_H | 51 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 61 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 31 |
|  |  | Bands NR_FDD_FR1_B | 31 |
|  |  | Bands NR_TDD_FR1_C | 32 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 32 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 33 |
|  |  | Bands NR_FDD_FR1_G | 34 |
|  |  | Bands NR_FDD_FR1_H | 34 |
| Highest reported value (Cell 2) | 86 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 52 |
|  |  | Bands NR_FDD_FR1_B | 53 |
|  |  | Bands NR_TDD_FR1_C | 53 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 54 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 54 |
|  |  | Bands NR_FDD_FR1_G | 55 |
|  |  | Bands NR_FDD_FR1_H | 56 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

<End of modified section 1>