3GPP TSG-RAN5 Meeting # 100	R5-234935

# Tolouse, France, 21st August 2023 – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2895 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex C Test frequencies calculations corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Determination of test frequencies for asymmetric channel bandwidths for frequency bands whose TX-RX carrier centre frequency separation can vary wihtin a range is not fully defined as FTx-Rx_separation value is not clear. This impacts variable duplex frequency bands (frequency bands n91, n92, n93 and n94) and band n24. - In the case of band n24, understood as a symmetric NR band (once UL sub-band is chosen) but supporting asymmetric channel bandwidth combinations, it is not clear whether test frequencies should be calculated using annex C.2.1 or annex C.2.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Define criteria to choose FTx-Rx_separation for frequency bands where TX-RX carrier centre frequency separation can vary within a range in clause C.2.3. - Update title of section C.2.3 to focus only on asymmetric bandwidth combinations. - Define criteria to choose FTx-Rx_separation for frequency bands where TX-RX carrier centre frequency separation can vary within a range in clause C.2.1 (impacting band n24 mainly). - Clarification of the UL sub-band to be used in case more than one (impacting band n24 mainly). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing frequencies for similar bands will not follow a uniform criterion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.2.1.1, C.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

## C.2.1	Determination of test frequencies for symmetric NR bands and symmetric uplink and downlink channel bandwidth combinations

### C.2.1.1	Determination of test frequencies for Low-, Mid- and High-Range

Downlink:

| FDL_LowRange = Ceil((FDL_Low + CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.1.1-Eq1 |
| --- | --- |
| FDL_MidRange = Round((FDL_Low + BWDL/2) / ΔFRaster) * ΔFRaster | C.2.1.1-Eq2 |
| FDL_HighRange = Floor((FDL_High - CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.1.1-Eq3 |

FDL_LowRange is rounded up and FDL_HighRange is rounded down to obey to the minimum guard band according to clause 5.3.3 of TS 38.101-1 [7] and TS 38.101-2 [8].

Uplink:

| FUL_LowRange = FDL_LowRange - FTx-Rx_separation | C.2.1.1-Eq4 |
| --- | --- |
| FUL_MidRange = FDL_MidRange - FTx-Rx_separation | C.2.1.1-Eq5 |
| FUL_HighRange = FDL_HighRange - FTx-Rx_separation | C.2.1.1-Eq6 |

For those FDD frequency bands with two UL sub-bands, FTx-Rx_separation shall be chosen using the following criteria:

- For Low-Range and Mid-Range, use FTx-Rx_separation for lower UL sub-band.

- For High-Range, use FTx-Rx_separation for higher UL sub-band.

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

## C.2.3	Determination of test frequencies for asymmetric uplink and downlink channel bandwidth combinations

### C.2.3.1	General

The following procedure is used to calculate test frequencies for NR bands supporting asymmetric UL and DL channel bandwidths as described below, where CBWUL and CBWDL refer to the carrier’s UL and DL channel bandwidths; and BWUL and BWDL refer to the band’s total UL and DL bandwidths.

The procedure is also used to calculate test frequencies for symmetric UL and DL bandwidth combinations for asymmetric NR bands.

For FDD bands supporting asymmetric uplink and downlink bandwidth combinations a deviation of ΔFTX-RX (C.2.3.1-Eq1) is to be added to the default Tx-Rx carrier centre frequency separation, FTx-Rx_separation (TS 38.101-1 [7] clause 5.3.6).

| ΔFTX-RX = ¦(CBWDL – CBWUL)/2¦ | C.2.3.1-Eq1 |
| --- | --- |

For the case of asymmetric NR bands and symmetric UL and DL bandwidth combinations ΔFTX-RX = 0.To meet the Tx-Rx frequency separation requirement for asymmetric NR bands were the supported overall UL bandwidth is smaller than the supported overall DL bandwidth it may not be possible to cover the full DL frequency range for all UL and DL channel bandwidth combinations. For CA when such band is only used for DL CC the full range can be used for all DL channel bandwidths.

To maximize the tested frequency range for the non-CA case the UL frequency range, as being smaller than the DL frequency range, need to be used as the starting point to calculate the uplink and downlink test frequencies.

### C.2.3.2	Determination of Low-, Mid- and High-Range for asymmetric uplink and downlink bandwidth combinations

The following procedure is used to determine the test frequencies for Low-, Mid- and High-Range for bands supporting asymmetric UL and DL bandwidth combinations.

1.	Calculate uplink carrier centre frequencies:

| FUL_LowRange = Ceil((FUL_Low + CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq1 |
| --- | --- |
| FUL_MidRange = Round((FUL_Low + BWUL_Band/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq2 |
| FUL_HighRange = Floor((FUL_Low + BWUL_Band – CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq3 |

For those FDD frequency bands with two UL sub-bands, for Low- and Mid-Range, use lower sub-band and for High-Range use higher sub-band. In these cases, BWUL_Band refers to the sub-band.

2.	Calculate the downlink frequencies:

Calculate the DL carrier centre frequencies from the UL frequencies in step 1.

| FDL_LowRange = FUL_LowRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq4 |
| --- | --- |
| FDL_MidRange = FUL_MidRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq5 |
| FDL_HighRange = FUL_HighRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq6 |

For those FDD frequency bands where TX-RX carrier centre frequency separation can have multiple values, FTx-Rx_separation shall be chosen using the following criteria:

- For Low-Range, use FTx-Rx_separation that minimizes FDL_LowRange.for each SCS.

- For Mid-Range, use FTx-Rx_separation closer to FDL_LowRange + (FDL_HighRange – FDL_LowRange)/2.

- For High-Range, use FTx-Rx_separation that maximizes FDL_HighRange for each SCS.

3.	Check that the calculated centre test frequencies in step 2 for the BWDL fits within the bands DL frequency range:

If FDL_LowRange is smaller than the lowest frequency of the band then recalculate the minimum FDL_LowRange and modify the associated FUL_LowRange, FDL_MidRange and FUL_MidRange as:

| FDL_LowRange = Ceil((FDL_Low + CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq7 |
| --- | --- |
| FUL_LowRange = FDL_LowRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq8 |
| FDL_MidRange = Round((FDL_LowRange + FDL_HighRange)/2 / ΔFRaster))* ΔFRaster | C.2.3.2-Eq8a |
| FUL_MidRange = FDL_MidRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq8b |

If FDL_HighRange is larger than the higher frequency of the band then recalculate the maximum FDL_HighRange and modify the associated FUL_HighRange, FDL_MidRange and FUL_MidRange as:

| FDL_HighRange = Floor((FDL_Low + BWDL_Band – CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq9 |
| --- | --- |
| FUL_HighRange = FDL_HighRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq10 |
| FDL_MidRange = Round((FDL_LowRange + FDL_HighRange)/2 / ΔFRaster))* ΔFRaster | C.2.3.2-Eq11 |
| FUL_MidRange = FDL_MidRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq12 |

### C.2.3.3	Determination of test frequencies for a Mid range adjacent inter-frequency cell for FR2 RRM multicell testing

The following procedure is used to determine the test frequencies for Mid-Range adjacent inter-frequency cell used for RRM FR2 NR multi-cell in NR SA and EN-DC test cases. The reason for using an adjacent inter-frequency cell to the Mid-range cell for FR2 instead of using Low- or High- Range test frequencies as used for FR1 is to reduce test system complexity.

In addition to the definition of parameters in clause C.1 the following parameters are used to calculate the test frequencies for the Mid adjacent inter-frequency cell:

| Parameter | Description |
| --- | --- |
| FMid | Carrier centre frequency (MHz) of the Mid-range cell |
| CBWMid | Channel bandwidth (MHz) of the Mid-range cell |
| CBWAdjecent | Channel bandwidth (MHz) of the adjacent cell |

1.	Calculate the Mid-range adjacent cell carrier centre frequencies:

| FMidRangeAdjecentCell = Ceil((FMid + (CBWMid + CBWAdjecent)/2) / ΔFRaster) * ΔFRaster | C.2.3.3-Eq1 |
| --- | --- |

2.	Calculate SSB and CORESET#0 parameters as described in clause C.4.

## <<< END OF CHANGES 2>>>