3GPP TSG-RAN5 Meeting # 100	R5-234949

# Tolouse, France, 21st August 2023 – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2907 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test frequencies for band n91 and asymmetric channel bandwidths |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test frequencies for band n91 and asymmetric channel bandwidths are incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test frequencies for band n91 and asymmetric channel bandwidths are corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing frequencies for band n91 will not be correct or complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.91 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on R5-234935. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

###### 4.3.1.1.1.91	Reference test frequencies for NR operating band n91

Table 4.3.1.1.1.91-1: Test frequencies for NR operating band n91 and SCS 15 kHz

| UL/DL Bandwidth combination[MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 1429.5 | 285900 | 1427.25 | 285450 | 0 | 15 | 3573 | 285870 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |  | - | - | - | - | - | - |
| 10/5 | 25 | Downlink | Low | 1429.5 | 285900 | 1427.25 | 285450 | 0 | 15 | 3573 | 285870 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 | Uplink | Low | 837 | 167400 | 832.32 | 166464 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 751.6 | 150320 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 857 | 171400 | 851.24 | 170248 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES >>>