3GPP TSG-RAN5 Meeting # 100	R5-234952

# Tolouse, France, 21st August 2023 – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2910 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test frequencies for band n94 and asymmetric channel bandwidths |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Test frequencies for band n94 and asymmetric channel bandwidths are incorrect. - Only 15kHz synchronization raster is defined for band n94 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test frequencies for band n94 and asymmetric channel bandwidths are corrected. - Symmetric channel bandwidth test frequencies corrected using appropriate synchronization raster. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing frequencies for band n94 will not be correct or complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.94 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on R5-234935. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

###### 4.3.1.1.1.94	Reference test frequencies for NR operating band n94

Table 4.3.1.1.1.94-1: Test frequencies for NR operating band n94 and SCS 15 kHz

| UL/DL Bandwidth combination[MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 1434.5 | 286900 | 1432.25 | 286450 | 0 | 15 | 3584 | 286810 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1453.89 | 290778 | 102 |  | 3687 | 294990 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 1514.5 | 302900 | 1421.53 | 284306 | 504 |  | 3787 | 302930 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 5/10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 15 | 3585 | 286830 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | 3681 | 294510 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | 3774 | 301950 | 6 | 1 | 1 (2) | 507 |
|  | 25 | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 5/15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) | 507 |
|  | 25 | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 5/20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) | 509 |
|  | 25 | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 15 | 3585 | 286830 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | 3681 | 294510 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | 3774 | 301950 | 6 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| 10/15 | 70 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) | 507 |
|  | 52 | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| 10/20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) | 509 |
|  | 52 | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| 15/15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.39 | 176078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 899.31 | 179862 | 6 |  | - | - | - | - | - | - |
| 20/20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 890 | 178000 | 880.46 | 176092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 894.38 | 178876 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.94-2: Test frequencies for NR operating band n94 and SCS 30 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 1437 | 287400 | 1432.68 | 286536 | 0 | 15 | 3591 | 287310 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1474.5 | 294900 | 1433.46 | 286692 | 102 |  | 3687 | 294990 | 6 | 0 | 3 (8) | 220 |
|  |  |  | High | 1512 | 302400 | 1326.24 | 265248 | 504 |  | 3780 | 302430 | 10 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |  | - | - | - | - | - | - |
| 10/15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 15 | 3592 | 287330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | 3678 | 294270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | 3767 | 301450 | 18 | 0 | 2 (7) | 1022 |
|  | 24 | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |  | - | - | - | - | - | - |
| 10/20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 15 | 3590 | 287290 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (5) | 1018 |
|  | 24 | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |  | - | - | - | - | - | - |
| 15/15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 15 | 3592 | 287330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | 3678 | 294270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | 3767 | 301450 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.66 | 176132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 898.5 | 179700 | 6 |  | - | - | - | - | - | - |
| 20/20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 15 | 3590 | 287290 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 890 | 178000 | 880.82 | 176164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 893.66 | 178732 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES >>>