3GPP TSG-RAN5 Meeting #100	R5-234633

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0511 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE capabilities for additional Rel-17 EN-DC configurations with PC2 band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek (Wuhan) Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, ENDC_UE_PC2_R17_NR_TDD-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of UE physical layer capabilities for Rel-17 EN-DC configurations with PC2 band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added UE physical layer capabilities for following inter-band EN-DC configurations with FR1 for two bands with a PC2 band: - DC_12A_n77A - DC_14A_n77A - DC_30A_n77A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE physical layer capabilities for Rel-17 EN-DC configurations with a PC2 band not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2B.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

Table A.4.3.2B.2.3.1-3: Inter-band EN-DC within FR1 (two bands) PC2 UE RF Baseline Implementation Capabilities

| Item |  | EN-DC configuration |  | Inter-band EN-DC within FR1 (two bands) PC2 UE RF Baseline Implementation Capabilities |  | Ref. |  | Release |  | Mnemonic |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | DC_39A_n41A |  | LTE Frequency band: 1880-1920 MHz NR Frequency band: 2496-2690 MHz |  | 38.101-3, 6.2B.1.3 |  | Rel-16 |  | pc_Band39_nrBand41_ PC2_Supp |  |  |  |
| 2 |  | DC_39A_n79A |  | LTE Frequency band: 1880-1920 MHz NR Frequency band: 4400-5000 MHz |  | 38.101-3, 6.2B.1.3 |  | Rel-16 |  | pc_Band39_nrBand79_ PC2_Supp |  |  |  |
| 3 |  | DC_41A_n79A |  | LTE Frequency band: 2496-2690 MHz NR Frequency band: 4400-5000 MHz |  | 38.101-3, 6.2B.1.3 |  | Rel-16 |  | pc_Band41_nrBand79_ PC2_Supp |  |  |  |
| 4 |  | DC_3A_n78A |  | LTE Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band: 3300-3800 MHz |  | 38.101-3, 6.2B.1.3 |  | Rel-16 |  | pc_Band3_nrBand78_ PC2_Supp |  |  |  |
| 5 |  | DC_3A_n41A |  | LTE Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band: 2496-2690 MHz |  | 38.101-3, 6.2B.1.3 |  | Rel-16 |  | pc_Band3_nrBand41_ PC2_Supp |  |  |  |
| 6 |  | DC_1A_n78A |  | LTE Frequency band: 1920-1980 MHz (UL),2110- 2170 MHz (DL) NR Frequency band: 3300-3800 MHz |  | 38.101-3, 6.2B.1.3 |  | Rel-17 |  | pc_Band1_nrBand78_ PC2_Supp |  |  |  |
| 7 |  | Void |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  | DC_8A_n78A |  | LTE Frequency band: 703-748 MHz (UL), 758-803 MHz (DL) NR Frequency band: 3300-3800 MHz |  | 38.101-3, 6.2B.1.3 |  | Rel-17 |  | pc_Band8_nrBand78_ PC2_Supp |  |  |  |
| 9 |  | DC_2A_n77A |  | LTE Frequency band: 1850-1910 MHz (UL),1930-1990 MHz (DL) NR Frequency band: 3300-4200 MHz1 |  | 38.101-3, 6.2B.1.3 |  | Rel-17 |  | pc_Band2_nrBand77_ PC2_Supp |  |  |  |
| 10 |  | DC_5A_n77A |  | LTE Frequency band: 824-849 MHz (UL),869- 894 MHz (DL) NR Frequency band: 3300-4200 MHz1 |  | 38.101-3, 6.2B.1.3 |  | Rel-17 |  | pc_Band5_nrBand77_ PC2_Supp |  |  |  |
| 11 |  | DC_13A_n77A |  | LTE Frequency band: 777-787 MHz (UL),746-756 MHz (DL) NR Frequency band: 3300-4200 MHz1 |  | 38.101-3, 6.2B.1.3 |  | Rel-17 |  | pc_Band13_nrBand77_ PC2_Supp |  |  |  |
| 12 |  | DC_66A_n77A |  | LTE Frequency band: 1710-1780 MHz (UL),2110-2200 MHz (DL) NR Frequency band: 3300-4200 MHz1 |  | 38.101-3, 6.2B.1.3 |  | Rel-17 |  | pc_Band66_nrBand77_ PC2_Supp |  |  |  |
| 13 |  | DC_12A_n77A |  | LTE Frequency band: 699-716 MHz (UL),729- 746 MHz (DL) NR Frequency band: 3300-4200 MHz1 |  | 38.101-3, 6.2B.1.3 |  | Rel-17 |  | pc_Band12_nrBand77_ PC2_Supp |  |  |  |  |
| 14 |  | DC_14A_n77A |  | LTE Frequency band: 788-798 MHz (UL),758-768 MHz (DL) NR Frequency band: 3300-4200 MHz1 |  | 38.101-3, 6.2B.1.3 |  | Rel-17 |  | pc_Band14_nrBand77_ PC2_Supp |  |  |  |  |
| 15 |  | DC_30A_n77A |  | LTE Frequency band: 2305-2315 MHz (UL),2350-2360 MHz (DL) NR Frequency band: 3300-4200 MHz1 |  | 38.101-3, 6.2B.1.3 |  | Rel-17 |  | pc_Band30_nrBand77_ PC2_Supp |  |  |  |  |
| NOTE 1: In the USA this band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.3.2B.2.3.1-3a: Inter-band EN-DC within FR1 (two bands) NR part power class UE RF Baseline Implementation Capabilities (Rel-16 and forward)

| Item |  | EN-DC configuration |  | UE Physical Layer Baseline Implementation Capabilities |  | Ref. |  | Release |  | Mnemonic |  | Supported NR part power class |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | DC_39A_n41A |  | DC_39A_n41A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-16 |  | pc_Band39_nrBand41_powerClassNRPart_r16 |  |  |  |
| 2 |  | DC_39A_n79A |  | DC_39A_n79A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-16 |  | pc_Band39_nrBand79_powerClassNRPart_r16 |  |  |  |
| 3 |  | DC_41A_n79A |  | DC_41A_n79A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-16 |  | pc_Band41_nrBand79_powerClassNRPart_r16 |  |  |  |
| 4 |  | DC_3A_n78A |  | DC_3A_n78A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-16 |  | pc_Band3_nrBand78_powerClassNRPart_r16 |  |  |  |
| 5 |  | DC_3A_n41A |  | DC_3A_n41A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-16 |  | pc_Band3_nrBand41_powerClassNRPart_r16 |  |  |  |
| 6 |  | DC_1A_n78A |  | DC_1A_n78A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-17 |  | pc_Band1_nrBand78_powerClassNRPart_r17 |  |  |  |
| 7 |  | DC_8A_n78A |  | DC_8A_n78A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-17 |  | pc_Band8_nrBand78_powerClassNRPart_r17 |  |  |  |
| 8 |  | DC_2A_n77A |  | DC_2A_n77A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-17 |  | pc_Band2_nrBand77_powerClassNRPart_r17 |  |  |  |
| 9 |  | DC_5A_n77A |  | DC_5A_n77A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-17 |  | pc_Band5_nrBand77_powerClassNRPart_r17 |  |  |  |
| 10 |  | DC_13A_n77A |  | DC_13A_n77A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-17 |  | pc_Band13_nrBand77_powerClassNRPart_r17 |  |  |  |
| 11 |  | DC_66A_n77A |  | DC_66A_n77A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-17 |  | pc_Band66_nrBand77_powerClassNRPart_r17 |  |  |  |
| 12 |  | DC_12A_n77A |  | DC_12A_n77A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-17 |  | pc_Band12_nrBand77_powerClassNRPart_r17 |  |  |  |  |
| 13 |  | DC_14A_n77A |  | DC_14A_n77A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-17 |  | pc_Band14_nrBand77_powerClassNRPart_r17 |  |  |  |  |
| 14 |  | DC_30A_n77A |  | DC_30A_n77A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-17 |  | pc_Band30_nrBand77_powerClassNRPart_r17 |  |  |  |  |

<End of Changes>