3GPP TSG-RAN5 Meeting #100	R5-234145

# Toulouse, France, 21st Aug 2023 – 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2826 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update LoggedMeasurementConfiguration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2023-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To align the local IE LoggedMeasurementConfiguration-v1700-IEs with the LoggedMeasurementConfiguration message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The local IE has been expanded. The IE table in clause 4.6.3 has been marked as void. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be aligned with the core specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1, 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### –	LoggedMeasurementConfiguration

Table 4.6.1-5AA: LoggedMeasurementConfiguration (Thres1)

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |  |  |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |  |  |  |  |
| plmn-Identity-r16 |  | plmn-Identity in USIM |  | MCC/MNC=MCC/MNC in USIM |  |  |  |
| traceId-r16 |  | ‘0EF’H |  | OCTET STRING (SIZE (3)) |  |  |  |
| } |  |  |  |  |  |  |  |
| traceRecordingSessionRef-r16 |  | ‘1A’H |  | OCTET STRING (SIZE (2)) |  |  |  |
| tce-Id-r16 |  | ‘5’H |  | OCTET STRING (SIZE (1)) |  |  |  |
| absoluteTimeInfo-r16 |  | Set to value corresponding to the absolute time when the message is sent |  | BIT STRING (SIZE (48)) |  |  |  |
| areaConfiguration-r16 |  | Not present |  |  |  |  |  |
| plmn-IdentityList-r16 |  | Not present |  |  |  |  |  |
| bt-NameList-r16 |  | Not present |  |  |  |  |  |
| wlan-NameList-r16 |  | Not present |  |  |  |  |  |
| sensor-NameList-r16 |  | Not present |  |  |  |  |  |
| loggingDuration-r16 |  | min120 |  |  |  |  |  |
| reportType CHOICE { |  |  |  |  |  |  |  |
| periodical SEQUENCE { |  |  |  |  |  | PERIODICAL |  |
| loggingInterval-r16 |  | ms2560 |  | 2.56s |  |  |  |
| } |  |  |  |  |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |  |  | EVENT |  |
| eventType-r16 CHOICE { |  |  |  |  |  |  |  |
| outOfCoverage |  | NULL |  |  |  | OUT-OF-COVERAG |  |
| eventL1 SEQUENCE { |  |  |  |  |  | EVENTL1 |  |
| l1-Threshold CHOICE { |  |  |  |  |  |  |  |
| rsrp |  | Thres1 |  | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |  |  |
| } |  |  |  |  |  |  |  |
| hysteresis |  | 0 |  |  |  |  |  |
| timeToTrigger |  | ms100 |  | 0.1s |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| loggingInterval-r16 |  | ms2560 |  | 2.56s |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |  |  |  |
| nonCriticalExtension |  | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  | LOG-MEAS-R17 |  |
| sigLoggedMeasType-r17 |  | Not present |  |  |  |  |  |
|  |  | true |  |  |  | SigLogMDT |  |
| earlyMeasIndication-r17 |  | Not present |  |  |  |  |  |
|  |  | true |  |  |  | EarlyMeas |  |
| areaConfiguration-v1700 |  | Not present |  |  |  |  |  |
| nonCriticalExtension |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| PERIODICAL | Configuration of LoggedPeriodicalReport |
| EVENT | Configuration of LoggedEventTrigger |
| OUT-OF-COVERAGE | Configuration of outOfCoverage |
| EVENTL1 | Configuration of eventL1 |
| LOG-MEAS-R17 | Configuration of LoggedMeasurement in R17 |
| SigLogMDT | Signalling based logged measurements |
| EarlyMeas | Log measurements on early measurement |

<End of modified section>

<Start of next modified section>

#### –	LocationMeasurementInfo

Table 4.6.3-65: LocationMeasurementInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LocationMeasurementInfo ::= CHOICE { |  |  |  |
| eutra-RSTD SEQUENCE (SIZE (1..maxInterRAT-RSTD-Freq)) OF EUTRA-RSTD-Info { | 1 entry |  |  |
| EUTRA-RSTD-Info[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | ARFCN-ValueEUTRA |  |  |
| measPRS-Offset | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.6.3-65A: Void

<End of modified section>