3GPP TR 37.890 V0.21.0 (2023-09)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Feasibility Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations (Release 18)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Radio, LTE, NR

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	10

3.1	Definitions	10

3.2	Symbols	10

3.3	Abbreviations	11

4	Regulatory aspects	12

4.0	General	12

4.1	ITU Region 1	12

4.1.1	Europe	12

4.1.1.1	Introduction	12

4.1.1.1a	European standardisation	14

4.1.1.2	Licensed operations	15

4.1.1.3	Unlicensed operations	15

4.1.1.3.0	Background	15

4.1.1.3.1	Coexistence aspects	16

4.1.1.3.1a	ECC Report 302	16

4.1.1.3.1b	ECC Report 316	17

4.1.1.3.1c	CEPT Report 075	18

4.1.1.3.1d	ECC Decision (20)01	18

4.1.1.3.1e	CEPT Report 073	19

4.1.1.3.2	European Commission Decision 2021/1067	20

4.1.2	Regional Commonwealth in the field of Communications (RCC) countries	20

4.1.3	UK	20

4.1.4	Saudi Arabia	21

4.1.5	United Arab Emirates	21

4.1.6	African Telecommunication Union	21

4.1.7	Morocco	22

4.1.8	Kenya	22

4.1.9	Qatar	22

4.1.10	Jordan	22

4.1.11	Russian Federation	23

4.1.12	South Africa	23

4.2	ITU Region 2	24

4.2.1	USA	24

4.2.1.1	Introduction	24

4.2.1.2	Licensed operations	25

4.2.1.3	Unlicensed operations	26

4.2.1.3.0	Overview	26

4.2.1.3.1	Standard-power operations	26

4.2.1.3.2	Low-power operations	27

4.2.1.3.3	Common requirements to the entire 6GHz band	27

4.2.2	Canada	27

4.2.3	Brazil	28

4.2.4	Peru	29

4.2.5	Chile	29

4.2.6	Mexico	29

4.2.7	Honduras	30

4.2.8	Costa Rica	30

4.2.9	Colombia	30

4.2.10	Dominican Republic	31

4.2.11	Argentina	31

4.3	ITU Region 3	32

4.3.1	China	32

4.3.2	South Korea	32

4.3.3	Hong Kong SAR	33

4.3.4	Australia	33

4.3.5	New Zealand	33

4.3.6	Japan	34

4.3.7	Malaysia	35

4.3.8	Singapore	35

4.4	ITU-R	36

4.5	Regulatory parameters comparative for license-exempt	37

5	Possible band plans	39

5.1	Licensed operations in 6 GHz	39

5.2	Unlicensed operations in 6 GHz	39

6	Conclusions	40

Annex A: Change history	41

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

A feasibility study on 6 GHz for LTE and NR operations was approved at 3GPP TSG RAN #78 [2]. It aims at investigating the existing regulatory framework in different regions for the frequency range 5.925-7.125 GHz and to monitor the ongoing work within the regulatory organisations on this frequency range for both licensed and unlicensed broadband wireless use. This will provide useful information for consideration of the frequency range 5.925-7.125 GHz for potential LTE operations and NR operations if this frequency range becomes available for operation.

# 1	Scope

The present technical report documents the regulatory aspects for the frequency range 5.925-7.125 GHz in different regions for both unlicensed and licensed operations.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-172804: "Feasibility Study on 6 GHz for LTE and NR", Ericsson, Verizon Wireless, Qualcomm Incorporated.

[3]	ITU-R Radio Regulations, Articles, Edition 2016.

[4]		FCC ONLINE TABLE OF FREQUENCY ALLOCATIONS, 47 C.F.R. § 2.106, December 13, 2017.

[5]	FCC 17-104, Notice of Inquiry, "Expanding Flexible Use in Mid-Band Spectrum Between 3.7 and 24 GHz".

[6]	Comments of IEEE 802, in GN Docket No. 17-183.

[7]	APPLE INC., BROADCOM LIMITED,,CISCO SYSTEMS, INC., FACEBOOK, INC., GOOGLE LLC, HEWLETT PACKARD ENTERPRISE, INTEL CORPORATION,MEDIATEK INC., MICROSOFT CORPORATION, and QUALCOMM INCORPORATED, in GN Docket No. 17-183.

[8] 	Reply Comments of the Wireless Internet Service Providers Association, in GN Docket No. 17-183.

[9]	Comments of Ericsson, in GN Docket No. 17-183.

[10]	Comments of T-Mobile USA, in GN Docket No. 17-183.

[11]	Comments of Verizon, in GN Docket No. 17-183.

[12]	Reply Comments of the Satellite Industry Association, in GN Docket No. 17-183.

[13]	Reply Comments of the Fixed Wireless Communications Coalition, in GN Docket No. 17-183.

[14]	Comments of Dynamic Spectrum Alliance, in GN Docket No. 17-183.

[15]	Comments of the National Spectrum Management Association, in GN Docket No. 17-183.

[16]	Comments of CTIA, in GN Docket No. 17-183.

[17]	Reply Comments of Cisco Systems, Inc., in GN Docket No. 17-183.

[18]	Reply Comments of WI-FI Alliance, in GN Docket No. 17-183.

[19]	PART 15 - Radio Frequency Devices, Title 47 of electronic Code of Federal Regulations.

[20]	The European Table of Frequency Allocations and applications in the frequency range 8.3 kHz and 3000 GHz (ECA Table), October 2017.

[21]	RSCOM17-53rev1- Mandate to CEPT to study and identify harmonised compatibility and sharing conditions for wireless access systems including radio local area networks in the band 5925-6425 MHz for the provision of wireless broadband services.

[22]	ETSI TR 103 524 System Reference document (SRdoc), "Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) in the band 5 925 MHz to 6 725 MHz", v1.1.1, October 2018.

[23]	CEPT/ERC/REC 74-01: "Unwanted Emissions in the Spurious Domain".

[24]	ECC Report 302, Sharing and compatibility studies related to Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) in the frequency band 5925-6425 MHz, May 2019.

[25]	FCC Notice of Proposed Rulemaking. FCC 18-147. October 24, 2018.

[26]	ETSI TR 103 612, "IMT cellular networks; Mobile/Fixed Communication Network (MFCN) in the frequency range 6 425 - 7 125 MHz", v1.1.1, December 2019.

[27]	ETSI TR 103 631, "Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) in the band 6 725 MHz to 7 125 MHz", v1.1.1, March 2019.

[28]	ECC Report 302, "Sharing and compatibility studies related to Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) in the frequency band 5925-6425 MHz".

[29]	CCSA-TC5-WG8-2019-003 Project Proposal on the feasibility study of IMT system using 5925-7125MHz frequency band, http://www.ccsa.org.cn/tc/meeting.php?meeting_id=6243#

[30]	World Radiocommunication Conference 2019 (WRC-19) Provisional Final Acts, ITU-R https://www.itu.int/dms_pub/itu-r/opb/act/R-ACT-WRC.13-2019-PDF-E.pdf

[31]	Report and order and further notice of proposed rulemaking, FCC 20-51.

[32]	ECC Report 316, “Sharing studies assessing short-term interference from Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) into Fixed Service in the frequency band 5925-6425 MHz", 21 May 2020

[33]	CEPT Report 075, "to study feasibility and identify harmonised technical conditions for Wireless Access Systems including Radio Local Area Networks in the 5925-6425 MHz band for the provision of wireless broadband services"; Report B: Harmonised technical parameters for WAS/RLANs operating on a coexistence basis with appropriate mitigation techniques and/or operational compatibility/coexistence conditions, operating on the basis of a general authorisation, November 2020.

[34]	ECC Decision (20)01; “On the harmonised use of the frequency bands 5945 to 6425 MHz for the implementation of Wireless Access Systems including Radio Local Area Networks (WAS/RLAN)”, November 2020.

[35]	ETSI TR 103 524, "System Reference document (SRDoc); Wireless access systems including radio local area networks (WAS/RLANs) in the band 5925 MHz to 6725 MHz".

[36]	EN 303 687, "6 GHz RLAN Harmonised Standard for access to radio spectrum", Draft

[37]	RP-201438, LS to 3GPP TSG RAN – ENG (18th meeting of the RCC Commission on Spectrum and Satellite Orbits),

[38]	CEPT Report 073, "to study feasibility and identify harmonised technical conditions for Wireless Access Systems including Radio Local Area Networks in the 5925-6425 MHz band for the provision of wireless broadband services"; Report A: Assessment and study of compatibility and coexistence scenarios for WAS/RLANs in the band 5925-6425 MHz, Approved on 6 March 2020 by ECC.

[39]	Korea’s Ministry of Science and ICT, "Technical standards for radio equipment for radio stations", URL: https://www.law.go.kr/admRulLsInfoP.do?admRulSeq=2100000196974

[40]	Ofcom, "Improving spectrum access for Wi-Fi; Spectrum use in the 5 GHz and 6 GHz bands", July 2020, URL: https://www.ofcom.org.uk/__data/assets/pdf_file/0036/198927/6ghz-statement.pdf

[41]	Communications & Information Technology Commission, "Spectrum Outlook for Commercial and Innovative Use 2021- 2023", January 2021, URL: https://www.citc.gov.sa/ar/new/publicConsultation/Documents/Spectrum%20Outlook%20for%20Commercial%20and%20Innovative%20(2021-2023).pdf

[42]	Innovation, Science and Economic Development Canada, "Decision on the Technical and Policy Framework for Licence-Exempt Use in the 6 GHz Band", May 2021, URL: https://www.ic.gc.ca/eic/site/smt-gst.nsf/eng/sf11698.html

[43]	National Telecommunications Agency (ANATEL), "ACT NO. 1306", February 2021, URL: https://sei.anatel.gov.br/sei/modulos/pesquisa/md_pesq_documento_consulta_externa.php?eEP-wqk1skrd8hSlk5Z3rN4EVg9uLJqrLYJw_9INcO7uvjUt3vSOwT_4Z5fukj9yIzPErY4KWH5cpE9W_9hcTZkCG-vLPIdpXyuhgMG-L9M-uBLoSdAAXO0clb3SIt1i

[44]	Ministerio de Transportes y Comunicaciones, "Resolución Ministerial N° 373-2021-MTC/01", April 2021, URL: https://cdn.www.gob.pe/uploads/document/file/1861732/Resolución%20Ministerial%20nro%20373-2021-MTC/01.pdf

[45]	MINISTERIO DE TRANSPORTES Y TELECOMUNICACIONES; SUBSECRETARÍA DE TELECOMUNICACIONES, "RESOLUCIÓN 1985 EXENTA", October 2020, URL: https://www.bcn.cl/leychile/navegar?idNorma=1109333&idParte=9841504&idVersion=&r_c=6

[46]	RP-210957, Liaison statement to 3GPP TSG RAN on inclusion of the frequency band 6425-7125 MHz in 3GPP specification for 5G-NR/IMT-2020 systems, Regional Commonwealth in the filed of Communications.

[47]		COMMISSION IMPLEMENTING DECISION (EU) 2021/1067, on the harmonised use of radio spectrum in the 5 945-6 425 MHz frequency band for the implementation of wireless access systems including radio local area networks (WAS/RLANs), 17 June 2021.

[48]		The National Telecommunications Commission (CONATEL) of Honduras, "Resolution NR 003/21", March 2021, URL: http://www.conatel.gob.hn/doc/Regulacion/resoluciones/2021/NR003-21.pdf

[49]		The Superintendencia de Telecomunicaciones (SUTEL), "DECRETO EJECUTIVO N° 42924-MICITT", April 2021, URL: https://www.imprentanacional.go.cr/pub/2021/04/30/ALCA87_30_04_2021.pdf

[50]	Communications & Information Technology Commission, "Radio Spectrum Allocation and Use Regulation for WLAN Application", June 2021, URL: https://www.citc.gov.sa/en/new/publicConsultation/Documents/144207-en.pdf

[51]	Communications & Information Technology Commission, “Public Consultation on Spectrum Light Licensing”, August 2021, URL: https://www.citc.gov.sa/ar/new/publicConsultation/Documents/EN_PublicConsultationonLightLicensing-144301.pdf

[52]	Communications & Information Technology Commission, “WLAN regulations”, January 2022, URL: https://www.citc.gov.sa/en/mediacenter/pressreleases/Pages/2022022101.aspx

[53]	UAE Telecommunication Regulatory Authority, "Ultra-Wide Band and Short Range Devices", Version 4.0, December 2020, URL: https://tdra.gov.ae/-/media/About/regulations-and-ruling/EN/UWB-and-SRD-Regs-V4-0-pdf.ashx

[54]	National Telecommunications Regulatory Agency (ANRT), "Decision ANRT/DG/N°07/21", May 2021, URL: https://www.anrt.ma/sites/default/files/decision_a2fp_-vf-_mod_07.05.2021.pdf

[55]	MINISTERIO DE TRANSPORTES Y TELECOMUNICACIONES; SUBSECRETARÍA DE TELECOMUNICACIONES, "RESOLUCIÓN 1325 EXENTA", July 2021, URL: https://www.bcn.cl/leychile/navegar?idNorma=1162978&idParte=10255091

[56]	Agencia Nacional del Espectro (ANE), "Por medio de la cual se modifica la Resolución 105 de 2020 y se actualiza el Cuadro Nacional de Atribución de Bandas de Frecuencia", October 2021, URL: https://www.ane.gov.co/Documentos%20compartidos/ArchivosDescargables/Normatividad/Planeacion_del_espectro/RESOLUCIÓN%20No%20000105%20DE%2027-03-2020(1)%20(1).pdf

[57]	African Telecommunication Union, "ATU-R Recommendation 005-0", July 2021, URL: https://www.atuuat.africa/wp-content/uploads/2021/08/En_ATU-R-Recommendation-005-0.pdf

[58]	African Telecommunication Union, "2nd ATU Preparatory Meeting for WRC-23 (APM23-2); Agenda Item 1.2", September 2021, URL: https://docs.google.com/document/d/1u0Oz6o3gnH6oTGY-sE6_BTarNf2RUuSx/edit

[59]		RCC Recommendation 1/21 "Harmonization of the technical conditions for 5G-NR / IMT-2020 systems in the RCC countries in the frequency band 6 425-7 125 MHz or in its portions" (RP-213605).

[60]	Recommendation ITU-R SM.329-12, Unwanted emissions in the spurious domain, 09/2012.

[61]	The Australian Communications and Media Authority, "Radiocommunications (Low Interference Potential Devices) Class Licence Variation 2022 (No. 1)", March 2022, URL: https://www.legislation.gov.au/Details/F2022L00249/Html/Text

[62]	HKCA 1081, "PERFORMANCE SPECIFICATION FOR RADIOCOMMUNICATIONS APPARATUS OPERATING IN THE 6 GHz BAND FOR WIRELESS LOCAL AREA NETWORK", April 2022, URL: https://www.ofca.gov.hk/filemanager/ofca/en/content_401/hkca1081.pdf

[63]	Radio Spectrum Management, "WLAN use in the 6 GHz Band: Outcomes and Other Updates", August 2022, URL: https://www.rsm.govt.nz/projects-and-auctions/completed-projects/wlan-use-in-the-6-ghz-band/

[64]	Ministry of Internal Affairs and Communications, "Technical conditions for introduction of 6GHz band wireless LAN", April 2022, URL: https://www.soumu.go.jp/menu_news/s-news/01kiban12_02000142.html

[65]	Malaysian Communications and Multimedia Commission (MCMC), "Class Assignment No. 1 of 2022", January 2022, URL: https://www.mcmc.gov.my/skmmgovmy/media/General/CA-No-1-of-2022_-signed_19012022.pdf

[66]	Communications Authority of Kenya (CAK), "Guidelines On the Use of Radiofrequency Spectrum by Short Range Devices", June 2022, URL: https://www.ca.go.ke/wp-content/uploads/2022/07/Guidelines-on-the-Use-of-Radiofrequency-Spectrum-by-Short-Range-Devices-2022.pdf

[67]	Communications Regulatory Authority (CRA), "Class License for the use of RLAN devices over 5925-6425 MHz Band", April 2022, URL: https://www.cra.gov.qa/-/media/System/D/2/5/8/D258CF18B83A5613B0D590193CB799CB/Class-License-WIFI-6E-Final-English-032022--V3.ashx

[68]	The Hashemite Kingdom of Jordan Telecommunications Regulatory Commision (TRC), "Short Range Station License", May 2022, URL: https://trc.gov.jo/EchoBusV3.0/SystemAssets/PDF/AR/الطيف%20الترددي%20الراديوي/ترخيص%20الترددات/التعليمات%20والاجراءات/20be3e3e-3013-47cf-8cbc-b0280118acc0_SRD%20license.pdf

[69]	<void>

[70]	MINISTERIO DE TRANSPORTES Y TELECOMUNICACIONES; SUBSECRETARÍA DE TELECOMUNICACIONES, "RESOLUCIÓN 2844 EXENTA  MODIFICA RESOLUCIÓN N° 1.985 EXENTA, DE 2017, DE LA SUBSECRETARÍA DE TELECOMUNICACIONES", September 2022, URL: https://www.bcn.cl/leychile/navegar?idNorma=1181305&idVersion=2022-09-14

[71]	Agencia Nacional del Espectro (ANE), "RESOLUCIÓN No. 000737", November 2022, URL: https://www.mintic.gov.co/portal/715/articles-273182_recurso_1.pdf

[72]	Instituto Dominicano de las Telecomunicaciones (INDOTEL), "RESOLUCIÓN NÚM. 082-2022", September 2022, URL: https://transparencia.indotel.gob.do/media/217225/res_082_2022.pdf

[73]	Korea’s Ministry of Science and ICT, "Technical standards for radio equipment for radio stations", URL: https://www.law.go.kr/admRulLsInfoP.do?admRulSeq=2100000217697&vSct=202275

[74]	Ministry of the digital development and communication, "Заседание ГКРЧ от 23 декабря 2022 г. (протокол № 22-65)", December 2022, URL: https://digital.gov.ru/ru/documents/8628/

[75]	Instituto Federal de Telecomunicaciones, "El pleno del IFT clasifica la banda de 5925-6425 MHz como espectro libre", February 2023, press release 013/2023

[76]		Ente Nacional de Comunicaciones (ENACOM), "Resolución 756/2023", May 2023, URL: https://www.boletinoficial.gob.ar/detalleAviso/primera/287126/20230524

[77]	Ministry of Industry and Information Technology, "中华人民共和国无线电频率划分规定", July 2023, URL: https://www.miit.gov.cn/cms_files/filemanager/1226211233/attach/20236/9086700eed45430bafe236efd1096fd3.pdf

[78]	Public Consultation on Assignment Arrangements of Spectrum in 6/7 GHz Band Launched. URL: https://www.coms-auth.hk/en/media_focus/press_releases/index_id_2325.html

[79]	Independent Communications Authority of South Africa (ICASA), "NOTICE TO AMEND ANNEXURE B OF THE RADIO FREQUENCY SPECTRUM REGULATIONS", May 2023, URL: https://www.icasa.org.za/uploads/files/Notice-to-amend-Annexure-B-of-the-Radio-Frequency-Spectrum.pdf

[80]	The Infocomm Media Development Authority (IMDA), "Technical Specification for Short Range Devices", September 2023, URL: https://www.imda.gov.sg/~/media/imda/files/regulation%20licensing%20and%20consultations/ict%20standards/telecommunication%20standards/radio-comms/imdatssrd.pdf

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AP	Access Point

BAS		Broadcast Auxiliary Service

CARS		Cable Television Relay Service

CBTC	Communication-based Train Control systems

CEPT		European Conference of Postal and Telecommunications

CL	Client device

DA2GC	Direct Air-to-Ground Communications

EC		European Commission

ECC		European Electronic Communications Committee

EESS		Earth Exploration-Satellite Service

eLAA		enhanced LAA

ERC		European Radio-communication Commitee

ESV		Earth Stations on Vessel

FCC		Federal Communication Commission

FDP	Fractional Degradation in Performance

FM		Frequency Management – ECC Working Group

FS		Fixed Service

FSS		Fixed Satellite Service

IMT		International Mobile Telecommunications

ITS		Intelligent Transport System

LAA		Licensed Assisted Access

LPI	Low Power Indoor

LPR		Level Probing Radar

LTTS 		Local TV Transmission Service

MBR		Maritime Broadband Radio Links

MCL	Maximum Coupling Loss

NOI		Notice of Inquiry

NPRM		Notice of Proposed Rule Making

NR		New Radio

NR-U		NR Unlicensed

PMSE		Programme-Making and Special Events

RLAN		Radio Local Area Network

RR		Radio Regulation

SDARS	Sirius Satellite Radio Service

SE		Spectrum Engineering – ECC Working Group

SP	Standard Power

SRD		Short Range Device

TLPR		Tank Level Probing Radar

UNII		Unlicensed National Information Infrastructure

UWB		Ultra Wideband

VLBI		Very Long Baseline Interferometry (Radio Astronomy)

VLP	Very Low Power

WAS		Wireless Access System

WIA		Wireless Industrial Applications

# 4	Regulatory aspects

## 4.0	General

The Mobile allocation in the frequency range 5.925-7.125 GHz, which is allocated at International level to Mobile services on a primary basis in all Regions, as per ITU Table of frequency allocation [3] summarized in Table 4.0-1, has not yet been implemented at National level in most countries. On the contrary, this frequency range is already used by several services which may impose constraints on any new users.

Table 4.0-1: ITU allocation to services in 5.925-7.145 GHz

| Frequency range [MHz] | Allocations |  |  |
| --- | --- | --- | --- |
|  | Region 1 | Region 2 | Region 3 |
| 5 925 - 6 700 | FIXED 5.457 FIXED SATELLITE (Earth-to-space) 5.457A 5.457B MOBILE 5.457C 5.149 5.440 5.458 |  |  |
| 6 700 - 7 075 | FIXED FIXED SATELLITE (Earth-to-space) (space-to-Earth) 5.441 MOBILE 5.458 5.458A 5.458B |  |  |
| 7 075 – 7 145 | FIXED MOBLE 5.458 5.459 |  |  |

The FCC notice of proposed rulemaking is available in [25].

The CEPT 5925-6425 MHz regulatory requirements are available in [34].

## 4.1	ITU Region 1

### 4.1.1	Europe

#### 4.1.1.1	Introduction

According to the European Common Allocations Table (ECA Table) [20] in ERC Report 25 and the ECO Frequency Information System (EFIS), only the, 5.925-6.700 GHz frequency range is allocated on a primary basis to MS-Mobile Service, FS- Fixed Service and FSS-Fixed Satellite Service (earth to space), and on a secondary basis to Earth Exploration Satellite Service - EESS (passive); 6.700-7.075 GHz frequency range is allocated on a primary basis to FS and FSS (earth to space, space to earth), and on a secondary basis to EESS (passive); 7.075-7.145 GHz is allocated on a primary basis to FS and on a secondary basis to EESS (passive). In Russian federation, according to RR 5.459, 7.100-7.125 GHz is allocated to space operation service (Earth-to-space) on a primary basis. Generic SRD (UWB), LPR and TLPR also operate in 5.925-7.125 GHz frequency ranges on a no protection and no interference basis. More details are provided in Table 4.1.1.1-1 [20].

Table 4.1.1.1-1: European Allocation in 6 GHz

| Frequency Range [MHz] | Allocation | Applications/Notes |
| --- | --- | --- |
| 5 925 – 6 700 | MOBILE FIXED FIXED-SATELLITE (EARTH TO SPACE) Earth Exploration Satellite (Passive) 5.149 5.440 5.458 | ESV/Within 5925-6425 MHz FSS Earth stations/Priority for civil networks Fixed/Point-to-point Passive sensors (satellite)/For sea surface temperature, sea surface wind speed and soil moisture measurements Radio astronomy/Spectral line observations, VLBI Radiodetermination applications/Within 4500-7000 MHz for TLRP application and 6000-8500 MHz for LPR applications UWB applications/Generic UWB as well as UWB on-board aircraft regulation within 6.0-8.5 GHz |
| 6 700 – 7 045 | FIXED FIXED-SATELLITE (EARTH TO SPACE) (SPACE TO EARTH) 5.441 Earth Exploration Satellite (Passive) 5.458 5.458A 5.458B | FSS Earth stations/Within 6725-7025 MHz. Priority for civil networks Feeder links Fixed/ Point-to-point PMSE/ portable or mobile wireless video, cordless cameras, temporary P-t-P video links in 7-8.5 GHz tuning range Passive sensors (satellite)/For sea surface temperature, sea surface wind speed and soil moisture measurements Radiodetermination applications/Within 4500-7000 MHz for TLRP application and 6000-8500 MHz for LPR applications UWB applications/Generic UWN as well as UWB on-board aircraft regulation within 6.0-8.5 GHz |
| 7 045 – 7 145 | FIXED Earth Exploration Satellite (Passive) 5.458 | Fixed/ Point-to-point PMSE/ portable or mobile wireless video, cordless cameras, temporary P-t-P video links in 7-8.5 GHz tuning range Passive sensors (satellite) / For sea surface temperature, sea surface wind speed and soil moisture measurements Radiodetermination applications/Within 6000-8500 MHz for LPR applications UWB applications/Generic UWB as well as UWB on-board aircraft regulation within 6.0-8.5 GHz |

Allocation for services (ITS, UWB, WIA, non-specific SRDs) in adjacent bands is shown in Table 4.1.1.1-2 and 4.1.1.1-3.

Table 4.1.1.1-2: Adjacent services below 5.925 GHz

| Frequency Range [MHz] | Allocation | Applications/Notes |
| --- | --- | --- |
| 5 850 – 5 925 | FIXED FIXED-SATELLITE (EARTH TO SPACE) MOBILE 5.150 | BFWA/Within 5725-5875 MHz DA2GC/Within 5855-5875 MHz FSS Earth stations/ priority for civil networks ISM/Within 5725-5875 MHz MBR/Within 5852-5872 MHz and 5880-5900 MHz ITS/ Within 5875-5925 MHz and 5855-5875 MHz. Traffic safety applications within 5875-5905 MHz NON-specific SRDs/Within 5725-5875 MHz Radiodetermination applications/ Within 4500-7000 MHz for TLPR applications WIA/Within 5725-5875 MHz |

Table 4.1.1.1-3: Adjacent services above 7.145 GHz

| Frequency Range [MHz] | Allocation | Applications/Notes |
| --- | --- | --- |
| 7 145 – 7 190 | FIXED MOBILE SPACE RESEARCH (DEEP SPACE) (EARTH TO SPACE) Space Operation (Earth-to-space) 5.458 | Fixed/ Point-to-point PMSE/ portable or mobile wireless video, cordless camera, temporary video links in 7-8.5 GHz tuning range Radiodetermination applications/ within 6.0-8.5 GHz for LPR applications UWB applications/Generic UWB. On-board aircraft regulations within 6.0-8.5 GHz |

#### 4.1.1.1a	European standardisation

There are a number of ETSI deliverables that pertain to the 5925-7125 MHz frequency range. The relationship between these ETSI deliverables, System Reference Document (SRDoc), Technical Report (TR), the EC Mandate to CEPT, and Harmonized Standard (EN), is as shown in Figure 4.1.1.1a-1.

Figure 4.1.1.1a-1: Relationship between ETSI Deliverables and EC 6GHz Mandate.

The ETSI deliverables are as follows:

-	ETSI TR 103 524 [22]: this document provides information on the intended applications, the technical parameters, mitigation techniques, the relation to the existing spectrum regulation and additional new radio spectrum requirements for Wireless access systems including radio local area networks (WAS/RLANs). The present document presents justification on the need for additional new license exempt spectrum for Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) in the 5 925 MHz to 6 725 MHz frequency range to support future growth, particularly of WAS/RLANs with greater utilization and efficiency of this spectrum. This document was also suggesting considering the additional 6 425-6 725 MHz frequency range and not only the 5 925-6 425 MHz frequency range, which was opposed by two Administrations.

-	ETSI TR 103 631 [27]: this document provides information on the intended applications, the technical parameters, mitigation techniques, the relation to the existing spectrum regulation and additional new radio spectrum requirements for technology neutral wireless access systems including radio local area networks (WAS/RLANs) capable of operating in the 6 725 MHz to 7 125MHz range.

-	ETSI TR 103 612 [26]: this document considers the possibility of sharing the frequency range 6 425 - 7 125 MHz between the incumbent services and MFCN (Mobile/Fixed Communication Network) services. It was concluded that the frequency range 6 425 - 7 125 MHz has the potential to support the growing demand for mobile broadband and other use cases. Further feasibility studies are required to assess the challenges around the co-existence and compatibility of 5G NR technology with the incumbent services in the same frequency range and neighbouring frequencies respectively.

-	Draft ETSI EN 303 687 [36]: this Harmonised standard will specify technical characteristics and methods of measurements for Wireless access systems including radio local area networks (WAS/RLANs) operating in the band 5 925 MHz to 6 425 MHz.

#### 4.1.1.2	Licensed operations

ETSI TFES has finalized a Technical Report [26] on the "possibility of sharing the band 6425-7125 MHz between the incumbent services and MFCN (Mobile/Fixed Communication Network) services.". The summary of the document indicates that "The present document does not include the co-existence studies to analyse the risk of interference between incumbent services and the 5G NR technology, which can be the next step in the process of feasibility study".

The conclusions, state that "… further feasibility studies are required to assess the challenges around the co-existence and compatibility of 5G NR technology with the incumbent services in the same frequency range and neighbouring frequencies respectively. The present document can be considered as the base for such studies to determine the technical conditions to use the MFCN in this band".

#### 4.1.1.3	Unlicensed operations

##### 4.1.1.3.0	Background

In December 2017, EC Radio Spectrum Committee finalized the Mandate to CEPT [21] to study and identify harmonised compatibility and sharing technical conditions for a sustainable and efficient use on a shared basis of the frequency range 5.925-6.425 GHz for Wireless Access Systems including Radio Local Area Networks (WAS/RLANs).

CEPT created two project teams, ECC SE 45 (coexistence) and ECC FM 57 (regulatory). ECC SE 45 was tasked to undertake compatibility and sharing studies in the 5.925-6.425 GHz frequency range to support ECC FM 57, and the latter defined the regulatory rules for the band. The scope of the work items in these groups was the introduction of low power wireless access systems (including RLAN) in the frequency range 5.925-6.425 GHz under a licence-exempt / general authorisation regulatory regime, ensuring certainty of continued operation, development and protection of existing services (Fixed Services (FS), Fixed Satellite Service (FSS)) considering RR 5.440 and 5.458.

Currently, no WAS/RLAN or 5G NR coexistence studies have been undertaken within CEPT for the 6.425-7.125 GHz range.

ETSI has published the TR 103 524 [22] providing information on the technical parameters for WAS/RLANs to support the CEPT Work Items activities covering the frequency range 5.925 GHz to 6.425 GHz. In addition, it contains a request for considering additional frequencies up to 6.725 GHz. Furthermore, ETSI published TR 103 631 [27] providing information on the intended applications, the technical parameters, mitigation techniques, the relation to the existing spectrum regulation and additional new radio spectrum requirements for technology neutral wireless access systems including radio local area networks (WAS/RLANs) capable of operating in the 6725 MHz to 7125MHz range.

The ECC Report 302 [24] on sharing and compatibility studies related to Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) in the frequency band 5925-6425 MHz, developed by ECC SE45, used the technical characteristics specified in the ETSI TR 103 524 [22] for RLAN as starting point, and the technical parameters provided from ECC SE19 and ECC SE40 for FS and FSS respectively. It is to be noted that some of the technical parameters (for example max EIRP, TX unwanted emissions, etc.) for RLAN in the ETSI TR 103 524 may differ from the final rules for the band, depending on the results of the compatibility studies done by CEPT.

##### 4.1.1.3.1	Coexistence aspects

##### 4.1.1.3.1a	ECC Report 302

ECC Report 302 [24] was approved during ECC WGSE#82 meeting (May 27th-29th). This report was triggered by EC Mandate on 6GHz and deals with sharing and compatibilities studies between WAS/RLAN and existing incumbent services or applications in the 5925-6425 MHz band and in the adjacent bands under a non-protected basis, ensuring certainty of continued operation, development and protection of existing incumbent services. The work is related to the EC Mandate to CEPT on WAS/RLAN in the 5925-6425 MHz band with the aim of exploring the availability of additional spectrum for the provision of internet-based services with increased data capacity and speed.

This Report lists the various coexistence studies that have been done in ECC SE45.Three different scenarios (Low, Mid and High) have been investigated considering busy hour and market adoption factors. The Report provides some preliminary conclusions on coexistence with:

-	Fixed Service

Three types of studies have been done to evaluate interference from RLAN to FS networks.

The first study, based on MCL analysis, highlights two different types of areas where RLAN could possibly exceed the protection criteria (a circular area with small radius and a peak area largely down the boresight) and reveals critical scenarios without concluded about the likelihood of them.

The second study is a Monte Carlo study and analyses fixed links in UK and Netherlands. It shows that the long-term interference criterion is met (I/N value of -10dB not exceeded for more than 20% of time as advised by Recommendation ITU-R F.758). Another complementary protection criterion (FDP < 10%) was also studied and was exceeded for some of the link in the UK, some of those scenarios where exceedance occurred was considered as highly improbable. By restricting deployment to indoor only with a maximum EIRP of 200mW, almost all cases would be resolved. Based on these conditions, the conclusion is that sharing with FS is feasible.

The third study is based on three existing FS receivers in France and shows that outdoor RLAN operating with an EIRP of 1W would create interference from a large area around the FS link. But restricting the usage to indoor with an EIRP of 250mW would considerably reduce them. The statistically study based on Monte Carlo approach indicates that the long term protection criterion is not exceeded.

-	Fixed Satellite Service

A representative set of FS satellites with coverage over Europe was used for those two studies.

The first study is based on Monte Carlo methodology and considers the Mid scenario. Results show feasibility on spectrum sharing based on the taken assumptions.

The second study is based on static analysis considering average values for the Low, Mid and High scenarios for RLAN deployment model in Europe by 2025. Results show the protection criterion is satisfied except for the High scenario, the calculated interference level was found out to be close to the FSS protection criterion used in the study.

Further investigation was done to investigate the sensitivity of increasing the proportion of outdoor WAS/RLAN usage above the agreed parameters – up to 5% outdoor use, The results highlight the risk of excess interference for the high scenario with the most sensitive satellite should higher power outdoor usage increase beyond the agreed parameters (3% outdoor) To compensate for this, the report is suggesting limiting RLAN usage to indoor with an EIRP limit.

-	Road Intelligent Transportation Systems

A coexistence study, using the minimum coupling loss methodology, was done to assess the out-of-band emissions on Road-ITS service below 5925 MHz (adjacent band), considering a protection criterion of -6 dB I/N.

The results show that with appropriate WAS/RLAN Out-of-Band-Emissions limits road-ITS is protected plus with indoor usage obviously providing even better coexistence. Depending on the scenario, the RLAN out of band emissions should have a limit between -69 dBm/MHz and -36 dBm/MHz (main-lobe case) and between -59 dBm/MHz and -26 dBm/MHz (side-lobe case).

The most stringent requirement would be when the ITS antenna is integrated inside the vehicle (which might be an unlikely scenario).

The RLAN indoor usage scenario resulted in the least stringent Out of Band emissions below 5925 MHz.

-	Communication-based Train Control systems

Two types of study were done to investigate CBTC protection: one with CBTC operating in the adjacent band, and the other one with CBTC operating in-band.

The study with CBTC in the adjacent band shows that RLAN out of band emission of -29 dBm/5 MHz would be sufficient to protect CBTC if RLAN is deployed indoor. If RLAN would operate indoor starting at 5940 MHz, an in-band EIRP of 21.5 dBm/20 MHz would comply with CBTC blocking requirements. Also, with RLAN portable devices (outdoor scenario), the study shows that RLAN out of band emission of -42dBm/5 MHz and an EIRP of 4.7 dBm/20 MHz would ensure CBTC operation.

The in-band study for the S-train in Copenhagen concludes that the distance between RLAN device and CBTC receiver should be in the range of 180 and 600m to avoid interference on the CBTC receiver. Such distance may not be guaranteed so dedicated mitigations techniques, to be applied locally (i.e. for Copenhagen) would need to be specified.

-	Radio Astronomy

The number of Radio Astronomy sites in this frequency range is limited (around 19); compatibility would be then addressed on a case by case basis, at the national level. The Report suggests than an I/N threshold can be used to derive a zone around the RAS site following the applicable ITU-R Recommendations.  The zone contours could be considered as coordination or exclusion zones, which would need to be managed by the Regulator.

-	Ultra Wide Band

The introduction of WAS/RLANs could have an impact on the applications and users of UWB systems that can only operate over the frequency range under study in this Report. An MCL analysis studied a range of EIRP levels (0 dBm to 30 dBm) has shown that an individual RLAN would cause 3 dB sensitivity loss from between 30 m and 946 m away for location tacking systems and from 7 to 212m for UWB sensing applications. Aggregate interference studies using Monte Carlo simulations and RLAN activity factor shows that the probability exceeds 3 dB from 0.0024% to 3.3% depending on the scenario for location tracking devices.  For UWB sensing applications the probability exceeds 0.042% to 1.7%.

NOTE:	UWB operates under a non-interference regulatory regime and as such cannot claim protection from interference

##### 4.1.1.3.1b	ECC Report 316

The ECC Report 316 [32], presented two studies expanding upon the studies in ECC Report 302 [27] by assessing the possible short-term interference impact of the WAS/RLANs into point-to-point Fixed Service (FS) links:

-	Site-general Monte Carlo analysis;

-	Site-specific sensitivity analysis on LPI deployment assumptions based on radio coverage.

A combination of low power indoor (LPI) WAS/RLANs operating at power levels up to 200 mW and outdoor Very Low Power (VLP) portable WAS/RLANs up to 25 mW is used in the studies

Results of the site-general Monte Carlo analysis show that the percentage of events for which the short- term threshold (I/N = 19 dB) is not exceeded for more than 4.5 ⋅ 10-4% (which was used as a proxy for short- term protection criterion) for the studied combinations of FS antenna heights, population densities, and WAS/RLAN deployment. Various VLP outdoor percentages of WAS/RLAN deployment show no effect on the overall interference impact.

Results of the site-specific sensitivity analysis on LPI deployment assumptions based on radio coverage have shown that the size of the area from where a WAS/RLAN may create interference to the FS receiver is sensitive among others to the clutter loss, building entry loss, WAS/RLAN height and position (relative to FS main beam). For the WAS/RLAN RF activity factor of 2%, it should be noted that for the combinations where some positions depict the exceedance of the short-term threshold I/N = 19 dB, this interference may be created for more than 4.5 ⋅ 10-4% of the time in any month (for errored seconds); the probability of this occurring has not been studied.

##### 4.1.1.3.1c	CEPT Report 075

The CEPT Report 075 [33] addresses the possible designation of the frequency band 5925-6425 MHz for the implementation of Wireless Access Systems (WAS) including Radio Local Area Networks (RLAN), in accordance with Task 2 of the EC Mandate to CEPT.

Based on ECC Report 316 and the complementary studies done in parallel with the public consultation, the in-band Power Spectral Density (PSD) for VLP devices and out-of-band emission limits below 5935 MHz for LPI and VLP devices were  further assessed.

The recommended framework distinguishes the following use cases for use by WAS/RLAN in the 5945-6425 MHz frequency band:

-	Low power indoor (LPI) use with max 200 mW EIRP, only indoor.

-	Very low power (VLP) portable use with max 25 mW EIRP, indoor and outdoor.

The Report proposes that WAS/RLAN transmissions should be constrained to above 5945 MHz to take into account CBTC operations.

CEPT concludes that coexistence between WAS/RLAN operating in the 5945-6425 MHz band and existing services and systems within the band and in adjacent band would be technically feasible under the generic technical conditions detailed the annex of the Report and also given in ECC Decision(20)01, see clause 4.1.1.3.1d.

In addition, to maintain the protection of point-to-point FS, national administrations should consider an effective enforcement of the indoor restriction for LPI.

##### 4.1.1.3.1d	ECC Decision (20)01

The ECC Decision (20)01 [34] harmonized the 5.945-6.425 GHz frequency band for WAS/RLAN use, restricted to:

-	Indoor use only for LPI equipment. This LPI equipment could be:

-	LPI Access Point or bridge, power supplied from a wired connection, not battery powered, with an integrated antenna.

-	LPI client connected to a LPI access point or another LPI client. This LPI client could be battery powered or not.

-	Indoor and outdoor use for VLP equipment. The VLP device is a portable device, drones are prohibited.

Those equipment shall support an adequate spectrum sharing mechanism.

Moreover, LPI equipment shall comply with the limits captured in Table 4.1.1.3.1d-1 and VLP equipment with limits captured in Table 4.1.1.3.1d-2 [34].

Table 4.1.1.3.1d -1: Low Power Indoor (LPI) WAS/RLAN devices

| Parameter | Technical conditions |
| --- | --- |
| Permissible operation | Restricted for indoor use only (including trains where metal coated windows (note 1) are fitted and aircraft). Outdoor use (including in road vehicles) is not permitted. |
| Category of device | An LPI access point or bridge that is supplied power from a wired connection, has an integrated antenna and is not battery powered. An LPI client device is a device that is connected to an LPI access point or another LPI client device and may or may not be not battery powered. |
| Frequency band | 5945-6425MHz |
| Channel access and occupation rules | An adequate spectrum sharing mechanism shall be implemented. |
| Maximum mean e.i.r.p. for in-band emissions (note 2) | 23 dBm |
| Maximum mean e.i.r.p. density for in-band emissions (note 2) | 10 dBm/MHz |
| Maximum mean e.i.r.p. density for out-of-band emissions below 5935 MHz (note 2) | -22 dBm/MHz |
| Note 1: Or similar structures made of material with compatible attenuation characteristics. Note 2: The "mean e.i.r.p." refers to the e.i.r.p. during the transmission burst, which corresponds to the highest power, if power control is implemented. |  |

Table 4.1.1.3.1d -2: Very Low Power (VLP) WAS/RLAN devices

| Parameter | Technical conditions |
| --- | --- |
| Permissible operation | Indoors and outdoors Use on drones is prohibited |
| Category of device | The VLP device is a portable device. |
| Frequency band | 5945-6425MHz |
| Channel access and occupation rules | An adequate spectrum sharing mechanism shall be implemented. |
| Maximum mean e.i.r.p. for in-band emissions (note 1) | 14 dBm |
| Maximum mean e.i.r.p. density for in-band emissions (note 1) | 1 dBm/MHz |
| Narrowband usage maximum mean e.i.r.p. density for in-band emissions (note 1) (note 2) | 10 dBm/MHz |
| Maximum mean e.i.r.p. density for out-of-band emissions below 5935 MHz (note 1) | -45 dBm/MHz (note 3) |
| Note 1: The "mean e.i.r.p." refers to the e.i.r.p. during the transmission burst, which corresponds to the highest power, if power control is implemented. Note 2: Narrowband (NB) devices are devices that operate in channels bandwidths below 20 MHz. Narrowband devices also require a frequency hopping mechanism based on at least 15 hop channels to operate at a PSD value above 1 dBm/MHz. Note 3: ECC will study the appropriateness of this level of OOBE by 31/12/2024. In absence of the justified evidence, a value of -37 dBm/MHz will be adopted from 01/01/2025. |  |

##### 4.1.1.3.1e	CEPT Report 073

This CEPT Report 073 [38] contains the summaries of the technical studies and assessment of coexistence scenarios for Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) systems with incumbent systems in the 5925-6425 MHz band and adjacent bands, in response to Task 1 (Assessment and study of compatibility and coexistence scenarios in the band 5925-6425 MHz) of the EC Mandate on WAS/RLAN in the 5925-6425 MHz band.

Based on the studies described in ECC Report 302 (see clause 4.1.1.3.1b), the CEPT Report 073 concludes that coexistence between WAS/RLAN with existing services and systems within and adjacent to the band 5925-6425 MHz would be technically feasible under certain conditions.

For scenarios with indoor only WAS/RLAN deployment with a maximum e.i.r.p. in the range of 200-250 mW, the studies indicated that coexistence was feasible with FS when considering the long-term aggregated interference protection criterion and with FSS. Also, coexistence with CBTC systems and Road-ITS would be technically feasible assuming suitable measures such as a guard-band and requirements on WAS/RLAN in band and/or out-of-band emissions. These elements would limit the available spectrum to WAS/RLAN to less than the entire 5925-6425 MHz band.

Complementary studies and analyses  are covered by CEPT Report 075. These studies address FS short-term protection between point-to-point applications and WAS/RLAN indoor only deployments as well as potential WAS/RLAN portable devices that operate outdoor with power levels significantly lower than that for indoor use.

High power WAS/RLAN devices and outdoor WAS/RLAN devices other than those mentioned in the Report could present a risk of harmful interference to the Fixed Service , Fixed-Satellite Service, Communication based train control  systems and Road Intelligent Transport System  incumbents when sharing the band without restrictions.

Based on this risk of interference, the feasibility of outdoor WAS/RLAN deployment and high-power WAS/RLAN access points (APs) would require additional studies to address the interference to the incumbent systems.

##### 4.1.1.3.2	European Commission Decision 2021/1067

On June 17th 2021, The European Commission has published in the Official Journal the Commission Decision ([47]) on Wireless Access Systems / Radio Local Area Networks operating in the band 5945-6425 MHz.

The mandated technical conditions are those specified in ECC Decision (20)01 ([34]), as summarized in tables 4.1.1.3.1d-1 and 4.1.1.3.1d-2.

The EC Decision  2021/1067 has clarified further (Article 3) that "when introducing new applications into the 5 945-6 425 MHz frequency band or into adjacent frequency bands after the entry into force of this Decision, Member States shall not adopt technical and operational conditions applicable to any new application that unduly restrict the continued use of WAS/RLAN in the 5 945-6 425 MHz frequency band in accordance with this Decision".

### 4.1.2	Regional Commonwealth in the field of Communications (RCC) countries

The RCC Commission on Spectrum and Satellite Orbits approved their position on Agenda Item 1.2 of WRC-23 supporting the usage of IMT systems in the 6425-7125 MHz frequency range taking into account the conditions ensuring the compatibility of these systems with other radio systems using this frequency range [37].

In its 20th meeting (6-10 December 2021, Minsk, Belarus), the RCC Commission on Spectrum and Satellite Orbits approved the RCC Recommendation 1/21 “Harmonization of the technical conditions for 5F-NR/IMT-2020 systems in the RCC countries in the frequency band 6 425-7 125 MHz or in its portions” [59]. This recommendation provides the regulatory requirements by harmonized technical conditions (considered as regulatory requirements as stated in RCC Liaison Statement [59]) for RCC Administrations licensed operation of mobile service on a primary basis in 6 425-7 125 MHz band as further detailed in [59], as summarized hereafter.

The frequency band is a TDD band and consists of 35 reference frequency blocks of 20 MHz each. A frequency channel can consist of one or more adjacent frequency blocks but should not exceed 400 MHz. Additional information on the band arrangement can be found in [59].

The power of 5G-NR/IMT-2020 base stations and user equipment should not exceed the limits established by Article 21 of the Radio Regulations for the radio frequency band 6425-7125.

The unwanted emissions of 5G-NR/IMT-2020 base stations and user equipment in the spurious emissions domain shall be in compliance with the Category B limits for stations in the mobile service, as provided in the latest version of Recommendation ITU-R SM.329 [60].

NOTE:	The list of RCC members could be consulted at RCC website.

### 4.1.3	UK

On 24th July 2020, UK Ofcom has decided to make the lower 6 GHz band (5925-6425 MHz) available for the license exempt RLANs for indoor use with a maximum EIRP of 250mW and outdoor use with a maximum EIRP of 25mW, with the maximum mean EIRP density of 12.6mW/MHz in any 1 MHz band [40].

Equipment must not form part of a fixed outdoors installation when operating in 5925- 6425MHz. Aeronautical mobile use is not permitted. The Low Power Indoor apparatus may only be used within a building, onboard an aircraft or in any other enclosed space with attenuation characteristics at least as strong as those of either a building or an aircraft. ‘Onboard aircraft’ means the use of radio links for communications purposes inside an aircraft. Airborne use of the relevant equipment will be permitted within an aircraft only to establish a connection with a station or apparatus within the same aircraft.

### 4.1.4	Saudi Arabia

On 28th January 2021, Saudi Arabia Communications & Information Technology Commission announced its intention to allocate the entire 5925 – 7125 MHz band for license-exempt [41].

On 10th June 2021 Saudi Arabia Communications & Information Technology Commission (CICT) opened a public consultation to update the Radio Spectrum Allocation and Use Regulation for WLAN Applications [50].

On 22nd of August 2021, CITC opened a further consultation [51] on Light Licensing regime to optimize efficient use of spectrum addressing seven frequency ranges within 4-94 GHz, including the 6 GHz band for high power WLAN and Low Power 5G services (NR-U) (mobile).

On 1st January 2022, Saudi Arabia Communications & Information Technology Commission has published “WLAN Regulations” [52] which is now in effect and replaces previously published “Wireless Local Area Networks (WLAN/Wi-Fi) Usage Regulations”.  Technical conditions for operation in the 5925-7125MHz frequency range are as follows:

-	indoor access point with maximum EIRP of 1000mW and PSD of 10mW/MHz;

-	indoor client device with maximum EIRP of 250mW and PSD of 10mW/MHz.

### 4.1.5	United Arab Emirates

On December 2020, the telecommunications Regulatory Authority (TRA) has designated an additional 500 MHz radio spectrum, specifically 5925-6425MHz, for indoor use at an EIRP of 250 mW under class authorization [53].

### 4.1.6	African Telecommunication Union

In July 2021, ATU approved the ATU-R Recommendation 005-0 [57] recommending to its member states to designate the frequency band 5925–6425 MHz for use by WAS/RLAN equipment restricted to VLP (both outdoor and indoor use) and LPI use only on a non-exclusive, non-interference and non-protected basis, with the following operating conditions:

-	Low power indoor:

-	23dBm mean EIRP and 10dBm/MHz mean EIRP density;

-	Restricted to indoor use only including trains where metal coated windows are fitted and aircraft;

-	Outdoor use (including in road vehicles) is not permitted;

-	An LPI access point or bridge is a device that is supplied power from a wired connection, has an integrated antenna and is not battery powered.

-	An LPI client device is a device that is connected to an LPI access point or another LPI client device and may or may not be battery powered.

-	Very low power indoor and outdoor:

-	14dBm mean EIRP and 1dBm/MHz mean EIRP density;

-	Use on drones is prohibited;

-	Very Low Power device is a portable device.

-	An adequate spectrum sharing mechanism shall be implemented for channel access and occupation;

In September 2021, ATU Preparatory Meeting for World Radiocommunication Conference 2023 (APM23-2) agreed for band 6425-7125 MHz that [58]:

-	Conduct a survey among ATU Member States to identify services deployed at national level in the 6425 – 7125 MHz, with a view to protect existing services, and based on the results develop contributions to ITU-R WP5D, as appropriate;

-	Preliminarily support identification of the frequency band 6 425-7 125 MHz for IMT, taking into account the result of the coexistence studies in ITU-R.

### 4.1.7	Morocco

On May 2021, the Moroccan National Telecommunications Regulatory Agency (ANRT) published a new decision on Short Range Devices (SRD) in 5925-6425MHz frequency range [54] with the following technical parameters:

-	23dBm EIRP for indoor usage only, not allowed inside vehicles;

-	14dBm EIRP for indoor and outdoor usage for the portable devices, not allowed for unmanned flying devices (i.e. drones).

### 4.1.8	Kenya

On June 2022, Communications Authority of Kenya (CAK) issued “Guidelines On the Use of Radiofrequency Spectrum by Short Range Devices” [66] allowing the use of frequency band 5925-6425 MHz with the following technical requirements:

-	For the LPI mode:

-	23dBm (200 mW) mean EIRP;

-	10dBm/MHz mean EIRP density for in-band emissions;

-	Restricted to indoor use only, outdoor use including in road vehicles is not permitted;

-	An adequate spectrum sharing mechanism shall be implemented for channel access and occupation.

-	For the VLP mode:

-	14dBm (25 mW) EIRP;

-	1dBm/MHz mean EIRP density for in-band emissions;

-	Indoor and outdoor use, use of drones is prohibited;

-	VLP device is a portable device.

### 4.1.9	Qatar

On April 2022 the Qatar Communications Regulatory Authority (CRA) issued the Class License for the use of Radio Local Area Network (RLAN) devices over the lower part 5925 – 6425 MHz of the 6GHz band with the following technical characteristics [67]:

-	Low power mode with 23dBm EIRP for indoor use only;

-	Very low power mode with 14dBm for indoor and outdoor use.

### 4.1.10	Jordan

On May 2022 the TRC of Jordan updated the rules for WLAN devices allowing unlicensed operation in 5925-6425MHz frequency range with the following technical conditions [68]:

-	14dBm EIPR for portable devices operated indoor and outdoor;

-	23dBm EIRP for indoor usages only.

### 4.1.11	Russian Federation

On December 2022, Russian Ministry of the digital development and communication decided to open the 5925-6425MHz frequency range for the unlicensed usage with the following technical conditions:

-	for the frequency hopping devices, the allowed usage is only indoors with the channel bandwidth of 20MHz, with the minimum number of 15 hopping channels and the maximum PSD of 10dBm/MHz;

-	for the spread spectrum devices both LPI and VLP modes are allowed:

-	both modes are limited to indoor usage only;

-	the LPI mode maximum EIRP is 23dBm with the maximum PSD of 10dBm/MHz;

-	the VLP mode maximum EIRP is 14dBm with the maximum PSD of 1dBm/MHz.

### 4.1.12	South Africa

On May 2023, The Independent Communications Authority of South Africa (ICASA) has published an Amendment to Annexure B of Radio Frequency Spectrum Regulations, 2015 in respect of Radio Frequency Spectrum Licence Exemptions, effectively opening up the lower 6GHz spectrum band for the provision of Wi-Fi services [79]:

-	5925-6425MHz is opened for both LPI and VLP modes;

-	LPI mode can operate with 23dBm EIRP only indoors;

-	VLP mode can operate with 14dBm EIRP both indoors and outdoors;

-	EN 303687 should be used as a relevant performance standard;

-	Additional requirements are according to (EU) 2021/1067, ATU-R Rec.005-0 Annex 3.

## 4.2	ITU Region 2

### 4.2.1	USA

#### 4.2.1.1	Introduction

In US, 5.925-6.425 GHz frequency range is currently allocated for non-federal use for FSS (Fixed Satellite Service) and FS (Fixed Service) on a primary basis; while 6.425-7.125 GHz frequency range is allocated exclusively for non-federal use on a primary basis for FS at 6.525-7.125 GHz, Mobile Service at 6.425-6.525 GHz and 6.875-7.125 GHz, and FSS at 6.425-6.700 GHz and 7.025-7.075 GHz for uplink and at 6.700-7.025 GHz for both uplink and downlink. More details are provided in Table 4.2.1.1-1 [4] and Figure 4.2.1.1-1 below.

Table 4.2.1.1-1: US Frequency Allocations in 6 GHz

| Frequency Range [MHz] | Allocations – US Non Federal Table | FCC rule part(s) |
| --- | --- | --- |
| 5 925 – 6 425 | FIXED FIXED SATELLITE (Earth-to-space) NG181 | RF Devices (15) Satellite Communications (25) Fixed Microwave (101) |
| 6 425 – 6 525 | FIXED SATELLITE (Earth-to-space) MOBILE 5.450 5,458 | RF Devices (15) Satellite Communications (25) TV Broadcast Auxiliary (74F) Cable TV Relay (78) Fixed Microwave (101) |
| 6 525 – 6 700 | FIXED FIXED SATELLITE (Earth-to-space) 5.458 US342 | RF Devices (15) Satellite Communications (25) Fixed Microwave (101) |
| 6 700 – 6 875 | FIXED FIXED SATELLITE (Earth-to-space) (space-to-Earth) 5.441 5.458 5.458A 5.458B |  |
| 6 875 – 7 025 | FIXED NG118 FIXED SATELLITE (Earth-to-space) 5.441 MOBILE NG171 5.458 5.458A 5.458B | RF Devices (15) Satellite Communications (25) TV Broadcast Auxiliary (74F) Cable TV Relay (78) |
| 7 025 – 7 075 | FIXED NG118 FIXED SATELLITE (Earth-to-space) NG172 MOBILE NG171 5.458 5.458A 5.458B | RF Devices (15) TV Broadcast Auxiliary (74F) Cable TV Relay (78) |
| 7 075 – 7 125 | FIXED NG118 MOBILE NG171 5.458 |  |

Figure 4.2.1.1-1: US services in 6 GHz

Frequency ranges above 7.125 GHz are allocated to fixed federal services, while allocation of services in frequencies below 5.925 GHz is shown in Table 4.2.1.1-2.

Table 4.2.1.1-2: Adjacent services below 5.925 GHz

| Frequency Range [MHz] | Allocations US Federal Table | Allocations US Non Federal Table | FCC rule part(s) |
| --- | --- | --- | --- |
| 5 650 – 5 830 | RADIOLOCATION G2 5.150 US245 | Amateur 5.150 5.282 | RF Devices (15) ISM Equipment (18) Amateur Radio (97) |
| 5 830 – 5 850 |  | Amateur Amateur-Satellite 5.150 |  |
| 5 850 – 5 925 |  | FIXED SATELLITE (Earth-to-space) US245 MOBILE NG160 Amateur 5.150 | ISM Equipment (18) Private Land Mobile (90) Personal Radio (95) Amateur Radio (97) |

In August 2017, FCC issued a Notice of Inquiry (NOI) [5] asking inputs on potential opportunities for additional flexible access (mainly for wireless broadband services) in 5.925-6.425 GHz and 6.425-7.125 GHz frequency ranges. After reviewing received comments filed on October 2017 in GN Docket No. 17-183, FCC has issued a Notice of Proposed Rulemaking (NPRM) on October 24th, 2018 in [25] where the Commission proposes tailored rules to support compatibility of unlicensed operations in the 6 GHz band. Licensed operations in the band were not consider (comments are expected 60 days after publication, reply 90 days after publication).

#### 4.2.1.2	Licensed operations

According to comments in GN Docket No. 17-183, the upper part of 6 GHz frequency range (6.425-7.125 GHz) has been identified by some companies in [9, 10] as a good complement to millimetre wave spectrum for licensed mobile use cases in densely populated areas. which may be implemented by clearing and auctioning the spectrum, and relocating incumbents to alternative media or higher bands with a particular focus on transitioning of uses in more urban and suburban areas.

At the same time benefits that 6 GHz band provides for FS (allowing to accommodate long links) and FSS services (higher-frequency satellites cannot match C-band’s resistance to atmospheric attenuation) have been highlighted, as well as the risks in terms of costs and delay associated with clearing the spectrum, including to the disruption to users and displacement of investment associated with existing uses (particularly existing public safety services) [6, 7, 13, 14].

In its Report and Order [31], FCC decided to not allocate any spectrum for licensed operation in the 5.925-7.125 GHz frequency range.

#### 4.2.1.3	Unlicensed operations

##### 4.2.1.3.0	Overview

According to comments in GN Docket No. 17-183, the whole 5.925-7.125 GHz frequency range represents a unique opportunity to address the unlicensed spectrum crunch quickly and efficiently, due to the band’s favourable propagation characteristics, proximity to existing unlicensed operations, and ability to support the wide channels that advanced applications and gigabit services require [6,7,8,14,17,18]. It is anyway requested that the introduction of unlicensed use in such frequency range is subjected to the requirement for a detailed engineering analysis and mitigation proposals ensuring that incumbents operations are fully protected [6, 7, 8, 9, 10, 11, 13, 14, 16, 17, 18].

As adopted in the Report and Order [31], the Commission decided to implement the following four Unlicensed National Information Infrastructure (U-NII) sub-bands in the 6 GHz band:

-	U-NII-5: 5.925 GHz to 6.425 GHz (sharing with FS and FSS incumbents)

-	U-NII-6: 6.425 GHz to 6.525 GHz (sharing with BAS and CARS)

-	U-NII-7: 6.525 GHz to 6.875 GHz (same as U-NII-5)

-	U-NII-8: 6.875 GHz to 7.125 GHz (sharing with BAS and CARS)

The Commission further authorized:

-	Unlicensed standard-power access points in the U-NII-5 and U-NII-7 bands through use of an Automated Frequency Coordination (AFC).

-	Unlicensed indoor low-power access point in the entire 6 GHz band, opening up 320 MHz channels operating in that band.

##### 4.2.1.3.1	Standard-power operations

U-NII-5 and U-NII-7 bands are available under rules consistent with the existing rules for U-NII-1 and U-NII-3 bands to include the same permitted power levels (i.e. “standard power levels”) for access points.

Table 4.2.1.3.1-1 lists power characteristics of unlicensed devices for standard-power operation.

Table 4.2.1.3.1-1: Devices maximum power for standard-power operations

|  | Maximum EIRP | Maximum EIRP power spectral density |
| --- | --- | --- |
| Standard-Power Access Points (AFC controlled) | 36 dBm | 23 dBm/MHz |
| Client devices connected to standard power access point | 30 dBm | 17 dBm/MHz |

The Commission mandated to implement an automated frequency coordination (AFC) process for unlicensed use to protect incumbents. The AFC will use a centralized model.

Prior to any operation, standard-power access points (APs) access the AFC system to determine the authorized frequencies and their associated maximum permissible power level at AP’s geographic coordinates. Note that the list of available frequency ranges will not only be determined based on the AP’s maximum permissible power (36dBm), but also for lower power levels (down to 21 dBm by step of maximum 3 dB).

APs may transmit only on frequencies indicated as being available by the AFC system. Client devices will receive this list from the standard-power access point.

The AFC system is expected to obtain information on protected services from FCC databases (i.e. the Universal Licensing System, plus other information collected from Canada and Mexico to protect operations at the borders) and use it to determine frequency availability for APs based on FCC specified protection criteria.

The Commission listed several characteristics of the AFC (e.g., device registration to AFC, access security, fees to get information), what information should be managed (available frequencies, exclusion zone and how they will be determined) and how they should be maintained (information update, regular inquiry from devices). The Commission decided this AFC system will be operated by entities designated by the FCC Chief of the Office of Engineering and Technology.

Note that it’s expected the standard-power access point’s location and height from the ground shall be well determined to use efficiently the AFC information.

Also, it’s expected that client devices operating in U-NII-5 and U-NII-7 would be under standard-power access point’s control.

##### 4.2.1.3.2	Low-power operations

Low power indoor operations are also allowed in the entire 6 GHz band (U-NII-5, U-NII-6, U-NII-7 and U-NII-8).

U-NII-6 and U-NII-8 are currently used for mobile operations (broadcast stations, programming networks, …), the Commission has then mandated that unlicensed use in these bands will be limited in power level and for indoor operations use only (residential/business like operations). The Commission believes use of the AFC system in these bands would not efficiently protect incumbent services.

Table 4.2.1.3. 2-1 lists power characteristics of unlicensed devices for low-power operation.

Table 4.2.1.3.2-1: Devices maximum power for low-power operations

|  | Maximum EIRP | Maximum EIRP power spectral density |
| --- | --- | --- |
| Low power Access point (indoor only) | 30 dBm | 5 dBm/MHz |
| Client devices connected to low power access point | 24 dBm | -1 dBm/MHz |

Without AFC control, operations are limited to low power indoor, using a contention-based protocol. The access point devices cannot be weather resistant and shall have integrated antennas (prohibiting connection to other antennas).

Operations in moving vehicles (cars, trains, aircraft) and drones are prohibited, with one exception in the U-NII-5 band for large passenger aircraft operating over 10,000 feet.

##### 4.2.1.3.3	Common requirements to the entire 6GHz band

The emission limits outside the U-NII-5 and U-NII-8 bands is -27dBm/MHz EIRP.

In the U-NII-5 through U-NII-8 bands, the emission mask shall comply with:

-	20 dB PSD suppression at one megahertz of channel edge.

-	28 dB PSD suppression at one channel bandwidth from the channel centre.

-	40 dB PSD suppression at 1.5 channel bandwidth from the channel centre.

### 4.2.2	Canada

On 21st May 2021, Innovation, Science and Economic Development (ISED) department of Canada published "Decision on the Technical and Policy Framework for Licence-Exempt Use in the 6 GHz Band" [42] with the following key aspects:

-	D1: ISED will allow licence-exempt RLAN use in the 5925-7125 MHz band;

-	D3: Standard-power RLANs under the control of an AFC system will be permitted to operate on a licence-exempt basis in the 5925-6875 MHz frequency range:

-	standard-power devices would be subject to a maximum permitted EIRP of 36 dBm per channel;

-	a maximum permitted power spectral density of 23 dBm/MHz and that standard-power RLAN operation would only be permitted on specific frequency channels, with EIRP dictated by the AFC system;

-	to address coexistence with fixed-satellite services (FSS), ISED sought comments to limit the standard-power RLAN’s maximum EIRP to 125 mW per channel at elevation angles above 30 degrees over the horizon.

-	D4: Low-power indoor-only RLANs will be permitted to operate on a licence-exempt basis across the 5925-7125 MHz band with the use of a contention-based protocol (e.g. listen-before-talk):

-	subject to a maximum permitted EIRP of 30 dBm per channel; and

-	a maximum permitted power spectral density of 5 dBm/MHz.

-	D5: Indoor and outdoor very low-power RLAN devices will be permitted to operate on a licence-exempt basis across the 5925-7125 MHz band with the use of a contention-based protocol (e.g. listen-before- talk):

-	subject to a maximum permitted EIRP of 14 dBm per channel; and

-	a maximum permitted power spectral density of -8 dBm/MHz.

-	D6-D7: ISED will adopt an AFC system that is practical, consistent with the Canadian public interest and harmonized with the U.S. to the maximum extent possible. Canadian AFC rules will include a requirement to protect licensed fixed service systems and radioastronomy sites.

### 4.2.3	Brazil

On 26th February 2021, Brazilian National Telecommunications Agency approved Act 1306 [43] that lists the technical requirements for Broadband Wireless Access System for Local Area Networks operating 5.925 – 7.125 GHz band with the following key points:

-	For indoor access points and subordinate access points, they must meet the following conditions:

-	Average power is limited to a maximum of 30 dBm EIRP.

-	Average power spectral density is limited to a maximum of 5 dBm / MHz EIRP.

-	The equipment must be used in an indoor environment only.

-	For client equipment operating under the control of an indoor access point, they must meet the following conditions:

-	Average power is limited to a maximum of 24 dBm EIRP.

-	Average power spectral density is limited to a maximum of -1 dBm / MHz EIRP.

-	The equipment must be used in an indoor environment only.

-	For very low power devices, the following conditions must be met:

-	Average power is limited to a maximum of 17 dBm EIRP.

-	Average power spectral density is limited to a maximum of -5 dBm / MHz EIRP.

-	The RMS value for spurious emissions of any emission outside the range of 5.925 - 7.125 MHz must be limited to maximum spectral power density of EIRP of -27 dBm/MHz.

-	The spectral power density must meet the following emission mask:

-	Attenuation of 20 dB, with a distance of 1 MHz bandwidth from the channel.

-	Attenuation of 28 dB, with a distance of one channel spacing, from the centre of the channel, and

-	Attenuation of 40 dB, with a distance of 1.5 channel spacing, from the centre of the channel

-	All equipments must only use an antenna permanently integrated into the structure of the equipment.

-	It is mandatory to use a media access sharing system on these equipment.

### 4.2.4	Peru

On 30th April 2021, Peruvian Ministry of Transport and Communication has released a new resolution [44], which in particular updates regulatory parameters for the 6GHz band as presented below:

-	Frequency range 5925-7125MHz is assigned for the license-exempt operation;

-	For the indoor operation, the access point is allowed to transmit at maximum EIRP of 30dBm with the spectral density of 5dBm/MHz; and the client equipment maximum EIRP is 24dBm and the maximum spectral density is -1dBm/MHz;

-	Access points cannot be weatherproof;

-	Access points may have only built-in antennas and it is prohibited to provide the ability to connect other external antennas;

-	Access points cannot be battery operated;

-	Access points cannot operate on oil extraction platforms, automobiles, trains, ships and aircraft, with the exception of large passenger aircraft (only at 5925 - 6425 MHz) operating at more than 10,000 feet;

-	Operation and/or communication with drones is not allowed.

### 4.2.5	Chile

On 22th October 2020, Ministry of Transport and Communication of Chile published Resolution 1985 [45] according to which low power access equipment, also referred to as AP, may operate in the frequency band 5925-7125 MHz under exclusive use for indoor environment and the following transmission power requirements:

-	maximum EIRP of 30dBm;

-	maximum spectral density of 5dBm/MHz.

-	low-power AP equipment may only have integrated antennas, which cannot be removable or replaceable, nor may they have connectors that allow the connection of additional external antennas;

-	likewise, they should not be able to operate with internal batteries or have spaces for their installation

User terminal devices that are associated with low power AP equipment must operate at the following power levels:

-	maximum EIRP of 24dBm;

-	maximum spectral density of -1dBm/MHz.

On July 2021, the Chilean authority developed Resolution 1321 [55] updating Resolution 1985 and covering the technical standard for short range devices in Chile:

-	very low power devices with personal reach may operate outdoors, using internal batteries, in the 5925-7125 MHz frequency band;

-	the maximum average EIRP power is 17 dBm.

On September 2022, the Chilean authority issued a Resolución 2844 Exenta [70], which revises Resolución 1985 Exenta [45] as follows:

-	the frequency range for unlicensed operation is changed to 5925-6425MHz;

### 4.2.6	Mexico

In May 2021 the Instituto Federal de Telecomunicaciones (IFT) in Mexico has opened a public consultation for the 5925-7125 MHz frequency band with the objectives to establish the technical operating conditions of WAS/RLAN systems that could make use of the of the 5925-7125 MHz frequency band and to increase the harmonization based on international best practices and existing technological advances. IFT has emphasized that within the operating conditions of the band, its use was determined for low power indoors and very low power indoor and outdoor usage.

IFT has release a press release [75] informing on reservation of the 500 MHz in the low 6 GHz band for unlicensed operation. The Mexican regulator decided that the remaining 700 MHz in 6425 to 7125 MHz range will remain unchanged as not allocated, subject to further technology evolution, spectrum needs and service availability of Mexican market.

### 4.2.7	Honduras

The National Telecommunications Commission (CONATEL) of Honduras has published Resolution NR 003/21 [48], which updates the National Frequency Allocation Plan (PNAF) to open the use of the 5925-7125MHz frequency range for the license-exempt usage.

### 4.2.8	Costa Rica

The Superintendencia de Telecomunicaciones (SUTEL) in Costa Rica has published its Plan Nacional de Atribución de Frecuencias (PNAF) [49], according to which the 5925-7125MHz frequency range is opened for the license-exempt usage with the following technical conditions:

-	maximum EIRP is 30dBm;

-	maximum output power of the equipment is 24dBm;

-	unlicensed usage is allowed at the following conditions:

-	low power indoor applications (LPI) in accordance with the power limits indicated above;

-	very low power (VLP) indoors and outdoors with a maximum EIRP of 14 dBm;-	LPI applications are restricted to operation inside buildings for RLAN type systems (wireless local area networks) or similar, which use contention protocols to get access to the environment;

-	VLP applications are for portable or similar type devices, limited in power, allowing connectivity over short distances.

### 4.2.9	Colombia

On October 2021, Colombian National Agency (Agencia Nacional del Espectro) released a draft resolution [56] according to which the 5925-7125MHz frequency range is opened for the license-exempt operation. On November 2022 another resolution No. 000737 [71] was released setting the following technical parameters:

-	only indoor usage is allowed;

-	the use of devices located on oil rigs, automobiles, trains, boats and aircraft is not allowed with the exception of aircraft flying above 10,000 feet;

-	the operation of equipment intended to control or communicate with unmanned aerial vehicles is not allowed;

-	access point devices must be powered directly from the commercial electrical power network, the use of batteries is not allowed, and their structure must not support outdoor use;

-	access point devices must only work with an antenna integrated into their structure;

-	maximum EIRP of 30dBm for AP devices with maximum PSD of 5dBm/MHz;

-	maximum EIRP of 24dBm for CL devices with maximum PSD of -1dBm/MHz;

-	the spectral density should be attenuated 20 dB at a distance of 1 MHz from the end of the channel, 28 dB at a distance of one channel apart from the center of the channel, and 40 dB at a distance of 1.5 channels apart from the center of the channel;

-	emissions outside the 5925-7125MHz frequency range should be limited to -27 dBm/MHz;

### 4.2.10	Dominican Republic

On September 2022 the Instituto Dominicano de las Telecomunicaciones (INDOTEL) has published Resolución No. 082-2022 [72], which establishes operating conditions for Low and Very Low Power Equipment in the 5925-7125 MHz frequency band with the following conditions:

-	For indoor access points operating and for devices subordinate to it, the maximum power spectral density must not exceed 8 dBm/MHz. The maximum EIRP over the operating frequency band should not exceed 30 dBm.

-	For client-type devices the maximum power spectral density shall not exceed 2 dBm/MHz in any 1MHz chunk and the maximum EIRP over the operating frequency band should not exceed 24 dBm.

-	Very Low Power (VLP) devices with maximum power spectral density not exceeding -8 dBm/MHz and maximum EIRP over the operating frequency band not exceeding 14 dBm may be used outdoors.

-	For transmitters any emissions outside the 5925-7125 MHz frequency band must not exceed -27 dBm/MHz.

-	The power spectral density must be suppressed by 20dB at 1MHz outside the edge of the channel, by 28dB at channel width from the center from the channel and by 40dB at one and a half times the channel width from the center of the channel. At frequencies between 1MHz outside the channel edge and one channel width from the center of the channel, the limits should be linearly interpolated between 20 dB and 28 dB of suppression. At frequencies between one channel width plus 1MHz, and one channel with one-half times the channel width the limits must be linearly interpolated between 28 dB and 40 dB of suppression.

-	The operation of indoor access points and fixed client devices is prohibited in automobiles, trains, ships and airplanes, except for large aircraft while flying above 3000 meters and only access points indoor operating in the frequency band 5925-6425MHz.

-	Usage of indoor access points and fixed client-type devices is prohibited for control or communications with unmanned aircraft systems.

-	Indoor access points and fixed client-type devices must use an antenna integrated or permanently fixed to its body or structure, as well as use a contention-based communications protocol.

### 4.2.11	Argentina

On 24th May 2023, Argentinian Ente Nacional de Comunicaciones (ENACOM) decided to allocate full 6GHz band, 5925 - 7125MHz, for the unlicensed usage [76].

-	Operation is for exclusive indoor use, both private and for the provision of ICT services.

-	Emissions may not cause interference to authorized stations of a service assigned with primary category, nor claim protection against harmful interference caused by stations of a service assigned with category primary.

-	An access point or subordinate device may operate in the 5925-7125MHz band with a maximum Equivalent Isotropically Radiated Power (EIRP) of 30 dBm and a maximum EIRP spectral density of 5 dBm/MHz.

-	A client device can operate in the 5925-7125MHz band with a maximum Equivalent Isotropically Radiated Power (EIRP) of 24 dBm and a maximum EIRP spectral density of -1 dBm/MHz.

-	Emissions outside the band 5925-7125MHz should not exceed an EIRP of -27 dBm/MHz.

-	The operation of access points, client devices and subordinate devices is restricted to inside buildings only.

-	Both access points, client devices and subordinate devices must use a "contention protocol" that allows several users to share the same spectrum.

-	The operation of these devices is prohibited in vehicles (cars, trains, boats and planes). However, the operation of this equipment will be allowed in aircraft when they fly above 10,000 feet.

## 4.3	ITU Region 3

### 4.3.1	China

In April 2019, CCSA established a new project on technical report for the feasibility study of IMT system using 5925-7125 MHz frequency band [29].

This CCSA study includes identifying IMT parameters in 5925-7125MHz in order to develop coexistence between IMT system and incumbent services and applications. The project was finalized in April 2023.

China's Ministry of Industry and Information Technology published the new NTFA (national table for frequency allocation) which is effective from July 1st, 2023 [77]. In the new NTFA, the frequency band of 6425-7125 MHz is identified for IMT services.

### 4.3.2	South Korea

In June 2020, Korea’s Ministry of Science and ICT (MSICT) issued an amendment of technical standards that also includes proposed rules for the 5925–7125MHz frequency band, which was finally approved in October 2020 [39]. In May 2022, the usage of LPI mode inside metro was additionally allowed [73]. The key points are as follows:

-	The whole range of 5925–7125MHz is allocated for the license-exempt low power indoor:

-	the occupied channel bandwidth should be less or equal 160MHz;

-	the EIRP limit is 24dBm;

-	the maximum power spectral density is 2dBm/MHz;

-	a device shall use power supply from a wired connection, and a client battery powered device must communicate through the device connected to a wired power supply;

-	this frequency range cannot be used for indoor automotive or airplane deployments;

-	usage inside moving vehicles, except for metro, is prohibited;

-	usage inside metro is allowed only in the frequency range of 5925-6425MHz.

-	The 5925-6425MHz frequency range is allocated for the license-exempt very low power indoor and outdoor:

-	the occupied channel bandwidth should be less or equal 160MHz;

-	the EIRP limit is 14dBm;

-	the maximum power spectral density is 1dBm/MHz;

-	this frequency range cannot be used for drones;

-	automotive embedded systems shall use 6085-6425MHz.

-	Unwanted emissions at frequencies outside the operational frequency range shall be below -27dBm/MHz.

-	Unwanted emissions at frequencies outside the operational frequency of 5925-6445MHz (very low power under the conditions for that range described above) shall be below -34dBm/MHz.

-	Transmitter and receiver spurious emission limits should be as specified in table 4.3.2-1.

Table 4.3.2-1:	Transmitter and receiver spurious emission limits – South Korea

| Frequency Range | Maximum level (average) | Measurement bandwidth |
| --- | --- | --- |
| f<1GHz | -54dBm | 100kHz |
| f>1GHz | -47dBm | 1MHz |

### 4.3.3	Hong Kong SAR

In April 2022, the Office of the Communication Authority (OFCA) published HKCA 1081 [62] for devices operating in the 6GHz band. This specification defines the minimum performance requirements for radiocommunications devices operating in the 6 GHz band for wireless local area networks. The apparatus shall operate in the 5.925 – 6.425 GHz frequency range and shall meet the technical requirements specified in the standard EN 303 687. According to [62], the RF output power limit of 23 dBm EIRP and the power spectral density limit of 10 dBm/MHz EIRP as specified in clauses 4.3.2 and 4.3.3 of EN 303 687 are allowed to be increased by 1 dB to 24 dBm EIRP and 11 dBm/MHz EIRP respectively.

In 18 July 2023, the Secretary for Commerce and Economic Development (SCED) and the Communications Authority (CA) jointly launched a public consultation ([78]) on arrangements for assignment of 400 MHz of spectrum in the 6/7 GHz band (6570-6770 MHz and 6925-7125 MHz) for the provision of public mobile services, and the methods for setting the related spectrum utilisation fee (SUF) with earliest release timing at 2025.

### 4.3.4	Australia

In March 2022, The Australian Communications and Media Authority makes the following variation under subsection 132(1) of the Radiocommunications Act 1992 [61]:

-	Low power indoor transmitters:

-	Operating frequency 5925-6425MHz;

-	The transmitter must only be used indoors;

-	Maximum EIRP of 250mW;

-	The power spectral density of the transmitter must not exceed 12.5mW EIRP per MHz;

-	Contention-based protocols for multiple access, such as Carrier Sense Multiple Access (CSMA) or Multiple Access Collision Avoidance (MACA), must be implemented.

-	Very low power indoor/outdoor transmitters:

-	Operating frequency 5925-6425MHz;

-	Maximum EIRP of 25mW;

-	The power spectral density of the transmitter must not exceed 1.25mW EIRP per MHz;

-	Contention-based protocols for multiple access, such as Carrier Sense Multiple Access (CSMA) or Multiple Access Collision Avoidance (MACA), must be implemented.

### 4.3.5	New Zealand

In August 2022, the New Zealand Radio Spectrum Management (RSM) body agreed to make the 5925 – 6425 MHz frequency band available for Wireless Local Area Network (WLAN) under the Low Power Indoor (LPI) use and the Very Low Power (VLP) Indoor and Outdoor with the following technical conditions [63]:

-	Low power indoor:

-	Indoor use only within a building or within an enclosed space having attenuation characteristics at least equivalent to those of a building;

-	Maximum EIRP of 24dBm (250mW);

-	Maximum power density of 11dBm/MHz (12.6mW/Hz).

-	Very low power indoor/outdoor:

-	Maximum EIRP of 14dBm (25.11mW);

-	Maximum power density of 1dBm/MHz (1.26mW/Hz).

### 4.3.6	Japan

In April 2022 Japanese Ministry of Internal Affairs and Communications published the document describing technical conditions for the 6GHz band unlicensed systems:

-	LPI and VLP mode in the frequency range of 5925-6425MHz;

-	Maximum EIRP should be less than 200mW in LPI mode and less than 25mW in VLP mode;

-	For the LPI mode frequency use is limited to indoors;

-	Operating channel bandwidth size of 20, 40, 80, and 160MHz with the following centre frequencies:

-	20MHz channel: 5955, 5975, 5995, 6015, 6035, 6055, 6075, 6095, 6115, 6135, 6155, 6175, 6195, 6215, 6235, 6255, 6275, 6295, 6315, 6335, 6355, 6375, 6395, 6415 MHz;

-	40MHz channel: 5965, 6005, 6045, 6085, 6125, 6165, 6205, 6245, 6285, 6325, 6365, 6405 MHz;

-	80MHz channel: 5985, 6065, 6145, 6225, 6305, 6385 MHz;

-	160MHz channel: 6025, 6185, 6345 MHz;

-	Adjacent channel leakage power:

-	20MHz channel: -25dBc/20MHz for the 20MHz offset and -40dBc/20MHz for the 40MHz offset;

-	40MHz channel: -25dBc/40MHz for the 40MHz offset and -40dBc/40MHz for the 80MHz offset;

-	80MHz channel: -25dBc/80MHz for the 80MHz offset and -40dBc/80MHz for the 160MHz offset;

-	160MHz channel: -25dBc/160MHz for the 160MHz offset and -40dBc/160MHz for the 320MHz offset MHz;

-	Unwanted emission requirements:

-	For the LPI mode:

-	-27dBm/MHz for frequencies 5925MHz and below for all channel sizes;

-	-13dBm/MHz for frequencies from 6425MHz to X value limit, and -19dBm/MHz for frequencies above X value limit, where X limit depends on the channel size as follows:

-	X = 6435.9MHz for 20MHz (the reference channel at 6415MHz);

-	X = 6440.1MHz for 40MHz (the reference channel at 6405MHz);

-	X = 6440.4MHz for 80MHz (the reference channel at 6385MHz);

-	X = 6425.5MHz for 160MHz (the reference channel at 6345MHz).

-	For the VLP mode:

-	-37dBm/MHz for frequencies 5925MHz and below for all channel sizes;

-	-13dBm/MHz for frequencies from 6425MHz to X value limit, and -19dBm/MHz for frequencies above X value limit, where the X limit depends on the channel size as follows:

-	X = 6425.5MHz for 20MHz;

-	X = 6425.4MHz for 40MHz;

-	X = 6425.2MHz for 80MHz;

-	X = 6425.1MHz for 160MHz.

### 4.3.7	Malaysia

In January 2022, Malaysian Communications and Multimedia Commission (MCMC) published a new class assignment document, which in particular specifies unlicensed operation in the 5925-6425MHz frequency range with the following technical conditions:

-	The maximum EIRP for indoor use only is limited to 200mW;

-	The radiated PSD for indoor devices is limited to 10mW/MHz;

-	The maximum EIRP for indoor and outdoor devices is limited to 25mW;

-	The radiated PSD for outdoor and indoor devices is limited to 1.25mW/Hz;

-	The radiated PSD for narrowband usage (channel bandwidth below 20MHz) for outdoor and indoor devices is limited to 10mW/MHz;

### 4.3.8	Singapore

On September 2023 Singaporean Infocomm Media Development Authority (IMDA) announced that the 5925-6425MHz frequency range will be allocated for the unlicensed operation with the following parameters [80]:

- LPI mode

-	Maximum output power of 250mW;

-	Maximum mean EIRP density of 11dBm/MHz;

-	Outdoor use, including in road vehicles, is not permitted;

-	An LPI access point or bridge that is supplied power from a wired connection, has an integrated antenna and is not battery powered;

-	An LPI client device that is connected to an LPI access point or another LPI client;

-	Adequate spectrum sharing mechanism shall be implemented employing the contention-based protocol;

-	Recommended Measurement Methods are FCC Part 15E and ETSI EN 303 687.

- VLP mode

-	Maximum output power of 25mW;

-	Maximum mean EIRP density of 1dBm/MHz or 10dBm/MHz for the narrow-band usage;

-	Use on unmanned aircraft systems/drones is prohibited;

-	Adequate spectrum sharing mechanism shall be implemented employing the contention-based protocol;

-	Recommended Measurement Methods are FCC Part 15E and ETSI EN 303 687.

-	Transmitter spurious emissions are in accordance with ITU-R SM.329-12 [60], paragraph 4.3, Table 3, for "short range device above 30 MHz, Radio local area networks (RLAN), Citizens band (CB), cordless telephones, and radio microphones".

Table 4.3.8-1: Spurious domain emission limits

| Type of equipment | Limits |
| --- | --- |
| Short range device operating above 30 MHz. | –36 dBm for 9 kHz ≤ except frequencies below< 1 GHz –54 dBm for f within the bands 47-74 MHz, 87.5-118 MHz, 174-230 MHz, 470-862 MHz –30 dBm for 1 GHz ≤ f < (see ITU-R SM.329-12 [60], paragraph 2.5) |

## 4.4	ITU-R

WRC-19 (28 October to 22 November 2019) approved agenda for the 2023 world radiocommunication conference [30], in which the frequency bands 7 025-7 125 MHz and 6 425-7 025 MHz will be considered for IMT identification at WRC-23. In particular, to conduct and complete in time for WRC-23 the sharing and compatibility studies, with a view to ensuring the protection of services to which the frequency band is allocated on a primary basis, without imposing additional regulatory or technical constraints on those services, and also, as appropriate, on services in adjacent bands, for the frequency bands:

– 7 025-7 125 MHz (globally);

– 6 425-7 025 MHz (Region 1/EMEA);

CPM 23-1 (Nov. 25-26, 2019) assigned ITU-R WP5D as the Responsible Group for Agenda Item 1.2, and also defined the deadline for submission of technical and operational characteristics needed for sharing and compatibility studies as 15 June 2021.

## 4.5	Regulatory parameters comparative for license-exempt

The following table 4.5-1 compares the regulatory parameters for spectrum allocated to license-exempt.

Table 4.5-1: Comparative of the regulatory parameters for license-exempt

| Region | Country | Permissible operation (Note 1) | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- | --- |
| Region 1 | EU/CEPT | LPI (see 4.1.1) | 5945 – 6425MHz | 23dBm | 10dBm/MHz | -22 dBm/MHz (below 5935MHz) |
|  |  | VLP (see 4.1.1) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) | -45 dBm/MHz (below 5935MHz); |
|  | UK | LPI (see 4.1.3) | 5925 – 6425MHz | 24dBm | 11dBm/MHz | In accordance with directive 2014/53/EC |
|  |  | VLP (see 4.1.3) |  | 14dBm |  |  |
|  | UAE | LPI (see 4.1.5) | 5925 – 6425MHz | 24 dBm |  |  |
|  | Morocco | LPI (see 4.1.7) | 5925 – 6425MHz | 23 dBm |  |  |
|  |  | VLP (see 4.1.7) |  | 14 dBm |  |  |
|  | Saudi Arabia | LPI (see 4.1.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 10dBm/MHz |  |
|  | Kenya | LPI (see 4.1.8) | 5925 – 6425MHz | 23dBm | 10dBm/MHz |  |
|  |  | VLP (see 4.1.8) |  | 14dBm | 1dBm/MHz |  |
|  | Qatar | LPI (see 4.1.9) | 5925 – 6425MHz | 23dBm |  |  |
|  |  | VLP (see 4.1.9) |  | 14dBm |  |  |
|  | Jordan | LPI (see 4.1.10) | 5925 – 6425MHz | 23dBm |  |  |
|  |  | VLP (see 4.1.10) |  | 14dBm |  |  |
|  | Russian Federation | LPI (see 4.1.11) | 5925 – 6425MHz | 23dBm | 10dBm/MHz |  |
|  |  | VLP (see 4.1.11) |  | 14dBm | 1dBm/MHz |  |
|  | South Africa | LPI (see. 4.1.12) | 5925 – 6425MHz | 23dBm | 10dBm/MHz | In accordance with ETSI EN 303 687 |
|  |  | VLP (see 4.1.12) |  | 14dBm | 1dBm/MHz |  |
| Region 2 | US | SP (see 4.2.1) | 5925 – 6425MHz 6525 – 6875MHz | 36dBm (AP) 30dBm (CL) | 23dBm/MHz (AP) 17dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | LPI (see 4.2.1) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |
|  | Canada | SP (see 4.2.2) | 5925-6875 MHz | 36dBm | 23dBm/MHz |  |
|  |  | LPI (see 4.2.2) | 5925-7125 MHz | 30dBm | 5 dBm/MHz |  |
|  |  | VLP (see 4.2.2) |  | 14dBm | -8dBm/MHz |  |
|  | Brazil | LPI (see 4.2.3) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.2.3) |  | 17 dBm | -5 dBm/MHz |  |
|  | Peru | LPI (see 4.2.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  | Chile | LPI (see 4.2.5) | 5925 – 6425MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  |  | VLP (4.2.5) |  | 17 dBm |  |  |
|  | Costa Rica | LPI (see 4.2.8) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) |  |  |
|  |  | VLP (see 4.2.8) |  | 14dBm |  |  |
|  | Colombia | LPI (see 4.2.9) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  | Dominican Republic | LPI (see 4.2.10) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 8dBm/MHz (AP) 2dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.2.10) |  | 14dBm | -8dBm/MHz |  |
|  | Argentina | LPI (see 4.2.11) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
| Region 3 | South Korea | LPI (see 4.3.2) | 5925 – 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.3.2) | 5925 – 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range of the VLP mode) |
|  | Hong Kong | LPI (see 4.3.3) | 5925 – 6425MHz | 23dBm | 10dBm/MHz | In accordance with ETSI EN 303 687 |
|  |  | VLP (see 4.3.3) |  | 14dBm | 1dBm/MHz |  |
|  | Australia | LPI (see 4.3.4) | 5925 – 6425MHz | 24dBm | 11dBm/MHz |  |
|  |  | VLP (see 4.3.4) |  | 14dBm | 1dBm/MHz |  |
|  | New Zealand | LPI (see 4.3.5) | 5925 – 6425MHz | 24dBm | 11dBm/MHz |  |
|  |  | VLP (see 4.3.5) |  | 14dBm | 1dBm/MHz |  |
|  | Japan | LPI (see 4.3.6) | 5925 – 6425MHz | 23dBm |  | -27dBm/MHz (below 5925MHz) -13dBm/MHz and -19dBm/MHz (above 6425MHz) |
|  |  | VLP (see 4.3.6) |  | 14dBm |  | -37dBm/MHz (below 5925MHz) -13dBm/MHz and -19dBm/MHz (above 6425MHz) |
|  | Malaysia | LPI (see 4.3.7) | 5925 – 6425MHz | 23dBm | 10dBm/MHz |  |
|  |  | VLP (see 4.3.7) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) |  |
|  | Singapore | LPI (see 4.3.8) | 5925 – 6425MHz | 24dBm | 11dBm/MHz |  |
|  |  | VLP (see 4.3.8) |  | 14dBm | 1dBm/MHz |  |
| Note 1: For the exact conditions of operation, refer to the sub-clause which is mentioned in parenthesis (e.g. for the exact conditions for operating SP in US, refer to sub-clause 4.2.1.) |  |  |  |  |  |  |

# 5	Possible band plans

Editor’s note: List of the band plans which may contribute to harmonization with respect to the frequency usage in 5.925-7.125 GHz in the different regions

## 5.1	Licensed operations in 6 GHz

## 5.2	Unlicensed operations in 6 GHz

# 6	Conclusions

Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-03 | RP-79 | RP-180119 | - | - | - | Skeleton report of TR 37.890 | 0.0.0 |
| 2018-03 | RP-79 | RP-180547 | - | - | - | Added the following Text Proposals: - RP-180164: “TP to TR 37.890 Abbreviations section” - RP-180165: “TP to TR 37.890 General Regulatory Framework section” - RP-180548: “TP to TR 37.890 Regulatory Aspects USA section” - RP-180167: “TP to TR 37.890 EU Regulatory Aspects Europe section” | 0.1.0 |
| 2018-03 | RP-79 | RP-180565 | - | - | - | Updated version number | 0.2.0 |
| 2018-06 | RP-80 | RP-181021 | - | - | - | Corrected Table 4.1.1.3-1 according to TP RP-181018 | 0.3.0 |
| 2018-12 | RP-82 | RP-182311 |  |  |  | Updated with outcomes from FCC NPRM | 0.4.0 |
| 2019-06 | RP-84 | RP-190961 |  |  |  | Updated with outcomes from CEPT SE45 coexistence studies and information from China | 0.5.0 |
| 2019-09 | RP-85 | RP-192008 |  |  |  | Updated with agreed TP from last RAN#84 meeting and ETSI TFES TR timeline update | 0.6.0 |
| 2019-12 | RP-86 | RP-192510 |  |  |  | Fixed formatting issue (not aligned with ETSI styles) Added section 4.4 | 0.7.0 |
| 2020-07 | RP-88e | RP-201124 |  |  |  | Updated with FCC Report and Order decisions | 0.8.0 |
| 2020-09 | RP-89e | RP-201944 |  |  |  | Updated with CEPT additional information/clarification (RP-202072) and RCC information (RP-202027). | 0.9.0 |
| 2020-12 | RP-90e | RP-202273 |  |  |  | Updated with CEPT conclusions (RP-202831) | 0.10.0 |
| 2021-03 | RP-91e | RP-210857 |  |  |  | Updated with decision from South Korea | 0.11.0 |
| 2021-06 | RP-92e | RP-211179 |  |  |  | Updated with decisions from RCC, UK, Canada, Brazil, Peru and Chile. (RP-211515) | 0.12.0 |
| 2021-09 | RP-93e | RP-211929 |  |  |  | Updated with EC Decision 2021/1067, decisions from Honduras and Costa Rica and additional information for Saudia Arabia and Mexico. (RP-212575). | 0.13.0 |
| 2021-12 | RP-94-e |  |  |  |  | Withdrawn | 0.14.0 |
| 2022-03 | RP-95-e | RP-220914 RP-220915 |  |  |  | Updated with RCC Recommendation Updated with updates from Saudi Arabia and Chile, decisions in United Arab Emirates, ATU, Morocco and Colombia | 0.15.0 |
| 2022-06 | RP-96 | RP-221570 |  |  |  | Updated with decisions from Australia and Hong Kong | 0.16.0 |
| 2022-09 | RP-97-e | RP-222011 |  |  |  | Updated with decisions from New Zealand, Japan and Malaysia | 0.17.0 |
| 2022-12 | RP-98e | RP-223483 |  |  |  | Updated with decisions from Kenya, Qatar, Jordan, Chile, Colombia, Dominic Republic and Japan. | 0.18.0 |
| 2023-03 | RP-99 | RP-230414 |  |  |  | Updated with decision from Russian federation and regulation updates from South Korea and Mexico | 0.19.0 |
| 2023-06 | RP-100 | RP-231008 |  |  |  | Updated with decision from Argentina | 0.20.0 |
| 2023-09 | RP-101 | RP-231703 |  |  |  | Editorial clean up (MCC) Updated with decision from South Africa and Singapore Updates from China and Honk Kong SAR | 0.21.0 |