3GPP TSG RAN meeting #101																	RP-231704

Bangalore, India, September 11-15, 2023

Source:				Ericsson

Title:					TP to TR 37.890 capturing latest updates and fixing some editorial issues

Agenda item:		9.2.1

Document for:	Approval

# Introduction

The RAN-led study item on 6 GHz band for LTE and NR captures the latest status of Regulators decision for the 6 GHz frequency range.

This text proposal is capturing the latest updates from China, fixes some drafting rules issues (e.g. hanging paragraph, tables’ title, empty rows in table 4.5-1) and clean up some other text.

The comments below have been added to clarify each update, they should be removed when the TP will be added to the TR.

# Text proposal

<Start of changes>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	RP-172804: “Feasibility Study on 6 GHz for LTE and NR”, Ericsson, Verizon Wireless, Qualcomm Incorporated.

[3]					ITU-R Radio Regulations, Articles, Edition 2016;

[4]		FCC ONLINE TABLE OF FREQUENCY ALLOCATIONS, 47 C.F.R. § 2.106, December 13, 2017;

[5]	FCC 17-104, Notice of Inquiry, “Expanding Flexible Use in Mid-Band Spectrum Between 3.7 and 24 GHz”;

[6]	Comments of IEEE 802, in GN Docket No. 17-183;

[7]	APPLE INC., BROADCOM LIMITED,,CISCO SYSTEMS, INC., FACEBOOK, INC., GOOGLE LLC, HEWLETT PACKARD ENTERPRISE, INTEL CORPORATION,MEDIATEK INC., MICROSOFT CORPORATION, and QUALCOMM INCORPORATED, in GN Docket No. 17-183;

[8] 	Reply Comments of the Wireless Internet Service Providers Association, in GN Docket No. 17-183;

[9]	Comments of Ericsson, in GN Docket No. 17-183;

[10]	Comments of T-Mobile USA, in GN Docket No. 17-183;

[11]	Comments of Verizon, in GN Docket No. 17-183;

[12]	Reply Comments of the Satellite Industry Association, in GN Docket No. 17-183;

[13]	Reply Comments of the Fixed Wireless Communications Coalition, in GN Docket No. 17-183;

[14]	Comments of Dynamic Spectrum Alliance, in GN Docket No. 17-183;

[15]	Comments of the National Spectrum Management Association, in GN Docket No. 17-183;

[16]	Comments of CTIA, in GN Docket No. 17-183;

[17]	Reply Comments of Cisco Systems, Inc., in GN Docket No. 17-183;

[18] 	Reply Comments of WI-FI Alliance, in GN Docket No. 17-183;

[19]	PART 15 - Radio Frequency Devices, Title 47 of electronic Code of Federal Regulations;

[20]	The European Table of Frequency Allocations and applications in the frequency range 8.3 kHz and 3000 GHz (ECA Table), October 2017;

[21]	RSCOM17-53rev1- Mandate to CEPT to study and identify harmonised compatibility and sharing conditions for wireless access systems including radio local area networks in the band 5925-6425 MHz for the provision of wireless broadband services.

[22]	ETSI TR 103 524 System Reference document (SRdoc), “Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) in the band 5 925 MHz to 6 725 MHz”, v1.1.1, October 2018

[23]	CEPT/ERC/REC 74-01: “Unwanted Emissions in the Spurious Domain”;

[24]	ECC Report 302, “Sharing and compatibility studies related to Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) in the frequency band 5925-6425 MHz, May 2019.

[25]	FCC Notice of Proposed Rulemaking. FCC 18-147. October 24, 2018

[26]	ETSI TR 103 612, "IMT cellular networks; Mobile/Fixed Communication Network (MFCN) in the frequency range 6 425 - 7 125 MHz", v1.1.1, December 2019

[27]	ETSI TR 103 631, "Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) in the band 6 725 MHz to 7 125 MHz", v1.1.1, March 2019

[28]	ECC Report 302, “Sharing and compatibility studies related to Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) in the frequency band 5925-6425 MHz”

[29]	CCSA-TC5-WG8-2019-003 Project Proposal on the feasibility study of IMT system using 5925-7125MHz frequency band, http://www.ccsa.org.cn/tc/meeting.php?meeting_id=6243#

[30]	World Radiocommunication Conference 2019 (WRC-19) Provisional Final Acts, ITU-R https://www.itu.int/dms_pub/itu-r/opb/act/R-ACT-WRC.13-2019-PDF-E.pdf

[31]				Report and order and further notice of proposed rulemaking, FCC 20-51

[32]	ECC Report 316, “Sharing studies assessing short-term interference from Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) into Fixed Service in the frequency band 5925-6425 MHz”, 21 May 2020

[33]	CEPT Report 075, “to study feasibility and identify harmonised technical conditions for Wireless Access Systems including Radio Local Area Networks in the 5925-6425 MHz band for the provision of wireless broadband services”; Report B: Harmonised technical parameters for WAS/RLANs operating on a coexistence basis with appropriate mitigation techniques and/or operational compatibility/coexistence conditions, operating on the basis of a general authorisation. , November 2020

[34]	ECC Decision (20)01; “On the harmonised use of the frequency bands 5945 to 6425 MHz for the implementation of Wireless Access Systems including Radio Local Area Networks (WAS/RLAN)”, November 2020

[35]	ETSI TR 103 524, "System Reference document (SRDoc); Wireless access systems including radio local area networks (WAS/RLANs) in the band 5925 MHz to 6725 MHz"

[36]	EN 303 687, “"6 GHz RLAN Harmonised Standard for access to radio spectrum", Draft

[37]	doc 2.1_LS to 3GPP TSG RAN – ENG (18th meeting of the RCC Commission on Spectrum and Satellite Orbits), [to be updated]

[38]	CEPT Report 073, “to study feasibility and identify harmonised technical conditions for Wireless Access Systems including Radio Local Area Networks in the 5925-6425 MHz band for the provision of wireless broadband services”; Report A: Assessment and study of compatibility and coexistence scenarios for WAS/RLANs in the band 5925-6425 MHz, Approved on 6 March 2020 by ECC.

[39]	Korea’s Ministry of Science and ICT, "Technical standards for radio equipment for radio stations", URL: https://www.law.go.kr/admRulLsInfoP.do?admRulSeq=2100000196974

[40]	Ofcom,  "Improving spectrum access for Wi-Fi; Spectrum use in the 5 GHz and 6 GHz bands", July 2020, URL: https://www.ofcom.org.uk/__data/assets/pdf_file/0036/198927/6ghz-statement.pdf

[41]	Communications & Information Technology Commission, "Spectrum Outlook for Commercial and Innovative Use 2021- 2023", January 2021, URL: https://www.citc.gov.sa/ar/new/publicConsultation/Documents/Spectrum%20Outlook%20for%20Commercial%20and%20Innovative%20(2021-2023).pdf

[42]	Innovation, Science and Economic Development Canada, "Decision on the Technical and Policy Framework for Licence-Exempt Use in the 6 GHz Band", May 2021, URL: https://www.ic.gc.ca/eic/site/smt-gst.nsf/eng/sf11698.html

[43]	National Telecommunications Agency (ANATEL), "ACT NO. 1306", February 2021, URL: https://sei.anatel.gov.br/sei/modulos/pesquisa/md_pesq_documento_consulta_externa.php?eEP-wqk1skrd8hSlk5Z3rN4EVg9uLJqrLYJw_9INcO7uvjUt3vSOwT_4Z5fukj9yIzPErY4KWH5cpE9W_9hcTZkCG-vLPIdpXyuhgMG-L9M-uBLoSdAAXO0clb3SIt1i

[44]	Ministerio de Transportes y Comunicaciones, "Resolución Ministerial N° 373-2021-MTC/01", April 2021, URL: https://cdn.www.gob.pe/uploads/document/file/1861732/Resolución%20Ministerial%20nro%20373-2021-MTC/01.pdf

[45]	MINISTERIO DE TRANSPORTES Y TELECOMUNICACIONES; SUBSECRETARÍA DE TELECOMUNICACIONES, "RESOLUCIÓN 1985 EXENTA", October 2020, URL: https://www.bcn.cl/leychile/navegar?idNorma=1109333&idParte=9841504&idVersion=&r_c=6

[46]	RP-210957, Liaison statement to 3GPP TSG RAN on inclusion of the frequency band 6425-7125 MHz in 3GPP specification for 5G-NR/IMT-2020 systems, Regional Commonwealth in the filed of Communications.

[47]		COMMISSION IMPLEMENTING DECISION (EU) 2021/1067, on the harmonised use of radio spectrum in the 5 945-6 425 MHz frequency band for the implementation of wireless access systems including radio local area networks (WAS/RLANs), 17 June 2021.

[48]		The National Telecommunications Commission (CONATEL) of Honduras, "Resolution NR 003/21", March 2021, URL: http://www.conatel.gob.hn/doc/Regulacion/resoluciones/2021/NR003-21.pdf

[49]		The Superintendencia de Telecomunicaciones (SUTEL), "DECRETO EJECUTIVO N° 42924-MICITT", April 2021, URL: https://www.imprentanacional.go.cr/pub/2021/04/30/ALCA87_30_04_2021.pdf

[50]	Communications & Information Technology Commission, "Radio Spectrum Allocation and Use Regulation for WLAN Application", June 2021, URL: https://www.citc.gov.sa/en/new/publicConsultation/Documents/144207-en.pdf

[51]	Communications & Information Technology Commission, “Public Consultation on Spectrum Light Licensing”, August 2021, URL: https://www.citc.gov.sa/ar/new/publicConsultation/Documents/EN_PublicConsultationonLightLicensing-144301.pdf

[52]	Communications & Information Technology Commission, “WLAN regulations”, January 2022, URL: https://www.citc.gov.sa/en/mediacenter/pressreleases/Pages/2022022101.aspx

[53]	UAE Telecommunication Regulatory Authority, "Ultra-Wide Band and Short Range Devices", Version 4.0, December 2020, URL: https://tdra.gov.ae/-/media/About/regulations-and-ruling/EN/UWB-and-SRD-Regs-V4-0-pdf.ashx

[54]	National Telecommunications Regulatory Agency (ANRT), "Decision ANRT/DG/N°07/21", May 2021, URL: https://www.anrt.ma/sites/default/files/decision_a2fp_-vf-_mod_07.05.2021.pdf

[55]	MINISTERIO DE TRANSPORTES Y TELECOMUNICACIONES; SUBSECRETARÍA DE TELECOMUNICACIONES, "RESOLUCIÓN 1325 EXENTA", July 2021, URL: https://www.bcn.cl/leychile/navegar?idNorma=1162978&idParte=10255091

[56]	Agencia Nacional del Espectro (ANE), "Por medio de la cual se modifica la Resolución 105 de 2020 y se actualiza el Cuadro Nacional de Atribución de Bandas de Frecuencia", October 2021, URL: https://www.ane.gov.co/Documentos%20compartidos/ArchivosDescargables/Normatividad/Planeacion_del_espectro/RESOLUCIÓN%20No%20000105%20DE%2027-03-2020(1)%20(1).pdf

[57]	African Telecommunication Union, "ATU-R Recommendation 005-0", July 2021, URL: https://www.atuuat.africa/wp-content/uploads/2021/08/En_ATU-R-Recommendation-005-0.pdf

[58]	African Telecommunication Union, "2nd ATU Preparatory Meeting for WRC-23 (APM23-2); Agenda Item 1.2", September 2021, URL: https://docs.google.com/document/d/1u0Oz6o3gnH6oTGY-sE6_BTarNf2RUuSx/edit

[59]		RCC Recommendation 1/21 “Harmonization of the technical conditions for 5G-NR / IMT-2020 systems in the RCC countries in the frequency band 6 425-7 125 MHz or in its portions” (RP-213605).

[60]	Recommendation ITU-R SM.329-12, Unwanted emissions in the spurious domain, 09/2012.

[61]	The Australian Communications and Media Authority, "Radiocommunications (Low Interference Potential Devices) Class Licence Variation 2022 (No. 1)", March 2022, URL: https://www.legislation.gov.au/Details/F2022L00249/Html/Text

[62]	HKCA 1081, " PERFORMANCE SPECIFICATION FOR RADIOCOMMUNICATIONS APPARATUS OPERATING IN THE 6 GHz BAND FOR WIRELESS LOCAL AREA NETWORK", April 2022, URL: https://www.ofca.gov.hk/filemanager/ofca/en/content_401/hkca1081.pdf

[63]	Radio Spectrum Management, "WLAN use in the 6 GHz Band: Outcomes and Other Updates", August 2022, URL: https://www.rsm.govt.nz/projects-and-auctions/completed-projects/wlan-use-in-the-6-ghz-band/

[64]	Ministry of Internal Affairs and Communications, "Technical conditions for introduction of 6GHz band wireless LAN", April 2022, URL: https://www.soumu.go.jp/menu_news/s-news/01kiban12_02000142.html

[65]	Malaysian Communications and Multimedia Commission (MCMC), "Class Assignment No. 1 of 2022", January 2022, URL: https://www.mcmc.gov.my/skmmgovmy/media/General/CA-No-1-of-2022_-signed_19012022.pdf

[66]	Communications Authority of Kenya (CAK), "Guidelines On the Use of Radiofrequency Spectrum by Short Range Devices", June 2022, URL: https://www.ca.go.ke/wp-content/uploads/2022/07/Guidelines-on-the-Use-of-Radiofrequency-Spectrum-by-Short-Range-Devices-2022.pdf

[67]	Communications Regulatory Authority (CRA), "Class License for the use of RLAN devices over 5925-6425 MHz Band", April 2022, URL: https://www.cra.gov.qa/-/media/System/D/2/5/8/D258CF18B83A5613B0D590193CB799CB/Class-License-WIFI-6E-Final-English-032022--V3.ashx

[68]	The Hashemite Kingdom of Jordan Telecommunications Regulatory Commision (TRC), "Short Range Station License", May 2022, URL: https://trc.gov.jo/EchoBusV3.0/SystemAssets/PDF/AR/الطيف%20الترددي%20الراديوي/ترخيص%20الترددات/التعليمات%20والاجراءات/20be3e3e-3013-47cf-8cbc-b0280118acc0_SRD%20license.pdf

[69]	<to be updated>

[70]	MINISTERIO DE TRANSPORTES Y TELECOMUNICACIONES; SUBSECRETARÍA DE TELECOMUNICACIONES, "RESOLUCIÓN 2844 EXENTA  MODIFICA RESOLUCIÓN N° 1.985 EXENTA, DE 2017, DE LA SUBSECRETARÍA DE TELECOMUNICACIONES", September 2022, URL: https://www.bcn.cl/leychile/navegar?idNorma=1181305&idVersion=2022-09-14

[71]	Agencia Nacional del Espectro (ANE), " RESOLUCIÓN No. 000737", November 2022, URL: https://www.mintic.gov.co/portal/715/articles-273182_recurso_1.pdf

[72]	Instituto Dominicano de las Telecomunicaciones (INDOTEL), “RESOLUCIÓN NÚM. 082-2022”, September 2022, URL: https://transparencia.indotel.gob.do/media/217225/res_082_2022.pdf

[73]	Korea’s Ministry of Science and ICT, "Technical standards for radio equipment for radio stations", URL: https://www.law.go.kr/admRulLsInfoP.do?admRulSeq=2100000217697&vSct=202275

[74]	Ministry of the digital development and communication, "Заседание ГКРЧ от 23 декабря 2022 г. (протокол № 22-65)", December 2022, URL: https://digital.gov.ru/ru/documents/8628/

[75]	Instituto Federal de Telecomunicaciones, “El pleno del IFT clasifica la banda de 5925-6425 MHz como espectro libre”, February 2023, press release 013/2023

[76]		Ente Nacional de Comunicaciones (ENACOM), "Resolución 756/2023", May 2023, URL: https://www.boletinoficial.gob.ar/detalleAviso/primera/287126/20230524

[77]	Ministry of Industry and Information Technology of the People’s Republic of China, new National Table for Frequency Allocation, URL: https://www.miit.gov.cn/jgsj/wgj/gzdt/art/2023/art_92c8962a03a44a37becc2963cb3c8df9.html

<End of changes>

<Start of changes>

#### 4.1.1.3	Unlicensed operations

#### 4.1.1.3.0	Background

In December 2017, EC Radio Spectrum Committee finalized the Mandate to CEPT [21] to study and identify harmonised compatibility and sharing technical conditions for a sustainable and efficient use on a shared basis of the frequency range 5.925-6.425 GHz for Wireless Access Systems including Radio Local Area Networks (WAS/RLANs).

CEPT created two project teams, ECC SE 45 (coexistence) and ECC FM 57 (regulatory). ECC SE 45 was tasked to undertake compatibility and sharing studies in the 5.925-6.425 GHz frequency range to support ECC FM 57, and the latter defined the regulatory rules for the band. The scope of the work items in these groups was the introduction of low power wireless access systems (including RLAN) in the frequency range 5.925-6.425 GHz under a licence-exempt / general authorisation regulatory regime,, ensuring certainty of continued operation, development and protection of existing services (Fixed Services (FS), Fixed Satellite Service (FSS)) considering RR 5.440 and 5.458.

Currently, no WAS/RLAN or 5G NR coexistence studies have been undertaken within CEPT for the 6.425-7.125 GHz range.

ETSI has published the TR 103 524 [22] providing information on the technical parameters for WAS/RLANs to support the CEPT Work Items activities covering the frequency range 5.925 GHz to 6.425 GHz. In addition, it contains a request for considering additional frequencies up to 6.725 GHz. Furthermore, ETSI published TR 103 631 [27] providing information on the intended applications, the technical parameters, mitigation techniques, the relation to the existing spectrum regulation and additional new radio spectrum requirements for technology neutral wireless access systems including radio local area networks (WAS/RLANs) capable of operating in the 6725 MHz to 7125MHz range.

##### 4.1.1.3.1 	Coexistence aspects

<End of changes>

<Start of changes>

#### 4.2.1.3	Unlicensed operations

##### 4.2.1.3.0	Overview

According to comments in GN Docket No. 17-183, the whole 5.925-7.125 GHz frequency range represents a unique opportunity to address the unlicensed spectrum crunch quickly and efficiently, due to the band’s favourable propagation characteristics, proximity to existing unlicensed operations, and ability to support the wide channels that advanced applications and gigabit services require [6,7,8,14,17,18]. It is anyway requested that the introduction of unlicensed use in such frequency range is subjected to the requirement for a detailed engineering analysis and mitigation proposals ensuring that incumbents operations are fully protected [6, 7, 8, 9, 10, 11, 13, 14, 16, 17, 18].

As adopted in the Report and Order [31], the Commission decided to implement the following four Unlicensed National Information Infrastructure (U-NII) sub-bands in the 6 GHz band:

- U-NII-5: 5.925 GHz to 6.425 GHz (sharing with FS and FSS incumbents)

- U-NII-6: 6.425 GHz to 6.525 GHz (sharing with BAS and CARS)

- U-NII-7: 6.525 GHz to 6.875 GHz (same as U-NII-5)

- U-NII-8: 6.875 GHz to 7.125 GHz (sharing with BAS and CARS)

The Commission further authorized:

- Unlicensed standard-power access points in the U-NII-5 and U-NII-7 bands through use of an Automated Frequency Coordination (AFC).

- Unlicensed indoor low-power access point in the entire 6 GHz band, opening up 320 MHz channels operating in that band.

<End of changes>

<Start of changes>

### 4.3.1	China

In April 2019, CCSA established a new project on technical report for the feasibility study of IMT system using 5925-7125 MHz frequency band [29].

This CCSA study includes identifying IMT parameters in 5925-7125MHz in order to develop coexistence between IMT system and incumbent services and applications.

The MIIT published its new NTFA (national table for frequency allocation) which is effective from July 1st 2023. In the new NTFA, the frequency band of 6425-7125 MHz is identified for IMT [77].

<End of changes>

<Start of changes>

### 4.3.2	South Korea

In June 2020, Korea’s Ministry of Science and ICT (MSICT) issued an amendment of technical standards that also includes proposed rules for the 5925–7125MHz frequency band, which was finally approved in October 2020 [39]. In May 2022, the usage of LPI mode inside metro was additionally allowed [73]. The key points are as follows:

-	The whole range of 5925–7125MHz is allocated for the license-exempt low power indoor:

-	the occupied channel bandwidth should be less or equal 160MHz;

-	the EIRP limit is 24dBm;

-	the maximum power spectral density is 2dBm/MHz;

-	a device shall use power supply from a wired connection, and a client battery powered device must communicate through the device connected to a wired power supply;

-	this frequency range cannot be used for indoor automotive or airplane deployments;

-	usage inside moving vehicles, except for metro, is prohibited;

-     usage inside metro is allowed only in the frequency range of 5925-6425MHz.

-	The 5925-6425MHz frequency range is allocated for the license-exempt very low power indoor and outdoor:

-	the occupied channel bandwidth should be less or equal 160MHz;

-	the EIRP limit is 14dBm;

-	the maximum power spectral density is 1dBm/MHz;

-	this frequency range cannot be used for drones;

-	automotive embedded systems shall use 6085-6425MHz.

-	Unwanted emissions at frequencies outside the operational frequency range shall be below -27dBm/MHz.

-	Unwanted emissions at frequencies outside the operational frequency of 5925-6445MHz (very low power under the conditions for that range described above) shall be below -34dBm/MHz.

-	Transmitter and receiver spurious emission limits should be as specified in table 4.2.3-1.

Table 4.3.2-1:	Transmitted and receiver spurious emission limits – South Korea

| Frequency Range | Maximum level (average) | Measurement bandwidth |
| --- | --- | --- |
| f<1GHz | -54dBm | 100kHz |
| f>1GHz | -47dBm | 1MHz |

<End of changes>

<Start of changes>

## 4.5	Regulatory parameters comparative for license-exempt

The following table 4.5-1 compares the regulatory parameters for spectrum allocated to license-exempt.

## Table 4.5-1: Comparative of the regulatory parameters for license-exempt

| Region |  | Country |  | Permissible operation (Note 1) |  | Frequency range |  | Maximum mean EIRP for in-band emissions |  | Maximum mean EIRP density for in-band emissions |  | Maximum mean EIRP density for out-of-band emissions |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Region 1 |  | EU/CEPT |  | LPI (see 4.1.1) |  | 5945 – 6425MHz |  | 23dBm |  | 10dBm/MHz |  | -22 dBm/MHz (below 5935MHz) |  |
|  |  |  |  | VLP (see 4.1.1) |  |  |  | 14dBm |  | 1dBm/MHz 10dBm/MHz (for the narrowband usage) |  | -45 dBm/MHz (below 5935MHz); |  |
|  |  | UK |  | LPI (see 4.1.3) |  | 5925 – 6425MHz |  | 24dBm |  | 11dBm/MHz |  | In accordance with directive 2014/53/EC |  |
|  |  |  |  | VLP (see 4.1.3) |  |  |  | 14dBm |  |  |  |  |  |
|  |  | UAE |  | LPI (see 4.1.5) |  | 5925 – 6425MHz |  | 24 dBm |  |  |  |  |  |
|  |  | Morocco |  | LPI (see 4.1.7) |  | 5925 – 6425MHz |  | 23 dBm |  |  |  |  |  |
|  |  |  |  | VLP (see 4.1.7) |  |  |  | 14 dBm |  |  |  |  |  |
|  |  | Saudi Arabia |  | LPI (see 4.1.4) |  | 5925 – 7125MHz |  | 30dBm (AP) 24dBm (CL) |  | 10dBm/MHz |  |  |  |
|  |  | Kenya |  | LPI (see 4.1.8) |  | 5925 – 6425MHz |  | 23dBm |  | 10dBm/MHz |  |  |  |
|  |  |  |  | VLP (see 4.1.8) |  |  |  | 14dBm |  | 1dBm/MHz |  |  |  |
|  |  | Qatar |  | LPI (see 4.1.9) |  | 5925 – 6425MHz |  | 23dBm |  |  |  |  |  |
|  |  |  |  | VLP (see 4.1.9) |  |  |  | 14dBm |  |  |  |  |  |
|  |  | Jordan |  | LPI (see 4.1.10) |  | 5925 – 6425MHz |  | 23dBm |  |  |  |  |  |
|  |  |  |  | VLP (see 4.1.10) |  |  |  | 14dBm |  |  |  |  |  |
|  |  | Russian Federation |  | LPI (see 4.1.11) |  | 5925 – 6425MHz |  | 23dBm |  | 10dBm/MHz |  |  |  |
|  |  |  |  | VLP (see 4.1.11) |  |  |  | 14dBm |  | 1dBm/MHz |  |  |  |
| Region 2 |  | US |  | SP (see 4.2.1) |  | 5925 – 6425MHz 6525 – 6875MHz |  | 36dBm (AP) 30dBm (CL) |  | 23dBm/MHz (AP) 17dBm/MHz (CL) |  | -27 dBm/MHz (outside operational range) |  |
|  |  |  |  | LPI (see 4.2.1) |  | 5925 – 7125MHz |  | 30dBm (AP) 24dBm (CL) |  | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |  |  |
|  |  | Canada |  | SP (see 4.2.2) |  | 5925-6875 MHz |  | 36dBm |  | 23dBm/MHz |  |  |  |
|  |  |  |  | LPI (see 4.2.2) |  | 5925-7125 MHz |  | 30dBm |  | 5 dBm/MHz |  |  |  |
|  |  |  |  | VLP (see 4.2.2) |  |  |  | 14dBm |  | -8dBm/MHz |  |  |  |
|  |  | Brazil |  | LPI (see 4.2.3) |  | 5925 – 7125MHz |  | 30dBm (AP) 24dBm (CL) |  | 5dBm/MHz (AP) -1dBm/MHz (CL) |  | -27 dBm/MHz (outside operational range) |  |
|  |  |  |  | VLP (see 4.2.3) |  |  |  | 17 dBm |  | -5 dBm/MHz |  |  |  |
|  |  | Peru |  | LPI (see 4.2.4) |  | 5925 – 7125MHz |  | 30dBm (AP) 24dBm (CL) |  | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |  |  |
|  |  | Chile |  | LPI (see 4.2.5) |  | 5925 – 6425MHz |  | 30dBm (AP) 24dBm (CL) |  | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |  |  |
|  |  |  |  | VLP (4.2.5) |  |  |  | 17 dBm |  |  |  |  |  |
|  |  | Costa Rica |  | LPI (see 4.2.8) |  | 5925 – 7125MHz |  | 30dBm (AP) 24dBm (CL) |  |  |  |  |  |
|  |  |  |  | VLP (see 4.2.8) |  |  |  | 14dBm |  |  |  |  |  |
|  |  | Colombia |  | LPI (see 4.2.9) |  | 5925 – 7125MHz |  | 30dBm (AP) 24dBm (CL) |  | 5dBm/MHz (AP) -1dBm/MHz (CL) |  | -27 dBm/MHz (outside operational range) |  |
|  |  | Dominican Republic |  | LPI (see 4.2.10) |  | 5925 – 7125MHz |  | 30dBm (AP) 24dBm (CL) |  | 8dBm/MHz (AP) 2dBm/MHz (CL) |  | -27 dBm/MHz (outside operational range) |  |
|  |  |  |  | VLP (see 4.2.10) |  |  |  | 14dBm |  | -8dBm/MHz |  |  |  |
|  |  | Argentina |  | LPI (see 4.2.11) |  | 5925 – 7125MHz |  | 30dBm (AP) 24dBm (CL) |  | 5dBm/MHz (AP) -1dBm/MHz (CL) |  | -27 dBm/MHz (outside operational range) |  |
| Region 3 |  | South Korea |  | LPI (see 4.3.2) |  | 5925 – 7125MHz |  | 24dBm |  | 2dBm/MHz |  | -27 dBm/MHz (outside operational range) |  |
|  |  |  |  | VLP (see 4.3.2) |  | 5925 – 6425MHz |  | 14dBm |  | 1dBm/MHz |  | -34 dBm/MHz (outside operational range of the VLP mode) |  |
|  |  | Hong Kong |  | LPI (see 4.3.3) |  | 5925 – 6425MHz |  | 23dBm |  | 10dBm/MHz |  | In accordance with ETSI EN 303 687 |  |
|  |  |  |  | VLP (see 4.3.3) |  |  |  | 14dBm |  | 1dBm/MHz |  |  |  |
|  |  | Australia |  | LPI (see 4.3.4) |  | 5925 – 6425MHz |  | 24dBm |  | 11dBm/MHz |  |  |  |
|  |  |  |  | VLP (see 4.3.4) |  |  |  | 14dBm |  | 1dBm/MHz |  |  |  |
|  |  | New Zealand |  | LPI (see 4.3.5) |  | 5925 – 6425MHz |  | 24dBm |  | 11dBm/MHz |  |  |  |
|  |  |  |  | VLP (see 4.3.5) |  |  |  | 14dBm |  | 1dBm/MHz |  |  |  |
|  |  | Japan |  | LPI (see 4.3.6) |  | 5925 – 6425MHz |  | 23dBm |  |  |  | -27dBm/MHz (below 5925MHz) -13dBm/MHz and -19dBm/MHz (above 6425MHz) |  |
|  |  |  |  | VLP (see 4.3.6) |  |  |  | 14dBm |  |  |  | -37dBm/MHz (below 5925MHz) -13dBm/MHz and -19dBm/MHz (above 6425MHz) |  |
|  |  | Malaysia |  | LPI (see 4.3.7) |  | 5925 – 6425MHz |  | 23dBm |  | 10dBm/MHz |  |  |  |
|  |  |  |  | VLP (see 4.3.7) |  |  |  | 14dBm |  | 1dBm/MHz 10dBm/MHz (for the narrowband usage) |  |  |  |
| Note 1: For the exact conditions of operation, refer to the sub-clause which is mentioned in parenthesis (e.g. for the exact conditions for operating SP in US, refer to sub-clause 4.2.1.) |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of changes>