3GPP TSG RAN meeting #101										RP-231712

Bangalore, India, September 11-15, 2023

## Status Report to TSG

Agenda item:			9.3.4.1

| WI / SI Name | NR support for dedicated spectrum less than 5MHz for FR1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_FR1_lessthan_5MHz_BW |  |  |  |  |
| Unique ID | 941012 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-231425 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 95% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Man Hung Ng |
|  | Company | Nokia |
|  | Email | man_hung.ng@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

An allocation of 0.25 TU is requested for RAN4#108bis and RAN4#109 meetings to complete the open issues in core part.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#114

17 contributions were submitted and summarized in [22]. The agreements will be implemented into the specifications by corresponding specification editors following the usual RAN1 procedure.

Agreement For 3MHz channel BW, the 15 PRBs CORESET#0 is obtained by puncturing the 9 upper PRBs of 𝑁RB CORESET = 24 CORESET#0. Agreement For 3MHz channel BW, for kssb and PRB offset for the determining the CORESET#0 position in frequency domain, - kssb follows legacy configuration. - Note: based on existing specifications, UE does not expect other values than kssb = 0 for 12PRBs CORESET#0, [kssb = 8 for 15PRBs CORESET#0 with offset 0 PRBs, kssb = 4 for 15RPBs CORESET#0 with offset 2 PRBs] - PRB offset - For 12 PRBs CORESET#0 transmission BW, PRB offset = 0 - For 15 PRBs CORESET#0 transmission BW, at least PRB offset = 0, [2] are supported - FFS if offset = 1, 3 are supported. Note: the above assumes that PRB offset to be the frequency gap between the first PRB of the actually transmitted 12/15 PRBs CORESET#0 and the first CRB overlapping with the first PRB of the actually transmitted 12 PRBs SS/PBCH block, how to capture it in the specification is left to editors. Agreement For CORESET#0 for less than 5MHz transmission bandwidth, the UE assumes the same precoding being used per REG bundle. Note: This depends on UE implementation whether channel estimation can be performed for partial REG bundle, if any. Agreement Revise the previous agreement as below, Agreement For 3MHz channel BW, for kssb and PRB offset for the determining the CORESET#0 position in frequency domain, - kssb follows legacy configuration. - Note: based on existing specifications, UE does not expect other values than kssb = 0 for 12PRBs CORESET#0, [kssb = 8 [or 20] for 15PRBs CORESET#0 with offset 0 PRBs, kssb = 4 [or 16] for 15RPBs CORESET#0 with offset 2 PRBs] - PRB offset - For 12 PRBs CORESET#0 transmission BW, PRB offset = 0 - For 15 PRBs CORESET#0 transmission BW, at least PRB offset = 0, [2] 2 are supported - FFS if offset = 1, 3 are supported. Note: the above assumes that PRB offset to be the frequency gap between the first PRB of the actually transmitted 12/15 PRBs CORESET#0 and the first CRB overlapping with the first PRB of the actually transmitted 12 PRBs SS/PBCH block, how to capture it in the specification is left to editors. Note: the range of the PRB offsets is only applicable to the introduced new sync. raster points for 3MHz channel BW and 100kHz channel raster. Agreement For 3MHz channel BW, Table 13-0 is used by the UE for CORESET#0 configuration only when the detected SSB is from a new synch raster point in RAN4. FFS: any specification impact. Agreement For dedicated spectrum with less than 5MHz transmission BW, if a UE is not provided initialDownlinkBWP, the location and size of the initial DL BWP is equal to that of CORESET#0 transmission BW, i.e., - For 12 PRBs CORESET#0 transmission BW, the initial DL BWP has 12 PRBs. - For 15 PRBs CORESET#0 transmission BW, the initial DL BWP has 15 PRBs. Note: whether/how to reflect the above is up to Editor. Conclusion (based on RAN1#113 agreement) For 2 symbols and 3 symbols for 15 PRBs CORESET#0, both interleaved and non-interleaved CCE to REG mapping are supported. Agreement For 5MHz channel BW, 20 PRBs CORESET#0 transmission BW is supported. - The upper 4 PRBs of the 𝑁RB CORESET = 24 CORESET#0 are punctured to obtain 20 PRBs CORESET#0. - Table 13-0 is used for configuring 20 PRBs CORESET#0 - Maximum number of CORESET#0 symbols is 3. Minimum number of CORESET#0 symbols is 2. - PRB offset = 0 - Only interleaved CCE to REG mapping is supported. - REG bundle size = 6 - Kssb follows legacy configuration. - Note: The 20 PRBs CORESET#0 is only valid for the new sync. raster (=921.45 MHz) for band n100, 5MHz channel BW - Note 1: UE can be configured with 20 PRBs BWP for UE capable of 20 PRBs CORESET#0

#### 2.1.2	Remaining Open issues

None.

## 2.2	RAN2

The required changes for this feature in the RAN2 specifications will be discussed together with other features in the Rel-18 feature list discussion.

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#108 Main

13 contributions were submitted and summarized in [1].

Draft CRs on system parameters were endorsed in [2-3], where the agreements on system parameters for NR <5MHz are specified.

CR on UE RF and EMC requirements were agreed in [4-5], where the agreements on UE RF and EMC requirements for NR <5MHz are specified.

#108 RRM

13 contributions were submitted and summarized in [6].

WF on RRM requirements for NR FR1 less than 5MHz BW was approved in [7], where the agreements and open issues on RRM requirements for NR <5MHz are listed.

Draft CRs on RRM requirements were endorsed in [8-11] and Big CR on RRM requirements was agreed (pending email approval) [12], where the agreements on RRM requirements for NR <5MHz are specified.

#108 BSRF

13 contributions were submitted and summarized in [13].

Draft CRs on BS RF requirements were endorsed in [14-18] and Big CR on BS RF requirements was agreed [19], where the agreements on BS RF requirements for NR <5MHz are specified.

CR on BS requirements was agreed in [20], where the agreements on BS EMC requirements for NR <5MHz are specified.

LS reply to ECC WGFM was approved in [21], where certain technical parameters for smaller than 5 MHz channel bandwidths in band n100 are provided, and the reasons why RAN4 has specified the in-block conducted output power requirements and additional unwanted emission limits for bands n100 and n101 are explained.

#### 2.4.2	Remaining Open issues

Core part:

- Treat the maximum conducted output power topic as an Rel-18 maintenance item for band n100.

- The number of samples required in different RRM requirements are further to be updated based on the simulation results.

Performance part:

Specify necessary UE/BS performance requirements for NR operation in dedicated FDD FR1 spectrum allocations from approximately 3MHz up to below 5MHz, corresponding to the core requirements:

- Specify necessary RRM performance requirements (RAN4)

- Specify necessary UE demodulation performance and CSI reporting requirements (RAN4)

- Specify necessary BS demodulation performance requirements (RAN4)

- Specify necessary BS conformance tests (RAN4)

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

[1]	R4-2314220, “Topic summary for [108][138] NR_FR1_lessthan_5MHz_BW”, Moderator (Nokia).

[2]	R4-2314762, “Draft CR 38.101-1 on Introduction of dedicated spectrum less than 5MHz for FR1 in clause 5.4 (sync raster)”, Nokia, Nokia Shanghai Bell.

[3]	R4-2314763, “Draft CR to TS 38.104 - Sync raster for 3 MHz channel bandwidth”, Ericsson.

[4]	R4-2314764, “CR to 38.101-1 Introduction of dedicated spectrum less than 5MHz for FR1”, Nokia, Ericsson, ZTE Corporation, Apple, Qualcomm Inc, Huawei, Mediatek.

[5]	R4-2313248, “CR to TS 38.124: Introduction of 3 MHz channel bandwidth”, Ericsson.

[6]	R4-2314163, “Topic summary for [108][218] NR_FR1_lessthan_5MHz_BW”, Moderator (Nokia, Nokia Shanghai Bell).

[7]	R4-2314273, “WF on RRM requirements for NR support for dedicated spectrum less than 5MHz for FR1”, Nokia.

[8]	R4-2314269, “draftCR on link recovery requirements for less than 5MHz”, Huawei, HiSilicon.

[9]	R4-2314481, “Draft CR for RLM for less than 5MHz”, MediaTek inc.

[10]	R4-2314485, “draftCR for 38.133 - inclusion of NR  intra-frequency measurements for 3MHz channel bandwidth”, Nokia, Nokia Shanghai Bell.

[11]	R4-2314443, “Draft CR to introduce RRC connected state mobility requiremnts for NR less than 5MHz”, Ericsson.

[12]	R4-2314487, “Big CR to TS 38.133 on RRM core requirements for NR support for dedicated spectrum less than 5MHz for FR1”, Nokia.

[13]	R4-2314240, “Topic summary for [108][304] NR_FR1_lessthan_5MHz_BW_BSRF”, Moderator (Nokia).

[14]	R4-2311669, “Draft CR to TS 38.104 on introduction of 3 MHz channel bandwidth in clauses 6.3 and 6.6”, Nokia, Nokia Shanghai Bell.

[15]	R4-2312019, “draftCR to TS38.104: the introduction of 3 MHz channel bandwidth”, ZTE Corporation.

[16]	R4-2313245, “Draft CR to TS 38.104 - Introduction of 3 MHz channel bandwidth”, Ericsson.

[17]	R4-2313593, “Draft CR to TS38.104: updated in-band blocking requirements for 3 MHz channel bandwidth (7.4.2)”, Huawei, HiSilicon.

[18]	R4-2313594, “Draft CR to TS38.104: updated RX IMD requirements for 3 MHz channel bandwidth (7.7)”, Huawei, HiSilicon.

[19]	R4-2311670, “Big CR to TS 38.104 on introduction of 3 MHz channel bandwidth”, Nokia, Nokia Shanghai Bell.

[20]	R4-2311675, “CR to TS 38.113 on introduction of 3 MHz channel bandwidth in clause 6.1”, Nokia, Nokia Shanghai Bell.

[21]	R4-2313910, “Reply LS on NR bandwidth smaller than 5 MHz at 900 MHz”, Nokia, Nokia Shanghai Bell, UIC.

[22]	R1-2308587, “Summary#3 of discussion on enhancements to operate NR on dedicated spectrum less than 5MHz”, Moderator (Lenovo).

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template