3GPP TSG RAN Meeting #101								RP-231717

Bangalore, India, September 11-15, 2023

## Status Report to TSG

Agenda item:			9.3.1.4

| WI / SI Name | NR MIMO evolution for downlink and uplink |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MIMO_evo_DL_UL |  |  |  |  |
| Unique ID | 940096 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-223276 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 12/2023 | Performance part: 06/2024 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 90% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |
| Rapporteur | Name | Yubo Yang |
|  | Company | Huawei |
|  | Email | yangyubo1@huawei.com |
| Rapporteur | Name | Riki Okawa |
|  | Company | NTT DOCOMO |
|  | Email | riki.ookawa.rp@nttdocomo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#114, the following agreements were made.

Multi-TRP enhancement

Agreement

On unified TCI framework extension for S-DCI based MTPR, a UE capability is used as the threshold for PDSCH reception scheduled/activated by DCI format 1_0/1_1/1_2 if the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2

- Note: Whether to reuse the legacy UE capability (timeDurationForQCL) as the threshold and corresponding candidate values are discussed in Rel-18 UE feature AI

Agreement

On unified TCI framework extension for S-DCI based MTRP, if twoPHRMode is configured, and two SRS resource sets for CB/NCB are configured (i.e., TDM PUSCH repetition is configured):

- If the UE provides a Type 1 PHR for a reference PUSCH transmission associated with the first indicated joint/UL TCI state, the UL PC parameter setting and PL-RS are obtained from the first indicated joint/UL TCI state.

- If the UE provides a Type 1 PHR for a reference PUSCH transmission associated with the second indicated joint/UL TCI state, the UL PC parameter setting and PL-RS are obtained from the second indicated joint/UL TCI state

Note: The support for two PHR modes for Rel-18 SFN/SDM based sTXMP will independently discussed.

Conclusion

The following is not supported in Rel-18:

On unified TCI framework extension, support a first and a second UL PC parameter settings for PUSCH/PUCCH/SRS configured in BWP-UplinkDedicated

- If the first/second indicated joint/UL TCI state applied to PUSCH/PUCCH/SRS transmission doesn’t include an UL PC parameter setting for PUSCH/PUCCH/SRS, the UE shall apply the first/second UL PC parameter setting for PUSCH/PUCCH/SRS configured in the corresponding UL BWP

  - For M-DCI based MTRP operation, the first and the second indicated joint/UL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

Agreement

On unified TCI framework extension for S-DCI based MTRP, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of AP CSI-RS resources in an AP CSI-RS resource set for BM/CSI is smaller than a threshold for AP CSI-RS reception:

- FFS: If there is any other DL signal in the same symbols as the AP CSI-RS

- If there is no DL signal in the same symbols as the AP CSI-RS:

  - If the UE is in FR1 or the UE supports the capability of two default beams for S-DCI based MTRP in FR2, the UE shall apply the first or the second indicated joint/DL TCI state to the AP CSI-RS according to the RRC configuration(s) provided to the AP CSI-RS resource or AP CSI-RS resource set

    - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold.

  - Otherwise, the UE shall apply the first indicated joint/DL TCI state to the AP CSI-RS.

- FFS: The definition of other DL signals

- Note: Whether to reuse the legacy UE capability (beamSwitchTiming/beamSwitchTiming-r16) as the threshold for AP CSI-RS reception is discussed in Rel-18 UE feature AI

Agreement

On unified TCI framework extension, if the UE supports NCJT CSI, the UE should support resource-level RRC configuration for informing that the UE shall apply the first or the second indicated joint/DL TCI state to AP CSI-RS resource

Agreement

On unified TCI framework extension for S-DCI based MTRP and M-DCI based MTRP, after NW response to TRP-specific BFR request, for PUSCH/PUCCH/SRS transmission applying the new beam (qnew):

- The values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l provided by p0AlphaSetforPUSCH associated with a value of ul-powercontrolId for the corresponding cell

- The value of PO_UE_PUCCHqu and the PUCCH power control adjustment state l provided by p0AlphaSetforPUCCH associated with a value of ul-powercontrolId for the corresponding cell

- The values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and the SRS power control adjustment state l provided by p0AlphaSetforSRS associated with a value of ul-powercontrolId for the corresponding cell

The value of ul-powercontrolId for the corresponding cell used above is the smallest one if the PUSCH/PUCCH/SRS transmission is associated with the first indicated joint/UL TCI state

The value of ul-powercontrolId for the corresponding cell used above is the smallest one if the PUSCH/PUCCH/SRS transmission is associated with second indicated joint/UL TCI state

For M-DCI based MTRP, the first/second indicated TCI states correspond to the indicated TCI states specific to CORESET pool index values 0 and 1, respectively

Agreement

On unified TCI framework extension for M-DCI based MTRP, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of AP CSI-RS resources in an AP CSI-RS resource set for BM/CSI is smaller than a threshold for AP CSI-RS reception:

- If there is no other DL signal in the same symbols as the AP CSI-RS:

  - If the UE is in FR1 or the UE supports the capability of default beam per coresetPoolIndex for M-DCI based MTRP in FR2:

    - Alt1: The UE shall apply the first or the second indicated joint/DL TCI state to the AP CSI-RS according to the RRC configuration(s) provided to the AP CSI-RS resources or AP CSI-RS resource set

    - Note: If the UE supports the capability of two default beam per coresetPoolIndex for M-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold.

  - Otherwise, the UE shall apply the indicated joint/DL TCI state specific to coresetPoolIndex value 0 to the AP CSI-RS resource set.

- FFS: If there is any other DL signal in the same symbols as the AP CSI-RS

- FFS: The definition of other DL signals

- Note: Whether to reuse the legacy UE capability (beamSwitchTiming/beamSwitchTiming-r16) as the threshold for AP CSI-RS reception is discussed in Rel-18 UE feature AI

Agreement

Support joint configuration of the presence of “TCI states selection” field for DCI format 1_1 and DCI format 1_2 in the same DL BWP

Agreement

On unified TCI framework extension for S-DCI based MTRP, the RRC configuration for indicating whether the first, second, or both of the indicated joint/DL TCI states is/are applied to PDSCH reception scheduled/activated by DCI format 1_0 can be provided per DL BWP

Agreement

On unified TCI framework extension for S-DCI based MTRP, if twoPHRMode is configured, and two SRS resource sets for CB/NCB and multipanelScheme for SDM/SFN are configured:

- If the UE determines that one or both Type 1 PHRs are based on an actual PUSCH transmission

  - If the actual PUSCH transmission applies both first and second indicated joint/UL TCI states, the UE provides the first {power headroom, configured maximum output power} associated with the first indicated joint/UL TCI state for the actual PUSCH transmission, and the second {power headroom, configured maximum output power} associated with the second indicated joint/UL TCI state for the actual PUSCH transmission

  - If the actual PUSCH transmission applies only the first indicated joint/UL TCI state, the UE provides the first {power headroom, configured maximum output power} associated with the first indicated joint/UL TCI state for the actual PUSCH transmission

    - FFS: How to provide the second report for a reference PUSCH transmission?

  - If the actual PUSCH transmission applies only the second indicated joint/UL TCI state, the UE provides the second {power headroom, configured maximum output power} associated with the second indicated joint/UL TCI state for the actual PUSCH transmission

    - FFS: How to provide the first report for a reference PUSCH transmission?

- FFS: If the UE determines that both Type 1 PHRs are based on reference PUSCH transmissions, how to provide the first and second reports for reference PUSCH transmissions, respectively?

Agreement

For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, support indication of additionalPCI in the PDCCH order

- as baseline capability: support PRACH triggering towards servingCell PCI or active additionalPCI.

Conclusion

For inter-cell/intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, there is no consensus to introduce RAR-less solution in Rel-18 from RAN1 perspective.

Conclusion

For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, for the optional feature where the overlapping duration of the later of the two UL transmissions is reduced the following is concluded:

- There is no consensus to specify the overlapping duration in RAN1 specifications.

Agreement

For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement and PDCCH order CFRA, indicate a representation of the TAG ID with 1 bit (either the first TAG ID or the second TAG ID for the serving cell) as part of TA command in RAR

Note: For intra-cell multi-DCI based Multi-TRP operation, only a single NTA,offset is configured.

Conclusion

For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, no consensus on introducing the following optional UE capability:

optional UE capability: support PRACH triggering towards servingCell PCI, active additionalPCI, or up to 1 inactive additionalPCI

Agreement

For inter-cell multi-DCI based multi-TRP operation with two TAGs configured in Spcell, when the PDCCH order is transmitted from a TRP associated with additionalPCI, PDCCH RAR and PDSCH RAR of a CFRA are both QCLed with the CORESET associated with the Type I CSS set

CSI enhancement

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’ for Capability 2 when NTRP>1, decide, in RAN1#114, based on the following alternatives:

- Alt1. r is dependent only on the value of NTRP (exact dependence TBD) and reported

- Alt2. r is dependent only on the value of NTRP and sub-carrier spacing (exact dependence TBD), reported

- Alt3. r is taken from a set of candidate values, e.g. {14, 28, legacy Z2’}

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each CSI-RS resource in the CSI-RS resource set no later than CSI reference resource and drops the report otherwise.

- FFS: Whether different behaviour is introduced when dynamic TRP selection is configured

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the CPU occupation,

- X is a “common value” and not dependent on NTRP or any other parameter value, reported by the UE from a set of candidate values {1, 1.5, 2} according to UE capability

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP, support

- that the bitmap is absent when all the coefficients are non-zero for Rel-17 based type-2 CJT codebook

- if NTRP =1, that the NTRP-bit bitmap (for dynamic TRP selection) is not reported

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding CBSR, whether to use only 1 bit or 2 bits per beam in a beam-group restriction is up to RAN2

- Note: RAN1 has previously agreed to support only 2 hypotheses per beam in a beam-group restriction for Rel-18 Type-II CJT codebook

- Send an LS to RAN2 regarding this agreement

LS is endorsed in R1-2308396.

Conclusion:

For the Rel-18 Type-II codebook refinement for CJT mTRP, there is no consensus on supporting the following proposals:

- amending the current agreement on reordering the unequal Ln and αn combinations without permutation in Table 5.2.2.2.8-1 and Table 5.2.2.2.9-1 in TS 38.214 in descending order, so that the smaller Ln/αn  values are assigned less priority

- for UCI part 2, amending the current agreement on encoding G0 and G1 together, and G2 independently

- regarding CSI calculation and measurement, adding the following restriction: a UE can assume that the configured NTRP CSI-RS resources comprising the CMR are located in the same RB(s)

- the need for specifying UE assumption(s) on the TCI state(s) associated with the configured NTRP CSI-RS resources comprising the CMR (assuming NTRP >1).

  - Note: It is understood that this issue is left for implementation

- regarding channel and interference measurement restriction, supporting additional UE behaviour beyond the current specification

- introducing an indicator in Part 2 CSI to indicate the reported per layer per TRP NZC bitmaps at least when the reported NNZC in Part 1 CSI is less than the number of per layer per TRP NZC bitmaps

- introducing an indicator for the number of all-zero per layer per TRP NZC bitmaps in Part 1 CSI

- specifying any TRP selection criterion

- specifying the following UE behavior: a UE does not expect to be configured with more than 1 value of Ltotal=n=1NTRPLn.

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the CPU occupation, the value of Y is reported by the UE (as a part of UE capability reporting) and not dependent on any codebook parameter value

- FFS (by RAN1#114): The candidate value(s) of Y

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ for Capability 2, decide, in RAN1#114, based on the following alternatives:

- Alt1: the value of r is dependent only on N4 (exact dependence TBD)

- Alt2: the value of r is dependent only on N4 and sub-carrier spacing (exact dependence TBD)

- Alt3: r is taken from a set of candidate values, e.g. {14, 28, legacy Z2’}

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z for Capability 2 associated with P/SP-CSI-RS, decide, in RAN1#114, based on the following alternatives:

- Alt1: w=14.(KP–1).d or 14. KP.d, where d denotes the CSI-RS periodicity

- Alt2: w=14 or 28 (fixed)

Agreement

For the Type-II codebook refinement for high/medium velocities, the UE reports a CSI report only if receiving at least X consecutive CSI-RS transmission occasion for each CSI-RS resource in the CSI-RS resource set no later than CSI reference resource, and/or one CSI-IM occasion for interference measurement, else drops the report otherwise.

- X=1 for AP-CSI-RS.

- X=KP for P/SP-CSI-RS, where KP denotes the scaling factor of active P/SP-CSI-RS resource counting

Note: This includes the cases of CSI report (re)configuration, serving cell activation, BWP change, activation of SP-CSI, or DRX configuration

Conclusion:

For the Type-II codebook refinement for high/medium velocities, there is no consensus on supporting the following proposals:

- in case of A/SP-CSI reporting repetition towards M-TRP, the start of CSI reporting window is slot l=n+δ, where n is the last uplink slot carrying the CSI report

- when the configured value of N4 is >1, if Z’ is not satisfied for CSI calculation, the UE falls back to N4=1

- for AP-CSI associated with P/SP-CSI-RS, to determine the SCS for Z', SCS of all associated P/SP-CSI-RS resource shall be considered in the minimum operation (cf. clause 5.4 in TS38.214).

- regarding channel and interference measurement restriction, additional UE behaviour beyond the current specification.

- 2-stage PDCCH triggering mechanisms to prevent the latency and throughput reduction of UL-SCH due to the PUSCH conveying aperiodic Type-II-Doppler CSI: A 1st PDCCH to trigger CSI measurement/computation (and AP CSI-RS, if applicable), and, a 2nd PDCCH to trigger report.

- when P/SP-CSI-RS is configured as the CMR, any restriction in CSI-RS periodicity

Agreement

For the Rel-18 TRS-based TDCP reporting, when Y delay(s) are configured, regarding CPU occupation, the value of X={1,2} is reported and not dependent on the configured value of D or whether phase reporting is ON/OFF

Agreement

For the Rel-18 TRS-based TDCP reporting, the supported values of KTRS (number of configured TRS resource sets) are {1,2,3}

- The candidate values {2,3} are UE optional

Agreement

For the Rel-18 TRS-based TDCP reporting, since all the UCI parameters are included in UCI part 1, TDCP reporting utilizes 1-part UCI

Conclusion:

For the Rel-18 TRS-based TDCP reporting, there is no consensus on supporting the following proposals:

- additional D value(s)

- TRS resource configuration where all the configured KTRS resource sets are aperiodic

- further restricting the use of any of the supported D values to any additional condition

- reverting the agreement for Dbasic from 1 slot to 5 slots

- when KTRS>1 resource sets are configured, restricting the number of configured resources for any configured resource set

Conclusion:

For the Rel-18 TRS-based TDCP reporting, there is no consensus on supporting the following:

- additional dropping rules beyond the current specification for CSI reporting

- regarding amplitude quantization, refining the previous agreement by replacing the agreed amplitude quantization entry index “15” (“1111” in binary) with an “invalid” value

- regarding channel and interference measurement restriction, supporting TD measurement restriction

Conclusion:

For the Rel-18 Type-II codebook refinement for CJT mTRP, there is no consensus on supporting different CSI reporting and dropping behaviour when dynamic TRP selection is configured

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z associated with P/SP-CSI-RS,

- W (unit of symbols) is reported by UE

  - To be finalized as part of Rel-18 UE feature discussions

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the CPU occupation, the candidate values of Y are {2/3, 1, 2, 3}

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding Z/Z’ for Capability 2 when NTRP>1, r=legacy Z2’

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ for Capability 2, r=legacy Z2’

Reference signal enhancement

Agreement

Confirm the following WA in RAN1#113.

  - For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, for case 2) in RAN1#113 agreement,

    - For 1CW,

      - 2) For row 24-30, 55-60, 69-80, support at least row 73-80 without MU restriction. Support row 24-30 with MU restriction (UE does not expect to be coscheduled with another UE in the same CDM group). Remove row 55-60.

        - FFS: for row 69-72

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, for case 3) and 4) in RAN1#113 agreement,

  - Remove row 81-82.

  - Remove row 83.

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, for case 5) in RAN1#113 agreement,

  - Support row 0-3.

Above does not imply that the support of MU-MIMO for two codewords

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for case 3) in RAN1#113 agreement, support Alt.3-2.

  - Alt.3-2: Remove row 60-62.

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, for case 4) in RAN1#113 agreement, support Alt.4-2.

  - Alt.4-2: Remove row 153-158.

Agreement

  - If Rel.18 eType1 DMRS ports with maxLength=2 is configured for PDSCH, the antenna ports field in DCI format 1_1/1_2 is

    - Alt. 1: increased by 2-bit from Rel.17.

  - If the number of rows of DMRS ports combinations for Rel.18 eType2 DMRS ports with maxLength=2 for PDSCH exceeds 128, the above is also applied to Rel.18 eType2 DMRS ports with maxLength=2 for PDSCH.

Agreement

If enhanced-dmrs-Type_r18 is configured for PDSCH, support the following values of PT-RS EPRE to PDSCH EPRE per layer per RE:

Table 4.1-2A: PT-RS EPRE to PDSCH EPRE per layer per RE ()

| epre-Ratio | The number of PDSCH layers with DM-RS associated to the PT-RS port |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 0 | 0 | 3 | 4.77 | 6 | 7 | 7.78 | 8.45 | 9 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | reserved |  |  |  |  |  |  |  |
| 3 | reserved |  |  |  |  |  |  |  |

Agreement

Confirm the following Working Assumption in RAN1#112 for CB based PUSCH:

  - To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables.

    - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table).

      - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1].

Agreement

Confirm the following Working Assumption in RAN1#112bis-e for CB based PUSCH:

- Adopt Table 7.3.1.1.2-12B/13B/14B/15B/16B/17B/20B/21B/22B/23B to support signalling >4 ranks PUSCH with Rel-15 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH.

- FFS: Whether/how some of bits in the antenna ports field can be reused for other purpose for >4 ranks PUSCH.

Table 7.3.1.1.2-12B: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-4 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-13B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-14B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-16B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-4 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-17B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-5 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-20B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-4 | 1 |
| 1 | 2 | 0,1,2,3,6 | 2 |
| 12-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-21B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-5 | 1 |
| 1 | 2 | 0,1,2,3,6,8 | 2 |
| 2-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-22B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Agreement

If UE does not support the orphan RE capability (i.e. UE can receive PDSCH without the scheduling restriction for FD-OCC length 4 in Rel.18 eType 1 DMRS), all the following scheduling restriction is applied for PDSCH transmission with fdmSchemeA or fdmSchemeB:

  - 1) The number of consecutively scheduled PRBs for PDSCH for each TCI-state is even.

    - If the precoding granularity is set to ‘wideband’, the total number of PRBs allocated to UE should be multiple of 4 to ensure the number of consecutively scheduled PRBs for PDSCH for each TCI-state is even.

  - 2) The number of PRBs offset of scheduled PDSCH for each TCI-state from point A (common resource block 0) is even.

Agreement

For 8Tx PUSCH, when the ptrs-Power configures 00, Alt.2 is supported for the factor () for partial coherent TPMIs:

  - Alt.2: 10 log10(Lx) +10 log10(Qp), where Lx is the number of PUSCH layers in the antenna group which are precoded coherently with the PUSCH layer / DMRS port where PTRS port x is associated with, and Qp is the number of PTRS ports scheduled to the UE.

Agreement

For partial/non-coherent PUSCH, if 2 port PTRS is configured in maxNrofPorts in PTRS-UplinkConfig, and if more than 4 layers is configured in maxMIMO-Layers [or MaxMIMO-LayersDCI-0-2 in PUSCH-ServingCellConfig],

  - Alt.1: The size of PTRS-DMRS association field is 4-bit in DCI format 0_1 [or DCI format 0_2].

Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 | 3 | 4th DMRS port which shares PTRS port 1 |

Agreement

The following MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports is not supported:

  - For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group.

    - UE does not expect such MU-MIMO within a CDM group

Agreement

If UE is scheduled with PDSCH with 2CWs with Rel.18 eType1/eType2 DMRS ports with maxLength=1/2,

  - Alt.1) the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.

Agreement

For the antenna ports indication in DCI format 0_1/0_2 for Rel.18 eType1 DMRS ports with maxLength = 2 for PUSCH, following Table 7.3.1.1.2-46, Table 7.3.1.1.2-47, Table 7.3.1.1.2-48, and Table 7.3.1.1.2-49 are supported.

  - Note: Row(s) agreed for PUSCH does not imply it is also agreed for PDSCH.

Table 7.3.1.1.2-46: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=2, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 2 | 0 | 2 |
| 7 | 2 | 1 | 2 |
| 8 | 2 | 2 | 2 |
| 9 | 2 | 3 | 2 |
| 10 | 2 | 4 | 2 |
| 11 | 2 | 5 | 2 |
| 12 | 2 | 6 | 2 |
| 13 | 2 | 7 | 2 |
| 14 | 1 | 8 | 1 |
| 15 | 1 | 9 | 1 |
| 16 | 2 | 8 | 1 |
| 17 | 2 | 9 | 1 |
| 18 | 2 | 10 | 1 |
| 19 | 2 | 11 | 1 |
| 20 | 2 | 8 | 2 |
| 21 | 2 | 9 | 2 |
| 22 | 2 | 10 | 2 |
| 23 | 2 | 11 | 2 |
| 24 | 2 | 12 | 2 |
| 25 | 2 | 13 | 2 |
| 26 | 2 | 14 | 2 |
| 27 | 2 | 15 | 2 |
| [28 | 1 | 8 | 2] |
| [29 | 1 | 9 | 2] |
| [30 | 1 | 12 | 2] |
| [31 | 1 | 13 | 2] |

Table 7.3.1.1.2-13-47: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=2, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 2 | 0,2 | 1 |
| 4 | 2 | 0,1 | 2 |
| 5 | 2 | 2,3 | 2 |
| 6 | 2 | 4,5 | 2 |
| 7 | 2 | 6,7 | 2 |
| 8 | 2 | 0,4 | 2 |
| 9 | 2 | 2,6 | 2 |
| 10 | 1 | 8,9 | 1 |
| 11 | 2 | 8,9 | 1 |
| 12 | 2 | 10,11 | 1 |
| [13 | 2 | 8,10 | 1] |
| 14 | 2 | 8,9 | 2 |
| 15 | 2 | 10,11 | 2 |
| 16 | 2 | 12,13 | 2 |
| 17 | 2 | 14,15 | 2 |
| [18 | 2 | 8,12 | 2] |
| [19 | 2 | 10,14 | 2] |
| [20 | 2 | 9,11 | 1] |
| [21 | 2 | 1,3 | 1] |
| [22 | 2 | 0,2 | 2] |
| [23 | 2 | 1,3 | 2] |
| [24 | 2 | 4,6 | 2] |
| [25 | 2 | 5,7 | 2] |
| [26 | 2 | 8,10 | 2] |
| [27 | 2 | 9,11 | 2] |
| [28 | 2 | 12,14 | 2] |
| [29 | 2 | 13,15 | 2] |
| [30 | 1 | 0,1 | 2] |
| [31 | 1 | 8,9 | 2] |
| [32 | 1 | 4,5 | 2] |
| [33 | 1 | 12,13 | 2] |

Table 7.3.1.1.2-14-48: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| [3 | 2 | 8-10 | 1] |
| [4 | 2 | 8,9,12 | 2] |
| [5 | 2 | 10,11,14 | 2] |
| 6 | 1 | 0,1,8 | 1 |
| 7 | 2 | 0,1,8 | 1 |
| 8 | 2 | 2,3,10 | 1 |
| [9 | 1 | 0,1,8 | 2] |
| [10 | 1 | 4,5,12 | 2] |
| [11 | 2 | 0,1,8 | 2] |
| [12 | 2 | 4,5,12 | 2] |
| [13 | 2 | 2,3,10 | 2] |
| [14 | 2 | 6,7,14 | 2] |
| [15 | 2 | 5,8,9 | 2] |
| [16 | 2 | 7,10,11 | 2] |
| [17 | 2 | 7,12,13 | 2] |
| 18-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15-49: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=2, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 2 | 0,1,4,5 | 2 |
| 2 | 2 | 2,3,6,7 | 2 |
| 3 | 2 | 0,2,4,6 | 2 |
| [4 | 2 | 8-11 | 1] |
| 5 | 2 | 8,9,12,13 | 2 |
| 6 | 2 | 10,11,14,15 | 2 |
| [7 | 2 | 8,10,12,14 | 2] |
| 8 | 1 | 0,1,8,9 | 1 |
| 9 | 2 | 0,1,8,9 | 1 |
| 10 | 2 | 2,3,10,11 | 1 |
| [11 | 1 | 0,1,8,9 | 2] |
| [12 | 1 | 4,5,12,13 | 2] |
| [13 | 2 | 0,1,8,9 | 2] |
| [14 | 2 | 4,5,12,13 | 2] |
| [15 | 2 | 2,3,10,11 | 2] |
| [16 | 2 | 6,7,14,15 | 2] |
| 17-31 | Reserved | Reserved | Reserved |

Agreement

For the antenna ports indication in DCI format 0_1/0_2 for Rel.18 eType2 DMRS ports with maxLength = 1 for PUSCH, following Table 7.3.1.1.2-54, Table 7.3.1.1.2-55, Table 7.3.1.1.2-56, and Table 7.3.1.1.2-57 are supported.

  - Note: Row(s) agreed for PUSCH does not imply it is also agreed for PDSCH.

Table 7.3.1.1.2-54: Antenna port(s), transform precoder is disabled, dmrs-Type=eType2, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6 | 3 | 0 |
| 7 | 3 | 1 |
| 8 | 3 | 2 |
| 9 | 3 | 3 |
| 10 | 3 | 4 |
| 11 | 3 | 5 |
| 12 | 1 | 12 |
| 13 | 1 | 13 |
| 14 | 2 | 12 |
| 15 | 2 | 13 |
| 16 | 2 | 14 |
| 17 | 2 | 15 |
| 18 | 3 | 12 |
| 19 | 3 | 13 |
| 20 | 3 | 14 |
| 21 | 3 | 15 |
| 22 | 3 | 16 |
| 23 | 3 | 17 |
| 24-31 | Reserved | Reserved |

Table 7.3.1.1.2-17-55: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 3 | 0,1 |
| 4 | 3 | 2,3 |
| 5 | 3 | 4,5 |
| 6 | 2 | 0,2 |
| 7 | 1 | 12,13 |
| 8 | 2 | 12,13 |
| 9 | 2 | 14,15 |
| 10 | 3 | 12,13 |
| 11 | 3 | 14,15 |
| 12 | 3 | 16,17 |
| [13 | 2 | 12,14] |
| [14 | 3 | 13,15] |
| [15 | 2 | 13,15] |
| 16-31 | Reserved | Reserved |

Table 7.3.1.1.2-18-56: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 3 | 0-2 |
| 2 | 3 | 3-5 |
| [3 | 2 | 12-14] |
| [4 | 3 | 12-14] |
| [5 | 3 | 15-17] |
| 6 | 1 | 0,1,12 |
| 7 | 2 | 0,1,12 |
| 8 | 2 | 2,3,14 |
| 9 | 3 | 0,1,12 |
| 10 | 3 | 2,3,14 |
| 11 | 3 | 4,5,16 |
| [12 | 3 | 13,15,17] |
| 13-31 | Reserved | Reserved |

Table 7.3.1.1.2-19-57: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 1 | 3 | 0-3 |
| [2 | 2 | 12-15] |
| [3 | 3 | 12-15] |
| 4 | 1 | 0,1,12,13 |
| 5 | 2 | 0,1,12,13 |
| 6 | 2 | 2,3,14,15 |
| 7 | 3 | 0,1,12,13 |
| 8 | 3 | 2,3,14,15 |
| 9 | 3 | 4,5,16,17 |
| 10-31 | Reserved | Reserved |

Agreement

For the antenna ports indication in DCI format 0_1/0_2 for Rel.18 eType2 DMRS ports with maxLength = 2 for PUSCH, following Table 7.3.1.1.2-62, Table 7.3.1.1.2-63, Table 7.3.1.1.2-64, and Table 7.3.1.1.2-65 are supported.

Note: Row(s) agreed for PUSCH does not imply it is also agreed for PDSCH.

Table 7.3.1.1.2-20-62: Antenna port(s), transform precoder is disabled, dmrs-Type=eType2, maxLength=2, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 3 | 0 | 1 |
| 7 | 3 | 1 | 1 |
| 8 | 3 | 2 | 1 |
| 9 | 3 | 3 | 1 |
| 10 | 3 | 4 | 1 |
| 11 | 3 | 5 | 1 |
| 12 | 3 | 0 | 2 |
| 13 | 3 | 1 | 2 |
| 14 | 3 | 2 | 2 |
| 15 | 3 | 3 | 2 |
| 16 | 3 | 4 | 2 |
| 17 | 3 | 5 | 2 |
| 18 | 3 | 6 | 2 |
| 19 | 3 | 7 | 2 |
| 20 | 3 | 8 | 2 |
| 21 | 3 | 9 | 2 |
| 22 | 3 | 10 | 2 |
| 23 | 3 | 11 | 2 |
| 24 | 1 | 0 | 2 |
| 25 | 1 | 1 | 2 |
| 26 | 1 | 6 | 2 |
| 27 | 1 | 7 | 2 |
| 28 | 1 | 12 | 1 |
| 29 | 1 | 13 | 1 |
| 30 | 2 | 12 | 1 |
| 31 | 2 | 13 | 1 |
| 32 | 2 | 14 | 1 |
| 33 | 2 | 15 | 1 |
| 34 | 3 | 12 | 1 |
| 35 | 3 | 13 | 1 |
| 36 | 3 | 14 | 1 |
| 37 | 3 | 15 | 1 |
| 38 | 3 | 16 | 1 |
| 39 | 3 | 17 | 1 |
| 40 | 3 | 12 | 2 |
| 41 | 3 | 13 | 2 |
| 42 | 3 | 14 | 2 |
| 43 | 3 | 15 | 2 |
| 44 | 3 | 16 | 2 |
| 45 | 3 | 17 | 2 |
| 46 | 3 | 18 | 2 |
| 47 | 3 | 19 | 2 |
| 48 | 3 | 20 | 2 |
| 49 | 3 | 21 | 2 |
| 50 | 3 | 22 | 2 |
| 51 | 3 | 24 | 2 |
| 52 | 1 | 12 | 2 |
| 53 | 1 | 13 | 2 |
| 54 | 1 | 18 | 2 |
| 55 | 1 | 19 | 2 |
| 56-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-21-63: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 3 | 0,1 | 1 |
| 4 | 3 | 2,3 | 1 |
| 5 | 3 | 4,5 | 1 |
| 6 | 2 | 0,2 | 1 |
| 7 | 3 | 0,1 | 2 |
| 8 | 3 | 2,3 | 2 |
| 9 | 3 | 4,5 | 2 |
| 10 | 3 | 6,7 | 2 |
| 11 | 3 | 8,9 | 2 |
| 12 | 3 | 10,11 | 2 |
| 13 | 1 | 0,1 | 2 |
| 14 | 1 | 6,7 | 2 |
| 15 | 2 | 0,1 | 2 |
| 16 | 2 | 2,3 | 2 |
| 17 | 2 | 6,7 | 2 |
| 18 | 2 | 8,9 | 2 |
| 19 | 1 | 12,13 | 1 |
| 20 | 2 | 12,13 | 1 |
| 21 | 2 | 14,15 | 1 |
| 22 | 3 | 12,13 | 1 |
| 23 | 3 | 14,15 | 1 |
| 24 | 3 | 16,17 | 1 |
| [25 | 2 | 12,14 | 1] |
| 26 | 3 | 12,13 | 2 |
| 27 | 3 | 14,15 | 2 |
| 28 | 3 | 16,17 | 2 |
| 29 | 3 | 18,19 | 2 |
| 30 | 3 | 20,21 | 2 |
| 31 | 3 | 22,23 | 2 |
| 32 | 1 | 12,13 | 2 |
| 33 | 1 | 18,19 | 2 |
| 34 | 2 | 12,13 | 2 |
| 35 | 2 | 14,15 | 2 |
| 36 | 2 | 18,19 | 2 |
| 37 | 2 | 20,21 | 2 |
| [38 | 3 | 13,15 | 1] |
| [39 | 2 | 13,15 | 1] |
| 40-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-22-64: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 3 | 0-2 | 1 |
| 2 | 3 | 3-5 | 1 |
| 3 | 3 | 0,1,6 | 2 |
| 4 | 3 | 2,3,8 | 2 |
| 5 | 3 | 4,5,10 | 2 |
| [6 | 2 | 12-14 | 1] |
| [7 | 3 | 12-14 | 1] |
| [8 | 3 | 15-17 | 1] |
| [9 | 3 | 12,13,18 | 2] |
| [10 | 3 | 14,15,20 | 2] |
| [11 | 3 | 16,17,22 | 2] |
| 12 | 1 | 0,1,12 | 1 |
| 13 | 2 | 0,1,12 | 1 |
| 14 | 2 | 2,3,14 | 1 |
| 15 | 3 | 0,1,12 | 1 |
| 16 | 3 | 2,3,14 | 1 |
| 17 | 3 | 4,5,16 | 1 |
| [18 | 1 | 0,1,12 | 2] |
| [19 | 1 | 6,7,18 | 2] |
| [20 | 2 | 0,1,12 | 2] |
| [21 | 2 | 6,7,18 | 2] |
| [22 | 2 | 2,3,14 | 2] |
| [23 | 2 | 8,9,20 | 2] |
| [24 | 3 | 0,1,12 | 2] |
| [25 | 3 | 6,7,18 | 2] |
| [26 | 3 | 2,3,14 | 2] |
| [27 | 3 | 8,9,20 | 2] |
| [28 | 3 | 4,5,16 | 2] |
| [29 | 3 | 10,11,22 | 2] |
| [30 | 3 | 7,12,13 | 2] |
| [31 | 3 | 9,14,15 | 2] |
| [32 | 3 | 11,16,17 | 2] |
| [33 | 3 | 9,18,19 | 2] |
| [34 | 3 | 18,19,20 | 2] |
| [35 | 3 | 21,22,23 | 2] |
| [36 | 3 | 13,15,17 | 1] |
| 37-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23-65: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 3 | 0-3 | 1 |
| 2 | 3 | 0,1,6,7 | 2 |
| 3 | 3 | 2,3,8,9 | 2 |
| 4 | 3 | 4,5,10,11 | 2 |
| [5 | 2 | 12-15 | 1] |
| [6 | 3 | 12-15 | 1] |
| 7 | 3 | 12,13,18,19 | 2 |
| 8 | 3 | 14,15,20,21 | 2 |
| 9 | 3 | 16,17,22,23 | 2 |
| 10 | 1 | 0,1,12,13 | 1 |
| 11 | 2 | 0,1,12,13 | 1 |
| 12 | 2 | 2,3,14,15 | 1 |
| 13 | 3 | 0,1,12,13 | 1 |
| 14 | 3 | 2,3,14,15 | 1 |
| 15 | 3 | 4,5,16,17 | 1 |
| [16 | 1 | 0,1,12,13 | 2] |
| [17 | 1 | 6,7,18,19 | 2] |
| [18 | 2 | 0,1,12,13 | 2] |
| [19 | 2 | 6,7,18,19 | 2] |
| [20 | 2 | 2,3,14,15 | 2] |
| [21 | 2 | 8,9,20,21 | 2] |
| [22 | 3 | 0,1,12,13 | 2] |
| [23 | 3 | 6,7,18,19 | 2] |
| [24 | 3 | 2,3,14,15 | 2] |
| [25 | 3 | 8,9,20,21 | 2] |
| [26 | 3 | 4,5,16,17 | 2] |
| [27 | 3 | 10,11,22,23 | 2] |
| 28-63 | Reserved | Reserved | Reserved |

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, for case 2) in RAN1#113 agreement,

  - Remove row 69-72.

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for case 4) and 5) for 2CW in RAN1#113 agreement,

  - Remove row 2-3.

Agreement

Confirm the following Working Assumption in RAN1#113:

  - For > 4 layers PUSCH with Rel.15 Type1/Type2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports, for partial coherent UL codebook, support Alt.2:

    - Alt.2: DMRS ports combination(s) for full/non-coherent UL codebook is reused.

  - The same DMRS port tables are used for full/partial/non-coherent UL codebook.

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, for 2 CW case in RAN1#113 agreement,

  - Support row 8-11.

  - Support row 12-15.

  - Remove row 16-19.

  - Remove row 24-35.

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 1 CW case in RAN1#113 agreement,

  - Support row 9-10 and row 20-23 with MU restriction (i.e. UE does not expect to be co-scheduled with another UE in the same CDM group).

  - Remove row 33-34 and row 44-46

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 2CW in RAN1#113 agreement,

  - Support row 8-11.

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, for 1CW case in RAN1#113 agreement,

  - Support row 9-10 and row 20-23 with MU restriction (i.e. UE does not expect to be co-scheduled with another UE in the same CDM group).

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, for 1CW case in RAN1#113 agreement,

  - Remove 129-132

  - Support row 133-152 without MU restriction

  - Support row 42-47 with MU restriction (i.e. UE does not expect to be co-scheduled with another UE in the same CDM group).

  - Remove row 100-105.

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, for 1CW case in RAN1#113 agreement,

  - Remove 67-68, 78-80.

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, for 2 CW case in RAN1#113 agreement,

  - Support row 0-5.

  - Support row 10-13.

  - Support row 14-17.

  - Support row 18-21.

  - Remove row 22-25.

  - Support row 26-29.

  - Remove row 30-37.

  - Remove row 46-69.

Agreement

For the antenna ports tables for Rel.18 eType1 DMRS ports with maxLength = 2 for PUSCH in RAN1#114 agreement, at least support the following rows:

  - Row 28-31 for rank 1 with the following modification.

Table 7.3.1.1.2-46: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=2, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| [28 | 1 | 0 8 | 2] |
| [29 | 1 | 1 9 | 2] |
| [30 | 1 | 8 12 | 2] |
| [31 | 1 | 9 13 | 2] |

Agreement

For the antenna ports tables for Rel.18 eType1 DMRS ports with maxLength = 2 for PUSCH in RAN1#114 agreement, at least support the following rows:

  - Row 20 for rank 2.

  - Row 15-17 for rank 3.

  - Row 30-33 for rank 2.

Agreement

For the antenna ports tables for Rel.18 eType2 DMRS ports with maxLength = 1 for PUSCH in RAN1#114 agreement, at least support the following rows:

  - Row 14-15 for rank 2.

  - Row 12 for rank 3.

Agreement

For the antenna ports tables for Rel.18 eType2 DMRS ports with maxLength = 2 for PUSCH in RAN1#114 agreement, at least support the following rows:

  - Row 38-39 for rank 2.

  - Row 30-36 for rank 3.

Agreement

When finer time-delay-domain granularity for SRS cyclic shift hopping is configured, K is 2

  - FFS (to be decided this week) Support of K=4

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s = 2, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot the port, down select from the following options:

  - Option 1: The first subset includes ports {1000, 1001, 1004, 1005}, and the second subset includes {1002, 1003, 1006, 1007}.

Agreement

For the SRS hopping formula in cyclic shift hopping or comb offset hopping except for SRS configured with TDM, let N=128 and M=7:

- For cyclic shift hopping: αi=2πnSRScs,inSRScs,max+2πfCS, hopKnSRScs,max, where fCS, hop=S{(m=0Mc8t+m∙2m) mod Y}, and

  - t=modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l',

  - If cyclicShiftHoppingSubset is not configured, S{n}=n.

    - If cyclicShiftHoppingFinerGranularity is not configured, Y=nSRScs,max and K=1.

    - If cyclicShiftHoppingFinerGranularity is configured, Y=K.nSRScs,max, and K=2

  - If cyclicShiftHoppingSubset is configured, S{n} denotes the (n+1)th element of the configured subset, Y is the number of elements in the subset, and K=1.

- For comb offset hopping: k0(pi)=nshiftNscRB+kTC(pi)+fTC, hop+koffsetl' mod KTC, where fTC, hop=S{(m=0Mc8t+m∙2m) mod Y}, and

  - t=mod(SFN,N)Nslotframe,μNsymbslot+ns,fμNsymbslot+l'',

    - l''=l0+l' if R=1 or UE is provided with combOffsetHoppingWithRepetition=Per-symbol; otherwise, l'' is the OFDM symbol index of the first symbol across the R repetitions within the slot.

  - If combOffsetHoppingSubset is not configured, S{n}=n, and Y=KTC.

  - If combOffsetHoppingSubset is configured, S{n} denotes the (n+1)th element of the configured subset, and Y is the number of elements in the subset.

Conclusion

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s and repetition factor R, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when cyclic shift hopping is configured for the SRS resource,

- Option A4: Do not support cyclic shift hopping for 8-port SRS with TDM.

Conclusion

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s and repetition factor R, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when comb offset hopping is configured for the SRS resource,

  - Option B5: Do not support comb offset hopping for 8-port SRS with TDM.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when sequence/group hopping is configured for the SRS resource, the time-domain behavior of hopping depends only on the OFDM symbol index l’ of each symbol.

Conclusion

When finer time-delay-domain granularity for SRS cyclic shift hopping is configured, K = 4 is not supported.

Agreement

When a subset of comb offsets for comb offset hopping is configured, and when a subset of cyclic shifts for cyclic shift hopping is configured, support the following option for configuring the subset S={S(0), S(1), …, S(z-1)} with 1<z<Z, where Z=KTC for comb offset hopping and Z=nSRScs,max for cyclic shift hopping, and:

  - Option 1b: S(0), S(1), …, S(z-1) are configured via a Z-length bitmap with S(i-1) being the i-th bit set as 1.

Enhanced uplink transmission

Agreement

When the single-DCI based PUSCH SDM/SFN is configured, the codepoint ‘11’ of the DCI field SRS resource set indicator is reserved.

Agreement

Regarding how to configure multi-DCI based STxMP PUSCH+PUSCH in RRC,

- Introduce a new RRC parameter to indicate the multi-DCI based STxMP PUSCH+PUSCH. The multi-DCI based STxMP PUSCH+PUSCH is configured when the new RRC parameter is configured, two different coresetPoolIndex values are configured and two SRS resource sets for CB/NCB are configured.

When multi-DCI based STxMP PUSCH+PUSCH is configured, the DCI field SRS resource set indicator is not present.

Agreement

- For single-DCI based STxMP PUSCH SFN transmission, reuse Table 7.3.1.1.2-25 and Table 7.3.1.1.2-26 of 38.212 to indicate the association between PTRS port(s) and DMRS port(s) when one PTRS port and two PTRS ports are configured for the SFN scheme, respectively.

- For single-DCI based STxMP PUSCH SDM scheme, when maxNrofPortsforSdm = 1, the 2-bit “PTRS-DMRS association” DCI field indicates the association between PTRS-DMRS port and the DMRS port according to the existing Table 7.3.1.1.2-25 in 38.212.

Agreement

For the single-DCI based STxMP PUSCH SFN scheme, the maximal number of layers is up to 2

Agreement

Introduce one RRC parameter in PUCCH-config to configure STxMP SFN scheme. When this RRC parameter is configured:

- When two indicated TCI states are applied to one PUCCH resource, the STxMP SFN scheme is enabled to this PUCCH resource.

- When one TCI state is applied to one PUCCH resource, the sTRP transmission is enabled to this PUCCH resource.

- It is subject to UE capability to support enabling Rel-18 STxMP SFN scheme and the Rel-17 repetition number parameter pucch-RepetitionNrofSlots in a same PUCCH resource at the same time.

When this RRC parameter is not configured:

- When two indicated TCI states are applied to one PUCCH resource, the Rel-17 TDM scheme with unified TCI states is enabled to this PUCCH resource.

- When one TCI state is applied to one PUCCH resource, the sTRP transmission is enabled to this PUCCH resource.

Agreement

- When the single-DCI based STxMP PUSCH SDM/SFN scheme is configured, the DCI field Second PTRS-DMRS association is not present in the DCI.

- If the single-DCI based STxMP PUSCH SDM scheme is configured, the bit-width of PTRS-DMRS association field is determined as follows:

  - when maxRank = 1, maxRankSdm =1 and maxNrofPortsforSdm = 2, the bit-width is 0 bit;

  - otherwise, 2 bits.

- If the single-DCI based STxMP PUSCH SFN scheme is configured

  - when maxRank = 1 and maxRankSfn = 1, the bit-width of PTRS-DMRS association field is 0 bit.

  - Otherwise, 2 bits

Agreement

Support single-DCI based SDM and SFN scheme in CG-PUSCH within one CG configuration

- For Type-1 CG-PUSCH, configure two SRI fields and two TPMI fields in CG configuration.

  - For Type-1 CG-PUSCH single-DCI based SFN, the UE may assume the association between UL PT-RS port(s) and DM-RS port(s) defined by value 0 in Table 7.3.1.1.2-25 or value "00" in Table 7.3.1.1.1.2-26 described in Clause 7.3.1 of [5, TS38.212].

    - Note: it is the same behavior as Type1 CG-PUSCH for sTRP transmission.

  - For Type-1 CG-PUSCH single-DCI based SDM, the UE may assume the association between UL PT-RS port(s) and DM-RS port(s) defined by value 0 in Table 7.3.1.1.2-25 or value "00" in Table 7.3.1.1.1.2-25a described in Clause 7.3.1 of [5, TS38.212].

- For Type-2 CG-PUSCH, the SRS resource set indicator/SRI fields/TPMI fields in the activation DCI of the SDM/SFN are applied to the activated CG PUSCH.

Agreement

When multi-DCI based STxMP PUSCH+PUSCH is configured,

- the existing rules for resolving overlapping PUSCH for the cases of one PUSCH overlapping with another PUSCH in time in one serving cell specified in legacy specifications at least for CG+DG overlap, CG+CG overlap, CG+PUSCH with SP-CSI overlap, or PUSCH with SP-CSI + PUSCH with SP-CSI overlap are performed separately for each coresetPoolIndex value.

Agreement

The maximum number of PTRS port in a PUSCH of multi-DCI based STxMP PUSCH+PUSCH is restricted to 1.

Agreement

For SDM scheme, maximum of 2 PTRS ports can be configured if UE has reported the capability of supporting full-coherent UL transmission.

  - Where there are at most 1 PTRS port per SRS resource set

Agreement

For partially coherent uplink precoding by an 8TX UE, Ng=2,

- Alt1. Following combinations of layer splitting are supported

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups |
| --- | --- | --- |
| 5 |  | (2,3) |
| 7 |  | (3,4) |

Agreement

For partially coherent precoding by an 8TX UE with Ng antenna groups, down-select from,

- Alt-1: All Rel-15 full-coherent 4TX and 2 TX UL precoders are supported for Ng=2 and Ng=4, respectively

  - For Ng=2 and Ng=4, up to N bits are used for joint TPMI/TRI indication, respectively

    - Number of bits (value of N) depend on the configured max rank

  - A single TPMI/TRI field is indicated

  - FFS: Mapping between TPMI/TRI codepoint and Rel-15 precoders

Agreement

For an 8TX UE, reuse DL principle that DC format 0_2 does not support 2 CW transmission.

Conclusion

For an 8TX UE, there is no consensus that Rel-17 mTRP PUSCH repetition transmission is supported in Rel-18.

Conclusion

For an 8TX UE, there is no consensus that CSI only transmission (UL-SCH=”0”) for rank>4 is supported in Rel-18.

Agreement

For partially coherent precoding by an 8TX UE with Ng=2 and Ng=4 antenna groups, up to N=10 bits are used for joint TPMI/TRI and split-layer indication.

- The actual number of bits (value of N) depends on the configured max rank

- FFS: Mapping between TPMI/TRI codepoint and Rel-15 precoders

Agreement

For an 8TX UE, with Ng=2, configured for full power transmission with ‘fullpowerMode1’, at least following precoder is supported

Rank = 1

12211111111

- FFS other additions

Agreement

For an 8TX UE, with Ng=4, configured for full power transmission with ‘fullpowerMode1’, at least following precoders are supported per rank,

| Rank 1 | Rank 2 | Rank 4 |
| --- | --- | --- |
| 12211111111 | 1221010010110100101 | None |

- FFS other additions

Agreement

For an 8TX UE, with Ng=8, configured for full power transmission with ‘fullpowerMode1’, at least following precoders are supported,

| Rank 1 | Rank 2 | Rank 3 | Rank 4 |
| --- | --- | --- | --- |
| 12211111111 | 1221010010110100101 | FFS | FFS |

- FFS other additions

- Precoders for rank>4 are not introduced in Rel-18

Agreement

For an 8TX UE, configured for full power transmission with ‘fullpowerMode2’ for Ng=2

- UE power capability is indicated per antenna group, where for an indicated group, full power is supported for all ranks

  - For when Ng=2, a single bit is used to indicate which of the antenna group has full power capability.

Agreement

For an 8TX UE, configured for full power transmission with ‘fullpowerMode2’,

- Subject to UE capability, a maximum of 2 or 4 SRS resources are supported in an SRS resource set with usage set to 'codebook',

- An SRS resource set can be configured with one or more of 1-, 2-, 4-, or 8-port SRS resources.

Conclusion

- Full power transmission with ‘fullpowerMode2’ with TPMI capability reporting when Ng=4 is not supported in Rel-18

- Full power transmission with ‘fullpowerMode2’ with TPMI capability reporting when Ng=8 is not supported in Rel-18

Agreement

For an 8TX UE, with Ng=4 and Ng=8, configured for full power transmission with ‘fullpowerMode1’, at least following precoders are supported per rank,

122100100010001100100010001

#### 2.1.2	Remaining Open issues

Multi-TRP enhancement

- Maintenance issues on extension of Rel-17 unified TCI framework

- Maintenance issues on power control for UL single DCI for multi-TRP operation where unified TCI framework extension is assumed

- Maintenance issues on Two TAs for UL multi-DCI for multi-TRP operation

CSI enhancement

- Maintenance issues on CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1

- Maintenance issues on Rel-16/17 Type-II codebook refinement for CJT mTRP and its associated CSI reporting, targeting FDD FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs

Reference signal enhancement

- Maintenance issues on DMRS enhancement to increase the number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM.

- Maintenance issues on DMRS enhancements for support of more than 4 layers SU-MIMO PUSCH.

- Maintenance issues on SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization

- Maintenance issues on SRS enhancements to enable 8 Tx UL operation

Enhanced uplink transmission

- Maintenance issues on UL precoding indication for PUSCH to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable):

- Maintenance issues on UL transmission for 8TX UE

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#123 (Toulouse, France, August 2023)

Two TAs for multi-DCI multi-TRP

Each joint/UL TCI state is associated with either TAG1 or TAG2 by RRC configuration.

RAN2 do not assume any restriction on grouping serving cells/TRPs to TAGs unless RAN1 indication comes.

RAN2 assumes the current 4 TAGs per cell group is sufficient to support Rel-18 mDCI mTRP with 2 TAs.

At least when both TATs for a SpCell are expired, 1-8 are applied to all TRPs of all serving cells.

At least when both TATs for a SCell are expired (assuming PTAG(s) of the cell group still running), 1-7 are applied to all TRPs associated to the TAG with the expired TAT (including both TRPs of the concerned SCell).

At least when both TATs for a SpCell are expired, 1-8 are applied to all TRPs of all serving cells.

At least when both TATs for a SCell are expired (assuming PTAG(s) of the cell group still running), 1-7 are applied to all TRPs associated to the TAG with the expired TAT (including both TRPs of the concerned SCell).

Working assumption:

We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs;

    - When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired,

    - when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired.

1.	not perform any uplink transmission except the Random Access Preamble and MSGA transmission;

2.	flush all HARQ buffers;

3.	notify RRC to release PUCCH, if configured;

4.	notify RRC to release SRS, if configured;

5.	clear any configured downlink assignments and configured uplink grants;

6.	clear any PUSCH resource for semi-persistent CSI reporting;

7.	maintain NTA (defined in TS 38.211 [8]) of this TAG;

8.	consider all running timeAlignmentTimers as expired.

For inter-cell PDCCH order CFRA to the additionalPCI,

- PDCCH order indicates which additionalPCI’s PRACH configuration to be used (according to RAN1 agreement),

The following is taken as baseline (for intra-cell case): for CBRA, we reuse the mechanism agreed for CFRA case, i.e. use the RA RAR to indicate the TAG.

Unified TCI extension to mTRP operation

The following information can be indicated by the MAC CE (for separate DL/UL TCI mode):

    - if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs,

    - if the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s)

    - if the unified TCI codepoint is for all, or sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}

#### 2.2.2	Remaining Open issues

- FFS whether 2-6 are applied to the TRP with TAT expired for two TAs in multi-DCI for multi-TRP operation

- MAC CE format review for single-DCI based multi-TRP operation

- RRC parameters review for RAN1/RAN4 features

## 2.3	RAN3

n/a

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#108 August 21 – August 25 2023, Toulouse, France

The following agreements and conclusions were made in RAN4 #108:

- RF related:

LS on configured Tx power for STxMP in FR2 was approved in R4-2314911.

WF on NR MIMO evo DL UL UERF was approved in R4-2314698.

<PCMAXf,c,k>

-	LS is sent to RAN1 to inform that RAN4 will introduce PCMAXf,c,k for STxMP (See R4-2314698)

-	How to incorporate the PCMAXf,c,k in to the spec will be discussed in RAN4#108-bis

<MPRf,c,k/A-MPRf,c,k>

-	MPRf,c,k/A-MPRf,c,k will be further discussed and determined in RAN4#108-bis from the following options

- Option 1: MAX[(MPRk , A-MPRk, MPRp, A-MPRp) ] +3dB in lower bound for beam k and p

- Option 2: MAX(X, MPRf,c,k, A- MPRf,c,k), X = 10*log10(number of UL TCI-states indicated for [STxMP]) dB in lower bound

- Option 3: Define ‘per-panel’ requirements of MPRf,c,k = MPRf,c + 3dB, and A-MPRf,c,k = A-MPRf,c + 3dB

- Option 4: Reuse MPRf,c and A-MPRf,c requirements, and add 3dB relaxation to lower bound

- Option 5: Do not extend the current MPR concept at least in this release.

- Option 6: Other proposals based on legacy MPR/A-MPR requirements are not precluded for RAN4#108-bis

<P-MPRf,c,k>

-    In RAN4#108-bis, it will be discussed how to ensure EIRP compliance

<PUMAXf,c,k>

-	Whether to introduce PUMAXf,c,k will be determined based on the discussion results of MPRf,c,k/A-MPRf,c,k

<New signalling>

-	Whether to introduce new signalling for overlapped beams indication depends on the discussion results of MPRf,c,k/A-MPRf,c,k in RAN4#108-bis

<Testability>

-	RAN4 will check the testability issue before PUMAXf,c,k is introduced, e.g., sending LS to RAN5 and/or other means

- RRM related:

WF on R18 NR MIMO RRM requirements was approved in R4-2314347.

Reply LS on TDCP Measurements was approved in R4-2314348.

Issue 1-1-1: Whether to define TDCP measurement delay requirements?

Agreement:

  - Do not define TDCP measurement delay requirements

Issue 1-1-2: Whether to define TDCP measurement accuracy requirements?

Agreement:

  - Further identify feasibility of methodology and test setup to define TDCP accuracy requirements including at least ideal TDCP definition, channel models

  - Define TDCP accuracy requirements subject to conclusions of feasibility analysis

Issue 1-1-5: Besides RAN1’s definition, for TDCP calculation, whether to define additional information for averaging across RX ports?

Agreement:

  - For TDCP measurement averaging across RX ports should be left to UE implementation.

    - The reported TDCP amplitude is expected to between the minimum and maximum measured values across the Rx branches if receiver diversity is used.

Issue 2-1-1: If UE doesn’t support STxMP and not capable of supporting RTD>CP, whether to define MTTD requirement?

Agreement:

  - No need to specify MTTD requirement.

Issue 2-1-3: How to handle overlapping UL transmissions?

Agreement:

  - RAN1 has agreements to apply scheduling restriction.

  - No RAN4 spec impact.

Issue 3-1-1: For eUTCI, whether to support simultaneous reception in mTRP scenarios in FR2?

Agreement:

  - Deprioritize requirements for eUTCI with simultaneous reception in DL in FR2

    - Further check in RAN4 #108bis on the workload and expected scope for the work and make a final decision on the respective requirements

Issue 3-1-4: For sDCI or mDCI mTRP, whether to define separate TCI state switching requirements?

Agreement:

  - Define separate TCI state switching requirements.

Issue 3-1-5: For mDCI mTRP, how to specify RRM requirements for eUTCI if UE cannot support simultaneous DL reception in FR2?

Agreement:

  - RRM requirements for eUTCI

    - For UEs not supporting two TAs, reuse Rel-17 unified TCI state switching requirements [with association of coresetPoolIndex].

    - For UEs supporting two TAs and not capable to support RTD > CP reuse Rel-17 unified TCI state switching requirements [with association of coresetPoolIndex]

    - For UEs supporting two TAs and capable to support RTD > CP the requirements are FFS

    - The TCI state switching requirements cover both known and unknown target TCI state cases

Issue 3-1-6: For mDCI mTRP, whether to specify RRM requirements for RRC based switching delay requirements?

Agreement:

  - Do not specify RRC based TCI state switch delay requirements.

Issue 3-1-7: For sDCI mTRP if only one single TCI state is switched, how to specify TCI state switch delay requirement for eUTCI if UE cannot support simultaneous DL reception in FR2?

Agreement:

  - Reuse Rel-17 unified TCI state switching requirements in this case, including MAC-CE based TCI and DCI based TCI state switching.

Issue 3-1-9: For sDCI mTRP if dual TCI state is switched, how to specify DCI based TCI state switch delay for eUTCI if UE cannot support simultaneous DL reception in FR2?

Agreement:

  - Both for DL and UL, the legacy DCI based TCI state switch delay requirements can be reused.

#### 2.4.2	Remaining Open issues

- RF related

- CR to introduce ‘per-panel’ configured transmitted power (PCMAXf,c,k) for STxMP

- Whether to consider MPRf,c,k/A-MPRf,c,k/P-MPRf,c,k in this release

- Whether to consider testability issue in this release

- Whether to introduce PUMAXf,c,k will be determined based on the discussion results of MPRf,c,k/A-MPRf,c,k

- Whether to introduce new signalling for overlapped beams indication depends on the discussion results of MPRf,c,k/A-MPRf,c,k in RAN4#108-bis

- RRM related

- Further study on feasibility of TDCP measurement accuracy requirements.

- Whether to reuse legacy SRS switching RRM requirements for 8TX UL?

- Whether to track DL RS to determine DL reference timing.

- Whether to introduce TAG management for multi-TRP with 2 TAs

- Whether to introduce RRM requirements for eUTCI if UE can support sTxMP?

- Whether to further enhance mDCI TCI state switching requirements for eUTCI for two TAs, if UE cannot support simultaneous DL reception?

- How to specify sDCI MAC-CE based TCI state switching requirements for eUTCI, if UE cannot support simultaneous DL reception?

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template