3GPP TSG RAN Meeting #101									           RP-231730

Bangalore, India, September 11-15, 2023

## Status Report to TSG

Agenda item:			9.3.4.5

| WI / SI Name | Enhanced NR support for high speed train scenario in frequency range 2 (FR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_HST_FR2_enh |  |  |  |  |
| Unique ID | 950079 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222272 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2023 | Performance part: 03/2024 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 100% | Performance Part: 50% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | WANG, He (Jackson) |
|  | Company | Samsung |
|  | Email | h0809.wang@samsung.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.4	RAN4

#### 2.4.1	Agreements

###### RAN4 #108 (Toulouse, France, 21–25 August, 2023)

1. UE RF requirement

Multi-panel reception:

- CR (R4-2314757) is agreed for RF requirements of FR2 HST multi-panel operation.

- 2AoA spherical coverage: No performance degradation shall be expected compared to 1AoA requirement in Rel-17.

Intra-band CA:

- CR (R4-2313024) is agreed for power class 6 intra-band CA requirements.

2. RRM requirement

Big CR (R4-2313541) to TS 38.133 for the enhanced NR support for FR2 HST is email-approved with the following endorsed dCRs contained:

- R4-2313798, DraftCR on RLM requirements for R18 FR2 HST, Qualcomm

- R4-2314285, DraftCR on HST Requirements for SSB based BFD, Xiaomi

- R4-2314286, DraftCR on MRTD requirements for HST FR2 multi-panel Rx UE, Intel

- R4-2314459, DraftCR on L1-RSRP requirements for R18 FR2 HST, Huawei, HiSilicon

- R4-2314289, DraftCR on HST FR2 Enhanced CA Inter-Frequency Cell Detection, Nokia

- R4-2314290, DraftCR on intra-frequency measurement requirements for R18 FR2 HST CA, Huawei

- R4-2314291, DraftCR on inter-frequency measurement requirements in Idle mode for FR2 HST, Samsung

- R4-2314296, DraftCR on SCell activation delay for HST FR2, Ericsson

- R4-2314292, DraftCR on HST FR2 Enhanced TCI State Switch, Nokia

- R4-2314293, DraftCR on UE timing requirements for R18 FR2 HST, Huawei

CR (R4-2314480) to TR 38.854 is agreed for HST FR2 enhanced tunnel deployment.

Reply LS (R4-2314299) to RAN2 on MAC-CE Based Indication for Cross-RRH TCI State Switch is approved.

WFs on FR2 HST RRM requirements are approved in R4-2314297 (part 1) and R4-2314298 (part 2), with details provided as follows:

Way forwards on simultaneous multi-panel operation for train roof-mounted FR2 high power devices are provided as:

Sub-topic 1-1: Possible scenarios analysis in Rel-18 FR2 HST

Agreement

- Define the RRM requirements for Rel-18 FR2 HST Multi-Rx based on the following scenarios described in R4-2314160 clause 1.2.1:

  - Scenario A & D: The FR2 PC6 UE uses two active panels pointed forward and backward to receive DL signal transmitted from two RRHs with different QCL TypeD RSs at the same time.

Sub-topic 1-2: Whether need to define a new or reuse the existing (simultaneousReceptionDiffTypeD-r16) capability for multi-panel simultaneous reception

Agreement

- Define a new UE capability [simultaneousReceptionFR2HST-r18] to indicate support of simultaneous multi-panel reception for Rel-18 FR2 PC6 UE

  - Details can be discussed in Rel-18 feature list discussion.

    - The conclusion from Rel-18 Multi-RX WI could be considered.

Sub-topic 1-3: Whether to define the new scheduling restriction requirements for data+L1 measurement

Way Forward:

- In the maintenance of WI, RAN4 will discuss the feasibility and try to define enhanced scheduling/measurement restriction requirements for data+L1 measurement for Rel-18 FR2 HST multi-panel simultaneous reception

  - FFS, whether/how to capture in the TS that no scheduling/measurement restrictions apply for the case of simultaneous data reception on one panel and SSB reception on the other panel in case of data and SSB transmission from different RRHs

Sub-topic 1-4: Whether to/How to define the new measurement restriction requirements for simultaneous L1+L1 measurement

Agreement:

- There is no need to consider measurement restriction enhancement on SSB configuration for neighboring RRHs for Rel-18 FR2 HST multi-panel simultaneous reception

Sub-topic 1-5: Whether to/How to reduce the sweeping factor N1

Agreement:

- Introduce enhanced scaling factors in L1 measurement requirements for PC6 UE supporting Rel-18 multi-panel reception and when highSpeedDeploymentTypeFR2-r17 is configured as bidirectional:

  - Baseline:

    - N1 = [1.5] when highSpeedMeasFlagFR2-r17 = set1;

      - One company have the concern on the impact of N1=[1.5], and may further study the feasibility in the maintenance of Rel-18.

    - N1 = [4] when highSpeedMeasFlagFR2-r17 = set2

- Use the following text proposal for the enhancement for RLM, BFD and L1-RSRP measurement:

  - Note: Scaling factor N=2 when highSpeedMeasFlagFR2-r17 is configured to set1 or scaling factor N=6 when highSpeedMeasFlagFR2-r17 is configured to set2, if UE is not supporting [simultaneousReceptionFR2HST-r18] or when highSpeedDeploymentTypeFR2-r17 is not configured as bidirectional; Scaling factor N=[TBD] when highSpeedMeasFlagFR2-r17 is configured to set1 or scaling factor N=[4] when highSpeedMeasFlagFR2-r17 is configured to set2, if UE is supporting [simultaneousReceptionFR2HST-r18] and when highSpeedDeploymentTypeFR2-r17 is configured as bidirectional.

Way forwards on intra-band carrier aggregation scenario are provided as:

Sub-topic 2-1: Inter-frequency measurement requirements

Issue 2-1-1: Whether need to support separate CA and inter-frequency enhancements for Rel-18 FR2 HST

Agreements

- Introduce per-UE capability to indicate whether the UE is capable of supporting the enhanced RRM requirements for Connected and Idle mode inter-frequency measurements for FR2 HST

- FFS whether to introduce a cell specific network signalling to inform UE whether to apply the enhanced RRM requirements for Connected and Idle inter-frequency measurement for FR2 HST.

- FFS: Do not introduce new network indication or UE capabilities to support CA enhancements for R18 FR2 HST

Agreement:

- A single per-UE capability is introduced for FR2 HST CA and inter-frequency measurement enhancement:

  - [measurementEnhancementCAInterFreqFR2-r18] indicates whether the FR2 PC6 UE supports the enhanced RRM requirements for carrier aggregation as specified in TS 38.133 and the UE supports the enhanced RRM requirements for inter-frequency measurements in connected and idle mode as specified in TS 38.133.

Agreement:

- Reuse Rel-17 signaling highSpeedMeasFlagFR2 to inform UE whether to apply the enhanced RRM requirements for inter-frequency measurement for FR2 HST in Connected mode.

- Reuse Rel-17 signaling highSpeedMeasFlagFR2 to inform UE whether to apply the enhanced RRM requirements for intra-frequency measurement on SCC for FR2 HST in Connected mode.

Issue 2-1-2: Whether need to introduce signalling for idle mode inter-frequency measurement requirements

Agreement:

- Reuse Rel-17 IE highSpeedMeasFlagFR2-r17 in SIB to inform UE whether to apply the enhanced RRM requirements for inter-frequency measurement for FR2 HST in idle mode.

Sub-topic 2-2: Intra-frequency measurement requirements on SCC

Agreement:

- For UE supporting power class 6 with [highSpeedMeasFlagFR2] configured, the TSSB_measurement_period_intra specified for UE supporting [measurementEnhancementCAInterFreqFR2-r18] shall apply for SCC.

  - Specification change is required, because new capability [measurementEnhancementCAInterFreqFR2-r18] is introduced.

Sub-topic 2-3: The RRM impact for SCell activation delay requirement for Rel-18 FR2 HST UE

Agreement:

- SCell activation delay with 3ms Tactivation_time can be applied without having to fulfil legacy conditions in HST FR2 scenario, and the below text proposal is used in CR:

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the FR2 power class 6 UE is configured with highSpeedMeasFlagFR2-r17, Tactivation_time is 3 ms, provided - the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band. Editor Notes: FFS additional condition/capability is needed.

Way forwards on UL TX timing adjustment are provided as:

Issue 1-1-1: MAC-CE based cross-RRH indication

Agreement

- If MAC-CE bit indicates “1” then for the TCI state switch, UE may expect that there is a large timing difference between DL signals from two non-collocated RRHs.

- If MAC-CE bit indicates “0” then for the TCI state switch, UE may expect that the timing difference between DL signals from two RRHs or from the same RRH is smaller than a quarter of CP.

Issue 1-1-2: One-shot large timing adjustment: DL propagation delay difference threshold

Agreement

- When MAC-CE indicates “1”

  - When highSpeedLargeOneStepUL-TimingFR2-r17 is enabled for HST PC 6 UE supporting [largeOneStepUL-timingFR2-r17] capability, UE shall apply the one shot large timing adjustment specified in Clause 7.1.2.3 in TS 38.133 on the first UL transmission after TCI state switch without checking the DL timing difference threshold.

- When MAC-CE indicates “0”

  - Gradual timing adjustment requirements in Clause 7.1.2.1 in TS 38.133 apply to the first UL transmission after TCI state switch, i.e., one shot large UL timing adjustment is not applied.

Issue 1-1-3: TCI state switching delay

Agreement

- When MAC-CE indicates “1”

  - Rel-17 TCI state switching delay requirements in HST FR2 scenarios from TS 38.133, Clause 8.10.3A are followed without changes.

- When MAC-CE indicates “0”,

  - UE follows the Rel-15 TCI state switching requirements as described in TS 38.133 Clause 8.10.3

Issue 1-1-4: UE feature definition for cross-RRH MAC CE based indication solution

Agreement

- RAN4 sees a need to define enhanced MAC CE indication in HST FR2 deployments and corresponding [highSpeedTCISwitchEnhMAC-CE-FR2-r18] UE capability.

- The detail of the capability can be discussed in Rel-18 feature list discussion.

Issue 1-1-5: Enhanced/Unified TCI state activation/deactivation

Agreement

- RAN4 LS only affects the MAC CEs intended for indicating target TCI state for PDCCH in 6.1.3.15 in TS 38.321.

Way forwards on tunnel deployment are provided as:

Issue 2-1: UL interruption in tunnel scenario

Way forward

- The following issue has been identified by a company that may need further study in the maintenance stage:

  - Option 1: RAN4 to study the UL interruption issue that may occur in multi-panel reception inside tunnel:

    - Option 1a: switching the UL to the other connected RRH before the UL failure happens

    - Option 1b: UL shall not be configured for the link-failure-prone RRH

    - Option 1c: Allow UE to adjust UL timing autonomously in the case of beam failure

  - Other options are not precluded.

Issue 2-2: Use of DRX in the tunnel deployment

Way forward

- The following issue has been identified by a company that may need further study in the maintenance stage:

  - In the HST FR2 Tunnel deployment, the reliable mobility performance for PC6 UEs at maximum speed of 350 km/h may not be guaranteed when DRX is enabled.

Issue 2-3: Indication of tunnel deployment

Agreement

- Not to introduce signalling indicating tunnel deployment

3. Demodulation requirement

WF (R4-2313919) on FR2 HST demodulation requirements is approved, with the following agreements and way forwards contained, and the simulation assumption (R4-1313920) for FR2 HST demodulation is approved.

Way forwards on deployment and channel modelling:

Sub-topic 1-1: Deployment and Channel Model for Demodulation requirement with simultaneous Rx reception in open space scenario

Issue 1-1-1:  Channel model for demodulation requirement for PDSCH with simultaneous multi-Rx reception

Agreement:

- Use the modified channel models for simultaneous multi-Rx reception scenario

| Panel 1 | fst=fdcosθ(t) cosθt=Ds_offset+Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθ(t)=cost modDsv, t>Dsv |
| --- | --- |
| Panel 2 | fst=fdcosθ(t) cosθt=-Ds-Ds_offset+vtDmin2+Ds-Ds_offset+vt2, 0<t≤2Ds_offsetv--Ds_offset+vtDmin2+-Ds_offset+vt2, 2Ds_offsetv<t≤Dsv cosθ(t)=cost modDsv, t>Dsv |

Issue 1-1-2:  Starting point for channel model for PDSCH requirements with Multi-Rx Reception

Agreement:

- The train location is denoted as y = y0 + vt. The starting train location y0 corresponding to t=0 is y0= (- DS_offset)

Issue 1-1-3:  Whether need to include relative power for channel model for PDSCH requirements with Multi-Rx Reception

Agreement:

- Further discuss whether and how to model power profile across Rx chains under demodulation test cases with fixed FRC

  - Test feasibility need to be taken into account when selecting suitable test set-up.

Issue 3-2-12: Number of MCS for PDSCH requirement for multi-Rx reception

Agreement:

- It is feasible to define PDSCH demodulation requirement per Rx panel with different MCS per Rx panel

  - Companies are encouraged to check the feasibility of MCS pair (MCS 17, MCS [13]) or (MCS 17, MCS [19]) in next meeting

  - Other feasible MCS pair are not precluded

Issue 1-1-4:  How to model relative power for channel model for PDSCH requirements with Multi-Rx Reception (if relative power is introduced)

Agreement:

- FFS

  - Option 1

    - Option 1a: Define power profiles based on the free-space pathloss

    - Option 1b: Define power profiles that considers UE and RRH beam gains in addition to pathloss

  - Option 2

    - Model the relative power for HST FR2 multi-Rx channel modelling as following:

      - Power level Pk (dB) for the signal from kth RRH, normalized to the total power received from all visible RRHs, is given by:

        - Pk=-20lgy-xk-10lgi∈i|i*Ds±Ds-Ds_offset≤a<i*Ds±Ds+Ds_offset1y-xi2 for k*Ds±Ds-Ds_offset≤a<k*Ds±Ds+Ds_offset

        - Pk=-20lgy-xk-10lgi∈i|i*Ds+Ds_offset≤a+0, 1*Ds<i*Ds+Ds-Ds_offset1y-xi2 for k*Ds+Ds_offset≤a+0, 1*Ds<k*Ds+Ds-Ds_offset

Issue 1-1-5:  Requirements need to be introduced for scenario A and scenario B in Bi-directional deployment scenario for PDSCH demodulation requirement

Agreement:

- Define single requirement for scenario A and scenario B in Bi-directional deployment scenario for PDSCH demodulation requirements, considering Scenario B-1 for requirement

| Parameters | Description | Scenario B-1 |
| --- | --- | --- |
| Ds | Inter-RRH distance | 700m |
| Dmin | Distance between rail track and RRH | 150m |
| v | Train velocity | 350km/h |
| fd | Maximum Doppler frequency shift | 9722Hz |
| fc | Carrier frequency | 30GHz |
| Drx_panel | Distance between two UE Rx panels | 0m |
| Ds_offset | Switching transmission point between adjacent RRHs | 100m |

Sub-topic 1-2 Deployment and Channel Model in tunnel scenario

Issue 1-2-1: Channel Model in Tunnel Scenario for UE demodulation

Way forward:

- Option 1:

  - RAN4 not to consider further discussions on channel modelling for Tunnel scenario for FR2 HST DPS

- Option 2: Reuse the channel model in RAN4 spec 38.101-4, i.e, single path with LoS propagation, for performance requirement study of FR2 HST-DPS in tunnel deployment with updated Ds, Dmin and Ds_offset

  - Uni-directional scenario

    - Doppler shift fst=fd cosθt (Hz) for PDSCH received at UE is given by

cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθt=cosθt modDsv, t>Ds/v Parameter; Value. TBD. Ds; 700 m. Ds_offset; 5m. Dmin; 1 m. v; 350 km/h. fd; 9722 Hz.

- Interested companies can provide the simulation results for initial performance evaluation in tunnel scenario deployment in the next meeting

Issue 1-2-2: Channel Model for Demodulation requirements in Tunnel Scenario with Single-panel reception for BS demod

Way forward:

- Option 1:

  - Reuse the channel model in RAN4 spec 38.101-4, i.e, single path with LoS propagation, for performance requirement study of FR2 HST-DPS in tunnel deployment with updated Ds, Dmin and Ds_offset

cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθt=cosθt modDsv, t>Ds/v Parameter; Value. TBD. Ds; 700 m. Ds_offset; 5m. Dmin; 1 m. v; 350 km/h. fd; 19444 Hz.

- Interested companies can provide the simulation results for initial performance evaluation in tunnel scenario deployment in the next meeting

Issue 1-2-3: Channel Model for Demodulation requirements in Tunnel Scenario with Bi-directional deployment with simultaneous multi-Rx reception for UE demodulation

Way forward:

- Option 1:

  - RAN 4 to reuse open-space propagation condition for two-sided reception (including Doppler and power profiles) for the tunnel deployment with tunnel-specific parameters as follows

| Parameters | Description | Tunnel |
| --- | --- | --- |
| Ds | Inter-RRH distance | 700m |
| Dmin | Distance between rail track and RRH | 1m |
| v | Train velocity | 350km/h |
| fd | Maximum Doppler frequency shift | 9722Hz |
| fc | Carrier frequency | 30GHz |
| Drx_panel | Distance between two UE Rx panels | 0m |
| Ds_offset | Switching transmission point between adjacent RRHs | [10, 5]m |

- Interested companies can provide the simulation results for initial performance evaluation in tunnel scenario deployment in the next meeting

Issue 1-2-4: Channel Model for Demodulation requirements in Tunnel Scenario with Bi-directional deployment for BS demodulation

Way forward:

- Option 1:

  - RAN4 to consider uni-directional scenario for testing HST FR2 Tunnel deployment when UE is moving in the same direction with the serving beam and Ds_offset = 10m (Option 2)

- Option 2: Considering alternative scenario with train moving in the same direction with serving beam. - Uni-directional scenario - Doppler shift fst=fd cosθt (Hz) for PUSCH received at gNB is given by cosθt=-Ds_offset +Ds-vtDmin2+-Ds_offset+Ds-vt2, 0<t≤Dsv cosθt=cosθt modDsv, t>Ds/v Parameter; Value. TBD. 350 km/h. 19444 Hz. Ds_offset; 10m.

- Interested companies can provide the simulation results for initial performance evaluation in tunnel scenario deployment in the next meeting

Way forwards on UE demodulation for CA requirement:

Issue 2-1-3: Release independent for CA PDSCH demodulation requirements requirement

Agreement

- From demodulation point of view, it is feasible to define Rel-18 FR2 HST PDSCH CA requirements which are release independent from Rel-17 according to the UE capability with PC6 UE

- The RRM conclusion on release independence of Rel-18 FR2 HST CA requirements shall be taken into account

Way forwards on PDSCH requirement with multi-Rx reception

Sub-topic 3-1: General for PDSCH requirement with multi-Rx reception

Issue 3-1-1: UE processing assumption for the FFT window

Agreement:

- Encourage companies to evaluate the performance difference with assumption on FFT (single FFT across Rx chains, and separate FFT per RF chain)

Sub-topic 3-2: Test setup for PDSCH requirement with multi-Rx reception

Issue 3-2-1: Transmission schemes

Agreement:

- Do not consider single-DCI based multi-TRP scheduling for PDSCH requirement

Issue 3-2-3: PDSCH resource scheduling for requirements

Agreement:

- Only cover full overlapping

Issue 3-2-4: Layer combination for full overlapping

Agreement:

- 2+2

Issue 3-2-6: PDSCH rate matching in mTRP transmission

Agreement:

- PT-RS allocation does not overlap with PDSCH allocation per TRP

Issue 3-2-7: PDSCH scheduling and Number of DMRS in TDD DL special slot

Agreement:

- PDSCH scheduling in the special DL slot is assumed with 3 DMRS configuration

Issue 3-2-8: Number of SSB and TCI state configuration for each cell

Agreement:

- Maximum 8 SSB and TCI states configuration for each cell is configured

Issue 3-2-9: Number of active TCIs tracking

Agreement:

- RAN4 to assume the baseline behaviour for FR2 HST UE under test is to track 2 Active TCI states, one per panel

Issue 3-2-10: TRS periodicity

Agreement:

- RAN4 to choose 10ms TRS periodicity to align with existing FR2 HST requirements

Issue 3-2-11: PDSCH allocation timeline in the UE Demod Test

Agreement:

- PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered.

- The detail of scheduling pattern can be further discussed in the next meeting

  - Option 1: RAN4 to follow the same approach used in FR1 HST DPS requirements and FR2 HST DPS requirements (Bidirectional) and define a PDCCH/PDSCH allocation timeline independent per each Panel, including a number of slots in which PDCCH and PDSCH are DTX and throughput statistics are not considered, starting from the TCI state switch and until the UE has received one TRS from the new RRH (TfirstTRS) and has had time to process it (TTRSproc), including THARQ and TMAC proc;

  - Other options are not precluded.

Issue 3-2-13:  Power scaling for two served RRH

Agreement:

- Apply a constant scaling factor 1/2 to the transmitted PDSCH signal from each TRxP, as same as Rel-16 multi-TRP SDM transmission scheme

Issue 3-2-14: Applicability rule for PDSCH requirement with simultaneous multi-Rx reception

Agreement:

- Simultaneous multi-Rx reception requirements are appliable for UE with the following capabilities at least:

  - UE power class 6 (PC6)

  - [simultaneousReceptionDiffTypeD-r16]

  - maxNumberActiveTCI-PerBWP > 1

- Additional UE necessary capabilities should be considered as

  - Multi-DCI based multi-TRP transmission

  - Overlapping PDSCHs in time and fully overlapping in frequency and time

- Other UE capability could be considered if introduced in the RRM session

Issue 3-2-15: Simulation Assumption for initial simulation purpose

- Companies are encouraged to provide simulation results for PDSCH requirement with multi-Rx reception in the RAN4#108bis meeting based on simulation assumption in R4-2313920

Way forwards on demodulation requirements for tunnel scenario

Sub-topic 4-1: Test Scope of demodulation requirements of tunnel scenario

Issue 4-1-1: Whether to define UE demodulation requirements for tunnel deployment scenario in FR2 HST

Way forward:

- Option 1

  - UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

- Option 2

  - RAN4 does not define PDCSH demodulation requirements for Tunnel scenario.

- Option 3

  - RAN4 does not define PDCSH demodulation requirements for Tunnel scenario. If no meaningful difference in between open space and tunnel deployments is indicated, no new tunnel propagation conditions need to be introduced and conformance can be concluded based on open-space requirements, e.g., in HST FR2 Scenario A with two-panel reception.

- Option 4

  - If PDSCH requirements with DPS transmission scheme is introduced, the DPS 1a and 1b can be considered for Uni-directional and Bi-directional scenario, separately

Issue 4-1-2: Whether to define BS demodulation requirements for tunnel deployment scenario in FR2 HST

Way forward:

- Option 1

  - BS demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

- Option 2

  - If PUSCH requirement is introduced, single set requirement for PUSCH in tunnel scenario based on Bi-directional scenario in the tunnel scenario could be considered

- Option 3

  - RAN4 does not define PUCSH demodulation requirements for Tunnel scenario.

- Option 4

  - If found to be needed, introduce single set requirement for PUSCH in tunnel scenario based on uni-directional scenario with tunnel-specific parameters.

#### 2.4.2	Remaining Open issues

Core Part:

- No remaining work expected.

Performance Part:

- Specify the necessary RRM test cases based on the outcome on corresponding core part.

- Specify the demodulation performance requirements for intra-band carrier aggregation (CA) HST scenario.

- Specify the necessary demodulation performance requirements for simultaneous multi-panel reception.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#108 contributions:

- R4-2313015	CR for power class 6 intra-band CA requirements	Huawei, HiSilicon, Samsung, Nokia, Nokia Shanghai Bell

- R4-2313024	CR for power class 6 intra-band CA requirements	Huawei, HiSilicon, Samsung, Nokia, Nokia Shanghai Bell

- R4-2311268	RF requirement for simultaneous multi-panel operation for train roof-mounted FR2 high power devices	Nokia, Nokia Shanghai Bell

- R4-2311783	FR2 HST RF	Qualcomm, Inc.

- R4-2312511	Discussion on RF requirements for FR2 HST multi-panel operation	Samsung

- R4-2312512	Feature CR for RF requirements of FR2 HST multi-panel operation	Samsung

- R4-2313016	On Multi-panel RF requirements for NR FR2 HST enhancement	Huawei, HiSilicon

- R4-2313228	Feature CR for RF requirements of FR2 HST multi-panel operation	Samsung

- R4-2313541	Big CR to TS38.133 on enhanced NR support for FR2 HST	Samsung

- R4-2313798	DraftCR on RLM for FR2 HST Enh	Qualcomm Incorporated

- R4-2311850	CR on HST Requirements for SSB based BFD	Xiaomi

- R4-2312188	On Simultaneous Multi-Panel Reception in HST FR2 Enhanced	Nokia, Nokia Shanghai Bell

- R4-2312189	Updated Simulation Results for RRM Performance with Simultaneous L1 Measurements in HST FR2	Nokia, Nokia Shanghai Bell

- R4-2312341	DraftCR on MRTD requirements for HST FR2 multi-panel Rx UE-s	Intel Corporation

- R4-2312359	Simultaneous multi-panel operation for train roof-mounted FR2 high power devices	Ericsson

- R4-2312871	Discussion on multi-panel simultaneous reception in FR2 eHST	Huawei, HiSilicon

- R4-2312872	DraftCR on L1-RSRP requirements for R18 FR2 HST	Huawei, HiSilicon

- R4-2313542	Discussion on simultaneous multi-panel operation for train roof-mounted FR2 high power devices	Samsung

- R4-2313543	Draft CR on scheduling availability of UE during L1-RSRP measurement for FR2 HST	Samsung

- R4-2313707	RRM aspects for simultaneous multi-panel reception for FR2 HST	Qualcomm Incorporated

- R4-2312190	On RRM Impacts by CA in HST FR2 Enhanced	Nokia, Nokia Shanghai Bell

- R4-2312191	Draft CR to TS 38.133 on HST FR2 Enhanced CA Inter-Frequency Cell Detection	Nokia, Nokia Shanghai Bell

- R4-2312360	Carrier aggregation for train roof-mounted FR2 high power devices	Ericsson

- R4-2312428	Discussion on intra-band CA and inter-frequency measurement in FR2 eHST	Huawei, HiSilicon

- R4-2312429	CR on intra-frequency measurement requirements for R18 FR2 HST CA	Huawei, HiSilicon

- R4-2312637	CR on intra-frequency measurement requirements for R18 FR2 HST CA	Huawei, HiSilicon

- R4-2313544	Draft CR on Inter-frequency measurement requirements in Idle mode for FR2 HST	Samsung

- R4-2312192	On UL Timing Adjustment in HST FR2 Enhanced	Nokia, Nokia Shanghai Bell

- R4-2312193	Draft CR to 38.133 on HST FR2 Enhanced TCI State Switch	Nokia, Nokia Shanghai Bell

- R4-2312361	UL TX timing requirement for train roof-mounted FR2 high power devices	Ericsson

- R4-2312873	Discussion on UL timing adjustment for R18 FR2 HST	Huawei, HiSilicon

- R4-2312874	DraftCR on UE timing requirements for R18 FR2 HST	Huawei, HiSilicon

- R4-2313545	Discussion on UL timing adjustment solutions	Samsung

- R4-2312196	On RRM Aspects of Tunnel Deployment Scenarios in HST FR2 Enhanced	Nokia, Nokia Shanghai Bell

- R4-2312197	Draft CR to TR 38.854 on HST FR2 Enhanced Tunnel Deployment	Nokia, Nokia Shanghai Bell

- R4-2312362	RRM aspects for tunnel deployment scenario for train roof-mounted FR2 high power	Ericsson

- R4-2312198	Big CR to TR 38.854 on HST FR2 Enhanced	Nokia, Nokia Shanghai Bell

- R4-2312357	draft CR on Scheduling availability of UE performing radio link monitoring on FR2 for HST FR2	Ericsson

- R4-2312358	draft CR on SCell activation delay for HST FR2	Ericsson

- R4-2312363	Reply LS on MAC-CE Based Indication for Cross-RRH TCI State Switch	Ericsson

- R4-2313546	Discussion on MAC-CE based indication for Cross-RRH TCI state switch and Reply LS	Samsung

- R4-2313709	Discussion on Reply LS on MAC-CE Based Indication for Cross-RRH TCI State Switch	Qualcomm Incorporated

- R4-2312199	On Channel Modelling in HST FR2 Enhanced	Nokia, Nokia Shanghai Bell

- R4-2312210	View on channel modeling for Rel-18 FR2 HST demodulation requirement in tunnel scenario	Samsung

- R4-2312493	FR2 HST Enh. UE Demod: General and Channel Modeling	Qualcomm Incorporated

- R4-2313088	FR2 HST Enh. UE Demod Simulation Results	Qualcomm Incorporated

- R4-2313654	Discussion on deployment and channel modelling for HST FR2	Huawei, HiSilicon

- R4-2312207	Discussion and simulation results for PDSCH with CA	Samsung

- R4-2312211	Simulation results summary for Rel-18 FR2 HST demodulation requirement	Samsung

- R4-2312494	FR2 HST Enh. UE Demod for CA	Qualcomm Incorporated

- R4-2312792	Simulation results of CA PDSCH demodulation requirements for FR2 HST	Ericsson

- R4-2312795	[NR_HST_FR2_enh-Perf] HST FR2 Enhanced: UE Demodulation PDSCH Requirements with Carrier Aggregation	Nokia, Nokia Shanghai Bell

- R4-2312798	[NR_HST_FR2_enh-Perf] Simulation Results on HST FR2 Enhanced with Carrier Aggregation	Nokia, Nokia Shanghai Bell

- R4-2313655	Discussion on UE PDSCH CA demodulation requirements for HST FR2	Huawei, HiSilicon

- R4-2313656	Simulation results on UE PDSCH CA demodulation requirements for HST FR2	Huawei, HiSilicon

- R4-2312208	Discussion and simulation results for PDSCH requirements with multi-Rx reception	Samsung

- R4-2312495	FR2 HST Enh. UE Demod with simultaneous multi-panel reception	Qualcomm Incorporated

- R4-2312793	UE demodulation requirements for FR2 HST multi-Rx reception	Ericsson

- R4-2312796	[NR_HST_FR2_enh-Perf] HST FR2 Enhanced: UE Demodulation PDSCH Requirements with Multi-Rx Chain DL Reception	Nokia, Nokia Shanghai Bell

- R4-2312797	[NR_HST_FR2_enh-Perf] Simulation Results on HST FR2 Enhanced with Multi-Rx Chain DL Reception	Nokia, Nokia Shanghai Bell

- R4-2313657	Discussion on UE PDSCH requirements with multi-Rx demodulation requirements for HST FR2	Huawei, HiSilicon

- R4-2313658	Simulation results for PDSCH requirements with Multi-Rx for HST FR2	Huawei, HiSilicon

- R4-2312200	On Demodulation Aspects of Tunnel Deployment Scenarios in HST FR2 Enhanced	Nokia, Nokia Shanghai Bell

- R4-2312209	View on demodulation requirements for tunnel deployment scenario for Rel-18 FR2 HST	Samsung

- R4-2312496	FR2 HST Enh. UE Demod with Tunnel Deployment	Qualcomm Incorporated

- R4-2312794	Tunnel deployment scenario for FR2 HST enhancements	Ericsson

- R4-2313659	Discussion on reference tunnel deployment scenario	Huawei, HiSilicon

- R4-2314757	Feature CR for RF requirements of FR2 HST multi-panel operation	Samsung, Nokia, Nokia Shanghai Bell, Qualcomm, Huawei

- R4-2314285	DraftCR on HST Requirements for SSB based BFD	Xiaomi

- R4-2314286	DraftCR on MRTD requirements for HST FR2 multi-panel Rx UE	Intel

- R4-2314459	DraftCR on L1-RSRP requirements for R18 FR2 HST	Huawei, HiSilicon

- R4-2314289	DraftCR on HST FR2 Enhanced CA Inter-Frequency Cell Detection	 Nokia

- R4-2314290	DraftCR on intra-frequency measurement requirements for R18 FR2 HST CA	Huawei

- R4-2314291	DraftCR on inter-frequency measurement requirements in Idle mode for FR2 HST	Samsung

- R4-2314296	DraftCR on SCell activation delay for HST FR2	Ericsson

- R4-2314292	DraftCR on HST FR2 Enhanced TCI State Switch		Nokia

- R4-2314293	DraftCR on UE timing requirements for R18 FR2 HST	Huawei

- R4-2314480	CR to TR 38.854 on HST-FR2 Enhanced Tunnel Deployment	Nokia, Nokia-Shanghai-Bell, Samsung

- R4-2314299	Reply LS to RAN2 on MAC-CE Based Indication for Cross-RRH TCI State Switch	Nokia

- R4-2314297	WF on FR2 HST RRM requirements (part 1)	Samsung

- R4-2314298 	WF on FR2 HST RRM requirements (part 2)	Nokia

- R4-2313919  	WF on FR2 HST demodulation requirements	Samsung

- R4-1313920	Simulation assumption for FR2 HST demodulation	Ericsson

- R4-2314217	Topic summary for [108][135] NR_HST_FR2_enh_UERF		Moderator (Samsung)

- R4-2314160	Topic summary for [108][215] NR_HST_FR2_enh_part1	Moderator (Samsung)

- R4-2314161	Topic summary for [108][216] NR_HST_FR2_enh_part2	Moderator (Nokia)

- R4-2314260 	Topic summary for [108][324] NR_HST_FR2_enh_Demod	Moderator (Samsung)

- R4-2314342	Ad-hoc minutes for NR FR2 HST RRM requirements	Samsung

- R4-2313918	Ad-hoc minutes for FR2 HST demodulation	Samsung

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template