3GPP TSG-RAN Meeting #101	RP-231745

Bangalore, India, September 11-15, 2023

Agenda item:			9.3.1.9

Source:			Xiaomi

Title:		Consideration on sidelink positioning scope reduction

Document for:	 	Discussion and decision

- Introduction

At RAN2#123, RAN2 concluded that:

Agreement:

RAN2 see risk to completion of sidelink positioning with the current scope.

In this contribution, we provide our views on  how to reduce the scope of sidelink positioning to ensure to complete the WI on time.

Discussion

The following table provides a list of potential areas that can be discussed for scope reduction:

| Areas | Comment |
| --- | --- |
| - Multiple target UE | This is related to locate multiple target UE in one LCS request. |
| - Groupcast/Broadcast support | Issue 1: whether to support groupcast and broadcast Issue 2: whether to support session-based groupcast and broadcast |
| - Forwarding | This is related to a UE performing forwarding of SLPP message between LMF and another UE, or between two other UEs. For in-coverage, partical coverage, out of coverage, there are different views on whether forwarding functionality is needed or not. |
| - Partial coverage | This is related to forwarding. At least for partial coverage, companies have the same view that forwarding functionality is needed. |
| - LMF involvement | Whether LMF will involve in the SL positioning procedure. |
| - SL-PRS resource coordination | For TDOA, multiple anchor UEs are involved and need to send SL-PRS. It needs to be ensured that the anchor UEs doesn’t send SL-PRS colliding with each other. |
| - Positioning methods | RAN1 has supported RTT, AOA, TDOA and RTOA. Whether to support all the positioning methods can be discussed. |
| - Mobility | The UE who participates in the SL positioning may move, as a result, the selected anchor UE may becomes not suitable. Anchor UE reselection may be needed. |
| - Whether server UE is colocated with target UE or anchor UE | Still there options on the table: - Option 1: Server UE is always co-located with target UE. - Option 2: Server UE is either co-located with target UE or anchor UE. - Option 3: Sever UE can be a separate UE other than target UE and anchor UE. |
| - Meta data in Discovery message | RAN2 haven’t reach agreement on what information can be included in discovery message. |

Views on scope reduction:

- Multiple Target UE:

- Since SA2 has not finished their design on multiple target UEs, it would be not possible for RAN2 to design procedure and signalling accordingly. It is suggested to exclude multiple target UE case.

Proposal 1: Multiple target UE is not considered in Rel-18.

- Groupcast/Broadcast:

- The major motivation to support groupcast/broadcast for session-based operation is to support multiple target UE. If multiple target UE is down-scoped,  groupcast/broadcast for session-based operation can also be down scoped.

- For session-less based operation for broadcast, it is not related to multiple target UE, and SA3 has already finished their work on security design for groupcast/broadcast, it can still be kept.

Proposal 2: Groupcast/Broadcast with session-based operation is  not considered in Rel-18.

Proposal 3: Groupcast with sessionless operation is  not considered in Rel-18.

Proposal 4: Broadcast with sessionless operation is  still considered in Rel-18.

- Forwarding:

- SA2 has sent LS in  to SA3 to ask the security/privacy concern over transmitting location relate information between LMF and target UE via located UE. Since SA3 has never address this issue before, it would be difficult for SA3 to finish the solution on this issue. A similar issue would also exist when transmitting location relate information between two UEs via a third UE.

- Q5: For Network based SL positioning for UE without NAS connection defined in clause 5.5.3 of TS 23.586, are there any security or privacy issues identified for transmitting location related information of the UEs (including the location of the Located UE(s), the Ranging/SL Positioning measurement data/result and the location of the Target UE) between the LMF serving one of the Located UEs and the Target UE via Located UE? Are there any other security or privacy issues in the solution of Network based SL positioning for UE without NAS connection? If any, the security or privacy issues may need to be studied by SA3

- Thus, forwarding functionality can be down scoped.

Proposal 5: Forwarding functionality in SLPP is not considered in Rel-18.

- Partial coverage:

- The scenario of partial coverage is highly related to forwarding functionality. If forwarding is not to be supported, partial coverage should be down-scoped.

Proposal 6: Partial coverage is not considered in Rel-18.

- LMF involvement:

- LMF involvement would complicate the discussion, all the procedures would have to discuss for the case with LMF involvement and  without LMF involvement. For example, when LMF is involved, anchor UE selection and server UE selection procedure can be different from UE only mode. Also, the assistant information coordination procedure may also be different. To simplify the discussion, it is suggested to not consider LMF involvement case in Rel-18.

Proposal 7: LMF involvement is not considered in Rel-18.

- SL-PRS resource coordination:

- For UE autonomous resource allocation, it is possible that the resources selected by different anchor UEs are collided. But this happens rare, it may not need to be addressed in Rel-18.

- Also, the selected bandwidth, comb size, number of symbols for SL-PRS may also be different, but it has less impact on the SL positioning accuracy if all the UEs choose resource that can fulfil the positioning QoS requirement.

Proposal 8: SL-PRS resource coordination in SLPP is not considered in Rel-18.

- Positioning methods

- RAN1 has supported RTT, AOA, TDOA and RTOA. RAN2 work on support each of the positioning methods is not high. RAN2 can continue to support all the positioning methods based on RAN1 design.

Proposal 9: All the positioning methods (RTT/AOA/TDOA/RTOA) supported by RAN1 is still considered in Rel-18.

- Mobility:

- The UE who participates in the SL positioning may move, as a result, the selected anchor UE may becomes not suitable. Anchor UE reselection may be needed. However, it is very rare that the SL positioning procedure will last long and anchor UE reselection will be needed. For periodic location report, different SL positioning procedure can be triggered, within each procedure, anchor UE selection is performed. No anchor UE reselection is needed.

Proposal 10: Mobility related issues is not considered in Rel-18.

- Whether server UE is colocated with target UE or anchor UE:

- RAN2 has discussed this issues for many meeting, but still no conclusion can be made. To support server UE co-located with anchor UE or a separate UE, the server UE is assumed to have SL connection with all other participating UEs. Otherwise, SLPP message forwarding may be required.

- It is suggested RAN to down-select among the 3 options:

- Option 1: Server UE is always co-located with target UE.

- Option 2: Server UE is either co-located with target UE or anchor UE.

- Option 3: Sever UE can be a separate UE other than target UE and anchor UE.

Proposal 11: RAN to down-select among the 3 options:

Option 1: Server UE is always co-located with target UE.

Option 2: Server UE is either co-located with target UE or anchor UE.

Option 3: Sever UE can be a separate UE other than target UE and anchor UE.

- Meta data in discovery message:

RAN2 has discussed this for many meetings, and the only agreement is still to include the UE roles suggested by SA2, any other paramters are FFS:

Agreement:

FFS which (if any) additional parameters can be included (as optional or mandatory) in the metadata in the discovery message for anchor and server UE selection; it should be based on technical requirements for the fields and how they will be used.

It is suggested to downscope this.

Proposal 12: Any other parameters except UE role in the meta data field of discovery message is not considered in Rel-18.

How to handle the down-scoped areas in future:

Most of the down-scoped areas are due to time limit in rel-18. Thus, it should be possible to continue the work in Rel-19. Which areas can be included in Rel-19 can be discussed in Rel-19 scope discussion, and are RAN2 led.

Proposal 13: The down-scoped areas can still be considered in Rel-19 as RAN2-led objectives.

- Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1: Multiple target UE is not considered in Rel-18.

Proposal 2: Groupcast/Broadcast with session-based operation is  not considered in Rel-18.

Proposal 3: Groupcast with sessionless operation is  not considered in Rel-18.

Proposal 4: Broadcast with sessionless operation is  still considered in Rel-18.

Proposal 5: Forwarding functionality in SLPP is not considered in Rel-18.

Proposal 6: Partial coverage is not considered in Rel-18.

Proposal 7: LMF involvement is not considered in Rel-18.

Proposal 8: SL-PRS resource coordination in SLPP is not considered in Rel-18.

Proposal 9: All the positioning methods (RTT/AOA/TDOA/RTOA) supported by RAN1 is still considered in Rel-18.

Proposal 10: Mobility related issues is not considered in Rel-18.

Proposal 11: RAN to down-select among the 3 options:

Option 1: Server UE is always co-located with target UE.

Option 2: Server UE is either co-located with target UE or anchor UE.

Option 3: Sever UE can be a separate UE other than target UE and anchor UE.

Proposal 12: Any other parameters except UE role in the meta data field of discovery message is not considered in Rel-18.

Proposal 13: The down-scoped areas can still be considered in Rel-19 as RAN2-led objectives.

- Reference

[1] S2-2310025 Reply LS on Reply LS on security aspects for Ranging/Sidelink Positioning.