3GPP TSG-RAN Meeting 101	Tdoc RP-231746

Bangalore, India, September 11-15, 2023

Agenda Item:	8A.2.1

Source:	Ericsson

Title:	Discussion on Release 19 AI enhancements for PHY

Document for:	Discussion

# 1	Introduction

RAN held a Rel.19 workshop In June 2023 and endorsed a document (RWS-230488) containing the following page related to AI/ML for PHY:

Hence, to continue the discussion, RANP need to assess the outcome of the SI and whether the SI has been successfully completed to determine whether to start a normative WI or to continue the SI in Rel.19.

The paper outline is as follows:

- Outcome of RAN1 part of the SI and progress of RAN2 and RAN4 parts are reviewed in Section 2;

- Our preferences for new Rel-19 use cases are provided in Section 3; and

- Our conclusion, i.e., the Rel-19 contents are discussed in Section 4.

# 2	Review of 3GPP’s outcome and progress in the Rel-18 SI

## 2.1 General aspects (RAN1 discussion)

### 2.1.1 Methodology

On a high level, RAN1 has achieved the above objectives from the Rel-18 SID.  However, we note the following observation. The objective “AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes” was agreed but not widely followed.

AI/ML models were widely studied using intermediate KPIs in RAN1 evaluations, while system level simulation results using AI/ML models were less frequently reported. Studies indicate that intermediate KPIs do not correlate well with system level performance. Cross-checking of company results has not been performed.

### 2.1.2 Overhead, power consumption, memory storage, and hardware requirements

We note the following on power consumption, memory storage, and hardware requirements (including for given processing delays).

- No use cases have studied or evaluated power consumption.

- For memory storage of AI/ML model for model inference, all use cases have extensively reported the number of AI/ML model parameters. The reported number of model parameters may have implications for RAN2.

- No use cases have studied or evaluated hardware requirements as it was concluded that it is out of 3GPP scope.

### 2.1.3	LCM for one-sided models

The high-level stages of AI/ML model life cycle management (LCM) are model training, model deployment, model inference, and model monitoring. 3GPP has listed many model LCM options for each use case. Our view is as follows:

- One-sided models in the NW (beam management and positioning): Model LCM is an implementation issue for NW vendors.

- One-sided models in the UE (beam management, positioning, and CSI prediction): Depending on the use case, there may be specification impact relating to functionality activation, deactivation, and monitoring. However, detailed model-level LCM is an implementation issue for the UE/chipset vendor.

On functionality: The UE can be equipped with several AI models for a certain functionality (e.g., positioning, beam management, or CSI prediction). The LCM of individual models within the functionality is transparent to the NW. In functionality-based LCM, the NW may indicate activation/deactivation/fallback/switching of an AI functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI).

- For one-sided model in the UE, prioritize the functionality-based LCM approach in an eventual Rel.19 WI.

### 2.1.4	Model Transfer for UE-sided models

Enabling the NW to train a model on site-specific data and then transfer the trained model to the UE may appear like an attractive solution to overcome potential AI model generalization issues. However, we believe that AI model transfer from the NW to the UE presents significant technical challenges that would need to be solved before standardization is possible. Moreover, no use cases in the Rel-18 study item have provided strong motivation to standardize model transfer; simply put, there is no need or justification to consider model transfer at this stage. The beam management, positioning, and CSI prediction use cases have feasible alternatives using one-sided models (either UE-sided or NW-sided) that do not need model transfer. Similarly, the two-sided CSI use case is discussing training solutions that do not require model transfer.

The technical challenges of NW-to-UE model transfer are large; for example, we foresee the following problems:

      - The AI model will likely need to be lowered specifically for the UE’s target hardware.

      - One option is that the NW transfers a high level, hardware agnostic, representation of the model to the UE, and the UE performs target-specific model lowering. However, the model lowering process is complicated (e.g., often involving quantization and fine tuning) and, therefore, this poses a significant challenge for the UE.

      - Another option is that the NW transfers a generic “one-size-fits-all” lowered model that is not optimized for the UE’s hardware. This option may require the UE to use generic hardware that can run a model provided by an entity (NW-side or neutral-side), which is not optimized by nature.

      - The standardization effort to define and maintain an open-format for AI models is excessive.

      - Proprietary model information is disclosed across vendors when using open format.

      - It posses a heavy burden on the 3GPP network to store, register, maintain, retrieve, and transfer UE models (a plethora of different UE models are expected, considering factors like different UE vendors, different UE releases, different PHY functionalities, different deployment scenarios, and different chipset capabilities).

Similar challenges have been observed in other working groups that discussed model transfer for AI/ML use cases. For instance, details of the Model Deployment/Update process as well as the use case specific AI/ML models transferred via this process were out of RAN3 Rel-17 study scope and the proceeding Rel-18 work Item (TR 37.817 V17.0.0). Similarly, during Rel-17, SA2 has concluded that 3GPP standardized sharing of models across different vendor environments is not deemed feasible in that release of the specifications (TR 23.700-91). No further studies or updates on the topic have taken place.

Given that this is a new area for RAN1, the first approach is to start with the simplest alternatives similar to what was done in other working groups. It is proposed to first conclude the standardizing of alternatives without model transfer prior to discussing any model transfer solution. Based on this analysis, we conclude that standardizing model transfer is not justified for the RAN1 use cases considered in the Rel-18 SI.

- In an eventual Rel-19 WI, model transfer from NW to UE should be deprioritized.

### 2.1.5 Data collection for NW-sided models

For use cases where the AI/ML model on the NW side needs to be trained using DL L1 channel measurements performed by UEs, it is important to standardize appropriate data collection mechanisms for the UE’s measurements to be reported to the NW over the air interface. This is to ensure that the NW side can efficiently train models by collecting data with unified data format from different UE/chipset vendors, which is essential for reducing the complexity and data processing cost of the AI/ML model design.

In addition, by standardizing data collection via UE report over the air interface, the data quality can be tested and determined by use of the 3GPP standard. Moreover, the network may identify a need of collecting extra training data from a specific area or at a specific period of a day or week to improve its AI/ML model design, hence, a standard data collection procedure is required to support the network to collect the training data when needed. Data collection is also a key-aspect of the AI/ML life cycle; hence, it is important to address this stage for the completeness of the study item.

In a Rel-19 WI, specify UE reporting-based data collection mechanisms for NW-side AI/ML model training and monitoring.

## 2.2 Study item use cases

### 2.2.1 CSI

For the UE-side CSI compression sub use case: Our high-level assessment of the situation in RAN1 part of the SI is the following:

- Performance: A large amount of simulation results has been provided (mainly using intermediate KPI for 1 layer) but there has not been any discussion on analysis or conclusions discussed and drawn among companies based on these results.

  - There is a large variance in the presented results across companies, it has not been possible to cross-company verification.

  - The largest gain is in the CSI report overhead reduction KPI, but it is noted that CSI report overhead reduction does not provide any net system DL throughput performance gain since eType-II CSI is not coverage limited.

- Specification impact: About 25 agreements has been made to study the necessity, feasibility, and potential specification impact of aspects related to CSI compression but none of these studies has been performed and discussed in RAN1. The value of these agreements lies in defining options and terminology. At each WG meeting, a new set of studies has been added.  Hence, there is a very large number of outstanding studies to be performed by RAN1 before this feature can be mature for a WI.

We further assume that two-sided AI models are trained outside 3GPP. However, we again note that the industry has not converged on scalable, robust, and cost-efficient manner solution for training two-sided AI models in a multi-vendor ecosystem.

Continue study of CSI compression in Rel-19 to address the long list of agreed studies and to further quantify performance gains versus specification impact.

We continue this section with a few examples and observations from the SI on CSI compression in RAN1:

For the two-sided CSI compression sub use case, evaluations have been provided for intermediate KPI i.e., squared generalized cosine similarity. We note that even considering the standardized Rel-16 eType-II codebook, the absolute values of the SGCS intermediate KPI shows quite a spread. The codebook does leave room for different implementations but starting a work with a up to 11% difference in baseline can make it difficult to assess the gains.

Figure 1: Comparison of baseline performance using the SGCS intermediate KPI.

Looking at UPT results from system level evaluations there is quite a big spread in reported gains. However, it seems fair to say that AI/ML show performance gains at higher resource utilization and lower UL overhead. A general observation could be that the gains are from a few percentages up to 15% in mean UPT.

Figure 2: Mean UPT gain reported by different companies, showing the 0th, 25th, 50th, 75th, and 100th percentile (potentially excluding a few outliers), as well as the mean (marked by “x”). The results cover 3 different UL overhead regimes, and 3 different RU regimes.

Instead of UPT gains, some companies report uplink CSI reporting overhead gains, and those appear to be larger (up to 60%). The potential impact of such savings on, for example, coverage have not been thoroughly investigated. However, our investigation showed  that eType-II CSI does not have a coverage issue. Therefore, the overall gain of this uplink reporting overhead gains are questionable at this point.

CSI compression demonstrate modest performance benefits (up to 15% for reasonable RU) but large (~60%) CSI overhead savings, although it is questionable whether such CSI overhead savings can be turned into a benefit in network performance KPIs as there is not a general coverage problem with existing CSI reporting.

Computational complexity in terms of nominal FLOPs have been reported in all use cases. To enable fair comparisons of computational complexity of AI/ML based algorithms with legacy algorithms, optimized computational complexity using agreed optimizations and hardware targets need to be used. Moreover, a method to define baseline computational complexity needs to be defined. This has not been covered so far in the SI.

As an example, the following figure illustrates gains versus computational complexity for the two-sided CSI compression use case. There are about a factor 4000 in size between the largest and the smallest models, and even if the largest and smallest models are excluded as outliers, there are almost a factor 100 in size between the presented models. Moreover, the graph does not contain a common understanding of the complexity of the baseline.

Figure 3: Mean UPT gains versus complexity (nominal FLOPs), for high resource utilization and low UL overhead for the two-sided CSI compression use case.

For the UE-side CSI prediction sub use case: Our high-level assessment of the situation in RAN1 part of the SI is the following:

- Performance: Some amount of simulation results has been provided but there has not been any discussion on analysis or conclusions discussed and drawn among companies based on these results.

  - There is a large variance in the presented results across companies (between a loss of 5% to a gain of 76% for intermediate KPI and between 1% loss to 23% gain for FTP traffic), and it has not been possible to cross-company verification.

- Specification impact: Specification impact was only discussed at one meeting although the UE sided monitoring is rather well discussed for other use cases.  While 3GPP has a good understanding of potential specification impacts, there are some important issues that were not resolved in the SI.

For CSI compression, many options are left on the table relating to performance monitoring and data collection. It is our view that these issues need more study and evaluations before normative work may commence.

Continue study of UE-side CSI prediction in Rel-19 to further quantify performance gains versus specification impact.

The use case may move into a normative phase, if 3GPP agrees there is potential performance gains over using Rel.18 classical CSI prediction. In addition, data collection for monitoring needs to be further studied

### 2.2.2 Beam management.

For beam management use case: Our high-level assessment of the situation in RAN1 part of the SI is the following:

- Performance: RAN1 evaluations have focussed on intermediate KPIs, with few system-level simulations evaluating end-KPI performance (e.g., downlink throughput).

  - The potential gains of AI/ML-based solutions are mainly reported as a reduction in the number of UE measurements and/or NW RS-transmission overhead.

  - Full system-level evaluations targeting end-KPIs (e.g., downlink throughput) appear to present significant challenges with, for example, aligning on non-trivial baselines.

  - AI/ML-based solutions can accurately predict the “best/good” downlink beams in a large set of scenarios using less RS overhead than simple full beam sweeping.

  - AI/ML-based solutions marginally outperform non-trivial conventional baselines (e.g., hierarchal beamforming with tailored beam patterns).

  - Generalization capability has been properly evaluated for TX-beam prediction, while for beam pair prediction the generalizability to different RX-beam patterns for NW-sided models is still questionable.

  - Majority of conclusions regarding generalization capability are made for spatial beam prediction.

- Specification impact: Specification impact for a UE-sided model would mainly comprise of how the UE can report a predicted set of beams with associated confidence information. In addition, there would likely be impact for data collection in respect to model training and monitoring.

  - For NW-sided model, the specification impact would be limited, mainly comprising how to collect relevant data from UE (DL Tx beam measurements), and how to configure a UE with a non-measured (predicted) beam during inference operation.

  - It is expected RAN2 should investigate the possible data collection and how UEs can signal its capability in running beam prediction models.

  - We expect that RAN4 will need to address the measurement accuracy of L1-RSRP, the evaluations in Rel-18 SI indicate a significant performance drop when considering the existing measurement requirements in comparison with perfect measurements. Other topics for RAN4 investigations could comprise of how to ensure reliable UE reported confidence of predictions and reliable performance monitoring KPI reports.   .

Hence, intermediate KPI results indicate performance gains in terms of reducing UE measurements and RS overhead with AI/ML-based beam management in comparison to full beam sweeping.

The results based on the intermediate KPIs indicate that AI/ML-based solutions can accurately predict the “best/good” downlink beams in a large set of scenarios using less RS overhead than simple full beam sweeping. However, the gains are less substantial when comparing to non-trivial conventional baselines (e.g., hierarchal beamforming with tailored beam patterns). The potential gains of AI/ML-based solutions are mainly observed as a reduction in the number of UE measurements and/or NW RS-transmission overhead. It is unclear to what extent such gains will translate into NW system KPIs due to lack of system-level simulations.

Sufficient progress on the different stages of AI/ML have been studied in our view, however, one of the main challenges regarding the feasibility of TX/RX beam pair prediction have not been addressed properly. It is proposed that such sub use case should continue in a future study item. Hence, the DL beam prediction use case is in our view mature for a WI.

Continue normative work in Rel-19 for the beam prediction use case based on DL TX beam predictions.

### 2.2.3 Positioning

The positioning use case covers two sub use cases: direct AI/ML positioning and AI/ML assisted positioning. For both sub use cases under positioning, the UE position error (two dimensional) is the end-KPI. The AI/ML assisted sub use case has additional intermediate KPIs. The agreed baselines are existing Rel-16/Rel-17 positioning methods.

For the positioning use case: Our high-level assessment of the situation in RAN1 part of the SI is the following:

- Performance: Extensive evaluations have been provided for both sub use cases, covering end- and intermediate-KPIs. It is generally agreed that AI/ML based fingerprinting type of solutions may bring large gains for heavy non-line of sight scenarios.

  - Extensive generalization studies demonstrated that “fingerprinting” solutions for heavy NLoS scenarios do not generalize well.

- Specification impact: Training and inference data will consist of channel measurements, but it is still not agreed if a full channel impulse response, a power delay profile, a delay profile, or path information should be used.

  - Also the format of a measurement report is still under discussion. How many time domain samples that are needed/reasonable, from a complexity/signalling overhead vs performance point of view, is also still under evaluation and discussion.

Furthermore, 5 solution cases were defined and evaluated. The agreed cases were:

- Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

As stated above, significant performance gains were demonstrated for UE positioning for both assisted and direct AI/ML positioning. These performance gains were shown using statistical 3GPP channel models (the 3GPP indoor factory scenario (InF)).

Based on the results provided so far in the SI, we believe there is enough performance gains to move into normative work for positioning, for solutions with UE side and NW side models.

Continue with normative work in Rel-19 for the positioning use case and the exact details of which (e.g., which sub use cases) can be left to further discussion until December plenary.

### 2.2.4 Protocol aspects (RAN2)

RAN1 is actively discussing challenges around data collection, AI/ML model transfer, and AI/ML model lifecycle management. While progress has been made in RAN2, RAN1 has not yet replied to RAN2’s LS of data collection requirements and assumptions. Since RAN1 has not concluded on data collection requirements for training, it is our prediction that it will not be possible to start a RAN2 work item immediately in Rel19.

RAN2 has a Rel-19 SI to further clarify data collection assumptions and requirements and limit the scope of a later RAN2 WI.

### 2.2.5 Interoperability and testability aspects (RAN4)

RAN4 is actively discussing challenges around AI/ML model lifecycle management, performance monitoring and (multi/cross-vendor) training. While significant progress has been made, RAN1 has not yet provided sufficient input to interoperability and testability aspects in RAN4.

For the CSI compression use case which use two-sided models, the challenges in RAN4 are large and requires a lot of additional time for studies, it might be so that RAN4 need to move into a new paradigm to be able to define testing of two-sided models, defining reference models etc. Hence, for CSI compression, taking the overall RAN perspective and the rather modest gains seen from RAN1 studies, it is not warranted to start a WI on CSI compression based on AI/ML.

RAN4 work has just started but it’s clear that more TU is needed in Rel-19 for studies for two-sided use cases.

# 3. New use cases to be studied in Rel-19

We believe that it is useful to evaluate more use cases to better understand the potential for introducing and standardizing AI/ML for PHY. In this section, we share our preferences for a possible new use case for study in Rel-19.

## 3.1 Mobility

The learnings from Rel-18 AI PHY beam management use cases, including the AI/ML model performance evaluation methodology and model LCM aspects (e.g., data collection, model monitoring, model fallback, etc.), can be largely reused for studying new AI-based mobility use cases, e.g., AI-based RRM measurements prediction in time/spatial/frequency domain for mobility optimization. Potential benefits include reduced measurement/reporting overhead, reduced latency, and enhanced reliability for mobility handling.

The performance gain versus complexity should be carefully studied for each AI-mobility sub use case. AI/ML model for the mobility use case can be deployed either at the UE-side or at the NW-side. Hence, if AI mobility is considered as a new use case in Rel-19 AI PHY, both UE-sided and NW-sided models should be considered. Different from the AI based mobility optimization study conducted in RAN3, this new AI mobility use case study should focus on the interactions between the UE side and NW side over the Uu interface. Like for all UE-sided model use cases, for UE-sided AI-mobility models, the sub use cases need to be carefully selected to ensure RAN4 testability of UE performance and ensure predictable UE behaviour.

Study AI/ML based mobility focusing on the interactions between the UE side and NW side over the Uu interface

## 3.2 General data collection for proprietary AI/ML models

The current study item has used synthetic channels for generating data in the evaluation of each (sub-)use case. However, the importance of data collection in networks to support AI/ML PHY use cases has been recognized during study; indeed, extensive discussions have been carried out (still ongoing) in the study item on how to support data collection for model training and monitoring of the identified Rel-18 use cases.

Discussions on supporting data collection for powering payload reception with AI/ML functionality has so far not been treated within the study item discussions, but it is seen as an important component to allow extensive use of AI/ML at the air interface. For example, sequences of bits, known to both receiver and transmitter, can be defined to train such AI/ML based receivers, using the transmitted bits as labels (i.e., ground truths) during training using for example binary cross-entropy loss function.

In certain circumstances, existing receivers already have the capability to unequivocally identify the transmitted data bits, i.e., by relying on the bits decoded after successful CRCs. However, the application of this approach is not always feasible since a correct data decoding cannot be guaranteed in situations where AI/ML receivers should also operate adequately, e.g., those with low signal-to-noise-plus-interference ratios or with noticeable hardware-impairments. In these cases, the definition and transmission of bit sequences known to both receiver and transmitter facilitates an efficient collection of the data required to develop AI/ML receivers and decoupling data collection to service requirements on the payload transmission.

Specifying a general data collection framework for support of proprietary powered AI/ML receivers allows for e.g. improved receiver performance through, for example, AI/ML enhanced channel estimation, demodulation, and symbol de-mapping. The functionality can also be a steppingstone to more radical 6G concepts.

Study data collection procedures and methods for supporting development of proprietary AI/ML-enhanced models for payload reception, e.g., triggering of transmission of known data sequences.

# 4. Further discussion on Rel-19 SI/WI structure

The exact scope of Rel-19 normative work, if any, should be based on the SI’s conclusions in the TR [2]. We are supportive of normative work for selected Rel-18 (sub) use cases that demonstrate sufficient performance versus complexity gains and for which the study is completed.

We believe that the primary purpose of the AI/ML for NR air interface work in 5G advanced is to pave the way for AI/ML features in 6G. Therefore, 3GPP does not need to rush into Rel-19 normative work for features that are poorly understood in terms of specification impacts and/or performance gains.

Moreover, we believe that higher “performance vs complexity gain” thresholds are needed for normative work on (sub) use cases requiring significant departures from current practices. We believe that it is in 3GPP’s best interest to continue the study of (or even down prioritize) Rel-18 sub use cases for which the specification impact and/or potential performance gains are not well understood and agreed in [2].

In our view, these parts of the SI have potential to be ready for a Rel-19 WI under the assumption that RAN4 study has completed a study of the testing methodology for single sided models.

- Positioning based on single sided AI/ML model on either the NW side or UE side

- Beam management with at least spatial DL TX-beam prediction based on single sided AI/ML model on either the NW side or UE side.

On the other hand, these components most have not progressed sufficiently during the Rel-18 SI and need to be continued to be studied in Rel-19 due to either lack of proper RAN4 study or lack of RAN1 study.

- CSI compression based on two-sided AI/ML models, mainly RAN1 and RAN4 studies.

- CSI prediction based on UE-sided AI/ML model.

Hence, we propose to continue in Rel.19, the study of Rel-18 use cases for which the specification impact, testing methodology, and potential performance gains are not well understood and agreed in [2].

Rel-18 sub use cases that do not provide sufficient performance gains in proportion to its standardization complexity and effort needed to realize (e.g., training) should not be part of a work item.

Rel-18 sub use cases for which the specification impact, testing methodology, and potential performance gains are well understood and provide sufficient performance gains or benefits in proportion to its standardization complexity and effort needed to realize (e.g., training) can be part of a single work item within Rel-19 timeframe.

At the same time, we believe that is useful to evaluate more use cases to better understand potential performance gains, protocol aspects, implications for interoperability and testability, and 3GPP evaluation methodologies and requirements. Here the further exploration of two-sided models needs to be pursued, especially in RAN4. Therefore, we are supportive of a new Rel-19 SI that further explores the potential of AI/ML for PHY.

Rel-19 includes an AI/ML for NR air interface study item covering a new set of use cases. The SI covers the entire release and may have a significantly larger RAN4 part if two-sided models continue to be considered in 3GPP.

We believe that the SI and WI on AI PHY can be done in parallel.

# 6. Conclusions

For Rel.19 (whole release)

- Support a work item for single sided models for

  - Positioning

  - Beam management for DL TX prediction

  - Data collection of UE measurements (UE to NW)

- Continue study and potentially add new use cases to the study in RAN1/RAN2/RAN4

  - Mobility

  - Data collection for AI-enhanced receivers.

- Continue to study

  - CSI prediction (RAN1)

  - CSI compression (RAN1) and two sided models in general (RAN4)

Our proposals in this document are summarized as:

Proposal 1	For one-sided model in the UE, prioritize the functionality-based LCM approach in an eventual Rel.19 WI.

Proposal 2	In an eventual Rel-19 WI, model transfer from NW to UE should be deprioritized.

Proposal 3	In a Rel-19 WI, specify UE reporting-based data collection mechanisms for NW-side AI/ML model training and monitoring.

Proposal 4	Continue study of CSI compression in Rel-19 to address the long list of agreed studies and to further quantify performance gains versus specification impact.

Proposal 5	Continue study of UE-side CSI prediction in Rel-19 to further quantify performance gains versus specification impact.

Proposal 6	Continue normative work in Rel-19 for the beam prediction use case based on DL TX beam predictions.

Proposal 7	Continue with normative work in Rel-19 for the positioning use case and the exact details of which (e.g., which sub use cases) can be left to further discussion until December plenary.

Proposal 8	RAN2 has a Rel-19 SI to further clarify data collection assumptions and requirements and limit the scope of a later RAN2 WI.

Proposal 9	Study AI/ML based mobility focusing on the interactions between the UE side and NW side over the Uu interface

Proposal 10	Study data collection procedures and methods for supporting development of proprietary AI/ML-enhanced models for payload reception, e.g., triggering of transmission of known data sequences.

Proposal 11	Rel-19 includes an AI/ML for NR air interface study item covering a new set of use cases. The SI covers the entire release and may have a significantly larger RAN4 part if two-sided models continue to be considered in 3GPP.

# References

RP-213560, Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface, 3GPP TSG RAN Meeting #94e, December 6-17, 2021.

TR 38.843, Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface, 3GPP Release 18.