3GPP TSG-RAN #101	Tdoc RP-231748

Bangalore, India, September 11th  -15th, 2023

Agenda Item:	8A.2.2

Source:	Ericsson

Title:	Discussion on Release 19 MIMO enhancements

Document for:	Discussion

# 1	Introduction and summary

RAN held a Rel.19 workshop in June 2023 and endorsed a document (RWS-230488) containing the following page related to MIMO:

The key take-away is the third bullet stating the importance to focus on topics where there are  real and urgent commercial needs. Based on this guidance we see the need for these enhancements in the MIMO area:

- CSI reporting enhancements

  - for larger antenna arrays being developed for the new 6-7 GHz band (n104)

  - for addressing shortcoming of current NR link adaptation procedures (CQI enhancement only)

  - for making C-JT implementation realizable by introducing UE assisted cross-TRP calibration reporting.

- Uplink coverage and capacity enhancement for which there “always” is a commercial interest.

- Reducing the load on RRC due to beam management procedures for FR2

- Addressing specification limitations on reciprocity based MIMO in commercial networks.

- Support enhancements enabling lower complexity 8 antenna UE receivers with up to 8-layer DL transmission

# 2	Motivation and proposals for Evolved MIMO

In this section we discuss problems and shortcomings of the current specifications in six different areas and propose the specification-based solutions including some initial evaluation results to demonstrate the benefits in each area.

It should be noted that some of the proposed areas are not strictly MIMO enhancements, but they could be handled by other WI in Rel-19 if better suited (e.g., in an XR WI or Coverage WI).

## 2.1 CSI enhancements

### 2.1.1	CSI reporting enhancements for larger massive MIMO antenna arrays targeting the new “higher” FR1 frequency bands

The licensed n104 band (6.425-7.125 GHz) is gaining attraction for deployments in Asia and Europe. The use of this “high” FR1 band comes with coverage challenges. This development motivates the introduction of large antenna arrays, where “large” means an increase in the number of antenna elements (AE) by roughly 4 times, from ~200 AE to ~800 AE or possibly even more.

A larger antenna array provides an opportunity to better maintain the uplink and downlink coverage on par with the lower bands, allowing mobile network operators to a large extent reuse existing site grids. Note that the antenna array for band n104 may not be physically larger than the typical deployed massive MIMO antenna at e.g., 3-4 GHz, but electrically larger (in terms of wavelengths).

Electrically larger antenna arrays (increased number of antenna elements) are motivated for the n104 band (6.425-7.125 GHz) to maintain coverage and existing site grids as frequency is increased.

A straightforward implementation solution when increasing the number of AE is to increase the antenna subarray size. However, as described in [5], this reduces the angular range coverage of the antenna array. Furthermore, increasing the subarray size may generate interference towards non-3GPP systems such as fixed links and various satellite services in band n104. In [5], we provide simulation results describing the performance with different subarray sizes.

In this new band, it is likely that more than 8 columns will be implemented for band n104 since this can be done without extending the size beyond existing products. However, the current 24 and 32 port codebooks do not facilitate simultaneous steerability in azimuth and elevation for antennas with more than 8 columns.

Hence, extending specification to FD-MIMO for at least 16 column antenna arrays is important, but also 12 column massive MIMO is of interest for product implementation. Likewise, for hybrid beamforming, the current specification supports up to 4 elevation (“analog”) beams (selected by the UE and reported using CRI), but only 8 horizontal (“digital”) ports for a selected elevation beam. The limit to 8 ports in current specifications for the horizontal precoding means in practice a limitation to 4 antenna columns only, which is a small antenna array not suitable for the new n104 band.

A valid question to ask is that given this new n104 band is for TDD, why not utilize channel reciprocity (i.e., SRS) to determine the MIMO precoder? This is certainly possible for a single carrier system, but since the n104 band is rather wide in bandwidth, it will likely utilize carrier aggregation (CA) in the downlink.

To utilize channel reciprocity, this means a need to implement SRS carrier switching solutions in gNB and UE, which will interrupt traffic on the serving cell while the UE switches to an SCell to perform an SRS transmission.  With multiple carriers configured in CA, the traffic interruption and delay in obtaining reciprocity channel estimates becomes prohibitively long. In addition, these higher carrier frequencies have challenging uplink coverage.

Hence, for TDD systems with CA, it is crucial to support CSI reporting for carriers which do not have an uplink.

In summary, there are solid motivations to enhance the 3GPP specifications to support massive MIMO antenna solutions for the higher frequency bands within FR1 to allow development of large antenna arrays. Since this band is not used by 3GPP technologies yet, there is no issue with legacy terminal support if an enhancement is introduced in Rel.19 even though the feature itself is band agnostic in the specifications.

The expected enhancements are for CSI-RS resource and CSI reporting enhancements for both digital and hybrid implementation choices of massive MIMO advanced antenna systems. What we see is important for standardization of large arrays is to support the following ranges of larger antenna arrays to provide the necessary flexibility in gNB implementation of massive MIMO. Any combination of rows and columns from these two bullets should be supported:

- gNB with 8,12 and 16 antenna columns

- gNB with 2,3 and 4 vertical subarrays (i.e. antenna subarray rows)

Hence the maximum number of required CSI-RS ports to measure for a large antenna CSI report is 2*4*16 = 128 ports, which is the same that Rel.18 UE already support for C-JT CSI reporting by aggregation of 32 port CSI-R resources. In addition, we assume most (or all) of the needed changes is to modify the size of the DFT precoder (i.e. N1 and N2 parameters) since the W2 part of the codebook remains the same.

Since this is a commercially important and urgent enhancement, we are fine to limit to Type I codebook only, to shorten the time to market, but are open to include Type II as well if time allows (for completeness). We thus propose the following:

For FR1 operation, introduce support for NZP CSI-RS and CSI reporting for large antenna arrays with 8, 12 and 16 columns including simultaneous vertical and horizontal (FD-MIMO) precoding, by

        - Extension of NZP CSI-RS resource beyond 32 ports (48, 64, 72, 96, 128 ports) with a reuse of legacy CSI-RS resource definitions

        - Extension of CSI reporting for at least the Type I PMI reporting/codebook for the associated extended NZP CSI-RS resources with up to 128 ports

        - Extension of CSI reporting for CRI based reporting with up to 32 CSI-RS ports per resource (hybrid beamforming)

### 2.1.2	CQI reporting enhancement to address shortcomings of current NR link adaptation procedures

Link adaptation aims to adjust the coding and modulation of a transmission to the channel quality. In DL, the NW selects the modulation and coding based on CSI measurement reports from the UE, which were performed on CSI-RS received in the past. In addition, HARQ is also a tool to perform link adaptation since the effective coding rate decreases when multiple retransmissions are performed.

Link adaptation is based on reported measurements on sparse transmission of CSI-RS depicted in Figure 8.

Figure 1: Transmission of PDSCH and CSI-RS over several slots. Note that the channel quality can only be estimated from the sparse CSI-RS transmission.

Just looking at Figure 1, it seems challenging to estimate the transmission quality of the dense PDSCH transmissions only based on the sparse CSI-RS transmissions: for example, how can we ensure that the interference situation in the sparse CSI-RS resources is representative of the interference situation over all the PDSCH transmissions between each CSI-RS occasion? In addition, since the CSI-RS measurements are performed separately from the PDSCH reception, receiver impairments, DMRS channel and interference estimation errors, frequency offsets and phase noise impairments may not be captured.

The CSI feedback on measurements on CSI-RS are inaccurate since, the CSI-RS transmissions are sparse in time and even if the CSI-RS is configured densely in time, the UE may receive the CSI-RS and the PDSCH using different receiver algorithms which leads to inaccuracies in CSI reporting.

The link adaptation resulting from the HARQ retransmission operates on the actual PDSCH receptions directly and thus avoids many of the issues related to the feedback based on CSI-RS. On the other hand, it may suffer from long latency: if the initial selection of modulation and coding scheme is off, it may take many retransmissions to recover from that, causing a large delay, since the retransmissions are separated by at least a few ms. For some services, e.g., XR, this delay may be too long to be useful:

Performing link adaptation based on HARQ retransmissions is slow and may be unable to reflect fast variations in channel or interference.

Despite these shortcomings, the link adaptation solution has not been enhanced during the NR specification – in fact, it is the same solution that was implemented during the early LTE releases. There have been attempts to improve the support in the URLLC work, but due to a lack of focus, these attempts have been unsuccessful.

In summary, the link adaptation based on CSI reports is inaccurate, since the resources where the measurement is made are sparse and separated from the resources where the PDSCH is transmitted. On the other hand, the (implicit) link adaptation based on HARQ feedback leads to long delays, since multiple retransmissions may be needed to reduce the effective coding rate.

To avoid these issues, we propose to introduce additional CSI feedback, which is determined from the actual received PDSCH quality of a previous scheduling. When a gNB receives this new information, it can directly adjust the MCS for the next, similar, transmission, without the need for outer-loop LA adjustments based only on ACK NACK feedback. Moreover, if the PDSCH could not be decoded, the gNB can use the post-decoding CQI to improve the re-transmission: the gNB can compare the CQI used to select the modulation and coding scheme with the CQI measured at the PDSCH reception. If the CQI used for the initial selection is much higher than the CQI measured at the PDSCH reception, the gNB could adjust the time- and/or frequency-domain allocation and modulation of the retransmission, to increase the chance for successful reception already at the next retransmission. Note that the specification supports changing the time/frequency allocation of a retransmission, but the feedback does not provide sufficient information to the gNB to make an intelligent use of this freedom.

Similar ideas have been floated in previous releases, e.g., DMRS based CQI and soft-HARQ-ACK. The DMRS based CQI is yet another approximation for the “true” CQI for the received PDSCH. The soft-HARQ-ACK is on the other hand a better representation of such.

The benefit of the proposed CSI feedback is thus twofold:

- to select the modulation and coding scheme for the initial PDSCH transmission, and

- to adjust the number of retransmissions in case the PDSCH could not be decoded.

We will now present evaluation results for DMRS-based CSI, demonstrating the latter benefit only: the link adaptation for the initial transmission still relies on the baseline link adaptation. We compare the DMRS-CQI scheme with two baseline schemes:

- TB-based transmission (denoted as “TB-tx”)

- CBG-based transmission (denoted as “CBG-tx”), where CBG (Codeblock group) HARQ-ACK is enabled, and UE is configured with 8 CBGs.

Figure 2 shows simulation results in Urban macro (UMa) scenario, respectively, with the different link adaptation schemes. All schemes are simulated with outer loop (OL) or without outer loop (no OL). Reciprocity-based precoding and proportional fair scheduling is used in this system level simulation. More details can be found in [1]. The results are depicted in Figure 2.

- Figure 2 Urban macro (UMa) DL capacity evaluation for LA schemes for AR/VR (left figure) and cloud gaming (right figure).

Based on the results in Figure 2, we observe

For comparable BLER targets in link adaption, DMRS-based CQI gives a significant downlink XR capacity improvement compared to a baseline TB- or CBG-based scheme.

We will also illustrate the performance benefits of soft-HARQ-ACK, for which results are shown in the Figure 3 below. The following scenarios were simulated and presented in the figure:

- A baseline case, where the outer loop LA adjustment is triggered on NACKs,

- Three cases where the outer loop LA adjustment is triggered on both NACK and ACK_low_margin, where the ACK_low_margin means PDSCH pass with low decoding margin. ACK_low_margin is reported if the LDPC decoder succeeded, but not within a limited number of iterations of:

  - “3 iterations”

  - “5 iterations”

  - “3/4/5 iterations” with 3 iterations for QPSK, 4 for 16QAM and 5 for 64QAM

The simulation uses an outer loop target rate of 1%, a value that is chosen smaller than for the MBB case, but not so small that convergence will be impossible.

The resulting BLER will then be lower than the outer loop target rate since the decoder is allowed to run several additional iterations even after ACK_low_margin is reported. The actual resulting BLER value will depend on where the event threshold is set. A smaller number of iterations before reporting an ACK_low_margin will lead to a lower BLER but will, due to the larger resulting back-off, also reduce the resulting throughput, which can be seen comparing the curves for 3 and 5 iteration thresholds.

As is visible from the figure, to reach a low resulting BLER also at lower SNR, a lower threshold may be used. The curve marked “3/4/5 iterations” uses different thresholds for different modulations. In this way the BLER is further reduced on lower SNR; while on higher SNR, excessive backoff is avoided. More details can be found in [4].

Figure 3 Resulting BLER and throughput after triggering outer loop on decoding success after a limited number of decoding iterations. 52 PRB allocation. Outer-loop target rate of 1%. If the outer loop is triggered before the decoder is finished decoding, the resulting BLER will decrease.

We have shown that both DMRS-CQI and soft-HARQ-ACK provide significant performance benefits. Based on these results, we propose

Study, and if beneficial, specify CQI reporting based on actual received PDSCH quality (e.g., PDSCH DMRS or PDSCH decoding based CQI).

### 2.1.3	CSI enhancement to support Coherent-JT with realistic non-ideal T/F sync

Coherent-JT in Rel.18 MIMO work item considered only ideal time/frequency synchronization, i.e., the DL timing and carrier frequency at multiple TRPs were assumed to be perfectly aligned. This is not the case in practice and there will be some DL timing and carrier frequency differences among TRPs.

There are a number of challenges in supporting CJT in practice.  Firstly, propagation delays between different TRPs and a UE can be different. Furthermore, the DL timing difference between TRPs will add on top of these propagation delay differences.  These combined effects lead to large delay differences that result in a very frequency selective composite channel. In type II CJT CSI feedback currently being specified in Rel-18, a precoding matrix per PMI subband is reported.  The smaller the PMI subband size, the larger delay spread that can be handled. However, even with the smallest PMI subband size, the tolerable delay spread is rather small:

The maximum channel delay spread that can be handled by type II CB based CJT is a function of PMI subband size and SCS. Even at the minimum PMI subband size, the maximum delay spread that can be tolerated by Rel-18 CJT is rather small.

In [5], we discuss a simple toy example where a UE receives a signal transmitted from two TRPs with various delay differences between the two TRPs and with an ideal co-phasing being applied to the signals. Due to the presence of a delay difference between two TRPs, a linear phase change across subcarriers occurs. When the co-phasing is per PMI subband, the signals would not be phase aligned on some subcarriers and a notch would appear at some subcarriers. As the delay difference increases, the notch becomes deeper.  Based on the example, we observe

CJT does not work well in presence of some modest delay differences between TRPs even with the minimum PMI subband size of 2RBs.

Secondly, even though a same nominal transmit frequency may be assumed at multiple TRPs, due to local oscillator stability, there will be some actual transmit frequency difference between the multiple TRPs.  In 3GPP RAN4, the maximum transmit frequency error for a base station is specified in TS38.104 and the most stringent requirement is +/-0.05ppm. Even with +/-0.05ppm, there will be some residual frequency errors.  These frequency errors means that the relative phase of the signal received from different TRPs will change over time.  For example, at 1GHz carrier frequency, the relative phase can change up to 180 degrees over 5ms time period. In other words, the time between when a CSI is measured and the time at which the CSI is applied needs to be much less than 5ms. This is not practical in most cases.

The above issues were raised during Rel-18 CJT discussion.  Although companies acknowledged the need for considering realistic T/F sync when evaluating CJT, these issues were not considered in Rel-18 due to workload and lack of time.

One potential solution is for the UE to measure and report the delay and/or frequency differences between TRPs by the UE so that the gNB can perform delay and/or frequency pre-compensation to preserve the performance benefits of CJT. Since delay and frequency pre-compensation may also be needed for reciprocity based CJT, the report should be a standalone report and may not be coupled with any codebook based CJT CSI (e.g., Type II) feedback.

Error! Reference source not found. shows the impact of frequency drift and timing error for a MU-MIMO system level simulation with CJT using the Rel-18 CJT Type II codebook and full buffer traffic. The simulation is conducted in Dense Urban scenario and UEs that are served by more than one TRPs are filtered for illustrating the performance loss. The frequency drift is set to 10 ppb (parts per billion) at 4GHz, which corresponds to 40 Hz frequency offset, or 144 degrees of phase drift over a CSI reporting periodicity of 10 ms. The timing error for different TRPs are assumed to be uniformly distributed between 0 and 1 cyclic prefix, which is between 0 and 4.69 us for 15 kHz subcarrier spacing. For the simulated case, we can see that the frequency drift alone and the timing error alone can cause 6% and 12%, while having both frequency drift and timing error can cause 26% performance loss. Hence, the impact of frequency and timing error can be significant and must be considered.

Figure 4: Impact of frequency drift and/or timing error for CJT with Type II codebook

Therefore, we have the following proposal:

Introduce enhancements that enable C-JT under realistic time/frequency synchronization among TRPs by reporting

- delay difference(s) between TRPs, and

- frequency difference(s) between TRPs.

## 2.2	Uplink coverage and capacity enhancements

Uplink coverage and capacity are key metrics in network designs, and enhancements have been of interest to operators in each release of NR.  However, UE RF complexity and power limitations tend to make such enhancements challenging from a technical viewpoint, potentially limiting their appeal to UE vendors. Therefore, it is important to target solutions that address use cases that are of sufficient interest to UE vendors.

### 2.2.1	Multi-layer UL MIMO coverage

The NR uplink has two waveforms: precoded (DFT-S-) and non-precoded (CP-) OFDM. DFT-S-OFDM has a lower peak to average power ratio than CP-OFDM, which allows the UE to operate its power amplifier closer to its maximum output power (theoretically ~3 dB closer for QPSK as measured by the cubic metric). This higher output power can be used to improve uplink coverage, especially for higher modulation orders. It also enables more efficient PA operation, and consequently reduced current drain in the UE.

In Rel-18 discussions of the UL-MIMO and the coverage enhancement work items, both multi-layer DFT-S-OFDM and fast DFT-S-OFDM to CP-OFDM switching were considered.  However, the scope of Rel-18 work items was large, especially for MIMO, and the need to specify both multi-layer DFT-S-OFDM and dynamic switching between DFT-S-OFDM and CP-OFDM was questioned, which led to only dynamic switching between DFT-S-OFDM and CP-OFDM being specified in Rel-18.

Given that dynamic switching between DFT-S-OFDM and CP-OFDM is supported in Rel-18, it is possible to improve coverage with single layer DFT-S-OFDM while retaining the ability to dynamically select multi-layer transmission with CP-OFDM when channel conditions improve.  However, dynamic switching does obviously not improve the coverage of multi-layer MIMO transmission.

Dynamically switching to DFT-S-OFDM from CP-OFDM can improve coverage in rank 1 channel conditions, and therefore its performance benefit is primarily for cell edge throughput.  This dynamic switching doesn’t provide a benefit when channel conditions allow for rank 2 or more, which is common in large portions of cells in NR deployments.

A first observation is that that rank 2 or higher transmission can be quite common in a cell, and that multi-layer transmission can be a mechanism to deliver higher power especially for non-coherent UL MIMO UEs. The figure below shows a histogram of the UL MIMO rank in a cell when the gNB has 4 or 32 Rx antennas.  Rel-15 non-coherent UL MIMO transmission is used, and FTP model 1 traffic is used. Resource utilization is roughly 40%. The details of the simulation setup and further discussion can be found in [3].

Figure 5	UL MIMO rank histograms for 4 and 32 Rx gNB

Very few UEs transmit only with rank 1. In the 4 Rx case, less than 1% of the UEs transmit rank 1, while for 32 gNB Rx antennas, rank 2 is always used. One major reason for the use of high rank is that non-coherent UEs gain 3 dB more power by transmitting two layers.

To quantify the relative gains of fast DFT-S/CP-OFDM switching and multi-layer DFT-S-OFDM, we considered 3 scenarios where the two first ones are possible in today’s specifications:

- Rel-15 codebook-based UL MIMO using CP-OFDM,

- Rel-18 fast switching between CP-OFDM and DFT-S-OFDM for rank 1, but CP-OFDM for rank 2, and

- Proposed Rel-19 DFT-S-OFDM for both rank 1 and rank 2.

Observing that there is 1 and 3 dB MPR in 38.101 in section 6.2.2 for QPSK with DFT-S-OFDM and CP-OFDM, respectively, in the edge and outer bands, and a similar difference in power (of 1.5 dB) for the inner bands, in these simulations we model DFT-S-OFDM transmission maximum power as 22 dBm, while CP-OFDM is 20 dBm.  We expect these differences are conservative given that 38.101 captures minimum performance requirements and given the higher amounts of cubic metric observed as discussed above. Furthermore, techniques to further reduce MPR for DFT-S-OFDM are being studied in Rel-18, and if these are specified, there may be additional coverage benefits of DFT-S-OFDM, and therefore multi-layer DFT-S-OFDM.

The system level simulations further use FTP model 1 traffic, and link adaptation selection between rank 1 and 2. Results from both 4 Tx and 2 Tx are given Error! Reference source not found. and Error! Reference source not found. below, respectively, for 20, 50, and 70% cell load, as gains over the CP-OFDM baseline. More details can be found in [5].

Table 1: System throughput gains over legacy Rel.15 CP-OFDM for fast switching and multi-layer DFT-S-OFDM: 2Tx, xpol.

|  | Mean Throughput Gain |  |  |  | Cell Edge Throughput Gain |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20% |  | 50% | 70% | 20% | 50% | 70% |
| Rel.18 Fast-Switching |  | 4% | 2% | 1% | 17% | 27% | 34% |
| Rel.19 DFT-S-OFDM |  | 11% | 14% | 17% | 31% | 33% | 34% |

Table 2: System throughput gains over legacy Rel.15 CP-OFDM for fast switching and multi-layer DFT-S-OFDM: 4Tx, directional.

|  | Mean Throughput Gain |  |  | Cell Edge Throughput Gain |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20% | 50% | 70% | 20% | 50% | 70% |
| Rel.18 Fast-Switching | 5% | 8% | 9% | 68% | 63% | 59% |
| Rel.19 DFT-S-OFDM | 12% | 16% | 19% | 70% | 78% | 81% |

Examining the results, we see that the main gains from fast switching are for cell-edge throughput, especially at low loads.  The 2 Tx gains range from 17—34%, while the 4 Tx gains are 59—68%.  However, the mean throughput gains from fast switching are more modest: up to 4% and 9% with 2 and 4 Tx, respectively.

The Rel.18 fast DFT-S/CP-OFDM switching has strong cell edge gains over configured DFT-S- & CP-OFDM waveforms, but less benefit to mean cell throughput.

By contrast, while multi-layer DFT-S-OFDM further improves cell edge gains, it additionally provides strong mean throughput gains, especially at high load and with 2 Tx.  Mean throughput gains of 11—17% and 12—19% are found for 2 and 4 Tx, respectively, which are up to 16% and 10% better than fast switch for 2 and 4 Tx (at 70% resource utilization).

Cell edge gains for 2 Tx with multilayer DFT-S-OFDM range from 31% to 34% and up to 14% better than fast switching (at 20% resource utilization), while for 4 Tx they are 70—81%, and up to 12% better than fast switching (at 70% resource utilization.) These higher mean throughputs from rank 2 DFT-S-OFDM as compared to fast switching are natural since the higher throughput from rank 2 is available over more of the cell.  Similarly, since rank 2 is commonly used in the cell, multi-layer DFT-S-OFDM can even improve cell edge performance.

The proposed Rel.19 multi-layer DFT-S-OFDM has substantial mean throughput gains (e.g., ~15% or ~20% for 2 or 4 Tx) at the system level, as well as for cell edge (e.g., up to ~35 or ~80% for 2 or 4 Tx).

Since it improves coverage of rank 2+ UL MIMO, multilayer DFT-S-OFDM improves mean throughput substantially over fast DFT-S/CP-OFDM switching, by up to, e.g., 16% and 10% for 2 Tx and 4 Tx, respectively.  Multi-layer DFT-S-OFDM also improves cell edge throughput over fast switching by up to, e.g., ~14% or ~12% for 2 or 4 Tx.

Based on these observations, which is important to operators to improve the uplink throughput for a larger area of the cells, we propose

Specify rank>1 DFT-S-OFDM for PUSCH as it improves the portion of the network area where rank 2+ PUSCH can be used.

### 2.2.2	Uplink network capacity loss due to blocking by a narrowband SRS transmission

In FR2, only PUSCH frequency-domain resource allocation type 1 is supported (contiguous set of PRBs). A UE near cell edge need to transmit a narrowband SRS that will render unused RBs for other UEs’ PUSCH. An illustration is provided in Figure 2, where two UEs scheduled in the same slot and a narrowband SRS transmission (e.g., for SRS configured with inter-slot FH and repetition factor 4) occupies a small set of PRBs in the middle of the BWP. This result in poor spectrum utilization and thus network capacity loss in FR2.

Figure 6: An example of PRB allocation for two UEs in the same UL slot and where one of the UEs is transmitting narrowband SRS with inter-slot frequency hopping.

As described in [5], as similar issue exists in FR1 when intra-slot SRS FH is configured. Thus, we observe that narrowband SRS transmissions may result in poor spectrum utilization in FR2 and in FR1 when intra-slot SRS FH is configured.

Based on the above observations, there is motivation for enhancing PUSCH scheduling to support better spectrum utilization in the UL when narrowband SRS transmissions are present, leading to the proposal:

Introduce FDM-based rate matching of PUSCH in both FR1 and FR2 to avoid collision with an SRS transmission.

In FR2, such FDM-based rate matching of PUSCH may involve supporting non-contiguous PRB allocation for PUSCH. To support PUSH rate matching around SRS configured with intra-slot FH and/or when SRS transmissions from different UEs are in the same slot but occupy different PRBs, there may be additional UL capacity gains by introducing reserved resources on a per OFDM symbol level (e.g., “zero-power SRS”) together with PUSCH rate matching to adjust the coded data rate to match the allocated resources.

### 2.2.3	Uplink limitations for UEs with antenna switching

- In legacy NR, for UEs equipped with less than half the number of TX ports compared to the number of RX ports (i.e., for t1r4, t1r6, t2r6, t2r8, t1r8), antenna selection for PUSCH is to a large extent up to UE implementation.

- The reason for this is that these antenna switching schemes requires more than 2 SRS resources, while there is a limit of maximum 2 SRS resources for codebook-based UL transmission (except for the Rel-16 full power mode 2, where up to 4 SRS resources can be configured). Antenna selection for PUSCH based on UE implementation can in many cases become sub-optimal since the UE is not aware of the current UL interference situation, both between different UEs in the same cell (i.e., intra-cell interference for UL MU-MIMO) and due to interference from UEs belonging to different cells (i.e., inter-cell interference for UL SU/MU-MIMO), which will deteriorate the UL performance. In addition, UE implementation based PUSCH antenna selection may in some case not be fast enough, which could deteriorate the UL performance even further due to aging of the UL channel.

In addition, for UL codebook-based operation with multi-port SRS resources, the network can indicate one out of the two configured SRS resources (using the single-bit SRI in DCI), and the corresponding PUSCH transmission will then be transmitted over the SRS ports (and hence corresponding UE antennas) associated with the selected SRS resource.

Since it is up to UE implementation to determine the “SRS-port to UE-antenna”-mapping, it is also up to the UE to determine which UE antennas that belongs to which SRS resources, which limits the PUSCH UE antenna selection flexibility for the network. For example, assume that a 2t4r UE is configured with two SRS resources for codebook-based UL transmission; SRS resource 1 consisting of SRS port 1 and SRS port 2, and SRS resource 2 consisting of SRS port 3 and SRS port 4. In this case, the SRI can either select PUSCH transmission over SRS port 1 and SRS port 2 (by indicating SRS resource 1 in the SRI) or over SRS port 3 and SRS port 4 (by indicating SRS resource 2 in the SRI), which might be sub-optimal for example if the network has determined that the best PUSCH performance will be attained if the UE transmit over e.g. SRS port 1 and SRS port 3.

Hence, we propose to evaluate the benefits of this feature and if beneficial specify this as part of Rel.19

Study, and if beneficial, specify enhanced closed-loop antenna selection for PUSCH.

## 2.3	FR2 signalling enhancements

Due to the multiple beams transmitted from one cell, a UE in FR2 needs to be informed when it moves between coverage areas of different beams. Every NR release has streamlined this signalling, but there are still improvement areas.

### 2.3.1	Signalling enhancement for TRS

The QCL source for the PDCCH/PDSCH reception is the TRS, and since a TRS always is associated with an SSB via QCL association (‘QCL-Type-C’ and ‘QCL-TypeD’ where applicable), it is preferred to configure a TRP at FR2 with a one-to-one mapping between an SSB and a TRS (i.e., for each transmitted SSB, a TRS is transmitted in the same beam as the SSB).

However, for TRPs at mmWave frequencies with many beams and carriers, it is not possible to configure the UE with one TRS per SSB per carrier due to that UEs support a limited number of TRSs across all configured carriers (indicated by maxConfiguredResourceSetsAllCC in 38.306).

This means that when a UE moves to a new SSB beam which is not associated with an TRS (for one or more of the DL carriers), the TRP needs to update the QCL of a TRSs such that it is associated with the new SSB. The QCL update of the TRS requires RRC signaling, which introduces additional latency and overhead signaling in the system.

For TRPs in FR2 with many beams and DL carriers, the QCL source of TRS need to be updated when a UE moves around in the cell. This requires RRC signalling, which introduces additional latency, overhead and NW side processing complexity.

As described above, a TRP with many beams and DL carriers will require QCL updates of TRS using RRC signaling when a UE moves around in the cell, which introduces additional latency and overhead signaling in the system. To mitigate this issue, it would be beneficial to introduce a more dynamic QCL source indication for TRSs, hence we propose:

Introduce dynamic switching for QCL source of TRS.

In practice, this would imply that we allow that a semi-persistent TRS is used as QCL source for PDCCH/PDSCH DMRS. The UE would use this semi-persistent TRS also for radio link monitoring and beam failure detection. As the QCL source for semi-persistent CSI-RS can be updated using MAC CE, RRC reconfigurations are avoided.

### 2.3.2	Signalling enhancement for SRS

For mmWave base stations, SSB is typically transmitted in semi-wide beams, while the data is transmitted/received in narrower beams. One simplified schematic example of this is illustrated in Figure 7, where the TRP is configured with two SSB beams, and where there are three narrow beams per SSB beam. To ensure that the TRP uses a suitable narrow beam for a UE, the TRP can configure the UE to frequently report the best SSB beam and trigger the UE to transmit a multi-symbol SRS resource based on the received report. The TRP can then sweep through the narrow beams within the best reported SSB beam while receiving the multi-symbol SRS resource, and in this way determine a suitable narrow TRP beam. To achieve good performance, the spatial relation of the SRS resource must be equal to the reported SSB. However, updating the spatial relation of an SRS resource requires MAC-CE signaling, which leads to significant latency and overhead signaling in the system.

For TRPs in FR2 where SRS transmissions are used to evaluate narrow TRP beams within a new best reported SSB beam, MAC-CE signalling is required to update the spatial relation of the SRS transmission, which introduces additional latency, overhead and NW side processing complexity

Figure 7: A schematic simplified example of a TRP that is configured with two SSB beams, and where there are three narrow beams per SSB beam, which is communicating with a UE with two panels in opposite directions.

To mitigate this issue, it would be beneficial to introduce a more dynamic update of the spatial QCL source of an SRS resource. Such a possibility exists in legacy: each SRS resource can be configured with a different spatial relation/UL TCI state. However, the number of SRS resources are too small to make it possible to configure one SRS resource per SSB. Hence, we propose

Enhance the DCI-based spatial relation/TCI state indication for SRS.

## 2.4	Critical issue with reciprocity-based DL SU/MU-MIMO in commercial networks

A complication of reciprocity-based DL precoding relying on SRS antenna switching is that the current RAN4 requirements allow quite large SRS output power differences at different UE antenna ports. This can result in large channel estimation errors in the reciprocity-based DL channel estimation, leading to degraded DL MIMO performance. SRS power differences are largely caused by different insertion losses (IL) associated with sounding y UE antennas using x<y Tx chains by configuring xTyR antenna switching. Since such antenna switching take place only in the UL (Rx chains are directly connected to UE antennas and, hence, does not experience the same IL), there is a mismatch between the reciprocity-based DL channel estimate and true DL channel due to SRS power imbalances.

This SRS power imbalance issue has been discussed in RAN4 and in RAN1 as a TEI-18 and there was a common view in RAN4 that some RAN1 enhancements are needed. Several possible solutions have been identified by RAN4 in a LS sent to RAN1 in RAN1#112e-bis but no conclusion to the issue was agreed in RAN1#114.

To improve the quality of the reciprocity-based channel estimate, UE should report the SRS power imbalance such that the gNB can compensate for these effects when making DL scheduling decisions.

The Table below summarizes the system level performance loss for UEs with SRS power imbalance compared to UEs with no SRS power imbalance for reciprocity-based DL MU-MIMO transmission and for 4 RX and 8 RX UEs. The simulation is conducted in UMa scenario with non-full buffer traffic with MU-MIMO scheduling.

The SRS power imbalance is modelled as x dB power offset relative to the first SRS port. For UEs with 4 RX, the SRS power imbalance for the four SRS ports is 0, -3, -6, and -9 dB, while for UE with 8 RX the SRS power imbalance for the eight SRS ports is 0, 0, -3, -3, -6, -6, -9, and -9 dB.

From Table 3, a considerable performance drop can occur in the presence of SRS power imbalance, for both 4 RX and 8 RX UEs. The performance loss is in general larger at the cell edge and for higher resource utilization.

Table 3: Performance loss due to SRS TX power imbalance for 4 and 8 RX UE.

|  | Mean throughput loss |  |  | Cell edge throughput loss |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Resource utilization | 20% | 50% | 70% | 20% | 50% | 70% |
| 4 RX UE with TX power imbalance | -10% | -17% | -25% | -16% | -26% | -36% |
| 8 RX UE with TX power imbalance | -3% | -11% | -28% | -7% | -20% | -38% |

Based on the above identified problem, we propose the following:

Support the gNB to obtain information of per SRS port imbalance to enable gNB side CSI correction.

# 3	Thoughts on other areas

During the Rel-19 workshop, some areas were discussed. Here we will share our view on two of them.

## 3.1	UE-initiated beam reporting/switching

Over the past few releases, there have been proposals on UE-initiated beam reporting. Such reporting would resemble the reporting for L3 mobility, where the UE performs measurements on (periodic) reference signals, only reporting the measurement results to the NW if certain conditions are fulfilled, i.e., if some events occur. This reporting is known as event-driven reporting, and 38.331 defines several different types of events, e.g., target cell becomes x dB better than serving cell. Another example of event-driven reporting in legacy is the power headroom reporting: the UE notifies the NW that the power head room has changed if some conditions have changed.

In principle, event-driven reporting could be applied also to the (intra-cell) beam reporting. The UE could perform measurement on (periodic) DL reference signals, and once certain conditions are fulfilled, the report could be sent to the network.

Fundamentally, a solution for event-driven reporting is composed of several parts:

The event definition: what would trigger the UE to send a report to the NW. Usually, the triggering mechanism would be based on quantities included in the report.

The mechanism to request UL resources for the transmission of the report. This step is optional: for power headroom reporting, the UE does not request UL resource to transmit the report: the report is only sent if the UE anyway has UL resources.

The reporting format: what the report would look like: what reporting quantities would be included, and how would the measurements be identified.

These three aspects are coupled: the requirements of each step would depend on the design choice of the other.

The main benefit of event-driven reporting is that it reduces signaling overhead, since the measurements are only sent to the NW if certain conditions are fulfilled.  On the other hand, event-driven reporting is typically slower than NW-initiated reporting, due to the time it takes to request UL resources for the transmission of the report:

Event-driven reporting reduces signaling overhead and is slower than NW-initiated reporting, due to the inherent delay to request UL resources for the transmission of the report.

Since beam management is performed on L1/L2, the event-driven reporting would have to operate on L1/L2 as well. Defining events on L1/L2 is somewhat new: essentially only one event is defined in RAN1 specifications: the radio link monitoring – the beam failure detection event is basically the same. This type of event is quite simplistic since it is only triggered once the serving link drops below a certain threshold. An event suitable for beam reporting would have to consider the quality of other beams, and it should trigger earlier, since the intent is to discover the best beam.

In contrast, the RRC specification defines a large number of events, where the quality of serving and neighbour cells is considered jointly. Also, filtering is applied to the measurements, both across beams and over time. Furthermore, the event is only triggered if the filtered measurements fulfil a condition for certain period of time. In summary:

The current events defined in RAN1 specifications are inadequate for event-driven beam reporting, whereas the L3 events for mobility are complex and versatile.

In our view, to be used as a tool to reduce signaling overhead, the beam reporting events would have to resemble the L3 events. Since RAN1 has never specified events of this complexity, this would probably be too big of a task for one release.

Since the aim is to reduce signaling overhead, it is not an option to rely on configured grant resources to transmit the UL report: the resources must be dynamically allocated by the network. The main question is then if the UE would request the UL resources (like the reports from L3), or if the UE would send the report opportunistically (like the power headroom reports). If the UE requests the UL resources, would we rely on existing mechanisms (legacy scheduling request) or define a new mechanism (similar to SCell/mTRP BFR). Defining a dedicated mechanism to request UL resources would be a quite time-consuming task for RAN1.

Defining the format and content of the report would be more in line with RAN1 typically does. There are already many reporting methods and formats defined, to be sent as UCI over either PUSCH or PUCCH. The legacy reporting has been designed to deliver measurement results, which can be used by the NW rather quickly. Consequently, the measurement reports cannot be retransmitted: the thought is that any retransmission would arrive too late to be useful, and that the UE would anyway send a new measurement report with update content only a short time later. However, this is not true for event-driven reporting: that report would only be sent once, since the aim was to reduce signaling overhead. Defining a reporting format without retransmissions does not seem like a good idea.

As mentioned, an efficient solution for event-driven reporting must consider all three aspects: event definition, request of UL resources and report format/content. Deriving optimized solutions for all three aspects in one release would seem impossible. Also, designing an optimized solution for one of the aspects while leaving the other aspects for a later release would most likely not lead to a functional solution. The remaining option is to use simplistic options for some of the aspects, while focusing the design effort on only one part:

A solution for event-driven UE reporting would rely on simplistic solutions for some aspects while deriving workable solutions for some aspects.

To be more specific: when designing a solution for event-driven reporting, the following approach could be used:

- A simplistic event is assumed: this could be polling by the NW, or a timer.

- No definition of a dedicated mechanism to request UL resources.

- Define a reporting format that can be applied to the simplistic mechanism described under bullet 1 and 2, but also to future extensions of 1 and 2.

The bulk of the work would thus be to define the report format and content. Note that the design would have to take into account that the report would be sent in response to an event, which means that the report would only be sent once.

## 3.2	UL-only nodes

UL performance is becoming a bottleneck mainly due to the reason that the UE has lower transmit power and fewer transmit antennas, and the widespread commercial network uses DL-heavy TDD patterns. One solution to improve UL performance is densification of network deployment. To simplify densification, UL-only nodes have been proposed. As such node would not be capable of any DL transmission, it may reduce the hardware cost.

Note that the cost of a site is dominated by factors other than hardware cost. For example, the site rental cost and cost of transport would be the same for an UL-only node as for a normal node. So, in the end, the lower hardware cost may not lead to a significant reduction in total site cost:

As the site cost is not dominated by the node cost, the total cost for an UL-only site may be similar to the total cost for a DL/UL site.

However, there may be other reasons why an UL-only node would be beneficial. An UL-only node will probably have lower energy consumption than a DL/UL node. Also, from a regulatory point of view, an UL-only node may be easier to deploy.

An additional point to note is that UL-only nodes may be deployed without changing the standard: there is nothing that prevents the NW deploying nodes without DL transmissions. The UL reception in such nodes would be somewhat suboptimal, since power control and timing advance would be performed with respect to the DL/UL node:

The NW can already today deploy UL-only nodes, but UL performance will be suboptimal.

In our understanding, the RAN1 specification efforts to provide an optimized support for UL-only nodes are small. The unified TCI state in Rel-17, LTM, and the 2TA feature in Rel-18 provides a good foundation for the required enhancements. As long as the functionality is based on these building blocks, the effort should be limited. However, the experience from the 2TA work teaches us that work on improving the RACH procedures can be quite time consuming, and this must be avoided.

Furthermore, it is important to avoid any impact on RAN2 (user plane) and RAN3 (intra-RAN signalling). The work must be confined to RAN1, and potentially RAN4.

## Simplifying 5-8 layer DL MIMO implementation

- In downlink, up to 8 layer MIMO reception in the UE has been supported in specifications since LTE Rel.10, but there has not been wide commercial support in UE side for LTE or NR. Now with increasing demands on spectral efficiency, higher order MIMO start to draw larger interest, but it is costly and complicated to implement such receiver in a UE. There is some industry discussion to simplify UE implementation to make it more commercially attractive and we believe this is a good direction to meet the operator’s demands.

It is beneficial to support enhancements enabling lower complexity 8 antenna UE receivers with up to 8-layer DL transmission.

# 4	Conclusion

The coverage of the new frequencies in the higher FR1 bands is challenging. At the same time, with the smaller wavelengths in these bands, the number of antenna elements can be increased without increasing the physical antenna size. However, for antennas with hundreds of antenna elements, the current support of up to 32 antenna ports is insufficient: the angular coverage of the subarray becomes small, and due to the lack of steerability, the performance will be degraded.

In many deployments, a large part of the UEs can transmit with two or more layers. This means that the performance of UL multi-layer transmission is still below potential, since only CP-OFDM can be used for multi-layer transmission.

Narrowband SRS transmissions lead to a capacity loss since they effectively block the entire carrier. In FR2, the PUSCH allocation must be continuous in the frequency domain, and in FR1, the intra-slot frequency hopping makes it impossible to reuse the remainder of the carrier.

Current UL transmission schemes will cause interference in neighboring cells. There is no possibility to avoid transmitting towards neighboring base stations, since all the UL transmission schemes are aiming to improve the received signal strength at the targeted cell.

The CJTs specified in Rel-18 relies on ideal frequency and time synchronization which is clearly unrealistic. Let’s go for realistic CJT!

In FR2 deployments with many carriers and SSB beams, the number of TRSs that can be RRC configured to the UE is not large enough, leading to undesirable RRC configurations to handle intra-cell UE movement.

To instruct a UE to transmit SRS in a certain direction, MAC CE signaling is required. Since a common scenario is to just transmit one probing SRS, using MAC CE is inappropriate.

The current link adaptation is based on measurements of sparse CSI-RS transmissions. This makes link adaptation sensitive to interference variations since the reception quality at the CSI-RS transmissions may not be representative of the reception quality during the PDSCH transmissions. Using HARQ retransmissions to adjust the effective coding rate is too slow, due to the feedback delay, which makes HARQ ineffective for new services, such as XR.

UE-initiated reporting on L1/L2 can be used for many purposes. Specifying such a scheme involves several components, some of which have not been specified by RAN1 earlier. If it is agreed to specify event-driven reporting in Rel-19, it is crucial to limit the scope to avoid excessive workload.

UL-only nodes can be used to enhance UL performance using nodes that consume less energy. Provided that there is no impact to the user plane or RAN architecture, specifying support for UL-only nodes could be considered.

It is beneficial to support enhancements enabling lower complexity 8 antenna UE receivers with up to 8-layer DL transmission.

Based on the above motivations, this list of enhancements are proposed for Rel.19 MIMO from our perspective

- Introduce support for NZP CSI-RS and CSI reporting for large antenna arrays with 8, 12 and 16 columns including simultaneous vertical and horizontal (FD-MIMO) precoding:

  - Extension of NZP CSI-RS resource beyond 32 ports (48, 64, 72, 96, 128 ports)

  - Extension of the PMI reporting/codebook for the associated extended NZP CSI-RS resource including support for FD-MIMO, i.e., (N1,N2)={(8,3),(8,4),(12,2),(12,3),(12,4),(16,2),(16,3),(16,4)}

  - Enhancement in specifications for hybrid beamforming implementations (e.g., time domain elevation beamforming and frequency domain horizontal precoding) to support 8,12 and 16 antenna columns

- Further enhanced uplink coverage and capacity by specifying

  - Support for multi-layer DFT-S-OFDM for PUSCH and

  - Support for PUSCH rate matching around SRS transmissions of another user

  - Support for closed loop antenna selection for PUSCH

- Support UE reporting of delay difference(s) and frequency difference(s) between TRP motivated by C-JT in non-ideally synchronized networks

- Support dynamic switching of QCL source for TRS and enhance the DCI-based spatial relation/TCI state indication for SRS

- Support UE reporting of information related to SRS power imbalance

- Study, and if beneficial, specify CQI reporting based on actual received PDSCH quality (e.g., PDSCH DMRS or PDSCH decoding based CQI)

# 5	References

R1-2208402, Discussion on capacity enhancements for XR, Ericsson, RAN1#110bis-e, October 2022

3GPP TS 38.211, NR; Physical channels and modulation, RAN, V17.4.0, RAN1#99, March 2023

R1-2008419, “PUSCH coverage enhancement”, Ericsson, RAN1#103-e

R1-2102745, CSI Feedback Enhancements for IIoTURLLC, Ericsson, RAN1#104bis-e

RWS-230154, Discussion on Release 19 MIMO enhancements, Ericsson, RAN Rel-19 WS, Taipei, June 2023