3GPP TSG RAN Meeting #101	                                            			    RP- 231752

Bengaluru, India, September 11-15, 2023

Agenda Item:	9.2.2

Source:	Apple

Title:	Views and consideration on Ambient IoT

Document for:	Discussion/Decision

# Introduction

In RAN#97-e, a RAN SID for ambient power enabled IoT has been approved and further revised in RAN#98-e with following objectives below in [1]. In RAN#100, the study item is further extended by one quarter [2].

This study targets at a new 3GPP IoT technology, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The study shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power.

In terms of energy storage, the study will consider the following device characteristics:

•	Pure batteryless devices with no energy storage capability at all, and completely dependent on the availability of an external source of energy

•	Devices with limited energy storage capability that do not need to be replaced or recharged manually.

Device categorization based on corresponding characteristics (e.g. energy source, energy storage capability, passive/active transmission, etc.) may be discussed during the study, in relation with the relevant use cases.

The device’s peak power consumption shall be limited by its practical form factor for the intended use cases, and shall consider its energy source.

  - Identify the suitable deployment scenarios and their characteristics, at least for the use cases/services agreed in SA1’s “Study on Ambient power-enabled internet of Things”, comprising among at least the following aspects

    - Indoor/outdoor environment

    - Basestation characteristics, e.g. macro/micro/pico cells-based deployments

    - Connectivity topologies, including which node(s) , e.g. basestation, UE, relay, repeater, etc. can communicate with target devices

    - TDD/FDD, and frequency bands in licensed or unlicensed spectrum

    - Coexistence with UEs and infrastructure in frequency bands for existing 3GPP technologies

    - Device originated and/or device terminated traffic assumption

NOTE: There can be more than one deployment scenario identified for a use case, and a deployment scenario may be common to more than one use case.

NOTE: Where more than one deployment scenario is identified for a use case, the trade-offs between them should also be studied.

NOTE: The study shall not prioritize deployment aspects that should be coordinated with SA, e.g. public or private network, with or without CN connection.

NOTE: A representative use case can be studied for a group of use cases that have similar requirements.

  - Formulate a set of RAN design targets based on the identified deployment scenarios and their characteristics for the relevant use cases, at least including

      - Power consumption

      - Complexity

      - Coverage

      - Data rate

      - Positioning accuracy

NOTE: The requirements from SA1 on the relevant use cases shall be taken into consideration.

NOTE: The study shall aim to provide better coverage compared to existing non-3GPP technologies for the relevant use cases.

NOTE: Other RAN design targets in relation to connection density, mobility, security, latency, reliability etc. may be discussed, if necessary for the relevant use cases.

NOTE: Detailed definitions of the RAN design targets should be discussed during the study.

  - Compare and assess the feasibility of meeting the design targets for relevant use case on the basis of the deployment scenario(s) appropriate to it, and identify assumptions on required functionality to be supported.

NOTE: This is not to require a detailed WG-level of analysis.

Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies.

In this contribution, we provide our views on remaining aspects related to design targets for ambient IoT. In addition, we also share our views on some key recommendations of this study for further potential study at WG level in Rel-19.

# Discussion

For design target consideration, power consumption requirement is one of the critical aspects. For the power consumption target, currently 2 options are considered for device A including  ≤ 10 μW or ≤ 1 μW. In our view, to be comparable with RFID devices, ≤ 1 μW should be considered. However, achieving such low power consumption may require quite detailed analysis and discussion. This would be more suited to study in a potential new study in WGs. For this RAN study, we can keep both the options. Similarly, for device B, there are 2 options under consideration including device A ≪ device B or device A ≤ device B. In our view, this will depend on the target power consumption value for device A. If ≤ 10 μW is selected for device A, then Device A ≤ Device B should be selected, otherwise if  ≤ 1 μW is selected for device A, then Device A ≪ Device B should be selected.

Proposal 1: For device design target for power consumption, both options can be included in the TR for device A, i.e. power consumption target during transmitting/receiving is  ≤ 10 μW and  ≤ 1 μW

Proposal 2: For device design target for power consumption, depending up on what option can be is selected for device A, corresponding selection of the power consumption comparison between device A and device B can be made:

- If device A power consumption ≤ 1 μW is selected, then Device A ≪ Device B < Device C

- If device A power consumption ≤ 10 μW is selected, then Device A ≤ Device B < Device C

For other design targets including coverage, data rate, maximum TB size, latency, positioning accuracy, connection/device density and device speed, the baseline requirements should be based on the representative use-cases. Furthermore, mapping of specific design target requirements to device types might be needed. However, considering the limiting time and scope of RAN study, we think that more detailed design requirements values and corresponding mapping to device types should be handled in potential study item in the WGs, based on prioritization of use-cases.

Proposal 3: For design targets including coverage, data rate, maximum TB size, latency, positioning accuracy, connection/device density and device speed, the baseline requirements should be based on the representative use-cases

Proposal 4: For specific device types, corresponding design target values is needed

- this could be more appropriate for WGs level study based on prioritization of use-cases

On the device speed, there is an FFS whether absolute or relative speed is considered. In our view, this could depend on which topology is applied. For connectivity topology (4), since UE bases reader is used and it could be mobile, therefore, design target speed could be relative. For other topologies where BS based reader is used, absolute speed should be considered.

Proposal 5: For device speed design target, consider relative speed at least for topology (4)

- for other topologies including (1), (2) and (3), only absolute speed can be considered

In RAN study for ambient IoT, a superset of use-cases, deployment scenarios, connectivity topologies, device types and design targets have been considered. For further study and potential work on ambient IoT at WG level, a more focused approach should be considered. It is not realistic to consider all the aspects and corresponding values/types for WG level study and work. Therefore, in our view, main recommendation from this RAN study to suggest only 1 set of 1-2 use-cases for further consideration in Rel-19 WG level study. Based on the recommended use-cases, other aspects such as deployment scenarios, connectivity topologies, device types can be determined. Furthermore, this would allow to determine a clear design target requirement.

In our view, prioritization of use-cases should be based on current market needs that may not be supported optimally by existing LPWA devices. From this point of view, we think inventory/logistics and sensor use-cases can be recommended. Based on these use-cases, device types to be recommended can be further determined. For inventory/logistics use-case, typically only device terminated traffic type should be sufficient to be supported, i.e. the ambient IoT device will primarily responds to the signal from the reader. Therefore, from this point of view device type A should be recommended. However, for inventory/logistics management in outdoors, the coverage requirement are typically a few couple of hundreds of meters. Such requirement might be challenging to be supported with device type A as there is no amplification of the signal. Therefore, considering such coverage requirement device type B would be needed as well. For the other use-case of sensor-based devices, supporting only device terminated traffic may not suffice. For sensor-based devices, it is reasonable to assume that the device is periodically monitoring and transmitting the data corresponding to the sensory function. Or depending on critical measurements, the device should be able to trigger to the reader. This basically would require the support of device originated traffic. Therefore, for supporting device originated traffic, device type C would be needed.

Proposal 6: RAN should recommend only a subset of 1-2 uses-cases for further consideration in Rel-19 for a potential WG level study to have a focused scope

- Inventory/logistics and sensor use-cases could be the recommended to be supported by RAN WG level study and potential normative work

Proposal 7: RAN should recommend device type C for further consideration in Rel-19 for a potential WG level study to have a focused scope (considering both inventory/logistics and sensor use-cases)

- However, if only inventory/logistics use-cases are recommended, then RAN can recommend device type A&B

# Conclusion

In this contribution, we have provided following proposals on the following remaining aspects for ambient IoT:

Proposal 1: For device design target for power consumption, both options can be included in the TR for device A, i.e. power consumption target during transmitting/receiving is  ≤ 10 μW and  ≤ 1 μW

Proposal 2: For device design target for power consumption, depending up on what option can be is selected for device A, corresponding selection of the power consumption comparison between device A and device B can be made:

- If device A power consumption ≤ 1 μW is selected, then Device A ≪ Device B < Device C

- If device A power consumption ≤ 10 μW is selected, then Device A ≤ Device B < Device C

Proposal 3: For design targets including coverage, data rate, maximum TB size, latency, positioning accuracy, connection/device density and device speed, the baseline requirements should be based on the representative use-cases

Proposal 4: For specific device types, corresponding design target values is needed

- this could be more appropriate for WGs level study based on prioritization of use-cases

Proposal 5: For device speed design target, consider relative speed at least for topology (4)

- for other topologies including (1), (2) and (3), only absolute speed can be considered

Proposal 6: RAN should recommend only a subset of 1-2 uses-cases for further consideration in Rel-19 for a potential WG level study to have a focused scope

- Inventory/logistics and sensor use-cases could be the recommended to be supported by RAN WG level study and potential normative work

Proposal 7: RAN should recommend device type C for further consideration in Rel-19 for a potential WG level study to have a focused scope (considering both inventory/logistics and sensor use-cases)

- However, if only inventory/logistics use-cases are recommended, then RAN can recommend device type A&B

# Reference

- 3GPP RP-223396, “SID revised: Study on Ambient IoT”, Huawei, HiSilicon

- 3GPP RAN#100, “Meeting Report (draft)”

- 3GPP RAN#99-e, “Meeting Report”

- 3GPP TR 22.840 V0.2.0, “Study on Ambient power-enabled Internet of Things (Release 19)”