3GPP TSG RAN meeting #101										RP-231756

Bangalore, India, September 11-15, 2023

## Status Report to TSG

Agenda item:			9.3.4.15

| WI / SI Name | Channel raster enhancement |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_channel_raster_enh |  |  |  |  |
| Unique ID | 991045 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-231469 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 50%: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | R4 |
| --- | --- | --- |
| Rapporteur | Name | Christian Bergljung |
|  | Company | Ericsson |
|  | Email | Christian.Bergljung@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#106bis

The work plan was agreed in R4-2306599.

In the work plan for this meeting:

- Discussion of necessary changes of the channel raster for the BS and UE and the applicability of the channel raster entries for support of functionality specified in RAN1 and RAN2 specifications

- Discussion of the need for UE capability to indicate support of configuration of a narrower UE channel BW inside a wider gNB channel BW in all cases

Agreements:

A WF listing candidate solutions was agreed in R4-2306598.

RAN4#107

In the work plan for this meeting:

- the applicability of the more flexible frequency location also to [the carrier resource grid indicated in] SIB1 or not

  - the need for an additional channel raster and if so, its granularity,

- Decision on the need for a UE capability to indicate support of configuration of a narrower UE channel BW inside a wider gNB channel BW in all cases and channel raster enhancements

- LS to RAN2 on a UE capability if needed

Agreements:

A WF was agreed in R4-2310269 listing the following agreement

- There is no backwards compatibility issue, the carrierBandwidth advertised in SIB1 does not have to be placed on the 100kHz raster.

and the following candidate solutions for further study (a shorter list than that in R4-2306598)

1- 	Approach 1: Specify a new channel raster

1-	New channel raster step size:

Option 1: 5 kHz

Option 2: 10 kHz

Option 3: 50kHz

2-	The new channel raster should be specified for:

both UE and gNB.

3-	For which bands this new channel raster should be specified:

All FR1 bands below 3GHz that that currently have 100 kHz channel raster

2-	Approach 2: Do not specify new channel raster entries

- Alternative 1

1-	Clarify in clause 5.4.2.2 of both the BS and UE specifications that the “RF channel” is mapped to the channel raster at the centre of a carrier grid of a serving cell for at least one numerology as advertised in SIB1.

2-	The network should be able to use the RRC specification for configuring the UE with locations of the UE-specific channel BW within a wider cell-specific bandwidth;

- Alternative 3:

1-	For operating bands with a 100 kHz channel raster, the UE can signal a capability to support a UE specific channel BW that

• consists of a contiguous subset of RBs from SCS-SpecificCarrier in SIB1 and

• is a maximum transmission BW configuration

• but need not be centered on the channel raster.

2-	For UEs with the capability to support a	 UE specific channel BW off the 100 kHz raster in corresponding operating bands, the natural raster for the UE specific channel BW is the RB grid of the carrier bandwidth in SIB1. (For a given numerology and location of the SIB1 carrier bandwidth, its RB grid is considerably sparser than the proposed channel rasters and it includes only valid frequency locations, hence rather the RB grid of the carrier bandwidth in SIB1 should be specified as raster for the UE specific channel BW than a new channel raster.)

Way forward:

For the next meeting, companies are encouraged to detail the expected specification’ updates of their preferred approach.

RAN4#108

A WF was agreed in R4-2314681 listing the following approaches

One of the following approaches/alternatives to be chosen:

1-	Approach 1: Specify a new channel raster

The new channel raster step size: 10 kHz

2-	Approach 2: Do not specify new channel raster entries

- Alternative 1 [the second sub-bullet modified as compared to the WF from RAN4#107]

- Clarify in clause 5.4.2.2 of both the BS and UE specifications that the “RF channel” is mapped to the channel raster at the centre of a carrier grid of a serving cell for at least one numerology as advertised in SIB1.

- The network should be able to use the RRC specification for configuring the UE with locations of the UE-specific channel BW within a wider cell-specific bandwidth subject to UE capability; a subset of requirements applies for the UE-specific CHBW within a wider carrier

- Alternative 3: [from RAN4#107]

- For operating bands with a 100 kHz channel raster, the UE can signal a capability to support a UE specific channel BW that

•	consists of a contiguous subset of RBs from SCS-SpecificCarrier in SIB1 and

•	is a maximum transmission BW configuration

•	but need not be centered on the channel raster.

- For UEs with the capability to support a UE specific channel BW off the 100 kHz raster in corresponding operating bands, the natural raster for the UE specific channel BW is the RB grid of the carrier bandwidth in SIB1. (For a given numerology and location of the SIB1 carrier bandwidth, its RB grid is considerably sparser than the proposed channel rasters and it includes only valid frequency locations, hence rather the RB grid of the carrier bandwidth in SIB1 should be specified as raster for the UE specific channel BW than a new channel raster.)

the same as for RAN#107 but with only one raster candidate granularity for Approach 1 and the following agreement on a UE capability (notwithstanding the approach selected):

Agreement:

A new UE capability will be specified to support the WI objectives

Open issues:

- The new UE capability should be per band.

- FFS from which release should the UE capability be applicable.

- Whether the capability should (at least for some bands) be mandatory from Rel-18 onwards.

Way forward

For the next meeting, companies are encouraged to comment on open issues and/or to detail the expected specification updates of their preferred approach.

The progress is almost in line with the original work plan for RAN4#107.

#### 2.4.2	Remaining Open issues

All objectives of the WI

- Specify necessary changes to the UE channel raster such that configuring a narrower UE channel BW inside a wider gNB channel BW is always possible [RAN4].

- Changes to BS channel raster can be considered if required [RAN4].

- specify the corresponding UE capability, if needed, to enable changes to the channel raster [RAN2,RAN4]

are open but

- agreement on the introduction of a UE capability to support the WI objectives

- fewer options for the two approaches considered

The changes made to TN will also be made to NTN.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## References

RAN4#106bis

[1]	R4-2304462, Work plan for the WI on channel raster enhancement, Ericsson

[2]	R4-2306599, Work plan for the WI on channel raster enhancement, Ericsson

[3]	R4-2304262, Views on the UE channel raster for FR1, Intel Corporation

[4]	R4-2304263, Clarification of UE capability enhancement of channel raster in bands below 3GHz, Intel Corporation

[5]	R4-2304264, Clarification of UE capability enhancement of channel raster in bands below 3GHz, Intel Corporation

[6]	R4-2304265, Clarification of UE capability enhancement of channel raster in bands below 3GHz, Intel Corporation

[7]	R4-2304266, Clarification of UE capability enhancement of channel raster in bands below 3GHz, Intel Corporation

[8]	R4-2304334, Initial considerations for the flexible channel raster, Apple

[9]	R4-2304335, 	Flexible channel raster for the NTN bands, Apple

[10]	R4-2304463, On changing the BS and UE channel raster, Ericsson

[11]	R4-2304653, Discussion on UE and BS channel raster, CMCC

[12]	R4-2304664, Discussion on channel raster for SIB1 CBW and UE-specific CBW, China Telecom

[13]	R4-2304876, Discussion on channel raster enhancement, Nokia, Nokia Shanghai Bell

[14]	R4-2305308, Discussion on Channel raster enhancement, Huawei, HiSilicon

[15]	R4-2305353, Discussion on channel raster enhancement, ZTE Corporation

[16]	R4-2305517, Channel Raster Enhancements, Qualcomm Incorporated

[17]	R4-2305585, Channel raster enhancement, Samsung Electronics Co., Ltd

[18]	R4-2305702, Consideration on channel raster enhancement, MediaTek Inc.

[19]	R4-2305828, 10 kHz raster for bands that currently specify the 100 kHz raster, T-Mobile USA

[20]	R4-2304336, UE capability for the flexible channel raster, Apple

[21]	R4-2304464, On a UE capability for channel raster enhancement, Ericsson

[22]	R4-2304654, Discussion on UE capability of new channel raster, CMCC

[23]	R4-2304877, Discussion on UE capability for channel raster enhancement, Nokia, Nokia Shanghai Bell

[24]	R4-2305560, UE capability for channel raster enhancement, Huawei, HiSilicon

[25]	R4-2305703, Consideration on enhanced channel raster capability, MediaTek Inc.

[26]	R4-2306205, Topic summary for [106-bis-e][126] NR_channel_raster_enh, Moderator (Ericsson)

[27]	R4-2306288, Topic summary for [106-bis-e][126] NR_channel_raster_enh, Moderator (Ericsson)

[28]	R4-2306598, WF on channel raster enhancement, Ericsson

RAN4#107

[29]	R4-2308786, Channel Raster Enhancements Scope, Qualcomm Incorporated

[30]	R4-2307501, Discussion on channel raster enhancement, Nokia, Nokia Shanghai Bell

[31]	R4-2307580, Discussion on UE and BS channel raster, CMCC

[32]	R4-2307737, Options for enhancing the channel raster, Ericsson

[33]	R4-2307763, Discussion on Channel raster enhancement, Huawei, HiSilicon

[34]	R4-2307847, Discussion on UE and BS channel raster, ZTE Corporation

[35]	R4-2307976, Discussion on channel raster for SIB1 CBW and UE-specific CBW, China Telecom

[36]	R4-2308385, Further discussion on NR channel raster enhancement, MediaTek Inc.

[37]	R4-2308787, Channel Raster Enhancements, Qualcomm Incorporated

[38]	R4-2309049, Further considerations for the flexible channel raster, Apple

[39]	R4-2309711, Proposal for the 100 kHz raster issue, T-Mobile USA, TELUS

[40]	R4-2309712, Input on the Way Forward on channel raster enhancements, T-Mobile USA

[41]	R4-2307738, Clarification of the channel raster and capability for UE-specific channel bandwidth, Ericsson

[42]	R4-2307739, Clarification of the channel raster, Ericsson

[43]	R4-2307740, Clarification of the channel raster, Ericsson

[44]	R4-2307502, Discussion on UE capability for channel raster enhancement, Nokia, Nokia Shanghai Bell

[45]	R4-2307581, Discussion on UE capability of new channel raster, CMCC

[46]	R4-2307764, UE capability for channel raster enhancement, Huawei, HiSilicon

[47]	R4-2307848, Discussion on UE capability, ZTE Corporation

[48]	R4-2308386, Further discussion on UE capability for channel raster enhancement, MediaTek Inc.

[49]	R4-2309050, UE capability for the flexible channel raster, Apple

[50]	R4-2307741, Draft LS on a capability for UE-specific channel bandwidth location, Ericsson

[51]	R4-2310012, Topic summary for [107][129] NR_channel_raster_enh, Moderator (Ericsson)

[52]	R4-2310269, WF on NR channel raster enhancement, Ericsson

RAN4#108

[53]	R4-2313503, Discussion on NR channel raster enhancement. Google Inc.

[54]	R4-2311067, Discussion on channel raster for SIB1 CBW and UE-specific CBW, China Telecom

[55]	R4-2311220, Further considerations for the flexible channel raster, Apple

[56]	R4-2311819, Discussion on UE and BS channel raster, CMCC

[57]	R4-2312020, Discussion on UE and BS channel raster, ZTE Corporation

[58]	R4-2312163, More on the channel raster enhancement, Ericsson

[59]	R4-2312314, Discussion on channel raster enhancement, Huawei, HiSilicon

[60]	R4-2312439, Channel raster enhancements, Qualcomm Incorporated

[61]	R4-2312525, Discussion on channel raster enhancement, Nokia, Nokia Shanghai Bell

[62]	R4-2313616, On channel raster enhancement for TN, MediaTek Inc.

[63]	R4-2312164, Clarification of the channel raster and capability for UE-specific channel bandwidth, Ericsson

[64]	R4-2312165, Clarification of the channel raster, Ericsson

[65]	R4-2312166, Clarification of the channel raster, Ericsson

[66]	R4-2312315, Draft CR to introduce 10 kHz channel raster in Rel-18, Huawei, HiSilicon

[67]	R4-2311221, Flexible channel raster for NTN, Apple, Globalstar, Ligado Networks, Inmarsat, Hughes/Echostar

[68]	R4-2311820, Discussion on NTN channel raster, CMCC

[69]	R4-2313617, On channel raster enhancement for NTN, MediaTek Inc.

[70]	R4-2312167, Clarification of the channel raster and capability for UE-specific channel bandwidth, Ericsson

[71]	R4-2311821, Discussion on UE capability of new channel raster, CMCC

[72]	R4-2312021, Discussion on UE capability, ZTE Corporation

[73]	R4-2312316, UE capability for channel raster enhancement, Huawei, HiSilicon

[74]	R4-2312440, Channel raster enhancement capability, Qualcomm Incorporated

[75]	R4-2312526, Discussion on UE capability for channel raster enhancement, Nokia, Nokia Shanghai Bell

[76]	R4-2313618, UE capability for channel raster enhancement, MediaTek Inc.

[77]	R4-2312168, Draft LS to RAN2 on a capability for UE-specific channel bandwidth location, Ericsson

[78]	R4-2314211, Topic summary for [108][129] NR_channel_raster_enh, Moderator (Ericsson)

[79]	R4-2314681, WF on channel raster enhancement, Ericsson

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template