3GPP TSG RAN #101                                    		                                       RP-231772

Bangalore, India, September 11st – 15th, 2023

Source:	OPPO

Title:	Discussion on MIMO enhancement for Rel-19

Agenda Item:	8A.2.2

Document for:	Discussion

# Introduction

In this paper we provide our views on the MIMO enhancement in Rel-19.

# Discussions

## Support 3Tx UL MIMO

It is desired that the NR system can provide various types of services. Some types of services require very heavy UL transmission. While those service require heavy UL transmission, the uplink transmission in NR system has always been restricted by limited coverage and limited throughput. We have noticed that boosting the UL throughput has been attracting interests from many companies [1]. However, in reality, the commercial handheld UEs, for example the smart phones in the current market, are generally restricted by 2 Tx chains. Therefore, the UL throughput of commercial handheld UEs is limited by that. Even though the NR specification supports 4 Tx chain or even more, 4 Tx chains are not likely to be applied in the commercial UEs in the near future due to various commercial factors, including the PA cost and limited size of commercial cell phones.

On the other hand, due to the successful evolution of hardware, the advanced smartphone is capable  of supporting 3 Tx chains in one same frequency band, which enables UE to boost the UL throughput through using one more chain than 2 Tx chains. Compared with the UE with 2 Tx chains, the UE with 3 Tx chains can provide significant gain in UL throughput (up to 50% gain in peak throughput). That can provide much better user experience for UL-heavy services.

Figure 1: The evolution of hardware in smartphone:  supporting 3 Tx chains in one band

We conducted simulation to prove the performance gain of 3 Tx chains for UL-MIMO.  Please check the Appendix for the simulation assumptions. The throughput performance results are shown in Figs 2 and 3, respectively. It can be observed that for both codebook-based PUSCH and non-codebook based PUSCH, the 3Tx UL-MIMO can provide ~50% throughput gain over the 2 Tx UL-MIMO transmission.

Figure 2: Performance of 3 Tx UL MIMO of non-codebook based

Figure 3: Performance of 3 Tx UL MIMO of codebook based

To support the 3Tx UL MIMO, we propose the following enhancements:

- Specify 3-port codebook.  A 3-port codebook shall be specified to support 3 Tx chains in one band.

- Specify SRS resource for 3Tx. In current specification, the SRS resource can only have 1, 2, 4 or 8 port. To support 3 Tx chain UE, a SRS resource with 3 port is needed for training the uplink channel.

- We also need to consider the corresponding UE capability and RAN4 requirements.

Proposal 1: In rel-19, specify signaling/mechanism to enable 3 Tx UL MIMO transmission, including

- 3-port codebook

- 3-port SRS resource

- Corresponding UE capability and RAN4 requirements

## Enhancement on multi-TA in mTRP system

In Rel-18, the function of multi-TA for M-DCI mTRP system is supported. The TA acquisition mechanism specified in Rel-18 relies on the RACH procedures. For both inter-cell and intra-cell mTRP, two TA groups are associated with one serving cell. In RACH-based TA acquisition mechanism, NW needs to reserve and pre-configure the RACH resources for each involved TRP including those with inactive PCIs. That would cost additional UL resources in most of the scenarios. In some sense, it could also impair the UL performance due to the extra overhead of RACH resources for the other TRPs.

To resolve this issue, one solution is RACH-less TA acquisition. In RACH-less TA acquisition approach, UE measures the difference of DL timing reference between reference TRP (e.g. serving cell) and TA-target TRPs (e.g. non-serving cell(s) with different PCIs from serving cell for the case of inter-cell mTRP). Then the UE is able to calculate the TA of the target TRP. Surely, one precondition is that UE already obtains the TA of the reference TRP.

Another issue of the multi-TA mTRP in Rel-18 relates to the framework of spatial relation. The feature of multi-TA for mTRP lacks off the support to legacy UL spatial relation in Rel.15/16. It is solely based on the unified TCI framework that was specified in Rel-17 and Rel-18. That would cause difficulty to the operation of commercial NWs and UEs. Specifically, a UE that supports the function of multi-TA mTRP would have to support the unified TCI framework as a prerequisite. But as we know, most of the current deployed NW implements the Rel.15/16 UL spatial relation framework, i.e., the legacy UL multi-beam operation.

With above being said, we propose to resolve those issues in Rel-19.

Proposal 2: In Rel-19, enhance the function of multi-TA for mTRP transmission:

- Specify RACH-less TA acquisition based on the measurements of DL timing reference of different TRPs.

- Specify multiple TAs for multiple TRPs associated with legacy Rel-15/16 spatial relation framework

## Enhancement on uplink multi-panel transmission

Multi-panel UL simultaneous transmission is specified in Rel-18. However, the design in Rel-18 assumes symmetric panels in the UE. It assumes the two panels in one UE have the same number of ports, the same type coherence capability, the same maximum number of layers and same type of full power transmission. However, multi-panel UL simultaneous transmission is mainly targeted for CPE/FWA/vehicles type UEs. Asymmetric panels are envisioned for CPE/FWA/vehicles type UEs. To effectively support those type UEs, we shall consider to enhance the multi-panel transmission to support asymmetric panels.

Rel-18 specifies the transmission of PUSCH+PUSCH in multi-DCI based multiple TRP system. That can significantly improve the uplink throughput since now the UE can transmit two PUSCHs simultaneously. However, the cases of PUSCH+PUCCH and PUCCH+PUCCH are still not supported. That would restrict the flexibility of system operation and limit the system throughput/latency enhancement. Due to this restriction, the system has to be very careful on uplink scheduling and that could impair system uplink performance. To resolve this issue, we propose to enhance the multi-panel transmission in multi-DCI based system to support the transmission of PUSCH+PUCCH and PUCCH+PUCCH.

Proposal 3: In Rel-19, enhance the multi-panel uplink transmission:

- Extend the single-DCI based multi-panel uplink transmission and multi-DCI based multi-panel uplink transmission to UEs with asymmetric panels

- Specify PUCCH+PUCCH multi-panel transmission in multi-DCI based multi-TRP system

- Specify PUSCH+PUCCH multi-panel transmission in multi-DCI based multi-TRP system

## Enhancement on multi-beam operation

Multi-beam operation was specified in Rel-15 and then in the following Rel-16/17/18, we continue to enhance multi-beam operation. However, up to now, all the beam reporting and beam switch/update specified in NR up to Rel-18 are NW-initiated methods. The system configures/requests the UE to measure some beams and then report the best beams. The system indicates the UE to switch to some beams for DL reception and/or UL transmission. The main problem of NW-initiated beam reporting and beam switch is that it would result in latency and signaling overhead. The reason is that UE is always the first one who know the variation of beams and the change in the best beam but the NW does not know until the UE reports the measurement result.

In Rel-18, we introduced the function of LTM. In LTM, for the L1-RSRP measurement and reporting, the UE would be required to measure and report much more beams than the L1-RSRP measurement/reporting of intra-cell multi-beam operation. Due to that, we would face more severe issues of the latency and signaling overhead of this NW-initiated beam reporting and beam switch.

To resolve the issue, we propose to specify UE-initiated beam reporting and beam switch. The UE can trigger the beam reporting when it finds the beam variation is large enough. That can reduce the UL signaling overhead because the UE only reports beam measurement results only when it is necessary. That can also reduce the beam reporting latency because the UE can report the measurement result immediately whenever it is needed. The UE-initiated beam switch can reduce the latency of beam switch and also reduce the signaling overhead of beam switch. Furthermore, UE-initiated beam switch can also reduce the possibility of beam link failure since the UE always knows which beam is the best. Therefore, we propose to specify UE-initiated beam reporting and UE-initiated beam switch/update.

Proposal 4: In Rel-19, Specify signaling/mechanism to facilitate UE-initiated beam management procedure, including UE-initiated beam reporting and UE-initiated beam switch/update

## Other Enhancements

In addition to our proposed objectives and topics in previous section, we also consider the following enhancements.

The first topic is enhancement to support UL-only TRP for UL-MIMO [2]. In NR system, DL and UL could have totally different coverage. For example, the macro-BS could have very large DL coverage but the UL coverage is very limited. In some deployment scenarios, UL-only TRRP can improve the UL coverage and then UL throughput with very low cost. That can also improve the system power efficiency. Due to that, we are open to consider the issue of UL-only TRP for UL-MIMO transmission.  In terms of the UL-MIMO transmission schemes, we are open to consider TRP-specific scheme, like DPS or some TRP joint receiption, for example non-coherent joint reception at multiple UL-only TRPs.

Proposal 5: Consider the enhancements for enabling UL-only TRP for UL-MIMO and advanced features for UL-distributed MIMO in Rel-19

Another topic is enhancement for CSI/PMI feedback to facilitate the channel-based beamforming [3]. In NR, we have introduced quite many enhancements on CSI/PMI feedback that can be used for the system to select baseband precoder, i.e., digital precoder. However, NR system also supports high frequency band and hybrid beamforming. How to obtain CSI of system with hybrid beamforming is always a challenge for high frequency band NR system. A mechanism that can obtain the CSI effectively and efficiently in the system with analog beamforming would be critical to the performance of such NR systems. Therefore, we are open to introduce some enhancement on CSI feedback for system with analog beamforming [3].

Proposal 6: Consider the following CSI enhancement in Rel-19:

- Specify CSI feedback to facilitate the channel-based beamforming

- Specify CSI-RS with antenna switching for CSI feedback for the UE to identify the raw channel for beamforming weight selection

# Conclusion

In this contribution, we present our views on the MIMO enhancement in Rel-19 and the following proposals are made:

Proposal 1: In rel-19, specify signaling/mechanism to enable 3 Tx UL MIMO transmission, including

- 3-port codebook

- 3-port SRS resource

- Corresponding UE capability and RAN4 requirements

Proposal 2: In Rel-19, enhance the function of multi-TA for mTRP transmission:

- Specify RACH-less TA acquisition based on the measurements of DL timing reference of different TRPs.

- Specify multiple TAs for multiple TRPs associated with legacy Rel-15/16 spatial relation framework

Proposal 3: In Rel-19, enhance the multi-panel uplink transmission:

- Extend the single-DCI based multi-panel uplink transmission and multi-DCI based multi-panel uplink transmission to UEs with asymmetric panels

- Specify PUCCH+PUCCH multi-panel transmission in multi-DCI based multi-TRP system

- Specify PUSCH+PUCCH multi-panel transmission in multi-DCI based multi-TRP system

Proposal 4: In Rel-19, Specify signaling/mechanism to facilitate UE-initiated beam management procedure, including UE-initiated beam reporting and UE-initiated beam switch/update

Proposal 5: Consider the enhancements for enabling UL-only TRP for UL-MIMO and advanced features for UL-distributed MIMO in Rel-19

Proposal 6: Consider the following CSI enhancement in Rel-19:

- Specify CSI feedback to facilitate the channel-based beamforming

- Specify CSI-RS with antenna switching for CSI feedback for the UE to identify the raw channel for beamforming weight selection

# Reference

- RP-211652   Moderator’s summary for discussion [RAN93e-R18Prep-02] UL enhancements

- RWS-230290 Views on Rel 19 MIMO evolution, ZTE, Sanechips

- RWS-230097 Views on Rel-19 MIMO Enhancement, Google

# Appendix

The simulation assumption for 3Tx UL MIMO is listed in the table below: