3GPP TSG RAN Meeting #101	RP- 231778 
Bangalore, India, September 11-15, 2023

Agenda Item:	8A.2.8

Source:	OPPO

Title:	Discussion on further enhancement for XR in Rel19

Document for:	Discussion, Decision

Introduction

This paper will share our view on R19 XR evolution.

Discussion

R19 new area

In TS 22.261, there are requirements to support coordinated transmission for multi-modality flows. In some cases, these multiple types of flows, e.g. video/audio stream, and haptic or sensor data for a more immersive experience are linked with/located at a single UE. In other cases, such multiple types of flows are associated with multiple UEs.

Figure 1. Multi-modal interactive system (in TS 22.261)

As described in TS 22.261, immersive multi-modal VR applications have critical synchronization requirements between different media components, in order to avoid having a negative impact on the user experience. The table below shows an example of synchronization requirements.

Table 1: Typical synchronization thresholds for immersive multi-modality VR applications (in TS 22.261)

| Media components | synchronization threshold (note 1) |  |
| --- | --- | --- |
| audio-tactile | audio delay: 50 ms | tactile delay: 25 ms |
| visual-tactile | visual delay: 15 ms | tactile delay: 50 ms |
| NOTE 1: for each media component, “delay” refers to the case where that media component is delayed compared to the other. |  |  |

To support the requirement of coordinated transmission for multi-modality flows, R18 SA2 targets the following objectives.

#1) Support of policy control enhancements to support multi-modality flows for single UE, based on AF provisioned information. #2) Support of policy control enhancements to support multi-modality flows for multiple UEs. - AF may indicate the multiple flows belong to the same multi-modal service for multiple UEs. - PCF generates related policies for each of the multiple UEs.

SA2 concludes to enhance QoS policies to implicitly implement the coordination/sync between UEs or QoS flows of a UE (mainly to handle inter-flow level sync), which does not have any RAN impact. However, in our view, the R18 solution could not implement accurate synchronization transmission between multi-modality flows, thus, we prefer RAN to have further study on how to support multi-modality services. In our view, the following aspects could be considered, including e.g. whether to have the per-packet level sync transmission between multi-modality flows, whether to know the sync information between multi-modality flows and what is the impact to scheduling/transmission in MAC/PDCP.

R19 XR work scope includes the multi-modality service-related objectives [RAN2, RAN3]

- Intra-UE synchronous transmission for multi-modal XR application

- Inter-UE synchronous transmission for multi-modal XR application

R18 XR progress and missed enhancements

Enhancements related to power saving [RAN2]

On R18 XR power saving aspects, RAN2 focuses on the work to handle non-integer XR traffic periodicity and finally agrees to introduce rational numbers in DRX cycle configuration. Meanwhile, the SFN warp-around issue is resolved by using a method similar to the one designed for CG in R16 IIoT. At this stage, the design of the fundamental functionality is already stable and only detailed issues are left. Thus, we think no further enhancement is to be pursued in R19.

Enhancements related to capacity [RAN1, RAN2, RAN3]

According to the RAN1 agreement in Rel-18, an XR UE transmits in CG PUSCH a UTO-UCI that provides gNB with information about whether a future CG PUSCH transmission occasion would be unused by the UE so that the gNB can release the corresponding TO resources to make it used by other UEs. When a DG PUSCH transmission overrides the CG PUSCH, as allowed in Rel-17, the UTO-UCI cannot be transmitted. To avoid such dropping of UTO-UCI, it was once proposed in RAN1 WI phase to allow UTO-UCI being transmitted instead on the overriding DG-PUSCH. Such proposal was left behind due to the desire to focusing on main functionality. We think the solution to handle the dropping of UTO-UCI due to DG-CG collision should be picked up in Rel-19.

Another missed enhancement from RAN1 perspective is the finer granularity of unused resource indication in UTO-UCI. What Rel-18 has is to indicate unused TO as a whole. That is to say, even if UE only needs to consume a small amount of RBs in a TO, it has to mark all configured RBs in the TO as “not unused”, which still leads to quite a bit waste of RB resources. In comparison, RB-level indication of unused RB resources was shown in simulations to be able to achieve about 17% of RB savings. The detailed simulation assumption and results are given in Appendix. Therefore we propose to consider an Rel-19 upgrade of UTO-UCI to have finer indication granularity (e.g., RB-level indication).

In R18, the specification supports the UE configuring with multiple CG PUSCH transmission occasions in a period of a single CG as well as reporting the unused CG PUSCH occasion(s) by using UTO-UCI. It requires the enhancement of the CG occasion determination, the HPI determination, the design of UTO-UCI, etc. In addition, R18 also covers the DG-related enhancement to improve the capacity, including supporting a new and static BS table for the quantization error reduction as well as the DSR reporting for timely scheduling.

Moreover, PDCP discard operation of PDU Set also falls in the scope of capacity enhancement because it can free up radio resources. Although the FEC-based discard is discussed in the SI phase, R18 eventually narrows that scope to PSI/PSIHI-based enhancement due to the lack of the SA2’s support. Considering SA2 already endorsed the SID of XRM ph2 which contains potential FEC enhancement [2], RAN2 could further consider the FEC-based discard in R19.

Enhancements for XR Awareness [RAN2, RAN3]

The concept of PDU set/data burst is introduced in SA2 in R18 as finer granularity to handle the XR service. To enable the PDU set/data burst-based operation, R18 concludes that the network and the UE need to identify the PDU set/data burst-related information. Also, the UE/SMF is responsible for providing XR assistance information to the gNB for the proper CG/power saving configuration. As no more information (except for FEC) is required for the XR service based on the SID of XRM ph2 [2], we can only focus on the FEC-related information.

Others

There are still some issues raised in the SI phase, which seems reasonable but not captured in the WI phase. One is the MG-less XR transmission, which means the XR service can be transmitted in the MG duration, which can avoid the delay of time-sensitive traffic due to MG.

R19 XR work scope includes the following enhancements missed from R18 [RAN1, RAN2, RAN4]

- UTO-UCI to be transmitted on DG-PUSCH that overrides the CG-PUSCH carrying the UTO-UCI [RAN1]

- Finer resource indication granularity for UTO-UCI [RAN1]

- FEC-based PDU Set discarding [RAN2]

- MG-less XR transmission [RAN2, RAN4]

Conclusion

We have the following proposals:

Proposal 1	R19 XR work scope includes the multi-modality service-related objectives [RAN2, RAN3]

-	Intra-UE synchronous transmission for multi-modal XR application

-	Inter-UE synchronous transmission for multi-modal XR application

Proposal 2	R19 XR work scope includes the following enhancements missed from R18 [RAN1, RAN2, RAN4]

-	UTO-UCI to be transmitted on DG-PUSCH that overrides the CG-PUSCH carrying the UTO-UCI [RAN1]

-	Finer resource indication granularity for UTO-UCI [RAN1]

-	FEC-based PDU Set discarding [RAN2]

-	MG-less XR transmission [RAN2, RAN4]

Reference

[1] RP-230786 Updated WID on XR Enhancements for NR

[2] S2-2310035 New SID on 5GS XRM Ph2

[3] SP-221326 New WID: Architecture Enhancements for XR and media services (XRM)

[4] TS 22.261 V19.3.0 (2023-06)

Appendix

Table A-1 Comparison of different UTO-UCI indication granularity

|  | Avg. number of consumed RBs (per UE per packet) |
| --- | --- |
| Case 1: UE always releases the unused CG PUSCH resource on a per-TO basis. | 679.1 |
| Case 2: UE releases the unused CG-PUSCH RBs at least on a RB-level in the last transmitted CG-PUSCH TO. | 562.2 |

Table A-2: Simulation assumptions for above comparison

| Parameters | Value |
| --- | --- |
| Scenario | Dense Urban hexagonal layout with (19, 3) Sectors |
| Inter-BS distance | 200m |
| Carrier frequency | 4GHz |
| Duplex Mode / Simulation bandwidth | 100MHz |
| SCS | 30KHz |
| TDD pattern | DDDSU |
| BS Antenna Configuration | 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) (dH, dV) = (0.5λ, 0.5λ) |
| UE Antenna Configuration | 4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| Transmit Power | 51dBm |
| Antenna Height | 3 m for BS and 1.5 m for UE |
| Receiver Noise Figure | 5 dB for BS and 9 dB for UE |
| UE speed | 3km/h |
| Scheduling Algorithm | SU-MIMO+PF |
| Channel estimation | Realistic |
| BS receiver | MMSE-IRC |