3GPP TSG-RAN WG #101	RP-231785

Bangalore, India, Sep 11 – Sep 15, 2023

Agenda Item:	8A.2.13.2

Source:	OPPO

Title:	Motivation for R19 enhancement of NR SL Relay

Document for:	Discussion, Decision

Introduction

This paper is to discuss the motivation for R19 NR SL Relay enhancement.

Background

SL Relay in LTE

During LTE Rel-13 enhanced device-to-device topic, the L3 relay was specified, which was to support the extension of network coverage using L3-based UE-to-Network Relays, including service continuity (if needed), based on Release 12 D2D communication, considering applicability to voice, video.

Figure 1 Rel-13 LTE-based L3 UE-to-Network Relay

Furthermore, in LTE Rel-15, there was a study on further enhanced device-to-device, which was to study the possibility of  a common solution supporting the following use cases:

i. UE to network relaying over non-3GPP access (Bluetooth/WiFi).

ii. UE to network relaying over LTE sidelink.

iii. Unidirectional and bidirectional UE to network relay.

Figure 2 Rel-15 LTE-based L2 UE-to-Network Relay (Study only)

SL Relay in NR

When it comes to NR, in Rel-17, both L2 and L3 UE-to-Network Relay are completed, where in the study phase, RAN WGs focused on mechanism(s) with minimum specification impact to support the SA requirements for sidelink-based UE-to-network and UE-to-UE relay, and later in normative phase, focused on UE-to-Network only, covering both layer-3 relay and layer-2 relay.

Figure 3 Rel-17 NR-based L3 UE-to-Network Relay (Non-N3IWF scheme)

Figure 4 Rel-17 NR-based L2 UE-to-Network Relay

And furthermore in Rel-18, the study on UE-to-UE relay are converted to normative phase, covering both Layer-3 relay and Layer-2 relay. Furthermore, it also includes enhancement of L2 U2N Relay, in terms of service-continuity and multi-path Relay.

Figure 5 Rel-18 NR-based L3 UE-to-UE Relay

Figure 6 Rel-18 NR-based L2 UE-to-UE Relay

I.e., so far in NR, basic components for both U2U and U2N, and both L2 and L3 relay have been specified (till the end of Rel-18).

1) For UE-to-Nework Relay, it allows not only single-path, but also multi-path relaying (1 direct path + 1 indirect path, limited to intra-gNB scenario, including both PC5-based and non-3GPP ideal connection).

2) For UE-to-UE Relay, it is limited to single-path relaying only.

For both cases, they are limited to single-hop, and unicast case only.

Discussion

Multi-Hop Relay

Stage-1 (TS 22.261) specification has defined the KPI for multi-hop UE-to-Network Relay

Table 1 KPI for Multi-hop UE-to-Network Relay

| Scenario | Max. data rate (DL) | Max. data rate (UL) | End-to-end latency (note 7) | Area traffic capacity (DL) | Area traffic capacity (UL) | Area user density | Area | Range of a single hop (note 8) | Estimated number of hops |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| InHome Scenario (note 1) | 1 Gbit/s | 500 Mbit/s | 10 ms | 5 Gbit/s/ home | 2 Gbit/s /home | 50 devices /house | 10 m x 10m – 3 floors | 10 m indoor | 2 to 3 |
| Factory Sensors (note 2) | 100 kbit/s | 5 Mbit/s | 50 ms to 1 s | 1 Gbit/s /factory | 50 Gbit/s /factory | 10000 devices /factory | 100 m x 100 m | 30 m indoor / metallic | 2 to 3 |
| Smart Metering (note 3) | 100 bytes / 15 mins | 100 bytes / 15 mins | 10 s | 200 x 100 bytes / 15 mins /hectare | 200 x 100 bytes / 15 mins /hectare | 200 devices /hectare | 100 m x 100 m | > 100 m indoor / deep indoor | 2 to 5 |
| Containers (note 4) | 100 bytes / 15 mins | 100 bytes / 15 mins | 10 s | 15000 x 100 bytes / 15 mins /ship | 15000 x 100 bytes / 15 mins /ship | 15000 containers /ship | 400 m x 60 m x 40 m | > 100 m indoor / outdoor / metallic | 3 to 9 |
| Freight Wagons | 100 bytes / 15 mins | 100 bytes / 15 mins | 10 s | 200 x 100 bytes / 15 mins /train | 200 x 100 bytes / 15 mins /train | 120 wagons /train | 1 km | > 100 m outdoor / tunnel | 10 to 15 |
| Public Safety (note 5) | 12 Mbit/s | 12 Mbit/s | 30 ms | 20 Mbit/s /building | 40 Mbit/s /building | 30 devices /building | 100 m x 100 m – 3 floors | > 50 m indoor (floor or stairwell) | 2 to 4 |
| Wearables (note 6) | 10 Mbit/s | 10 Mbit/s | 10 ms | 20 Mbit/s per 100 m2 | 20 Mbit/s per 100 m2 | 10 wearables per 100 m2 | 10 m x 10 m | 10 m indoor / outdoor | 1 to 2 |

The coverage extension requirement comes from different use cases, but especially from public safety.

Technically, to reach coverage extension, besides 3GPP-based multi-hop solution, other solution candidates include

- Either relying on lower frequency spectrum, yet that is not always available, and even if it is available, the bandwidth of lower frequency range is quite limited;

- Or relying on APP-layer solution, yet that is also questionable due to the a) security concern, b) additional latency, and 3) unavailable network control, considering the APP-level re-processing.

So a 3GPP-based multi-hop solution is preferred.

Multi-hop sidelink Relay is included in Rel-19 package.

Enhanced Multi-Path UE-to-Network Relay

Till Rel-18, the multi-path UE-to-network relay is limited in two aspects:

- The multi-path is limited to the scenario of 1 direct path + 1 indirect path

- The path switching of multi-path is limited to intra-gNB scenario (i.e., only single-path to single-path switching is supported for inter-gNB scenario)

For the first issue, considering SA2 has, in Rel-18, define the emergency-specific RSC based scheme, i.e., if limiting the multi-path scheme to single indirect path, for an out-of-coverage remote UE, it can only choose to connect with a single emergency-based RSC, or a single non-emergency based RSC, but not both. It limits the traffic type that an out-of-coverage remote UE can support.

Therefore, it is necessary to increase the number of indirect path that a remote UE can support simultaneously.

L2 Multiple indirect path based UE-to-network Relay is included in Rel-19 package.

Secondly, considering that existing tools have been specified by RAN2 for multi-path relay, there seems no need to limit it to intra-gNB scenario, but beneficial to extend it to inter-gNB case, by adding RAN3 related component into the toolbox.

Inter-gNB multi-path based path switching is included in Rel-19 package.

Conclusion

We have the following proposals:

Proposal 1	Multi-hop sidelink Relay is included in Rel-19 package.

Proposal 2	L2 Multiple indirect path based UE-to-network Relay is included in Rel-19 package.

Proposal 3	Inter-gNB multi-path based path switching is included in Rel-19 package.