3GPP TSG RAN Meeting #101	RP-231790

Bangalore, India, September 11-15, 2023

Agenda Item:	8A.2.2

Source:		 		vivo

Title:					Views on MIMO evolution in Rel-19

Document for:	Discussion

Introduction

MIMO has been a key technology in mobile communication systems, and has been evolving in every release of 3GPP specifications. With ever increasing demand of data volume and various application scenarios, further enhancement of MIMO technology in Rel-19 becomes inevitable:

With potential allocation of new spectrum beyond 7GHz for mobile communication systems, enhanced massive MIMO technology with larger antenna array becomes crucial for enhanced throughput and coverage.

Furthermore, channel aging has become a critical issue in the real network to harvest MIMO gain in moderate and high-speed scenarios. It is also an important area for improvement in Rel-19, which impacts both UL and DL.

For UL, whose performance has always been of concern for the operators, open/semi-open-loop transmission can be supported to achieve robust UL performance in moderate to high velocities.

For DL, CSI acquisition for moderate and high velocity UEs is another important aspect to further look into in Rel-19, especially in TDD bands which are the most widely used global bands for 5G and beyond.

As a second level of importance, beam-based operation in new spectrum beyond 7GHz, enhanced multi-TRP based operation, CSI enhancement etc. can also be considered as candidates for Rel-19 work scope if there are enough TUs allocated.

In section 2 below, we discuss potential enhancements for Rel-19 MIMO evolution in details.

Discussion

Enhanced massive-MIMO (eM-MIMO)

Up to Rel-18, NR supports various CSI feedback types in a single framework including Type I/II CSI, eType II CSI with up to 32 ports and also supports NCJT/CJT CSI feedback. Larger antenna array is necessary to ensure sufficient coverage in spectrum range beyond 7GHz. Below we provide initial evaluation results comparing different number of CSI-RS ports against 32 ports (baseline) assuming different antenna architectures. Simulation assumptions can be found in Table 1 in Appendix.

Performance comparison of different port layouts for array size 16×16×2

Performance comparison of different port layouts for array size 8×16×2

Performance comparison of different port layouts for array size 4×16×2

It can be observed that for different array sizes increasing number of ports provides significant gain, increasing number of horizontal ports provides better performance than increasing number of vertical ports.

Furthermore, with large antenna array in higher spectrum (beyond 7GHz), there is higher possibility of UEs being in near-field where the assumption on planar wave front is not valid. With the antenna aperture of 1 meter at 7GHz (wavelength is ~ 4.3cm), theoretical near-field distance is about 46 meters from base station, which means significant number of UEs may fall into this region. One way to address this issue is by specifically designing antenna architecture, for example, by splitting large antenna array into smaller sub-arrays. Each sub-array has smaller antenna aperture, hence the near-field effect can be mitigated or minimized. Multiple sub-arrays can be equivalent to multiple logical panels at the gNB placed side-by-side. Below, we provide initial evaluation on performance of CJT CSI feedback over multiple logical panels and eType II CSI feedback for overall single large antenna array to study the impact on far-field UEs. 64 ports array ({16,16,2,1,1,4,8}) is logically divided into 4 sub-arrays ({8,8,2,2,2,2,4}, 4 panels) with 16 ports each or 2 sub-arrays ({16,8,2,1,2,4,4}, 2 panels) with 32 ports each, and CJT CSI feedback is assumed. For 64 ports array, eType II CSI with parameter combination of 1 in current spec is assumed. Detailed simulation assumptions can be found in Table 1 in Appendix.

Performance comparison of different number of sub-arrays for 64 ports array

For similar performance, overhead of CJT CSI feedback over multiple logical panels is larger than eType II CSI feedback for overall single large antenna array due to increased number of rows of W2. In other words, for similar overhead CJT CSI feedback over multiple logical panels has performance loss compared to eType II CSI feedback for overall single large antenna array due to smaller pv,β,Ln values.

Considering both payload and performance, CJT CSI feedback with two logical panels is better than four logical panels.

Evaluate and study system performance as well as CSI overhead to support large antenna array considering different port layouts including sub-array architecture. If needed, specify necessary specification enhancement.

Enhanced performance in high-speed scenario

Precoder cycling for PUSCH

In Rel-18, CSI feedback is enhanced for moderate to high-speed scenario to improve DL performance. Similarly, UL performance should be enhanced for moderate to high speed where open/semi-open-loop precoding is a promising technique to guarantee the performance. Below we evaluate PUSCH performance for closed-loop precoding and precoder cycling in frequency domain for 4Tx and 8Tx UL transmission with link level simulations. For 4Tx PUSCH transmission, only full-coherent precoders from full/partial/non-coherent codebook in Rel-15 are used for both closed-loop wideband precoding and precoder cycling. And, for 8Tx PUSCH transmission, Rel-18 8Tx full-coherent codebook is assumed for both closed-loop wideband precoding and precoder cycling. In the case of precoder cycling, a precoder is randomly selected from the set of full-coherent precoders per PRG, a PRG is defined as 4 consecutive PRBs. Detailed simulation assumptions can be found in Table 4 in Appendix.

Performance comparison for 4Tx UL transmission

For closed-loop wideband precoding, only full-coherent precoders from FullPartialNon-coherent codebook subset defined in Rel-15 are used for TPMI indication. And, for precoder cycling PUSCH transmission, only full-coherent precoders are used. In Figure 5, performance of closed-loop wideband precoding and precoder cycling are compared for vehicular speeds of 3km/h, 30km/h, 60km/h and 120km/h with different SRS periodicity configured.

Performance comparison between precoder cycling and closed-loop wideband precoding with different vehicular speed for 4Tx rank=1 PUSCH transmission

It can be observed that for rank =1 PUSCH transmission, precoder cycling performs better compared to 4Tx closed-loop wideband precoding at higher vehicular speeds and performance of precoder cycling is not sensitive to SRS periodicity.

Performance comparison for 8Tx UL transmission

For closed-loop wideband precoding, only full-coherent precoders from Full-coherent codebook defined in Rel-18 are used for TPMI indication. And, for precoder cycling PUSCH transmission, only full-coherent precoders are used. In Figure 6, performance of closed-loop wideband precoding and precoder cycling are compared for vehicular speeds of 3km/h, 30km/h, 60km/h and 120km/h with different SRS periodicity configured for rank=1 transmission.

Performance comparison between precoder cycling and closed-loop wideband precoding with different vehicular speed for 8Tx rank=1 PUSCH transmission

It can be observed that for rank =1 PUSCH transmission, precoder cycling performs better compared to 8Tx closed-loop wideband precoding at higher vehicular speeds, and performance of precoder cycling is not sensitive to SRS periodicity.

Study and evaluate, specify if needed, performance of precoder cycling for PUSCH transmission with 4Tx and 8Tx.

Enhanced CSI acquisition for TDD high-speed scenarios

In Rel-18, UE-side CSI prediction based on pure PMI reporting has been introduced. To further improve the transmission performance for TDD high-speed scenarios, gNB-side CSI prediction enhancement for SRS-based DL CSI acquisition should be considered in Rel-19. Based on channel reciprocity, gNB can estimate the DL channel based on SRS reception, then the channel estimation result can be used to predict CSI for subsequent PDSCH transmission.

For UE-side CSI prediction based on PMI reporting, multiple P/SP CSI-RS occasions can be used to predict CSI for N4 occasions. Generally, a similar principle can be followed for SRS-based CSI prediction for TDD high-speed scenarios. However, the main difference is potential problem of phase continuity for SRS transmission in uplink, especially when the gap between SRS occasions is large, even 5ms might cause phase discontinuity at the UE. In other words, phase discontinuity would cause the channel estimation results of multiple SRS occasions unusable for CSI prediction.

There is potential problem of phase continuity at UE when the channel estimation results of multiple SRS occasions are used for CSI prediction.

In the current spec, the maximum number of SRS repetitions is up to 14 symbols, which enables a feasible way for symbol-level CSI prediction. Therefore, to avoid the phase discontinuity at UE, symbol-level CSI prediction based on SRS repetitions can be considered for TDD scenarios in Rel-19. The CSI for PDSCH transmission can be predicted based on SRS repetitions on the latest SRS occasion as shown in Figure 7.

Symbol-level CSI prediction based on repetitions on one SRS occasion

In Figure 7, the frame format DDDDUDDDDU is shown as an example. SRS with repetitions is transmitted in the UL slot, then channel estimation can be performed on each repetition symbol at the gNB side. There are R channel estimation results when the number of SRS repetitions is configured as R. Based on the channel estimation result of each symbol, the channel of the subsequent four DL slots can be predicted per symbol. For PDSCH scheduling in DL slots, the corresponding precoders can be calculated based on the predicted channel of respective DL slots.

- In addition, to evaluate the performance of symbol-level CSI prediction based on symbol-level SRS repetitions, the link-level simulation result is given in Figure 8, the simulation assumptions can be found in Table 3 in Appendix. According to the simulation results, it is observed that symbol-level CSI prediction based on symbol-level SRS repetitions can provide appreciable throughput gain for PDSCH.

PDSCH throughput of symbol-level CSI prediction based on symbol-level SRS repetitions

Symbol-level CSI prediction based on symbol-level SRS repetitions can provide appreciable throughput gain for PDSCH in TDD high-speed scenarios.

However, when a large number of repetitions is configured for SRS for a UE, the overall capacity becomes an issue. Therefore, SRS capacity enhancement and/or overhead reduction should be further studied for TDD high-speed scenarios in Rel-19.

Moreover, another scheme based on Doppler information reported for multiple paths to assist CSI prediction for TDD scenarios can also be considered. In this scheme, CSI-RS resource(s) are used for Doppler information measurement for multiple paths, then Doppler information is reported to gNB for assisting CSI prediction, combined with the channel estimation results based on SRS reception.

Study gNB-side CSI prediction for TDD high-speed scenarios in Rel-19, considering the following two aspects:

SRS capacity enhancement and/or overhead reduction

Doppler information reporting based on CSI-RS

Other enhancements in Rel-19

Beam-related enhancement

Fast beam training for large antenna array

Generally, to determine at least one accurate analog beam pair for subsequent transmission, all combinations of Tx beams on the gNB side and Rx beams on the UE side need to be swept and measured. And, in the process of analog beam training, only one beam pair, i.e., one Tx beam and one Rx beam can be measured at one time. For example, as shown in left part of Figure 9, 24 beam pairs need to be measured by UE and it would at least cost (24·X) ms, where X ms is the duration of measurement on one beam pair. For extremely large antenna array, such as 2046 or 4096, the beam-width becomes narrower and the number of beams is huge. As shown in right part of Figure 9, the number of antenna elements on the gNB side is four times that of the left part, and it is the same for the UE side. If legacy beam training mechanism is reused, 384 beam pairs need to be measured and duration of the whole process of beam training turns out to be (384·X) ms, which is too long. It is harmful for system throughput performance, especially for mobility scenarios, where beam measurement needs to be performed frequently. Therefore, fast beam training for extremely lager antenna array needs to be studied to reduce latency.

Illustration of beam training

As the number of antenna elements increases, the width of beam will be narrower, and the number of beams and the duration of beam measurement will be multiplied.

To reduce latency of beam measurement, study fast beam training for extremely large antenna array.

UE-initiated beam reporting

- In the current specification, beam reporting based on L1 measurement can only be triggered by the network. And, only L3 reporting is allowed to be triggered by UE, which is used for RRM. Compared to NW-initiated beam reporting, UE-initiated beam reporting has following advantages:

Lower latency and less signaling overhead compared to NW-initiated semi-persistent and aperiodic beam reporting. For UE-initiated beam reporting, measurement results will be reported once when a pre-defined criteria/event is satisfied, rather than to wait for NW signaling (e.g., MAC CE and DCI).

Less reporting overhead especially compared to periodic beam reporting. For UE-initiated beam reporting, measurement results are reported only when a pre-defined criteria/event is satisfied, rather than periodic reporting on PUCCH resource.

- UE-initiated beam reporting has potential to improve overall system performance hence should be studied in Rel-19

Study UE-initiated beam reporting on physical layer in Rel-19.

Triggering of AP-TRS for tracking together when TCI state activation

According to RAN4 requirement on TCI state activation, for the TCI state(s) not involved in the current active TCI state list, it is not activated until the first slot that is after slot n+THARQ+3Nslotsubframe,μ+Tfirst-SSB+TSSB-proc, where UE receives PDSCH carrying MAC CE TCI state activation command at slot n. As shown in Figure 10, Tfirst-SSB is the time of first SSB transmission after MAC CE command is decoded by the UE. The SSB shall be quasi-co-located with the activated TCI state with respect to QCL-Type A or QCL-Type C. According to RAN1 spec, for the TCI state(s) for PDCCH/PDSCH, QCL-Type C cannot be configured and the source RS with QCL-Type A must be TRS.  In other word, the time of Tfirst-SSB+TSSB-proc is used to monitor TRS and achieve fine synchronization. Therefore, a potential approach to reduce TCI state activation latency is to trigger aperiodic TRS for tracking while activating TCI state, where the aperiodic TRS can be determined by the source RS with respect to QCL-Type A of the TCI state(s) indicated in the TCI state activation command.

TCI state activation timeline

To reduce TCI state activation latency, study aperiodic TRS tracking is triggered together with TCI state activation.

Enhanced multi-TRP operation

Interference management

Maximizing the SINR is essential for improving throughput, hence study on interference management is important. As we know, the received signal is the superposition of four parts: the desired signal, intra-cell interference, inter-cell interference, and noise. The precoding matrix and receiving matrix can be used to amplify the desired signal and suppress the interference. Due to fewer antenna ports at the UE and more antenna ports at the gNB, impact of the precoding matrix on signal amplification and interference suppression is greater than the receiving matrix. Therefore, the UE assisted interference management for precoding matrix adjustment can be considered for downlink transmission.

Below we provide initial evaluation results comparing different number of coordination cells for interference management. UE may select at least part of the cells in the coordination/measurement cell set for PMI reporting. Non-serving cell may adjust their precoding matrix to maximize the SLNR/SINR. In Figure 11, we compare the system performance with and without coordination among cells. Simulation assumptions can be found in Table 2 in Appendix.

Performance comparison of different size of measurement/coordination set for interference management

When the number of coordination cells is 2, PMI reporting for non-serving cell can obtain 12% performance gain for downlink transmission. With the increase of the number of coordination cells, additional performance gain is smaller.

Evaluate and study system performance as well as CSI overhead to support UE assisted interference management for throughput improvement. If needed, specify necessary spec enhancement.

Enhanced CSI

CJT+NCJT CSI

In Rel-16, non-coherent joint transmission (NCJT) from two TRPs are supported where coherent phasing can’t be guaranteed between TRPs so that DL precoding is applied per TRP. In order to let the network has more accurate CSI for NCJT, CSI enhancement for NCJT is supported in Rel-17 with two reported RIs and corresponding two PMIs based on Type I codebook and one jointly derived CQI for a pair of CSI-RS resources of two TRPs. Rel-18 supports more advanced joint transmission (CJT) from up to four TRPs with perfect phase coherence among the TRPs. CSI enhancement derived from Rel-16 eType II codebook and Rel-17 eType II port selection codebook is supported by reporting independent SD bases for each individual TRP, independent or joint FD bases among the TRPs and coefficients for the bases within each TRP and among TRPs, which can help the network decide the precoder across TRPs.

However, in reality, various multi-TRP deployment schemes may exist in the network, some TRPs can achieve coherent phasing while others cannot. Figure 12 depicts a network deployment with mixed TRP coordination types, comprising non-coherent joint transmission among clusters, each of which is formed by TRPs capable of coherent joint transmission. Joint transmission among a group of TRPs capable of CJT and NCJT is possible to achieve higher performance and potential CSI enhancement based on both CJT codebook and NCJT codebook can be discussed.

Illustration of mixed TRP deployment

Introduce necessary enhancement to support various multi TRP operation in a single framework.

Joint CSI measurement and report across multi-carrier aggregation

- CSI computing at UE side is becoming complex and the feedback overhead is also increasing with various enhancements in previous releases. In current specification, each CSI report is only associated to one carrier or serving cell. When UE is configured with CSI reporting for multi-carriers, e.g. 4 carriers, the UE would perform four-fold CSI computation which causes even more complexity and overhead.

- One potential scheme to address above issues is to introduce joint CSI measurement and reporting across multi-carriers mainly targeting for contiguous carriers in intra-band as exemplary illustration in Figure 13. Taking eTypeII-codebook as an example, for the CSI feedback, spatial domain beam can be shared for the contiguous carriers assuming these carriers are being scheduled from the same gNB, also frequency domain compression and CQI calculation can be treated within aggregated wideband across the contiguous carriers since the gap between neighboring carriers can be small. It is clear that joint CSI processing across multi-carriers would reduce CSI computation complexity and reporting overhead, besides this, full channel information and better performance of channel estimation based on CSI-RS can be achieved with improved resolution of the delay and the rejection of interference and white noise is also enhanced due to larger band.

CSI design across two CA aggregation

Study joint CSI measurement and report across multi-carrier aggregation.

UE-side CSI prediction enhancement for higher UE speed

Rel-18 supports basic features of CSI prediction with reasonable performance for moderate speed, e.g., 30km/h. However, performance degrades significantly for higher UE speed. As seen in Figure 14, NMSE performance for AR based prediction degrades along with the increase of UE speed. Specifically, the switch point where AR prediction gain compared to sample-and-hold vanishes at 12-ms or 6-ms for prediction window length in 60km/h and 120km/h. It means the supported CSI and CSI-RS configurations cannot provide gain over non-prediction case when UE speed is high. Hence to support high MU performance for higher UE speeds, UE-side Type II based CSI prediction enhancement is needed for higher UE speed, e.g., up to 120km/h. Possible solutions include enhancement on CSI-RS and/or CSI configurations to capture larger Doppler spread.

| a) 30km/h | b) 60km/h |
| --- | --- |
| c) 120km/h |  |

NMSE performance for Rel-18 CSI prediction configurations in different UE speeds

Performance of Rel-18 CSI-RS/CSI configurations for CSI prediction degrades significantly for higher UE speed (e.g., 60 km/h, 120 km/h), even lower than non-prediction case.

Study UE-side CSI prediction enhancement for higher UE speed, e.g., up to 120km/h, in Rel-19

Enhance CSI-RS and/or CSI configurations to capture larger Doppler spread

Performance monitoring for UE-side CSI prediction

UE-side CSI prediction based on pure PMI reporting is being specified in Rel-18. However, performance of UE-side CSI prediction depends on many aspects (e.g., UE speed, channel time-variation, UE algorithm, etc.). Prediction performance/reliability varies dynamically along with these aspects. For example, as seen in the evaluations in Figure 14, performance varies significantly along with UE speeds. Hence performance monitoring for UE-side CSI prediction is needed to ensure the reliability. We propose to support performance monitoring for UE-side CSI prediction in Rel-19, which can be done by UE reporting of performance metrics to enable gNB monitoring of the prediction performance.

Performance of UE-side CSI prediction depends on many aspects (e.g., UE speed, channel time-variation, UE algorithm, etc.). Prediction performance/reliability varies dynamically along with these aspects.

Study performance monitoring for UE-side CSI prediction to ensure reliability in Rel-19

Consider UE reporting of performance metrics to enable gNB monitoring of the prediction performance

Conclusions

Based on discussion, analysis and observations in section 2 we have a set of proposals to consider for Rel-19 MIMO enhancement. Given the scope of this item and available TUs, we have a primary set of proposals which are essential in Rel-19 and a secondary set of proposals which can be considered if there are enough TUs available as below.

Primary set of proposals:

- Evaluate and study system performance as well as CSI overhead to support large antenna array considering different port layouts including sub-array architecture. If needed, specify necessary specification enhancement.

Study and evaluate, specify if needed, performance of precoder cycling for PUSCH transmission with 4Tx and 8Tx.

Study gNB-side CSI prediction for TDD high-speed scenarios in Rel-19, considering the following two aspects:

SRS capacity enhancement and/or overhead reduction

Doppler information reporting based on CSI-RS

Secondary set of proposals:

To reduce latency of beam measurement, study fast beam training for extremely large antenna array.

Study UE-initiated beam reporting on physical layer.

To reduce TCI state activation latency, study aperiodic TRS tracking is triggered together with TCI state activation.

Evaluate and study system performance as well as CSI overhead to support UE assisted interference management for throughput improvement. If needed, specify necessary spec enhancement.

Introduce necessary enhancement to support various multi TRP operation in a single framework.

Study joint CSI measurement and report across multi-carrier aggregation.

Study UE-side CSI prediction enhancement for higher UE speed, e.g., up to 120km/h

Enhance CSI-RS and/or CSI configurations to capture larger Doppler spread

Study performance monitoring for UE-side CSI prediction to ensure reliability

Consider UE reporting of performance metrics to enable gNB monitoring of the prediction performance

Appendix

System-level simulation assumptions for eM-MIMO

| Parameter | Value |  |
| --- | --- | --- |
| Duplex, Waveform | FDD, OFDM |  |
| Multiple access | OFDMA |  |
| Scenario | Dense Urban (macro only) 200m ISD. Typical 57-sector |  |
| Frequency Range | FR1 only, 6.7GHz |  |
| Antenna setup at gNB | (16,16,2), or (8,16,2), or (4,16,2); (dH, dV) = (0.5, 0.8)λ |  |
| Antenna setup and port layouts at UE | (1,1,2,1,1,1,1), (dH, dV) = (0.5, 0.5)λ |  |
| BS Tx power | Per TRP 41 dBm for 10MHz |  |
| BS antenna height | 25m |  |
| UE antenna height & gain | Follow TR36.873 |  |
| UE receiver noise figure | 9dB |  |
| Modulation | Up to 256QAM |  |
| Coding on PDSCH | LDPC Max code-block size=8448bit |  |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs | 52 for 15 kHz SCS |  |
| Simulation bandwidth | 20 MHz (10 MHz DL + 10 MHz UL) for 15kHz as a baseline |  |
| Frame structure | Slot Format 0 (all downlink) for all slots |  |
| MIMO scheme | MU-MIMO with rank 1 is a baseline |  |
| MIMO layers | For all evaluation, the maximum MU layers are 8 |  |
| CSI feedback | Feedback assumption at least for baseline scheme: CSI feedback periodicity (full CSI feedback): 5 ms Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |  |
| Traffic model | Full buffer |  |
| UE distribution | According to TS 38.802 - DU and UMa: 80% indoor (3km/h), 20% outdoor (30km/h) |  |
| UE receiver | MMSE-IRC as the baseline receiver |  |
| Evaluation Metric | Throughput and CSI feedback overhead as baseline metrics |  |
| UE number per TRP | 5 |  |

System-level simulation assumptions for interference management

| Parameter | Value |  |
| --- | --- | --- |
| Duplex, Waveform | FDD, OFDM |  |
| Multiple access | OFDMA |  |
| Scenario | InH. Typical 12-TRP. |  |
| Frequency Range | FR1 only, 2.2GHz |  |
| Antenna setup and port layouts at gNB | (4,4,2,1,1,2,4), (dH,dV) = (0.5, 0.5)λ |  |
| Antenna setup and port layouts at UE | (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |  |
| BS Tx power | Per TRP 21 dBm for 10MHz |  |
| BS antenna height | Follow TR36.873 |  |
| UE antenna height & gain | Follow TR36.873 |  |
| UE receiver noise figure | 5dB |  |
| Modulation | Up to 256QAM |  |
| Coding on PDSCH | LDPC Max code-block size=8448bit |  |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs | 52 for 15 kHz SCS |  |
| Simulation bandwidth | 20 MHz (10 MHz DL + 10 MHz UL) for 15kHz as a baseline |  |
| Frame structure | Slot Format 0 (all downlink) for all slots |  |
| MIMO scheme | SU-MIMO with rank 1/2 is a baseline |  |
| CSI feedback | Feedback assumption at least for baseline scheme CSI feedback periodicity (full CSI feedback): 5 ms Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |  |
| Traffic model | Full buffer |  |
| UE receiver | MMSE-IRC as the baseline receiver |  |
| Evaluation Metric | Throughput as baseline metrics |  |
| UE number per TRP | 1 |  |
| Codebook | Non-PMI |  |

Link-level simulation assumptions for gNB-side CSI prediction

| Parameter | Value |
| --- | --- |
| Frame format | DDDDUDDDDU |
| Carrier Frequency | 4 GHz |
| Subcarrier spacing | 15kHz |
| Channel Model | CDL-C |
| Delay spread | DS=300ns |
| UE velocity | 30km/h |
| BS antenna configuration | 16 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,2,4), (dH, dV) = (0.5, 0.8)λ |
| UE antenna configuration | 2 Rx: (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1), (dH, dV) = (0.5, 0.5)λ |
| Precoding | SVD, PRG=4 |
| DMRS configurations | Single symbol DMRS, pos = [3 11] |
| SRS configurations | 2 port, R=14, periodicity=5ms |
| Link adaptation | AMC enabled |
| MIMO Rank | Up to 2 layers |
| CSI applying delay | 2 slots |
| Channel estimation | Realistic MMSE channel estimation |
| Receiver type | MMSE |
| Prediction | AR prediction |

Link-level simulation assumptions for PUSCH precoder cycling

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 4GHz |
| Subcarrier spacing | 30kHz |
| Waveform | CP-OFDM |
| Channel Model | CDL-D, DS=200ns |
| gNB antenna assumption | 16Rx |
| UE antenna assumptions | 4 and 8Tx |
| PUSCH scheduling | 40PRB, 64QAM and code rate=0.7 single layer No HARQ one PUSCH scheduling per 10 slots |
| Precoding scheme | Baseline: wideband TPMI Precoder cycling: random TPMI each PRG=4 PRB |
| Receiver | Channel estimation: MMSE filter Equalizer: MMSE-based |