3GPP TSG RAN Meeting #101                                                           RP-231798

Bangalore, India, September 11-15, 2023

Source:	vivo

Title:	Rel-19 Channel Modeling for ISAC and New Spectrum (7-24GHz) (RAN1-led)

Agenda Item:	8A.2.11

Document for:	Discussion

Introduction

After discussion in Rel-19 workshop, a RAN chair’s summary of Rel-19 workshop [1] has been released, including the channel modeling as follows:

- Strong interest in exploring study in new areas (RAN plenary involvement?) - Very strong interest in channel modeling for integrated sensing and communication (ISAC), and also interest in other possible areas (e.g., use cases, scenarios, key performance metrics (KPIs), potential spec impact, etc.) - Reasonable interest in exploring study in new spectrum, such as 7-24GHz - Channel modeling study only? - Reasonable interest in RIS (Reconfigurable Intelligent Surface) related study - Channel modeling study only? - Very challenging if study is to be carried out for all the above items - Which one(s) should be the focus? Necessity for future standard evolution?

In this contribution, we express our views on channel modeling, and discuss how the channel model impacts the RAN1-based Rel-19 study in each interest. Then, we are trying to make the prioritization on each interest in Rel-19 study, accordingly. In addition, we investigate the sensing performance of both mono-static and bi-static models, relying on system level simulations as well as experiments.

This contribution can be summarized for two interested channel models in terms of the discussion points and the detailed contents, as follows:

| Channel Model Types | Discussion Points | Contents |
| --- | --- | --- |
| General discussion | Aspects of channel model study in Rel-19 | Prioritize the channel model studies on ISAC and FR 7-24 GHz in Rel-19, due to the duplicated workload between ISAC and RIS. |
| Channel model for ISAC | Sensing coverage | Demonstrate the importance of bi-static and mono-static sensing and the necessity of their coexistence from the performance perspective. |
|  | Bi-static sensing | Briefly discuss the bi-static channel model with newly introduced propagation model. |
|  | Mono-static sensing | Briefly discuss the mono-static channel model with newly introduced propagation model. |
|  | RCS | Describe the characteristics of typical RCSs and the relation between bi-static and mono-static sensing. |
|  | Experiment investigation | Adjust the path-loss of sensing channel between the empirical formula (based on TR 38.901) and the experiment results according to the various RCSs in InH and UMi LoS scenarios. |
| Channel Model for FR3 | Near-field range | Describe the channel modeling in FR3, as compared to the channel model specified in TR38.901. Model near-field propagation and spatial non-stationarity. |

Aspects of Channel Model Study in Rel-19

Rel-19 scope has been discussed for serval meetings. Due to the last release for NR, it is not preferable to deliver new features from Rel-19. It is envisaged, therefore, two achievements may be potentially fulfilled in Rel-19; one is to complete and enhance the existing NR features, such as AI/ML completion and enhancement for each use case as a WI, and the other is to make advance preparations towards 6G, such as the design of new channel models. Since ISAC, FR 7-24GHz, and RIS are belonging to 6G technologies, the studies on the corresponding channel model are preferably started prior to the 6G studies in Rel-19.

The channel modeling for RIS should be deprioritized as opposed to that for ISAC and FR 7-24GHz. This is because there are a lot of similarities and commonalities in the channel modeling between ISAC and RIS, except for RCS definition and configuration. This implies that as long as the channel modeling for ISAC is completed, the channel modeling for RIS can be simply imitated by defining a new RCS based on the RIS inherent materials and functionalities. Equally treating the studies of both channel models in Rel-19 may incur the duplicated workload. This should be avoided.

There are a lot of similarities and commonalities in the channel modeling between ISAC and RIS, except for RCS definition and configuration.

RAN1 prioritizes the studies on the channel models of ISAC and FR 7-24GHz.

As we will discuss in Section 3, since the channel model for ISAC is tightly coupled with the use cases and sensing modes, the typical use cases and sensing modes should be determined prior to the channel modeling. The following option can be taken into consideration for the ISAC channel model studies:

- Start the RAN- or RAN1-level discussion on use cases and sensing modes from the 1st quarter of 2024. The channel models corresponding to the use cases and the sensing modes are started from the 2nd quarter of 2024 in RAN1.

Start RAN- or RAN1-level discussion on use cases and sensing modes from the 1st quarter of 2024 and start the corresponding channel modeling from the 2nd quarter of 2024 in RAN1.

As we will discuss in Section 4, the channel model specified for the frequencies from 0.5 to 100 GHz in TR 38.901 already supports the frequency range of 7-24 GHz. Indeed, this could simplify the designs of channel model. The remaining issue on the channel modeling, however, is how to capture the characteristics of the near-field region in case of the large antenna aperture and the high frequency, which RAN1 has never done before. This implies that the channel modeling for the near-field region should be discussed for the advance preparation towards 6G technologies such as the near-field MIMO.

Regarding FR 7-24GHz, RAN1 starts the channel modeling in the 1st quarter of 2024, focusing on the near field scenario.

Start the channel modeling for FR 7-24 GHz from the 1st quarter of 2024, focusing on the near field scenario in RAN1.

Channel Model for ISAC

According to the summary of Rel-19 workshop [1], the majority companies are supportive of studying the channel model for ISAC as Rel-19 study item. Indeed, this is a right direction, but one important factor needs to be emphasized that unlike the channel model specified for NR in TR 38.901, the channel model for ISAC is much more complicated than what 3GPP has experienced so far. Furthermore, the channel model for ISAC is tightly coupled with the use cases and sensing modes.

As we elaborated in our contribution in the previous RAN meeting [2], the aspects of ISAC study mainly contain use case, sensing mode, channel model, and specification. Strictly speaking, it is almost not possible to isolate one aspect from others, particularly between use case, sensing mode, and channel model. In the typical use cases of intruder detection [3], for instance, highway/railway intruder detection and yard intruder detection can be categorized as the scenarios of outdoor and indoor, respectively. The former requires the mono-static sensing mode, while the latter requires the bi-static sensing mode. As we will discuss later, both sensing modes are completely different especially for sensing channel modeling in terms of sensing architecture and interface involved in sensing. In other words, if we are interested in the use case of highway/railway intruder detection, we need to study the mono-static sensing mode-based channel model accordingly. If we are interested in the use case of yard intruder detection, we need to study the bi-static sensing mode-based channel model accordingly.

As illustrated in Figure 1, however, there exist six sensing modes, categorized by gNB mono-static, gNB bi-static, gNB-to-UE bi-static, UE-to-gNB bi-static, UE mono-static, and UE bi-static. Each of them is potentially implemented in different use case, and each requires different channel model.

Figure 1: Six sensing modes for ISAC.

Due to the limited time unit assigned to the channel model study in Rel-19, we should find out an efficient way that can ensure to complete the study of channel models potentially applied for 6G use cases. This needs us to prioritize the studies of the channel models, associated with the use case and the sensing mode which are most interesting to stakeholders for 6G.

The channel model for ISAC is tightly coupled with the use cases and sensing modes.

RAN1 prioritizes the studies of the channel models, corresponding to the use case and the sensing mode.

Coverage Investigation

In the previous section, we have recognized that ISAC behaves the tight association between channel models, the use cases and the sensing modes. A question may be raised that which channel models should be prioritized. To this question, we investigate both mono-static sensing and bi-static sensing from the perspective of the achievable coverage.

Due to the UE diversity, theoretically, the received SNR for bi-static sensing is normally higher than that for mono-static sensing. For the detailed description regarding UE diversity, we can refer to Annex-1 in section 6. Here, we are trying to elaborate how UE diversity for sensing can be realized by involving a “Good-UE” other than a “Bad-UE”, so that the sensing coverage can be improved significantly, as illustrated in Figure 2.

Figure 2: Bi-static sensing mode with a selected UE involved.

According to the sensing requirements, it is reasonably assumed that NW is aware of the sensing area or UE-diversity zone, where at least a candidate of sensing UE exists and the target potentially appears as well. The sensing area is depicted as a circle with a radius of r. It is worthwhile noting that the less the radius of r, the larger the UE diversity, but the less the number of sensing UE candidates, and vice versa. We define that a UE who is in the sensing area denotes “Good-UE”, while a UE who is out of the sensing area denotes “Bad-UE”. The NW always strive to select a “Good-UE” to achieve the UE-diversity for sensing.

It is noted that the NW is not aware of the target centric-circle area prior to the sensing in reality, but rather than the circle area where the targets are uniformly distributed and so is the “Good-UE”. For the sake of simplicity, nevertheless, it is assumed that the sensing area is the target centric-circle area in our simulation, because we believe that the performance tendency in SNR should be the same.

With bi-static sensing mode, configuring a sensing area with a proper radius of r can achieve UE-diversity so as to improve the sensing coverage.

To make the comparison of the coverage between bi-static sensing and mono-static sensing, we conduct the system level simulation with the simulation assumptions briefly listed in Table 1.

Table 1: Simulation assumptions.

| Parameters | Assumptions |
| --- | --- |
| Pathloss | Ld1,d2=PLd1+PL(d2)+10logc24πf2, in TR 38.901 |
| Scenario | 3GPP TR38.901 InH and UMi |
| Center frequency | 4 GHz |
| Frequency bandwidth | 100MHz |
| Distance between gNBs | 20m for InH, and 200m for UMi |
| gNB height | 3m for InH, and 10m for UMi |
| gNB Tx power | 24dBm for InH, and 44dBm for UMi |
| gNB noise figure | 5 dB |
| gNB antenna pattern | Indoor wall-mount for InH, 3sector-direction for UMi |
| gNB antenna gain | 5dBi for InH 8dBi for UMi |
| gNB antenna configuration | (M,N,P,Mg,Ng;Mp,Np)=[4,2,2,1,1,4,2] |
| UE distribution | 10 UE per Cell, random-drop |
| UE height | 1m |
| UE Tx power | 23 dBm |
| UE noise figure | 7dB |
| UE antenna pattern | Omni-direction |
| UE antenna gain | 0dBi |
| UE antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,2) |
| Target RCS | 1 m2 |

We investigate the coverage performance of UE-to-gNB bi-static sensing and gNB-to-UE bi-static sensing with the various radius r=5,10,20 meters for InH scenario and the various radius r=5,15,30,60 meters for UMi scenario, as opposed to that of gNB-based mono-static sensing, as shown in Figure 3 and Figure 4. For the reference purpose, we also plot the CDF of wideband SNR for up-link communication link.

Figure 3: CDF of wideband SNR for both mono-static sensing and bi-static sensing in the scenario of InH.

Figure 4: CDF of wideband SNR for both mono-static sensing and bi-static sensing in the scenario of UMi.

From the simulation results, the observations can be made as follows:

- gNB mono-static sensing has a poor coverage but offers a high accuracy in the sensing range due to its inherent architecture.

- gNB-to-UE or UE-to-gNB bi-static sensing offers a wide coverage but has a compatibly low accuracy in sensing range due to the uncertainties of the timing and UE location.

  - The difference of the coverages between gNB-to-UE bi-static sensing and UE-to-gNB bi-static sensing is almost negligible.

- Bi-static sensing and mono-static sensing modes can be co-existing to each other.

  - Sensing area can be divided into two zones, one is for gNB-toUE or UE-to-gNB bi-static zone, and the other for gNB mono-static zone. The proper zone boundary is around SNR of 20dB in InH scenario, but -7dB in UMi scenario.

- Up-link communication link offers a higher wideband SNR than sensing link. For instance, the SNR gap between communication up-link and gNB-based sensing mono-static link is about 45dB and 35dB for the scenarios of InH and UMi, respectively. This implies that as long as the sensing requirements between UE and gNB are fulfilled, the communication quality of the interface in between should be no longer a problem.

Considering the trade-off between the sensing coverage and the sensing accuracy, we believe that bi-static sensing and gNB mono-static sensing can complementally help each other so that the overall sensing performance can be improved.

From the performance perspective and the potential use cases, therefore, the bi-static sensing mode (gNB-to-UE, UE-to-gNB) and gNB mono-static sensing mode are more important than other sensing modes.

Bi-static sensing and gNB mono-static sensing modes can be co-existing to each other so as to improve the overall sensing performance;

- gNB mono-static sensing has a poor coverage but offers a high accuracy in the sensing range, while gNB-to-UE or UE-to-gNB has a wide coverage but offers a compatibly low accuracy in sensing range.

As a study of NR sensing channel model in Rel-19, prioritize the bi-static sensing mode (gNB-to-UE, UE-to-gNB) and gNB mono-static sensing mode.

Channel model for bi-static sensing

In the legacy stochastic communication channel model in TR 38.901, the channel parameters such as delay, power, and angle coefficients are randomly generated, and the clusters with very low power are excluded. In the case that the sensing targets are the part of the communication channel clusters, it may mismatch the rays in a cluster and result in the ambiguity of sensing target if the legacy channel generation approach is directly adopted for ISAC channel generation. This may incur the inaccuracy issue on the sensing measurement. In the case that the sensing targets are the non-communication channel clusters, the existing communication channel model has not been considered so far. Therefore, the legacy statistical communication channel model is not suitable for ISAC study and evaluation, it is necessary to study a new method of generating ISAC channel.

To strive to reuse the channel model specified in TR 38.901, we introduce a new propagation model with both multiple sensing clusters and multiple communication clusters as illustrated in Figure 5, where the channel generation in each cluster can mimic what TR 38.901 does. It is worthwhile noting that the channel generation for communication cluster is unchanged in this case, so as to minimize the impact on the communication channel and avoid the mismatching in case that the sensing and communication channels are jointly modeled simultaneously.

Figure 5: The new propagation model with multiple sensing clusters and multiple communication clusters for bi-static sensing mode.

By doing this way, the ISAC channel can be modeled by additionally introducing the sensing channel modeling, and its general framework can follow-up TR38.901.

The channel modeling for gNB-to-UE or UE-to-gNB bi-static sensing based on TR 38.901 requires the moderate workload and its study can be completed within Rel-19 work plan.

Design channel model for gNB-to-UE and/or UE-to-gNB sensing modes in Rel-19.

In the proposed propagation model for bi-static sensing mode, there are two parts considered in sensing channel modeling:

- Sensing target channel: this part describes the propagation of electromagnetic waves passing through sensing target(s), and contains the information of the sensing target(s).

- Non-sensing target channel: this part mainly models the influence of non-sensing target(s), i.e., the environment object(s). All propagation paths included in this part do not pass through the sensing target(s), which can be considered as interference for sensing purpose.

Two methodologies of modeling the sensing target channel can be classified:

- Segmented channel modeling： based on the location of the sensing target, the channel between the sensing transmitter and the sensing target, and the channel between the sensing target and the sensing receiver are generated, respectively. Then the two channels are combined into a complete channel of sensing target. In this method, the following issues should be addressed:

  - Sensing target modeling, e.g., how to model the antenna pattern of sensing target, and the phase effect caused by sensing target.

    - The electromagnetic wave passing through the sensing target with physical size will have multiple different incident/reflection points [4], which can be equated to antennas. Hence, it is reasonable that the sensing target can be modeled as a node with multiple antennas. For example, the antenna configuration of sensing target can be consistent with the sensing receiver or sensing transmitter so as to not change the channel rank. The antenna gain of sensing target can be set to 0dBi, where the related signal power impact caused by sensing target is considered separately in the RCS.

  - Channel coefficients generation based on two segmented channels, e.g., how to combine the two segmented channels.

    - After the transmission of electromagnetic wave through multiple channels, the transmission process of multiple channels can be equivalent to a single channel through convolution operation [5]. Hence, we can convolve the two segmented channels into a complete channel. Besides, in order to reduce the complexity of convolution, some simplifications can be carried out, e.g., only a portion of the cluster is reserved for the convolution according to metric value, and the metric is obtained by multiplying the cluster power of the two channels.

- Cascaded channel modeling： one sensing target is modeled as a single cluster, and the channel between the sensing transmitter and the sensing receiver is modeled as a whole that only contains the cluster passed through the sensing target.

This approach only considers the line-of-sight propagation between the sensing transmitter and the sensing target, and that between the sensing target and the sensing receiver, without considering multiple reflections, which can be regarded as a simplified method.

Non-sensing target channel is not relevant to the information of sensing target and can be considered as interference or clutter for the purpose of sensing environment. Two possible methods to model non-sensing target channel are classified as follows:

- Non-sensing target channel can be stochastically modeled by using TR38.901 procedure, in which the corresponding NLOS clusters can be regarded as environment object(s).

- First, randomly generate the location of the environment object(s). Then the non-sensing target channel can be generated based on the location of environment object(s), using the same method of modeling sensing target channel.

Sensing channel can be obtained by combining sensing target channel and non-sensing target channel. However, the correlation and relationship between sensing channel and communication channel need to be further studied. For example:

- Whether sensing channel and communication channel share the same environment objects, and how their correlation are modeled.

- Whether and how sensing channel and communication channel are combined, e.g., how cluster powers are distributed in case that the sensing and communication channels are jointly modeled simultaneously.

Sensing target channel that contains the information of the sensing target(s) can be generated by either segmented channel modeling or cascaded channel modeling.

Non-sensing target channel that does not pass through sensing target(s) can be modeled by either using TR 38.901 procedure or using the same method of modeling sensing target channel based on the location generated randomly.

The correlation and relationship between sensing channel and communication channel need to be further studied.

Once the performance evaluation is performed in the sensing link, the clusters dedicated for communication are seen as an interference. However, once the performance evaluation is performed in the communication link, the cluster links for sensing is seen as an interference. It should be emphasized that the characteristics and behaviors associated with the communication channel, such as link budget and short-term fading channel, should not be statistically varied accordingly, as opposed to that of the legacy NR.

Studying the new propagation model should avoid the changes of statistic characteristics and behaviors associated with the communication channel.

Channel model for gNB mono-static sensing

Similar to the newly proposed channel model for bi-static sensing model, the channel model for gNB mono-static sensing can be done by the same way. As illustrated in Figure 6, we can employ the new propagation model with multiple sensing clusters and multiple communication clusters for mono-static sensing mode as well. The key step to generate the channel for gNB mono-static mode, however, is to determine the propagation environment, e.g., LOS or NLOS (with/without a direct path for sensing channel).

Figure 6: The propagation model with multiple sensing clusters and multiple communication clusters for mono-static sensing mode.

The small-scale fading channel generation for mono-static channel modeling can mimic the way for bi-static channel modeling.

TR 38.901 currently does not support a channel model for self-transmitting and self-receiving mode for gNB/UE, thus a new channel modeling framework and method are required.

Design channel model for gNB mono-static sensing modes in Rel-19.

RCS model

The RCS, σ, is a target-specific quantity that depends on many factors. The computational formula of the RCS is only possible for simple targets. The RCS of simple geometric targets mainly depends on the ratio of the structural dimensions of the targets to the wavelength. Practically, the RCS of a target depends on the physical geometry and exterior features of the target, the direction of the illuminating radar, the radar transmitters frequency, the electrical properties of the target’s surface.

The shape and size of the distribution are different, but always the same relative to the aspect angle of the illuminating radar, regardless of the angle from which the radar illuminates this sphere. The RCS, σ, is defined as

σ=4πr2PrPt

where Pt is the power density of the transmitter at the radar target in [W/m²], Pr is the scattered power density at the receiving site in [W/m²], r is the radius of the sphere.

The typical RCSs according to the target shape, size and the reflected direction can be formulated as in Table 2.

Table 2: The formulations for the typical RCSs.

| Reflection | RCS formulation |
| --- | --- |
| from a sphere | σ=πr2 |
| at a cylinder | σmax=2πrhλ |
| at a flat plate | σmax=4πb2h2λ2 |
| at an inclined plate | σ=0 |

The various RCSs for the typical targets can be modeled as a fixed value and obeying some probability distribution, as exemplified in Table 3 and Table 4, respectively.

Table 3: The RCSs for the typical targets.

| Targets | RCS [m²] | RCS [dBsm] |
| --- | --- | --- |
| bird | 0.01 | -20 |
| man | 1 | 0 |
| cabin cruiser | 10 | 10 |
| automobile | 100 | 20 |
| truck | 200 | 23 |
| corner reflector | 20379 | 43.1 |

Table 4: The RCS distributions for the typical targets [6].

| Targets | RCS distribution |
| --- | --- |
| man | Uniform distribution ∈(-20, 0) [dBsm] or Gaussian distribution N(-10,(10/3)2)∈(-20, 0) [dBsm] |
| compact car | Uniform distribution ∈(-5, 25) [dBsm] or Gaussian distribution N(10,52)∈(-5, 20) [dBsm] |
| large automobile | Uniform distribution ∈(15, 40) [dBsm] or Gaussian distribution N55/2,25/62∈(15, 40) [dBsm] |
| motorcycle | Uniform distribution ∈(-10, 20) [dBsm] or Gaussian distribution N5,52∈(-10, 20) [dBsm] |

As mentioned before, the RCS is a complex model related to the frequency and direction of the illuminating radar. When considering RCS in channel model, to reduce RCS modeling complexity, RCS can be simply modeled in large-scale channel model, e.g., using the RCS values of typical objects listed in the above tables directly. Besides, it is worthwhile noting that, these RCSs listed in the above table are usually applied to mono-static sensing, which are different from the RCS reflected in bi-static sensing.

To investigate the RCS, we also conduct a measurement experiment in Annex-2 of Section 7. From the measured results, the observations can be made as follows:

- In the mono-static experiment, the measured RCS value, i.e., -1.04dBsm, in mono-static sensing of sphere, basically matches to RCS empirical formula value of sphere, i.e., σMo=πr2=π×(0.5/2)2=-1.05 dBsm.

- In the bi-static experiment, the measured RCS value, i.e., -15.52dBsm, in bi-static sensing, is a little different from the RCS empirical formula in mono-static sensing, i.e., σMo=πr2=π×(0.15/2)2=-17.54 dBsm.

In mono-static experiment, the measured RCS value matches to RCS empirical formula value of sphere, while in bi-static experiment, the measured RCS value is a little different from the RCS empirical formula in mono-static sensing.

Study the RCS model for both bistatic sensing and mono-static sensing.

Adjustment of path-loss for sensing channel

In our system level evaluation discussed in Section 3.1, we have employed the newly modified propagation model to generate the pathloss based on the communication link pathloss model specified in TR 38.901, as

Ld1,d2=PLd1+PLd2+10log10λ24π-10log10σRCS

Eq. 1

where the same pathloss, PLd, is utilized twice except for the parameter of the distance between the sensing transmitter and the target, d1, the distance between the target and the sensing receiver, d2, and σRCS is the target RCS.

With regard to the pathloss formular in Eq. 1, two factors need to be investigated by our experiment. One is, according to the pathloss for InH in LOS and UMi in LOS scenarios specified in Table 7.4.1-1 of TR 38.901, both pathlosses contain a constant factor with 32.4dB for adjustment. This imposes a constant factor in the entire sensing pathloss to be used by twice, perhaps resulting in an inaccurate loss adjustment in the overall sensing channel.

The other is, since the RCS is added into the pathloss formular directly, it may need somewhat loss adjustment. To this end, we investigate the RCS based on the experiment as opposed to the empirical formula of RCS. In our experiment, the RCSs for InH in LOS and UMi in LOS scenarios are RCSInH=1.7 m2 or RCSInH=0.07 m2, and RCSUMi=1.6m2, depending on the angle of reflection and the type of target, respectively. It is noted that the detailed RCS investigation has been discussed in Section 3.4.

It is noted that, for the detailed definitions regarding the pathlosses of PLd1, and PLd2 and the specific RCSs for InH and UMi in LOS scenario, we can refer to Annex-3 in Section 8.

Our experiment is to investigate whether an overall loss adjustment is necessary and how much the adjustment range is reasonable.

Based on the experiment investigations in both InH and UMi scenarios, the gaps of pathloss between the empirical formula and the experiment results can be summarized in Table 5. For the detailed experiment assumptions and measured results, we can refer to the Annex-3 in Section 8.

Table 5: the gaps of pathloss between the empirical formula and the experiment results.

| Scenario | Gaps of pathloss in dB |
| --- | --- |
| InH in LOS | 0dB for the maximum gap, and –7~0 dB for average gap. |
| UMi in LOS | –7~0 dB |

In the scenario of InH LOS with σInH = 1.7m2 as the maximum RCS, the gap of pathloss between the empirical formula and the experiment results is fairly small, while with σInH = 0.07m2 as the average RCS, the gap of pathloss between the empirical formula and the experiment results is about 0dB and -7dB in the short range and the long range, respectively.

In the scenario of UMi LOS with σUMi = 1.6m2, the gap of pathloss between the empirical formula and the experiment results is about 0dB and -7dB at the short range and the long range, respectively.

Introduce an adjustment factor Δadj in pathloss as PLsen(d3D) = PLcom(d3D) +Δadj, where the exact value of Δadj should be figured out in RAN1 study.

Channel Model for 7-24GHz Range

3GPP working group has studied a new spectrum, releasing a technical report for the study item on 7.125 – 24.250 GHz frequency range, and covering the regulatory framework study, general RF aspects, as well as BS- and UE-specific aspects in May 2021 [7].

Accordingly, the selected frequency sub-ranges and example frequencies are listed in Table 5.2-1 [7]. The NR deployment scenarios for the 7-24 GHz frequency range are summarized in Table 5.6.5-1 [7]. However, the exemplified frequency sub-ranges were selected mainly for purpose of RF technology analyses during RAN4 SI.

In the 5G era, there were three usage scenarios identified based on the user demands, namely, eMBB, mMTC and URLLC. In the time frame of 6G, however, these three usage scenarios are expected to evolve into a more advanced version. Beyond that, ubiquitous connectivity, ISAC and AI intelligence and communication. There are still some challenges which have to be overcome, including the associated electromagnetic modelling (especially for the near-field), optimal waveform design, joint beamforming, power saving mechanisms as well as flexible duplexing and control.

The channel model specified for the frequencies from 0.5 to 100 GHz has been released in TR 38.901, that already supports the frequency range of 7-24 GHz. The remaining issue on the channel modeling of this frequency range, however, is how to capture the characteristics of the near-field. This is because given the large aperture of extremely largescale antenna arrays (ELAAs) and high frequencies, 6G networks with the frequency range of 7-24 GHz exhibit a large near-field region on the order of dozens of meters.

Near-field propagation effect has been studied and verified in many scenarios. In the indoor scenario exemplified in [8], the channel impulse response using a virtual uniform circular antenna (UCA) array is measured in consideration of both LOS and Obstructed-LOS (OLOS) cases as shown in Figure 7. It is shown that the propagation delay and the signal strength vary across UCA antennas. Although the variance of the propagation delay is only 2~3ns, a considerable phase error could be raised in the high frequency band due to the spherical wave propagation.

Under the modeling of near-field propagation, thus, it is emphasized that the obstacles in the near-field may incur the partial blockage for UCA antennas, known as a spatial non-stationary phenomenon. This phenomenon is crucial, especially in an ELAAs, that should be captured in FR3 channel modeling as well.

Figure 7: The channel impulse response across UCA antennas measured in the indoor scenario [8].

In this section, we briefly study the channel modeling for the frequency range of 7-24 GHz in consideration of near-field characteristics, mainly focusing on the spherical wave propagation and the spatial non-stationary phenomenon.

Modeling of spherical wave propagation

For FR1 region, the wavelength is usually much larger than the differences of propagation distance among antennas in the antenna array. Thus, the planar wave propagation can be assumed in the channel model, and the clusters is distributed in the far-field region. For FR3 region, the wavelength shrinks to 12mm~42mm, the near-filed phenomenon may easily occur in case that the aperture size of antenna array is kept divergent due to the various services in 6G. Hence, a planar wave model and a spherical wave model, representing as a far-field cluster and near-field cluster, respectively, should be introduced to express the propagation difference among the antennas as shown in a) and b) of Figure 8.

Figure 8: The illustration of planar wave and spherical wave propagation.

From the electromagnetics aspects [9], the boundary of near- and far-field region is derived according to the maximum allowable phase error, also known as Fraunhofer distance or Rayleigh distance,

Rf=π4D2ϕλ

where D is the maximum aperture size of the antenna array, ϕ denotes the maximum allowable phase error, λ denotes the wavelength. For example, for 10GHz frequency point, the wavelength λ=30mm, and assuming the aperture size of the antenna array is D=0.5m, the allowable phase error is ϕ=π8. The near-field boundary is derived as Rf=16.7m. It means that the scattering objects within the near-field boundary should be modeled as a near-field cluster, and the propagation path between the antenna array and near-field cluster should be derived under the spherical wave assumption. Therefore, after the near-field boundary is determined, the near-field clusters and far-field clusters are classified according to the distance from the antenna array to the targets, and the channel responses under the spherical wave and planar wave assumption are generated, accordingly.

When the frequency increases, the scattering objects are more likely to be within the near-field boundary in case of the large antenna aperture.

Considering the near-field boundary, the near-field clusters and the far-field clusters would be identified according to the distance away from the antenna array and the channel responses under the spherical wave and planar wave assumption are generated, accordingly.

When FR3 frequency is applied, a larger number of antennas are integrated in a panel thanks to the shrinking size of the antenna elements. The extremely large-massive MIMO could realize so called “pencil” beam with higher beamforming gain to enhance the coverage performance. On the other hand, the beam-width of the pencil beam are narrow inversely proportional to the antenna number in the array. In the legacy channel model in TR 38.901, only a few far-field clusters with a low probability are illuminated by the pencil beams that correspond to the channel response, which indicates that most of the sweeping beams are unresponsive. If the near-field phenomenon occurs, the near-field cluster is more likely to be illuminated in the beam sweeping, as exemplified in Figure 9 b). The sweeping beams pointing to an angle in the far field pass by the near-field cluster before the pencil beam formed in the far-field. The equivalent angle range in the far-field can be regarded as the shadow of the near-field cluster. Considering the near-field cluster, the signal strength variation may be described smoothly in the case of pencil beam sweeping.

Figure 9: An example of far-field cluster distribution and hybrid far- and near-field cluster distribution.

The modeling of the near-field cluster has been studied in COST 2100 [10]. A near-field cluster is represented by the propagation delay, AoD/ZoD and AoA/ZoA. The location of a cluster in [10] is achieved by the geometry-based random generation. To support the near-field channel model, thus, the method of the cluster location generation and the near-/far-field boundary should be studied as an additional procedure at the step of cluster generation as shown in channel coefficient generation procedure. Then the near-field clusters could be determined according to the locations. It is noted that the cluster location generation is restricted by the propagation delay of the corresponding path. Therefore, it is suggested that the near-field clusters are determined after the step-5 of “Generate delays” (defined in TR 38.901) as indicated by the red color in Figure 10.

The spherical wave should be considered for near-field clusters, the azimuth and elevation angle and the phase change gradually and non-linearly with the position of the antenna elements. To model the spherical wave accurately, a non-linear model of the angle and phase variance across the antenna array can be studied in FR3 near-field channel model. One direct method is promising to derive the distance and angle variance via the vector between near-field cluster and the corresponding antenna element or antenna group (assuming the plane wave is used within the antenna group). The other method is to decompose the spherical wave into a series of plane waves [11], which increase the total number of clusters. The spherical wave model should be involved in the step-7 of “Generate arrival & departure angle” (defined in TR 38.901) as indicated by the red color in Figure 10.

Figure 10: The channel coefficient generation procedure with the modification of the near-field cluster based on TR 38.901.

In FR3 channel model considering spherical wave propagation, the following aspects should be studied:

- The method of the cluster location generation and the near-/far-field boundary at the step of “Generate delay” based on TR 38.901.

- The model of the angle and the phase variance across the antenna array with respect to the spherical wave from a near-field cluster at the step of “Generate arrival & departure angles” based on TR 38.901.

Modeling of spatial non-stationarity

In near field, the spatial non-stationary phenomenon for the large scale MIMO should also be considered in FR3 channel model. It may occur when the propagation path is blocked, or the power of the scattered signal for the directional cluster focuses on a part of the antenna array, as exemplified in Figure 11, where the antenna array is divided to LOS region and blockage region. Due to the spatial non-stationary impact, the antenna elements at different spatial positions incur different channel multipath characteristics. COST 2100 channel model defines the visibility region of the antenna array for each cluster to support the model of spatial non-stationarity. In TR 38.901, the spatial consistency is considered for the case of correlation of nearby UEs, which can be regarded as the spatial non-stationarity at UE side. In FR3 channel model, the spatial non-stationarity at gNB side should be studied when the scale of the antenna array goes large. To model the variance of the signal strength among the antenna array, there should be an additional operation in terms of the antenna position when generating the cluster power.

Figure 11: An example of the spatial non-stationarity for FR3 channel model.

In FR3 channel model, the spatial non-stationarity at gNB side should be studied at the step of generating the cluster power when the scale of the antenna array goes large.

Conclusions

In this contribution, we have expressed our views on channel modeling, and discussed the channel model impacts on the RAN1-based Rel-19. The observations and proposals are summarized as follows:

Observation 1: There are a lot of similarities and commonalities in the channel modeling between ISAC and RIS, except for RCS definition and configuration.

Observation 2: The channel model for ISAC is tightly coupled with the use cases and sensing modes.

Observation 3: With bi-static sensing mode, configuring a sensing area with a proper radius of r can achieve UE-diversity so as to improve the sensing coverage.

Observation 4: Bi-static sensing and gNB mono-static sensing modes can be co-existing to each other so as to improve the overall sensing performance;

A) gNB mono-static sensing has a poor coverage but offers a high accuracy in the sensing range, while gNB-to-UE or UE-to-gNB has a wide coverage but offers a compatibly low accuracy in sensing range.

Observation 5: The channel modeling for gNB-to-UE or UE-to-gNB bi-static sensing based on TR 38.901 requires the moderate workload and its study can be completed within Rel-19 work plan.

Observation 6: Sensing target channel that contains the information of the sensing target(s) can be generated by either segmented channel modeling or cascaded channel modeling.

Observation 7: Non-sensing target channel that does not pass through sensing target(s) can be modeled by either using TR 38.901 procedure or using the same method of modeling sensing target channel based on the location generated randomly.

Observation 8: TR 38.901 currently does not support a channel model for self-transmitting and self-receiving mode for gNB/UE, thus a new channel modeling framework and method are required.

Observation 9: In mono-static experiment, the measured RCS value matches to RCS empirical formula value of sphere, while in bi-static experiment, the measured RCS value is a little different from the RCS empirical formula in mono-static sensing.

Observation 10:  In the scenario of InH LOS with σInH = 1.7m2 as the maximum RCS, the gap of pathloss between the empirical formula and the experiment results is fairly small, while with σInH = 0.07m2 as the average RCS, the gap of pathloss between the empirical formula and the experiment results is about 0dB and -7dB in the short range and the long range, respectively.

Observation 11:  In the scenario of UMi LOS with σUMi = 1.6m2, the gap of pathloss between the empirical formula and the experiment results is about 0dB and -7dB at the short range and the long range, respectively.

Observation 12: When the frequency increases, the scattering objects are more likely to be within the near-field boundary in case of the large antenna aperture.

Observation 13: In the scenario of InH LOS, the pathloss measured in the communication link are well aligned with the pathloss generated by the empirical formula.

Observation 14: USRP hardware prototype is reliable for the investigation of the pathloss test.

Observation 15: With a triangular corner reflector during its moving, the extracted test samples distribute with a relatively large variation (about 15dB).

Observation 16: With a car surface reflector during its moving, the extracted test samples distribute with a relatively small variation (less than 5dB).

Observation 17: The scenario of UMi LOS behaves two-step pathloss with a turn-point at the range of 53m.

Proposal 1: RAN1 prioritizes the studies on the channel models of ISAC and FR 7-24GHz.

Proposal 2: Start RAN- or RAN1-level discussion on use cases and sensing modes from the 1st quarter of 2024 and start the corresponding channel modeling from the 2nd quarter of 2024 in RAN1.

Proposal 3:  Start the channel modeling for FR 7-24 GHz from the 1st quarter of 2024, focusing on the near field scenario in RAN1.

Proposal 4: RAN1 prioritizes the studies of the channel models, corresponding to the use case and the sensing mode.

Proposal 5: As a study of NR sensing channel model in Rel-19, prioritize the bi-static sensing mode (gNB-to-UE, UE-to-gNB) and gNB mono-static sensing mode.

Proposal 6: Design channel model for gNB-to-UE and/or UE-to-gNB sensing modes in Rel-19.

Proposal 7: The correlation and relationship between sensing channel and communication channel need to be further studied.

Proposal 8: Studying the new propagation model should avoid the changes of statistic characteristics and behaviors associated with the communication channel.

Proposal 9: Design channel model for gNB mono-static sensing modes in Rel-19.

Proposal 10: Study the RCS model for both bistatic sensing and mono-static sensing.

Proposal 11:  Introduce an adjustment factor Δadj in pathloss as PLsen(d3D) = PLcom(d3D) +Δadj, where the exact value of Δadj should be figured out in RAN1 study.

Proposal 12: Considering the near-field boundary, the near-field clusters and the far-field clusters would be identified according to the distance away from the antenna array and the channel responses under the spherical wave and planar wave assumption are generated, accordingly.

Proposal 13: In FR3 channel model considering spherical wave propagation, the following aspects should be studied:

A) The method of the cluster location generation and the near-/far-field boundary at the step of “Generate delay” based on TR 38.901.

B) The model of the angle and the phase variance across the antenna array with respect to the spherical wave from a near-field cluster at the step of “Generate arrival & departure angles” based on TR 38.901.

Proposal 14:  In FR3 channel model, the spatial non-stationarity at gNB side should be studied at the step of generating the cluster power when the scale of the antenna array goes large.

References

- 3GPP TSG RAN #100, RWS-230488, RAN Chair’s Summary of Rel-19 Workshop, June 15-16, 2023.

- 3GPP TSG RAN #101, RP-230378, Initial Study on Integrated Sensing and Communication for NR, March 20-23, 2023, vivo.

- 3GPP TR 22.837, Feasibility Study on Integrated Sensing and Communication, V19.0.0, 2023-06.

- A. M. Haimovich, R. S. Blum and L. J. Cimini, "MIMO Radar with Widely Separated Antennas," in IEEE Signal Processing Magazine, vol. 25, no. 1, pp. 116-129, 2008.

- Stankovic, V., & Stankovic, L. (2010). Continuous and discrete time signals and systems. Upper Saddle River, NJ: Prentice-Hall.

- Huawei，5G-A通感仿真评估研究报告框架-3.3车路章节，IMT-2020(5G)推进组通信感知任务组第五次会议，2022年7月7日.

- 3GPP TR 38.820, “7-24 GHz frequency range”, v16.1.0, 2021-03.

- Z. Yuan, J. Zhang, et al., “Spatial Non-Stationary Near-Field Channel Modeling and Validation for Massive MIMO Systems,” IEEE Transactions on Antennas and Propagation, Vol. 71, No. 1, Jan. 2023.

- K. T. Selvan and R. Janaswamy, “Fraunhofer and Fresnel Distances: Unified derivation for aperture antennas,” IEEE Antennas and Propagation Magazine, Vol. 59, No. 4, Aug. 2017.

- L. Liu, C. Oestges, J. Poutanen, et al., “The COST 2100 MIMO channel model,” IEEE Wireless Communications, Vol. 19, No. 6, Dec. 2012.

- L. Sanguinetti, A. A. D’Amico, and M. Debbah, “Wavenumber-Division Multiplexing in Line-of-Sight Holographic MIMO Communications,” IEEE Transactions on Wireless Communications, Vol. 22, No. 4, Apr. 2023.

- H. Buddendick and T. F. Eibert, “Acceleration of Ray-Based Radar Cross Section Predictions Using Monostatic-Bistatic Equivalence,” IEEE Trans. On Antenna and Propagation, Vol. 58, No. 2, February 2010.

Annex-1: SNR Analysis between Mono-static and Bi-static Sensing

Basically, the SNRs offered by the mono-static sensing and the bi-static sensing are different. The sensing SNR equation, in general, can be expressed as

SNRSensing=PTxTx Power∙14πRTx2SpreadFactor∙GTxTx-Ant.Gain∙σbRCS∙14πRRx2SpreadFactor∙GRxλ24πRx-Ant.Aperture∙1kBT0BFn

=PTxGTxGRxλ2σb4π3RTx2RRx2∙kBT0BFnL

where kB is the Boltzman’s constant and equal to 1.38×10-23w/(Hz⁰K), T0 denotes a reference temperature in degrees Kelvin (⁰K), B is the effective noise bandwidth of the sensing and has the units of Hz, Fn is the sensing noise figure and is dimensionless, or has the units of w/w, L is a term included to account for all losses that must be considered when using the sensing range equation, and GTx and GRx are the transmit antenna and receive antenna gains, respectively.

For mono-static sensing, the distance between the transmitter and the target, RTx, and the distance between the receiver and the target, RRx, are the same, namely, RTx=RTx, while for bi-static sensing, the distance between the transmitter and the target, RTx, and the distance between the receiver and the target, RRx, are different. Therefore, the SNR gain achieved by the bi-static sensing, as opposed to monostatic sensing, ∆SNR, can be defined as

∆SNR=SNRBiSNRMo=RTx2GRxBiRRx2GRxMo

where GRxBi and GRxMo are the receive antenna gains for the bistatic sensing and the monostatic sensing, respectively.

If the transmitter is a gNB, who is far away from the target (i.e., RTx→ large), and the receiver is a UE, who is near the target (i.e., RRx→ small), the SNR gain could be significantly large.

It should be noted that, RRx between the UE and the target can be controlled by the network, e.g., MAC scheduler, meaning that the network can always select the UE, who is near the target, for sensing. This is called UE diversity.

Annex-2: Experiment for RCS

In this section, we conduct a measurement experiment of RCS. The measurement system is based on the vector network analyzer (VNA). The measurements are conducted in an anechoic chamber. The measuring materials is the registration sphere, which is placed on the rotary table. Besides, two directional antennas are respectively fixed on the tripods as illustrated in Figure 12. The measurement assumptions are briefly listed in Table 6.

Figure 12: The measurement system for RCS.

Table 6: Measurement assumptions.

| Parameters | RCS in mono-static case | RCS in bi-static case |
| --- | --- | --- |
| Center frequency | 6.5GHz | 28GHz |
| Frequency bandwidth | 1GHz | 1GHz |
| Rotation angle interval | 5.0° | 5.0° |
| Angle between Tx & Rx | 0° | 10.0° |
| Measuring material | registration sphere | registration sphere |
| Diameter of the registration sphere | 0.5m | 0.15m |

Based on the measurement system and assumptions, we measure the value of RCS for mono-static and bi-static, as illustrated in Figure 13 and Figure 14, respectively. The mean value of RCS at all angles for mono-static and bi-static are -1.04dBsm and -15.52dBsm, respectively.

Figure 13: The measured value of RCS for mono-static.

Figure 14: The measured value of RCS for bi-static.

From the measured results, the observations can be made as follows:

- In mono-static experiment, the measured RCS value, i.e., -1.04dBsm, in mono-static sensing of sphere, basically matches to RCS empirical formula value of sphere, i.e., σMo=πr2=π*(0.5/2)2=-1.05 dBsm.

- In bi-static experiment, the measured RCS value, i.e., -15.52dBsm, in bi-static sensing, is a little different from the RCS empirical formula in mono-static sensing, i.e., σMo=πr2=π*(0.15/2)2=-17.54 dBsm.

Annex-3: Experiment for Indoor and Outdoor Pathloss

In this section, first of all, we calibrate our hardware system (USRP) in the InH LOS scenario of Uu communication link, ensuring that the tested pathloss aligns what we defined in TR 38.901. Then, we try to figure out the gap of pathloss between the empirical formula and the experiment results in both InH and UMi LOS scenarios.

The experiment assumptions in our experiment investigation are briefly listed in Table 7, where it can figure out that the channel matrix for the signal processing is 410 sub-carriers × 400 OFDM symbols.

It should be noted here that in the pathloss test of the sensing channel, since the transmitter and the receiver belong to the same device, the perfect time and frequency synchronization can be achieved. Hence the 2D-FFT operation can be directly performed in the delay-Doppler domain after the signal detection, so that the power of the path reflected only by the sensing target can be extracted.

In the pathloss test of the communication link, on the other hand, the transmitter and receiver belong to different devices, incurring the timing off-set and carrier frequency off-set. Thus, the delay-Doppler spectrum after 2D-FFT operation will be dispersed. For simplicity, therefore, we perform an IFFT operation on the frequency domain signal to convert the signal to the delay domain, and then complete signal detection and signal power calculation in the delay domain.

Table 7: The experiment assumptions.

| Parameters | Assumptions |
| --- | --- |
| Center frequency | 4 GHz |
| Frequency bandwidth | 400 MHz |
| Sub-carrier spacing | 120 kHz |
| Configuration of sensing signal | front-loaded PDSCH-DMRS, DMRS with comb-2: Note-1 |
| Time duration for sensing signal | One sensing signal every 4-time slots within 200ms, equivalent to 400 OFDM symbols |
| Transmit antenna gain | 18 dBi |
| Receive antenna gain | 12 dBi |
| Transmit PA | 40 dB for InH and 60 for UMi |
| Receive LNA | 40 dB for InH and UMi 34 dB for Uu communication test |
| UE Tx power | -25 dBm: Note-2 |
| Experiment scenarios | InH LOS: 40m×10m room, transmit and receive antenna height 1.5m, and transmit and receive antenna spacing 2.4m UMi LOS: Parking lot, transmit and receive antenna height 2m, and transmit and receive antenna spacing 1.6m |
| Sensing target | InH LOS: Triangular corner reflector UMi LOS: Dongfeng Peugeot 308 car (see photo below) |

Note-1: To reduce the computational load of radar signal processing, the result of channel estimation is extracted by 4-times in the frequency domain, and finally it is the sensing signal with 1 subcarrier for every 8 subcarriers.

Note-2: Since the sensing signal only occupies 1/8 of the number of subcarriers in the frequency domain, the equivalent transmit power of the sensing signal is -34 dBm, i.e., -25-10log108=-34 dBm.

Uu-Link Calibration in InH LOS Scenario

In order to ensure the pathloss consistency between the tested communication link and the empirical formula, we perform an experiment measurement of the pathloss in the InH LOS scenario of Uu communication link, based on the USRP hardware prototype. The hardware diagram is depicted in Figure 15, where the hardware part of the communication test system is mainly composed of USRP1 and USRP2 in the transmitter and the receiver, respectively.

Figure 15: The diagram of Uu communication link channel path-loss test.

For more details, after the transmitted signal generated by USRP1 is amplified by the PA with the value of 40dB, it is transmitted through the transmit antenna with the antenna gain of 18dBi. The received signal from the receive antenna with the antenna gain of 12dBi is processed in USRP2 after the LNA with 34dB. USRP2 conducts down-conversion, ADC, LS channel estimation and so forth, and outputs the channel estimation results. The PC performs off-line signal processing to obtain the RSRP, whereby the pathloss information corresponding to the communication link is estimated.

In the pathloss test of the communication link, we perform an IFFT operation on the frequency domain signal to convert the signal to the delay domain signal, and then calculate the signal power in the delay domain.

In the calibration, we consider the empirical propagation model in the scenario of InH to generate the pathloss model specified in TR 38.901, as

PLInH-LOS=32.4+17.3log10(d3D)+20log10(fc)

where d3D is the distance from the transmit antenna to the receive antenna.

Figure 16 depicts the pathloss in dB as a function of distance from the transmit antenna to the receive antenna in the scenario of InH LOS, where each point is plotted based on the measurement in the different range.

It can be observed that there is a certain deviation of 4dB between the test results and the empirical formula results, and we believe that the results are well aligned.

Figure 16: Experiment pathloss results of communication link in the scenario of InH LOS.

In the scenario of InH LOS, the pathloss measured in the communication link are well aligned with the pathloss generated by the empirical formula.

USRP hardware prototype is reliable for the investigation of the pathloss test.

Pathloss Investigation in both InH and UMi LOS Scenarios

Due to the assumptions of mono-static sensing mode, the same pathloss formula, PLd, is utilized twice except for the parameters of the distance, d1, from the transmitter to the target, and the distance, d2, from the target to the receiver. In our system level evaluation discussed in Section 3.1, the newly modified propagation model is utilized to generate the pathloss based on the communication link pathloss model specified in TR 38.901, as

Ld1,d2=PLd1+PLd2+10log10λ24π-10log10σRCS

Eq. 2

According to the pathloss for InH in LOS and UMi in LOS scenarios specified in Table 7.4.1-1 of TR 38.901, for instance, it can be expressed as

PLInH-LOSd3D=32.4+17.3log10(d3D)+20log10(fc)

Eq. 3

PLUMi-LOS=PL110m≤d2D≤dBS'PL2dBS'≤d2D≤5km

PL1=32.4+21log10(d3D)+20log10(fc)

PL2=32.4+40log10(d3D)+20log10(fc)-9.5log10((dBP')2+(hBS-hUT)2)

dBS'=4hBS'hUT'fcc

Eq. 4

where d3D is the one-way distance from the device to the sensing target, and dBS'≈53.3m in our experiment, hBS and hUT are the actual antenna heights at BS and UT, respectively, and hBS' and hUT' are effective antenna heights at the BS and the UT, respectively, with hBS'=hBS-hE, and hUT'=hUT-hE, and for UMi, hE=1m.

We conduct an experiment based on mono-static sensing mode, with the diagram of the USRP-based prototype as depicted in Figure 17, where a USRP connected with baseband RF modules, and transmit/receive antennas.

Figure 17: The diagram of sensing channel path-loss test relying on the USRP-based prototype with mono-static sensing mode.

For more details, after the transmitted signal is amplified by the PA with the value of 40dB for InH and 60dB for UMi, it is transmitted through the transmit antenna with the antenna gain of 18dBi. The signal returns to the receive antenna after being reflected by the sensing target, and the received signal is processed after the LNA. USRP conducts down-conversion, ADC, LS channel estimation and so forth, and outputs the channel estimation results. The PC performs off-line radar signal processing to obtain the delay-Doppler spectrum, whereby the path information corresponding to the sensing target is extracted.

In the scenario of InH LOS, the RCS for the triangular corner reflector used for the comparison with the experiment results is empirically given by

σInH=4πL43λ2, m2

Eq. 5

where L is the height of the triangular corner reflector. In our experiment, with the size of L=0.22m, the maximum RCS is 1.7m2, and the average RCS is 0.07m2.

Figure 18 depicts the pathloss in dB as a function of distance between the target and the test device in the scenario of InH LOS, where each point in the scatter plot of the measured results represents the change in RCS due to the change in the angle of the corner reflector relative to the test device during the experiment.

Figure 18: Experiment results of sensing pathloss in the scenario of InH LOS.

It can be observed that if assuming the average RCS with σInH=0.07m2, the gap of pathloss between the empirical formula and the experiment results is about 1dB, while if assuming the maximum RCS with σInH=1.7m2, the gap of pathloss between the empirical formula and the experiment results increases to 13dB. In addition, due to the instability of the triangular corner reflector during its moving, we confirm that the extracted test samples distribute with a relatively large variation (about 15dB).

In the scenario of InH LOS with σInH=1.7m2 as the maximum RCS, the gap of pathloss between the empirical formula and the experiment results is fairly small, while with σInH=0.07m2 as the average RCS, the gap of pathloss between the empirical formula and the experiment results is about 0dB and -7dB in the short range and the long range, respectively.

With a triangular corner reflector during its moving, the extracted test samples distribute with a relatively large variation (about 15dB).

Furthermore, in the 10GHz frequency band, empirically, the relevant RCS in the front of the car is about 10m2 [12]. Considering the RCS is inversely proportional to the square of the wavelength, thus, the RCS in the front of the car, in the 4GHz frequency band of the UMi LOS scenario, is approximated to 1.6m2, i.e., by 10m2/10/42=1.6m2.

Figure 19 depicts the pathloss in dB as a function of distance between the target and the test device in the scenario of UMi LOS, where each point in the scatter plot of the measured results represents the change in RCS due to the change in the angle of the corner reflector relative to the transmitter during the experiment.

Figure 19: Experiment results of sensing pathloss in the scenario of UMi LOS.

It can be observed that if assuming σUMi=1.6m2, the gap of pathloss between the empirical formula and the experiment results is about 2dB. In addition, two confirmations are made as follows:

- Due to the relative stability of the car surface during its moving, the extracted test samples distribute with a relatively small variation (less than 5dB).

- Two-step pathloss behaves with a turn-point at the range of 53m.

In the scenario of UMi LOS with σUMi=1.6m2, the gap of pathloss between the empirical formula and the experiment results is about 0dB and -7dB at the short range and the long range, respectively.

With a car surface reflector during its moving, the extracted test samples distribute with a relatively small variation (less than 5dB).

The scenario of UMi LOS behaves two-step pathloss with a turn-point at the range of 53m.