3GPP TSG-RAN Meeting #101	RP-231859

Bangalore, India, September 11-15, 2023

Agenda item:	9.3.1.9

Source: 	Qualcomm Incorporated

Title: 	Views on scope reduction for Rel-18 WI on Expanded and Improved NR Positioning

Document for:		Discussion and Decision

# 1.	Introduction

At RAN2#123, it was concluded [1]:

Þ SL pos: RAN2 see some risk for the completion with the current scope.

In this contribution, we provide our view on the scope reduction for the Rel-18 Work Item on "Expanded and Improved NR Positioning" [2].

# 2.	Discussion

## 2.1	Sidelink Positioning

### 2.1.1	WI objectives/Tasks

For sidelink positioning, the WID [2] includes the following main objectives/tasks:

1.	Specify/design a SL-PRS.

-	This is a RAN1 task, and RAN1 has completed the design. The RAN1 design of the SL-PRS need to be supported in RAN2 specifications to enable any sidelink positioning using NR PC5.

2.	Specify support for sidelink positioning methods: RTT-type solutions, SL-AoA, SL-TDOA.

-	RAN1 has defined measurements and associated attributes for the above methods. However, RAN2 must design the system support to enable the above positioning methods, including SLPP design, triggering criteria (upper-lower layer interactions (SCI, priority handling, etc.)), synchronization mechanisms between (anchor) UEs, etc.).

3.	Resource allocation for SL-PRS (scheme 1 (network-centric) and scheme 2 (UE autonomous)).

-	This is also a RAN1 task, however, with many RAN2 specification impacts, incl. Uu RRC and MAC.

4.	Transmit power control for SL-PRS transmissions.

-	This is a RAN1 task which seems to have no (or little)  RAN2 specification impacts.

5.	Specify support for unicast, groupcast and broadcast of SL-PRS transmissions.

-	This is a RAN1 task which seems to have no (or little)  RAN2 specification impacts.

6.	Specify signalling/procedures to support SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios.

-	This is the main RAN2 task, which includes the design of a new protocol (SLPP) and associated procedures, incl. signalling/procedures between UEs and signalling/procedures between UEs and LMF.

-	The RAN2 discussions and contributions around this objective comprise the following main sub-tasks:

a.	SLPP transport (incl. protocol layering, architecture, reliable transport features);

b.	SLPP session handling (incl. session-based and session-less operation);

c.	SLPP positioning procedures (incl. (among others) with and without LMF involvement, hybrid SL/Uu positioning, coverage scenarios (in-coverage, out-of-coverage, partial coverage), relaying for partial coverage scenarios, anchor/server UE selection, assignment of roles to UEs (e.g., server UE separate from anchor/target UE, etc.), discovery aspects (e.g., metafield content));

d.	Cast-types supported for SLPP signalling (unicast, groupcast, broadcast) and group positioning  ((application-layer) managed and unmanaged groupcast);

e.	Support for multiple target UEs;

f.	SLPP content (e.g. common aspects and SL-PRS support).

7.	Sidelink positioning and ranging service authorizations.

-	This is primarily a RAN3 task without RAN2 impacts.

### 2.1.2	Proposed scope reduction

Given the RAN2 status and agreements so far [3],[4],[5], we think a (probably significant) scope reduction of the objectives/tasks listed in section 2.1.1 would be necessary to complete the remaining work in the two RAN2 meetings left for this WI [2]. However, any reduced WI scope must still result in a workable/useful specification, but with reduced capabilities/features compared to what was originally intended.

In the following, we provide our views on the objectives/tasks listed in section 2.1.1 above impacting RAN2.

##### Supported SL positioning methods.

RTT-type and SL-AoA based solutions do not require synchronization among UEs involved in a session. In addition, RTT/AoA methods would be required to support ranging between two or more UEs (i.e., the determination of distance and direction between UEs).

TDOA based solutions on the other hand require (precise) synchronization between anchor UEs, either for the transmission of SL-PRS (analogous to DL-TDOA in Uu positioning) or for the measurement of SL-PRS (analogous to UL-TDOA in Uu positioning). RAN1 discussed synchronization procedures [3],[4],[6], incl. support for SFN/DFN initialization time, different types of synchronization sources, determining RTDs, etc., but left the details up to RAN2. RAN2 had not any discussions on this topic yet, and support for such synchronization procedures (which may or may not involve 5GC (LMF)) may require a similar set of procedures as those available in NRPPa for Uu positioning. Hence, the RAN2 workload can be reduced by prioritizing RTT and AoA type solutions. In addition, SL-TDOA solutions would not support the ranging objective of the WI.

Proposal 1:	Regarding the sidelink positioning methods supported, prioritize RTT and AoA type solutions.

##### Resource allocation for SL-PRS

RAN1 essentially completed the design of SL-PRS resource allocation Scheme 1 (network centric) and Scheme 2 (UE autonomous) for both, shared resource pools and dedicated resource pools. However, RAN2 had just recently started to investigate L2/L3 impacts, but without any significant progress/agreements yet [5]. For scheme 1, the NW schedules SL-PRS resources to be used by the UE for SL-PRS transmission. For scheme 2, the UE autonomously determines SL-PRS resources within the resources configured by the gNB (or pre-configured by the network).

For scheme 1 the UE needs to be in the RRC_CONNECTED state, whereas scheme 2 also works for UEs either in RRC_IDLE state or even out-of-coverage.

This item/objective, although RAN1-centric, has significant MAC and RRC impacts, similar to the sidelink resource allocation modes 1 and 2 for sidelink communication. Prioritizing scheme 2 would reduce the RAN2 workload and would cover most use cases, such as V2X and IIoT.

Proposal 2:	Regarding the resource allocation for SL-PRS, prioritize resource allocation Scheme 2 (UE autonomous).

##### Signalling/procedures to support SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios

(a) SLPP transport (incl. protocol layering, architecture, reliable transport features)

This task seems progressing well in RAN2, with draft Stage 2 and Stage 3 specifications available [7],[8]. The "reliable transport" feature in the draft TS [8] is largely based on LPP and therefore, supports (so far) unicast signalling only (see also item (d) below). A scope reduction seems not needed for this task.

(b) SLPP session handling (incl. session-based and session-less operation)

At RAN2#120, it was agreed that "sidelink positioning supports a session-based concept in SLPP, in which signalling messages within a session can be associated with one another by the involved UEs" and "at least in the case that positioning methods are supported that do not require a mutual exchange of SLPP messages associated with one another among UEs, SLPP session less operation can be supported" [9].

Some progress on session-based operation has been made at RAN2#123 [5], although, some aspects would still require further investigations; in particular in relation to groupcast/broadcast signalling and group positioning (see also item (d) below). Session-less operation on the other hand, has not progressed much in RAN2 yet. Given that session-based operation allows SLPP operation in a structured and coordinated manner (similar to LPP) and is progressing in RAN2, we propose to prioritize session-based operation as baseline.

Proposal 3:	Regarding SLPP session handling, prioritize session based SLPP operation.

(c) SLPP positioning procedures and scenarios

Many inconclusive discussions happened in RAN2 around this general task. Fundamental to sidelink positioning and ranging is UE peer-to-peer operation. This core functionality should be completed first before investigating the benefits of 5GC/LMF involvement, hybrid SL/Uu positioning, relaying, etc. The RAN2 workload can be reduced significantly by de-prioritizing the involvement of 5GC for e.g., service request handling, assistance data distribution, result calculation, etc. The 5GC support can still be included (e.g. based on SA2, CT1 and CT4 support) but RAN2 support is made low priority.

Proposal 4:	Regarding sidelink positioning procedures and scenarios, prioritize PC5-only scenarios.

(d) Cast-types supported for SLPP

At RAN2#119bis-e, it was agreed that "unicast/one-to-one operation is assumed as baseline for exchange of sidelink positioning signalling" [9]. Unicast link establishment requires a sequence of messages between two participating UEs before SLPP signalling can be exchanged. This results in signalling overhead and latency which increases as the number of session participants increases. Unicast operation is clearly suboptimal for many use cases such as V2X.

However, application layer managed groupcast or connection-less groupcast requires proper SLPP session management, since any application layer managed group may result in only a subset of UEs with the ability to join an SLPP session (e.g., may require (among others) SLPP session modification). There is no RAN2 progress on group positioning and further investigations may be required which seem not feasible to complete in the remaining allocated time for this Work Item. For the sake of progress and considering the amount of RAN2 meeting time left for this Release, we support:

Proposal 5:	Regarding cast-types for SLPP signalling, prioritize SLPP support for unicast.

(e) Support for multiple target UEs versus single target UE

At RAN2#122, it was agreed that "SLPP can support multiple target UEs in the same session when LCS requests" [4]. If an SLPP session would be constrained to support only a single target UE, an "LCS request" to perform sidelink positioning for multiple target UEs would require as many SLPP sessions as there are target UEs. Each separate session will require its own capability exchange, assistance data transfer, and location information transfer, resulting in very inefficient positioning operation.

The SLPP design must support multiple UEs and a particular UE role (e.g., anchor UE or target UE) seems agnostic to SLPP. I.e., 'multiple targets UEs' and 'single target UE' have identical functionality with respect to SL-PRS transmission and SL-PRS measurement, where target UE(s) measure SL-PRS from all other (e.g., anchor) UEs whereas non-target UEs (e.g., anchor UEs) only measure SL-PRS from target UE(s). This should not make any difference to SLPP message design, because target UEs and non-target UEs would continue to send/receive the same message types. E.g. the one 'single target UE' can send/receive the same SLPP message types as each of the 'multiple target UEs'.

Therefore, we do not observe any difference in RAN2 workload by limiting SLPP support to a single target UE. However, we can agree that any SLPP impacts needed for multiple target UEs that are not needed for a single target UE are de-prioritized.

Proposal 6:	Regarding Support for multiple target UEs, SLPP can support multiple target UEs in the same session to the extent that no new SLPP impacts are added that are not needed for a single target UE.

(f) SLPP content

This sub-task is expected to be very time consuming for some RAN2 participants and to require significant RAN2 time in a meeting or via Email (e.g. to review an initial SLPP definition) since a whole new protocol must be created with precise and detailed support for RAN1 SL-PRS agreements. However, this does not seem possible to down-scope. But it could be possible to move much of this offline to reduce RAN2 meeting impacts.

## 2.2	Non-Sidelink Positioning Objectives [2]

##### Integrity for NR Positioning

RAN2 is completing the Stage 2 [7] and Stage 3 [10] specification work, and only minor open issues are left. Hence, no scope reduction seems required for this objective.

##### Low Power, High Accuracy Positioning (LPHAP)

This objective has several sub-tasks [2]:

(a) Extending eDRX cycle beyond 10.24s in RRC_INACTIVE

RAN1 agreed that the eDRX cycle lengths supported for eRedCap are also sufficient for positioning in RRC_INACTIVE state [6]. Therefore, no additional work seems needed compared to eRedCap.

(b) SRS configuration enhancements

This objective includes the specification of a validity area for the SRS for positioning configuration in RRC_INACTIVE state,  pre-configuration of one or multiple SRS for positioning configurations, and SRS for positioning activation/request procedures. There is good progress on these objectives and most agreements are in place to finalize Stage 2 and Stage 3 specifications [3],[4],[5].

(c) DL-PRS measurements in RRC_IDLE state

RAN2 concluded that DL-PRS measurements in idle state and reporting the results in connected state is feasible [4]. Some clarifications may be required in Stage 2 specifications, but no Stage 3 impacts are foreseen.

(d) Alignment between (e)DRX and DL-PRS

RAN2 agreed that at least UE-initiated on-demand PRS request procedure is supported for the alignment of the PRS configuration to the fixed (e)DRX configuration [5], which should be possible to complete within the remaining meeting time allocated for this WI.

Therefore, this objective is progressing well and scope reduction seems not required.

##### RedCap UE positioning, PRS bandwidth aggregation, carrier phase positioning

For theses objectives, no RAN2 specific/new design seems required (e.g., new messages or procedures). Support for these features seem to require the implementation of the RAN1 agreed parameter into RAN2 specifications as extensions to the existing NR positioning methods.

Proposal 7:		No scope reduction is required for the non-sidelink related objectives of the WID.

Although, the RedCap UE positioning, PRS bandwidth aggregation, carrier phase positioning would not require the design of new signalling and procedures, the implementation of the RAN1 parameter to support these features may require significant RAN2 time. Also, the completion of sidelink positioning (even with reduced scope) and remaining tasks for integrity and LPHAP will likely consume significant time for RAN2 participants. Hence, RAN2 should limit the discussion of positioning items/enhancements not in scope of this Work Item on 'Expanded and Improved NR Positioning' and/or should not discuss de-prioritized items as part of TEI-18.

Proposal 8:	RAN2 does not consider more positioning TEI-18 proposals.

# 3.	Summary

In this contribution, we provided our view on the scope reduction for the Rel-18 Work Item on "Expanded and Improved NR Positioning" [2]. The following proposals were made.

Proposal 1:	Regarding the sidelink positioning methods supported, prioritize RTT and AoA type solutions.

Proposal 2:	Regarding the resource allocation for SL-PRS, prioritize resource allocation Scheme 2 (UE autonomous).

Proposal 3:	Regarding SLPP session handling, prioritize session based SLPP operation.

Proposal 4:	Regarding sidelink positioning procedures and scenarios, prioritize PC5-only scenarios.

Proposal 5:	Regarding cast-types for SLPP signalling, prioritize SLPP support for unicast.

Proposal 6:	Regarding Support for multiple target UEs, SLPP can support multiple target UEs in the same session to the extent that no new SLPP impacts are added that are not needed for a single target UE.

Proposal 7:		No scope reduction is required for the non-sidelink related objectives of the WID.

Proposal 8:	RAN2 does not consider more positioning TEI-18 proposals.

# References

[1]	RAN2#123 Chair Notes, RAN2 Chairman (MediaTek).

[2]	RP-231460, "Revised WID on Expanded and Improved NR Positioning", Intel Corporation.

[3]	RP-230327, "Status Report to TSG", RAN#99.

[4]	RP-231092, "Status Report to TSG", RAN#100.

[5]	R2-2308964, "Report from session on positioning and sidelink relay", RAN2#123, Session Chair (MediaTek).

[6]	RAN1#114 Chair Notes, RAN1 Ad-Hoc Chair (Huawei).

[7]	R2-2309207, "Running Stage 2 CR for 'Expanded and improved NR positioning'", Qualcomm Incorporated (Rapporteur).

[8]	R2-2309183, "TS 38.355 v0.0.5", Intel Corporation.

[9]	RP-222982, "Status Report to TSG", RAN#98e.

[10]	R2- 2309181, "LPP running CR for RAT-dependent integrity", CATT.