3GPP TSG-RAN WG3 Meeting #121	R3-233866

# Toulouse, France, 21st – 25th August, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0296 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for change of gNB-CU-UP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CPUP_Split-Core |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 38.401 for the Intra-gNB-CU-CP Change of gNB-CU-UP (figure 8.9.5-1), the step 10 Path Update procedure is performed is mentioned, however the text description is as 10-12. Path Switch procedure is performed to update the DL TNL address information for the NG-U towards the core network. This makes implementation difficulty to decide whether to use of NGAP: Path Switch Request procedure or NGAP: PDU Session Resource Modify Indication procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the intra-gNB-CU-CP Change of gNB-CU-UP, to give correction to say the PDU Session Resource Modify Indication procedure is performed. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) on the Intra-gNB-CU change of gNG-CU-UP, on NGAP procedure to use. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confuse which NGAP procedure to use hence cause inter-operability issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change part

8.9.5	Change of gNB-CU-UP

Figure 8.9.5-1 shows the procedure used for the change of gNB-CU-UP within a gNB.

Figure 8.9.5-1: Change of gNB-CU-UP

1.	Change of gNB-CU-UP is triggered in gNB-CU-CP based on e.g., measurement report from the UE.

2-3.	Bearer Context Setup procedure is performed as described in Section 8.9.2.

4.	F1 UE Context Modification procedure is performed to change the UL TNL address information for F1-U for one or more bearers in the gNB-DU.

5-6.	Bearer Context Modification procedure (gNB-CU-CP initiated) is performed to enable the gNB-CU-CP to retrieve the PDCP UL/DL status and to exchange data forwarding information for the bearer.

7-8.	Bearer Context Modification procedure is performed as described in Section 8.9.2.

9.	Data Forwarding may be performed from the source gNB-CU-UP to the target gNB-CU-UP.

10-12.	PDU Session Resource Modify Indication procedure is performed to update the DL TNL address information for the NG-U towards the core network.

13-14.	Bearer Context Release procedure (gNB-CU-CP initiated) is performed as described in Section 8.9.3.

< skip unchanged part >

End of change part