3GPP TSG RAN meeting #101										RP-231926

Bangalore, India, September 11-15, 2023

Status Report to TSG

Agenda item:			9.3.1.8

| WI / SI Name | Network energy savings for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | Netw_Energy_NR |  |  |  |  |
| Unique ID | 981037 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-230566 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 75% (RAN1 100%) | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Yi Wang |
|  | Company | Huawei |
|  | Email | wangyi6@huawei.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

##### RAN1#114

Conclusion

- No further work on BM enhancements for R18 NES.

- No further work on TCI configuration enhancement for R18 NES.

Conclusion

- No further enhancements for PMI reduction in R18 NES.

- No further enhancements for RI reduction in R18 NES.

- No support of UE reporting PDSCH power reduction tolerance in R18 NES.

Agreement

For SD and/or PD adaptation without UE complexity reduction, CPU counting of A/SP-CSI reporting is based on OCPU=i=1NKsi for CSIs reporting corresponding to N indicated sub-configurations from L configured sub-configurations in a CSI report.

Agreement

For sub-configuration triggering of A-CSI, an indication for N sub-configurations out of L sub-configurations for a triggering state is configured in CSI-AssociatedReportConfigInfo.

- No change to current CSI request field in DCI.

Conclusion

There is no consensus to support the following:

Option 1: support indication of spatial and/or transmission power adaptation in one of the following approaches (same approach for SD and PD adaptation) in addition to the agreed triggering/activation signalling

- Alt 1: MAC-CE/RRC for indication of corresponding subConfig ID that gNB has applied as adaptation

  - Note: need to take this RAN2 LS in R1-2306380 into account

- Alt 2: UE specific DCI

  - A new field in existing non-fallback UE specific DCI formats is introduced

    - If agreed, the number of bits are to be discussed at CR stage

Agreement

- For each sub-configuration in a CSI reportConfig, for Type 1 SD adaptation only, and Type 2 SD adaptation only, support,

  - {codebookConfig (for Type 2 SD only) is common for all sub-configurations

  - {reportQuantity, reportFreqConfiguration} is not configured in any sub-configuration and the legacy/original parameters are used for all sub-configurations.

  - cqi-Table is common for all sub-configurations

  - for indicating # of ports in a port subset = 2, legacy IE twoTX-CodebookSubsetRestriction can be used for this subConfig in Type 1 SD.

Conclusion

No simultaneous configuration of Type 1 SD and Type 2 SD adaptation in a same CSI report configuration

Agreement

- For power domain adaptation only, all CSI-RS resource(s) (which can be one or more) in the CSI-RS resource set for channel measurement are associated with each sub-configuration provided in a CSI report configuration

- Each sub-configuration contains an offset value (e.g. X) that is commonly applied to all the resources within the resource set. For a CSI-RS resource in CSI resource configuration, the PDSCH to CSI-RS EPRE offset (e.g. Y) for CSI calculation is determined based on powerControlOffset (e.g. Z) value in CSI resource configuration and the offset value configured in CSI sub-configuration in the report configuration.

  - Only legacy values are applicable for the resulted power control offset values

  - It is expected that the sub-configuration leads to a value no larger than power control offset value provided in CSI resource configuration

Agreement

For joint operation of SD and PD, each subConfig contains corresponding parameters for an SD adaptation and/or parameters for a PD adaptation

Agreement

For sub-configuration triggering of SP-CSI on PUSCH report, an indication for N sub-configurations out of L sub-configurations for a triggering state is configured in CSI-SemiPersistentOnPUSCH-TriggerState.

- No change to current CSI request field in DCI.

Agreement

Down-select from the below for priority rule determination for CSI reporting of multiple sub-configurations

- Option 1: The priority of the CSI report containing CSIs for multiple sub-configurations, is determined according to the clause 5.2.5 of TS 38.214.

  - 1-b) A sub-configuration level priority is determined by the order of sub-configuration index. For Part 2 CSI corresponding to each sub-configuration, omission is at subConfig level. Follow legacy dropping rules for a CSI report containing multiple CSIs.

    - CSI mapping rule across sub-configurations follow legacy specification principle

    - Sub-configuration index with lower value has higher priority

    - Sub-configuration index is configured in CSI report config

Agreement

For CSIs across multiple sub-configurations in one CSI reportConfig map different sub-configurations based on RAN1#114 agreement in 9.7.1

- For Part 2 priority reporting level

  - Option 1: for a given band type from {wideband, even subband, odd subband}, the omission order follows the priority order determined by sub-configuration index

Agreement

For N(>1) CSIs reporting with multiple sub-configurations without payload/complexity reduction,

- Each CSI can be a single-part, or two-part CSI, and contains the same types of CSI parameters/quantities as legacy, when applicable/if reported;

- The mapping order of CSI fields of one sub-configuration is as legacy mapping order of CSI fields of one CSI report;

- Part 2 CSI priority reporting level follows wideband CSI first, then even subband CSI and odd subband CSI;

Agreement

For a CSI report configuration containing sub-configuration(s), if a CSI-RS resource is referred by M sub-configurations among X sub-configurations, the CSI-RS resource is counted M times and CSI-RS ports within the CSI-RS resource are counted by

- Option 2A: maxs=1MPs, P for Type 1 SD adaptation, and M×P for Type 2 SD or PD adaptation.

- P is nrofPorts configured in NZP-CSI-RS-Resource and Ps is the number of CSI-RS ports in sub-configuration s derived from port subset indication.

- It is understood that further discussions are necessary

Agreement

For Type 1 SD for multi-panel case,

- Introduce a new mixed codebook combination {Type 1 Single Panel, Type 1 Multi Panel, Null} in R18 for FG codebookComboParameterAddition (indicating the UE supports the mixed codebook combinations in a slot)

- Note: gNB can configure either Type 1 single panel codebook or Type 1 multi-panel codebook for a sub-configuration from one or multiple sub-configurations within one CSI report configuration if a UE reports support of multi-panel operation.

Agreement

DCI format 2_X, for activation and deactivation of cell DTX and DRX configuration,

- at least includes following fields,

  - N information block field(s),

  - Spare/reserved padding bits to match the size configured for DCI 2_X (if needed)

- payload size is configurable and within the bounds set by existing RAN1 specification

- an information block field contains signaling of activation or deactivation of ‘a configuration of cell DTX and/or DRX’ of ‘a serving cell’

- for serving cell configured with SUL, the same bit is applicable for both NUL and SUL

Above applies at least for sTRP case.

Agreement

For at least the case where one cell DTX/DRX pattern is configured, an information block field of DCI format 2_X for activation and deactivation of cell DTX and DRX configuration supports the following:

- Separate (activation/deactivation) signaling for cell DTX and cell DRX, i.e. one activation/deactivation signaling sub-field for cell DTX configuration and one activation/deactivation signaling sub-field for cell DRX configuration

  - Separate 1 bit indication for each of activation/deactivation for one cell DTX and one cell DRX

Above does not imply that multiple DTX/DRX patterns is not supported.

Agreement

Support new RNTI (e.g. nes-RNTI) which is configured by higher layer, for scrambling of DCI format 2_X

Agreement

From RAN1 point of view, DCI format 2_X supports activation/deactivation of cell DTX/DRX configuration of multiple serving cells and support activation/deactivation per cell

- UE monitor DCI format 2_X in one serving cell

Agreement

Delay that is applied after DCI Format 2_X reception that activate/deactivate cell DTX/DRX configuration is introduced in Rel-18.

Agreement

DCI format 2_X is monitored in the common search space

Note: Search space set configuration for DCI format 2_X is separately provided by higher layers

Agreement

The following high layer signaling are to be included to the RRC parameter list for new DCI format 2_X for activation and deactivation of cell DTX/DRX

- search space set configuration with new DCI format 2_X

- DCI size for new DCI format 2_X

Agreement

- An information block field of DCI format 2_X is variable size either 1 or 2 bits.

  - Higher layer signaling configures whether the activation/deactivation of cell DTX and/or cell DRX is indicated in DCI format 2_X for a serving cell.

    - If both cell DTX and cell DRX are configured for a serving cell,

      - 1st bit corresponds to activation/deactivation of cell DTX configuration, and

      - 2nd bit corresponds to activation/deactivation of cell DRX configuration,

    - otherwise, the 1 bit corresponds to the configured cell DTX or cell DRX configuration.

  - Note: this does not imply there may be separate higher layer signaling to enable L1 signaling based activation/deactivation for a cell DTX and/or cell DRX configuration. Signaling design is up to RAN2.

Agreement

For each serving cell configured with L1 signaling based activation/deactivation of cell DTX and/or cell DRX configuration, starting bit position of an information block of DCI format 2_X is provided by UE specific higher layer signaling.

Agreement

- UE is expected to apply cell DTX or DRX activation/deactivation change at beginning of the slot X where the SCS of slot X is with respect to the active DL or UL BWP of the serving cell, respectively.

- Slot X is the first slot whose beginning is no earlier than (i.e., same or after) beginning of slot n + D, where D is the delay and n is the slot containing the PDCCH of DCI format 2_X based on SCS of PDCCH.

| SCS of PDCCH (kHz) | Value of D (in unit of slot) |
| --- | --- |
| 15 | 3 |
| 30 | 6 |
| 60 | 12 |
| 120 | 24 |
| 480 | 96 |
| 960 | 192 |

Agreement

Rel-18 UE supporting cell DTX is not required to monitor the following signals/channels from the gNB, during non-active periods of cell DTX

- PDCCHs associated with DCI format 2_0 – DCI Format 2_5

Conclusion:

- HARQ-ACK of SPS PDSCH transmitted is not impacted by non-active period of cell DRX.

Agreement

For the FFS from agreement from RAN1 #112bis

- SRS for positioning is not impacted by cell DRX operation.

Conclusion

- The following channels are not impacted by non-active period of cell DRX

  - HARQ-ACK of a DCI format without scheduling a PDSCH

2.1.2	Remaining Open issues

None.

2.2	RAN2

2.2.1	Agreements

##### RAN2#123

Cell DTX/DRX:

Agreements:

1	Activation/deactivation is per serving cell.  FFS if the configuration is per cell or per MAC entity

2	RAN2 will reuse the start timer formula of the onDurationTimer from UE C-DRX (including SlotOffset) to specify the start of cellDTX-onDurationTimer (and cellDRX-onDurationTimer) in 38.321.

3	The gNB should ensures that there is at least partial overlapping between UE C-DRX on-duration and cell DTX/DRX on-duration.  It is up to network implementation to ensure the alignment.  We will capture this in stage 2 specification.

Understanding is that alignment means that the cell DTX/DRX and C-DRX periodicity should be multiple of each other.   FFS if we anything needs to be specified in stage 3 (i.e. in IE description)

4	As a baseline legacy C-DRX reconfiguration is used to change UE C-DRX configuration once Cell DTX/DRX is activated/deactivated.

5	RAN2 specifies cellDTX-onDurationTimer (and cellDRX-onDurationTimer) to have the same value range as UE C-DRX on-duration timer.

6	RAN2 specifies cellDTX-Cycle (and cellDRX-Cycle) to have the same value range as UE C-DRX Long cycle.

7	Separate DTX and DRX configuration means that the features can be enabled separately (i.e. Cell DTX can be configured without Cell DRX)

8	On-duration and Cycle parameters are common between cell DTX and DRX, when both are configured.  FFS if we have different start offset configuration for cell DTX and cell DRX

9	RAN2 will not introduce a MAC CE for cell DTX/DRX (de)activation.

10	Confirm working assumption, when the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

11	We focus on the case where DTX in RRC can only be configured when C-DRX is configured.  We will not optimize for the case where C-DRX is not configured.

Cell selection

Agreements

-	One single bit in SIB1 is introduced for controlling all “NES-capable UEs” to access a cell.  FFS what “NES capable UE” bit means.  The NES UE always follows the NES bit used for barring, if present.  If not present the UE shall follow legacy barring.

-	No new cell baring techniques for non-NES UEs will be specified.

-	No new cell re-selection techniques will be considered in this Rel-18

Connected mode mobility:

Agreements

1	We will support the CHO triggers for the use case of turning off the cell

2	(At least for cell DTX/DRX) Time-based CHO is not to be considered in NES.

3	Do not consider using an indication in SIB1 for triggering NES CHO execution condition

2.2.2	Remaining Open issues

Necessary discussion for cell DTX/DRX (e.g. whether the configuration is per cell or per MAC entity), SSB-less and fast SCell activation (e.g. stage-3 details), CHO enhancement (e.g. signalling for triggering NES specific CHO execution condition), and other details (e.g. RAN2 impacts of spatial/power domain techniques).

2.3	RAN3

2.3.1	Agreements

##### RAN3#121

- BLCRs/TPs

- BLCRs are endorsed in R3-233798 and R3-233804.

- TPs are agreed in R3-234668, R3-234669 and R3-234771.

- Editor notes in F1AP and XnAP

- Maximum SSB areas is 64

- In F1AP BLCR, keep the criticality “reject” for the SSBs within the cell to be Activated List IE included in the GNB-CU CONFIGURATION UPDATE message, and remove the FFS.

- Inter-node beam activation

- Introduce a new cause value “SSB not Available” in XnAP and F1AP to indicate that none of the requested SSB beam(s) can be successfully activated.

- Paging enhancement

- Take the texts in the SoD as baseline, and send the draft reply LS to SA2 in Oct. meeting when we receive RAN2 reply LS.

- Working assumption: To support CN paging enhancement, the recommended SSB list for Paging could be transferred in the NGAP UE context release complete message, and sent back in the NGAP Paging message in the transparent container

- Cell DTX/DRX

- WA: Support the exchange of the Cell DTX/DRX configuration over Xn.

2.3.2	Remaining Open issues

Restricting paging in a limited area for RRC idle UEs, and reply LS to SA2.

Cell DTX/DRX exchange over Xn interfaces, e.g., activation/deactivation status exchange.

Editor’s note in the BLCRs, and potential specification impact based on the other WG progress.

2.4	RAN4

2.4.1	Agreements

##### RAN4#108

###### BS RF part

Agreement

A WF (R4-2314935) was approved.

Issue 1-1: TAE

Agreement:

- Alternative #1:

  - Do not specify the BS TAE requirements of SSB-less operation for FR1 co-located inter-band CA

  - Define the side condition of RTD to ensure UE performance in RRM part.

- Alternative #2:

  - Specify BS TAE requirements

    - FFS on values

Issue 1-4: other UE related considerations

Agreement:

- No new UE RF requirements are defined for R18 NES feature.

###### RRM part

Issue 1-2-1: RTD conditions for scenario 1

- Agreements

    - Define requirements for RTD ≤ CP

      - note: the CP corresponding to the [largest SCS across CCs or SCell SCS]

    - FFS if additional requirements shall be considered for RTD ≤ 260ns

    - Performance requirements (RRM and/or demodulation) need to be introduced for the scenario with RTD ≤ CP to guarantee proper performance on the SCell

Issue 2-1-2: RRM impacts of Cell DTX/DRX – SSB based measurement

Agreement:

RAN4 to reuse the legacy requirement for SSB-based measurement in Cell DTX/DRX

Issue 2-3: RRM impacts of Cell selection/reselection (obj#4)

Agreements:

Obj#4 (mechanism to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques) has no RRM impact for objective#4.

Issue 2-4: RRM impacts of CHO (obj#5)

Agreements:

RAN4 shall wait for more progress of RAN2 on CHO to investigate whether there is RRM impacts.

2.4.2	Remaining Open issues

Open issues are captured in WF R4-2314381.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

##### RAN1#114

| R1-2306472 | Techniques in spatial and power domains | Nokia, Nokia Shanghai Bell |
| --- | --- | --- |
| R1-2306473 | Enhancements on cell DTX/DRX mechanism | Nokia, Nokia Shanghai Bell |
| R1-2306504 | CSI enhancements for network energy saving | Huawei, HiSilicon |
| R1-2306505 | Cell DTX/DRX mechanism for network energy saving | Huawei, HiSilicon |
| R1-2306552 | Higher layer signalling for Rel-18 NES | Huawei, HiSilicon |
| R1-2306575 | Discussions on network energy savings techniques in spatial and power domains | Ruijie Network Co. Ltd |
| R1-2306577 | Discussions on enhancements on cell DTX/DRX mechanism | Ruijie Network Co. Ltd |
| R1-2306624 | Spatial and Power Adaptations for Network Energy Savings | FUTUREWEI |
| R1-2306625 | Cell DTX/DRX for NES | FUTUREWEI |
| R1-2306657 | Discussion on NES techniques in spatial and power domains | Spreadtrum Communications |
| R1-2306658 | Discussion on enhancements on cell DTXDRX mechanism | Spreadtrum Communications |
| R1-2306762 | Discussions on NES techniques in spatial and power domain | vivo |
| R1-2306763 | Discussions on enhancements on cell DTX/DRX mechanism | vivo |
| R1-2306802 | Spatial and power domain adaptation for network energy saving | Panasonic |
| R1-2306803 | Cell DTX/DRX enhancement for network energy saving | Panasonic |
| R1-2306828 | Discussion on NWES techniques in spatial and power domain | Intel Corporation |
| R1-2306829 | Discussion on enhancements on cell DTX/DRX mechanism | Intel Corporation |
| R1-2306946 | Discussion on network energy saving techniques in spatial and power domains | NEC |
| R1-2306948 | Cell DTX/DRX Configuration for Network Energy Saving | NEC |
| R1-2306963 | Network Energy Saving in Spatial and Power Domain | Google |
| R1-2306964 | Network Energy Saving on Cell DTX and DRX | Google |
| R1-2307099 | Network Energy Saving techniques in spatial and power domain | CATT |
| R1-2307100 | DTX/DRX for network Energy Saving | CATT |
| R1-2307139 | Discussion on NES techniques in spatial and power domains | ZTE, Sanechips |
| R1-2307140 | Discussion on cell DTX/DRX | ZTE, Sanechips |
| R1-2307162 | Discussion on NW energy saving techniques in spatial and power domains | Fujitsu |
| R1-2307163 | Discussion on cell DTX/DRX mechanism | Fujitsu |
| R1-2307207 | Discussion on spatial and power domains enhancements for network energy saving | CMCC |
| R1-2307208 | Discussion on cell DTX/ DRX mechanism | CMCC |
| R1-2307290 | Discussion on remaining issues for spatial and power domain enhancements to support network energy saving | Apple |
| R1-2307291 | Discussion on remaining issues for cell DTX/DRX mechanism | Apple |
| R1-2307396 | Discussion on techniques in spatial and power domains | xiaomi |
| R1-2307397 | Discussions on cell DTX-DRX for network energy saving | xiaomi |
| R1-2307404 | On network energy saving techniques in spatial and power domains | KT Corp. |
| R1-2307483 | Discussion on spatial and power domain enhancements for NW energy savings | NTT DOCOMO, INC. |
| R1-2307484 | Discussion on enhancements on Cell DTX/DRX mechanism | NTT DOCOMO, INC. |
| R1-2307540 | Discussion on techniques in spatial and power domains | OPPO |
| R1-2307541 | Discussion on enhancements on cell DTX/DRX mechanism | OPPO |
| R1-2307623 | Discussion on spatial and power domain CSI adaptation for NES | China Telecom |
| R1-2307624 | Discussion on the mechanism of Cell DTX/DRX for NES | China Telecom |
| R1-2307690 | Remaining Issues on NES techniques in spatial and power domains | Samsung |
| R1-2307691 | Remaining issues on cell DTX/DRX mechanism | Samsung |
| R1-2307748 | Discussion on NES techniques in spatial and power domains | ETRI |
| R1-2307749 | Discussion on cell DTX/DRX mechanism | ETRI |
| R1-2307765 | Discussion of NES techniques in spatial domain and power domain | Transsion Holdings |
| R1-2307766 | Discussion on Enhancement on cell DTX DRX mechanism | Transsion Holdings |
| R1-2307779 | Discussion on TCI state and beam management for NES spatial and power domain adaptation | FGI |
| R1-2307792 | Discussion on NES techniques in spatial and power domains | LG Electronics |
| R1-2307793 | Discussion on cell DTX/DRX mechanism | LG Electronics |
| R1-2307813 | Network energy saving techniques in spatial and power domains | Lenovo |
| R1-2307814 | Enhancements on cell DTX/DRX mechanism | Lenovo |
| R1-2307821 | Discussion on techniques in spatial and power domains | InterDigital, Inc. |
| R1-2307822 | Discussion on enhancements on cell DTX/DRX mechanism | InterDigital, Inc. |
| R1-2307838 | Considerations of Cell DTX/DRX Signaling and Reliability | Fraunhofer IIS, Fraunhofer HHI |
| R1-2307891 | Discussion on NES techniques in spatial and power domain | ITRI |
| R1-2307938 | Techniques in spatial and power domains | Qualcomm Incorporated |
| R1-2307939 | Enhancements on cell DTX and DRX mechanism | Qualcomm Incorporated |
| R1-2307984 | Spatial Domain Adaptation for NES | Fraunhofer IIS, Fraunhofer HHI |
| R1-2307987 | NW energy saving techniques in spatial and power domains | Ericsson |
| R1-2307988 | RAN1 aspects of cell DTX/DRX | Ericsson |
| R1-2308008 | Discussion on spatial and power adaptations for network energy savings | CEWiT |
| R1-2308009 | Discussion on cell DTX/DRX mechanism for network energy saving | CEWiT |
| R1-2308081 | On NES Techniques in spatial and power domains | MediaTek Inc. |
| R1-2308082 | On enhancements of cell DTX/DRX mechanism | MediaTek Inc. |
| R1-2308105 | Discussion on spatial/power domain adaptation | ASUSTeK |
| R1-2308106 | Discussion on cell DTX/DRX | ASUSTeK |
| R1-2308250 | FL summary#1 for SD and PD adaptation for R18 NES | Moderator (Huawei) |
| R1-2308251 | FL summary#2 for SD and PD adaptation for R18 NES | Moderator (Huawei) |
| R1-2308252 | FL summary#3 for SD and PD adaptation for R18 NES | Moderator (Huawei) |
| R1-2308253 | FL summary#4 for SD and PD adaptation for R18 NES | Moderator (Huawei) |
| R1-2308350 | Summary of issues for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2308351 | Discussion summary #1 for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2308352 | Discussion summary #2 for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2308496 | Discussion summary #3 for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2308497 | Discussion summary #4 for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2308599 | FL summary#5 for SD and PD adaptation for R18 NES | Moderator (Huawei) |
| R1-2308600 | Final summary for SD and PD adaptation for R18 NES | Moderator (Huawei) |
| R1-2308620 | Discussion summary #5 for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2308642 | Comments collection for RRC parameters for R18 NES | Moderator (Huawei) |

##### RAN2#123

| R2-2307017 | Reply LS on Cell DTX/DRX activation/deactivation (R1-2306246; contact: Huawei, Intel) | RAN1 |
| --- | --- | --- |
| R2-2307053 | Reply LS on 3GPP work on Energy Efficiency (S1-231805; contact: Nokia) | SA1 |
| R2-2307063 | Reply LS on the enhancements to restricting paging in a limited area (S2-2307984; contact: Qualcomm) | SA2 |
| R2-2307073 | LS on 3GPP work on Energy Efficiency (S4-231111; contact: Qualcomm) | SA4 |
| R2-2307147 | Joint Operation between UE C-DRX and Cell DTX | NEC |
| R2-2307149 | Discussion on RRC_IDLE/INACTIVE UE behavior due to NES | Xiaomi |
| R2-2307150 | Discussion on Cell DTX/DRX configuration and operation | Xiaomi |
| R2-2307151 | Discussion on UE mobility due to NES cell | Xiaomi |
| R2-2307152 | Discussion on inter-band SSB-less SCell | Xiaomi |
| R2-2307177 | Remaining issues for Cell DTX_DRX | Samsung Electronics Co., Ltd |
| R2-2307178 | Various (RRC Procedure, Measurement, SR, CG etc.) alignment aspects | Lenovo |
| R2-2307179 | CHO Procedure in NES Mode | Lenovo |
| R2-2307220 | Discussion on cell selection and reselection for NES | Samsung Electronics GmbH |
| R2-2307221 | Discussion on Connected mode mobility for NES | Samsung Electronics GmbH |
| R2-2307456 | Discussion on CHO for NES | Ericsson |
| R2-2307458 | MAC CE for activating/deactivating SP CSI report configurations for NES | Ericsson |
| R2-2307475 | Discussion on the CHO Enhancements for NES | Google Inc. |
| R2-2307519 | On NES SSB-less SCell operation | Nokia, Nokia Shanghai Bell |
| R2-2307524 | Discussion on cell DTX and DRX | Huawei, HiSilicon |
| R2-2307525 | Discussion on cell selection/reselection for NES | Huawei, HiSilicon |
| R2-2307527 | Outcome of [Post122][307][NES] DTX/DRX – alignment, single/multiple configurations, parameter values (Huawei) | Huawei, HiSilicon |
| R2-2307528 | Running 38.331 CR - Introduction of Network energy savings for NR | Huawei, HiSilicon |
| R2-2307645 | Discussion on CHO enhancement | NEC |
| R2-2307647 | Cell DTX-DRX Mechanism | Qualcomm Incorporated |
| R2-2307648 | SSB-less Interband CA | Qualcomm Incorporated |
| R2-2307649 | NES Connected mode mobility | Qualcomm Incorporated |
| R2-2307650 | Restricting Paging to limited area | Qualcomm Incorporated |
| R2-2307713 | Cell DTX/DRX NES Techniques | III |
| R2-2307717 | Discussion on cell DTX/DRX | KDDI Corporation |
| R2-2307760 | SPS/CG and MAC CE Activation/Deactivation in Cell DTX/DRX | Samsung |
| R2-2307765 | Reselection and Paging handling for NES | Nokia, Nokia Shanghai Bell |
| R2-2307766 | CHO on NES | Nokia, Nokia Shanghai Bell |
| R2-2307787 | Remaining issues on Cell DTX/DRX | Nokia, Nokia Shanghai Bell |
| R2-2307808 | Discussion on key issues of Cell DTX/DRX | Apple |
| R2-2307809 | Detailed mechanism of legacy UE barring in NES cell | Apple |
| R2-2307810 | Further Discussion on RAN2 work of inter-band SSB-less CA | Apple |
| R2-2307811 | Further Discussion on CHO enhancement in NES | Apple |
| R2-2307897 | TAT aspects under Cell-DTX/Cell-DRX | FGI |
| R2-2307898 | Discussion on cell DTX and DRX | FGI |
| R2-2307911 | Single configuration with multiple DTX_DRX patterns and explicit signalling | BT Plc, Nokia, T-Mobile, Deutsche Telekom, NTT Docomo, KDDI, ZTE |
| R2-2307917 | Discussion on CHO enhancements for NES | Sharp |
| R2-2308026 | Discuss on SSB-less SCell operation in NES | Lenovo |
| R2-2308027 | Cell selection/re-selection in NES | Lenovo |
| R2-2308031 | Discussion on CHO enhancement for NES | Huawei, Turkcell, HiSilicon |
| R2-2308044 | Work plan for NR network energy savings | Huawei, HiSilicon |
| R2-2308045 | Discussion on RAN1 and RAN3 led NES techniques | Huawei, HiSilicon |
| R2-2308154 | Skip monitoring of CSI-RS during non-active periods | Sony |
| R2-2308175 | Handover enhancement for NES | Sony |
| R2-2308179 | Discussion on DTX/DRX mechanism | OPPO |
| R2-2308180 | Discussion on SSB-less Scell operation | OPPO |
| R2-2308181 | Discussion on cell selection reselection | OPPO |
| R2-2308182 | Discussion on connected mode mobility | OPPO |
| R2-2308198 | CHO enhancement for NES cell | Quectel |
| R2-2308279 | The remaining issues on cell DTX and DRX | ZTE corporation, Sanechips |
| R2-2308280 | Discussion on SSB-less SCell operation for NES | ZTE corporation, Sanechips |
| R2-2308281 | Consideration on cell selection and reselection enhancements for NES | ZTE corporation, Sanechips |
| R2-2308282 | The remaining issues on connected mode mobility | ZTE corporation, Sanechips |
| R2-2308301 | Discussion on cell DTXDRX | CMCC |
| R2-2308302 | Discussion on connected mode mobility | CMCC |
| R2-2308333 | Discussion on cell barring and reselection for NES | CMCC |
| R2-2308359 | On SBB/SIB-less NES solutions | Dell Technologies |
| R2-2308388 | Remaining issues on Cell DTX/DRX | InterDigital |
| R2-2308389 | Cell selection and resection for NES | InterDigital |
| R2-2308390 | CHO for NES | InterDigital |
| R2-2308391 | SSB-less Scell operation | InterDigital |
| R2-2308393 | Running CR to 38.321 for Network Energy Savings | InterDigital |
| R2-2308394 | TS 38.321 open issues for NES | InterDigital |
| R2-2308448 | UL considerations for Cell DTX/DRX | NEC Telecom MODUS Ltd. |
| R2-2308449 | Procedure for legacy UEs camping on NES cells | NEC Telecom MODUS Ltd. |
| R2-2308492 | Running CR to 38300 for Network energy savings | Ericsson |
| R2-2308493 | Further aspects on cell DTX/DRX | Ericsson |
| R2-2308494 | Remaining aspects for NES Cell selection/reselection | Ericsson |
| R2-2308533 | Cell Selection and Re-Selection for NES | Fraunhofer IIS, Fraunhofer HHI |
| R2-2308534 | Cell DTX/DRX traffic adaptations | Fraunhofer IIS, Fraunhofer HHI |
| R2-2308593 | Discussion on DTX/DRX mechanism | LG Electronics Inc. |
| R2-2308606 | Discussion on Cell DTX/DRX | Fujitsu |
| R2-2308608 | Discussion on Connected mode mobility for network energy savings | Fujitsu |
| R2-2308681 | Access restriction | LG Electronics |
| R2-2308697 | CHO enhancement for different NES use cases | LG Electronics Inc. |
| R2-2308703 | Support multiple configuration of Cell DTX/DRX | ETRI |
| R2-2308727 | Leftover issues of Cell DTX/DRX | CATT, Turkcell |
| R2-2308728 | Enhancements on SSB-less SCell operation | CATT |
| R2-2308729 | Consideration on Cell Selection/Re-selection on NES cells | CATT |
| R2-2308730 | CHO procedure enhancements | CATT |
| R2-2308737 | Remaining issues for cell DTX-DRX | vivo |
| R2-2308738 | UE C-DRX configuration used upon cell DTX activation | vivo, Fraunhofer IIS, Huawei, HiSilicon, LG Electronics Inc., MediaTek Inc., NEC, Xiaomi |
| R2-2308739 | Discussion on cell selection/re-selection | vivo |
| R2-2308740 | Conditional handover enhancement for network energy saving | vivo |
| R2-2308784 | Access restrition for NES | LG Electronics Inc. |
| R2-2308794 | Further aspects on cell DTX/DRX | Ericsson |
| R2-2308831 | Discussion on Cell selection | NTT DOCOMO INC. |
| R2-2308886 | Cell DTX/DRX NES Techniques | CEWiT |

##### RAN3#121

| R3-234668 | (TP to Netw_Energy_NR BLCR for TS 38.423) Network energy saving techniques | Huawei, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, Qualcomm Incorporated, Samsung, CATT |
| --- | --- | --- |
| R3-234771 | (TP to Netw_Energy_NR BLCR for TS 38.300) Introduction of Network Energy Saving | ZTE, Huawei, Ericsson, Nokia, Nokia Shanghai Bell |
| R3-234669 | (TP to Netw_Energy_NR BLCR for TS 38.473) Introduction of Network Energy Saving | Ericsson |
| R3-233804 | (BLCR to 38.473) Introduction of Network Energy Saving | Ericsson, Huawei, Samsung, ZTE, CATT, Intel |
| R3-233798 | (BLCR to 38.423) Network energy saving techniques | Huawei, Samsung, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, CATT, Intel |
| R3-233990 | WI Work plan for R18 network energy savings | Huawei |
| R3-233991 | (TP to Netw_Energy_NR BLCR for TS 38.423) Network energy saving techniques | Huawei |
| R3-233992 | (TP to Netw_Energy_NR BLCR for TS 38.473, 38.413, 38.470 and 38.300) Network energy saving techniques | Huawei |
| R3-234134 | (TP to TS 38.423) Support of network energy saving techniques | Nokia, Nokia Shanghai Bell |
| R3-234135 | (TP to TS 38.473) Beam deactivation decision and signalling for energy saving | Nokia, Nokia Shanghai Bell |
| R3-234426 | left issues on network energy saving | ZTE |
| R3-234427 | TPs to BL CRs for network energy saving | ZTE |
| R3-234670 | (TP to Netw_Energy_NR BLCR for TS 38.300) Introduction of Network Energy Saving | ZTE, Huawei, Ericsson, Nokia, Nokia Shanghai Bell |
| R3-234193 | Discussion on NES related issues | Lenovo |
| R3-234319 | Introduction of Network Energy Saving | Ericsson |
| R3-234320 | Text Proposal on F1AP: Introduction of Network Energy Saving | Ericsson |
| R3-233708 | Reply LS on Cell DTX/DRX activation/deactivation | RAN1(Huawei/Intel) |
| R3-233729 | Reply LS on the enhancements to restricting paging in a limited area | SA2(Qualcomm) |
| R3-233930 | Inter-node beam activation and paging enhancements | Qualcomm Incorporated |
| R3-234308 | Discussion on Cell DTX/DRX for NES | CATT |
| R3-234309 | (TPs to TS38.423/TS38.473 BL CR) Discussion on new cause and paging enhancement for NES | CATT |
| R3-233944 | Discussion on network energy saving | Samsung |
| R3-233945 | Introduction of Network Energy Saving for Paging IDLE UE | Samsung |
| R3-234614 | Summary of offline discussions on CB: # R18ES | Huawei |

##### RAN4#108

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2311165 | Discussion on RRM requirements of SSBless inter-band CA | Qualcomm Incorporated |
| R4-2311326 | On RRM requirements for NES | Apple |
| R4-2311327 | On SSB-less SCell operation for NES | Apple |
| R4-2311400 | Discussion on BS requirements for inter-band CA with SSB-less operation | Fujitsu Limited |
| R4-2311447 | Discussion on BS RF requirement of network Energy saings | Huawei, HiSilicon |
| R4-2311557 | RF requirements for Network energy savings for NR | Nokia, Nokia Shanghai Bell |
| R4-2311575 | SSB-less operation for FR1 inter-band co-located CA | Nokia, Nokia Shanghai Bell |
| R4-2311576 | Simulation results on SSB-less operation | Nokia, Nokia Shanghai Bell |
| R4-2311577 | RRM requirements impact due to Cell DTX/DRX | Nokia, Nokia Shanghai Bell |
| R4-2311623 | Discussion on RRM requirements for SSB-less SCell operation | CATT |
| R4-2311644 | Discussion on the UE RF impact and BS TAE requirement for NES | CATT |
| R4-2311680 | Discussion on RRM requirements for Network energy saving for NR | MediaTek inc. |
| R4-2311699 | Discussion on TAE requirements for network energy saving | NEC |
| R4-2311801 | Discussion on RRM impact on network energy saving | CMCC |
| R4-2311802 | Discussion on network energy saving RF requirements | CMCC |
| R4-2312284 | Discussion on NES RRM requirement | LG Electronics Inc. |
| R4-2312338 | NR network energy saving RRM aspects - SSB-less Scell | Intel Corporation |
| R4-2312339 | NR network energy saving RRM aspects - other aspects | Intel Corporation |
| R4-2312340 | Discussion on TAE requirements for SSB-less SCell operations | Intel Corporation |
| R4-2312355 | discussion on RRM requirements for NR network energy saving | Samsung |
| R4-2312420 | Discussion on RRM requirements for NES | Huawei, HiSilicon |
| R4-2312431 | Discussion on SSB-less SCell operation | Huawei, HiSilicon |
| R4-2312543 | RF aspects on SSB-less SCell operation for FR1 inter-band CA | China Telecom |
| R4-2312544 | RRM requirements on SSB-less SCell operation for FR1 inter-band CA | China Telecom |
| R4-2312589 | Discussion on RF architecture for inter-band SSB-less Scell operation | vivo |
| R4-2312910 | Discussion on NES UE RF impact | Ericsson |
| R4-2312911 | Discussion on NES BS RF impact | Ericsson |
| R4-2313117 | Discussion on SSB-less SCell operation of Network energy saving for NR | ZTE Corporation |
| R4-2313175 | Discussion on BS RF requirement impacts from NES perspective | ZTE Corporation |
| R4-2313176 | Discussion on UE RF requirement impacts from NES perspective | ZTE Corporation |
| R4-2313304 | Discussion on other RRM requirement impacts for network energy saving | vivo |
| R4-2313305 | Discussion on SSB-less SCell operation for network energy saving | vivo |
| R4-2313384 | Discussion on NES UE RF requirements | LG Electronics UK |
| R4-2313812 | On SCell activation procedures for SSB-less inter-band SCell in FR1 | Ericsson |
| R4-2314179 | Topic summary for [108][234] Netw_Energy_NR | Moderator (Huawei) |
| R4-2314233 | Topic summary for [108][151] Netw_Energy_NR | Moderator (Huawei) |
| R4-2314935 | WF on RF requirements for NES | Huawei |
| R4-2314344 | Ad-hoc minutes for NR Network Energy Saving RRM requirements | Huawei |
| R4-2314381 | WF on Network Energy Saving RRM requirements | Huawei |

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template