3GPP TSG RAN Meeting #101	RP-231931

Bangalore, September 11 – 15, 2023

Agenda Item:	8A.2.7

Source:	Huawei, HiSilicon

Title:	NTN enhancements in Rel-19

Document for:	Discussion

# Introduction

In Rel-19 workshop, a large number of different enhancements for NTN (Non-Terrestrial Networks) were proposed, including DL/UL coverage enhancement (Rel-18 focuses only on UL coverage for VoIP service), regenerative satellite, mobility enhancement, support for Redcap, and store-and-forward IoT NTN [1]. According to RAN Chair summary [2], there is a need to do prioritization among the proposals and to focus on critical needs for commercial deployment.

- Strong desire to further evolve Non-Terrestrial networks (NTN) in both NR and Internet of Things (IoT) areas - The proposed possible areas for enhancements are quite broad and heavy - Need to focus on the critical needs for commercial deployments and maintain a reasonable workload - Especially considering it’s a continuation from previous releases

In this contribution, we share our view on the main candidate topics that were proposed for Rel-19 NTN in the workshop.

# Discussion

## NR NTN evolution

### Downlink coverage enhancement

Rel-18 NTN coverage enhancement focuses on UL physical channels enhancement to support direct mobile access and VoIP service [3], where techniques such as repetition and DMRS bundling are evaluated and NTN specific enhancements are discussed. For downlink, whether and how much DL coverage enhancements are needed depends on the satellite parameter assumptions. It is observed in [4] that with limited total transmission power and based on the required SNR of physical channels supported for NR NTN up to Rel-17, a satellite can only serve a small part of the potential coverage area at a time.

For downlink coverage analysis, we consider two scenarios (fixed per-beam power and fixed total satellite power), as below.

Fixed per-beam power

In TR 38.821 [5] only ERIP density is defined for satellites. The per-beam transmit power of a satellite is assumed to be fixed, and how many beams can be supported by a satellite is left to implementation based on the satellite capability. Such parameter assumption is used for link budget evaluation in Rel-18 coverage enhancement.

There are proposals in Rel-19 workshop to consider additional body loss for downlink coverage analysis.  If additional body loss is to be considered, we suggest 4 dB for S-band is used following parameters in [6].

Proposal 1: If additional body loss is to be considered for downlink link budget evaluation, 4 dB can be used.

Table 1 provides downlink coverage gap analysis for cases with and without body loss, for both LEO-600 and LEO-1200 satellite orbit. Coverage gap is defined as Gap=SNRrequired-CNR. The results and simulation parameters are based on [3]. Only satellite parameter Set-1 is considered for the coverage gap analysis, as Rel-18 NTN UL coverage enhancement takes only Set-1 as performance baseline.

Table 1 Coverage gap analysis for DL channels

| DL channels | 3GPP Satellite Parameter Set | CNR (without body loss) (dB) | Required SNR (dB) | Coverage gap (dB) | Coverage gap (with 4dB body loss) (dB) |
| --- | --- | --- | --- | --- | --- |
| PDSCH for VoIP | LEO600-set1 | -1.3 | -12.8 | -11.5 | -7.5 |
|  | LEO1200-set1 | -0.7 | -12.8 | -12.1 | -8.1 |
| PDSCH for 1Mbps | LEO600-set1 | -1.3 | -7.9 | -6.6 | -2.6 |
|  | LEO1200-set1 | -0.7 | -7.9 | -7.2 | -3.2 |
| Msg4 PDSCH | LEO600-set1 | -1.3 | -7.5 | -6.2 | -2.2 |
|  | LEO1200-set1 | -0.7 | -7.5 | -6.8 | -2.8 |
| PDCCH in USS and Type3 CSS set | LEO600-set1 | -1.3 | -10.3 | -9.0 | -5.0 |
|  | LEO1200-set1 | -0.7 | -10.3 | -9.6 | -5.6 |
| PDCCH in Type0, 1, 2 CSS set | LEO600-set1 | -1.3 | -7.7 | -6.4 | -2.4 |
|  | LEO1200-set1 | -0.7 | -7.7 | -7.0 | -3.0 |
| PSS detection (Single SSB reception) | LEO600-set1 | -1.3 | -4.5 | -3.2 | 0.8 |
|  | LEO1200-set1 | -0.7 | -4.5 | -3.8 | 0.2 |
| PSS detection (4 SSB blocks combining) | LEO600-set1 | -1.3 | -9.0 | -7.7 | -3.7 |
|  | LEO1200-set1 | -0.7 | -9.0 | -8.3 | -4.3 |

According to the results listed in Table 1, there is no gap for all the evaluated channels for both LEO600 and LEO1200 scenarios when body loss is not considered.

While if the additional 4 dB body loss is considered, the performance of PSS detection under single SSB reception is observed to have a 0.2 and 0.8 dB coverage gap for LEO1200 and LEO600, respectively. Nevertheless, PSS combining from multiple SSB blocks can be utilized at UE side by implementation given that the gaps between combined SSB block transmissions is small, e.g. within the same SS burst. Therefore, with such approach, the observed marginal coverage gap can be easily eliminated for single PSS detection. Specifically, with 4 PSS combined, the coverage gap observed above is eliminated and 3.7/4.3 dB SNR margin can be obtained for LEO600/LEO1200.

When comparing different channels, performance of DL synchronization and control channels have slightly larger SNR margin as compared to Msg4 PDSCH. Therefore if downlink becomes worse, due to e.g. reduced per-beam transmission power, Msg4 PDSCH may become the bottleneck, though the difference between different channels are not large.

Fixed total satellite power

According to analysis in [4] more than 1000 beams are needed to cover footprint of a satellite for minimum UE elevation angel of 30 degree. It is clearly not feasible to transmit such a large number of beams at the same time due to limited total power on a satellite. Therefore, beam scheduling in a TDM manner or beam hopping is needed to support system-level coverage extension. As power used in a beam is determined by the total satellite power and the maximum number of simultaneous beams, further study on potential satellite parameters in terms of total transmit power, number of simultaneous satellite beams and antenna assumption are needed.

With limited number of beams, the beam scheduled by network needs to better match the traffic characteristics to maximize the network power utilization. For example, for areas like deserts or oceans with very limited number of UEs, the SSB periodicity can be extended compared to the default 20ms for network power saving. For the UEs in connected mode, the beam hopping pattern can be indicated to the UEs. By awareness of such a pattern, e.g. the beams used for initial access and transmission in a particular area, the UE can can skip transmission and receptions during the “coverage holes” when beams are scheduled to other directions.

Proposal 2: Satellite parameter configurations including total satellite power constraint and number of simultaneous satellite beams can be studied for downlink coverage enhancement.

Proposal 3: Beam hopping extending system-level coverage and assistant information to better match corresponding traffic characteristics can be studied in Rel-19, e. g. to indicate to the UE about the beam hopping relevant information.

### Uplink coverage/capacity enhancement

Due to the large propagation distance and path loss for satellite access scenario, UL coverage performance is a bottleneck for FR1 UE to access to SAN. In Rel-17, FR1 PC3 handheld UE for satellite access has been standardized as a baseline and in Rel-18 the antenna gain of handheld UE was changed from 0dBi to -5.5dBi assuming more realistic UE devices. So the objectives related to NTN coverage enhancement were established to improve UL coverage of NTN UE in Rel-18. According to our contribution in [8], it can be observed that even with the best performance of DMRS bundling setting there is still a coverage gap of 0.26 dB. Thus, in order to address the coverage gaps, the most efficient solution is to improve UE maximum transmit power directly. Therefore NR high power UE was proposed in [9] and also in our Rel-19 RAN4 proposal [10].

When FDD high power UE is introduced for smartphones, enhancement in transmission is needed e.g. for flexible scheduling subject to SAR restriction. In the current specification for TDD band, high power UE reports UE capability of maximum UL duty cycle based on SAR restriction, i. e. the maximum UL percentage that UE can operate in high power mode. If high power transmission is allowed, the network can restrict the uplink-downlink configuration of TDD frame to ensure the UL duty cycle will not exceed the UE reported one. For FDD, which allows flexible and continuous UL scheduling, restricting the UL transmission for each frame is not needed as SAR restriction applies for a long time duration, e. g. within several minutes. To optimize the UL capacity or performance, while satisfying the SAR restriction, UE can be allowed to transmit with high power for much longer time compared to TDD, e. g. continuous UL transmission with high power for several frames and only DL reception for the next several frames. The adaptation is based on the real services. Based on the above analysis, enhancements that allow flexible UL scheduling while supporting high power transmission from UE side to improve the UL performance for FDD is needed.

Proposal 4: Enhancements that allow flexible UL scheduling while supporting high power transmission from UE side is needed for FDD high power UEs.

### Regenerative payload

Regenerative payload can reduce the delay of Uu transmission and requires less gateway compared with transparent payload. It is therefore beneficial to support regenerative payload on satellites.  One straightforward and also future proof way to support regenerative payload is to have full gNB on board.

For RAN1 and RAN2, the design of Uu interface is generally common for transparent payload and regenerative payload. One potential difference between transparent payload and regenerative payload may be the handling of delay for the Uu interface, in which case regenerative payload does not include the delay from the feeder link. In Rel-17/Rel-18, the reference point at which uplink time synchronization to be achieved is configured by the network. The UE needs to know the delay (i.e. common TA) between the reference point and gNB. In the regenerative case, the network can configure the reference point as the point of satellite with the common TA equals to 0. In this case, the regenerative payload has no impact on RAN1 and RAN2 stage 3 specification.

For RAN3, in Rel-16 NR NTN SI, the feeder link switch has been studied for the gNB on board architecture. The agreed solutions are as followings:

- The existing NG procedure without modification can support the feeder link switch over when the satellite remains in the coverage area of current AMF.

- For the case that the satellite moves into a coverage of a new AMF, the current specification has supported that the satellite/gNB setup the NG with the new AMF, while still keep the NG with the old AMF.

In summary, all the existing stage 3 procedures can already support regenerative payload, without critical needs missing. On the other hand, it is worth updating stage 2 specifications to explicitly describe what has been supported, i.e. stage 2 specification needs updates to reflect support of regenerative payload for NTN.

Proposal 5: to support regenerative payload with full gNB on board, the only specification change is to modify 38.300 specification to describe regenerative payload with full gNB on board is supported.

### Mobility enhancement

According to what has been discussed and specified in Rel-17 and Rel-18, it seems that mobility between TN-NTN is well supported including both Xn/Ng based handover and cell (re)selection procedures with assistant NTN information. There seems no critical issues to be further enhanced.

Observation 1: there are no critical issues for the TN-NTN mobility.

For NTN-NTN mobility, in Rel-18, RAN2 has agreed to support the hard satellite switch without handover command in the unchanged PCI scenario. For the soft satellite switch without handover command in the unchanged PCI scenario, RAN1 has agreed that it is feasible under the following conditions. RAN1 also confirms that the same UE behavior may be applied for soft satellite. The LS has been sent to RAN2 and RAN2 can continue the solution discussion in Rel-18. If there is no enough time to converge in Rel-18, RAN2 should continue specifying the solution for the unchanged PCI scenario in Rel-19.

Question 2: If it is feasible to support soft satellite switching without PCI change? Reply: Under the following conditions: - UE is not required to connect to two satellites simultaneously during soft satellite switching. - Interference avoidance/mitigation between two satellites may potentially be done by gNB implementation at least to ensure non-colliding SSB with same PCI at UE side. - UE is provided with the information on new common TA, K_mac, ephemeris and cell-specific K-offset are applied during resynchronization to new satellite. - UE may be provided with the information if needed to detect the SSB of the new satellite for soft satellite switching. - The same UE behavior may be applied for soft satellite switching and hard satellite switching. RAN1 concludes it is feasible for soft satellite switching without PCI change.

Proposal 6: specify soft satellite switch mechanism with unchanged PCI, if it cannot be supported in Rel-18.

During the discussion in RAN2, some companies also propose to support the satellite switch without handover command in the changed PCI scenario. In our understanding, whether to change PCI or not may up to different implementation and network planning. According to the reply LS from RAN1, in the soft satellite switch with unchanged PCI scenario, the gNB need to ensure the interference avoidance/mitigation. It may increase the complexity of gNB. In our views, the network can choose to change PCI (even when unchanged PCI is also feasible from the specification perspective) to avoid the interference between two satellites and reduce the complexity of gNB.

Proposal 7: Study and specify satellite switch without handover command for RRC_CONNECTED UEs in PCI changed scenario.

### Redcap for NTN

Redcap is introduced in Rel-17 to reduce the device cost and complexity compared to high-end eMBB devices of Rel-15/Rel-16. Three typical use cases, i.e. Industrial wireless sensors (IWSN), Video Surveillance, and Wearables. The corresponding requirements for Redcap are summarized in [7].

If Redcap is supported in NTN directly, considering the limited power and large path loss, it is necessary to evaluate whether the data rate requirement for Redcap can be satisfied. The set-1 LEO 600 in Table 6.1.1.1-1 of [5]and a bandwidth of 30 MHz are used for evaluation. Frequency reuse factor of 3 is used to reduce the inter-beam interference, and the bandwidth of each beam is 10 MHz. Based on the above assumptions, the resulted 5% user experienced data rate in NTN  is provided in Table 2 together with the data rate for TN Redcap services. The DL data rate of 347.2 kbps and UL data rate of 199.0 kbps present a large gap with the required data rate of the three typical use cases defined for TN redcap. And the large RTT would also be a challenge for the latency requirement of some specific use cases.

Table 2 Use cases and requirements

| Video Types | Data rate | E2E latency | Reliability |
| --- | --- | --- | --- |
| IWSN | <2Mbps | <100ms or 5-10 ms (safety related sensors) | 99.99% |
| Video Surveillance | 2-4 Mbps (economic video); 7.5-25 Mbps (High-end video) | <500ms | 99%-99.9% |
| Wearables | 5-50 Mbps in DL; 2-5 Mbps in UL | - | -- |
| Redcap NTN | 347.2 kbps for DL 199.0 kbps for UL | 25.77ms (RTT of LEO600) | -- |

Observation 2: With direction combination of Redcap and NTN, none of the requirements for TN Redcap use cases can be satisfied.

To support IoT services with data rate higher than NB-IoT NTN and eMTC NTN , it might be beneficial to further support redcap NTN. However, based on the above analysis, the KPIs required for TN redcap can’t be met by NTN Redcap, whether new use cases or even a new type of IoT terminal based on NR protocol should be introduced for NTN needs further discussion.

Proposal 8: Whether new use cases or even a new type of IoT terminal based on NR protocol should be introduced for NTN needs further discussion.

## IoT NTN evolution

In Rel-18 NR NTN, RAN2 introduces some enhancements for the cell reselection from NTN to TN in order to reduce the UE power consumption and also introduce the mobility enhancement in NTNs in order to reduce the signaling overhead. In our understanding, some of these features are helpful for IoT NTN. Therefore IoT NTN can reuse some Rel-18 NR NTN enhancements.

Proposal 9: Re-use the solutions introduced in Rel-18 NR NTN cell reselection enhancement for IoT NTN, if needed.

PC3 IoT NTN UE was introduced into RAN4 in Rel-18. Although UL coverage is not an issue for IoT NTN UE, the UL performance and data rate is not very good especially for GEO scenario.. With 26 dBm transmission power, it is expected that throughput can be doubled in comparison to 23 dBm transmission power.

Proposal 10: Introduce 26 dBm high power UE in Rel-19 for IoT NTN to improve the UL performance.

High power UE for IoT NTN is included in our RAN4 contribution for Rel-19 [10] as well.

# Conclusion

In the contribution, NTN enhancement in Rel-19 is discussed. We have the following observations and proposals:

Observation 1: there are no critical issues for the TN-NTN mobility.

Observation 2: If Redcap is supported in NTN, none of the requirements for TN Redcap use cases can be satisfied.

Proposal 1: If additional body loss is to be considered for downlink link budget evaluation, 4 dB can be used.

Proposal 2: Satellite parameter configurations including total satellite power constraint and number of simultaneous satellite beams can be studied for downlink coverage enhancement.

Proposal 3: Beam hopping extending system-level coverage and assistant information to better match corresponding traffic characteristics can be studied in Rel-19, e. g. to indicate to the UE about the beam hopping relevant information.

Proposal 4: Enhancements that allow flexible UL scheduling while supporting high power transmission from UE side is needed for FDD high power UEs.

Proposal 5: to support regenerative payload with full gNB on board, the only specification change is to modify 38.300 specification to describe regenerative payload with full gNB on board is supported.

Proposal 6: specify soft satellite switch mechanism with unchanged PCI, if it cannot be supported in Rel-18.

Proposal 7: Study and specify satellite switch without handover command for RRC_CONNECTED UEs in PCI changed scenario.

Proposal 8: Whether new use cases or even a new type of IoT terminal based on NR protocol should be introduced for NTN needs further discussion.

Proposal 9: Re-use the solutions introduced in Rel-18 NR NTN cell reselection enhancement for IoT NTN, if needed.

Proposal 10: Introduce 26 dBm high power UE in Rel-19 for IoT NTN to improve the UL performance.

# References

- RWS-230491, Meeting Report for RAN Rel-19 workshop.

- RWS-230488, RAN Chair’s Summary of Rel-19 Workshop

- R1-2205856, Discussion on coverage enhancement for NR NTN, Huawei, HiSilicon.

- RP-230568, Views on the coverage enhancements for non-terrestrial networks, Huawei, HiSilicon

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN).

- ITU-R M.2292-0, Characteristics of terrestrial IMT-Advanced systems for frequency sharing/interference analysis.

- RP-211574, Revised WID on support of reduced capability NR devices.

- R1-2302364, Discussion on coverage enhancement for NR NTN, Huawei, HiSilicon.

- RP-230487, New WID on Introduction of high power UE for satellite access, Huawei, HiSilicon

- RP-232383, RAN4-led topics in Rel-19, Huawei, HiSilicon

- R1-2308566, Reply LS to RAN2 on unchanged PCI, RAN1