3GPP TSG RAN Meeting #101									RP-231932

Bangalore, September 11-15, 2023

Agenda item:	8A.2.12.2

Source: 	Huawei, HiSilicon

Title: 	Multicarrier operation enhancements in Rel-19

Document for:	Discussion/Decision

# Introduction

Nowadays, 5G New Radio (NR) is being deployed on FR1 TDD bands globally (e.g. 3.5GHz and 2.6GHz), FR1 FDD bands in some countries (e.g. 700MHz or 2.1GHz), and FR2 bands in some countries (e.g. 24GHz and 39GHz). Furthermore, more spectrum in Sub3GHz deployed with 2/3/4G will be gradually refarmed to NR in the future. It can be anticipated that many operators will have quite several spectrum for NR, such as more than 4 bands. In order to improve the spectrum efficiency of multi-carrier operation, joint management of multiple bands similar to or as multi-bands serving cell is an inspiring and promising technical direction.

In Rel-18, the philosophy of multi-bands serving cell has been realized from the aspects of PDCCH and PUSCH. For PDCCH on aggregated carriers, single DCI is introduced to reduce the control overhead. For PUSCH among multiple carriers, UL Tx switching among 3 or 4 bands could fully utilize all UL carriers with limited Tx chains.

Furthermore, in Rel-19, more aspects can be considered as following.

- Joint PDSCH operation through fast carrier switching to achieve higher DL data rate.

- Joint CSI measurement operation to enhance Massive MIMO performance in CA scenario.

- Joint HARQ re-transmission operation to reduce latency.

- One cell with multiple SULs and one NUL associated with the single DL carrier.

In addition, Rel-18 leftovers such as joint PDCCH operation with different SCS among co-scheduled cells and support of more than one scheduling cell can be further discussed to enable wider range of network deployment and promote the advantages of multi-carrier scheduling.

# Discussion

## Enhancements on single DCI for multi-cell PDSCH/PUSCH scheduling

### Multi-cell scheduling with different SCS among co-scheduled cells

In Rel-18, multi-cell PUSCH/PDSCH scheduling with a single DCI is introduced to reduce the control overhead. However, due to limited TU, only same SCS among co-scheduled cells are specified, which limits the applicable scenario for this feature.

In actual network deployment, it is common that different cells in CA framework have different SCSs, e.g.  3.5GHz/2.6GH TDD + Sub-3GHz FDD. Therefore, to achieve similar benefits for TDD+FDD scenario, multi-cell PUSCH/PDSCH scheduling with a single DCI should be extended to support different SCS case, as illustrated in Figure 1.

Figure 1. Multi-cell PUSCH/PDSCH scheduling enhancement in Rel-19

Proposal 1: Support multi-cell PUSCH/PDSCH scheduling by DCI format 0_3/1_3 with different sub-carrier spacing among co-scheduled cells in Rel-19.

### Support more than one scheduling cell for the scheduled cell

In current standard, when the SCell is configured to be scheduled by other cells, the SCell cannot schedule itself unless it is reconfigured by RRC. If single DCI for multi-cell scheduling and legacy DCI for single-cell scheduling can only be monitored on the same scheduling cell, the control load on the scheduling cell will be heavy and it is possible to lead to CCE resource allocation failure.

To solve this problem, multi-cell scheduling DCI from one scheduling cell and single-cell scheduling DCI via self-scheduling can be supported. Figure 2 shows an example that PCell jointly schedules PCell and SCell by single DCI while the SCell can still support scheduling the SCell itself by legacy DCI. In this way, the DL control load on the scheduling cell can be mitigated and it will be beneficial for control channel load balance.

Figure 2. PCell jointly schedules PCell and SCell while the SCell can still schedule the SCell itself

Proposal 2: Multi-cell scheduling from one scheduling cell by single DCI and single cell scheduling from the scheduled cell via legacy DCI self-scheduling can be supported.

## Fast DL carrier switching across the configured DL carriers

It is common that the number of carriers available at the network side is more than that can be supported by UE CA capability, especially more and more spectrum will be deployed in the future by operators. To enable UE to make full use of the spectrum resource, network can decide the best carrier for PDSCH transmission based on the traffic and/or channel condition, and indicate UE to switch among carriers, as shown in Figure 3.

Figure 3. Dynamic switch among DL carriers

In Rel-17, the Network configures serving cell(s) and BWP(s) configuration to comply with capabilities, regardless whether the cells/BWPs are activated or not. In this case, cell switching is indicated by L3 signaling, where the interruption time is ~50ms for PCell and SCell. In Rel-18, the network pre-configures more cell(s) including serving cell(s) and candidate cell(s), which are cells relevant to UE CA capability. In this case, cell switching is indicated by L2 signaling, where the interruption time for PCell is 6~10ms and the interruption time for SCell is 20~50ms since SCell switching is not specifically enhanced in RAN2 and RAN4 in Rel-18.

Thus, it can be found that the interruption time is quite long based on the current switching mechanisms especially considering SCells, which would have significant impact on the UE experience. It is promising to do further enhancements in Rel-19 to support fast DL carrier switching.

One potential direction is to support carrier switching via L1-based BWP switching to reduce the interruption time. The network can configure multiple BWPs in each cell with at least one dormant BWP, only the activated carrier with non-dormant BWP as the current BWP is counted as one CA carrier consisting of the UE CA capability, i.e. the number of configured activated carriers can be larger than the number of DL carriers supported by UE CA capability. Then the Rel-15 1~2ms BWP switching can be reused to switch transmission cells. For example, as shown in Figure 4, network configures and activates 4 cells while the UE only has 2 simultaneous PDSCH transmission capability. In this case, the network can set two of the SCells as dormant BWP, which doesn’t occupy the PDSCH processing capability.

Figure 4. Carrier switch via L1-based BWP switching in Rel-19

Table 1 presents the interruption time for PCell switching and SCell switching in Rel-18 and Rel-19. The SCell switching delay is ~50ms in Rel-18. In Rel-19, the switching delay can be reduced to ~1ms by reusing the BWP switching mechanism. The corresponding evaluation result of DL UPT is provided in Figure 5, for which detailed simulation assumptions can be found in the Appendix A.

Table 1. Interruption time for PCell switching and SCell switching

|  | Rel-18 | Rel-19 |
| --- | --- | --- |
| PCell switching (with SCell or without) | 6ms | 6ms |
| SCell switching | 50ms | 1ms |

It can be observed that Rel-19 carrier switch via L1-based BWP switching can achieve about 25% gain compared with Rel-18. The performance gain comes from adapting the limited capability according to channel and traffic condition at TTI-level.

Figure 5. Performance evaluation results for DL UPT

As analyzed above, support fast DL carrier switching via L1-based BWP switching can achieve fast DL carrier switching with ~millisecond level switching delay. It can reuse the existing BWP switching mechanism, and thus there is almost zero impact in RAN1, which has the benefit of achieving promising gain with very limited standard effort.

Some other mechanisms to achieve faster DL carrier switching (e.g. ~hundreds of μs level) are proposed by other companies, which can be considered also, e.g. via RRC signaling to configure semi-static switching pattern or via other L1 signaling to achieve DL Rx switching in similar way as UL Tx switching.

Proposal 3: Support fast DL carrier switching, assuming that the number of configured activated carriers can be larger than the number of DL carriers supported by UE CA capability.

## Enhancements for efficient multi-carrier operation

### Joint CSI measurement across multiple carriers with one CSI report

Currently, the CSI is measured individually for each carrier. Therefore, Massive MIMO in CA scenario requires high UE complexity of CSI computation, which increases with the number of carriers. It is even worse considering mTRP. However, the capability to measure CSI is limited for each UE, e.g., 32 ports/all CC. Thus, with such limited capability, UE cannot obtain full channel information with limited capability of CSI measurement. For example, as shown in the Figure 6, for an antenna array with 32 Tx, the channel of three carriers is needed to be measured. With limited capability of CSI measurement, e.g., 32 ports/all CC, only CSI of 8 port can be measured for each carrier. As a result, there will be great performance loss due to the lack of full channel information.

Figure 6. Example of individual CSI measurement for each carrier

However, UE can measure multiple co-located neighboring carriers jointly if the total bandwidth of all carriers doesn’t exceed the maximum bandwidth per carrier, as shown in the Figure 7. Furthermore, by limiting the number of sub-bands across all the measured carriers, the joint CSI measurement has the similar UE complexity as measurement of single carrier in Rel-18. The performance comparison is given in Figure 8, for which detailed simulation assumptions can be found in the Appendix B. It can be observed that joint CSI measurement can provide 8%~30% performance gain with Type I codebook.

Figure 7. Example of joint CSI measurement among multiple carriers

Figure 8. Performance gain of joint CSI measurement

Since the CSI for multiple carriers are measured jointly, it is straightforward to report the measurement results in one CSI report.

Proposal 4: Support joint CSI measurement across multiple carriers with one CSI report.

### Cross-cell HARQ retransmission

Latency reduction can be achieved by cross-cell related features (i.e. cross-cell scheduling, cross-cell HARQ-ACK feedback and cross-cell HARQ retransmission), especially for TDD CA with different UL/DL configurations. In existing specification, some cross-cell related features have been supported, such as SCell cross-scheduling PCell while PCell can still schedule itself, single DCI cross-scheduling multiple cells and cross-cell HARQ-ACK feedback with DCI based mode or semi-static mode by slot-based PUCCH-cell pattern. However, some features like cross-cell HARQ retransmission is still missing and the advantages of multi-carrier scheduling cannot be fully utilized.

During the process of transmission, if one TB is not received correctly by UE, the gNB will retransmit it to guarantee the network performance. Since cross-cell HARQ retransmission has not been supported, once the TB on the scheduled cell is not received correctly, the network needs to wait for next proper scheduling opportunity to re-transmit TB on the same carrier with initial TB transmission, which will lead to large latency.

To solve this problem, cross-cell HARQ retransmission mechanism can be introduced to reduce the latency. As shown in Figure 9, when the TB transmitted on SCell is fail to be received, gNB can retransmit the TB on PCell with shorter latency, which facilitates transmission efficiency improvement.

Figure 9. Cross-cell HARQ retransmission

Proposal 5: Support cross-cell HARQ retransmission to reduce the latency.

### One cell with multiple SULs and one NUL associated with the single DL carrier

To support higher UL throughput, R18 UL Tx switching requires UE to support 3 or 4 CC DL CA, though there is no requirement on DL for UL heavy scenarios. Typically, UE only supports 2 DL CCs. In order to better support UL-heavy scenario with low DL UE complexity, it is beneficial to support a cell with multiple SULs, e.g. 1 cell = 1 DL + 1 NUL + 2 or more SUL. An example is as shown in Figure 10.

Figure 10. Multi-SUL cell (1 cell = 1 DL + 1 NUL + 2 SUL)

Proposal 6: Support one cell with multiple SULs and one NUL associated with the single DL carrier in Rel-19.

## Summary of potential directions for further discussions for Rel-19 MC WI

Based on the above discussions, the following directions can be set for further discussions for Rel-19 MC WI:

- Enhancements on single DCI for multi-cell PUSCH/PDSCH scheduling

- Multi-cell PUSCH/PDSCH scheduling by DCI format 0_3/1_3 with different sub-carrier spacing among co-scheduled cells

- More than one scheduling cell for a scheduled cell, where at least one scheduling cell configured with single DCI

- Fast DL carrier switching, assuming that the number of configured activated carriers can be larger than the number of DL carriers supported by UE CA capability

- FFS ~millisecond level switching delay or ~microsecond level switching

- Enhancements for more efficient multi-carrier operation on intra-band contiguous, intra-band non-contiguous or inter-band spectrum

- Joint CSI measurement across multiple carriers with one CSI report for all the carriers

- Cross-cell HARQ retransmission

- One cell with multiple SULs and one NUL associated with the single DL carrier

# Conclusion

In this paper, we provide our views on further enhancements for multi-carrier operation in Rel-19. Based on the above discussion, we have following proposals:

Proposal 1: Support multi-cell PUSCH/PDSCH scheduling by DCI format 0_3/1_3 with different sub-carrier spacing among co-scheduled cells in Rel-19.

Proposal 2: Multi-cell scheduling from one scheduling cell by single DCI and single cell scheduling from the scheduled cell via legacy DCI self-scheduling can be supported.

Proposal 3: Support fast DL carrier switching, assuming that the number of configured activated carriers can be larger than the number of DL carriers supported by UE CA capability.

Proposal 4: Support joint CSI measurement across multiple carriers with one reported CSI report.

Proposal 5: Support cross-cell HARQ retransmission to reduce the latency.

Proposal 6: Support one cell with multiple SULs and one NUL associated with the single DL carrier in Rel-19.

In addition, the following directions can be set for further discussions for Rel-19 MC WI:

- Enhancements on single DCI for multi-cell PUSCH/PDSCH scheduling

- Multi-cell PUSCH/PDSCH scheduling by DCI format 0_3/1_3 with different sub-carrier spacing among co-scheduled cells

- More than one scheduling cell for a scheduled cell, where at least one scheduling cell configured with single DCI

- Fast DL carrier switching, assuming that the number of configured activated carriers can be larger than the number of DL carriers supported by UE CA capability

- FFS ~millisecond level switching delay or ~microsecond level switching

- Enhancements for more efficient multi-carrier operation on intra-band contiguous, intra-band non-contiguous or inter-band spectrum

- Joint CSI measurement across multiple carriers with one CSI report for all the carriers

- Cross-cell HARQ retransmission

- One cell with multiple SULs and one NUL associated with the single DL carrier

# Appendix A

Table 2: System level simulation assumptions for fast DL carrier switching

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Deployment | 3GPP Dense Urban, 7 sites, 3 sectors/site |  |  |
| Frequency | 3.5 GHz | 2.1 GHz | 1.8 GHz |
| Bandwidth | 100 MHz + 100 MHz | 50 MHz | 50 MHz |
| BS antenna | TDD: 32T; FDD: 32T |  |  |
| UE antenna | Total 8R (max. 4 per band) |  |  |
| ISD | 200m |  |  |
| BS Tx power | 49 dBm |  |  |
| Number of UEs | 840/ All cells |  |  |
| MIMO mode | MU-MIMO with rank adaptation |  |  |
| Traffic model | FTP3, 1Mbyte/packet |  |  |
| Scheduling | PF |  |  |
| Simultaneous PDSCH capability | 2 |  |  |

# Appendix B

Table 3: System level simulation assumptions for joint CSI measurement

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Deployment | UMA, 21 cells |  |  |
| Frequency | 1.8 GHz | 2.1 GHz | 2.6 GHz |
| Bandwidth | 10 MHz | 10 MHz | 10 MHz |
| BS antenna | 32T |  |  |
| Channel mdoel | 3GPP channel model (Correlation modelling for multi-frequency simulations) |  |  |
| Traffic model | FTP3 |  |  |
| CSI-RS period | 40ms |  |  |
| Codebook | Type I |  |  |
| TAE among multi-CCs | 0~3us |  |  |