3GPP TSG RAN meeting #101										RP-231944

Banglaore, India, September 11th -15th 2023

## Status Report to TSG

Agenda item:			9.2.3

| WI / SI Name | Study on Self-Evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_IMT_2020_Sat_eval |  |  |  |  |
| Unique ID |  |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-231945 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2023 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 40 % | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN (ITU-R Ad-Hoc) |
| --- | --- | --- |
| Rapporteur | Name | Asbjörn Grövlen |
|  | Company | Ericsson |
|  | Email | asbjorn.grovlen@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.0	RAN

#### 2.0.1	Agreements

#### RAN#101

The following discussions took place:

- Presentation of description templates RP-232314 (IoT NTN) and RP-232591 (NR NTN)

  - Documents will be further reviewed on the ITU adhoc reflector.

- pCR on spectral efficiency and peak data rate agreed in RP-231946

- 37.911 v0.1.0 agreed in RP-232645

- Updated SID agreed in RP-231945

#### 2.0.2	Remaining Open issues

The following issues remain open

- Review of description templates

- Submission and review of compliance templates

- Development of the technical report

- Finalization of the submission to ITU

## 2.1	RAN1

#### 2.1.1	Agreements

#### RAN1#114

Agreements on evaluation methodology

Agreement

For DL and UL overhead:

- DL overhead is assumed to be 0.14

- UL overhead is assumed to be 0.08

Agreement

For the evaluation metrics that require usage of beam area, a beam area of 1415 km2 is assumed.

Agreement

The modeling of delay / RTT for SLS is up to companies to report.

Agreement

All the beams are considered for computing the area of connection density.

Agreement

For SLS to LLS metric of connection density, “pre-processing SINR” should be used instead of “pre-processing SNR”.

Agreement

Elevation angle of 90° is used for determining mean and standard deviation values of K-factor and delay spread for NTN TDL-C Rural channel model.

Agreement

For mMTC LLS (NR and eMTC), 16-QAM can be used in addition to QPSK to derive the SNR to SE mapping.

Agreement

For connection density evaluation with full buffer system-level simulation followed by link-level simulation, for SINR CDF distribution derivation, all the beams are assumed to be fully loaded.

Observation:

Based on RAN1 evaluations, for peak data rate and spectral efficiency the max code rate and modulation order is:

- For DL, the modulation scheme is Qm=6, and Rmax between 666/1024 and 822/1024

- For UL, the modulation scheme is Qm=4, and Rmax between 434/1024 and 553/1024

Agreement

3GPP will report peak data rate and spectral efficiency based on

- Rmax=822/1024 and Qm=6 for DL

- Rmax=553/1024 for and Qm=4 for UL

Agreement

For the link budget template:

- The attached xls to R1-2308233 is endorsed as the starting point for the link budget template.

- The target elevation angle is [30] degrees, and the link budget template includes the link margin (in dB) at that elevation angle.

- The LLS to derive the “required SNR” for the link budget template is performed under an NTN TDL-C rural channel model with the K-factor corresponding to the target elevation angle.

- The rest of the parameters (bits / #reps / others) and whether different values are used for different scenarios (eMBB-s, HRC-s, mMTC-s) are to be discussed via email discussion.

Agreements for self-evaluation results for NR NTN:

Agreement

The results of CL, Geometry SIR and Geometry SINR simulated on DL are to be reported within the attached template.

Attachment: R1-2306416_Calibration_results v00

Agreement

The attached template is used to collect evaluation results for eMBBs peak spectral efficiency and peak data rate.

- A single value should be captured in the TR 37.911, which is calculated based on the modulation order (Q_m) and maximum coding rate once they are agreed in 9.14.1

- There is no need to collect input for DL&UL OH in the template.

Attachment: 1. R1-2306416 - Att eMBBs_ Peak spectral efficiency_Peak data rate v01

Agreement

Downlink peak spectral efficiency is evaluated based on analytical method. The evaluation result is provided in the following Table.

The detailed assumptions will be captured in the TR 37.911.

NR NTN DL peak spectral efficiency (bit/s/Hz)

| SCS [kHz] |  | 30 MHz | Req. |
| --- | --- | --- | --- |
| FR1 | 15 | [3.71] | 3 |

Based on the above, NR NTN fulfils DL peak spectral efficiency requirement.

Agreement

Uplink peak spectral efficiency is evaluated based on analytical method. The evaluation result is provided in the following Table.

The detailed assumptions will be captured in the TR 37.911.

NR NTN UL peak spectral efficiency (bit/s/Hz)

| SCS [kHz] |  | 1.44 MHz | Req. |
| --- | --- | --- | --- |
| FR1 | 15 | [1.85] | 1.5 |

Based on the above, NR NTN fulfils UL peak spectral efficiency requirement

Agreement

DL peak data rate for NR NTN is evaluated based on the evaluation results of NR NTN peak spectral efficiency. Using the analytical way as provided in Report ITU-R M.2514. The evaluation result is provided in the following Table. It is observed that NR fulfils the DL peak data rate requirement.

NR NTN DL peak data rate (Mbit/s)

| SCS [kHz] |  | 30 MHz | Req. |
| --- | --- | --- | --- |
| FR1 | 15 | [111] | 70 |

Agreement

UL peak data rate for NR NTN is evaluated based on the evaluation results of NR NTN peak spectral efficiency. Using the analytical way as provided in Report ITU-R M.2514. The evaluation result is provided in the following Table. It is observed that NR fulfils the UL peak data rate requirement.

NR NTN UL peak data rate (Mbit/s)

| SCS [kHz] |  | 1.44 MHz | Req. |
| --- | --- | --- | --- |
| FR1 | 15 | [2.76] | 2 |

Agreement

The attached text proposal for TR 37.911 is endorsed from RAN1 perspective. Note: the detailed structure of the TR is up to the TR editor. Attachment: TP to TR37.911-Peak spectral efficiency and peak data rate-v02

Agreement

The attached template is used to collect evaluation results for eMBBs 5th percentile user spectral efficiency, average spectral efficiency, user experienced data rate and area traffic capacity

Note: OH as agreed in 9.14.1 will be removed from the template.

Attachment: R1-2306416 - Att eMBBs_Spectral Efficiency_User data rate_Area traffic capacity_v01

Agreement

The attached template is used to collect evaluation results for the TPR on mobility

Attachment: R1-2306416 Att eMBB-s_Mobility _v00

Agreement

The attached template is used to collect evaluation results for the TPR on Reliability

Attachment: R1-2306416  Att_ HRC-s-Reliability_v00

Agreement

The attached template is used to collect evaluation results for the TPR on connection density

Attachment: R1-2306416  Att_ConnectionDensity -v002

Agreement

RAN1 to provide at RAN1#114-bis evaluation results on Energy efficiency, including both network and device.

Agreement

For NR NTN Connection Density LLS:

- Agree 10% BLER as target for throughput for Full Buffer

- SINR - user spectral efficiency (SE) mapping graph for Full Buffer, where:

  - SE = nominal SE × (1-BLER)

Agreements for self-evaluation results for NR NTN:

Conclusion

No discussion needed regarding whether E-UTRA operating bands 255 and 256 can be used for Rel-17 IoT NTN, as it is already clear that this point does not affect the submission.

Agreement

For IoT NTN Connection Density LLS:

- Agree 10% BLER as target for throughput for NB-IoT Full Buffer and eMTC Full Buffer

- Companies to use QPSK-π/4 as well as BPSK-π/2 in evaluations for NB-IoT Full Buffer.

- Each company to provide SINR - user spectral efficiency (SE) mapping graph for NB-IoT Full Buffer and eMTC Full Buffer, where:

  - SE = nominal SE × (1-BLER)

#### 2.1.2	Remaining Open issues

Provide self-evaluation results against technical performance requirements for eMBB-s, including

  - User experienced data rate

  - 5th percentile user spectral efficiency

  - Average spectral efficiency

  - Area traffic capacity

  - Energy efficiency, including both network and device

  - Mobility

Provide self-evaluation results against technical performance requirements for mMTC-s, including

  - Connection density

## 2.2	RAN2

#### 2.2.1	Agreements

#### RAN2#123

Agreements:

- At the moment, RAN2 assumes the best-case scenario even though RAN2 understands that it might not be a common scenario in some cases. Additional scenarios can be considered during the self evaluation work

- RAN2 assumes that both UE and gNB are located at the satellite’s nadir, i.e., elevation angles are 90 degrees, for the calculation of round trip delay (RTD).

- Given the assumptions of Proposal 1, feeder and service link delays are included in the propagation delay computation (RTD).

- For the mobility interruption evaluation, RAN2 assumes that for now it is sufficient to consider beam-based mobility in NTN.

- From RAN2 perspective, satellite on-board delay can be considered negligible.

- RAN2 assumes the CP procedure defined in Figure 1 as the baseline for the CP evaluation.

- For the best-case scenario, RAN2 assumes a lossless scenario (p=0) for the User plane evaluation / RAN2 will not consider retransmissions.

8.   RAN2 assumes the following for the evaluation of CP and UP latency:

-        NR FDD

-        Only NTN bands are considered (n255, n256).

-        UE capabilities 1 & 2

-        Resource type mapping A &B

-        SCS 15 kHz for the baseline scenario. FFS other supported scenarios (e.g., 30 kHz).

[Post123][102]NTN Self Ev] CP/UP latency (Ericsson)

Scope: discuss the actual numbers for CP/UP latency and potentially draft a corresponding TP

Intended outcome: email discussion summary

Deadline: Long

#### 2.2.2	Remaining Open issues

Provide self-evaluation results against technical performance requirements for eMBB-s, including

  - Latency, including user plane latency and control plane latency

  - Mobility interruption time

## 2.3	RAN3

#### 2.3.1	Agreements

No

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

No discussion

#### 2.4.2	Remaining Open issues

Provide self-evaluation results for other requirements (including bandwidth) as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2, RAN4]

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

## 4.	References

#### RAN1#114

R1-2306407	Simulation results for the study on self-evaluation towards the IMT-2020 of the 3GPP Satellite RIT			THALES

R1-2306414	Feature Lead Summary #1 on self-evaluation results for NR NTN	Moderator (THALES)

R1-2306415	Feature Lead Summary #2 on self-evaluation results for NR NTN	Moderator (THALES)

R1-2306416	Feature Lead Summary #3 on self-evaluation results for NR NTN	Moderator (THALES)

R1-2306417	Feature Lead Summary #4 on self-evaluation results for NR NTN	Moderator (THALES)

R1-2306507	Evaluation methodology of IMT-2020 satellite	Huawei, HiSilicon

R1-2306508	Self-evaluation results for NR NTN	Huawei, HiSilicon

R1-2306509	Self-evaluation results for IoT NTN	Huawei, HiSilicon

R1-2306567	Discussion on the evaluation methodology of the self-evaluation	ZTE

R1-2306568	Discussion on the Self-evaluation results for NR NTN	ZTE

R1-2306569	Discussion on the Self-evaluation results for IoT NTN	ZTE

R1-2307067	Initial evaluation result for NR NTN	CATT

R1-2307247	Remaining discussion on evaluation methodology for IMT-2020 Satellite submission	Nokia, Nokia Shanghai Bell

R1-2307248	Initial evaluation results related to IMT-2020 Satellite for NR over NTN	Nokia, Nokia Shanghai Bell

R1-2307362	Self-evaluation results for NR NTN	xiaomi

R1-2307407	Initial evaluation results on NR-NTN for self-evaluation of IMT-2020 satellite radio interface technology			Panasonic

R1-2307496	Calibration results and peak data rate analysis for NR NTN	NTT DOCOMO, INC.

R1-2307551	Discussion on self-evaluation methodology for IMT-2020 satellite radio interface	OPPO

R1-2307552	Self-evaluation results for NR NTN	OPPO

R1-2307553	Self-evaluation results for IoT NTN	OPPO

R1-2307706	Discussion on remaining issues for evaluation methodology	Samsung

R1-2307707	Discussion on evaluation results for NR NTN	Samsung

R1-2307708	Discussion on evaluation results for IoT NTN	Samsung

R1-2307729	Consideration on IMT-2020 Satellite self-evaluation	THALES

R1-2307774	Satellite IMT-2020 self-evaluation: Peak data rate and spectral efficiency	Ericsson

R1-2307955	Further discussion on self-evaluation assumptions	Qualcomm Incorporated

R1-2307956	Self-evaluation results for NR NTN	Qualcomm Incorporated

R1-2307957	Self-evaluation results for IOT NTN	Qualcomm Incorporated

R1-2308033	Self-evaluation results for IoT NTN	MediaTek Inc.

R1-2308233	Feature lead summary on Evaluation Methodology for IMT-2020 satellite	Moderator (Qualcomm Incorporated)

R1-2308410	Feature Lead summary for 9.14.3: Self-Evaluation Results for IoT NTN	Moderator (MediaTek)

R1-2308550	Session notes for 9.14 (Study on Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology)	Ad-Hoc Chair (Huawei)

R1-2308748	[Post-114-ITU-Sat-01]  Email discussion on link budget template	Moderator (Qualcomm Incorporated)

#### RAN2#122

R2-2307322	Discussion on IMT-2020 Satellite self-evaluation for Latency and Mobility	THALES

R2-2307496	Self-Evaluation for NR NTN	Huawei, HiSilicon

R2-2307586	On CP and UP Latency for IMT-2020 NTN Self Evaluation	Nokia, Nokia Shanghai Bell

R2-2307624	RAN2 aspects on evaluation methodology for IMT-2020 Satellite	Qualcomm Incorporated

R2-2308508	Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology				ZTE Corporation, Sanechips

R2-2308903	Satellite IMT-2020 self-evaluation: CP latency	Ericsson

R2-2308905	Satellite IMT-2020 self-evaluation: UP latency	Ericsson

R2-2308982	CP/UP latency assumptions	Ericsson

#### RAN#101

RP-231560	Inputs on study on self-evaluation towards IMT-2020 submission of the 3GPP Satellite Radio	Qualcomm Incorporated

RP-231944	Status report for SI: Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology; rapporteur: Ericsson	Ericsson

RP-231945	Revised SID: Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology	Ericsson

RP-231946	pCR for TR 37.911 on peak spectral efficiency and data rate 	Ericsson

RP-232314	Update on IMT-2020 submission of 3GPP satellite RITs [IoT NTN]	MediaTek Inc.

RP-232591	Characteristics template for 3GPP 5G NR satellite access RIT (Release 17 and beyond)	Thales

RP-232645	TR 37.911 v0.1.0 on Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology	Ericsson