3GPP TSG RAN Meeting #101	RP-231948

Bangalore, India, September 11-15, 2023

Agenda Item:	8A.2.12.1

Source:	Ericsson

Title:	On a Rel-19 WI for low-power WUS and WUR for NR

Document for:	Discussion, Decision

# 1	Introduction

In the ongoing Rel-18 study ‘Study on low-power Wake-up Signal and Receiver for NR’ [1], different solutions for LP-WUS design, WUR architecture, and procedures, and the expected gains from those, are being evaluated. Based on the results in the draft SI TR [2], in this contribution we present our view on the content of a subsequent Rel-19 low-power WUS/WUR work item.

# 2	Discussion

The focus of the LP-WUS/WUR SI is IoT and wearables:

The study should primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables.

Since this is a subset of all UEs in the network, and since the WUS/WUR is a feature targeting UE energy consumption gain, it is beneficial if the required network changes for supporting WUS are small. That is, to ensure quick and widespread network support of the WUS/WUR feature it is beneficial to be able to enable WUR in gNB with a minimum of changes. More specifically, any new gNB hardware or new emissions/compliance requirements should be avoided.

Regarding WUS coexistence, it should be ensured that enabling WUR support does not have noticeable negative impacts on the system performance. First, when no WUS is transmitted to any UE in the cell, it should be ensured that the WUS/WUR feature has no negative impact at all. I.e., it must be possible to reuse the WUS radio resources for legacy transmissions to ensure spectral efficiency remains the same. Second, when WUS is transmitted to a UE in the cell, it must be possible to multiplex the WUS transmission with legacy NR transmissions and also to minimize the interference caused to legacy NR transmissions. Third, broadcast of new periodic signals for WUR has negative impact on control signalling overhead and network energy efficiency, also when no WUS is transmitted in the cell, and should be avoided (see LP-SS discussion below).

To ensure flexibility, both for the coexistence with the legacy system and for the WUR solution itself, it should be possible for the WUR to operate in different frequency bands, and with a flexible frequency location/allocation within the carrier according to configuration.

As mentioned, WUS/WUR is a feature for UE energy consumption reduction, and high energy consumption is a more severe problem for cell-edge UEs. Therefore, it is preferable if WUS coverage is not restricted to the central parts of the cell, so-called partial WUS coverage, such that the WUR gains apply also to cell-edge UEs, providing a full cell coverage.

Figure 1: Schematic illustration of WUS and main receiver coverage.

Moreover, specification impact and complexity will be considerable larger for partial WUS coverage since new procedures must be introduced for when the UE moves in or out of the WUS coverage in the cell. It could even be the case that the UE becomes unreachable in the downlink if it incorrectly assumes it is within WUS coverage when it is not. Since the LP-WUS/WUR addresses not only IoT use cases but also wearables and smart watches, DL reachability and paging performance must not be worse than legacy to ensure there is no deterioration of user experience. Therefore, WUS coverage should be provided with full cell coverage (matching paging PDCCH) and with the same paging success rate as legacy paging.

For full benefit of LP-WUS/WUR to end users, also at cell-edge, full cell WUS coverage without reduction of the paging success rate should be targeted in Rel-19.

From the SI results and conclusions of the draft TR [2], the gains compared to baseline for duty-cycled WUR operation in RRC_IDLE and RRC_INACTIVE are the largest ("up to more than 90%”, i.e., larger than in RRC_CONNECTED, and/or using continuous WUR.  Therefore, specifying support for this should be the main objective of the Rel-19 WI.

The Rel-19 scope can be contained by first specifying the solutions with the biggest gain in Rel-19: duty-cycled LP-WUS/WUR in Idle/Inactive.

From the evaluation results in the draft SI TR [2], it is clear that the main WUR gain comes from being able to keep the main receiver in a sleep state, e.g., the deep sleep/ultra-deep sleep state. The effect of the active WUR power does not have significant impact on the WUR gain when duty cycle is large. For example, in [3] it is shown that for the scenario with a 1.28 s DRX cycle, even for large increase in WUR active power, the WUR gain only decreases marginally e.g., from about 94% to 93%,

However, the price to pay for such marginal additional gain is high: if a less capable WUR with lower active power could not reuse legacy PSS/SSS for synchronization and RRM measurements, a new low-power sync signal, LP-SS, would have to be introduced. This means that a new always-on signal needs to be introduced for supporting one single feature which is against the lean NR design principle and may have negative impact on NW energy efficiency. Operation with less capable WUR also add significantly more system overhead to legacy paging procedures. For example, as discussed in [3], assuming 250 Idle UEs system overhead is increased by more than 50x-100x compared to baseline paging overhead for less capable WUR while with more capable WUR the power savings can be achieved with <1.25x overhead.

Also, the WUS monitoring time to achieve a given link performance target (e.g., considering coverage, payload size, etc.) for a less capable WUR would be longer compared with that needed for a more capable WUR. When such increased WUS monitoring time is considered together with the active WUR power during the monitoring time, the power saving gains from less capable WUR can become smaller/worse than gains achievable by a more capable WUR.

Based on the above, we propose the following Rel-19 WI scope, objectives, and design requirements:

Specify duty-cycled LP-WUS/WUR for RRC Idle/Inactive in Rel-19 with the following design requirements:

  - For gNB, transmission of LP-WUS should not require new hardware nor trigger new emissions/compliance requirements.

  - It should be possible to dynamically reuse unused LP-WUS radio resources for other NR transmissions.

  - It should be possible to multiplex LP-WUS with other NR transmissions in time or frequency domain without causing interference.

  - WUR should support band and carrier tuning and flexible frequency location within a carrier.

  - As a baseline for WUS monitoring, WUS monitoring is associated to PO/PEI occasion.

  - For UE subgrouping, PEI mechanism is used as baseline.

  - WUR operation should not deteriorate legacy paging reception success rate.

  - LP-WUS and WUR should have full coverage in the cell (matching paging PDCCH).

  - WUR architecture should be capable of legacy SSS reception.

  - WUR uses existing OFDMA based signals (SSS) for sync and serving cell measurements (i.e., no new LP-SS).

# Conclusion

The largest UE energy consumption reduction (>90% compared to baseline) has been identified for duty-cycled WUR in RRC Idle/Inactive. Therefore, this should be the main track of the Rel-19 WUR WI. To maximize the applicability of the WUR gain, WUS should have full coverage in the cell to reach all UEs, and no new HW should be required in gNB for widespread feature support. WUS should have good coexistence properties with legacy, and when WUS is not transmitted there should be no negative impact to legacy, i.e., ensuring dynamic reuse of radio resources and no additional overhead from new always-on broadcast signals, i.e., low-power sync-signal (LP-SS). The major part of the WUR gain comes from keeping the main receiver in a sleep state, and a somewhat higher active power for a WUR capable of legacy SSS reception has an insignificant impact on the WUR gain.

Observation 1	For full benefit of LP-WUS/WUR to end users, also at cell-edge, full cell WUS coverage without reduction of the paging success rate should be targeted in Rel-19.

Observation 2	The Rel-19 scope can be contained by first specifying the solutions with the biggest gain in Rel-19: duty-cycled LP-WUS/WUR in Idle/Inactive.

Based on the discussion in the previous sections we propose the following scope for a Rel-19 WI:

Proposal 1	Specify duty-cycled LP-WUS/WUR for RRC Idle/Inactive in Rel-19 with the following design requirements:

a.	For gNB, transmission of LP-WUS should not require new hardware nor trigger new emissions/compliance requirements.

b.	It should be possible to dynamically reuse unused LP-WUS radio resources for other NR transmissions.

c.	It should be possible to multiplex LP-WUS with other NR transmissions in time or frequency domain without causing interference.

d.	WUR should support band and carrier tuning and flexible frequency location within a carrier.

e.	As a baseline for WUS monitoring, WUS monitoring is associated to PO/PEI occasion.

f.	For UE subgrouping, PEI mechanism is used as baseline.

g.	WUR operation should not deteriorate legacy paging reception success rate.

h.	LP-WUS and WUR should have full coverage in the cell (matching paging PDCCH).

i.	WUR architecture should be capable of legacy SSS reception.

j.	WUR uses existing OFDMA based signals (SSS) for sync and serving cell measurements (i.e., no new LP-SS).

# References

RP-222644, ‘Revised SID: Study on low-power Wake-up Signal and Receiver for NR’, 3GPP TSG-RAN#97e, Sept 12 - 16, 2022.

R1-2307829, Draft TR 38.869, “Study on low-power wake up signal and receiver for NR”, v0.3.0, RAN1#114, Aug 2023.

R1-2308438, “Low power WUS evaluations”, Ericsson, RAN1#114, Toulouse, France, August 2023.